3GPP TSG RAN WG1 #121		 	R1-2506282

Bengaluru, India, Aug 25th – 29th, 2025

Source:	NTT DOCOMO, INC.

Title:	Maintenance of NR-NTN uplink capacity/throughput enhancement

Agenda Item:	8.11.3

Document for: 	Discussion/Decision

Introduction

At the RAN1#121 meeting, it was concluded that R19 NR-NTN WI was completed. In this contribution, maintenance of UL capacity/throughput enhancement will be discussed, and several proposals will be submitted.

Discussion

Phase continuity for PUSCH with OCC

In RAN1#120bis meeting [1], phase continuity for PUSCH with OCC has been discussed. Following agreement was made.

Agreement RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling

An LS was sent to RAN4 for checking whether the same phase continuity requirements as for DMRS bundling can be applied for OCC under the same conditions. In our understanding, regarding the conditions for maintaining phase continuity and power continuity, it should be confirmed that the same conditions that phase continuity applies for DMRS bundling can be directly reused for the duration of one OCC group with PUSCH, which can be further updated if RAN4 provide different views.

Proposal 1:

RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH under the same conditions that phase continuity and power consistency applies for DMRS bundling.

- Adopt following TP for TS38.214.

6.1.2.1 Resource allocation in time domain <Unchanged parts omitted> The UE shall maintain power consistency and phase continuity within an [OCC group] of PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, or 0_2, or PUSCH repetition Type A with a configured grant if any event described in Clause 6.1.7 is not occurring within the OCC group; otherwise, PUSCH transmissions within the OCC group are dropped. <Unchanged parts omitted>

As agreed, phase continuity and power consistency will be maintained within an OCC group. For example, for OCC length 4, this requirement needs to be satisfied during each 4 slots. As specified in R17/18, phase continuity and power consistency needs to be maintained within an actual TDW of DMRS bundling. For example, value 8 may be reported via UE capability FG 44-2 for the maximum duration, and nominal TDW length 6 can be configured via RRC signaling.

When OCC and DMRS bundling are enabled at the same time, there may be a case where UE needs to maintain phase continuity and power consistency beyond the reported value of the maximum duration for DMRS bundling. Figure 1 shows an example of this issue. In this example, OCC length 4 and nominal/actual TDW length 6 are configured by NW, and 12 repetitions are performed for a PUSCH transmission. As abovementioned, phase continuity and power consistency will be maintained within each 4 slots of OCC and within each 6 slots of DMRS bundling. However, actually this UE must maintain the phase continuity and power consistency during 12 slots as the boundaries of OCC group and nominal/actual TDW are not aligned. Although value 8 is reported for DMRS bundling size and the configured TDW length is less than the reported value, this UE will fail to handle this scheduled PUSCH transmission with OCC and DMRS bundling.

Fig. 1: An example of phase continuity for PUSCH when both OCC and DMRS bundling are enabled.

To avoid this situation, following alternatives can be considered.

- Alt 1: Nominal TDW size (= N) is the same as or is the multiples of OCC length (= L)

- Alt 2: UE is expected to maintain phase continuity and power consistency within K slots, where:

  - K is the least common multiple of N and L

  - K is not beyond the maximum duration during which UE is able to maintain power consistency and phase continuity

- Alt 3: UE does not expect that both OCC and DMRS bundling will be enabled for a given time.

For simplicity, Alt 1 is proposed from our side. Alt 3 is not preferred as DMRS bundling is an important feature for NTN that has been discussed in R18, and DMRS bundling should be applicable even when OCC is enabled.

Proposal 2:

When both OCC and DMRS bundling are enabled, it is expected that nominal TDW size (= N) is the same as or multiples of OCC length (= L).

- Adopt the following TP for TS 38.214.

6.1.2.1 Resource allocation in time domain <Unchanged parts omitted> For a PUSCH transmission with repetition Type A, a UE considers OCC operation enabled if it is configured with or indicated an OCC length, LOCC > 1. For a PUSCH transmission with repetition Type A and with OCC operation enabled, an OCC group is defined by Locc consecutive PUSCH repetitions and the integer number of OCC groups M is determined as M=K/Locc OCC group m includes PUSCH repetition m x Locc, m x Locc+1, .., (m+1) x Locc-1, where m=0,1, .., M-1, where repetition i within the OCC group will be multiplied with the OCC factor wi, where i is the repetition index within the OCC group. The OCC factor is for each repetition index i, based on the orthogonal sequence w_n (i) defined in Table 6.3.2.5A-1 and Table 6.3.2.5A-2 of [4, TS 38.211], where the sequence index n is derived from the DCI format scheduling the transmission for PUSCH or configured grant Type 2 PUSCH according to [5, TS 38.212, antenna port mapping to sequence], or provided by OCCsequenceIndexCGType1-r19 for configured grant Type 1 PUSCH, and provided for actual transmission according to clause 6.3.1.2a in [4, TS 38.211], where i=0,1 for OCC length 2 and i=0,1,2,3 for OCC length 4. If OCC operation and DMRS bundling are enabled for a PUSCH transmission with repetition Type A, it is expected that the duration of each nominal TDW given by pusch-TimeDomainWindowLength is the same as or multiples of an OCC length Locc. <Unchanged parts omitted>

Overlap issues

Overlap between PUCCH and inter-slot OCC with PUSCH repetitions

Regarding handling of overlap between PUCCH and inter-slot OCC with PUSCH repetitions for special case should be considered. In Rel-18 TEI, the following situation is allowed as shown in TS38.213, which was prohibited until Rel-17.

- A UE receives a UL grant to schedule PUSCH repetitions.

- After the RX of the UL grant, the UE receives a DL assignment scheduling a PUCCH for HARQ-ACK.

- The PUCCH can be overlapped with any of the PUSCH repetitions that will result in the HARQ-ACK multiplexing on the overlapped repetition, except for the first repetition.

Clause 9 of TS38.213 … If a UE - is provided enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, or enableType2HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, or enableType3HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, and - is not provided uci-MuxWithDiffPrio, and - transmits a repetition of a PUSCH transmission in a slot other than a first repetition, the UE includes, in a HARQ-ACK codebook, HARQ-ACK information associated with a PDSCH reception scheduled by a first DCI format, in a first PDCCH monitoring occasion, indicating a first resource for a first PUCCH transmission in the slot, when - the first resource overlaps with the repetition of the PUSCH transmission - the PUSCH transmission is scheduled by a second DCI format in a second PDCCH monitoring occasion, and - the UE multiplexes the HARQ-ACK codebook in the PUSCH transmission in the slot, and - the timeline conditions in clause 9.2.3 for PUCCH resource determination and the timeline conditions of Tproc,1mux and Tproc,2mux for multiplexing the HARQ-ACK information in the PUSCH, as described in clause 9.2.5, are satisfied, …

Fig.1: Scheduling allowed in R18 TEI.

The above restriction of “except for the first repetition” means that the HARQ-ACK multiplexing on the first repetition does not occur in the current specification. On the other hand, based on the agreement in R19 NR NTN, UCI in a PUCCH will be multiplexed on the first repetition even when the PUCCH is overlapped with a repetition within the first OCC group that is not the first repetition. Therefore, whether PUCCH overlapping with the first OCC group is allowed in the R18 TEI case or not is unclear and thus should be clarified.

For clarification of UE behavior in this situation, the following three alternatives can be considered:

- Alt 1: PUCCH can be overlapped with a PUSCH repetition within the first OCC group, which results in the HARQ-ACK multiplexing on the first repetition.

- Alt 2: PUCCH can be overlapped with a PUSCH repetition within other than the first OCC group, i.e., HARQ-ACK multiplexing on the first repetition is prohibited.

- Alt 3: UE does not expect to be configured with enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, or enableType2HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, or enableType3HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant.

From our side, Alt 2 is suggested as the outcome of Alt 1 is not aligned with the R18 TEI feature. Alt 3 is an excessive restriction and thus not preferred.

Fig.2a. Alt 1

Fig.2b. Alt 2

Proposal 3:

For the scheduling order supported in R18 TEI, PUCCH can be overlapped with a PUSCH repetition within other than the first OCC group, i.e., HARQ-ACK multiplexing on the first repetition is prohibited.

- Adopt following TP for TS38.213.

9 UE procedure for reporting control information <Unchanged parts omitted> If a UE - is provided enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, or enableType2HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, or enableType3HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, and - is not provided uci-MuxWithDiffPrio, and - transmits a repetition of a PUSCH transmission in a slot other than a first repetition, or other than repetitions within the first OCC group when OCC is enabled the UE includes, in a HARQ-ACK codebook, HARQ-ACK information associated with a PDSCH reception scheduled by a first DCI format, in a first PDCCH monitoring occasion, indicating a first resource for a first PUCCH transmission in the slot, when - the first resource overlaps with the repetition of the PUSCH transmission - the PUSCH transmission is scheduled by a second DCI format in a second PDCCH monitoring occasion, and - the UE multiplexes the HARQ-ACK codebook in the PUSCH transmission in the slot, and - the timeline conditions in clause 9.2.3 for PUCCH resource determination and the timeline conditions of Tproc,1mux and Tproc,2mux for multiplexing the HARQ-ACK information in the PUSCH, as described in clause 9.2.5, are satisfied, <Unchanged parts omitted>

Drop of PUSCH with OCC due to other than PUCCH overlap

At the previous meetings, it has been discussed and agreed how to handle overlap between PUSCH within an OCC group and PUCCH. Similarly, we believe that RAN1 should clarify UE behaviour for PUSCH drop due to other than PUCCH overlap; otherwise, system could not work well.

In the current specification, when a PUSCH repetition is involved with one of the overlapping/cancellation cases listed below, only the overlapped/cancelled PUSCH repetition is dropped and the remaining repetitions are transmitted, where the orthogonality of OCC is broken. For example, when PRACH overlaps within a slot of an OCC group (Case a below), the PUSCH in that slot is dropped, where OCC does not work well. Regarding solutions, the same handling as PUCCH overlap resulting in PUSCH drop can be reused for these cases.

- Case a: Overlapped with PRACH

- Case b: Overlapped with other PUSCH

  - e.g., DG PUSCH vs. CG PUSCH, CG PUSCH vs. CG PUSCH, PUSCH associated with C-RNTI vs. Msg3/MsgA PUSCH, A-CSI on PUSCH vs. SP-CSI on PUSCH, PUSCH with UL-SCH vs. SP-CSI on PUSCH

- Case c: Overlapped with other UL transmission that has a higher priority (based on priority index).

- Case d: Cancelled due to reception of a cancellation indication via DCI format 2_4

- Case e: Overlapped with prioritized DL reception in a RedCap/eRedCap UE with half-duplex FDD (HD-FDD)

Therefore, we submit the following TP.

Proposal 4:

When a repetition within an OCC group would be dropped due to other than PUCCH overlap, all repetitions within the same OCC group are dropped.

- Adopt the following TP for TS 38.214.

| Reason for change: | When one or more repetitions of a PUSCH repetition with OCC are dropped, the remaining repetitions in the same OCC group lose the OCC orthogonality. The current specification does not prohibit the UE from transmitting those remaining repetitions when the drop occurs due to other than PUCCH overlap. |
| --- | --- |
| Summary of change: | Introduce a UE behavior in clause 6.1.2.1 of TS 38.214, which is a similar manner to PUSCH drop due to PUCCH overlap, that, if the UE would not transmit a repetition of the PUSCH transmission with repetition Type A with OCC operation enabled due to other than PUCCH overlap, the UE does not transmit all repetitions of the PUSCH transmission in the OCC group. |
| Consequences if not approved: | UEs may transmit PUSCH repetitions whose OCC orthogonality is broken, leading to inefficient use of radio resources. |

6.1.2.1 Resource allocation in time domain <Unchanged parts omitted> For a PUSCH transmission with repetition Type A, a UE considers OCC operation enabled if it is configured with or indicated an OCC length, LOCC > 1. For a PUSCH transmission with repetition Type A and with OCC operation enabled, an OCC group is defined by Locc consecutive PUSCH repetitions and the integer number of OCC groups M is determined as M=K/Locc OCC group m includes PUSCH repetition m x Locc, m x Locc+1, .., (m+1) x Locc-1, where m=0,1, .., M-1, where repetition i within the OCC group will be multiplied with the OCC factor wi, where i is the repetition index within the OCC group. The OCC factor is for each repetition index i, based on the orthogonal sequence w_n (i) defined in Table 6.3.2.5A-1 and Table 6.3.2.5A-2 of [4, TS 38.211], where the sequence index n is derived from the DCI format scheduling the transmission for PUSCH or configured grant Type 2 PUSCH according to [5, TS 38.212, antenna port mapping to sequence], or provided by OCCsequenceIndexCGType1-r19 for configured grant Type 1 PUSCH, and provided for actual transmission according to clause 6.3.1.2a in [4, TS 38.211], where i=0,1 for OCC length 2 and i=0,1,2,3 for OCC length 4. For a PUSCH transmission scheduled by DCI format 0_1, 0_2 or 0_3 when OCC operation is not enabled, or 0_0 with CRC scrambled by TC-RNTI, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to table 6.1.2.1-2. For a PUSCH transmission scheduled by DCI format 0_1 or 0_2 when OCC operation is enabled, the redundancy version to be applied on the mth [OCC group] of the TB, where m = 0, 1, … (N∙K)/L_OCC -1, is determined according to table 6.1.2.1-2A. The UE shall maintain power consistency and phase continuity within an [OCC group] of PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, or 0_2, or PUSCH repetition Type A with a configured grant. For a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to the first row of Table 6.1.2.1-2. If the UE would not transmit a repetition of the PUSCH transmission with repetition Type A with OCC operation enabled due to other than PUCCH overlap, the UE does not transmit all repetitions of the PUSCH transmission in the OCC group. <Unchanged parts omitted>

Concurrence of UCI mux/drop and PUSCH drop due to other than PUCCH drop

At the previous meetings, UCI multiplexing/dropping rule in case of PUCCH/PUSCH overlap (Situation P) was agreed. Besides, PUSCH drop due to other than PUCCH/PUSCH overlap (Situation Q) could be discussed in this meeting as in the last section. Then, in this section, we will discuss the concurrence of these two situations.

We believe that the concurrence of these two situations could occur and thereby how to handle this concurrence case should be discussed/defined. For example, PUSCH repetition with OCC could overlap with a PUCCH for HARQ-ACK and a PRACH at different slots. The first overlap corresponds to Situation P and the HARQ-ACK would be multiplexed on PUSCH repetitions within the same OCC group. The second overlap corresponds to Situation Q and the PUSCH repetitions within the same OCC could be drop. In our understanding, the order of the two handling behaviors is not defined in the current spec. As a result, gNB does not know the outcome of overlap handling at the UE, which is not desirable.

Regarding solutions, the following three alternatives can be listed, and our preference is Alt 1.

- Alt 1: PUSCH drop for Situation Q is performed without UCI multiplexing/drop for Situation P.

- Alt 2: UCI multiplexing/drop for Situation P is applied first and PUSCH drop for Situation Q then.

- Alt 3: The concurrence case is defined as an error case.

Alt 2 is not preferred as performing UCI multiplexing/dropping first can result in unnecessary UCI drops. Fig.3 shows this disadvantage of Alt 2 by using the above example of PUSCH/PUCCH/PRACH overlaps. In Fig.3, the UE handles the overlap of the HARQ-ACK and PUSCH first, where HARQ-ACK is multiplexed onto all PUSCH repetitions within the same OCC group. Subsequently, the PUSCH/PRACH overlap is handled. All PUSCH repetitions within the same OCC group may be dropped as proposed in the last section, in this case, the multiplexed HARQ-ACK is also dropped. This unnecessary drop is avoided in Alt 1 and thus the HARQ-ACK is transmitted on the PUCCH.

Alt 3 may not be reasonable as it may be difficult for NW to avoid this overlap by its implementation. For example, PRACH occasions are predetermined and when the transmission happens may be decided after scheduling the PUCCH or the PUSCH.

Fig.3: Non-preferred handling order (Alt 2)

Proposal 5:

- When the concurrence of the following two cases occurs, PUSCH drop is performed for Case Q before UCI multiplexing/drop for Case P.

  - Case P: UCI multiplexing/dropping rule in case of PUCCH/PUSCH overlap

  - Case Q: PUSCH drop due to other than PUCCH/PUSCH overlap

- Adopt the following TP for TS 38.213.

# 9 UE procedure for reporting control information <Unchanged parts omitted> For the remaining of this clause, for a UE operating on an NTN serving cell, the timeline conditions for resolving time overlapping between a PUSCH transmission with repetitions in an OCC group [6, TS 38.214] and PUCCH transmissions are applicable with respect to the first repetition of the PUSCH transmission in the OCC group - if the UE would multiplex UCI from the PUCCH transmissions in the PUSCH, the UE multiplexes the UCI in all repetitions of the PUSCH transmission in the OCC group - if the UE would not transmit a repetition of the PUSCH transmission, the UE does not transmit all repetitions of the PUSCH transmission in the OCC group - the UE does not expect to transmit in different slots more than one PUCCHs that provide HARQ-ACK information or CSI reports and would overlap with the PUSCH transmission in the OCC group Resolving time overlapping between a PUSCH transmission with repetitions in an OCC group [6, TS 38.214] and PUCCH transmissions is applied after, if any, resolving drop of a repetition of the PUSCH transmission with repetition Type A with OCC operation enabled according to clause 6.1.2.1 of [6, TS 38.214]. <Unchanged parts omitted>

Editorial changes

Proposal 6:

- Adopt the following TP for TS 38.211.

## 6.2 Physical resources <Unchanged parts omitted> If inter-slot OCC is applied to PUSCH as described in clause XXX 6.1.2.1 of [6, 38.214], the UE transmission shall be such that the channel over which a symbol on the antenna port used for uplink transmission is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed if the two symbols are transmitted on slots within the same orthogonal cover code. <Unchanged parts omitted>

Conclusion

Proposal 1:

RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH under the same conditions that phase continuity and power consistency applies for DMRS bundling.

- Adopt following TP for TS38.214.

6.1.2.1 Resource allocation in time domain <Unchanged parts omitted> The UE shall maintain power consistency and phase continuity within an [OCC group] of PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, or 0_2, or PUSCH repetition Type A with a configured grant if any event described in Clause 6.1.7 is not occurring within the OCC group; otherwise, PUSCH transmissions within the OCC group are dropped. <Unchanged parts omitted>

Proposal 2:

When both OCC and DMRS bundling are enabled, it is expected that nominal TDW size (= N) is the same as or multiples of OCC length (= L).

- Adopt the following TP for TS 38.214.

6.1.2.1 Resource allocation in time domain <Unchanged parts omitted> For a PUSCH transmission with repetition Type A, a UE considers OCC operation enabled if it is configured with or indicated an OCC length, LOCC > 1. For a PUSCH transmission with repetition Type A and with OCC operation enabled, an OCC group is defined by Locc consecutive PUSCH repetitions and the integer number of OCC groups M is determined as M=K/Locc OCC group m includes PUSCH repetition m x Locc, m x Locc+1, .., (m+1) x Locc-1, where m=0,1, .., M-1, where repetition i within the OCC group will be multiplied with the OCC factor wi, where i is the repetition index within the OCC group. The OCC factor is for each repetition index i, based on the orthogonal sequence w_n (i) defined in Table 6.3.2.5A-1 and Table 6.3.2.5A-2 of [4, TS 38.211], where the sequence index n is derived from the DCI format scheduling the transmission for PUSCH or configured grant Type 2 PUSCH according to [5, TS 38.212, antenna port mapping to sequence], or provided by OCCsequenceIndexCGType1-r19 for configured grant Type 1 PUSCH, and provided for actual transmission according to clause 6.3.1.2a in [4, TS 38.211], where i=0,1 for OCC length 2 and i=0,1,2,3 for OCC length 4. If OCC operation and DMRS bundling are enabled for a PUSCH transmission with repetition Type A, it is expected that the duration of each nominal TDW given by pusch-TimeDomainWindowLength is the same as or multiples of an OCC length Locc. <Unchanged parts omitted>

Proposal 3:

For the scheduling order supported in R18 TEI, PUCCH can be overlapped with a PUSCH repetition within other than the first OCC group, i.e., HARQ-ACK multiplexing on the first repetition is prohibited.

- Adopt following TP for TS38.213.

9 UE procedure for reporting control information <Unchanged parts omitted> If a UE - is provided enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, or enableType2HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, or enableType3HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant, and - is not provided uci-MuxWithDiffPrio, and - transmits a repetition of a PUSCH transmission in a slot other than a first repetition, or other than repetitions within the first OCC group the UE includes, in a HARQ-ACK codebook, HARQ-ACK information associated with a PDSCH reception scheduled by a first DCI format, in a first PDCCH monitoring occasion, indicating a first resource for a first PUCCH transmission in the slot, when - the first resource overlaps with the repetition of the PUSCH transmission - the PUSCH transmission is scheduled by a second DCI format in a second PDCCH monitoring occasion, and - the UE multiplexes the HARQ-ACK codebook in the PUSCH transmission in the slot, and - the timeline conditions in clause 9.2.3 for PUCCH resource determination and the timeline conditions of Tproc,1mux and Tproc,2mux for multiplexing the HARQ-ACK information in the PUSCH, as described in clause 9.2.5, are satisfied, <Unchanged parts omitted>

Proposal 4:

When a repetition within an OCC group would be dropped due to other than PUCCH overlap, all repetitions within the same OCC group are dropped.

- Adopt the following TP for TS 38.214.

| Reason for change: | When one or more repetitions of a PUSCH repetition with OCC are dropped, the remaining repetitions in the same OCC group lose the OCC orthogonality. The current specification does not prohibit the UE from transmitting those remaining repetitions when the drop occurs due to other than PUCCH overlap. |
| --- | --- |
| Summary of change: | Introduce a UE behavior in clause 6.1.2.1 of TS 38.214, which is a similar manner to PUSCH drop due to PUCCH overlap, that, if the UE would not transmit a repetition of the PUSCH transmission with repetition Type A with OCC operation enabled due to other than PUCCH overlap, the UE does not transmit all repetitions of the PUSCH transmission in the OCC group. |
| Consequences if not approved: | UEs may transmit PUSCH repetitions whose OCC orthogonality is broken, leading to inefficient use of radio resources. |

6.1.2.1 Resource allocation in time domain <Unchanged parts omitted> For a PUSCH transmission with repetition Type A, a UE considers OCC operation enabled if it is configured with or indicated an OCC length, LOCC > 1. For a PUSCH transmission with repetition Type A and with OCC operation enabled, an OCC group is defined by Locc consecutive PUSCH repetitions and the integer number of OCC groups M is determined as M=K/Locc OCC group m includes PUSCH repetition m x Locc, m x Locc+1, .., (m+1) x Locc-1, where m=0,1, .., M-1, where repetition i within the OCC group will be multiplied with the OCC factor wi, where i is the repetition index within the OCC group. The OCC factor is for each repetition index i, based on the orthogonal sequence w_n (i) defined in Table 6.3.2.5A-1 and Table 6.3.2.5A-2 of [4, TS 38.211], where the sequence index n is derived from the DCI format scheduling the transmission for PUSCH or configured grant Type 2 PUSCH according to [5, TS 38.212, antenna port mapping to sequence], or provided by OCCsequenceIndexCGType1-r19 for configured grant Type 1 PUSCH, and provided for actual transmission according to clause 6.3.1.2a in [4, TS 38.211], where i=0,1 for OCC length 2 and i=0,1,2,3 for OCC length 4. For a PUSCH transmission scheduled by DCI format 0_1, 0_2 or 0_3 when OCC operation is not enabled, or 0_0 with CRC scrambled by TC-RNTI, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to table 6.1.2.1-2. For a PUSCH transmission scheduled by DCI format 0_1 or 0_2 when OCC operation is enabled, the redundancy version to be applied on the mth [OCC group] of the TB, where m = 0, 1, … (N∙K)/L_OCC -1, is determined according to table 6.1.2.1-2A. The UE shall maintain power consistency and phase continuity within an [OCC group] of PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, or 0_2, or PUSCH repetition Type A with a configured grant. For a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to the first row of Table 6.1.2.1-2. If the UE would not transmit a repetition of the PUSCH transmission with repetition Type A with OCC operation enabled due to other than PUCCH overlap, the UE does not transmit all repetitions of the PUSCH transmission in the OCC group. <Unchanged parts omitted>

Proposal 5:

- When the concurrence of the following two cases occurs, PUSCH drop is performed for Case Q before UCI multiplexing/drop for Case P.

  - Case P: UCI multiplexing/dropping rule in case of PUCCH/PUSCH overlap

  - Case Q: PUSCH drop due to other than PUCCH/PUSCH overlap

- Adopt the following TP for TS 38.213.

# 9 UE procedure for reporting control information <Unchanged parts omitted> For the remaining of this clause, for a UE operating on an NTN serving cell, the timeline conditions for resolving time overlapping between a PUSCH transmission with repetitions in an OCC group [6, TS 38.214] and PUCCH transmissions are applicable with respect to the first repetition of the PUSCH transmission in the OCC group - if the UE would multiplex UCI from the PUCCH transmissions in the PUSCH, the UE multiplexes the UCI in all repetitions of the PUSCH transmission in the OCC group - if the UE would not transmit a repetition of the PUSCH transmission, the UE does not transmit all repetitions of the PUSCH transmission in the OCC group - the UE does not expect to transmit in different slots more than one PUCCHs that provide HARQ-ACK information or CSI reports and would overlap with the PUSCH transmission in the OCC group Resolving time overlapping between a PUSCH transmission with repetitions in an OCC group [6, TS 38.214] and PUCCH transmissions is applied after, if any, resolving drop of a repetition of the PUSCH transmission with repetition Type A with OCC operation enabled according to clause 6.1.2.1 of [6, TS 38.214]. <Unchanged parts omitted>

Proposal 6:

- Adopt the following TP for TS 38.211.

## 6.2 Physical resources <Unchanged parts omitted> If inter-slot OCC is applied to PUSCH as described in clause XXX 6.1.2.1 of [6, 38.214], the UE transmission shall be such that the channel over which a symbol on the antenna port used for uplink transmission is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed if the two symbols are transmitted on slots within the same orthogonal cover code. <Unchanged parts omitted>

References

[1] Draft Report of 3GPP TSG RAN WG1 #120bis, R1-2502985, “Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput”, Moderator (MediaTek), Apr. 2025.