3GPP TSG RAN WG1 #122	                                   R1-2506324

Bengaluru, India, Aug 25th – 29h, 2025

Source:	WILUS Inc.

Title:	Discussion on Energy Efficiency for 6G Radio

Agenda item:	11.5

Document for:	Discussion/Decision

Introduction

In the TSG-RAN#108 meeting [1], a Working Group Study Item (SI) on 6G Radio was approved for Rel-20 [2]. The SI core objectives related to RAN1 aspects are as follows:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. …

Energy efficiency is a key objective for the sixth generation (6G) mobile communication systems, as it is crucial for achieving both Network Energy Saving (NES) and UE Power Saving (UPS). This integrated approach is essential for significantly reducing operational expenditures (OPEX) for network operators, extending UE battery life and addressing increasing environmental sustainability requirements. Unlike previous generations, 6G should study end-to-end energy-saving framework developed from the early stage of the standardization. This contribution provides our views on energy efficiency, with a focus on the implications of Cell DTX/DRX operations within the broader 6G NES framework. Key insights are drawn from 5G-Advanced, and key enhancements for 6G are identified.

Background

In 5G NR, network energy-saving mechanisms, including Cell DTX/DRX, were introduced in later releases (Rel-18 and Rel-19). While these features provided a basis for network power reduction, their effectiveness in real-world deployments was constrained by several limitations:

- Late Introduction and Fragmentation: Energy-saving features were not part of the initial 5G NR release. As a result, early deployments lacked a unified framework, leading to vender-specific or fragmented implementations with limited interoperability.

- Backward Compatibility Constraints: Many Rel-18/19 NES solutions often had limitations due to the need to ensure coexistence with legacy NR operations. For instance, on-demand SSB or SSB periodicity adaptation was only applicable to SCells, and Cell DTX/DRX was primarily for RRC_CONNECTED mode UEs. This significantly restricted the achievable NES gains in multi-cell and idle-mode scenarios.

- Continuous Common Signal Transmission: Rel-18 Cell DTX/DRX typically excluded common signals, such as Synchronization Signal Block (SSB), from discontinuous transmission. Consequently, gNBs were to required to transmit SSBs periodically (e.g., every 20ms) even during low or zero traffic periods. This constant transmission:

  - Prevented gNBs from entering deeper sleep modes.

  - Limited maximum possible sleep intervals.

  - Increased the frequency of active/sleep state transitions, reducing overall NES benefits and sustaining high energy consumption even under low network load conditions.

- Limited On-Demand Functionality: While Rel-19 introduced on-demand SIB1 for SCells and on-demand SSB for NCD-SSB in connected mode, the periodic transmission of cell-specific SSBs (CD-SSBs) remained mandatory, restricting the feasibility of extended deeper sleep operation.

- Lack of UE Traffic Information: The absence of timely UE traffic state information at the network side often prevented practical Cell DTX/DRX activation without risking negative impacts on user-perceived quality of service (QoS) experience.

- Separation of NES and UPS: The separate consideration of NES and UPS in distinct work items in 5G resulted in limitations in achieving overall system energy efficiency and often led to duplicated or inefficient mechanisms. This approach hindered the realization of holistic, end-to-end energy efficiency gains at the system level.

Discussion on Energy Efficiency for 6G Radio

6G Approach to Energy Efficiency

- Integrated Design from Day 1: The 6G Radio (6GR) should aim to develop a holistic, end-to-end energy-saving framework from Day 1 of standardization, applicable across all UE modes (Idle, Inactive, and RRC_CONNECTED). This unified approach will prevent fragmented deployments and enable coordinated optimization between network and UE.

- Quantitative KPIs and QoS-based Evaluation: 6GR should define measurable KPIs for energy efficiency, such as energy saving percentage over 5G, and metrics such as consumed energy per transmitted bit. Performance evaluation should be based on QoS satisfaction rather than solely on UE perceived throughput (UPT). This allows the network to schedule resources more flexibly and avoid unnecessary power consumption as long as user QoS is met. Updated power consumption models, including those for new FR3 bands, should be developed and studied.

- Joint Optimization and Design Simplification: The design must pursue joint optimization of NES and UPS. This includes harmonizing designs (e.g., aligning Cell DTX/DRX operation with UE-side DRX operation) and avoiding duplicated functionalities. The system should be simplified to reduce configuration complexity and enable more efficient Cell and UE management.

Considerations for Comprehensive NES across all UE modes

The primary objective of 6G Radio is to achieve comprehensive Network Energy Saving (NES) across all UE modes (Idle, Inactive, and RRC_Connected) in order to maximize both network-side and UE-side power savings. Learning from the limitations of 5G-Advanced NES solutions, which offered only limited benefits duo to their narrow scope and coexistence constraints with legacy UEs, 6GR should be designed from the outset with integrated and scalable NES technologies applicable to all UE modes.

- Energy-Oriented Initial Access: 6GR initial access mechanisms should be optimized for energy efficiency while ensuring fast and reliable connection establishment. This includes redesigning common channels and signals, like SSBs, to allow for flexible and energy-efficient transmission.

- Proposal 1: Study Energy-Efficient Initial Access for 6GR

  - Optimize initial access for both energy efficiency and fast, reliable connectivity by redesigning common channels and signals, such as SSBs, to support flexible and energy-efficient transmission.

- On-Demand SSB for Idle/Inactive UEs: A key proposal for 6GR is to study on-demand SSB transmission for UEs in Idle/Inactive mode.

  - This mechanism allows the gNB to transmit SSBs only when triggered by a Wake-Up Signal (WUS) from the UE, similar to on-demand SIB1 in 5G Rel-19.

  - This can lead to significant energy saving gains (e.g., up to 83.7% energy savings at zero or low load with Cat-1 BS power model [3]) by enabling deeper sleep for gNBs. The results demonstrate that this is a valuable candidate for further study in 6GR.

  - Such on-demand transmission extends the application scenarios for SSB reduction and can be particularly beneficial in new or re-farmed spectrums (e.g., 6GHz) where legacy UE constraints are minimal.

- Proposal 2: Study On-Demand SSB/SSB1 for 6GR

  - Enable on-demand SSB/SIB1 transmission for UEs in Idle, Inactive, or RRC_Connected modes to maximize energy savings and deep-sleep opportunities for gNBs.

- Extended SSB Periodicity: 6GR should study extended SSB periodicity beyond the typical 20ms assumed by UEs during initial access, potentially up to 160ms or more.

  - Longer SSB periodicities enable gNBs to increase deep-sleep durations, leading to substantial energy savings (e.g., up to 73% savings for 160ms periodicity compared to baseline [3]).

  - This feature is particularly applicable to greenfield deployments (new frequency bands) or to where legacy UE impacts can be minimized.

- Proposal 3: Study Extended SSB Periodicity for 6GR

  - Introduce longer SSB periodicities to enable deeper gNB sleep modes, leading to substantial energy savings.

- Extending Cell DTX/DRX to Common Signals in Multi-Cell Scenarios: Rel-18/19 introduced Cell-DTX/DRX for data channel energy saving, but common signals (like SSB, SIB1, Paging, and RACH) remained largely excluded. 6GR needs to extend the Cell DTX/DRX mechanism to common signals in multi-cell scenarios, where the anchor cell can serve legacy UEs, and NES cells can adapt their common signal transmissions for energy saving. Transmission and reception of common signals can be managed through the Cell DTX/DRX mechanism.

- Proposal 4: Study extending Cell DTX/DRX to common signals in multi-cell scenarios for 6GR

  - Manage both data and common signal transmission/reception through Cell DTX/DRX in multi-cell NES scenarios, enabling coordinated deep-sleep operation.

- Unified and Harmonized Design:

  - 6G design should avoid duplicated techniques pursuing identical functionality, leading to simpler and more efficient power saving mechanisms.

  - A fundamental principle for 6G is to achieve a harmonized NES-UPS. This includes alignment between Cell DTX/DRX with UE DRX to balance network and UE energy consumption while meeting QoS requirements.

  - This includes ensuring that NES techniques are applicable to all Idle/Inactive/RRC_CONNECTED mode UEs and various scenarios/deployments.

  - The power consumption model and assumptions for energy saving evaluation in 6G will also consider the new FR3 bands and the energy saving metric based on QoS satisfaction rather than throughput only.

Conclusion

In this contribution, we discussed the energy efficiency for 6GR Study Item and summarize our views as the following:

- Proposal 1: Study Energy-Efficient Initial Access for 6GR

  - Optimize initial access for both energy efficiency and fast, reliable connectivity by redesigning common channels and signals, such as SSBs, to support flexible and energy-efficient transmission.

- Proposal 2: Study On-Demand SSB/SSB1 for 6GR

  - Enable on-demand SSB/SIB1 transmission for UEs in Idle, Inactive, or RRC_Connected modes to maximize energy savings and deep-sleep opportunities for gNBs.

- Proposal 3: Study Extended SSB Periodicity for 6GR

  - Introduce longer SSB periodicities to enable deeper gNB sleep modes, leading to substantial energy savings.

- Proposal 4: Study extending Cell DTX/DRX to common signals in multi-cell scenarios for 6GR

  - Manage both data and common signal transmission/reception through Cell DTX/DRX in multi-cell NES scenarios, enabling coordinated deep-sleep operation.

References

- Draft Meeting Report of 3GPP TSG-RAN#108 (2025-06)

- RP-251881 New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- TR 38.864, “Study on network energy savings for NR”