3GPP TSG RAN WG1 #122                                  R1-2506334

Bengaluru, IN, Aug 25th – 29th, 2025

Agenda item:	10.6.1

Source: 	CSCN

Title:	Discussion on GNSS resilience for NR-NTN

Document for:	Discussion & Decision

- Introduction

In RAN#108 meeting, a new SID on GNSS resilient NR-NTN operation was approved, and the objectives of the study were obtained as following [1],

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

In this contribution, we discuss the impact of GNSS resilient scenarios on initial access and connected mode procedures, and propose some potential solutions for timing and frequency compensation.

- Discussion

Uplink time and frequency synchronization mechanisms in GNSS unavailable scenarios were initially studied during Releases 15-17. Based on previous discussions, this contribution further investigates timing and frequency compensation under the Rel-20 study item.

  - 2.1 GNSS resilient scenarios

The study item considers three possible scenarios for GNSS resilient operation. For Scenario 1, UE may not obtain GNSS information due to GNSS signal temporarily unavailable (e.g., indoor scenarios, signal blockage, or interference). For Scenario 2, UE receives GNSS signals, but the position accuracy is reduced (e.g., multipath effects, or weak signals). For Scenario 3, GNSS information is available but with increased GNSS measurement period for power saving purpose. Scenario 1 is the most challenging case with complete unavailability of GNSS information, while Scenarios 2 and 3 maintain varying degrees of partial GNSS availability. Given these conditions, Scenario 1 presents the most stringent requirements for time and frequency offset enhancement schemes, and the solutions developed for Scenario 1 will naturally be reused to Scenarios 2 and 3. Consequently, the study focusing on Scenario 1 should be given the highest priority.

Observation 1: Scenario 1 presents the most stringent requirements for time and frequency offset enhancement schemes, and the solutions developed for Scenario 1 will naturally be reused to Scenarios 2 and 3.

In NTN, to achieve synchronization during both initial access and connected modes, UE shall compute the time and frequency offset of the service link, and autonomously pre-compensate for it, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position, it shall not establish communication with the network until the GNSS information is regained. Therefore, enhancements for time and frequency compensation mechanisms are required during initial access and connected mode procedures under GNSS resilient scenarios.

Observation 2: Both initial access and connected mode procedures may be affected when GNSS information is unavailable.

Observation 3: Existing mechanisms for acquisition and maintenance of time and frequency synchronization rely on GNSS information.

In Scenario 1 where GNSS is temporarily unavailable, we can further classify the scenario into two sub-cases based on the duration of GNSS unavailable.

Case1, short-term GNSS unavailable, where the UE has previously obtained the reference time (synchronized with GNSS) and still maintains the reliability;

Case2, long-term GNSS unavailable, where the UE has no available reference time.

Proposal 1: The impact of GNSS unavailable duration should be researched, with long-term unavailability as the high-priority case.

To evaluate the impact of time and frequency compensation mechanisms on initial access and connected mode procedures for NR NTN, RAN1 should first specify the simulation scenarios and parameters related to GNSS resilient study.

Proposal 2: RAN1 to clarify the evaluation scenarios and parameter sets for the GNSS resilient study.

  - 2.2 Timing advance

### 2.2.1 Impact on initial access

Timing advance (TA) is carried out for uplink transmissions based on TA command so that gNB can receive uplink signals from different UEs at an aligned timing at gNB reception point. In NTN scenarios, since UEs are located at varying distances from the gNB, the propagation delay differs among UEs. Due to the long propagation delay between the gNB and the UE and differential delay, TA enhancements may be considered to align the UL transmission.

During Releases 15-17, many solutions about TA compensation in NR NTN were discussed. In Rel-16, the beam center is assumed as the reference point (RP), and common TA is used to compensate the RTT at a RP. Before sending Msg1, common TA is calculated and broadcasted by the network, and either gNB or UE compensates the common TA. Then, the UE subsequently handles differential TA adjustments. After sending Msg1, gNB detects the PRACH and transmits the RAR, and UE compensates TA based on the TA command from RAR [2].

In Case 1, where the UE has previously obtained the reference time and still maintains its reliability, the SIB9 based timestamp solution discussed in Rel-17 may be applicable [3]. SIB9 occupies approximately 120 bits and can be scheduled either in the same slot as other SIBs (such as SIB2/3/4) or independently, with a maximum transmission period of 5.12s. Additionally, SIB9 can achieve GPS/UTC time synchronization between BS and UE, with synchronization accuracy of 10ns. In this solution, the UE specific TA is estimated by the UE based on the GNSS acquired reference time at UE together with reference time as indicated by the network, without backward compatible issues. However, this approach requires clear criteria to determine whether the UE's stored reference time remains sufficiently reliable during GNSS unavailable period. Additionally, the transmission of SIB9 messages consumes some time-frequency resources. In beam hopping systems, the coverage time per beam is limited, and the available resources for broadcasting common signals are constrained. Therefore, the sufficient resource for both common signals and user data transmission should be ensured.

Proposal 3: For Case1, the SIB9 based solution could be considered to compensate timing offset.

Observation 4: The SIB9 based solution avoids backward compatibility issues, but requires sufficient time and frequency resource.

In Case 2, where the UE has no available reference time, gNB can broadcast essential assistant information (e.g., RP position), and 	UE calculates and compensates common TA based on this RP location and ephemeris information. The residual timing offset can then be estimated and corrected through PRACH based adjustment.

Proposal 4: For Case2, the gNB may broadcast assistance information (e.g., RP position) for the UE to calculate and compensate the common TA, while the residual offset could be estimated and corrected through PRACH.

Additionally, this RP can be configured as either cell-specific or beam-specific. For cell-specific RP configuration, the existing referenceLocation-r17 in SIB19 could be directly reused. However, due to the extensive coverage of NTN cells, a cell-specific RP may result in excessive differential delay, which cannot be handled by existing preamble formats. To address this issue, beam-specific RP configuration could be considered to limit the maximum differential delay within the beam footprint. However, this approach requires additional signaling design.

### 2.2.2 Impact on connected mode

The primary causes of TA variance during connected mode include UE mobility, satellite movement, and oscillator inaccuracy. Current open-loop timing offset compensation mechanisms depend on GNSS information at the UE side. However, this open-loop approach becomes unavailable in the GNSS resilient scenario, and the closed-loop compensation method should be employed to maintain time synchronization.

In the closed-loop compensation method, to maintain uplink timing in connected mode, the network sends timing advance commands in the MAC-CE to the UE. For GNSS resilient Case2, the UE has no available reference time for a long time, and TAC adjustments face several critical challenges. Firstly, the frequent TAC updates to compensate would generate excessive signaling overhead. Secondly, RTT causes TAC to arrive at the UE out of synchronization with current propagation conditions, leading to outdated compensation.

To solve the issues about TAC, the timing drift rate compensation method was discussed in Rel-16. The solution enables autonomous UE differential TA adjustment by leveraging predictable delay variations in NTN [4]. The UE can predict the delay drift within a limited time window based on the last timing advance commands and their updated values, or based on the observed DL reference signal tracking. In addition, the timing drift rate can be indicated from the network to UE, and the associated DL overhead should be carefully evaluated.

Observation 5: When UE has no available reference time for a long time, TA adjustments via TAC in the connected mode face critical challenges of excessive signaling overhead and outdated TAC.

When the RP is set as the cell/beam center, the calculated TA compensation between the satellite and RP may exceed the UE's actual required compensation in special scenarios, where the satellite operates at inclined angles or UE is at high altitudes. When the RP is set as the cell/beam edge, the differential TA may be excessive. Therefore, the RP position should be appropriately selected.

Proposal 5: The RP position should be appropriately selected.

  - 2.3 Frequency synchronization

### 2.3.1 DL frequency synchronization

In Rel-17, it’s concluded that the DL frequency compensation by gNB for the service link Doppler is not supported [5]. Thus, based on the existing specs, during initial access, UE first achieves DL frequency synchronization by detecting the SSB on GSCNs. Subsequently, based on the relative RB offset indicated in MIB, UE moves to the corresponding frequency to receive SIB1 and then, based on the relative frequency indicated in SIB1, receives SIB19. Only after receiving SIB19 does the UE obtain the ephemeris information, enabling it to calculate the service link frequency offset (FO) based on its own location and the ephemeris. Therefore, the availability of the UE's GNSS information has no impact on the initial access procedure before completing SIB19 decoding, i.e., before initiating the RACH procedure.

Observation 6: The availability of UE's GNSS information has less impact on the specs about initial access procedure before RACH initiation.

In connected mode, UE could maintain DL frequency synchronization via DL RS reception or FO estimation based on its GNSS and ephemeris information, while the later scheme can achieve higher precision. If GNSS information is unavailable, the RS-based synchronization maintenance mechanism still works, but some implementation impact might be introduced (e.g., increased synchronization complexity). Or, if needed, some DL frequency synchronization enhancements can be introduced.

### 2.3.2 UL frequency synchronization

In NTN, the high-speed movement of satellites induces significant Doppler FO. If the UE fails to compensate for this offset during PRACH transmission, gNB cannot successfully receive the PRACH within the allocated frequency-domain resources. Even the PRACH is successfully received, the residual FO may exceed the gNB's preamble FO estimation capability, leading to inaccurate FO estimation.

To mitigate such large Doppler offsets in UL transmission, the existing spec requires UE to calculate the service link FO using its GNSS position and ephemeris information, while the feeder link FO is managed by the network, as specified in the following [6].

TS38.300 The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained. In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation. The UE may be configured to report Timing Advance during Random Access procedures or in connected mode. In connected mode, event-triggered reporting of the Timing Advance is supported. While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link and transponder frequency error is left to the network implementation.

According to the spec, when GNSS information is temporarily unavailable and the UE lacks valid position information, UE cannot perform transmissions until the information is regained. This will lead to a significant system performance degradation in GNSS resilient scenarios.

To deal with this issue, similar to the TA compensation mechanisms, different schemes could be adopted for different GNSS unavailable cases.

For case 1, the SIB9-based scheme could be used for FO estimation without backward compatibility issues but with sufficient time and frequency resource requirements.

Proposal 6: For case1, the SIB9-based scheme could be adopted for FO estimation.

Moreover, the SIB9-based scheme enables accurate FO estimation, eliminating the need for a closed-loop synchronization mechanism. In other words, the differential FO indication from gNB is not needed at UE side.

Observation 7: The closed-loop frequency synchronization mechanism is not needed when using the SIB9-based scheme, since accurate FO could be estimated.

For Case 2, where the UE only has valid ephemeris information, the network needs to provide additional assistance information to help the UE estimate FO. Following a similar approach to TA compensation, the network can broadcast the cell/beam-specific RP position. Then, UE would autonomously calculate the FO and perform common FO compensation based on this RP and ephemeris information, while estimating the differential offset during the RACH procedure.

Proposal 7: For case 2, the RP indication-based scheme could be used, where the gNB broadcasts cell/beam-specific RPs to enable the autonomously common FO calculation and compensation at UE side.

Additionally, due to the still-extensive coverage of satellite beams, the differential FO after common FO compensation may still exceed UL frequency synchronization requirements. Therefore, the gNB needs to estimate this differential offset upon preamble reception and subsequently notify the UE of the compensation value. Consequently, when employing the common FO compensation method (i.e., the RP indication-based scheme), an additional closed-loop frequency synchronization mechanism is required to guarantee precise frequency synchronization.

Proposal 8: The closed-loop frequency synchronization mechanism should be introduced when using the RP indication-based common FO compensation scheme.

  - 2.4 RACH

### 2.4.1 PRACH

As mentioned above, the common time and frequency offset can be compensated by the UE with respect to a RP located within the cell/beam (e.g., the cell/beam center), while the differential time and frequency drifts should be estimated during the RACH procedure based on the preamble (generated from ZC sequence and its cyclic shift). The value of differential time and frequency offsets will be larger than those in existing TN/NTN systems, which leads to the reduction of the number of available cyclic shifts (CS).

Observation 8: Due to the large differential time and frequency offset, availability of CS multiplexing would be limited.

In accordance with the requirements of SID, the study should aim to minimize the impact of physical layer procedures and avoid changes to physical layer channels/signals. Besides, backward compatible issue is also one of the essential aspects specified in SID, once new/enhanced preamble formats are introduced, the backward compatibility cannot be guaranteed. Therefore, it is essential to assess the necessity of enhancing the PRACH, especially the preamble sequences.

Observation 9: Once some new/enhanced preamble formats are introduced, the backward compatibility cannot be guaranteed.

Proposal 9: The need of PRACH enhancement should be firstly specified before studying specific enhancement schemes.

If it is considered necessary to enhance the PRACH, then the schemes previously discussed in Rel-16 may be taken as the baseline. The approaches considered in Rel-16 are as follows [2]:

TR38.821 However, in case pre-compensation of timing and frequency offset is not performed at UE side for UL transmission, enhanced PRACH formats and/or preamble sequences should be supported with following options: ● Option-1: A single Zadoff-Chu sequence based on larger SCS, repetition number. Additional usage of CP and Ncs can be further determined in normative work [51], [52], [53], [54]. ● Option-2: A solution based on multiple Zadoff-Chu sequences with different roots [54], [55], [56] ● Option-3: Gold/m-sequence as preamble sequence with additional process, e.g., modulation and transform precoding [57], [51] ● Option-4: A single Zadoff-Chu sequence with combination of scrambling sequence [58], [59], [51] Further discussions to down select these candidates are needed in the normative work.

Proposal 10: If the PRACH enhancement is deemed necessary, the schemes studied in Rel-16 can be considered as the baseline.

Among these options, option 3 should be deprioritized as it may introduce significant spec impact and require significant workload.

Proposal 11: Option 3 for PRACH enhancement specified in TR38.821 should be deprioritized due to its substantial spec impacts and considerable workload.

### 2.4.2 RACH procedure

When receiving the preamble, the gNB performs detection within a fixed reception window. If the differential delay exceeds the supported range of PRACH CP, the gNB may fail to detect the preamble. This issue can be resolved through some implementation-based methods such as extending the preamble reception window length or enhancing the gNB receiver.

Moreover, as specified in the existing specs, the RAR window starts after an additional TTA+kmac msec, where TTA is calculated by the UE based on its GNSS information. When GNSS information is unavailable, inaccurate TTA estimation will cause displacement of the RAR window, thereby affecting RAR reception performance. Thus, necessary enhancements to the RAR reception window need to be considered in GNSS-resilient scenarios.

Additionally, if the RP for common time and frequency compensation is selected at the ground beam center, over-compensation may occur in certain scenarios (e.g., in inclined beam or high-altitude UE scenarios). In such cases, corresponding adjustments to both the preamble reception and RAR window would be required.

Proposal 12: Necessary optimizations of the preamble reception and RAR windows should be introduced.

- Conclusions

In this contribution, we provide some views on GNSS resilient NR-NTN operation. The proposals are summarized as following.

Observation 1: Scenario 1 presents the most stringent requirements for time and frequency offset enhancement schemes, and the solutions developed for Scenario 1 will naturally be reused to Scenarios 2 and 3.

Observation 2: Both initial access and connected mode procedures may be affected when GNSS information is unavailable.

Observation 3: Existing mechanisms for acquisition and maintenance of time and frequency synchronization rely on GNSS information.

Proposal 1: The impact of GNSS unavailable duration should be researched, with long-term unavailability as the high-priority case.

Proposal 2: RAN1 to clarify the evaluation scenarios and parameter sets for the GNSS resilient study.

Proposal 3: For Case1, the SIB9 based solution could be considered to compensate time offset.

Observation 4: The SIB9 based solution avoids backward compatibility issues, but requires sufficient time and frequency resource.

Proposal 4: For Case2, the gNB may broadcast assistance information (e.g., RP position) for the UE to calculate and compensate the common TA, while the residual offset could be estimated and corrected through PRACH.

Observation 5: When UE has no available reference time for a long time, TA adjustments via TAC in the connected mode face critical challenges of excessive signaling overhead and outdated TAC.

Proposal 5: The RP position should be appropriately selected.

Observation 6: The availability of UE's GNSS information has less impact on the specs about initial access procedure before RACH initiation.

Proposal 6: For case1, the SIB9-based scheme could be adopted for FO estimation.

Observation 7: The closed-loop frequency synchronization mechanism is not needed when using the SIB9-based scheme, since accurate FO could be estimated.

Proposal 7: For case 2, the RP indication-based scheme could be used, where the gNB broadcasts cell/beam-specific RPs to enable the autonomously common FO calculation and compensation at UE side.

Proposal 8: The closed-loop frequency synchronization mechanism should be introduced when using the RP indication-based common FO compensation scheme.

Observation 8: Due to the large differential time and frequency offset, availability of CS multiplexing would be limited.

Observation 9: Once some new/enhanced preamble formats are introduced, the backward compatibility cannot be guaranteed.

Proposal 9: The need of PRACH enhancement should be firstly specified before studying specific enhancement schemes.

Proposal 10: If the PRACH enhancement is deemed necessary, the schemes studied in Rel-16 can be considered as the baseline.

Proposal 11: Option 3 for PRACH enhancement specified in TR38.821 should be deprioritized due to its substantial spec impacts and considerable workload.

Proposal 12: Necessary optimizations of the preamble reception and RAR windows should be introduced.

- References

- RP-251863, Study on GNSS resilient NR-NTN operation, Jun. 9th-13rd, 2025.

- 3GPP TR 38.821, Solutions for NR to support NTN

- R1-2009748, FL Summary on enhancements on UL time and frequency synchronization for NR NTN, Oct. 26th–Nov. 13th, 2020

- R1-1904225, Fraunhofer IIS, Fraunhofer HHI, Thales, Timing Advance Adjustments for Satellite Communications (NTN), Apr. 8th-12th 2019

- R1-2111127, FL Summary #4 on enhancements on UL time and frequency synchronization for NR NTN, Nov. 11th– 19th, 2021

- 3GPP TS 38.300, NR; NR and NG-RAN Overall description; Stage-2