3GPP TSG RAN WG1 #122-bis			R1-2507020

Prague, Czech Republic, October 13th – 17th, 2025

Source:	CMCC

Title:	Discussion on AI/ML in 6GR interface

Agenda item:	11.6

Document for:	Discussion & Decision

Introduction

In RAN#108, study item on 6G Radio was approved with the following RAN1 led objectives [1], one of the objectives is to study AI/ML for 6GR and Radio Access Network, including identification on the AI/ML based use cases for 6GR. The related objective is as following:

The detailed objectives of the study are: - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In last meeting [3], we have made the following general agreement on the study points for each (sub-)use case companies proposed.

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., - no collaboration/interaction - UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

In this contribution, we provide our views on new AI/ML use cases and 5G-A use cases for 6GR interface.

General discussion on the selection principle of AI/ML use cases

6G AI/ML study in Rel-20 is expected to identify use cases with compelling trade-off between performance, complexity, etc. Before we discuss the evaluation methodology of each use case, we need to decide the categorization method of AI use cases.

Based on the FL summary in last meeting [4], more than 30 (sub-)use cases have been proposed by companies, covering nearly all the aspects of physical layer, including waveform, frame structure, channel coding, modulation, initial access, MIMO, energy efficiency, physical layer control, data scheduling and HARQ operation, ISAC, spectrum utilization and aggregation.

By RAN1#123, RAN1 need to collect AI/ML use cases in all potential components in physical layer design, targeting to select some use cases and the selected use cases will be distributed to respective related agenda from RAN1#124. We think in the first three meetings, RAN1 only needs to identify the definition of each (sub-)use cases including the model input/output, and some high-level evaluation methodology and specification impacts. The potential down-selection of (sub-)use cases can be performed in respective related agenda separately.

Proposal 1: It is proposed to identify/categorize (sub-)use cases based on the related RAN1 agenda of Rel-20 Study of 6GR.

Based on the FL summary in last meeting [4] and companies’ contribution, the following list of 6GR AI/ML (sub-)use cases can be summarized for further study:

- Waveform

AI for waveform

AI for PA non-linearity handling

- Frame structure

AI-powered adaptive frame structure

- Channel coding

AI/ML based channel coding

Scrambler/descrambler or interleaver/de-interleaver

- Modulation

AI for modulation and demodulation

Joint modulation and precoding

- Initial access

DL Tx beam spatial/time domain prediction in initial access

AI-based PRACH receiver

- MIMO

CSI prediction and CSI-RS/SRS overhead reduction

DMRS design with AI receiver/transceiver

CSI compression, including JSCC/JSCCM...

TPMI compression

Power control/Path loss production

RS sequence optimization

- Energy efficiency

AI for NES

AI based DRX

- Physical layer control, data scheduling and HARQ operation

JSCCM for HARQ

AI for HARQ

AI compression for DCI

DCI prediction

AI for link adaptation /MCS selection

- AI for ISAC

AI/ML-enabled RAN Digital Twin

- 6GR spectrum utilization and aggregation

Spectrum Sensing

- Service related

Traffic prediction

NET4AI related use case, e.g., token communication

- Led by other WG

LLM-Based Prediction of Measurement Events

Anomaly detection and fault prediction

Proposal 2: The following categorization structure of 6GR AI/ML (sub-)use cases can be considered:

- Waveform

- Frame structure

- Channel coding

- Modulation

- Initial access

- MIMO

- Energy efficiency

- Physical layer control, data scheduling and HARQ operation

- ISAC

- 6GR spectrum utilization and aggregation

- Service related

- Led by other WG

MIMO related AI/ML use cases

Case#1: AI/ML based CSI prediction

Motivation and use case description

In 6GR, with the consideration on new carrier frequency (e.g. U6G), for gNB with larger scale on antenna design (e.g. up to 128/256 TxRUs compared with 32/64 TxRUs in 5G, and up to 1024/2048 antenna elements compared with 128/192 antenna elements in 5G), the number of CSI-RS port will also increase rapidly compared with 5G, which will consume lots of CSI-RS overhead.

Besides, larger bandwidth (e.g. up to 200/400MHz compared with 100MHz in 5G) may appear in future 6G network, if the characteristics of frequency selection in 6G carrier frequency is relatively comparable to that in 5G frequency (e.g., 3.5G), then the frequency domain resources occupied by CSI-RS will also increase significantly.

In Rel-18/19 study on AI/ML CSI prediction in time domain, UE can use UE-sided model to predict future CSI(s) in prediction window, based on the historic CSI(s) in the measurement window. With a similar spirit as temporal CSI prediction, we propose to consider AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction.

For AI/ML-based spatial domain CSI prediction, as illustrated in Figure 1, it only requires UE to measure CSI of partial ports, and AI/ML model can use it as model input to predict full CSI of all ports as model output.

For AI/ML-based frequency domain CSI prediction, as illustrated in Figure 2, it only requires UE to measure CSI of partial subbands/RBs, and AI/ML model can use it as model input to predict full CSI of all subands/RBs as model output.

Overall, AI/ML based spatial/frequency CSI prediction can largely reduce measurement overhead with good channel acquisition accuracy.

Figure 1. AI/ML-based spatial domain CSI prediction

Figure 2. AI/ML-based frequency domain CSI prediction

Proposal 3: It is proposed to study AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction in 6GR study.

Proposal 4: For AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction use cases, AI model takes the measured channel matrix of partial ports or partial RBs as the input and outputs the channel matrix of all ports and all RBs.

Assumption on training types and model location for inference

Since channel characteristics are relatively stable in a specific scenario, offline training is enough. The label for CSI prediction use case is channel matrix of full ports/RBs, which can be constructed by measurement of CSI at UE.

Theoretically, both NW-sided model and UE-sided model can be considered here. However, if the model input is precoding matrixes, usually the AI/ML model used for CSI prediction cannot achieve comparable performance as that with raw channel matrixes as model input. In the legacy 5G CSI framework, UE can only report the PMI with format of precoding matrixes. Considering large reporting overhead of raw channel matrix in air interface, UE reporting explicit CSI, i.e., raw channel matrix, is not supported in 5G. So, the feasibility of NW side model needs further study, which requires explicit CSI, i.e., raw channel matrix reporting.

In this contribution, UE-sided AI model is considered, which takes the measured channel matrix of partial ports or partial RBs as the input and outputs the channel matrix of all ports and all RBs.

Proposal 5: For AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction use cases, offline training and at least UE-sided model should be considered.

Collaboration/interaction between UE and NW

Since AI/ML-based CSI prediction requires one-sided model, collaboration/interaction between UE and NW is not needed.

Potential specification impact

With UE-sided model, data collection (i.e. measurement of channel matrix) can reuse legacy CSI framework. Specification impact on sparse RS pattern configuration for inference and monitoring mechanisms can be further studied.

Summary of spatial/frequency domain CSI prediction

Above all, the definition, KPI, benchmark, model location, etc, of spatial/frequency domain CSI prediction use case are summarized in Table 1.

Table 1. Summary of spatial/frequency domain CSI prediction

| Use case | Spatial/Frequency domain CSI prediction |
| --- | --- |
| Model input | channel matrix of partial ports/RBs |
| Model output | channel matrix of full ports/RBs |
| Label | Measured channel matrix of full ports/RBs |
| Training types | Offline training |
| KPI | Intermediate KPI: SGCS Eventual KPI: Throughput |
| Benchmark | Option 1: Sample and hold. The channel matrixes for partial measured ports are used as the CSI for adjacent ports. The channel matrixes for measured RBs are used as the CSI for adjacent RBs. Option 2: Traditional channel interpolation algorithm |
| Preliminary result | Spatial domain CSI prediction: up to up to 18%, 45%, and 152% SGCS improvement when the ratio of measured ports to total ports is 1/2, 1/4, and 1/8. Frequency domain CSI prediction: up to 5%, 7%, and 9% SGCS improvement when the frequency density for measurement is 1/2, 1/4, 1/8. |
| Model location for inference | UE-sided model |
| Collaboration/interaction between UE and NW | No collaboration |
| High level potential specification impact | Specification impact on RS pattern configuration during inference and monitoring mechanism |
| Model backbone | Cmixer |

Proposal 6: For AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction use cases, RAN1 to take Table 1 in R1-2507020 for further study.

Performance evaluation

The simulation parameters, construction of data set and training parameters for AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction are summarized as in Table 2.

Table 2. Simulation assumption for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban |
| Channel model | According to TR 38.901 |
| SCS | 30kHz |
| Bandwidth | 20MHz |
| Carrier Frequency | 7GHz |
| Tx power | 44dBm |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4 ports: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| UE distribution | 80% outdoor (30km/h), 20% indoor (3km/h) |
| Data format | Float 64 |
| Data set | 42k samples, training set: validation set: test set = 8: 1:1 |
| Loss function | MSE |
| AI model | Cmixer |

Based on TR 38.843 [2], for the CSI-related use cases, i.e. CSI compression and temporal CSI prediction, SGCS (Squared Generalized Cosine Similarity) are widely used as the intermediate KPI for performance evaluation. Here is the definition of SGCS for temporal CSI prediction.

Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation.

Similarly, most evaluation results in Rel-18 study for AI interface only consider the throughput as the eventual KPI, which can be reused for AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction in 6GR study.

Proposal 7: Take the following metrics for performance evaluation for AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction in 6GR study.

- Intermediate KPI: SGCS

- Eventual KPI: Throughput

The benchmark of this use case needs to be aligned. Two options of benchmark of spatial/frequency domain CSI prediction sub use cases are considered here:

- Option 1: Sample and hold. The channel matrixes for partial measured ports are used as the CSI for adjacent ports. The channel matrixes for measured RBs are used as the CSI for adjacent RBs.

- Option 2: Traditional channel interpolation algorithm

Proposal 8: Benchmark of AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction need to be studied, the following two options can be considered:

- Option 1: Sample and hold. The channel matrixes for partial measured ports are used as the CSI for adjacent ports. The channel matrixes for measured RBs are used as the CSI for adjacent RBs.

- Option 2: Traditional channel interpolation algorithm

Regarding spatial domain CSI prediction, as shown in Table 3, we respectively compare SGCS of AI model with baseline of sample && hold and traditional interpolation, under different ratio of measured ports to all ports. The measured ports are uniformly selected from all ports, and the measurement pattern is fixed over different RBs.

Table 3. Simulation results of spatial domain CSI prediction

| Ratio of measured ports to all ports | 1/2 | 1/4 | 1/8 |
| --- | --- | --- | --- |
| Ports of model output | [0, 1, 2, ..., 31] | [0, 1, 2, ..., 31] | [0, 1, 2, ..., 31] |
| SGCS of benchmark 1 (Sample and hold) | 0.843 | 0.674 | 0.425 |
| SGCS of benchmark 2 (Traditional interpolation) | 0.877 | 0.680 | 0.374 |
| SGCS of AI | 0.996 | 0.978 | 0.942 |
| Gain of AI compared with sample and hold | 18% | 45% | 122% |
| Gain of AI compared with traditional interpolation | 14% | 44% | 152% |
| FLOPs | 19.7M | 19.5M | 19.4M |
| Number of model parameters | 0.12M | 0.12M | 0.12M |

With the decrease in number of measured ports, SGCS of two benchmarks, no matter sample and hold or traditional interpolation, decreases rapidly, because the correlation of channel matrix among different ports is weaker with sparser measurement CSI-RS pattern.

Compared to benchmark methods, the AI model improves spatial domain CSI prediction performance (in terms of SGCS) by up to 18%, 45%, and 152% when the ratio of measured ports to total ports is 1/2, 1/4, and 1/8, respectively.

On the other hand, a good prediction performance (>0.94 in terms of SGCS) for AI/ML based spatial domain CSI prediction can be achieved with CSI-RS overhead reduced to 1/2, 1/4, and 1/8, respectively.

Observation 1: Compared to benchmark methods like sample-and-hold or traditional interpolation, the AI model improves spatial domain CSI prediction performance (in terms of SGCS) by up to 18%, 45%, and 152% when the ratio of measured ports to total ports is 1/2, 1/4, and 1/8, respectively.

Observation 2: A good prediction performance (>0.94 in terms of SGCS) for AI/ML based spatial domain CSI prediction can be achieved with CSI-RS overhead reduced to 1/2, 1/4, and 1/8, respectively.

Regarding frequency domain CSI prediction, as shown in Table 4, we respectively compare SGCS of AI model with benchmark of sample & hold and traditional interpolation, under different frequency density for measurement. The measured RBs are uniformly selected from all RBs, and all ports are measured on each measured RB.

Table 4. Simulation results of frequency domain CSI prediction

| RB density for measurement | 1/2 | 1/4 | 1/8 |
| --- | --- | --- | --- |
| SGCS of benchmark 1 (Sample and hold) | 0.966 | 0.941 | 0.914 |
| SGCS of benchmark 2 (Traditional interpolation) | 0.952 | 0.929 | 0.902 |
| SGCS of AI | 0.999 | 0.995 | 0.986 |
| Gain of AI compared with sample and hold | 3% | 6% | 8% |
| Gain of AI compared with traditional interpolation | 5% | 7% | 9% |
| FLOPs | 19.1M | 18.6M | 18.3M |
| Number of model parameters | 0.12M | 0.12M | 0.12M |

With the decrease of frequency density for measurement, SGCS of two benchmarks, no matter sample and hold or traditional interpolation, decreases slightly, because the frequency domain correlation of channel matrix among different RBs is relatively high.

Compared to benchmark methods like sample-and-hold or traditional interpolation, the AI model improves frequency domain CSI prediction performance (in terms of SGCS) by up to 5%, 7%, and 9%, when the frequency density for measurement is 1/2, 1/4, 1/8, respectively.

On the other hand, a good prediction performance (>0.98 in terms of SGCS) for AI/ML based frequency domain CSI prediction can be achieved with CSI-RS overhead reduced to 1/2, 1/4, 1/8, respectively.

Observation 3: Compared to benchmark methods like sample-and-hold or traditional interpolation, the AI model improves frequency domain CSI prediction performance (in terms of SGCS) by up to 5%, 7%, and 9%, when the frequency density for measurement is 1/2, 1/4, 1/8, respectively.

Observation 4: A good prediction performance (>0.98 in terms of SGCS) for AI/ML based frequency domain CSI prediction can be achieved with CSI-RS overhead reduced to 1/2, 1/4, 1/8, respectively.

Regarding joint spatial and frequency domain CSI prediction, as shown in Table 5, we respectively compare SGCS of AI model with benchmark under different ratio of measured ports to all ports and different frequency density for measurement. The measured ports and RBs are uniformly selected from all ports and RBs, and measured port pattern is fixed on each measured RB.

Table 5. Simulation results of joint spatial and frequency domain CSI prediction

| Ratio of measured ports to all ports | 1/2 | 1/4 | 1/2 | 1/4 | 1/4 | 1/8 |
| --- | --- | --- | --- | --- | --- | --- |
| RB density for measurement | 1/2 | 1/2 | 1/4 | 1/4 | 1/8 | 1/4 |
| SGCS of benchmark 1 (Sample and hold) | 0.816 | 0.665 | 0.796 | 0.650 | 0.635 | 0.412 |
| SGCS of benchmark 2 (Traditional interpolation) | 0.838 | 0.652 | 0.819 | 0.638 | 0.624 | 0.408 |
| SGCS of AI | 0.995 | 0.981 | 0.993 | 0.982 | 0.97 | 0.946 |
| Gain of AI compared with sample and hold | 22% | 48% | 25% | 51% | 53% | 130% |
| Gain of AI compared with traditional interpolation | 19% | 50% | 21% | 54% | 55% | 132% |
| FLOPs | 18.9M | 18.8M | 18.5M | 18.4M | 18.3M | 18.4M |
| Number of model parameters | 0.12M | 0.12M | 0.12M | 0.12M | 0.12M | 0.12M |

The decrease of ratio of measured ports to all ports results in more severe reduction of SGCS compared to the decrease of RB density, since the frequency domain correlation of channel matrix among different RBs is higher than spatial domain correlation of channel matrix among different ports.

A good prediction performance (>0.94 in terms of SGCS) for AI/ML based spatial and frequency domain CSI prediction can be achieved with CSI-RS overhead reduced to 1/32.

Observation 5: A good prediction performance (>0.94 in terms of SGCS) for AI/ML based joint spatial and frequency domain CSI prediction can be achieved with CSI-RS overhead reduced to 1/32.

Case#2: AI/ML based DMRS enhancement

In 5G-NR, DMRS and data symbols are orthogonally allocated in time-frequency resources. In Release 18, the PDSCH DMRS port number was increased to 24 to support multi-user MIMO. This results in a PDSCH DMRS overhead of up to 16.7% (= 2 / 12, assuming PDCCH occupies 2 symbols) for a total of 24 front-loaded DMRS ports. If additional DMRS is configured, the maximum PDSCH DMRS overhead doubles to 33.3%.

In 6G, to achieve higher spectral efficiency, PDSCH DMRS port numbers may expand substantially, leading to increased DMRS overhead. Therefore, AI/ML-based DMRS enhancement can be considered. This involves combining low-overhead DMRS at the transmitter with an AI-assisted receiver to improve channel estimation/demodulation performance with reduced PDSCH DMRS overhead.

Proposal 9: It is proposed to study AI/ML-based DMRS enhancement in 6GR study.

Regarding low-overhead DMRS at the transmitter, two approaches can be considered:

- Approach 1: Sparse orthogonal DMRS, which can be further divided into:

  - Option 1 (Intra-RB sparse): Reduced DMRS density within a PRB.

  - Option 2 (Inter-RB sparse): DMRS mapping confined to specific PRB subsets.

  - Option 3 (Inter-slot sparse): DMRS allocation to partial slots during PDSCH/PUSCH repetition.

  - Combination of the above options.

- Approach 2: Non-Orthogonal DMRS and Superimposed with data (abbr., Superimposed Pilot (SIP)). In this case, data and pilot are non-orthogonally carried on the resource grid.

Proposal 10: Regarding low-overhead DMRS at the transmitter for AI/ML-based DMRS enhancement, consider

- Approach 1: Sparse orthogonal DMRS

  - Option 1 (Intra-RB sparse): Reduced DMRS density within a PRB.

  - Option 2 (Inter-RB sparse): DMRS mapping confined to specific PRB subsets.

  - Option 3 (Inter-slot sparse): DMRS allocation to partial slots during PDSCH/PUSCH repetition.

  - Combination of the above options.

- Approach 2: Non-Orthogonal DMRS and Superimposed with data (SIP)

Sparse Orthogonal DMRS

Use case description

Use case definition

For AI-assisted receiver for sparse orthogonal DMRS, three alternatives can be considered:

- Alt 1: AI-based channel estimation. This may achieve better channel estimation accuracy, improved interference resistance (including inter-UE and intra-UE interference), and/or lower DMRS overhead.

  - Model input: Received signal at DMRS REs

  - Model output: Channel matrix of the whole allocated REs

- Alt 2: AI receiver combining channel estimation and equalization.

  - Model input: DMRS pattern and received signal at the whole allocated Res

  - Model input: Interim value for demodulation

- Alt 3: AI receiver combining channel estimation, equalization, and demodulation processing.

  - Model input: DMRS pattern and received signal at the whole allocated Res

  - Model input: LLR for channel decoding

Figure 3. AI receiver for sparse orthogonal DMRS.

Proposal 11: For AI-assisted receiver for sparse orthogonal DMRS, three alternatives can be considered:

- Alt 1: AI-based channel estimation.

  - Model input: Received signal at DMRS REs

  - Model output: Channel matrix of the whole allocated REs

- Alt 2: AI receiver combining channel estimation and equalization.

  - Model input: DMRS pattern and received signal at the whole allocated Res

  - Model input: Interim value for demodulation

- Alt 3: AI receiver combining channel estimation, equalization, and demodulation processing.

  - Model input: DMRS pattern and received signal at the whole allocated Res

  - Model input: LLR for channel decoding

Training type

For AI-assisted receiver for sparse orthogonal DMRS, offline training at UE side should serve as the baseline approach. Nevertheless, considering the time-varying nature of channel environments, the need for online fine-tuning may be further studied.

Proposal 12: For AI-assisted receiver for sparse orthogonal DMRS, adopt offline training at UE side as baseline.

- FFS: needs for online fine-tuning

Label construction

Regarding label construction for offline training at UE side—specifically for acquiring channel matrix and data labels across allocated REs—both simulation-based and field data collection approaches may be considered:

- Simulation-based: Labels may be generated through system level simulation (SLS) or link level simulation (LLS), leveraging predefined channel models and transmission parameters

- Field data collection-based: Labels can be obtained via the reception of known reference signals (e.g., PRS or predefined data sequences) during actual transmission scenarios.

Proposal 13: For AI-assisted receiver for sparse orthogonal DMRS, the following label construction methods can be considered:

- Simulation-based: Utilize System Level Simulation (SLS) or Link Level Simulation (LLS) to generate labels.

- Field data collection-based: Acquire labels by receiving known reference signals or data files.

Model location for inference

For AI-assisted receiver for sparse orthogonal DMRS, UE-sided model is assumed for inference.

Proposal 14: For AI-assisted receiver for sparse orthogonal DMRS, consider UE-sided model for inference.

Collaboration/interaction between UE and NW

For AI-assisted receiver for sparse orthogonal DMRS, both model training and inference are executed entirely at the UE side. Consequently, collaboration/interaction between the UE and NW seems unnecessary for this specific functionality.

Proposal 15: For AI-assisted receiver for sparse orthogonal DMRS, explicit collaboration/interaction between the UE and NW is not required.

High level potential specification impact

Regarding specification impact for sparse orthogonal DMRS, a UE capability report should be introduced to indicate support for such AI-assisted receivers. This ensures network awareness of UE processing capabilities and optimizes resource allocation.

Although termed "sparse DMRS," existing NR DMRS patterns may be reused for 6GR UEs equipped with AI-assisted receivers without requiring new dedicated patterns. For instance:

- In poor channel conditions (e.g., low SNR, high mobility), traditional receivers typically necessitate additional DMRS symbols for accurate channel tracking.

- In contrast, AI-assisted receivers could achieve comparable performance using front-loaded DMRS only (effectively a special case of sparse DMRS), thereby reducing pilot overhead.

Additionally, dedicated sparse DMRS patterns optimized for AI-assisted receivers may be explored in 6GR, such as:

- Option 1 (Intra-RB sparse): Reduced DMRS density within a PRB (e.g., 2 REs per OFDM symbol).

- Option 2 (Inter-RB sparse): DMRS mapping confined to specific PRB subsets (e.g., allocated every two RBs).

- Option 3 (Inter-slot sparse): DMRS allocation to partial slots during PDSCH/PUSCH repetition (e.g., every two slots).

Proposal 16: For AI-assisted receiver for sparse orthogonal DMRS, introduce UE capability reporting for AI-assisted receivers supporting sparse orthogonal DMRS.

Proposal 17: For AI-assisted receiver for sparse orthogonal DMRS, either

- Reuse existing NR DMRS pattern, or

- Design new DMRS pattern, with the following options proposed for further study:

  - Option 1 (Intra-RB sparse): Reduced DMRS density within a PRB.

  - Option 2 (Inter-RB sparse): DMRS mapping confined to specific PRB subsets.

  - Option 3 (Inter-slot sparse): DMRS allocation to partial slots during PDSCH/PUSCH repetition.

Summary of Sparse Orthogonal DMRS

Table 6. Summary of Sparse Orthogonal DMRS.

| Use case | Sub-case A: AI-based channel estimation | Sub-case B: AI receiver combining channel estimation and equalization | Sub-case C: AI receiver combining channel estimation, equalization, and demodulation processing |
| --- | --- | --- | --- |
| Model input | Received signal at DMRS REs | DMRS pattern and received signal at the whole allocated REs | DMRS pattern and received signal at the whole allocated REs |
| Model output | Channel matrix of the whole allocated REs | Interim value for demodulation | LLR for channel decoding |
| Label | Channel matrix | Channel matrix | Channel matrix and data |
| Training types | Offline training at UE side as baseline. - FFS: needs for online fine-tuning |  |  |
| KPI | BLER and throughput |  |  |
| Model location for inference | UE-sided model for inference |  |  |
| Collaboration/interaction between UE and NW | No collaboration |  |  |
| Potential specification impact | - UE capability reporting for AI-assisted receivers supporting sparse orthogonal DMRS. - Potential design new DMRS pattern, with the following options proposed for further study: - Option 1 (Intra-RB sparse): Reduced DMRS density within a PRB. - Option 2 (Inter-RB sparse): DMRS mapping confined to specific PRB subsets. Option 3 (Inter-slot sparse): DMRS allocation to partial slots during PDSCH/PUSCH repetition. |  |  |
| Model backbone | - MLP-mixer |  |  |

Proposal 18: For AI/ML based sparse orthogonal DMRS, RAN1 to take Table 6 in R1-2507020 for further study.

Performance evaluation

Evaluation assumption

In our preliminary evaluation, Alt 1 (AI-based channel estimation using an MLP-Mixer model) is evaluated under the following DMRS patterns (as illustrated in Figure 4):

- Common configuration: Two FDM-multiplexed DMRS ports (e.g., port 0 and port 2), double-symbol DMRS structure.

- Baseline: Rel-15 DMRS pattern with one additional DMRS (48 REs per RB)

- Case 1 (symbol-level sparsity, 1/2 density): Rel-15 DMRS pattern with front-loaded DMRS only (24 REs per RB)

- Case 2 (subcarrier-level sparsity, 1/2 density): New DMRS pattern with one additional DMRS, using 2 REs per DMRS symbol per port (24 REs per RB)

- Case 3 (Joint symbol-and-subcarrier-level sparsity, 1/4 density): New DMRS pattern with front-loaded DMRS only, using 2 REs per DMRS symbol per port (12 REs per RB)

Figure 4. DMRS pattern for sparse orthogonal DMRS

The simulation parameters, construction of data set and training parameters for sparse orthogonal DMRS are summarized as in Table 7.

Table 7. Simulation assumption for sparse orthogonal DMRS

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| SCS | 15KHz |
| Channel | CDL-C |
| UE speed | 120km/h |
| Delay spread | 300ns |
| Tx antenna | 2 Tx ports, [M, N, P, Mg, Ng] = [1, 1, 2, 1, 1] |
| Rx antenna | 2 Rx ports, [M, N, P, Mg, Ng] = [1,1, 2, 1, 1] |
| TDRA | S = 2, L = 12, mapping type A |
| FDRA | 10RBs |
| Layer (rank) | 2 |
| Digital precoder | W = [1, 0; 0, 1]H |
| Channel estimation | Baseline: LS + Linear interpolation AI-assisted channel estimator (Alt 1): MLP-Mixer |
| Channel equalization | MMSE |
| MCS | TS 38.214 Table 5.1.3.1-1 MCS index table 1, MCS = 13 (16QAM, R = 490/1024) |
| SNR | 8~18 in 2dB steps |
| Data format | Float 64 |
| Data set | 15k samples for model training, with training set: validation set = 2: 1 The trained AI-based channel estimator replaces the traditional estimator during testing, utilizing an additional 6k samples for performance evaluation. |
| Loss function | MSE |
| KPI | BLER and throughput |

Preliminary simulation results

(a) BLER vs. SNR

(b) Throughput vs. SNR

Figure 5. Preliminary simulation results for sparse orthogonal DMRS.

Figure 5. presents the BLER and throughput performance of an AI-assisted receiver (specifically employing an AI-based channel estimator) for sparse orthogonal DMRS. Under high-speed conditions (120 km/h) and the CDL-C channel model with 300 ns delay spread, the following observations can be made:

- Case 2 (subcarrier-level sparsity, 1/2 density) demonstrates superior performance compared to other DMRS patterns:

  - Given 10% BLER target, it achieves approximately 6dB SNR gain relative to the baseline (i.e., 12 dB vs. 18 dB).

  - For throughput, it achieves 83% gain at 8 dB SNR, decreasing to 42% gain at 18 dB SNR, indicating that lower SNR values yield higher throughput gains compared to the baseline.

- Case 1 (symbol-level sparsity, 1/2 density) exhibits worse BLER performance than the baseline but provides 10%–14% throughput gain across the evaluated SNR range.

- Case 3 (Joint symbol-and-subcarrier-level sparsity, 1/4 density) shows significantly worse BLER performance but achieves 28%–9% throughput gain compared to the baseline.

Based on these results, the AI-assisted receiver with sparse orthogonal DMRS consistently outperforms legacy NR DMRS configurations using traditional channel estimators under high-speed (120 km/h) CDL-C channel conditions with 300 ns delay spread. Among the evaluated cases, Case 2 (subcarrier-level sparsity) delivers the best overall performance, characterized by:

- Approximately 6 dB SNR gain at the 10% BLER target (12 dB vs. 18 dB).

- Throughput gains ranging from 83% at 8 dB SNR to 42% at 18 dB SNR, with more substantial improvements observed at lower SNR values.

Observation 6: Under high-speed conditions (120 km/h) with a CDL-C channel model and 300 ns delay spread, AI-assisted receiver—specifically employing an AI-based channel estimator with sparse orthogonal DMRS—outperforms legacy NR DMRS configurations that use traditional channel estimators. Among the evaluated cases, Case 2 (subcarrier-level sparsity, 1/2 density) demonstrates the best overall performance, characterized by:

- Approximately 6 dB SNR gain at the 10% BLER target (12 dB vs. 18 dB).

- Throughput gains ranging from 83% at 8 dB SNR to 42% at 18 dB SNR, with more substantial improvements observed at lower SNR values.

- This case utilizes a new DMRS pattern featuring one additional double-symbol DMRS, with 2 REs per DMRS symbol per port (resulting in 24 REs per RB).

Non-Orthogonal DMRS and Superimposed with data (SIP)

Use case description

Use case definition

For Non-Orthogonal DMRS and Superimposed with data (SIP), AI receiver combining channel estimation, equalization, and demodulation processing can be considered:

- Model input: SIP pattern and received signal at the whole allocated REs

- Model input: LLR for channel decoding

The SIP receiver consists of two main modules: a Channel Estimator (Mixer-MLP based) and a Data Detector (Transformer based).

Figure 6. AI receiver for SIP.

Proposal 19: For Non-Orthogonal DMRS and Superimposed with data (SIP), AI receiver combining channel estimation, equalization, and demodulation processing can be considered.

- Model input: SIP pattern and received signal at the whole allocated Res

- Model input: LLR for channel decoding

Training type

For AI-assisted receiver for SIP, offline training at UE side should serve as the baseline approach. Nevertheless, considering the time-varying nature of channel environments, the need for online fine-tuning may be further studied.

Proposal 20: For AI-assisted receiver for SIP, adopt offline training at UE side as baseline.

- FFS: needs for online fine-tuning

Label construction

Regarding label construction for offline training at UE side,

- Channel Matrix Labels can be acquired through

  - Simulation-based: System Level Simulation (SLS) or Link Level Simulation (LLS), leveraging predefined channel models and transmission parameters, or,

  - Field data collection-based: Reception of known reference signals (e.g., PRS or predefined data sequences) during actual transmissions.

- Data Labels (including data, superimposed signals (i.e., data + SIP pilot) and received signals) can be generated via LLS by leveraging the channel matrix.

Proposal 21: For AI-assisted receiver for SIP, the following label construction methods can be considered:

- Channel Matrix Labels:

  - Simulation-based: Generated via SLS/LLS with predefined channel models and transmission parameters, or,

  - Field data collection-based: Acquired from known reference signals during actual transmissions.

- Data Labels (including data, superimposed signals (i.e., data + SIP pilot) and received signals):

  - Simulation-based: Generated via LLS using the channel matrix for superimposed signals and received signals.

Model location for inference

For AI-assisted receiver for SIP, UE-sided model is assumed for inference.

Proposal 22: For AI-assisted receiver for SIP, consider UE-sided model for inference.

Collaboration/interaction between UE and NW

For AI-assisted receiver for SIP, both model training and inference are executed entirely at the UE side. Consequently, collaboration/interaction between the UE and NW seems unnecessary for this specific functionality.

Proposal 23: For AI-assisted receiver for SIP explicit collaboration/interaction between the UE and NW is not required.

High level potential specification impact

Regarding specification impact for SIP, a UE capability report should be introduced to indicate support for SIP receivers. This ensures network awareness of UE processing capabilities and optimizes resource allocation.

The SIP transmission mechanism—specifically the non-orthogonal superposition of data and pilot signals within the same RE—requires standardized definition of the following technical characteristics:

- Non-orthogonal resource mapping patterns

- Receiver interference cancellation requirements

- Power allocation between data and pilot symbols

Proposal 24: For AI-assisted receiver for SIP, introduce UE capability reporting for supporting SIP receivers.

Proposal 25: Standardize the SIP transmission mechanism with definitions for:

- Non-orthogonal resource mapping patterns

- Receiver interference cancellation requirements

- Power allocation between data and pilot symbols

Summary of Non-Orthogonal DMRS and Superimposed with data (SIP)

Table 8. Summary of SIP.

| Use case | AI receiver combining channel estimation, equalization, and demodulation processing |
| --- | --- |
| Model input | DMRS pattern and received signal at the whole allocated REs |
| Model output | LLR for channel decoding |
| Label | Channel Matrix label and Data Labels (including data, superimposed signals (i.e., data + SIP pilot) and received signals) |
| Training types | Offline training at UE side as baseline. - FFS: needs for online fine-tuning |
| KPI | BLER and throughput |
| Benchmark | - DMRS pattern: Rel-15 DMRS pattern - Channel estimation: LS + Linear interpolation - Channel equalization: MMSE - Demodulation: LLR-based soft demodulation |
| Model location for inference | UE-sided model for inference |
| Collaboration/interaction between UE and NW | No collaboration |
| Potential specification impact | - UE capability reporting for AI-assisted receivers supporting SIP. - SIP transmission mechanism, e.g., - Non-orthogonal resource mapping patterns - Receiver interference cancellation requirements - Power allocation between data and pilot symbols |
| Model backbone | Channel Estimator (Mixer-MLP based) + Data Detector (Transformer based) |

Proposal 26: For AI/ML based Non-Orthogonal DMRS and Superimposed with data (SIP), RAN1 to take Table 8 in R1-2507020 for further study.

Performance evaluation

Evaluation assumption

Baseline: Traditional channel estimators using legacy NR DMRS configurations

- DMRS pattern: Rel-15 DMRS pattern with one additional two-symbols DMRS with two FDM-multiplexed ports (48 REs per RB)

- Channel estimation: LS + Linear interpolation

- Channel equalization: MMSE

- Demodulation: LLR-based soft demodulation(16QAM)

SIP:

- SIP pattern: (Pattern1) SIP DMRS spans over each RE for each SIP pilot port, wherein, different SIP DMRS ports are multiplexed via OCC. SIP DMRS power : data power = 0.1 : 0.9

- SIP receiver Model: Channel Estimator (Mixer-MLP based) + Data Detector (Transformer based).

- Training method: Supervised learning with Adam optimizer, MSE for channel estimation + BCE loss

(a) Baseline: Legacy DMRS pattern													(b) SIP pattern

The simulation parameters, construction of data set and training parameters for SIP are summarized as in Table 9.

Table 9. Simulation parameters for SIP.

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| SCS | 15KHz |
| Channel | CDL-C |
| UE speed | 120km/h |
| Delay spread | 300ns |
| Tx antenna | 2 Tx ports, [M, N, P, Mg, Ng] = [1, 1, 2, 1, 1] |
| Rx antenna | 2 Rx ports, [M, N, P, Mg, Ng] = [1,1, 2, 1, 1] |
| TDRA | S = 2, L = 12, mapping type A |
| FDRA | 10RBs |
| Layer (rank) | 2 |
| Digital precoder | W = [1, 0; 0, 1]H |
| MCS | TS 38.214 Table 5.1.3.1-1 MCS index table 1, MCS = 13 (16QAM, R = 490/1024) |
| SNR | 8~18 in 2dB steps |
| Data format | Float 64 |
| Data set | 300k samples for model training, with training set: validation set = 9: 1 The trained AI-based channel estimator replaces the traditional estimator during testing, utilizing an additional 15k samples for performance evaluation. |
| Loss function | 0.2 * MSE (for channel estimator loss) + 0.8 * BCE (for data detector loss) |
| KPI | BLER and throughput |

Preliminary simulation results

- BLER vs. SNR

- Throughput vs. SNR

Figure 7. Preliminary simulation results for SIP.

Figure 7. presents the BLER and throughput performance of an AI-assisted receiver—integrating channel estimation, equalization, and demodulation—for SIP under high-speed conditions (120 km/h) and the CDL-C channel model with 300 ns delay spread. Key observations are summarized below:

- Two alignment assumptions are considered in the evaluation:

  - Under TBS alignment, the coding rate for SIP data is effectively reduced, as the allocated resources increase while the TBS remains the same as in the baseline.

  - Under MCS alignment, the TBS for SIP data increases, as resource allocation grows while the MCS is kept consistent with the baseline.

- Under TBS alignment, SIP consistently outperforms the baseline:

  - Given 10% BLER target, SIP achieves an SNR gain of approximately 7 dB relative to the baseline (11 dB vs. 18 dB).

  - Throughput gains range from 10% to 35%, with the maximum gain of 35% attained at 10 dB SNR.

- Under MCS alignment, SIP performance varies with SNR:

  - At low SNR (below 10 dB), throughput loss between –91% and –11% is observed.

  - At high SNR (above 12 dB), throughput gains range from 34% to 56%, increasing with SNR.

  - Given 10% BLER target, an SNR gain of about 2.8 dB is achieved (15.2 dB vs. 18 dB).

These results demonstrate that the AI-assisted receiver for SIP consistently outperforms legacy NR DMRS configurations using traditional channel estimators under the specified high-speed channel conditions. Among the evaluated cases:

- TBS alignment provides consistent performance gains across the SNR range.

- MCS alignment yields better throughput performance at higher SNR values.

- At 18 dB SNR, the maximum BLER gain (from 10% down to 0.6%) is achieved under TBS alignment, while the maximum throughput gain (56%) occurs under MCS alignment.

Observation 7: Under high-speed conditions (120 km/h) with a CDL-C channel model and 300 ns delay spread, AI-assisted receiver—combining channel estimation, equalization, and demodulation—for SIP outperforms legacy NR DMRS configurations that use traditional channel estimators.

- TBS alignment provides consistent performance gains across the SNR range.

- MCS alignment yields better throughput performance at higher SNR values.

- At 18 dB SNR, the maximum BLER gain (from 10% down to 0.6%) is achieved under TBS alignment, while the maximum throughput gain (56%) occurs under MCS alignment.

Case#3: Joint source, channel coding, and modulation (JSCCM)

Motivation and use case description

Existing AI/ML-based CSI compression methods for CSI feedback only replace the CSI source coding with AI/ML networks, which may lead to a "cliff effect" due to mismatches between actual channel conditions and channel coding schemes. To further enhance CSI feedback efficiency, solutions that combine channel coding (even modulation) techniques with CSI source coding are being explored.

Figure 8. Framework comparison of the AI/ML-based CSI compression, JSCC, and JSCCM schemes.

For Joint Source and Channel Coding (JSCC), the UE first processes CSI data through an AI/ML-based encoder. The output is encoded complex values. These values are then quantized into binary bits. Next, the binary bits pass through a traditional modulation module before being transmitted. The network side follows the reverse process.

For Joint Source, Channel Coding, and Modulation (JSCCM), the process is more integrated. At the UE side, CSI data is fed into an AI/ML-based encoder. The encoder directly outputs modulated complex data, which is then sent over the channel. The network side follows the reverse process.

Proposal 27: To further enhance CSI feedback efficiency and accuracy, solutions that combine CSI source coding with channel coding (even modulation) techniques could be considered.

Summary of Joint source, channel coding, and modulation (JSCCM)

The definition, KPI, benchmark, model location of JSCCM use case are summarized in Table 10.

Table 10. Summary of JSCCM

| Use Case | Joint source, channel coding, and modulation (JSCCM) |
| --- | --- |
| Model input | CSI data, in the form of a full channel matrix or feature vectors |
| Model output | complex-valued modulated symbols |
| Label | Label free |
| Training types | Offline training at both UE and NW sides. FFS: needs for online finetuning |
| KPI | SGCS of CSI and reconstructed CSI |
| Benchmark | (a) Rel-18 AI/ML-based CSI compression with Polar codes and legacy QAM modulations (b) JSCC with legacy QAM modulations |
| Preliminary result | - JSCCM achieves an SGCS score above 0.9 when the SNR exceeds -10dB. - JSCCM models trained at lower SNRs have better SGCS performance. - At a given training SNR, the JSCCM with average power constraint maintains a performance advantage over the JSCCM with constant amplitude constraint throughout the inference SNR range. |
| Model location for inference | A two-sided model deployment at both the UE and the NW. |
| Collaboration/interaction between UE and NW | Need collaboration: - The UE and NW should deploy a pre-trained, matched encoder-decoder model pair. - The inter-vendor training collaboration can follow the procedure in Rel-20 CSI compression: - Option 3a-1; - Option 4-1; - Direction C; - Signaling for data collection and model updating. |
| Potential specification impact | Mechanism for switching: The specification needs to define a mechanism for switching between the JSCCM and traditional CSI feedback methods. |
| Model backbone | Transformer-based |

Proposal 28: For AI/ML based Joint source, channel coding, and modulation (JSCCM), RAN1 to take Table 10 in R1-2507020 for further study.

Performance evaluation

Evaluation assumption

The simulation parameters, construction of data set and training parameters for JSCCM are summarized as in Table 11.

Table 11. Simulation parameters

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban |
| Channel model | According to TR 38.901 |
| SCS | 30kHz |
| Bandwidth | 20MHz |
| Carrier Frequency | 4GHz |
| Tx power | 44dBm |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4 ports: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| UE distribution | 80% outdoor (30km/h), 20% indoor (3km/h) |
| Data format | Float 64 |
| Data set | 105,000 samples, training set: test set = 20:1 |
| Loss function | SGCS |
| AI model | Transformer-based model |

Preliminary simulation results

Figure 9. Framework comparison of the AI/ML-based CSI compression, JSCC, and JSCCM schemes.

Figure 9 presents the SGCS performance of JSCCM. The evaluation is conducted on a dataset of 105,000 samples, each containing CSI vectors from 10 subbands. The AI/ML-based encoder and decoder, implemented as 2-layer transformer models, are deployed at the UE and the network side (NW), respectively. At the UE, the complex-valued output from the encoder is fed into a constraint module to enforce properties (e.g., constant amplitude or average power), thereby shaping the transmitted signal into a standard modulation-like format. The output of the constraint module is 64 symbols.

For both the constant amplitude and average power constraints, we trained separate models at 10dB, 20dB, and 30dB SNR, and evaluated their performance across an SNR range of -15dB to 10dB. Our results show that, for both constraints, an SGCS score above 0.9 is achieved when the SNR exceeds -10dB, and models trained at lower SNRs have better SGCS performance. It is also observed that, at a given training SNR, the average power constrained model maintains a performance advantage over the constant amplitude constrained model throughout the inference SNR range from -15dB to 10dB.

Observation 8：For both constant amplitude and average power constraints, the JSCCM achieves an SGCS score above 0.9 when the SNR exceeds -10dB, and models trained at lower SNRs have better SGCS performance.

Observation 9：At a given training SNR, the JSCCM with average power constraint maintains a performance advantage over the JSCCM with constant amplitude constraint throughout the inference SNR range

Control related AI/ML use cases

A typical DCI payload occupies less than 100 bits and includes, but is not limited to, fields such as FDRA, TDRA, MCS, NDI, RV, HARQ Process Number, TPC, TPI. Both field tests and system-level simulation datasets indicate low mutual information between consecutive transmissions, and it is observed that the DCI payload can be highly predictable from historical payload sequences.

Hence, we propose to study the following potential AI/ML based DCI Tx/Rx enhancement,

- Case 4: Prior-Information-Aided DCI Channel Decoding. Current DCI channel decoding is performed independently per transmission. Integrating prior information (e.g., historical DCI payloads) as soft input can improve channel decoding performance. Related analysis is provided in Section 4.1, and further sources of prior information may be explored in future studies.

- Case 5: Lossless DCI Compression. The BS transmits the compressed DCI to UE. The compression can be realized using AI/ML-based predictors that exploit temporal redundancy in DCI sequences. Related analysis is provided in Section 4.2.

Case#4: Prior-Information-Aided DCI Decoding

Motivation and use case description

Generally, the DCI received by user equipment (UE) changes dynamically over time, depending on factors such as channel conditions, service demands, and network scheduling strategies. These changes are not entirely random but follow certain temporal patterns and channel adaptation mechanisms. For example, the DCI usually includes multiple fields, the MCS field often assigns higher-order values in better channel states. The Frequency Domain Resource Assignment (FDRA) field frequently allocates contiguous resource blocks (RBs) in good conditions to boost throughput, while employing reduced or fragmented RBs in poor conditions to enhance frequency diversity. Certain fields, such as those related to Hybrid Automatic Repeat Request (HARQ), may exhibit periodic or semipersistent behaviour. Furthermore, although the DCI contains many fields, not all fields change in every transmission. Many remain constant over extended periods. Thus, different DCI fields possess distinct update rates and temporal features, indicating temporal correlation and channel dependency. This inherent pattern makes DCI partially predictable, and both information entropy and temporal correlation serve as effective, quantifiable metrics for assessing its predictability.

Based on the above analysis, we propose a prior-information-aided DCI decoder, in order to enhance the existing DCI decoder. The prior-information-aided DCI decoder integrates a traditional DCI decoder with an AI/ML-based DCI prediction network. This network can extract and learn underlying patterns from historical DCIs, enabling accurate prediction of forthcoming DCI. Specifically, the network predicts each bit of the upcoming DCI individually, outputting corresponding soft information (e.g., LLR) for every bit position. This prediction output serves as valuable prior information for the decoder, significantly enhancing performance under poor channel conditions. To enhance prediction accuracy, extra assistant information can also be incorporated, such as CSI (including CQI, PMI, and RI), HARQ-ACK feedback, and SRS measurement results.

The procedure of prior-information-aided DCI decoder is shown in Figure 10. Here the in the figure, the “decoder” block is still traditional decoder. And the prior-information-aided DCI decoder is referred to as “AI/ML DCI decoder” for brevity.

Figure 10. Procedure for Prior-Information-Aided DCI Decoding

Proposal 29: It is proposed to study prior-information-aided DCI decoder at the receiver in 6GR study, where an AI/ML-based DCI prediction network could be used to provide the decoder with accurate prior information and improve the decoding performance significantly.

High level potential specification impact

Regarding prior-information-aided DCI decoder at the receiver, the following aspects could be considered:

- Optimal DCI Field Selection for Prediction: Prioritize DCI fields demonstrating strong temporal correlation and pattern stability, which benefit most from AI/ML-based prediction. The selection can be based on information entropy - lower entropy in the historical DCI field patterns correlates strongly with higher prediction accuracy.

- DCI Dataset Collection for AI/ML-based DCI prediction network training

- Alt 1: The transmitter collects the DCI dataset and distributes it to receivers for model training.

- Alt 2: Each receiver independently gathers its own dataset for training.

- Decoder Switching Mechanism:

- Define criteria for switching between the AI/ML-based DCI decoder and the conventional decoder (e.g., based on prediction confidence, channel conditions, or performance metrics). This is important since both the 6G BS and UE should keep same understanding on which decoder is used. So that the BS can make the optimal decision for PDCCH transmission.

- Indication mechanisms for AI/ML-based DCI decoder: For example, (1) a new DCI format. (2) an optimized CORESET/Search Space Set structure that reduces frequency/time resource allocation while maintaining reliable detection performance. The optimization is enabled by the significant performance gains of the AI/ML-based DCI decoder. (3) lower blind detection attempts due to the optimized CORESET/Search Space Set structure.

Proposal 30: Regarding prior-information-aided DCI decoder, the following aspects could be considered for further studies

- DCI dataset collection for AI/ML-based DCI prediction network training.

  - Alt 1: The transmitter collects the DCI dataset and distributes it to receivers for model training.

  - Alt 2: Each receiver independently gathers its own dataset for training.

- AI decoder/non-AI decoder fallback mechanism:

- Define criteria for switching between the prior-information-aided DCI decoder and the conventional non-AI decoder.

- Signaling/configuration design for prior-information-aided DCI decoder.

- Which DCI fields for prediction as model input

Summary of Prior-Information-Aided DCI Decoding

The definition, KPI, benchmark, model location of prior-information-aided DCI decoder use case is summarized in Table 12.

Table 12. Summary of Prior-Information-Aided DCI Decoding

| Use Case | Prior-Information-Aided DCI Decoding |
| --- | --- |
| Model input | LLR after demodulation, and the predicted LLR from the DCI prediction network. Extra assistant information can also be incorporated, such as CSI (including CQI, PMI, and RI), HARQ-ACK feedback, and SRS measurement results. |
| Model output | Decoded DCI payloads, and predicted LLR for the next transmission |
| Label | Label free |
| Training types | Offline training at the UE side. |
| KPI | BLER performance |
| Benchmark | Traditional DCI decoder: - For convolutional codes: BCJR decoder - For Polar codes: traditional BP decoder, and CA-SCL decoder |
| Preliminary result | - For enhanced Convolutional decoder: achieves >5 dB improvement at BLER@1% and ~1.5 dB at BLER@0.1% compared to the baseline BCJR decoder, where the dataset is from Redcap scenario. - For enhanced Polar decoder: achieves about 5 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder, where the dataset is from Redcap scenario. - For enhanced Polar decoder: achieves about 4 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder, where the dataset is from eMBB scenario. |
| Model location for inference | UE-sided model |
| Collaboration/interaction between UE and NW | No collaboration |
| Potential specification impact | - DCI dataset collection for AI/ML-based DCI prediction network training. - Alt 1: The transmitter collects the DCI dataset and distributes it to receivers for model training. - Alt 2: Each receiver independently gathers its own dataset for training. - AI decoder/non-AI decoder fallback mechanism: - Define criteria for switching between the prior-information-aided DCI decoder and the conventional non-AI decoder. - Signaling/configuration design for prior-information-aided DCI decoder. - Which DCI fields for prediction as model input |
| Model backbone | LSTM based or Transformer based |

Proposal 31: For AI/ML based Prior-Information-Aided DCI Decoding, RAN1 to take Table 12 in R1-2507020 for further study.

Performance evaluation

We evaluate the performance of the proposed prior-information-aided DCI decoder on two datasets, each consisting of DCI format 1_1 samples collected under a single-UE, single-base-station scenario.

Dataset 1 consists of over 7,000 DCI payloads collected in a Redcap scenario, split into an 80% training set and a 20% test set. The DCI payload consists of MCS and FDRA fields. Before constructing the AI/ML-based DCI prediction network, we calculated the normalized entropy and autocorrelation across the DCI payloads to quantize the predictability of DCI. Take the MCS field for example, the normalized entropy was 0.5446, indicating moderate randomness. Meanwhile, significant autocorrelation peaks at lag 1 (0.5889) and lag 10 (0.7032), suggesting strong short-term predictability and the presence of periodic patterns. Given the short-term predictability of DCI, using an LSTM-based AI network for prediction is feasible. Specifically, the AI/ML-based DCI prediction network employs a bidirectional LSTM network architecture with fully connected layers and dropout. The model is trained to predict DCI payloads by minimizing the binary cross-entropy (BCE) loss between predicted and ground-truth payloads. After training, the prediction network achieves an average prediction bit error rate (BER) of approximately 1% on the test dataset.

Dataset 2 is from an eMBB scenario and includes over 209,000 samples. The DCI payload contains 16 fields, such as MCS, FDRA, TDRA, ZP CSI-RS trigger, and HARQ process number. The normalized entropy across the DCI payloads was calculated to be 22.5%, indicating a relatively low level of randomness in the data. As for the DCI prediction model design, to handle the scaled-up dataset (from 7,000 to over 209,000 samples), the previously used LSTM architecture proved to have insufficient capacity. To more effectively capture the complex dependencies within this larger dataset, a transformer-based architecture was adopted for the DCI prediction network. After training, the prediction network achieves an average prediction bit error rate (BER) of less than 1% on the test dataset.

Evaluation assumption

The simulation parameters, construction of data set and training parameters for DCI prediction network are summarized as in Table 13.

Table 13. Simulation parameters

| Parameter | Value |
| --- | --- |
| Scenario | Urban Macro |
| layout | Hexagonal grid, 3 sectors per site, 7 macro sites |
| ISD | 500m |
| Channel model | According to TR 38.901 |
| SCS | 30kHz |
| Bandwidth | 20MHz |
| Carrier Frequency | 4GHz |
| Tx power | 53dBm |
| Duplex mode | Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] |
| Antenna setup and port layouts at gNB | 64 ports: (M, N, P, Mg, Ng; Mp, Np) =(12, 8, 2, 1, 1; 4, 8), (dH, dV) = (0.5, 0.8)λ, +45°, -45° polarization. |
| Antenna setup and port layouts at UE | 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |
| MIMO mode | MU-MIMO |
| DCI format | DCI format 1_1 |
| Traffic mode | FTP model 3, 0.5Mbyte for DL and 0.125 Mbytes for UL, <10%, 20%-40% and ≥50% for low, medium and high load, |
| Scheduling algorithm | Proportional Fair (PF) |
| UE distribution | 80% indoor in houses with 3km/h; 20% outdoor in car with 30km/h; 5 users per Macro TRP |
| Dataset | Dataset 1: over 7,000 samples, training set: test set = 8: 2 Dataset 2: over 209,000 samples, training set: validation set: test set = 7: 1.5: 1.5 |
| Loss function | Loss for dataset 1: BCE Loss for dataset 2: - Phase 1 (Feature classifier training): MSE - Phase 2 (Bit-level predictor training): 0.8 * BCE (for bit-lever loss) + 0.2 * MSE (for sample-level loss) |
| AI model | Model for Dataset 1: A bidirectional LSTM network architecture with fully connected layers and dropout. Model for Dataset 2: A transformer-based architecture with two phases. |

Preliminary simulation results

- Simulation results on Dataset 1 (Redcap scenario)

For the DCI prediction network trained on Dataset 1, the decoding process leverages a traditional convolutional code decoder with a rate of 1/3. This decoder utilizes both the LLR after demodulation and the predicted LLR from the network. BCJR decoding algorithm is used by looking both backward at history and forward at the future and recursively computes the exact a-posteriori probabilities conditioned on the entire observation sequence.

Then we conduct a comprehensive comparison of block error rate (BLER) performance between: (1) the legacy decoder alone without prior information, and (2) legacy decoder with prior information by AI/ML DCI prediction network.

Figure 11. Comparison of BLER Performance for the convolutional decoder in the RedCap Scenario.

The results in Figure 11 demonstrate significant gains: the AI/ML DCI decoder achieves >5 dB improvement at BLER@1% and ~1.5 dB at BLER@0.1% compared to the baseline.

Observation 10: The prior-information-aided DCI decoder (for enhanced Convolutional decoder) achieves >5 dB improvement at BLER@1% and ~1.5 dB at BLER@0.1% compared to the baseline, where the dataset is from Redcap scenario.

Beyond convolutional codes, the AI/ML DCI decoder can also enhance Polar code decoding. Still using Dataset 1, we enhance Polar BP decoding by incorporating prior knowledge as initial LLR values before iterative decoding begins. We evaluate the performance of the enhance Polar BP decoder and compare it with two traditional decoders: a conventional BP decoder without prior information, and a CA-SCL decoder with a list size of L = 8.

Figure 12. Comparison of BLER Performance for the Polar decoder in the RedCap Scenario.

As shown in Figure 12, the results demonstrate significant gains: the AI/ML DCI decoder achieves about 5 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder.

Observation 11: The prior-information-aided DCI decoder (for enhanced Polar decoder) achieves about 5 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder, where the dataset is from Redcap scenario.

- Simulation results on Dataset 2 (eMBB scenario)

We extend evaluate the performance of the enhance Polar BP decoder to Dataset 2, where DCI payloads are collected in eMBB scenarios and the DCI payload includes more fields.

Figure 13. Comparison of BLER Performance for the Polar decoder in the eMBB Scenario.

The comparison of BLER performance is presented in Figure 13. It is observed that, the AI/ML DCI decoder achieves about 4 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder.

Observation 12: The prior-information-aided DCI decoder (for enhanced Polar decoder) achieves about 4 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder, where the dataset is from eMBB scenario.

The significant performance gains observed in both Figure 12 and Figure 13 demonstrate the effectiveness of the proposed prior-information-aided DCI decoder across both RedCap and eMBB scenarios. Furthermore, the proposed decoder exhibits broad applicability, as it is compatible with both convolutional and Polar codes, and can handle DCI payloads that include multiple (>2) fields.

Observation 13: The proposed prior-information-aided DCI decoder demonstrates wide applicability. It is effective in:

- Multiple scenarios: Both RedCap and eMBB.

- Multiple coding schemes: Both convolutional codes and Polar codes.

- Complex payloads: DCI payloads containing multiple (>2) fields.

Case#5: Lossless DCI Compression

Use case description

Reducing DCI payload size can reduce PDCCH overhead, which is benefit for:

- Resource Reallocation: Saved resources (originally occupied by PDCCH) can be reallocated to PDSCH, boosting user-plane throughput in high-traffic scenarios.

- Capacity Expansion: The compressed payload alleviates PDCCH congestion, supporting more UEs in ultra-dense deployments while maintaining decoding reliability.

Use case definition

This use case is enabled by the upstream AI task of DCI payload prediction, which demonstrates high feasibility across all datasets, as indicated by low normalized information entropy and high sample-level accuracy (ranging from 95.62% to 99.81%, refer to section 4.2.3 for details). The prediction task leverages the high temporal correlation in sequential DCI payloads, making lossless DCI compression achievable with a very high compression ratio.

Figure 14. AI-based DCI payload predicator.

An AI-based DCI payload predicator is used to predict the current DCI payload from historical DCI payload sequences. The model is structured as follows:

- Model input: Historical DCI payloads

- Model output: Predicated DCI payload

The predictor consists of two main modules: a Feature Classifier and a Bit-Level Predictor.

- The Feature Classifier comprises an embedding layer, a positional encoding layer, a transformer encoder, and a classification layer.

- The Bit-Level Predictor comprises a Bi-LSTM layer, a positional encoding layer, a transformer encoder, and a bit-level prediction head.

The Feature Classifier extracts key features from the historical DCI payloads and outputs the predicted class of the current DCI payload. In our preliminary evaluation, this module is used to capture frequency scheduling information, as indicated by the FDRA field. For example, in adaptive bitrate video streaming:

- I-frames (Intra-coded frames) may require high throughput, corresponding to the scheduling of a large number of PRBs.

- P-frames (Predictive frames) generally require lower throughput, involving a smaller number of PRBs.

- Background traffic (e.g., heartbeat packets) may need minimal throughput, often scheduled with only one or two PRBs.

The frequency allocation snapshoot of the first 5000 DCI payload records for Dataset 1 and Dataset 2 are demonstrated in Figure 16 and Figure 17, respectively, for information.

Thus, traffic features can be classified based on the FDRA field. Both the historical DCI payloads and the predicted class from the Feature Classifier are fed into the Bit-Level Predictor to generate the bit-level sequence of the current DCI payload.

Proposal 32: For lossless DCI compression, the following AI-based DCI payload predictor can be further studied:

- Model input: Historical DCI payloads

- Model output: Predicated DCI payload

Figure 15. Lossless DCI compression framework.

The lossless DCI compression framework is illustrated in Figure 15. and operates as follows:

- At BS:

  - A DCI payload predictor is used to estimate the current DCI payload from historical sequences.

  - If the predicted payload matches the actual one (with accuracy between 95.62% and 99.81%, as validated by datasets), the BS transmits a short DCI format (e.g., containing an M=2-bit payload to indicate the legacy DCI format, plus a 6-bit CRC).

  - If the prediction does not match, the BS transmits the legacy DCI (e.g., with an L=55-bit payload and 24-bit CRC) as a fallback.

  - Compression ratio: M/L (where M ≪ L).

- At UE:

  - The UE uses an identical DCI payload predictor (trained at the BS and transferred to the UE) to generate the same predicted payload from local historical sequences.

  - Upon receiving a short DCI, the UE recovers the full legacy DCI payload from its local prediction.

Proposal 33: It is proposed to study AI-based lossless DCI compression in 6GR study to reduce PDCCH overhead.

Training type

Offline training for AI-based DCI payload predicator is conducted at the BS side, and the trained predictor is subsequently delivered to the UE via model delivery.

Proposal 34: For lossless DCI compression, the following training types for AI-based DCI payload predictor can be considered:

- Offline training at BS side, with the trained predictor transferred to the UE via model delivery.

Label construction

Labels for the AI-based DCI payload predictor are constructed directly from recorded DCI payload sequences in a prediction task setting. Datasets can be sourced from:

- Field data collection: DCI datasets are acquired from logged DCI payload records.

- System-level simulation: DCI payloads are generated by logging time-varying scheduling parameters—such as FDRA, TDRA, MCS, NDI, RV, HARQ Process Number, PDSCH-to-HARQ feedback timing, Antenna Ports, etc.—and interpreting them into corresponding DCI fields.

For instance, we constructed the following DCI payload datasets from both field and simulation sources:

- Field data: Field tests involved a single UE streaming CCTV video with adaptive bitrate for approximately one hour.

  - (Dataset 1) DL grant DCI: Captured ~20w records of DCI format 1_1 with 55-bit payloads.

  - (Dataset 2) UL grant DCI: Captured ~84w records of DCI format 0_1 with 51-bit payloads.

- Simulation data (Dataset 3): System-level simulations (SLS) were conducted for DL transmission in a typical Urban Macro (UMA) scenario with MU-MIMO, comprising 80% indoor and 20% outdoor UEs (refer to  for details). The dataset contains over 100w records across 21 cells, each serving ~5 UEs. A total of 2290 UEs were randomly dropped over 20 rounds, with each UE receiving ~500 DL scheduling assignments on average. The 35-bit DL grant DCI was interpreted from the logged time-varying scheduling parameters listed above.

Note: Samples of Dataset 1, Dataset 2 and Dataset 3 are presented in Table 18, Table 21 and Table 24 in Annex A, respectively, to provide an intuitive visualization.

As a prediction task, labels can be directly derived from the recorded DCI payload sequences.

Proposal 35: For lossless DCI compression, labels for AI-based DCI payload predictor training can be directly constructed from recorded DCI payload sequences in a prediction task setting. DCI payload datasets can be constructed from both field tests and system-level simulation.

- Field data collection: DCI datasets are acquired from logged DCI payload records.

- Simulation-based generation: DCI payloads are generated via system-level simulations by logging and interpreting time-varying scheduling parameters into DCI fields.

Model location for inference for AI-based DCI Payload Predicator

Model inference is executed at both BS and UE.

- Note: The predictor is trained at BS and deployed to the UE via model delivery. This ensures both BS and UE share identical predictors, guaranteeing aligned inference outputs.

Proposal 36: For lossless DCI compression, the following inference locations for AI-based DCI payload predictor can be considered:

- Model inference at both BS side and UE side, with BS-offline-trained predictors are transferred to UEs to ensure inference alignment.

Collaboration/interaction between UE and NW for AI-based DCI Payload Predicator

Model training is performed at BS, while inference occurs at both BS and UE. Model delivery from BS to UE is required to maintain inference alignment.

Proposal 37: For lossless DCI compression, the following collaboration/interaction for AI-based DCI payload predictor can be considered:

- Model delivery from BS to UE to ensure inference alignment.

High level potential specification impact

The following specification support is recommended:

- Introduction of a UE capability report to indicate support for Lossless DCI Compression.

- Standardization of model delivery procedures to ensure predictor consistency between BS and UE.

- Definition of a new short DCI format to indicate the use of AI-predicted legacy DCI content.

- A mechanism to synchronize the historical DCI payload records between BS and UE.

Proposal 38: For lossless DCI compression, the following specification support is recommended:

- Introduction of a UE capability report to indicate support for Lossless DCI Compression.

- Standardization of model delivery procedures to ensure predictor consistency between BS and UE.

- Definition of a new short DCI format to indicate the use of AI-predicted legacy DCI content.

- A mechanism to synchronize the historical DCI payload records between BS and UE.

Summary of Lossless DCI Compression

Table 14. Summary of Lossless DCI Compression.

| Use case | Lossless DCI Compression: DCI payload predicator as AI task, and Lossless DCI Compression as downstream application |
| --- | --- |
| Model input | DCI payload predicator: Historical DCI payloads |
| Model output | DCI payload predicator: Predicated DCI payload |
| Label | Recorded DCI payload sequences |
| Training types | DCI payload predicator: Offline training can be conducted at the BS side, with the trained predictor subsequently delivered to the UE via model delivery. |
| KPI | DCI payload predicator: BER and sample-level predication accuracy Lossless DCI Compression: Compression ratio after CRC attachment, and Aggregate overhead reduction |
| Benchmark | Traditional DCI design |
| Model location for inference | DCI payload predicator: Model inference is executed at both BS side and UE side. - Note: The predictor is offline-trained at BS and deployed to the UE via model delivery. This ensures both BS and UE share identical predictors, guaranteeing aligned inference outputs. |
| Collaboration/interaction between UE and NW | DCI payload predicator: Model delivery from BS to UE is required to maintain inference alignment. |
| Potential specification impact | - Introduction of a UE capability report to indicate support for Lossless DCI Compression. - Standardization of model delivery procedures to ensure predictor consistency between BS and UE. - Definition of a new short DCI format to indicate the use of AI-predicted legacy DCI content. - A mechanism to synchronize the historical DCI payload records between BS and UE. |
| Model backbone | DCI payload predicator: Feature Classifier (Transformer based) + Bit-Level Predictor (Bi-LSTM + Transformer based) |

Proposal 39: For AI/ML based Lossless DCI Compression, RAN1 to take Table 14 in R1-2507020 for further study.

Performance evaluation

Evaluation assumption

As discussed above, DCI payload datasets can be constructed from both field and simulation sources. The simulation parameters, construction of data set and training parameters for dataset construction from SLS are summarized as in Table 15.

Table 15. Simulation parameters to generate DCI payload from SLS.

| Parameter | Value |
| --- | --- |
| Scenario | Urban Macro |
| layout | Hexagonal grid, 3 sectors per site, 7 macro sites |
| ISD | 500m |
| Channel model | According to TR 38.901 |
| SCS | 30kHz |
| Bandwidth | 20MHz |
| Carrier Frequency | 4GHz |
| Tx power | 53dBm |
| Duplex mode | Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] |
| Antenna setup and port layouts at gNB | 64 ports: (M, N, P, Mg, Ng; Mp, Np) =(12, 8, 2, 1, 1; 4, 8), (dH, dV) = (0.5, 0.8)λ, +45°, -45° polarization. |
| Antenna setup and port layouts at UE | 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |
| MIMO mode | MU-MIMO |
| DCI format | DL grant DCI |
| Traffic mode | FTP model 3, 0.5Mbyte for DL and 0.125 Mbytes for UL, <10%, 20%-40% and ≥50% for low, medium and high load, |
| Scheduling algorithm | Proportional Fair (PF) |
| UE distribution | 80% indoor in houses with 3km/h; 20% outdoor in car with 30km/h; 5 users per Macro TRP |
| Data set | 100w samples with training set: validation set: test set = 7: 1.5: 1.5 |
| Loss function | Phase 1 (Feature classifier training): MSE Phase 2 (Bit-level predictor training): 0.8 * BCE (for bit-lever loss) + 0.2 * MSE (for sample-level loss) |
| KPI | BER and sample-level predication accuracy |

Preliminary simulation results

Table 16. Performance of each dataset for DCI payload predictor.

| Dataset | Description | Normalized information entropy H | BER | Sample-level predication accuracy |
| --- | --- | --- | --- | --- |
| (Dataset 1) DL grant DCI from field test | ~20w records of DCI format 1_1 with 55-bit payloads, captured from field tests involving a single UE streaming adaptive bitrate video for approximately one hour. | 22.5% | 0.15% | 95.62% |
| (Dataset 2) UL grant DCI from field test | ~84w records of DCI format 0_1 with 51-bit payloads, captured from field tests involving a single UE streaming adaptive bitrate video for approximately one hour. | 25.0% | 0.03% | 99.22% |
| (Dataset 3) DL grant DCI from SLS | Over 100w records of DL grant DCI with 35-bit payloads, across 21 cells and 2290 UEs | - Range: 2.9% ~ 18.3% - Mode: 16.6% of UEs with H ≈ 12.3% - Mean H across all UEs ≈ 11.2% | 0.01% | 99.81% |

As shown in Table 16, both field tests and system-level simulation datasets indicate low mutual information (H) between consecutive transmissions, and the DCI payload are highly predictable from historical payload sequences.

- For (Dataset 1) DL grant DCI from field test, the normalized information entropy is 22.5%, with a BER of 0.15% and a sample-level predication accuracy of 95.62%

- For (Dataset 2) UL grant DCI from field test, the normalized information entropy is 25.0%, with a BER of 0.03% and a sample-level prediction accuracy of 99.22%.

- For (Dataset 3) DL grant DCI from SLS, the average normalized information entropy across all UEs is 11.2%, with a BER of 0.01% and a sample-level prediction accuracy of 99.81%.

Observation 14: For lossless DCI compression, DCI payload prediction demonstrates high feasibility across all datasets, as evidenced by low normalized information entropy and high sample-level accuracy. Performance is summarized as follows:

- (Dataset 1) DL grant DCI from field test: Normalized information entropy = 22.5%, BER = 0.15%, sample-level predication accuracy = 95.62%

- (Dataset 2) UL grant DCI from field test: Normalized information entropy = 25.0%, BER = 0.03%, sample-level predication accuracy = 99.22%

- (Dataset 3) DL grant DCI from SLS: Normalized information entropy = 11.2% (mean), BER = 0.01%, sample-level predication accuracy = 99.81%

For compression performance:

In scenarios with probability p, the original DCI payload (L bits) appended with a 24-bit CRC is compressed into a short DCI payload (M bits) with a C -bit CRC. The compression ratio after CRC attachment is:

Compression Ratio=M+CL+24

This corresponds to an air-interface overhead reduction of 1-M+CL+24 relative to uncompressed transmission.

For the remaining cases (probability 1-p), legacy DCI transmission (L+24) bits is dynamically scheduled as a fallback. Consequently, the average air-interface overhead is:

Average Overhead=pM+C+1-pL+24

The overall compression ratio after CRC attachment becomes:

Overall Compression Ratio=pM+C+1-pL+24L+24=p∙M+CL+24+1-p

This yields an aggregate overhead reduction of p∙1-M+CL+24.

Assume a short DCI payload of M=2 bits (e.g., used to indicate the legacy DCI format) appended with a C=6 bit CRC. The compression metrics for both datasets are summarized in Table 17.

The significant DCI compression efficiency gain reveals the technical potential of AI/ML based lossless DCI compression techniques.

Table 17. Compression Efficiency Analysis.

|  | Length of legacy DCI (L) | Length of short DCI (M) | CRC of short DCI (C) | Sample-level predication accuracy (p) | Compression ratio after CRC attachment (M+CL+24) | Aggregate overhead reduction (p∙1-M+CL+24) |
| --- | --- | --- | --- | --- | --- | --- |
| (Dataset 1) DL grant DCI from field test | 55 bits | 2 bits | 6 bits | 95.62% | 10.13% | 85.9% |
| (Dataset 2) UL grant DCI from field test | 51 bits | 2 bits | 6 bits | 99.22% | 10.67% | 88.6% |
| (Dataset 3) DL grant DCI from SLS | 35 bit | 2 bits | 6 bits | 99.81% | 13.56% | 86.3% |

Observation 15: AI/ML based lossless DCI compression shows potential for significant DCI compression efficiency.

- (Dataset 1) DL grant DCI from field test: 95.62% of cases achieve 10.13% compression ratio after CRC attachment by compressing the original 53-bit payload with 24-bit CRC to a 2-bit short DCI with 6-bit CRC. The aggregate overhead reduction is 85.9%.

- (Dataset 2) UL grant DCI from field test: 99.22% of cases achieve 10.67% compression ratio after CRC attachment by compressing the original 51-bit payload with 24-bit CRC to a 2-bit short DCI with 6-bit CRC. The aggregate overhead reduction is 88.6%.

- (Dataset 3) DL grant DCI from SLS: 99.81% of cases achieve 13.56% compression ratio after CRC attachment by compressing the original 35-bit payload with 24-bit CRC to a 2-bit short DCI with 6-bit CRC. The aggregate overhead reduction is 86.3%.

Other 5G-A straightforward extension use cases

For the use cases already studied in 5G-A, including beam management, CSI prediction, CSI compression, etc, at least the evaluation method, assumptions and results in TR 380843 can be considered in Rel-20 6G study. As for the specification impacts of these use cases, it can be further considered in 6G WI scoping.

Proposal 40: 5G-A use cases and the corresponding study outcome (e.g. observations, conclusions, etc. in TR 38.843) can be considered for 6GR system design, at least including beam management, CSI prediction, and CSI compression.

Conclusions

In this contribution, we discussed AI/ML use cases for 6GR interface, and the following observations and proposals are made:

General selection principle of AI/ML use cases:

Proposal 1: It is proposed to identify/categorize (sub-)use cases based on the related RAN1 agenda of Rel-20 Study of 6GR.

Proposal 2: The following categorization structure of 6GR AI/ML (sub-)use cases can be considered:

- Waveform

- Frame structure

- Channel coding

- Modulation

- Initial access

- MIMO

- Energy efficiency

- Physical layer control, data scheduling and HARQ operation

- ISAC

- 6GR spectrum utilization and aggregation

- Service related

- Led by other WG

For Case#1: AI/ML based CSI prediction:

Proposal 3: It is proposed to study AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction in 6GR study.

Proposal 41: For AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction use cases, AI model takes the measured channel matrix of partial ports or partial RBs as the input and outputs the channel matrix of all ports and all RBs.

Proposal 5: For AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction use cases, offline training and at least UE-sided model should be considered.

Proposal 6: For AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction use cases, RAN1 to take Table 1 in R1-2507020 for further study.

Proposal 7: Take the following metrics for performance evaluation for AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction in 6GR study.

- Intermediate KPI: SGCS

- Eventual KPI: Throughput

Proposal 8: Benchmark of AI/ML-based spatial domain CSI prediction and AI/ML-based frequency domain CSI prediction need to be studied, the following two options can be considered:

- Option 1: Sample and hold. The channel matrixes for partial measured ports are used as the CSI for adjacent ports. The channel matrixes for measured RBs are used as the CSI for adjacent RBs.

- Option 2: Traditional channel interpolation algorithm

Observation 1: Compared to benchmark methods like sample-and-hold or traditional interpolation, the AI model improves spatial domain CSI prediction performance (in terms of SGCS) by up to 18%, 45%, and 152% when the ratio of measured ports to total ports is 1/2, 1/4, and 1/8, respectively.

Observation 2: A good prediction performance (>0.94 in terms of SGCS) for AI/ML based spatial domain CSI prediction can be achieved with CSI-RS overhead reduced to 1/2, 1/4, and 1/8, respectively.

Observation 3: Compared to benchmark methods like sample-and-hold or traditional interpolation, the AI model improves frequency domain CSI prediction performance (in terms of SGCS) by up to 5%, 7%, and 9%, when the frequency density for measurement is 1/2, 1/4, 1/8, respectively.

Observation 4: A good prediction performance (>0.98 in terms of SGCS) for AI/ML based frequency domain CSI prediction can be achieved with CSI-RS overhead reduced to 1/2, 1/4, 1/8, respectively.

Observation 5: A good prediction performance (>0.94 in terms of SGCS) for AI/ML based joint spatial and frequency domain CSI prediction can be achieved with CSI-RS overhead reduced to 1/32.

For Case#2: AI/ML based DMRS enhancement:

Proposal 9: It is proposed to study AI/ML-based DMRS enhancement in 6GR study.

Proposal 10: Regarding low-overhead DMRS at the transmitter for AI/ML-based DMRS enhancement, consider

- Approach 1: Sparse orthogonal DMRS

  - Option 1 (Intra-RB sparse): Reduced DMRS density within a PRB.

  - Option 2 (Inter-RB sparse): DMRS mapping confined to specific PRB subsets.

  - Option 3 (Inter-slot sparse): DMRS allocation to partial slots during PDSCH/PUSCH repetition.

  - Combination of the above options.

- Approach 2: Non-Orthogonal DMRS and Superimposed with data (SIP)

Proposal 11: For AI-assisted receiver for sparse orthogonal DMRS, three alternatives can be considered:

- Alt 1: AI-based channel estimation.

  - Model input: Received signal at DMRS REs

  - Model output: Channel matrix of the whole allocated REs

- Alt 2: AI receiver combining channel estimation and equalization.

  - Model input: DMRS pattern and received signal at the whole allocated Res

  - Model input: Interim value for demodulation

- Alt 3: AI receiver combining channel estimation, equalization, and demodulation processing.

  - Model input: DMRS pattern and received signal at the whole allocated Res

  - Model input: LLR for channel decoding

Proposal 12: For AI-assisted receiver for sparse orthogonal DMRS, adopt offline training at UE side as baseline.

- FFS: needs for online fine-tuning

Proposal 13: For AI-assisted receiver for sparse orthogonal DMRS, the following label construction methods can be considered:

- Simulation-based: Utilize System Level Simulation (SLS) or Link Level Simulation (LLS) to generate labels.

- Field data collection-based: Acquire labels by receiving known reference signals or data files.

Proposal 14: For AI-assisted receiver for sparse orthogonal DMRS, consider UE-sided model for inference.

Proposal 15: For AI-assisted receiver for sparse orthogonal DMRS, explicit collaboration/interaction between the UE and NW is not required.

Proposal 16: For AI-assisted receiver for sparse orthogonal DMRS, introduce UE capability reporting for AI-assisted receivers supporting sparse orthogonal DMRS.

Proposal 17: For AI-assisted receiver for sparse orthogonal DMRS, either

- Reuse existing NR DMRS pattern, or

- Design new DMRS pattern, with the following options proposed for further study:

  - Option 1 (Intra-RB sparse): Reduced DMRS density within a PRB.

  - Option 2 (Inter-RB sparse): DMRS mapping confined to specific PRB subsets.

  - Option 3 (Inter-slot sparse): DMRS allocation to partial slots during PDSCH/PUSCH repetition.

Proposal 18: For AI/ML based sparse orthogonal DMRS, RAN1 to take Table 6 in R1-2507020 for further study.

Proposal 19: For Non-Orthogonal DMRS and Superimposed with data (SIP), AI receiver combining channel estimation, equalization, and demodulation processing can be considered.

- Model input: SIP pattern and received signal at the whole allocated Res

- Model input: LLR for channel decoding

Proposal 20: For AI-assisted receiver for SIP, adopt offline training at UE side as baseline.

- FFS: needs for online fine-tuning

Proposal 21: For AI-assisted receiver for SIP, the following label construction methods can be considered:

- Channel Matrix Labels:

  - Simulation-based: Generated via SLS/LLS with predefined channel models and transmission parameters, or,

  - Field data collection-based: Acquired from known reference signals during actual transmissions.

- Data Labels (including data, superimposed signals (i.e., data + SIP pilot) and received signals):

  - Simulation-based: Generated via LLS using the channel matrix for superimposed signals and received signals.

Proposal 22: For AI-assisted receiver for SIP, consider UE-sided model for inference.

Proposal 23: For AI-assisted receiver for SIP explicit collaboration/interaction between the UE and NW is not required.

Proposal 24: For AI-assisted receiver for SIP, introduce UE capability reporting for supporting SIP receivers.

Proposal 25: Standardize the SIP transmission mechanism with definitions for:

- Non-orthogonal resource mapping patterns

- Receiver interference cancellation requirements

- Power allocation between data and pilot symbols

Proposal 26: For AI/ML based Non-Orthogonal DMRS and Superimposed with data (SIP), RAN1 to take Table 8 in R1-2507020 for further study.

Observation 6: Under high-speed conditions (120 km/h) with a CDL-C channel model and 300 ns delay spread, AI-assisted receiver—specifically employing an AI-based channel estimator with sparse orthogonal DMRS—outperforms legacy NR DMRS configurations that use traditional channel estimators. Among the evaluated cases, Case 2 (subcarrier-level sparsity, 1/2 density) demonstrates the best overall performance, characterized by:

- Approximately 6 dB SNR gain at the 10% BLER target (12 dB vs. 18 dB).

- Throughput gains ranging from 83% at 8 dB SNR to 42% at 18 dB SNR, with more substantial improvements observed at lower SNR values.

- This case utilizes a new DMRS pattern featuring one additional double-symbol DMRS, with 2 REs per DMRS symbol per port (resulting in 24 REs per RB).

Observation 7: Under high-speed conditions (120 km/h) with a CDL-C channel model and 300 ns delay spread, AI-assisted receiver—combining channel estimation, equalization, and demodulation—for SIP outperforms legacy NR DMRS configurations that use traditional channel estimators.

- TBS alignment provides consistent performance gains across the SNR range.

- MCS alignment yields better throughput performance at higher SNR values.

- At 18 dB SNR, the maximum BLER gain (from 10% down to 0.6%) is achieved under TBS alignment, while the maximum throughput gain (56%) occurs under MCS alignment.

For Case#3: Joint source, channel coding, and modulation (JSCCM):

Proposal 27: To further enhance CSI feedback efficiency and accuracy, solutions that combine CSI source coding with channel coding (even modulation) techniques could be considered.

Proposal 28: For AI/ML based Joint source, channel coding, and modulation (JSCCM), RAN1 to take Table 10 in R1-2507020 for further study.

Observation 8：For both constant amplitude and average power constraints, the JSCCM achieves an SGCS score above 0.9 when the SNR exceeds -10dB, and models trained at lower SNRs have better SGCS performance.

Observation 9：At a given training SNR, the JSCCM with average power constraint maintains a performance advantage over the JSCCM with constant amplitude constraint throughout the inference SNR range

For Case#4: AI/ML based Prior-Information-Aided DCI Decoding:

Proposal 29: It is proposed to study prior-information-aided DCI decoder at the receiver in 6GR study, where an AI/ML-based DCI prediction network could be used to provide the decoder with accurate prior information and improve the decoding performance significantly.

Proposal 30: Regarding prior-information-aided DCI decoder, the following aspects could be considered for further studies

- DCI dataset collection for AI/ML-based DCI prediction network training.

  - Alt 1: The transmitter collects the DCI dataset and distributes it to receivers for model training.

  - Alt 2: Each receiver independently gathers its own dataset for training.

- AI decoder/non-AI decoder fallback mechanism:

- Define criteria for switching between the prior-information-aided DCI decoder and the conventional non-AI decoder.

- Signaling/configuration design for prior-information-aided DCI decoder.

- Which DCI fields for prediction as model input

Proposal 31: For AI/ML based Prior-Information-Aided DCI Decoding, RAN1 to take Table 12 in R1-2507020 for further study.

Observation 10: The prior-information-aided DCI decoder (for enhanced Convolutional decoder) achieves >5 dB improvement at BLER@1% and ~1.5 dB at BLER@0.1% compared to the baseline, where the dataset is from Redcap scenario.

Observation 11: The prior-information-aided DCI decoder (for enhanced Polar decoder) achieves about 5 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder, where the dataset is from Redcap scenario.

Observation 12: The prior-information-aided DCI decoder (for enhanced Polar decoder) achieves about 4 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder, where the dataset is from eMBB scenario.

Observation 13: The proposed prior-information-aided DCI decoder demonstrates wide applicability. It is effective in:

- Multiple scenarios: Both RedCap and eMBB.

- Multiple coding schemes: Both convolutional codes and Polar codes.

- Complex payloads: DCI payloads containing multiple (>2) fields.

For Case#5: AI/ML based Lossless DCI Compression:

Proposal 32: For lossless DCI compression, the following AI-based DCI payload predictor can be further studied:

- Model input: Historical DCI payloads

- Model output: Predicated DCI payload

Proposal 33: It is proposed to study AI-based lossless DCI compression in 6GR study to reduce PDCCH overhead.

Proposal 34: For lossless DCI compression, the following training types for AI-based DCI payload predictor can be considered:

- Offline training at BS side, with the trained predictor transferred to the UE via model delivery.

Proposal 35: For lossless DCI compression, labels for AI-based DCI payload predictor training can be directly constructed from recorded DCI payload sequences in a prediction task setting. DCI payload datasets can be constructed from both field tests and system-level simulation.

- Field data collection: DCI datasets are acquired from logged DCI payload records.

- Simulation-based generation: DCI payloads are generated via system-level simulations by logging and interpreting time-varying scheduling parameters into DCI fields.

Proposal 36: For lossless DCI compression, the following inference locations for AI-based DCI payload predictor can be considered:

- Model inference at both BS side and UE side, with BS-offline-trained predictors are transferred to UEs to ensure inference alignment.

Proposal 37: For lossless DCI compression, the following collaboration/interaction for AI-based DCI payload predictor can be considered:

- Model delivery from BS to UE to ensure inference alignment.

Proposal 38: For lossless DCI compression, the following specification support is recommended:

- Introduction of a UE capability report to indicate support for Lossless DCI Compression.

- Standardization of model delivery procedures to ensure predictor consistency between BS and UE.

- Definition of a new short DCI format to indicate the use of AI-predicted legacy DCI content.

- A mechanism to synchronize the historical DCI payload records between BS and UE.

Proposal 39: For AI/ML based Lossless DCI Compression, RAN1 to take Table 14 in R1-2507020 for further study.

Observation 14: For lossless DCI compression, DCI payload prediction demonstrates high feasibility across all datasets, as evidenced by low normalized information entropy and high sample-level accuracy. Performance is summarized as follows:

- (Dataset 1) DL grant DCI from field test: Normalized information entropy = 22.5%, BER = 0.15%, sample-level predication accuracy = 95.62%

- (Dataset 2) UL grant DCI from field test: Normalized information entropy = 25.0%, BER = 0.03%, sample-level predication accuracy = 99.22%

- (Dataset 3) DL grant DCI from SLS: Normalized information entropy = 11.2% (mean), BER = 0.01%, sample-level predication accuracy = 99.81%

Observation 15: AI/ML based lossless DCI compression shows potential for significant DCI compression efficiency.

- (Dataset 1) DL grant DCI from field test: 95.62% of cases achieve 10.13% compression ratio after CRC attachment by compressing the original 53-bit payload with 24-bit CRC to a 2-bit short DCI with 6-bit CRC. The aggregate overhead reduction is 85.9%.

- (Dataset 2) UL grant DCI from field test: 99.22% of cases achieve 10.67% compression ratio after CRC attachment by compressing the original 51-bit payload with 24-bit CRC to a 2-bit short DCI with 6-bit CRC. The aggregate overhead reduction is 88.6%.

- (Dataset 3) DL grant DCI from SLS: 99.81% of cases achieve 13.56% compression ratio after CRC attachment by compressing the original 35-bit payload with 24-bit CRC to a 2-bit short DCI with 6-bit CRC. The aggregate overhead reduction is 86.3%.

For 5G-A straightforward extension use cases:

Proposal 40: 5G-A use cases and the corresponding study outcome (e.g. observations, conclusions, etc. in TR 38.843) can be considered for 6GR system design, at least including beam management, CSI prediction, and CSI compression.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #108.

- RP-233133, TR 38.843 v2.0.0, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface, Qualcomm

- Chairman’s Notes RAN1#122, Bengaluru, India, Aug 25th – 29th, 2025.

- R1-2506456, Moderator summary #2 on AI/ML for 6GR, Samsung (Moderator)

Annex A: AI/ML based DCI enhancement

Dataset 1 (DL grant DCI from field test)

Table 18  A slice of DCI payloads for Dataset 1.

| Index | DCI payload |
| --- | --- |
| 1 | 1100010000000000000001100000000000111100010000010000000 |
| 2 | 1110000000000000000000000000000100111000010000110000000 |
| 3 | 1110000000000000000000000000000100111100100000000000000 |
| 4 | 1110000000000000000000000000000000111100010000000000000 |
| 5 | 1110000000000000000000000000000100111100000000000000000 |
| 6 | 1110000000000000000000000000000000111100010000000000000 |
| 7 | 1100010000000000000000100000000100111100000000000000000 |
| 8 | 1101111000000000000000100000000000111100100000110000000 |
| 9 | 1110000000000000000000100000000100111100100000000000000 |
| 10 | 1100000000000010000000000100001000111100100000110010000 |
| 11 | 1100010000000000000000000000000100111100100000100000000 |
| 12 | 1100000000000001000000001100000000111100100000000010000 |
| 13 | 1100000000000010000001100000100100111100100000010010000 |
| 14 | 1100000000000010000000001100000000111100100000000000000 |
| 15 | 1100000000000010000000001100111100111100100000000000000 |
| 16 | 1100000000000010000000000100000000111100010000110000000 |
| 17 | 1100000000000010000000000000100100111100100000100000000 |
| 18 | 1100000000000010000001100001000000111001010000010000000 |
| 19 | 1100000000000011100001100001100000111100100000010000000 |
| 20 | 1100000000000010000001100001000100111000010000010000000 |
| … | … |

Table 19  A slice of predication results for Dataset 1.

| True DCI payload (label) | Predicate value | Error pattern | Rank |
| --- | --- | --- | --- |
| 1100000000000010000001100001011000111100100000100000000 | 1100000000000010000001100001011000111100100000100000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000010000000000100010100111000010000110000000 | 1100000000000010000000000100010100111000010000110000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000011000000000001111100110100010000110000000 | 1100000000000011000000000001111100110100010000110000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000010000001100001011100111100100000010000000 | 1100000000000010000001100001011100111100100000010000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000011000000000010101000111000010000110000000 | 1100000000000011000000000010101000111000010000110000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100011110000011111100001111001000110101010000000000000 | 1100011110000011111100001111001000110101010000000000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100011100000011111100000111001000110000010000110000000 | 1100011100000011111100000111001000110000010000110000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000011100000000011001000101101010000100000000 | 1100000000000011100000000011001000101101010000100000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000010000000000000010000111100010000100000000 | 1100000000000010000000000000010000111100010000100000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000001000000000001000100111100100000000000000 | 1100000000000001000000000001000100111100100000000000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000011111000000010110000111100100000000000000 | 1100000000000011111000000010110000111100100000000000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000011000000001111001100111000010000000000000 | 1100000000000011000000001111001100111000010000000000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100011111111101111101100011100100110100010000010000000 | 1100011111111111111101100011100100110100010000010000000 | 0000000000000010000000000000000000000000000000000000000 | 31 |
| 1100000000000011111000000011010100111100010000110000000 | 1100000000000011111000000011010100111100010000110000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000011000000001111001000111001010000000000000 | 1100000000000011000000001111001000111001010000000000000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000010000001100000010000111100010000010010000 | 1100000000000010000001100000010000111100010000010010000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000001111001100011000100111100010000010010000 | 1100000000000001111001100011000100111100010000010010000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1100000000000011110000001111000000111100010000000010000 | 1100000000000011110000001111000000111100010000000010000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| 1111100000000011111101100011100100111000010000010010000 | 1111100000000011111101100011100100111000010000010010000 | 0000000000000000000000000000000000000000000000000000000 | 1 |
| … | … | … | … |

Table 20  Top 10 error pattern statistics for Dataset 1.

| Error pattern | Frequency | Rank |
| --- | --- | --- |
| 0000000000000000000000000000000000000000000000000000000 | 0.956222848 | 1 |
| 0000000000000000000100000000000000000000000000000000000 | 0.00310307 | 2 |
| 0010000000000000000000000000000000000000000000000000000 | 0.002434716 | 3 |
| 0000000000000000000000100000000000000000000000000000000 | 0.001718623 | 4 |
| 0000000010000000000000000000000000000000000000000000000 | 0.001623144 | 5 |
| 0000000100000000000000000000000000000000000000000000000 | 0.001575405 | 6 |
| 0000001000000000000000000000000000000000000000000000000 | 0.001575405 | 7 |
| 0000000000010000000000000000000000000000000000000000000 | 0.001479926 | 8 |
| 0001000000000000000000000000000000000000000000000000000 | 0.001336707 | 9 |
| 0000000000000000000100100000000000000000000000000000000 | 0.001288967 | 10 |
| … | … | … |

Figure 16. Frequency allocation snapshoot of the first 5000 DCI payload records for Dataset 1.

Dataset 2 (UL grant DCI from field test)

Table 21  A slice of DCI payloads for Dataset 2.

| Index | DCI payload |
| --- | --- |
| 1 | 010000101111001001000111101010000011010100000000001 |
| 2 | 010000101111001001000111101011000011100100000000001 |
| 3 | 010000101111001001000111101001000011010100000000001 |
| 4 | 010000101111001001000111101010000011010100000000001 |
| 5 | 010000011101011101000000000100000011010100000000001 |
| 6 | 010000011001100110000011101110000011010100000000001 |
| 7 | 010000100001110010000100000000000100010100000000001 |
| 8 | 010000011001100110000011101111000011010100000000001 |
| 9 | 010000011001100110000011101101000011100100000000001 |
| 10 | 010000011101010101000000000100000111010100000000001 |
| 11 | 010000011101011001000100000100001000010100000000001 |
| 12 | 010000011001100110000011101110000011010100000000001 |
| 13 | 010000011101010101000000000000000100010100000000001 |
| 14 | 010000100001100110000100000000001011010100000000001 |
| 15 | 010000100101110111000000000000000011010100000000001 |
| 16 | 010000010000101010000100000100000111010100000000001 |
| 17 | 010000010100110011000000001100000011010100000000001 |
| 18 | 010000010000100010000001101000000011010100000000001 |
| 19 | 010000010000100010000001100100000011010100000000001 |
| 20 | 010000101111001001000111101010000011010100000000001 |
| … | … |

Table 22  A slice of predication results for Dataset 2.

| True DCI payload (label) | Predicate value | Error pattern | Rank |
| --- | --- | --- | --- |
| 010000010010011011000000111000000011010100000000001 | 010000010010011011000000111000000011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000010010011011000000111000000111010100000000001 | 010000010010011011000000111000000111010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000010110011000000000101100000011100100000000001 | 010000010110011000000000101100000011100100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001001000001011010100000000001 | 010000001110001010000001001000001011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000101001000000011010100000000001 | 010000001110001010000101001000000011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001001100000111010100000000001 | 010000001110001010000001001100000111010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001001100001011010100000000001 | 010000001110001010000001001100001011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001010100000011010100000000001 | 010000001110001010000001010100000011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001010000000111010100000000001 | 010000001110001010000001010000000111010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000101010000001011010100000000001 | 010000001110001010000101010000001011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001010000000011010100000000001 | 010000001110001010000001010000000011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001010100000111010100000000001 | 010000001110001010000001010100000111010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000100111100001011010100000000001 | 010000001110001010000100111100001011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001010100000011010100000000001 | 010000001110001010000001010100000011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001010000000111010100000000001 | 010000001110001010000001010000000111010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001010000000011010100000000001 | 010000001110001010000001010000000011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001010100000111010100000000001 | 010000001110001010000001010100000111010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000101010000001011010100000000001 | 010000001110001010000101010000001011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| 010000001110001010000001010100000011010100000000001 | 010000001110001010000001010100000011010100000000001 | 000000000000000000000000000000000000000000000000000 | 1 |
| … | … | … | … |

Table 23  Top 10 error pattern statistics for Dataset 2.

| Error pattern | Frequency | Rank |
| --- | --- | --- |
| 000000000000000000000000000000000000000000000000000 | 0.992189331 | 1 |
| 000000000000000000000010000000000000000000000000000 | 0.0011716 | 2 |
| 000100000000000000000000000000000000000000000000000 | 0.000833138 | 3 |
| 000000000000000000000001000000000000000000000000000 | 0.000755031 | 4 |
| 000000000010000000000000000000000000000000000000000 | 0.000755031 | 5 |
| 000000000000000000000000000000000001000000000000000 | 0.00070296 | 6 |
| 000001000000000000000000000000000000000000000000000 | 0.000494676 | 7 |
| 000010000000000000000000000000000000000000000000000 | 0.000416569 | 8 |
| 001100000000000000000000000000000000000000000000000 | 0.000260356 | 9 |
| 000000000000000000000000000000000000100000000000000 | 0.000260356 | 10 |
| … | … | … |

Figure 17. Frequency allocation snapshoot of the first 5000 DCI payload records for Dataset 1.

Dataset 3 (DL grant DCI from SLS)

Table 24  A slice of DCI payloads for Dataset 3.

| Index | DCI payload |
| --- | --- |
| 1 | 10100000100000001101100000100101110 |
| 2 | 10000000100000001101110000110011110 |
| 3 | 10000000100000101101100001000001110 |
| 4 | 10100000100000001101110000100101110 |
| 5 | 10000000100000001101100000110011110 |
| 6 | 10000000100000101101110001000001110 |
| 7 | 10000000000000001101100000100101111 |
| 8 | 10000000000000001101110000110011111 |
| 9 | 10000000000000101101100001000001111 |
| 10 | 10000000000000001101110000100101111 |
| 11 | 10000000000000001101100000110011111 |
| 12 | 10000000000000101101110001000001111 |
| 13 | 10000000000000001100000000100110010 |
| 14 | 10000000000000001101110000110101111 |
| 15 | 10000000000000001101100001000011111 |
| 16 | 10000000000000101101110001010001111 |
| 17 | 10000000000000001101100000100101111 |
| 18 | 10000000000000001101110000110011111 |
| 19 | 10000000000000101101100001000001111 |
| 20 | 10000000000000001101110000100101111 |
| … | … |

Table 25  A slice of predication results for Dataset 3.

| True DCI payload (label) | Predicate value | Error pattern | Rank |
| --- | --- | --- | --- |
| 10000000000000001101110001000011111 | 10000000000000001101110001000011111 | 00000000000000000000000000000000000 | 1 |
| 10000000000000101101100001010001111 | 10000000000000101101100001010001111 | 00000000000000000000000000000000000 | 1 |
| 10101000000000001100110000100111110 | 10101000000000001100110000100111110 | 00000000000000000000000000000000000 | 1 |
| 10000000000000001101100000110101111 | 10000000000000001101100000110101111 | 00000000000000000000000000000000000 | 1 |
| 10000000000000001101110001000011111 | 10000000000000001101110001000011111 | 00000000000000000000000000000000000 | 1 |
| 10000000000000101101100001010001111 | 10000000000000101101100001010001111 | 00000000000000000000000000000000000 | 1 |
| 10100000000000001100110000100111110 | 10100000000000001100110000100111110 | 00000000000000000000000000000000000 | 1 |
| 10000000000000001101100000110101111 | 10000000000000001101100000110101111 | 00000000000000000000000000000000000 | 1 |
| 10000000000000001101110001000011111 | 10000000000000001101110001000011111 | 00000000000000000000000000000000000 | 1 |
| 10000000000000101101100001010001111 | 10000000000000101101100001010001111 | 00000000000000000000000000000000000 | 1 |
| 10000000000000101101100001000001111 | 10000000000000101101100001000001111 | 00000000000000000000000000000000000 | 1 |
| 10000000000000001101110000100101111 | 10000000000000001101110000100101111 | 00000000000000000000000000000000000 | 1 |
| 10000000000000001101100000110011111 | 10000000000000001101100000110011111 | 00000000000000000000000000000000000 | 1 |
| 10000000000000101101110001000001111 | 10000000000000101101110001000001111 | 00000000000000000000000000000000000 | 1 |
| 11111101111000001101100000100101110 | 11111101111000001101100000100101110 | 00000000000000000000000000000000000 | 1 |
| 11111101111000001101110000110011110 | 11111101111000001101110000110011110 | 00000000000000000000000000000000000 | 1 |
| 11111101111000101101100001000001110 | 11111101111000101101100001000001110 | 00000000000000000000000000000000000 | 1 |
| 11111101111000001101110000100101110 | 11111101111000001101110000100101110 | 00000000000000000000000000000000000 | 1 |
| 10001000000000001101100000110011110 | 10001000000000001101100000110011110 | 00000000000000000000000000000000000 | 1 |
| … | … | … | … |

Table 26  Top 10 error pattern statistics for Dataset 3.

| Error pattern | Frequency | Rank |
| --- | --- | --- |
| 00000000000000000000000000000000000 | 0.998073327 | 1 |
| 00000000000000100000000000000000000 | 0.000665836 | 2 |
| 00000000000000000000000001000000000 | 0.000297501 | 3 |
| 00000000001000000000000000000000000 | 0.000198334 | 4 |
| 00000100000000000000000000000000000 | 0.000155834 | 5 |
| 00000000000000001000000000000000000 | 0.000113334 | 6 |
| 00000000000000000000000010000000000 | 0.000113334 | 7 |
| 00000000000000000000000000000000001 | 8.50003E-05 | 8 |
| 00000000100000000000000000000000000 | 4.25001E-05 | 9 |
| 00000000000000000000000100000000000 | 4.25001E-05 | 10 |
| … | … | … |

Annex B: Normalized information entropy

The information entropy H is defined as:

H=-i=1nPxilog2Pxi

where xi denotes the i-th unique DCI payload for a single UE, n is the total number of payload records for that UE, and Pxi is the occurrence frequency of xi.

To standardize comparisons, the normalized information entropy H is derived as:

H=Hbit size of DCI payload