3GPP TSG RAN WG1 #122bis		                                         R1-2507967

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:	9.5

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary of discussion on Rel-19 NR UE features for LP-WUS/WUR

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 9.5. regarding Rel-19 NR UE features for LP-WUS/WUR.

[122-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO), Ralf (AT&T) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

- Companies’ inputs

The following inputs have been provided in companies’ contributions in this meeting:

| - ZTE Corporation, Sanechips | It is worthwhile to discuss whether the OFDM WUR has the capability to detect the OOK pattern. According to the agreement shown as following, the number of overlaid sequences can be configured as 1, which means there is no information carried by overlaid sequence. Agreement For LP-WUS overlaid OFDM sequence configuration in a cell, - gNB configures number of sequences Nseq, and one or two root values. - Alt1: Nseq is 2^n, where the value range of n is 0~4, 0~3, 0~2 for M=1, 2, 4, respectively. - No separate UE capability for different n values. - Nroot=2, when Nseq=16,8,4 for M=1,2,4, where Nroot is the number of root values. - Nroot=1 or 2 when Nseq is other value - The number of CS Ncs is derived by Nseq/Nroot. However, according to the WID, the OFDM WUR and OOK WUR should obtain the same information The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported. Two solutions can be considered for addressing this issue Solution 1: OFDM WUR has the capability as part of 62-1a/62-2a to decode OOK ON-OFF pattern Solution 2: gNB is not expected to configure n=0 when OOK ON-OFF pattern carries information. Based on solution 1, we can add some description in FG 62-1a or 62-2a to explicitly capture that OFDM WUR can detect OOK ON-OFF pattern for LP-WUS. Based on solution 2, we need an agreement to restrict the configuration. Proposal 1: Discuss and decide whether OFDM WUR can decode OOK ON-OFF pattern for LP-WUS. |
| --- | --- |
| - vivo | 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per band] Per FS; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1, Option 1-2, both Option 1-1 and Option 1-2} Component 3 candidate values: {5ms, 13ms, 37ms} Component 7 candidate values: {2, 4, 6, 8}; Optional with capability signalling. 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per band] Per FS; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1, Option 1-2, both Option 1-1 and Option 1-2} Component 3 candidate values: {5ms, 13ms, 37ms} Component 7 candidate values: {2, 4, 6, 8}; Optional with capability signalling. |
| - Huawei, HiSilicon | Based on the latest version of UE feature of Rel-19 LP-WUS [1], there is one FFS point is the type of 62-2, 62-2a and 62-3. Depending on different UE implementation, LP-WUR may be supported for different bands (see the agreed LS below). Some discussion in RAN1/RAN2/RANP also considers the case that LP-WUS reception is only supported by UE in some of the bands. Therefore, the type (i.e., the granularity) of UE feature for LP-WUS should be per band for single carrier operation. Additionally, in RAN1#120bis, “per band” was adopted for FG 62-1 and FG 62-1a. Thus, for CONNECTED mode, for single carrier operations, the type should be also confirmed as “per Band”. However, for CA or DC operation, depending on different RF implementation, UE may only support specific band combinations for LP-WUS operation. For example, UE may support LP-WUS in band#A for single carrier case, also support LP-WUS in band#A with CA_band#A_band#B, but not support LP-WUS in band#A with CA_band#A_band#C, depending on the hardware implementation. Thus, CA and DC operation can be taken as separate rows with type ‘per band per BC’. Although the signaling overhead is the same between “per band per BC” and “per FS”, “per FS” has much smaller granularity of capability than “per band per BC” and thus introduces unnecessary flexibility. For example, “per FS” allows UEs to report supports of “LP-WUS” ONLY when UL-MIMO is supported or any other feature that is irrelevant to LR DL reception. Therefore, “per FS” is unnecessary. Agreement Send an LS to RAN2 and RAN4 to convey the following In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. Final LS in R1-2407559. Agreement: Adopt “Per band” for FG 62-1 and 62-1a:. Based on the discussion above, we have the following proposal - On the UE features of LP-WUS in CONNECTED mode, add the following 2 new rows where the type is per band per BC. - LP-WUS operation in CONNECTED mode based on OOK signal with CA/NR-DC - LP-WUS operation in CONNECTED mode based on overlaid OFDM sequence with CA/NR-DC Proposal 2: The table 1 in Appendix is adopted. 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1, Option 1-2, both Option 1-1 and Option 1-2} Component 3 candidate values: {5ms, 13ms, 37ms} Component 7 candidate values: {2, 4, 6, 8}; Optional with capability signalling. 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1, Option 1-2, both Option 1-1 and Option 1-2} Component 3 candidate values: {5ms, 13ms, 37ms} Component 7 candidate values: {2, 4, 6, 8}; Optional with capability signalling. 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signalling. 62-4; LP-WUS operation in CONNECTED mode based on OOK signal with CA/NR-DC; 1. Support of LP-WUS operation in CONNECTED mode based on OOK signal with CA/NR-DC; FG 62-2; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal with CA is not supported; Per band Per BC; n/a; n/a; n/a; Optional with capability signalling. 62-5; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence with CA/NR-DC; 1. Support of LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence with CA/NR-DC; FG 62-2a; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence with CA is not supported; Per band Per BC; n/a; n/a; n/a; Optional with capability signalling. |
| - Nokia | Proposals on FG62-2, 62-2a and 62-3: - FG type: Make the FGs ‘Per Band’ Index; Feature group; Components; Type. 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO; [Per band]. 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO; [Per band]. 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; [Per band]. |
| - OPPO | UE features for LP-WUS/WUR in CONNECTED mode We have not confirmed the types of FG 62-2 “LP-WUS operation in CONNECTED mode based on OOK signal” / 62-2a “LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence” / 62-3 “LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode”. To our understanding, the per band capability type for above FGs can be confirmed. Even the per FR could be possible, we cannot preclude the LR for connected mode can only support only some of frequency bands. Proposal 1: For FG 62-2, 62-2a and 62-3, confirm them as ‘per band’. Conflicted reporting values As the FGs splitting into more sub groups. It actually made few more duplicated capabilities. One example is the wake-up delay, which had been discussed in session for IDLE/INACTIVE mode. It is expected a delay value should be specified for UE to report. Although different UE may have different values, there may not need separated values between OOK and OFDM receivers for all RRC modes. The wake-up time is essentially the ramp-up time for main receiver. We should consider to report single value. In the beginning, we consider merging to a dedicate FG for each capability, to avoid conflict reported wake-up times for a UE. However, for simplicity, it is also OK to make Note to say the same value should be reported. The list of possible conflict values: FG 62-1/62-1a “5. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring” FG 62-2/62-2a “3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode” Proposal 2: In UE capability report the following pair of components should be reported with same values. - Component 5 for FG 62-1 and 62-1a. - Component 3 for FG 62-2 and 62-2a. Regarding the exact text change, we can have following added to the notes of FGs: Proposal 3: Including Notes for reporting minimum gaps: - FG 62-1: “Note: Same value should be reported for component 5 of FG62-1a, if supported” - FG 62-1a: “Note: Same value should be reported for component 5 of FG62-1, if supported” - FG 62-2: “Note: Same value should be reported for component 3 of FG62-2a, if supported” - FG 62-2a: “Note: Same value should be reported for component 3 of FG62-2, if supported” |
| - Samsung | FG 62-2, 62-2a and 62-3 During the last meeting, which FG type is supported for LP-WUS operation in CONNECTED mode was discussed as below: Regarding Type for FG 62-2/2a, adopt Alt-x - Alt-1: Adopt “Per band” for FG 62-2 and 62-2a for both single-band and multi-band operation - Alt-2: - Adopt “Per band” for FG 62-2 and 62-2a, and clarify 62-2/2a is defined for single-band operation, and define the following two new FGs - LP-WUS operation in CONNECTED mode based on OOK signal - LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) - Alt-2a: Adopt “Per FS” for FG 62-2 and 62-2a. Alt 2 was brought up to handle the special case in CA/DC operation with LP-WUS. Specifically, this is to take into account the case where, even within the same band, the support of LP-WUS in that band may vary depending on which band it is operating with in CA/DC. However, there seems to be no clear use case or reason why such an implementation should be specially considered. In addition, if the UE reports capability on a per-band basis, the gNB can determine whether LP-WUS can be supported in CA/DC based on the bands where the UE indicates LP-WUS support. Therefore, per-band reporting does not appear to cause any issue. Hence, Alt 1 is proposed. Proposal 1: Confirm “per band” in FGs 62-2/2a and 62-3 for both single-band and multi-band operation as follows: 62. NR_LPWUS; 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1, Option 1-2, both Option 1-1 and Option 1-2} Component 3 candidate values: {5ms, 13ms, 37ms} Component 7 candidate values: {2, 4, 6, 8}. 62. NR_LPWUS; 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1, Option 1-2, both Option 1-1 and Option 1-2} Component 3 candidate values: {5ms, 13ms, 37ms} Component 7 candidate values: {2, 4, 6, 8}. 62. NR_LPWUS; 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; [Per band]; n/a; n/a; n/a. |
| - Ericsson | FG 62-1 - Components - Suggest adding new component “9. Detection of LP-WUS information from binary OOK sequences” - This clarifies the difference on how UE obtains LP-WUS information compared to FG 62-1a FG 62-1a - Components - Suggest adding new component “8. Detection of LP-WUS information from OFDM overlaid sequences” - This clarifies the difference on how UE obtains LP-WUS information compared to FG 62-1 FG 62-2 - Components - Suggest adding new component “8. Detection of LP-WUS information from binary OOK sequences” - This clarifies the difference on how UE obtains LP-WUS information compared to FG 62-2a FG 62-2a - Components - Suggest adding new component “8. Detection of LP-WUS information from OFDM overlaid sequences” - This clarifies the difference on how UE obtains LP-WUS information compared to FG 62-2 In addition, the following was proposed in RAN1#122 but not yet resolved: Question 2-4a (new): Companies are encouraged to provide their preference on the following alternatives for Type for FG 62-2 and 62-2a - Alt-1: Adopt “Per band” for FG 62-2 and 62-2a for both single-band and multi-band operation - Alt-2: - Adopt “Per band” for FG 62-2 and 62-2a, and clarify 62-2/2a is defined for single-band operation, and define the following four new FGs - LP-WUS operation in CONNECTED mode based on OOK signal for CA - Type: Per FS - LP-WUS operation in CONNECTED mode based on OOK signal for DC - Type: Per FS - LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) for CA - Type: Per FS - LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) for DC - Type: Per FS - Alt-3: Adopt “Per FS” for FG 62-2 and 62-2a. In our view, Alt-1 “Per-band” should be supported for 62-2/2a due to lower complexity and signaling overhead compared to Alt-2. Also, Alt-3 is preferred over Alt2 as it is a cleaner solution. Thus, our first preference is Alt-1 and second preference is Alt-3. Our proposed updates (in red) to the FG descriptions are given below |
| - Apple | For connected mode, the following was proposed for discussion in RAN1#122 but no agreement was made: Question 2-4a (new): Companies are encouraged to provide their preference on the following alternatives for Type for FG 62-2 and 62-2a - Alt-1: Adopt “Per band” for FG 62-2 and 62-2a for both single-band and multi-band operation - Alt-2: - Adopt “Per band” for FG 62-2 and 62-2a, and clarify 62-2/2a is defined for single-band operation, and define the following four new FGs - LP-WUS operation in CONNECTED mode based on OOK signal for CA - Type: Per FS - LP-WUS operation in CONNECTED mode based on OOK signal for DC - Type: Per FS - LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) for CA - Type: Per FS - LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) for DC - Type: Per FS - Alt-3: Adopt “Per FS” for FG 62-2 and 62-2a. Alt-2 and Alt-3 achieve the same effect, but Alt-3 is simpler. Therefore, we propose to adopt Alt-3, i.e., “Per FS” for FG 62-2 and 62-2a. In this case, it also makes sense to have FG 62-3 as “Per FS”. Proposal: Support Alt-3: Adopt “Per FS” for FG 62-2 and 62-2a. In addition, adopt “Per FS” for FG 62-3. |
| - Qualcomm Incorporated | - LP-WUS procedure for idle/inactive state In this section, we discuss the UE capabilities for LP-WUS in idle and inactive states. This is based on the latest RAN1 UE features list for Rel-19 NR after RAN1 #122 in R1-2506627 and Session Notes of AI 9.5 in R1-2506411. - Nominal MO duration vs actual LP-WUS duration In RAN1 #121, it was agreed that both nominal LP-WUS MO X1 and actual LP-WUS duration X2 are configured by network. If the number of available OFDM symbols in nominal LP-WUS MO is smaller than X2, UE will skip monitoring LP-WUS in nominal MO. Otherwise, the UE monitors LP-WUS in the MO. When X1>X2, there can be many different combinations of available and unavailable OFDM symbols. For example, when X1=X2+1, there can be X1 cases where one OFDM symbol is unavailable for the UE to handle. This increases UE implementation complexity for determining whether and how LP-WUS should be monitored. For this, we propose that X1=X2 is the basic UE feature and X1>X2 is supported based on UE capability. RAN1 #121 Agreement Nominal MO duration (X1, in unit of OFDM symbols) and actual LP-WUS duration (X2, in unit of OFDM symbols) are configured. (Alt C) - A LP-WUS MO spans the nominal MO duration (i.e., the LP-WUS MO duration is the same as the nominal MO duration.) - If the number of available OFDM symbols within the nominal MO duration is no less than the actual LP-WUS duration, UE monitors LP-WUS on the first X2 available symbols within the LP-WUS MO. - Otherwise, UE does not monitor LP-WUS in this MO (i.e., the MO is dropped). Note: Any symbols that are not defined as unavailable are available symbols for LP-WUS.. Proposal 2.1: - For both FGs 62-1 and 62-1a, add a new component “nominal MO duration = actual LP-WUS duration” - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO - Introduce a new FG for “nominal MO duration larger than actual LP-WUS duration” for idle and inactive modes - LP-WUS procedure for connected state Issue 1: Nominal MO duration vs actual LP-WUS duration The agreed mechanism of LP-WUS mapping on available symbol(s) will create almost infinite variations of possible LP-WUS time-domain properties/structures. Even for a given UE on the serving cell where LP-WUS monitoring is configured, different LP-WUS MOs can have different time-domain properties/structures depending on how unavailable symbol(s) are overlapped with the LP-WUS MOs. Each UE has to store, on the WUR memory, all the possible LP-WUS time-domain properties/structures in the cell over the LP-WUS MOs, and the WUR has to monitor the various LP-WUS accordingly. UE implementation needs to make sure that the WUR can detect LP-WUS for every possible LP-WUS time-domain properties/structures including various LP-WUS segmentations due to unavailable symbol(s) in nominal MO durations and then wake up the MR for PDCCH monitoring after the minimum time gap. For OFDM WUR, the need for the mechanism itself is doubtful. As discussed in Section 2.1, the information bits carried by overlaid sequences can be acquired by the first 2K OOK symbols of the LP-WUS. The worst case is M = 1, N = 5, L = 2, in which case the LP-WUS for OFDM WUR spans 10 OFDM symbols. This is shorter than regular slot-based PDSCH. Therefore, it must not be so difficult for gNB to configure LP-WUS MOs such that the first 2K OOK symbols of each MO do not overlap with unavailable symbol(s). Having said that, it is quite unreasonable to mandate OFDM WUR to support the possible LP-WUS segmentations due to unavailable symbol(s). Fig. x We believe this mechanism creates a lot of problems such as IODT (testability), WUR memory requirement, and performances, and therefore, should be optional feature with UE capability signalling. If a UE does not report the UE capability signalling, the UE is expected to be configured with LP-WUS with X1 = X2, i.e., a LP-WUS spans a number of consecutive OFDM symbols, and if there is at least one OFDM symbol unavailable within the LP-WUS MO, the UE does not monitor the LP-WUS in the MO. Proposal 3.1: - For both FGs 62-2 and 62-2a, add a new component “nominal MO duration = actual LP-WUS duration” - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO - Introduce a new FG for “nominal MO duration larger than actual LP-WUS duration” Issue 2: Support of LP-WUS on the primary cell for CA RAN1 agreed to support LP-WUS monitoring on the primary cell to trigger PDCCH monitoring for all the serving cells for CA. Since there are various CA configurations supported for NR such as TDD-FDD CA, FR1-FR2 CA, licensed-unlicensed CA, etc, it is quite unreasonable to define the LP-WUS FGs simply “per-band”. A UE cannot declare support of LP-WUS procedure on a frequency band for any possible CA band combinations including the frequency band. There are two options to address the issue: - Opt.1: Define FGs 62-2 and 62-2a as “per-FS” - A UE can indicate on which frequency band(s), of which CA band combination(s), the UE supports the LP-WUS procedure, via the FG - Opt.2: Define FGs 62-2 and 62-2a as “per-band”, and introduce additional per-BC FGs indicating support of the LP-WUS procedure in the CA BC - NW can identify from the reported FG 62-2/2a and the new per-BC FG(s) on which frequency band(s), of which CA band combination(s), the UE supports the LP-WUS procedure We are open to either options – here we propose Option 1. Proposal 3.2: - Both FGs 62-2 and 62-2a are per-FS - Support of LP-WUR procedure in connected mode using OOK signal or OFDM overlaid sequence is reported per band per band combination Issue 3: FG type of 62-3 The FG type of 62-3 can follow the FG type of 62-2/2a. Proposal 3.3: - FG 62-3 is per-FS |
| - NTT DOCOMO, INC. | - Discussion - FG 62-2/2a - High priority issues (w/ ASN.1 impact) Regarding the reporting type, this has been discussed for multiple RAN1 meetings while no consensus has been reached yet. In the last RAN1 meeting, moderator provided following alternatives [2]: Question 2-4a (new): Companies are encouraged to provide their preference on the following alternatives for Type for FG 62-2 and 62-2a - Alt-1: Adopt “Per band” for FG 62-2 and 62-2a for both single-band and multi-band operation - Alt-2: - Adopt “Per band” for FG 62-2 and 62-2a, and clarify 62-2/2a is defined for single-band operation, and define the following four new FGs - LP-WUS operation in CONNECTED mode based on OOK signal for CA - Type: Per FS - LP-WUS operation in CONNECTED mode based on OOK signal for DC - Type: Per FS - LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) for CA - Type: Per FS - LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) for DC - Type: Per FS - Alt-3: Adopt “Per FS” for FG 62-2 and 62-2a. In our view, similar to idle/inactive modes, it would be straightforward to define the reporting type of FG 62-2/2a as “per band”. It was argued by some companies in the previous RAN1 meetings that CA case should be different capability. We are not sure whether it is necessary since FG 62-2/2a is the UE capability whether UE can receive LP-WUS by LR and trigger PDCCH monitoring. For LP-WUS reception itself, it is not related to CA and “per band”, where LP-WUS is monitored, would be enough. For triggering PDCCH monitoring, it is commonly controlled per cell group, including both intra-band CA and inter-band CA, and any differentiation within cell group would not be necessary. Proposal 1: - Reporting type of FG 62-2/2a is “per band” On the other hand, it was also agreed in previous RAN1 meeting that LP-WUS can be configured per cell group for DC case. Agreement - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell per cell-group - This does not imply that LP-WUS has to be configured on both cell groups - LP-WUS indication is applicable to all serving cells in the cell-group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS whether to support this case as separate UE capability from CA case - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell. We understand it would increase UE complexity to monitor LP-WUS for both MCG and SCG, and hence, this can be considered as additional capability. However, we are not sure where “per FS” is proper choice, since in this case, LP-WUS is monitored on both MCG and SCG and any differentiation of the monitored band among BC is not necessary. Proposal 2: - Introduce additional capability for “Support of LP-WUS monitoring in CONNECTED mode on both cell groups in NR-DC” - Prerequisite FG: FG 62-2 or 62-2a - Reporting type: per BC At RAN1#122, following agreement was made in AI9.6.3. Agreement: Nominal MO duration (X1, in unit of OFDM symbols) and actual LP-WUS duration (X2, in unit of OFDM symbols) are configured for LP-WUS in connected mode. (Alt C) - A LP-WUS MO spans the nominal MO duration (i.e., the LP-WUS MO duration is the same as the nominal MO duration.) - If the number of available OFDM symbols within the nominal MO duration is no less than the actual LP-WUS duration, UE monitors LP-WUS on the first X2 available symbols within the LP-WUS MO. - Otherwise, UE does not monitor LP-WUS in this MO (i.e., the MO is dropped). - Note: Any symbols that are not defined as unavailable are available symbols for LP-WUS. - Further discuss possible introduction of UE capability to restrict configuration of LP-WUS MO and LP-WUS duration. For example, - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO. As highlighted in yellow, it is FFS whether/how to introduce UE capability to restrict configuration of LP-WUS MO and LP-WUS duration. According to the proponent companies, it is difficult for UE to check whether unavailable OFDM symbols is overlapped with LP-WUS for every MO, since the unavailable symbols may be dynamically indicated in connected mode, unlike idle/inactive mode. However, the agreements related to the unavailable symbols in connected mode are almost the same as those in idle/inactive modes so far, and hence, we don’t see strong need to introduce such UE capability. Accordingly, this issue was discussed in the last RAN1 meeting under AI9.5, and the proponent company proposed to introduce restriction into FG 62-2/2a that “nominal MO duration = actual LP-WUS duration” but no other companies supported this. Considering that we are reaching the ASN.1 freeze on Dec’25, we can conclude this issue without introducing any additional UE capabilities. - FG 62-3 - High priority issues (w/ ASN.1 impact) Regarding the reporting type, FG 62-3 has prerequisite of FG 62-2/2a and it would be straightforward to have the same reporting type. Proposal 3: - Reporting type of FG 62-3 is same as those of FG 62-2/2a |

- Discussion

3.1	LP-WUS operation for IDLE/INACTIVE mode

Summary of companies’ inputs

- FG 62-1 - Components - Add “Detection of LP-WUS information from binary OOK sequences”: E/// - Add “nominal MO duration = actual LP-WUS duration”: QC, - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO - Note - “Note: Same value should be reported for component 5 of FG62-1a, if supported”: OPPO - FG 62-1a - Components - Add “Detection of LP-WUS information from OFDM overlaid sequences”: E/// - Add “nominal MO duration = actual LP-WUS duration”: QC, - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO - Other aspects - Discuss whether OFDM WUR can decode OOK ON-OFF pattern for LP-WUS: ZTE - Note - “Note: Same value should be reported for component 5 of FG62-1, if supported”: OPPO - New FG - nominal MO duration larger than actual LP-WUS duration: QC

Question 1-1:

Companie are encouraged to provide views on the need of following new components/FG in FG 62-1/1a:

- Q1-1-1: Introduce a new component “Detection of LP-WUS information from binary OOK sequences” in FG 62-1

- Q1-1-1a: Introduce a new component “Detection of LP-WUS information from OFDM overlaid sequences” in FG 62-1a

- Q1-1-2: Introduce a new component “nominal MO duration = actual LP-WUS duration” in FG 62-1 and 62-1a

  - Clarify “A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration” and “If there is at least one OFDM symbol unavailable for the LP-WUS MO within the ODFM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO”

  - Define a new FG for “nominal MO duration larger than actual LP-WUS duration”

| Company | Comment |
| --- | --- |
| vivo | - For Q1-1-1 and Q1-1-1a: Not needed, since the component #1 of FG 62-1 and FG62-1a seems to be already clear. - For Q1-1-2: Not needed This issue was concluded in RAN1#121 with no leftover for IDLE/INACTIVE. For CONNECTED mode, there was following FFS point, however, 1) it is for CONNECTED mode only and should be addressed in the main maintenance session, not directly in the UE feature session, 2) if it would be agreed, it should be an separate optional incapability, rather than a component of the basic capability as according to the agreement below. - Further discuss possible introduction of UE capability to restrict configuration of LP-WUS MO and LP-WUS duration. For example, - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO |
| Qualcomm | Q1-1-1, Q1-1-1a: Not necessary. Q1-1-2: This is necessary. We are OK to let main session to decide. We do not think such extreme flexibility of LP-WUS signal mapping can be accommodated in reality. |
| ZTE, Sanechips | Based on component 1 of 62-1a, OFDM WUR may not decode OOK ON-OFF pattern. Component 1 in 62-1 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK Component 1 in 62-1a 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence However, as indicated in the following agreement Agreement For LP-WUS overlaid OFDM sequence configuration in a cell, - gNB configures number of sequences Nseq, and one or two root values. - Alt1: Nseq is 2^n, where the value range of n is 0~4, 0~3, 0~2 for M=1, 2, 4, respectively. - No separate UE capability for different n values. - Nroot=2, when Nseq=16,8,4 for M=1,2,4, where Nroot is the number of root values. - Nroot=1 or 2 when Nseq is other value The number of CS Ncs is derived by Nseq/Nroot. The gNB could configure n=0, in this case, how the OFDM WUR obtain the same information as OOK WUR? For Q1-1-1a, it is not necessary, sine component1 clarified it is based on OFDM overlaid sequence. For new component “nominal MO duration = actual LP-WUS duration”, this issue already discussed before but no consensus. If needed, this should be decided by main session firstly. |
| DOCOMO | Q1-1-1/1a: Not necessary. Component 1 in FG62-1/1a is clear enough Q1-1-2: Open to discuss and would like to hear more companies view especially from UE side. Looks like this issue will be also discussed in Maintenance at least for connected mode in [M][FL1] Question 4.9-1 in the summary, and we are OK to further discuss there |
| Huawei, HiSilicon | Q1-1-1/1a: No need. Component 1 in FG62-1/1a is clear enough Q1-1-2: No need. ‘nominal MO duration > actual LP-WUS duration’ is probability the most popular scenario since it is difficult for gNB to reserve a clear 5MHz resource dedicated for LP-WUS. If the most popular scenario is not the basic capability, the whole feature is meaningless. |
| Apple | Q1-1-1/1a: Not necessary. Q1-1-2: Not necessary |
| Ericsson | Q1-1-1/1a: support as it clarifies how WUS information is obtained for OOK-WUR and OFDM-WUR. Q1-1-2: do not support. No agreement on this. Also, it is very hard to ensure “nominal MO duration = actual LP-WUS duration” due to presence of other channels/signals. |
| Samsung | Q1-1/1a: Not necessary. Component 1 is clear enough Q1-1-2: Considering that LP-WUS requires a lot of OFDM symbols, it is natural that nominal MO duration is larger than actual LP-WUS duration. |

Question 1-2:

Companie are encouraged to provide views on the need of following limitation for Component 5 between FG 62-1 and 62-1a:

- Same value should be reported for component 5 between FG 62-1 and FG 62-1a, when a UE reports both FG 62-1 and 62-1a

| Company | Comment |
| --- | --- |
| vivo | No action needed. UE is not mandated to report the same value for component 5 in FG62-1 and FG62-1a |
| Qualcomm | Not necessary. |
| ZTE, Sanechips | Currently, component 5 is ‘Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring’ according to the Appendix. in idle mode, how the UE report this to the gNB? |
| DOCOMO | Not necessary |
| Huawei, HiSilicon | No need. The wakeup delay may depend on different implementation. For a UE detecting OOK or OFDM, different timeline may be used, since different components may be turned on/off |
| Apple | No necessary |
| Ericsson | Not necessary. Different values can be used for FG62-1 and FG62-1a. |

3.2	LP-WUS operation for CONNECTED mode

Summary of companies’ inputs

- FG 62-2 - Components: - Add “Detection of LP-WUS information from binary OOK sequences”: E/// - Add “nominal MO duration = actual LP-WUS duration”: QC, - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO - Type - Per FS: vivo, Apple, QC, - Per band: HW/Hisi, Nokia, OPPO, Samsung, E///, DCM, - Note: - “Note: Same value should be reported for component 3 of FG62-2a, if supported”: OPPO - FG 62-2a - Components - Add “Detection of LP-WUS information from OFDM overlaid sequences”: E/// - Add “nominal MO duration = actual LP-WUS duration”: QC, - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO - Type - Per FS: vivo, Apple, QC, - Per band: HW/Hisi, Nokia, OPPO, Samsung, E///, DCM, - Other aspects - Discuss whether OFDM WUR can decode OOK ON-OFF pattern for LP-WUS: ZTE - Note: - “Note: Same value should be reported for component 3 of FG62-2, if supported”: OPPO - FG 62-3 - Type - Per FS: vivo, Apple, QC, - Per band: HW/Hisi, Nokia, OPPO, Samsung, - Other FGs - LP-WUS operation in CONNECTED mode based on OOK signal with CA/NR-DC: HW/Hisi - Type: - Per FS: HW/Hisi - LP-WUS operation in CONNECTED mode based on overlaid OFDM sequence with CA/NR-DC: HW/Hisi - Type: - Per FS: HW/Hisi - nominal MO duration larger than actual LP-WUS duration: QC - Support of LP-WUS monitoring in CONNECTED mode on both cell groups in NR-DC: DCM, - Per BC: DCM

Question 2-1:

Companies are encouraged to provide views on which of the alternatives can be taken for FG 62-2/2a/3’s type:

- Alt-1: Per band

- Alt-2: Per FS

| Company | Comment |
| --- | --- |
| Moderator | It seems to me that majority is in line with Alt-1. Several companies in Alt-1 camp (not many) also propose to define a new FG to cover potential missing capability that cannot be covered in FG 62-2/2a. In this case, it seems to me that the final result is quite similar to Alt-2. Hence, to me, the fundamental question is whether FG 62-2/2a with per band can cover all the scenarios or not. If not and if per FS can address the scenarios, Alt-2 is not a bad choice. |
| Vivo | Agree with Moderator’s views. We prefer Alt-2. |
| Qualcomm | OK with Alt-2. Per-FS resolves the issue. |
| ZTE, Sanechips | Considering alt2 may not need to define separate FG, for simplicity, it is slightly preferred. |
| DOCOMO | It is true that even if we go with Alt-1 for FG 62-2/2a, there are some missing cases (e.g., LP-WUS on both cell groups) which requires special handling. However, “per FS” should be introduced only if the motivation is well justified to avoid huge signalling overhead, even if it looks simpler. So, we prefer to introduce additional FG(s) only for the case where per band cannot work, rather than simply apply per FS for FG 62-2/2a. More clarification of the motivation (e.g., where the pain point comes) to introduce per FS is highly appreciated. |
| Huawei, HiSilicon | We suggest to postpone this discussion after some conclusions are made under 8.6. Depending on implementation, UE may or may not support LP-WUS for a band, for different band combinations. - For example, UE support CA_A_B and CA_A_C, where 4RX is required for band A and band C, while 2RX is required for band B. For MR operation with CA_A_B, 6 RX chains are needed, while for MR operation with CA_A_C, 8 RX chains are needed. So by legacy, UE should include at least 8 RX chains to support both CA_A_B and CA_A_C. - If LP-WUR is implemented by an additional separate RX chain, namely in total 9 RX chains in the UE, then for CA_A_C with LP-WUS configuration, UE can allocate RX chain to the combination 4+4+1 for MR_A/MR_C/LR_A. - However, if LP-WUR share one RX chain with MR, namely in total 8 RX chains in the UE, then - for CA_A_B with LP-WUS configuration, UE can allocate RX chain to the combination 4+2+1 for MR_A/MR_B/LR_A. - But for CA_A_C, there is no extra RX chain for LP-WUR. - To summarize, UE can support LP-WUS for band A with CA_A_B but not band A with CA_A_C. - So, if there can be possible simultaneous operation (including the collision during a switching gap), it should be per FS or per band per BC. - However, if UE is not required to operate simultaneously with MR and LR with enough switching gap, then defining it as per band is also OK, since no collision shall happen. |
| Apple | Alt-2 |
| Ericsson | Alt-1 is our first preference, and Alt-2 second preference. |
| Samsung | We prefer Alt 1, and we are open to discuss new FG to handle CA/DC case separately. |

Question 2-2:

Companie are encouraged to provide views on the need of following new components/FG in FG 62-2/2a:

- Q2-2-1: Introduce a new component “Detection of LP-WUS information from binary OOK sequences” in FG 62-2

- Q2-2-1a: Introduce a new component “Detection of LP-WUS information from OFDM overlaid sequences” in FG 62-2a

- Q2-2-2: Introduce a new component “nominal MO duration = actual LP-WUS duration” in FG 62-1 and 62-1a

  - Clarify “A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration” and “If there is at least one OFDM symbol unavailable for the LP-WUS MO within the ODFM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO”

  - Define a new FG for “nominal MO duration larger than actual LP-WUS duration”

| Company | Comment |
| --- | --- |
| vivo | For Q2-2-1 and 1a: Not needed. As component 1 of FG 62-2/2a seems clear. For Q2-2-2, Same comments as for idle/inactive. |
| Qualcomm | Q2-2-1, Q2-2-1a: Not necessary. Q2-2-2: This is necessary. We are OK to let main session to decide. We do not think such extreme flexibility of LP-WUS signal mapping can be accommodated in reality. |
| ZTE, Sanechips | Similar as previous question. |
| DOCOMO | Q2-2-1/1a: Not necessary as idle mode Q2-2-2: same comment as idle mode |
| Huawei, HiSilicon | No need. Save comments as for IDLE. |
| Apple | Not necessary |
| Ericsson | Q2-2-1/1a: support as it clarifies how WUS information is obtained. Q2-2-2: do not support. Also, it is very hard to ensure “nominal MO duration = actual LP-WUS duration” due to presence of other channels/signals. |
| Samsung | Similar as idle/inactive case. Not necessary. |

Question 2-3:

Companie are encouraged to provide views on the need of following limitation for Component 3 between FG 62-2 and 62-2a:

- Same value should be reported for Component 3 between FG 62-2 and 62-2a, when a UE reports both FG 62-2 and 62-2a

| Company | Comment |
| --- | --- |
| vivo | No action needed. UE is not mandated to report the same value for component 5 in FG62-2 and FG62-2a |
| Qualcomm | Not necessary. |
| ZTE, Sanechips | Not needed |
| DOCOMO | Not necessary |
| Huawei, HiSilicon | No need. |
| Apple | Not necessary |
| Ericsson | Not needed. |

- Conclusions

TBD

References

- R1-2506826	Discussion on WUR features	ZTE Corporation, Sanechips

- R1-2506884	UE feature for Rel-19 LP-WUS/WUR	vivo

- R1-2506922	UE features for Rel-19 LP-WUS	Huawei, HiSilicon

- R1-2507075	LP-WUS/WUR UE features	Nokia

- R1-2507164	Discussion for UE features of LP-WUS/WUR	OPPO

- R1-2507239	UE features for LP-WUS/WUR for NR	Samsung

- R1-2507268	UE features for LP-WUS/WUR for NR	Ericsson

- R1-2507660	Views on Rel-19 LP-WUS/WUR UE features	Apple

- R1-2507706	UE features for LP-WUS/WUR for NR	Qualcomm Incorporated

- R1-2507798	Discussion on UE features for LP-WUS/WUR for NR	NTT DOCOMO, INC.

Appendix: LP-WUS/WUR UE features after RAN1#122

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. The support of LP-SS based RRM measurement 5. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring 6. Support of all M values {1, 2, 4} for FR1 for LP-WUS 7. Support of M value 1 for 120 kHz SCS FR2 for LP-WUS 8. Support of all M values {1, 2, 4} for LP-SS |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: {capability 1, capability 2, capability 3} Note: According to RAN2 agreement, UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading. How to capture this is up to RAN2 | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence 2a. The support of SSB-based RRM measurement 5. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring 6. “Support of all M values {1, 2, 4} for FR1 for LP-WUS 7. “Support of M value 1 for 120 kHz SCS FR2 for LP-WUS |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 5 candidate values: {capability 1, capability 2, capability 3} Note: According to RAN2 agreement, UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading. How to capture this is up to RAN2 | Optional with capability signalling |
| 62. NR_LPWUS | 62-1b | LP-SS based RRM measurement in IDLE/INACTIVE mode when LP-SS overlaid sequence is configured | 1. Support of LP-SS based RRM measurement in IDLE/INACTIVE mode when LP-SS overlaid sequence is configured 2. Support of all M values {1,2,4} for LP-SS | 62-1a | YES | n/a | LP-SS based RRM measurement in IDLE/INACTIVE mode when LP-SS overlaid sequence is configured is not supported | Per band | n/a | n/a | n/a | Note: If LP-SS overlaid sequence is configured, and if both SSB based and LP-SS based thresholds are configured for RRM measurement, it is up to UE implementation which threshold to use | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {Option 1-1, Option 1-2, both Option 1-1 and Option 1-2} Component 3 candidate values: {5ms, 13ms, 37ms} Component 7 candidate values: {2, 4, 6, 8} | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Support of all M values {1, 2, 4} for FR1 6. “Support of M values {1, 2} for 60 kHz SCS for FR2, M value 1 for 120 kHz SCS FR2 7. Maximum number of codepoints to be checked by UE per MO |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {Option 1-1, Option 1-2, both Option 1-1 and Option 1-2} Component 3 candidate values: {5ms, 13ms, 37ms} Component 7 candidate values: {2, 4, 6, 8} | Optional with capability signalling |
| 62. NR_LPWUS | 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |