3GPP TSG RAN WG1 #122bis			R1-2508178

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	10.6.1

Source:	Moderator (Thales)

Title:	FL Summary #5 - NR-NTN GNSS resilience

Document for:	Discussion, Decision

## Introduction

This Feature Lead Summary (FLS) document aims to collect and align company views on the Release-20 study concerning GNSS-resilient NR-NTN operation (RP-251933 [1]). It provides a summary of the contributions submitted under agenda item 10.6.1 at TSG-RAN WG1 #122bis, along with key issues identified. The objective of this FLS is to facilitate consensus building and offer recommendations for prioritizing discussions, including considerations for the potential postponement of certain topics.

A total of 42 contributions have been submitted for discussion and decision at the current meeting. The list of company Tdocs can be found in the reference section of this FLS. A compilation of company proposals regarding NR-NTN resilience to GNSS unavailability and degradation is available in document R1-2507952.

SI Objectives are [1]:

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

# Topic#1 Evaluation assumptions and methodology

This section aims to further discuss the evaluation assumptions/parameters and the evaluation methodology.

## Background

On topic#1, RAN1#122 made the following agreements:

Agreement

For the study on GNSS resilient operation, scenarios should be considered where:

- Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link.

- Scenario 2: There is a previously acquired GNSS based position

  - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded

    - FFS: value of T

    - FFS: GNSS based UE location validity duration

    - FFS: GNSS accuracy

- Note: Two scenarios above belong to description as in SID.

Agreement

For the study on GNSS resilient operation, the following scenarios are considered for evaluation purposes:

| Satellite orbit | GSO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Satellite scenario parameters | At least Table 6.1.1.1-1 in TR 38.821, Parameters in Table 6.1.1.1-2 in TR 38.821could be considered. |  |  |
| Beam size (note 3) | At least values captured in Table 6.1.1.1-1/2 are considered, |  |  |
| Minimum elevation angle | 30° (LEO), 12.5° (GSO) |  |  |
| Frequency ranges/bands | FR1 NTN (2GHz/Ku: 14 GHz) and FR2 NTN (30 GHz) |  |  |
| UE type | Handheld (L/S band), VSAT (Ku/Ka) |  |  |
| UE speed (note 1) | L/S band: 3 km/h, 120km/h, 1500 km/h, Ku/Ka: 3 km/h , 120 km/h, 1500 km/h, (note 2) |  |  |
| Beam/cell type (note 4) | (Quasi)-Earth-fixed beams/cells and Earth-moving |  |  |
| Note 1: UE altitude is considered Note 2: 10 km altitude in case of aircraft scenario. 1500 km/h is for aircraft scenario. Note 3: Same values can be reused for other elevation angle than nadir. Beam size of the edge beam can be reported by companies. Note 4: Footprint of the beam could be: case 1: within the orbital plane, case 2: at 90° with respect to orbital plane. Companies to report the elevation angle of the beam. Note 5: Other parameters can be reported by companies. |  |  |  |

Note: The cyan highlighted parameters are prioritized for evaluation purposes.

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Proposal 2 Agree assumptions for nadir beam diameters for Ku-band and discuss whether Ka-band parameters are realistic. Proposal 3 The study should cover both Set-1 and Set-2 beam diameters with equal priority. Proposal 4 Discuss and agree assumptions for non-nadir beam sizes. |
| ZTE | Proposal 1: Scenario 2 (There is a previously acquired GNSS based position), which is aligned with “1/ temporarily unavailable defined in SID”, should be considered as the target if enhancement is expected for 5G NR-NTN. Proposal 2: The maximum GNSS error for low speed UE should be focused when evaluating GNSS accuracy for scenario 2. Proposal 3: For scenario 2, the GNSS accuracy can be assumed as V*T = 1km, where UE speed is assumed as V=3km/h and time duration T from the last acquired GNSS position is assumed as T=20min. |
| Xiaomi | Proposal 1. Given the linkage to RAN4 specification on timing adjustment accuracy and positioning margin, it’s suggested RAN1 needs to involve RAN4 in the location uncertainty value X determination. Proposal 2: For the scenario and case combination, consider interpretation 2, i.e., the following in the study for NR NTN GNSS resilient operation - Combination 1 (Scenario 1 – case a) - Combination 2 (Scenario 2 - case b) |
| CMCC | Proposal 2: For the evaluation of GNSS resilient NR-NTN operation, the following methods can be studied when the UE cannot rely on its GNSS for timing and frequency compensation on the service link, - RAT dependent positioning methods for UE to acquire its location - The UE performs timing and frequency compensation based on the reference location provided by network Proposal 3: For evaluating positioning performance of GNSS resilient NR-NTN operation, the evaluation metrics and assumptions of RAT dependent positioning methods study in Rel-18 NR NTN can be considered as a start point. Proposal 4: If considering the limitation of simultaneously active beams in different NR NTN deployment scenarios, the feasibility of network broadcasting the configuration of reference signals or the reference location need to be studied. Proposal 5: Regarding the evaluation of Scenario 2, the following factors need to be considered for determining the acceptable time duration T of GNSS information unavailable, - UE velocity - Timing/frequency error requirements - Elevation angle Proposal 6: The assistance information on GNSS based UE location validity duration can be reported by UE. Proposal 7: For horizontal location uncertainty within an area assumed in the evaluation regarding GNSS-resilient operation, the beam size can be the baseline for determining the uncertainty location area. Proposal 8: For the study on GNSS resilient operation, the detailed satellite parameters of Ku-band for evaluation purpose need to be defined. Proposal 9: It is suggested to prioritize the LEO based deployment in S-band for the study on GNSS resilient operation. Proposal 14: It should be further discussed whether a unified solution should be strived to solve both low speed mobility (3km/h, 120km/h) and high speed mobility (1500km/h) of the air traffic. |
| Tejas Networks | Proposal 1 : The GNSS unavailability duration, T, depends significantly on the cause of the outage, and assuming a fixed value for T is challenging. |
| HONOR | Proposal 1: RAN1 to define the GNSS accuracy and evaluate the threshold T at which the degraded GNSS accuracy exceeds network’s acceptable limits under different scenarios. Proposal 2: The GNSS based UE location validity duration can be equal to T. The extended validity duration can be determined based on the network indication. |
| LG Electronics | Proposal #1: In Rel-20 NR NTN, RAN1 should consider feasible GNSS-resilient scenarios to streamline the specification. Proposal #2: In Rel-20 NR NTN, Scenario 1 (where the UE cannot rely on GNSS) in FR2 should be deprioritized. |
| Eutelsat | Proposal 1: The GNSS resilience study should consider Ku band using 15 kHz, 30 kHz and 120 kHz SCS. |
| ETRI | Proposal 1. RAN1 to consider intentional threats, such as jamming and spoofing, as well as environmental impairments for practical GNSS performance degradation modelling. Intentional threats on GNSS may last several hours. Proposal 2. RAN1 to consider the following aspects from UE perspectives for GNSS performance degradation modelling: Detection mechanism to clarify GNSS status (and corresponding detection probability/latency on GNSS status) Time-To-Recover Max position drift / drift variation FFS, other aspects Proposal 3. RAN1 to consider the following aspects from system perspectives for GNSS performance degradation modelling: (Timing) pattern of GNSS-unavailability occurrence Maximum duration of GNSS-unavailable state (i.e., max outage duration) FFS, other aspects Proposal 4: RAN1 to consider the beam footprints projected onto the Earth’s surface for evaluation of NR-NTN GNSS resilience. - The values of X and T should be determined by considering the radius of edge beam, rather than by the radius of Nadir beam Proposal 7. RAN1 to consider the Scenario 1 as baseline Scenario 2 can be optionally considered during this SI |
| Nokia | Proposal 6: RAN1 to explicitly define the meaning of “degraded accuracy” including the relevant and testable metrics for classification of the accuracy of GNSS information. |
| MediaTek | Proposal 1: For the scenario 1 in GNSS resilient operation, study the maximum GNSS UE position error to support legacy RACH preamble formats during initial access. Proposal 2: RAN1 can first discuss scenario 1 to at least determine the maximum GNSS position error that can be supported in GNSS resilient operations. |
| Sharp | Proposal 1: A maximum estimation error should be defined for GNSS resilient operation. A reference distance indication may be a useful option but should not be specified as mandatary. |
| Qualcomm | Proposal 1: RAN1 to study how to perform nominal uplink time and frequency adjustments for GNSS-resilient UEs in Scenarios 1 and 2 - As a baseline for evaluations, RAN1 can assume that the UE is able to compensate for the nominal timing and frequency offset. - The nominal time and frequency adjustment can be based on a reference location inside the uncertainty area. - For evaluation purposes, the reference location can be set as follows: - Scenario 1: Reference location can be the center of the beam. - Scenario 2: Reference location can be the center of the uncertainty area. Proposal 2: RAN1 to study enhancements to the random access procedure to achieve the following requirements: - Requirement 1: Robustness to the presence of differential RTD/RTT within the uncertainty area (based on the agreed scenarios for evaluation). - Requirement 2: After random access completion, the residual timing and frequency errors should be within Xμs and Y ppm. - FFS: Values of X and Y. Proposal 3: For PRACH performance evaluation, RAN1 to consider additional timing and frequency errors as follows: - Timing error: Te = 0.39us for 15kHz SCS [3, TS 38.133, Table 7.1.2-1]. - Frequency error: fe=0.1 ppm. Proposal 4: For analytical characterization of current PRACH, adopt the following criteria: - A differential RTT is supportable if min(cyclic prefix duration, Sequence duration×maxNcsL) Differential RTT +Te - A differential RTD is supportable if Δf=Differential RTD+fefc≤ΔfRA2+γΔfRA - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0 Proposal 10: For the evaluation of frequency and timing adjustment methods, RAN1 will consider evaluation methodology: - Step 1: Drop the UE at a location where the satellite is visible at an elevation angle θ - Step 2: Propagate the satellite during the simulation time T (assuming a circular trajectory around the center of Earth) - Step 3: The UE applies an initial timing and frequency offset (randomly distributed within Te and fe, which are the residual timing and frequency errors after successful completion of random access) and updates the timing and frequency adjustments at time t based on a correction method - Step 4: The network evaluates the timing and frequency error after every TUL ms, assuming an uplink transmission every TUL ms - Step 5: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively - Using the steps mentioned above, RAN1 to evaluate timing and frequency adjustment mechanisms using the following KPI: - Number of TA and FA commands issued within T. |
| NTT DOCOMO | Proposal 1: For Scenario 2: - T is no smaller than the GNSS based UE location validity duration - FFS: upper bound of the value of T |
| CAICT | Proposal 1: Scenario 1 and Scenario 2 correspond to the evaluation Case a and Case b respectively. |
| CSCN | Proposal 1: The worst case of GNSS resilient operation, i.e., Scenario 1, should be prioritized. Solutions designed for Scenario 1 can be partially or fully adopted to address the timing and frequency synchronization of Scenario 2. |
| ST Engineering iDirect | Observation 1: The satellite position error can exceed 1 km, due to satellite maneuvers or due to limited accuracy of the TLE data. For NGSO satellites, the position can be as large as 12 km during collision avoidance maneuvers. Proposal 1: Add an evaluation parameter to consider a satellite position error of 1 km for GSO and 12 km for NGSO. Observation 2: High accelerations cause high delay variations. Proposal 2: Add an evaluation parameter that defines the satellite/UE acceleration, e.g., up to 10 m/s2 for UE mobility and up to 100 m/s2 for LEO mobility. |
| SageRAN | Proposal 1: The length of 1/2 CP can be used for time difference criteria. Proposal 2: For terminal with non-zero altitude, distance d can be determined by h0, h1 and α by d=RE+h12sin2α+h0-h12+2h0-h1RE+h1-RE+h1sinα where h0, h1 and α denote the altitude of satellite, the altitude of terminal and the elevation angle. Proposal 3: According to the low of sine and cosine, the change of geocentric angle can be derived with known sides and angles in the triangle formed by Earth Center, the satellite and the terminal. Proposal 4: The minimum time duration can be estimated assuming that the UE velocity is completely opposite to the satellite velocity. |

## Summary of companies’ contributions

This is a summary in bullet-points of companies’ proposals and observations on topic#1.

Ericsson: Observes missing/unclear nadir beam diameter info for Ku/Ka bands in TR 38.821 and proposes aligning on realistic assumptions for these parameters. Recommends that studies equally consider Set-1 and Set-2 beam diameters, and discuss non-nadir beam sizes.

vivo: Notes that outdated satellite ephemeris information introduces errors needing consideration.

ZTE: Observes that focusing on Scenario 2 (previously acquired GNSS position) is more aligned with industry needs than Scenario 1 (no GNSS). Proposes concentrating on low-speed UE GNSS error and using a set assumption of GNSS error as V×T for practical studies. V=3km/h/T=20min and V*T = 1km.

Xiaomi: Proposes involving RAN4 (for timing/positioning specs) in defining location uncertainty, and recommends using specific scenario/case combinations for NR NTN GNSS-resilient operations.

CMCC: Proposes multiple fallback methods when GNSS is unavailable (e.g., RAT-dependent positioning, network-provided reference location) and setting evaluation metrics accordingly. Suggests using beam size as baseline for uncertainty, prioritizing S-band LEO studies, and considering mobility diversity (low/high speed).

CATT: Observes that VSAT terminals face challenges with beam direction when GNSS is unavailable and that Scenario 2 (partial GNSS info) is more relevant, with GNSS accuracy as the key metric.

TCL: Proposes clarifying how validity duration of GNSS-based UE location is defined.

OPPO: Observes that uplink time/frequency sync solutions will be similar for both regenerative and transparent payloads and should focus on TA and frequency pre-compensation.

Tejas Networks: Proposes that GNSS unavailability duration (T) is highly variable and difficult to assume a fixed value.

HONOR: Proposes RAN1 should define GNSS accuracy and a threshold T for when degraded accuracy exceeds network limits, and that validity duration of location info should equal T or be extended per network indication.

NEC: Observes significant vertical error and altitude diversity challenges in NTN deployments, and notes limitations of studies assuming a fixed GNSS unavailability duration.

LG Electronics: Proposes that Rel-20 NR NTN should focus on feasible GNSS-resilient scenarios and deprioritize Scenario 1 (no GNSS in FR2).

Eutelsat: Observes Ku-band LEO-1200 is highest priority, with cell sizes up to 300 km for Ku band. Proposes GNSS resilience studies should cover Ku band using various numerology options (15, 30, 120 kHz).

ETRI: Proposes to model GNSS degradation including intentional threats like jamming, consider detection and recovery mechanisms, and use beam footprints for scenario evaluation. Observes real-world GNSS outages can last hours, with large differential position/timing/Doppler.

Nokia: Observes that with proper design, Scenario 1 is unlikely in deployment. Points out TA and FO uncertainty depend on UE velocity and time since last GNSS fix, and proposes explicit definition of “degraded accuracy.”

MediaTek: Observes position error should avoid physical layer changes, with time duration T linked to UE velocity. Proposes studying maximum GNSS position error support for legacy RACH formats based on scenario 1.

Sharp: Proposes defining a maximum estimation error for GNSS resilience, possibly including a reference distance indication as an option.

Qualcomm: Proposes a framework for nominal/differential offset compensation in Scenarios 1/2, strengthening random access robustness, setting error tolerances, and detailed evaluation methodologies.

NTT DOCOMO: Proposes that in Scenario 2, T (GNSS unavailability duration) should be at least as long as the validity duration of GNSS-based location.

CAICT: Proposes that Scenario 1 and Scenario 2 should correspond to Cases a and b for evaluation.

CSCN: Proposes prioritizing the worst case (Scenario 1) in GNSS resilience design, with solutions applicable to Scenario 2.

ST Engineering iDirect: Observes satellite position errors can be substantial due to maneuvers and proposes adding evaluation parameters for satellite position error and acceleration.

SageRAN: Proposes various mathematical criteria/methods for evaluating time differences and UE/satellite distances, factoring in altitude, elevation angle, and opposite velocities; observes minimum T for GNSS information absence for various scenarios should be considered.

## Initial proposal

### Proposal 1-1

Based on companies’ inputs, the following initial proposal is made:

Proposal 1-1-v0

For the evaluation of GNSS resilient NR-NTN operation, it is assumed that the UE performs timing and frequency compensation based on a reference location inside the uncertainty area. The reference location can be set as follows: - Scenario 1: Reference location can be the center of the beam. - Scenario 2: Reference location can be the center of the uncertainty area.

Companies are encouraged to comment on Proposal 1-1-v0

| Company | Comments |
| --- | --- |
| Samsung | It is unclear on the main difference between scenario 1 and scenario 2. For scenario 2, it is unclear how UE know the uncertainty area. Thus, considering scenario 1 is enough. |
| CATT | Support scenario 1. |
| HONOR | Support scenario 1. |
| Xiaomi | I guess we could agree to scenario 1 firstly. Regarding scenario 2, if the error is tolerable for PRACH in initial access phase, we can actually live with not compensating. |
| Sony | We view Scenario 1 as unnecessary as Scenario 2 covers both cases. Suggest rewording proposal: For the evaluation of GNSS resilient NR-NTN operation, it is assumed that the UE performs timing and frequency compensation based on a reference location inside the uncertainty area. The reference location can be set as follows: Reference location can be configured by the network on a beam specific basis in SIB19 or otherwise. |
| Toyota | The objective of this proposal is not clear. As pointed out by Samsung, the uncertainty area for Scenario 1 and 2 is not clear. For example, in Scenario 1, uncertainty area can be the entire cell. Without clearly defining/identifying the uncertainty area for different scenarios, not clear what is the meaning of the center of the uncertainty area. As reported in our contribution, when the entire footprint of a beam is considered as the uncertainty area, a single reference location is not sufficient for compensating time/frequency (see for example, Table C-1 in Appendix C). Thus, this agreement limits the choice of solutions unnecessarily and does not capture all proposed solutions. |
| Tejas Networks | Support Scenario 1 Reference location for frequency compensation can be the center of the beam Common TA parameter in the SIB19 for the time compensation can be the point within the beam closest to the satellite. |
| JHUAPL | At low elevation angle scenarios, the Doppler frequency or the TA rate spread is not approximately symmetric about the beam center. See the attached plots: the Doppler at the beam center is about 42 kHz, lower-edge it is about 34 kHz and the top-edge it is about 44 kHz (similar scenario for the TA rate). Reference location should be left to the gNB to specify. |
| CEWiT | We believe the beam itself can be considered an uncertainty area. Hence, it is reasonable to refer to it as the “beam and/or uncertainty area” rather than having two different scenarios. Furthermore, the reference point can be defined as the point with the minimum distance from the satellite within that particular beam, thereby avoiding the need for a negative timing advance (TA). |
| Panasonic | The purpose of defining the reference location for the evaluation is not clear to us. At least for scenario 2, reference location is not necessary because UE location obtained in the past can be used. For scenario 1, the reference location should be used for UE time and frequency pre-compensation, but it is not necessary to limit to beam center. |
| TCL | We prefer Scenario 1. |
| ZTE | The differential RTT and RTD can be directly obtained by the difference of maximum and minimum values. The introduction of RP is not needed. Moreover, regarding scenario 1, as the beam footprint may not be circle under lower elevation angle according discussion in previous meeting, the meaning of “center of the beam” seems not very clear. |
| Sharp | The necessarily of introducing a reference point is not clear. As long as a maximum error on time/freq compensations are specified, the reference point is not necessary. The use of the reference point may not reduce the estimation error depending on the relative location between the UE and the reference point. The center of the beam may be the starting point although it may not be the suitable for footprints with large angles. |
| ST Engineering iDirect | In RAN1#122, there was no agreement to define a reference solution. According to our understanding, the reference location is mentioned by several companies as a potential solution. As such, this proposal should be better positioned in Section 7 (potential solutions) of this FLS, rather than this section on evaluation assumptions. Note that for large cell sizes, the use of multiple reference locations inside the beam might be necessary. Also, the use of reference locations for an Earth-moving cell need to be analyzed. |
| LGE | The purpose of this proposal is unclear to us. We think that the evaluation assumptions should be aligned across companies first. In our understanding, the evaluation assumptions for differential delay/Doppler analysis different among companies. For example, some companies assume TO/FO pre-compensation based on a reference location and consider the remaining residual delay/Doppler as differential delay/Doppler. Others consider the minimum-maximum differential delay/Doppler within a specific area (e.g., cell or beam). Moreover, some companies consider round-trip Doppler, which expands the Doppler range of their analysis results. |
| Futurewei | It seems like the proposal provides a solution to perform timing and frequency compensation based on a reference location. It may be too early to discuss solutions without completing evaluations. However, if the intent of the proposal is for evaluation purposes, then it can be modified as follows: For the evaluation of GNSS resilient NR-NTN operation,. the reference location can be set as follows: - Scenario 1: Reference location can be the center of the beam. - Scenario 2: Reference location can be the center of the uncertainty area. Other reference points are possible. |
| Nokia | In our view, we should use the term “reference location” with a bit of caution. A reference location from RAN2 point of view is something deterministic, which is defined by the network. Regarding the scenario 2, this basically means that the “location” becomes UE specific, which would reflect the case where a UE is having a previously good/valid GNSS based location (or accurate geo-location). To our understanding, this should be the default operation mode. The scenario 1 simply reflects the situation where a UE does not have any knowledge of its geo-location, and would be a special case of scenario 2. |
| Ericsson | We support the proposal for Scenario 1 but would like a note to be added that other reference locations can also be considered. We propose to remove Scenario 2 from this proposal since in this case the center of the uncertainty area is the last GNSS position acquired by the UE, which means regular pre-compensation with an outdated GNSS position (the term "reference location" is confusing in this case). |
| OPPO | Given that scenario 1 (UE cannot rely on its GNSS) and scenario 2 (there is a previously acquired GNSS) have been both agreed in the last meeting, we support the proposal to determine the reference location for time and frequency pre-compensation for evaluation. |
| Apple | For scenario 1, Reference location can be the point with the shortest distance to the satellite to avoid introduce the negative TA. |
| ETRI | We think both scenario 1 and 2 are incorrect. For non-nadir beam, the reference location cannot be located at the center of the beam in practice. We suggest to use the scenario: the reference location can be 3dBbeamwidth2 . See our comments on Proposal 1-4 for more details. |
| Ofinno | We support scenario 1. |
| Spreadtrum | We prefer scenario 1. It is unclear how can UE know uncertainty area in scenario 2. |
| NEC | Uncertainty area covers the uncertainty of a beam or a cell. Thus, uncertainty area is a one scenario for all. |
| Huawei, HiSilicon | support |
| NTT DOCOMO | For scenario 1, a reference location could be defined, and the legacy NTN reference location can be considered as a starting point, further optimization may be needed regarding impact of large timing/frequency errors. For scenario 2, our understanding is that the previously acquired GNSS information could be applied for time/ frequency compensation, and introducing a reference location is not needed. |
| Lenovo | Support scenario 1 |
| CMCC2 | In general, we are fine with the proposal. As it was agreed in the last meeting that both Case a and Case b can be considered, Agreement: For evaluation regarding GNSS-resilient operation, based on GNSS information or the lack of it, it is assumed that the UE has a horizontal location uncertainty within an area - Case a: the location uncertainty area is the area served by the cell or beam. - Case b: the location uncertainty area is a circle of radius X km - X to be reported The scenario 1 in which the reference location can be the center of the beam is related to the Case a in the previous agreements. And the scenario 2 in which the reference location can be the center of the uncertainty area can be related to Case b. |

### Proposal 1-2

Based on companies’ inputs, the following initial proposal is made:

Proposal 1-2-v0

For PRACH performance evaluation for existing PRACH formats based on analytical characterization, adopt the following criteria: - PRACH tolerance is exceeded if - min(Ncp, GP, Sequence duration×maxNcsL) -Te ≤ Differential RTT, where; - Ncp is cyclic prefix duration - GP: Guard period - max Ncs is given by Tables 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211 - Te is an additional timing error to be reported. - Note: the delay spread is neglected - Or Differential Round Trip Delay+fefc≥ΔfRA2+γΔfRA - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0 - fe=0.1 ppm

Companies are encouraged to share views on Proposal 1-2-v0

| Company | Comments |
| --- | --- |
| Samsung | Without getting RAN4 inputs, it is unclear PRACH tolerance is meaningful. Thus, we need to send LS to RAN4 for getting exact condition for PRACH tolerance in order to identify whether it is issue or not. |
| CATT | It is better to send the LS to RAN4 and get the exact timing tolerance. |
| HONOR | Agree with sending LS to RAN4 |
| Xiaomi | We don’t think Te is necessarily reported, this can be defined in RAN4 spec TS 38.133 as cited in our contribution. |
| Sony | Agree with RAN4 LS. Second point looks incorrect, we think it should be “Differential Round Trip Doppler”. |
| Toyota | We support the general direction of analytical formulation for PRACH capability. Agree with Samsung that we need input from RAN4 on our assumptions. |
| Tejas Networks | Agree with the proposal |
| Panasonic | We generally support. “Differential Round Trip Delay” in the 2nd sub-bullet would be typo of Differential Round Trip Doppler”? Differential RTT and differential Round Trip Doppler should be clearly defined. Differential RTT should be (Max RTT in the uncertainty area) – (min RTT in the uncertainty area). i.e. not difference to the RTT of the reference location. Similarly, Differential Round Trip Doppler (RTD) should be (Max RTD in the uncertainty area) – (min RTD in the uncertainty area). |
| TCL | Agree |
| ZTE | GP may not need to be considered as short format may have zero GP. In second bullet is seems to be “differential round trip Doppler” instead of “differential round trip delay”. Moreover, the definition of ΔfRA is not clear. If it refers to SCS, the right part seems to beγΔfRA and γ=1 for unrestricted set? |
| Sharp | RAN4 input maybe needed. Different tolerance may be considered for different preamble formats and RO configurations. |
| ST Engineering iDirect | Agree with this proposal. We don’t think it is necessary to send an LS to RAN4 at this point. Note that different companies used different abbreviations in their tdocs. Qualcomm defined RTD as Round-trip Doppler, so the second sub-bullet needs to be corrected. For completeness, propose to define that ΔfRA refers to the SCS |
| LGE | We believe that aligning understanding of differential delay/Doppler should be a priority. If the differential delay/Doppler is defined as the min-max differential delay/Doppler within a certain area, the above proposal can be further discussed. In such a case, we have the following two comments. First, in our understanding, TO tolerance is derived assuming a single FFT window, and GP does not seem to need to be considered. Second, regarding Doppler, it's unclear why Differential Round Trip Doppler is considered. Even in a GNSS resilient scenario, we believe that the UE must meet the 0.1 ppm frequency error requirement in the downlink. Therefore, we understand that Doppler only affects the uplink in a "one-way" manner. Furthermore, it's unclear why SCS/2 is added to the right-hand side of the inequality for restricted sets A/B. |
| Futurewei | Could the proponent explain the rationale behind γ=4 and γ=2? RAN1 agree on a common approach to calculate differential RTT and differential RTD. |
| Nokia | To our understanding we should not consider N_CP in its pure form. First, in case there is differential RTT with respect to a reference point, there may positive and negative values, meaning that N_CP may not be able to cover this (far from). Hence, we should use N_CP/4 should be more useful to consider to allow for additional inaccuracies and potential multipath). There is a typo for the differential doppler (the text says differential round trip delay, while the equation refers to frequency). It may be beneficial to ensure that the “differential aspect to consider” is located on the same side of each equation. Also, we should refrain from using existing terminology (like T_e). |
| Ericsson | First equation: - Additional timing error Te in both directions (positive and negative) should be considered  Te should be 2Te. - Clarify that L is the ZC sequence length - Reference to Table 6.3.3.1-5 should be added (for ΔfRA=1.25 kHz). Second equation: - Differential Round Trip Delay should be Differential Round Trip Doppler - Differential Round Trip Doppler is twice the max residual frequency offset. Therefore, the fefc term should also be doubled. For the same reason, the first term on the right hand side should be ΔfRA. - No need to for ¦.¦ since the value is always positive. - Clarify that fc is the carrier frequency - In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively (corresponding to γ=1 and γ=3). For PRACH performance evaluation for existing PRACH formats based on analytical characterization, adopt the following criteria: - PRACH tolerance is exceeded if - min(Ncp, GP, Sequence duration×maxNcsL) -2Te ≤ Differential RTT, where; - Ncp is cyclic prefix duration - GP: Guard period - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211 - L is the ZC sequence length - Te is an additional timing error to be reported. - Note: the delay spread is neglected - Or Differential Round Trip Doppler+2fefc≥1+γΔfRA - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0 - fe=0.1 ppm - fc is the carrier frequency - Note: In practice, γ=1 and γ=3 for restricted sets A and B, respectively, should be assumed to avoid performance degradation. |
| OPPO | We agree with Panasonic that it should be “Differential Round Trip Doppler” in the 2nd sub-bullet. In addition, we suggest the following modification on the 2nd sub-bullet for better formulation: - Or Differential Round Trip Delay+fefc≥ΔfRA2+γΔfRA - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0.5 - fe=0.1 ppm |
| Apple | Just to clarify, here the differential RTT means the maximum differential RTT? |
| Spreadtrum | Whether differential RTT is maximum differential RTT need clarification. |
| Huawei, HiSilicon | We think the PRACH tolerance should include tolerance to both timing error and frequency error instead of selecting one between two. The preamble still can’t work if one aspect is satisfied. For differential RTT, the GP may not needed as it can be resolved by some scheduling restriction after RO occupying the full slot. For the 2nd equation, it should be differential round trip doppler values. To align the expressions, we recommend to use absolute values for both differential RTT and round trip doppler and then set γ=1 for restricted set A, γ=2for restricted set B. min(Ncp, GP, Sequence duration×maxNcsL) -Te ≤ ¦Differential RTT¦ Differential Round Trip Delay+fefc≥ΔfRA2+γΔfRA∨ - For restricted set A, γ=1. For restricted set B, γ=2. For non-restricted set, γ=0 - fe=0.1 ppm |
| NTT DOCOMO | Agree to send LS to RAN4. As the PRACH tolerance may exceed the Te/fe requirements for NTN, RAN4 clarification is needed. |

### Proposal 1-3

Based on companies’ inputs, the following initial proposal is made:

Proposal 1-3-v0

For the evaluation of frequency and timing adjustment methods, RAN1 will consider the following evaluation methodology: - Step 1: Drop the UE at a location where the satellite is visible at an elevation angle θ - Step 2: Propagate the satellite during the simulation time T (assuming a circular trajectory around the center of Earth) - Step 3: The UE applies an initial timing and frequency offset (randomly distributed within Te and fe, which are the residual timing and frequency errors after successful completion of random access) and updates the timing and frequency adjustments at time t based on a correction method - Step 4: The network evaluates the timing and frequency error after every TUL ms, assuming an uplink transmission every TUL ms - Step 5: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively - Using the steps mentioned above, RAN1 to evaluate timing and frequency control using the following metric: Number of TA and FA commands issued within T.

Companies are encouraged to share views on Proposal 1-3-v0

| Company | Comments |
| --- | --- |
| Samsung | Not support. This is optional evaluation. |
| CATT | Not needed. |
| HONOR | Not support. |
| Xiaomi | Why do we need this ? |
| Sony | Essentially support, but agree it may not be necessary. |
| Toyota | Not needed. |
| Tejas Networks | Not support |
| JHUAPL | This should be an optional simulation detail. |
| Panasonic | The intention is to decide if the overhead for issuing TA and FA commands may become too excessive? |
| TCL | We think is not necessary. |
| ZTE | Fine with the proposal. |
| Sharp | No need for this. |
| ST Engineering iDirect | Support the proposal to specify the evaluation methodology, to make it easier to compare simulation results from different companies, but the proposal should also consider UE mobility. Suggestion to modify step 1 as follows: UE follows a pre-defined trajectory with a certain speed and acceleration - The satellite is visible during the full trajectory - Elevation angle θ For initial simulations, a horizontal trajectory can be assumed |
| LGE | It is too early to discuss the evaluation methodology. We think that the need for a study on TAC/FAC enhancement and the consensus of RAN1 on evaluation should be discussed first. |
| Futurewei | Ok, to discuss |
| Nokia | We do not think it is very crucial to outline very specific details for the evaluations. It would be sufficient for companies to provide their evaluation methodology. |
| Ericsson | Ok. The size of the TA and FA commands might also be considered as metric. |
| OPPO | We also think the proposal is not needed. |
| Apple | It’s not really needed. if evaluated, UE speed needs to be considered. |
| NEC | We support to discuss the simulation details for closed-loop enhancement. |
| Huawei, HiSilicon | Generally fine with the steps. How to perform step 4, based on simulation or calculation? What is assumption of UE velocity? |
| NTT DOCOMO | Not needed. |
| Lenovo | Agree in principle with the steps, but we agree with others that we don’t need this. |

### Proposal 1-4

As Samsung proposed in R1-2507251, the following geometry model and the equations can be considered for the calculation of differential one-way delay and differential Doppler in the scenarios/cases under study.

Proposal 1-4-v0

For the calculation of differential one-way delay and differential Doppler, the following geometry model and the equations (1) through (11) are considered and will be captured in the TR 38.742. For the variables in the equations, refer to Figure below. d=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα; (1). φ1=φ-RcRE; (2). dref=RE2+RE+hs2-2RERE+hscosφ1; (3). φ=sin-1dRE+hscosα; (4). β=π2-α+φ; (5). β1=sin-1REdrefsinφ1; (6). γ=cos-1RE+hsdrefsinφ1; (7). fdest=vssinβ+vuecosαcfc; (8). fdref=vssinβ1cfc; (9). ∆t=d-drefc; (10). ∆fd=fdref-fdest; (11). In equations above, fdest is Doppler frequency if the UE has the perfect knowledge of both its location and speed. fdref is Doppler frequency calculated by the UE assuming the reference point and zero speed for the UE. ∆t is differential delay and ∆fd is differential Doppler Geometry model for the evaluations

Companies are encouraged to provide comments on Proposal 1-4-v0:

| Company | Comments |
| --- | --- |
| Samsung | Support as different companies have different assumption. Thus, it is better to have the common understanding on analytical analysis. |
| CATT | Support it |
| HONOR | Support it. |
| Xiaomi | We support capturing some common analytical analysis, but we believe the methodology listed in our paper for Differential Doppler computation as copied below can also be considered, we are open to further discussion. fd=fcc∙vradψ,λ=fcc∙v∙RsinψcosλR2+rs2-2Rrssinψ Figure A 1 Illustration of the elevation angle and central angle |
| Sony | Agree to defining unified geometry. Can discuss. |
| Toyota | Companies can report their analytical framework used for calculations. Can discuss if needed to agree on a common approach. |
| Tejas Networks | Support the proposal |
| JHUAPL | This is valid only when the LEO velocity vector is in the same plane as the center of the Earth, locations of UE and the satellite. This assumption should be stated clearly. |
| Panasonic | This calculation shows differential one-way delay/Doppler. But we are interested in “Differential Round Trip Delay” and “Differential Round Trip Doppler”. In our understanding, there can be a factor 4 between one-way and Round Trip values. We’d propose to add equations for computing Round Trip values. |
| ZTE | The model seems only consider the case where footprint is within orbital plane. The case where footprint is 90 degree with respect to orbital plane is not considered and hence not support the proposal. |
| Sharp | Support it. |
| Qualcomm | UE may not be located at the orbital plane of satellite. Agree with Xiaomi's formulation, we need to consider location uncertainty area centered around a point ψ,λ. As we have indicated in our paper, max differential RTD is observed for the uncertainty areas located at λ=90o. Also, the formula, if at all required, needs to be defined for a given location. No need to mention “reference location” since the definition of differential RTT, RTD does not depend on reference location. |
| ST Engineering iDirect | Support |
| LGE | We are supportive to have aligned evaluation assumptions especially for the analysis of differential delay and Doppler. In this sense, this proposal is one of examples as a method of calculating differential delay and Doppler. However, it can be applied for limited cases (e.g., along track). We are open to discuss its details or defining other methods. |
| Futurewei | Could the set of Doppler equations apply to a beam at 90° with respect to orbital plane and UE moving in any direction? The set of equations assume the beam center as a reference location? |
| Nokia | We do not think this is very beneficial compared to using the general models from TR38.821. |
| Ericsson | The equations and figure are to our understanding correct and we support their inclusion in the TR. A few comments: - Regarding the definition of differential delay, our view is that it is the max delay difference between any two points in a cell, not the difference between the reference location and a location at the cell edge (and similarly for differential Doppler). Could different terms be used for ∆t and ∆fd? - The reference location is assumed to be on the Earth surface, which is not a necessary restriction. - The use of φ and ϕ should be aligned (φ is used in the equations while ϕ is used in the figure). |
| Apple | Support. It’s important to align the understandings on differential delay and differential Doppler among the companies. |
| ETRI | The above equations are fine, as they just capture a geometry model. However, we think the following assumptions on the beam size of non-nadir beams should be further clarified. In this regard, we propose to add the following assumption: β-β1=3dBbeamwidth2 . Determining the beam size of non-nadir beams based on a fixed beam angle is more practical than using a fixed beam size (from our understanding, there are no commercial deployment examples for this fixed beam size for all satellite beams..). Because the evaluation results are primarily affected by the beam shape and size, the assumptions of the geometry model should be clearly defined. |
| NEC | Support and update the model with the capability to describe a UE which is not located at the orbital plane of satellite. |
| Huawei, Hisilicon | We think the differential delay and Doppler should be calculated in round trip |
| NTT DOCOMO | Support. |
| CMCC2 | Currently, there are certain discrepancies in scenario modeling among different companies, leading to variations in simulation results. These differences may impact the assessment of issues such as the usability of the current PRACH format. Therefore, it is necessary to establish a simulation modeling approach that is agreed upon by all companies. The current modeling approach only considers the two-dimensional plane. The goal of our simulation is to determine the range, or the maximum value, of time and frequency differential. Thus, modeling UE mobility speed as currently done is acceptable (velocity components in other directions relative to the UE-satellite vector would only be smaller). For three-dimensional scenarios, such as a beam at 90° with respect to the orbital plane, companies can first perform simple calculations to check whether the time and frequency offset range in such cases exceeds that of the current modeling approach. If not, there is no need to complicate the modeling. The current calculation of time and frequency offsets only considers the one-way direction. Regarding the question raised by some companies about whether round-trip effects need to be considered, we believe this can be addressed by applying a factor to the calculation results. Therefore, the current one-way modeling is sufficient. |

# Topic#2 Evaluation of differential delay and Doppler

## Background

## Companies’ proposals

### Scenario 1

For scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link.

| Companies | Proposals |
| --- | --- |
| Futurewei | Observation 1: For Case b of the GNSS-resilient evaluation, the maximum radius (X) of location uncertainty circle is equal to the radius of a satellite beam for different deployment scenarios. Observation 2: For the LEO-600, LEO-1200 and GSO scenarios, the differential round-trip Doppler shift increases with increasing elevation angles and X. Observation 3: For the LEO-600, LEO-1200 and GSO scenarios, the differential round-trip Doppler shift reaches the maximum at elevation angle 90° and maximum X (i.e., X=rbeam). Observation 4: For Case 2, the differential round-trip Doppler shift is higher than Case 1 for all X values and elevation angles (except at elevation angle = 90°). Observation 5: For the LEO-600, LEO-1200 and GSO scenarios, differential round-trip delay increases as X becoming larger and elevation angles becoming smaller. Observation 6: For the same value of X and elevation angles, there is no significant difference in differential round-trip delays between the LEO-600, LEO-1200 and GSO scenarios. Observation 7: For the LEO-600, LEO-1200 and GSO scenarios, differential round-trip delay reaches the maximum at the minimum elevation angle (i.e., 30° for LEO-600 and LEO-1200, and 12.5°for GSO) and maximum X. |
| Ericsson | Observation 5 For max differential Doppler, the highest values for footprint case 1 can be observed for the nadir beam and the edge beam perpendicular to the orbital plane. The same max differential Doppler is observed for footprint case 2 but only for the nadir beam. Observation 6 The max differential roundtrip Doppler for LEO and Set-1 is up to 4.2 ppm. Observation 7 The max differential roundtrip Doppler for LEO and Set-2 is up to 7.7 ppm. Observation 8 For max differential delay, the largest values can be observed for the edge beams. Observation 9 The max differential roundtrip delay for LEO and Set-1 is up to 1.4 ms assuming fixed angle beams and up to 0.5 ms assuming fixed size beams. Observation 10 The max differential roundtrip delay for LEO and Set-2 is up to 2.6 ms assuming fixed angle beams and up to 1.1 ms assuming fixed size beams. Observation 11 The max differential roundtrip delay for GSO and Set-1 is up to 6.1 ms assuming fixed angle beams and up to 1.6 ms assuming fixed size beams. Observation 12 The max differential roundtrip delay for GSO and Set-2 is up to 9.3 ms assuming fixed angle beams and up to 2.9 ms assuming fixed size beams. |
| Thales | Observation 2: If the UE uses the serving cell’s reference location rather than its actual position, and pre-compensates the common round trip delay based on this reference location, there will still be a differential delay that must be adjusted by the network (closed loop TA control) and/or the UE (open loop TA control). Observation 3: If the UE uses the serving cell’s reference location rather than its actual position, a residual frequency error may affect the calculated Doppler on the service link and, consequently, the performance of uplink frequency synchronization. Observation 4: In the absence of a GNSS fix, if the UE sets its position to the center of the NR cell (or NR beam), rather than its actual location, the maximum differential one-way delay and frequency offset are as follows: LEO 600km; LEO 1200km; GEO. dT (ms); dF (kHz); dF/dt (Hz/s); dT (ms); dF (kHz); dF/dt (Hz/s); dT (ms); dF (kHz); dF/dt (Hz/s). S Band; 0.0822; 2.102; 21.78; 0.1478; 1.815; 8.73; 0.4100; 0.072; 0.00. Ka Band; 0.0822; 21.023; 217.77; 0.1478; 18.147; 87.29; 0.4100; 0.72; 0.05. Proposal 4: In the absence of a GNSS fix, if the UE sets its position to the center of the NR cell (or NR beam), rather than its actual location, RAN1 to evaluate: - Differential delay scenarios in relevant deployment environments - Differential frequency offset/Doppler - Identification of typical and worst-case for both differential delay and Doppler. |
| vivo | Observation 2: In FR 1-NTN, for LEO 600km, the differential delay and frequency offset can be up to 288us and 4.2kHz; for LEO 1200km, the differential delay and frequency offset can be up to 520us and 3.63kHz; for GSO, the differential delay and frequency offset can be up to 1444us and 0.14kHz. Observation 3: In FR 2-NTN, for LEO 600km, the differential delay and frequency offset can be up to 115us and 25.22kHz; for LEO 1200km, the differential delay and frequency offset can be up to 231us and 24.2kHz; for GSO, the differential delay and frequency offset can be up to 636us and 0.95kHz. |
| Huawei | Observation 2: In FR1-NTN, when UE cannot rely on its GNSS for timing and frequency compensation on the service link and the horizontal location uncertainty area is the area served by the cell or beam (scenario 1 + case a), - for 1 footprint in a cell, the differential timing error and frequency error can be as large as 144us and 2.1ppm under LEO600, 260us and 1.81ppm under LEO1200, 818us and 0.03ppm under GEO; - for 4SSB per cell with 1 SSB covering 1 footprint, the differential timing error and frequency error in a cell can be as large as 359us and 5.23ppm under LEO600, 646us and 4.50ppm under LEO1200, 2036us and 0.03ppm under GEO; - for 4SSB per cell with 1 SSB covering 16 footprints, the differential timing error and frequency error in a cell can be as large as 1297us and 18.23ppm under LEO600, 2316us and 15.89ppm under LEO1200, 7481us and 0.03ppm under GEO. Observation 3: In FR2-NTN, when UE cannot rely on its GNSS for timing and frequency compensation on the service link and the horizontal location uncertainty area is the area served by the cell or beam (scenario 1 + case a), - for 1 footprint in a cell, the differential timing error and frequency error can be as large as 57.60us and 0.84ppm under LEO600, 115.36us and 0.81ppm under LEO1200, 357.39us and 0.03ppm under GEO; - for 64 SSB per cell with 1 SSB covering 1 footprint, the differential timing error and frequency error in a cell can be as large as 529us and 7.74ppm under LEO600, 1060us and 7.46ppm under LEO1200, 7482us and 0.03ppm under GEO. Observation 4: When UE cannot rely on its GNSS for timing and frequency compensation on the service link and the horizontal location uncertainty area is the area served by the cell or beam (scenario 1 + case a), - the differential delay in edge beam is much larger than that in nadir beam; - the differential doppler value in nadir beam is larger than that in edge beam across the orbit plane and then larger than that in edge beam along the orbit plane; - the larger the location uncertainty area is, the larger the differential delay and doppler values are. Observation 5: The time and frequency error in GNSS resilient scenario 1 far exceed the maximum tolerance of existing PRACH preamble formats. |
| Xiaomi | Proposal 3: Adopt the Differential delay and Doppler values in table 2 of R1-2506987 for further evaluation performance analysis including the performance evaluation of PRACH. |
| CMCC | Observation 2: The required CP lengths and SCSs (with restricted set and without restricted set) under LEO-600, LEO-1200, GEO with different beam footprint sizes are evaluated in Annex II of R1-2507012. |
| CATT | Observation 5: In Scenario 1, the maximum TA error is 812us, 258us, and 143us for GSO, LEO-1200, and LEO-600 respectively in L/S frequency band. The timing tolerance of existing PRACH format cannot meet the requirements of above TA error. The detals of timing errors for different orbits and frequency bands are shown in Table 3 of R1-2507117. Observation 6: In Scenario 1, the maximum FO error is 2711Hz, 2637Hz, and 2789.1Hz for GSO, LEO-1200, and LEO-600 respectively in L/S frequency band. The frequency offset tolerance of existing PRACH format except for format 0/1/2 can meet the requirements of above FO error. The detals of frequency offset errors for different orbits and frequency bands are shown in Table 3 of R1-2507117. |
| OPPO | Observation 2: The following observations are made for differential delay evaluation: - The maximal differential delay decreases as the elevation angle increases for each satellite scenario. - The GEO set 2 shows the worst performance in all the evaluated scenarios, with the maximal differential delay up to about 0.7ms and 0.45ms for 2 GHz and 30 GHz respectively. Observation 3: The following observations are made for differential doppler shift evaluation: - The maximal differential doppler shift increases as the elevation angle increases for each satellite scenario. - The LEO-600 set 2 shows the worst performance in all the evaluated scenarios, with the maximal differential doppler shift up to 3.77kHz and 31.47kHz for 2 GHz and 30 GHz respectively. |
| Tejas Networks | Observation 1: In scenarios where GNSS support is unavailable the UE's ability to perform UE-specific Timing Advance (TA) calculations is compromised and the UE can only apply the network-broadcasted common TA and is unable to perform UE-specific Doppler correction prior to transmitting the Msg1/MsgA PRACH preamble. Observation 6 : In aircraft scenarios for higher elevation angles, assuming the common TA reference point within the beam in the ground leads to negative values of differential delay which may not be detected by the existing PRACH preambles. Proposal 4: FFS on the location of the common TA reference point for aircraft scenarios under elevation angles closer to 90 degrees |
| Samsung | Observation 1: For LEO-600 2GHz, maximum differential delay is 71.8 us. Observation 2: For LEO-600 2GHz, maximum differential Doppler is 2107 Hz. Observation 3: For LEO-1200 2GHz, maximum differential delay is 129.2 us. Observation 4: For LEO-1200 2GHz, maximum differential Doppler is 1821 Hz. Observation 5: For GSO 2GHz, maximum differential delay is 405.8 us. Observation 6: For GSO 2GHz, maximum differential Doppler is 134 Hz, which happens for aircraft travelling at 1500 km/h. Observation 7: For GSO 14GHz, maximum differential delay is 178.8 us. Observation 8: For GSO 14GHz, maximum differential Doppler is 413 Hz, which happens for aircraft travelling at 1500 km/h. |
| ETRI | Observation 2: When the location uncertainty area is the area served by the beam, the maximum differential position (dP), delay (dT), and the differential Doppler values for Set-1 in Table 6.1.1.1-1 TR 38.821 are as follows: LEO 600km; LEO 1200km. dP (km); dT (ms); dP (km); dT (ms). BP 1; BP 2; BP 1; BP 2; BP 1; BP 2; BP 1; BP 2. 2 GHz; 25; 164.3784; 0.0720; 0.4872; 45; 269.7434; 0.1296; 0.8011. 30 GHz; 10; 58.16; 0.0288; 0.1697; 20; 96.09; 0.0577; 0.2806. dF (kHz). LEO 600km; LEO 1200km. 0 km/h; 3 km/h; 120 km/h; 0 km/h; 3 km/h; 120 km/h. BP 1; BP 2; BP 1; BP 2; BP 1; BP 2; BP 1; BP 2; BP 1; BP 2; BP 1; BP 2. 2 GHz; 2.1; 1.94; 2.1; 1.94; 2.09; 1.93; 1.81; 1.86; 1.81; 1.86; 1.81; 1.85. 30 GHz; 12.61; 11.65; 12.61; 11.65; 12.55; 11.6; 12.1; 11.18; 12.1; 11.18; 12.04; 11.13. Proposal 5. For the location uncertainty area defined as a circle of radius X km, X=165 km for Set-1 LEO 600km X=270 km for Set-1 LEO 1200km Proposal 6. RAN1 to consider the following values of maximum differential delay (dT), dT=0.49 ms for Set-1 LEO 600km dT=0.80 ms for Set-1 LEO 1200km |
| CCU | Observation 1: The maximum differential delay from the beam center to the beam edge is 0.2638 ms and 0.4982 ms for LEO-600 and LEO-1200 for Case a. Observation 2: The maximum differential Doppler from the beam center to the beam edge is 0.9712 ppm and 0.9320 ppm for LEO-600 and LEO-1200 with UE speed 3 km/h for Case a. Observation 3: The maximum differential Doppler from the beam center to the beam edge is 0.9758 ppm and 0.9370 ppm for LEO-600 and LEO-1200 with UE speed 120 km/h for Case a. Observation 4: The maximum differential delay from the beam center to the beam edge is 0.0723 ms and 0.13435 ms for LEO-600 and LEO-1200 for Case b (radius 25, 45 km). Observation 5: The maximum differential Doppler from the beam center to the beam edge is 1.0502 ppm and 0.9071 ppm for LEO-600 and LEO-1200 with UE speed 3 km/h for Case b (radius 25, 45 km). Observation 6: The maximum differential Doppler from the beam center to the beam edge is 1.0551 ppm and 0.9119 ppm for LEO-600 and LEO-1200 with UE speed 120 km/h for Case b (radius 25, 45 km). |
| Sony | Observation 3: For LEO-600 satellites operating at 2 GHz carrier frequency with a 25 km beam radius, the maximum differential Doppler is 2.1 kHz. Observation 4: For LEO-600 satellites operating at 2 GHz carrier frequency, elevation angles ≤ 22.5° can be compensated using a beam-center reference location. Observation 5: For LEO-600 satellites operating at 2 GHz carrier frequency with a 25 km beam radius, the maximum differential delay is 71 μs. |
| MediaTek | Observation 1: The frequency error and timing error increase with the GNSS UE position error depending on the UE velocity and the GNSS UE position measurement periodicity. Observation 2: At the beam edge at lowest elevation with angle β = 30 degrees, the timing error is 43.34 µs and 11.55 µs assuming a GNSS UE position error range of 7.5 km and 2 km respectively. Observation 3: At nadir, the frequency error is 0,630 ppm and 0.755 ppm assuming a GNSS UE position error range of 7.5 km with a UE velocity 3 km/h and 1000 km/h respectively. |
| Johns Hopkins University | Observation 1: In general, the satellite beam footprint is significantly larger at lower elevation angles. Observation 2: The rate of change of the timing advance can be used to compute the Doppler frequency at the UE. Proposal 1: RAN1 should study supporting initial access to low-elevation angle scenarios where the differential delay is higher than in high-elevation angle scenarios. |
| CEWiT | Observation 1: For Reference-Point based TA computation in LEO at 600 km with a beam radius of 25 km, the maximum timing error is 82.08 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 1.040 ppm, observed at an elevation angle of 90°. Observation 2: For Reference-Point based TA computation in LEO at 1200 km with a beam radius of 45 km, the maximum timing error is 147.75 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 0.936 ppm, observed at an elevation angle of 90°. Observation 3: For Reference-Point based TA computation in GSO at 35786 km with a beam radius of 125 km, the maximum timing error is 410.35 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 0.087 ppm, observed at an elevation angle of 90°. |

### Scenario 2

For Scenario 2: There is a previously acquired GNSS based position

| Companies | Proposals |
| --- | --- |
| vivo | Observation 4: If the positioning accuracy is 10km, only UL transmission in GSO at the inclination of 90 can satisfy the timing error and frequency error requirements in RAN 4. In other scenarios, the requirements cannot be satisfied. Observation 5: If the timing error due to UE moving is within [-29*64*Tc, 29*64*Tc], the GNSS validity period of a UE can be longer than 300s for a pedestrian UE, and as short as ~0.7s for a UE with velocity of 1500 km/h. Observation 6: If the timing error due to UE moving is within [-29*64*Tc, 29*64*Tc], the valid GNSS error can be up to ~300m. Observation 7: If the frequency error due to UE moving within [-0.1ppm, 0.1ppm], the GNSS validity period of a UE can be longer than 1000s for a pedestrian UE, but as short as ~5s for a UE with velocity of 1500 km/h. Observation 8: If the frequency error due to UE moving within [-0.1ppm, 0.1ppm], the maximum GNSS error X km can be up to ~2.24km for a UE with velocity of 1500km/h. Observation 9: If the maximum uncertainty area is a circle of radius of 25km, the maximum differential timing delay is up to 162.8 us, and the maximum differential frequency offset is up to 2.1ppm (i.e., 4.2 kHz in FR 1 and 63 kHz in FR 2). Observation 10: If compensation is based on UE location, the UE position accuracy may need to be 0.1km. |
| ZTE | Observation 3: The maximum round trip differential delay and differential Doppler are as follows: Case; S-band; Ka-band. Max differential delay (us); Max differential Doppler (kHz); Max differential delay (us); Max differential Doppler (kHz). LEO-600, Case a; 1044.978; 8.392; 414.464; 50.404. LEO-600, Case b, X=1km; 11.548; 0.336; 11.548; 5.04. LEO-600, Case b, X=5km; 57.734; 1.68; 57.734; 25.204. LEO-600, Case b, X=25km; 288.648; 8.392; 288.648; 125.908. LEO-1200, Case a; 1751.042; 7.252; 771.096; 50.404. LEO-1200, Case b, X=1km; 11.546; 0.16; 11.546; 2.42. LEO-1200, Case b, X=5km; 57.734; 0.808; 57.734; 12.092. LEO-1200, Case b, X=25km; 288.664; 4.032; 288.664; 60.452. GSO, Case a; 10768.444; /; 3823.138; /. GSO, Case b, X=1km; 13.016; /; 13.016; /. GSO, Case b, X=5km; 65.086; /; 65.086; /. GSO, Case b, X=25km; 325.432; /; 325.432; /. |
| Huawei | Observation 6: In scenario 2 + case b, when the elevation angle of a previously acquired GNSS based position is 30°, the maximum tolerable location uncertainty X and the time duration T is constrained by the tolerance to timing error of the preamble formats. Observation 7: In scenario 2 + case b, when the elevation angle of a previously acquired GNSS based position is 90°, the maximum tolerable location uncertainty X and the time duration T is constrained by the tolerance to frequency error of the preamble formats. |
| CATT | Observation 8: In scenario 2, it is recommended that the X should not exceed 5 km to meet the timing tolerance of existing PRACH formats. Otherwise, the solution in Scenario 1 can be used for PRACH enhancement. |
| Tejas Networks | Observation 2: For FR1-NTN S band scenarios, the maximum timing errors for Set 1 satellite parameters in case of location error equal to beam diameter are 291.13 us , 524.9 us and 1.46 us for LEO 600 km , LEO 1200 km and GEO respectively. Observation 3: When the user equipment (UE) location error and the satellite elevation angle are constant, the resulting differential delay remains nearly unchanged across different satellite altitudes. Observation 4 : For FR1-NTN, the timing error is upto 1.46 ms, 291 us and 525 us for GEO , LEO 600 km and LEO 1200 km respectively for scenario 2 which are significantly larger than the RAN4 requirement on the timing error limit Observation 5 : In Scenario 2, the worst-case location uncertainty region is equivalent to that of Scenario 1 when the GNSS outage duration T is sufficiently long and the resulting UE location error reaches the beam diameter. Observation 7: In the worst-case scenario, the maximum differential Doppler observed for nadir beams in Scenario 2 is equivalent to that in Scenario 1, provided the GNSS displacement error reaches the beam diameter. Observation 8 : In the worst-case scenario, the maximum differential delay observed for farther beams in Scenario 2 is equivalent to that in Scenario 1, provided the GNSS displacement error reaches the beam diameter. Proposal 5 : The worst-case differential delay and residual Doppler across all elevation angles and beam configurations in Scenario 2 are equivalent to those in Scenario 1, assuming the maximum GNSS displacement error spans the beam diameter. Consequently, any mitigation or compensation strategy developed for Scenario 1 is directly applicable to Scenario 2 as well. |
| NTT DOCOMO | Observation 1: For GNSS position error not larger than 2km, - for LEO-1200km, the maximum timing error is 11.5us, the maximum frequency error is 162Hz for S-band and 2431Hz for Ka-band; - for GSO, the maximum timing error is 13.02us, the maximum frequency error is 2.3Hz for S-band and 34Hz for Ka-band. |

## Summary of companies’ contributions

Several companies provided evaluation results on the differential delay and Doppler in different scenarios. The following is a recap of companies’ inputs:

Differential Delay:

LEO Scenarios:

- Range: Most reported maximum one-way (or round-trip as specified) differential delays in LEO span from a low of about 70–150 μs in small beams at low elevation angles (e.g., CEWiT, Thales, LGE, Samsung, CATT, Panasonic, Futurewei, Nokia, Sony), up to 1–3 ms in larger beams (ZTE, Ericsson, Spreadtrum/UNISOC, Johns Hopkins, Tejas Networks, Toyota). The absolute worst-case values (3+ ms) occur for very large beams or at very low elevation angles, enlarged-area at the edge of the satellite coverage .

- Trends: Differential delays increase with larger beam size, lower satellite elevation angles, and with user position uncertainty.

GEO Scenarios:

- Range: GEO differential delays are generally higher due to increased beam size, with most practical cases reporting between 400 and 1600 μs (Panasonic, CMCC, Thales, Sony, CEWiT, Futurewei, Samsung). Some GEO results, especially for very large beams, can approach 7–10 ms (Ericsson, ZTE, Huawei).

- Trends: Like LEO, the worst case in GEO occurs for beams of hundreds of kilometers at low elevation angles. However, for narrow beams or at higher elevations, the differential delay is considerably less.

Differential Doppler

LEO Scenarios:

- Range: Differential Doppler spans a wide range, from about 1–4 kHz at S-band frequencies (2 GHz) in typical beams (Thales, Xiaomi, Ericsson, Spreadtrum, Sony, Samsung, LGE, ETRI, Qualcomm, CCU, CEWiT), up to 25–50 kHz or more at higher frequencies (14–30 GHz, e.g., Panasonic, ZTE, Vivo, Xiaomi, LGE, Huawei), especially for large beams or high user velocity. At Ka-band, values exceeding 40 kHz are cited (ZTE, Panasonic, Xiaomi, CATT).

- Trends: Doppler spreads increase with higher frequency, greater user velocity, at 90° elevation, and larger beams.

GEO Scenarios:

- Range: GEO maximum Doppler differences are far less severe than LEO, typically below 1 kHz at S-band and only a few kHz at Ka-band (Thales, Panasonic, CMCC, Vivo, LGE, Futurewei, Samsung). Some contributions even consider GEO Doppler negligible (Ericsson, ZTE, OPPO, Sony, Nokia).

- Trends: The combination of nearly stationary satellites (in GEO) and the large slant range ensures limited Doppler spread within a beam, with the main contributor in GEO often being user (not satellite) motion.

The following table provides a detailed recap of companies’ evaluation results:

| Company | Max diff-delay (LEO) | Max diff-delay (GEO) | Max diff-Doppler (LEO) | Max diff-Doppler (GEO) |
| --- | --- | --- | --- | --- |
| Futurewei | ~145 μs (LEO-600, X=25km, 30°) ~261 μs (LEO-1200, X=45km, 30°) up to ~145 μs (LEO-600, 30°) up to ~115 μs (LEO-1200, 70°) | ~815 μs (GSO, X=125km, 12.5°) ~727 μs (GSO, X=125km, 30°) | Up to 4.22 kHz (2 GHz, 120km/h, 90°, LEO-600, X=25km) Up to 29.54 kHz (14 GHz, 120km/h, 90°, LEO-600, X=25km) Up to 27.19 kHz (30 GHz, 1500km/h, 90°, LEO-600, X=10km) | Up to ~10.3 kHz (2 GHz, 120km/h, 90°, GSO, X=125km) Up to ~71.9 kHz (14 GHz, 120km/h, 90°, GSO, X=125km) Up to ~1 kHz (30 GHz, 1500km/h, 90°, GSO, X=65km) |
| Ericsson | Up to 2.6 ms (Set-2, S-Band, fixed angle) Up to 1.1 ms (Set-2, S-Band, fixed size) Up to 1.4 ms (Set-1, S-Band, fixed angle) Up to 0.5 ms (Set-1, S-Band, fixed size) | Up to 9.3 ms (Set-2, S-Band, fixed angle) Up to 2.9 ms (Set-2, S-Band, fixed size) Up to 6.1 ms (Set-1, S-Band, fixed angle) Up to 1.6 ms (Set-1, S-Band, fixed size) | Up to 7.7 ppm (Set-2, S-Band) Up to 4.2 ppm (Set-1, S-Band) Up to 4.2 ppm (Set-2, Ka-Band) Up to 1.7 ppm (Set-1, Ka-Band) | Near 0 ppm (GEO, all bands) (Doppler due to satellite is negligible) + UE velocity can add up to 5.6 ppm (1500 km/h) |
| Spreadtrum, UNISOC | Set-1 S-band: 0.14 ms (30°, LEO-600), 0.26 ms (30°, LEO-1200) Set-2 S-band: 0.26 ms (30°, LEO-600), 0.54 ms (30°, LEO-1200) Set-1 Ka-band: 0.06 ms (LEO-600), 0.12 ms (LEO-1200) Set-2 Ka-band: 0.14 ms (LEO-600), 0.30 ms (LEO-1200) | Set-1 S-band: 0.58 ms (30°), 0.65 ms (12.5°), up to 1.3 ms (Set-2, 30°), 1.46 ms (12.5°) Ka-band: 0.32 ms (30°), 0.36 ms (12.5°), up to 0.81 ms (Set-2, 30°), 0.91 ms (12.5°) | Set-1 S-band: 2.1 kHz (LEO-600), 1.82 kHz (LEO-1200) Set-2 S-band: 3.76 kHz (LEO-600), 3.82 kHz (LEO-1200) Set-1 Ka-band: 12.6 kHz (LEO-600), 12 kHz (LEO-1200) Set-2 Ka-band: 31.5 kHz (LEO-600), 27.3 kHz (LEO-1200) | Not explicit (GEO device Doppler is low/omitted) |
| Thales | 82 μs (LEO-600, S/Ka, 8°); 148 μs (LEO-1200, S/Ka, 8°) | 410 μs (S/Ka, 8°) | S-band: up to 2.10 kHz (slant), 4.20 kHz UL equivalent; Ka-band: up to 21.02 kHz (LEO-600), 18.15 kHz (LEO-1200); drift up to ~218 Hz/s (Ka, LEO-600) | S-band: ~72 Hz (slant), 144 Hz UL equivalent; Ka-band: ~0.72 kHz |
| vivo | S-band (FR1, full beam/case a): 0.288 ms (LEO-600, 30°), 0.520 ms (LEO-1200, 30°); Ka-band (FR2): 0.115 ms (LEO-600 , 30°), 0.231 ms (LEO-1200, 30°) | S-band: 1.444 ms (30°); Ka-band: 0.636 ms (30°) | FR1: up to 4.2 kHz (LEO-600, 90°), 3.63 kHz (LEO-1200, 90°); FR2: up to 25.22 kHz (LEO-600, 90°), 24.2 kHz (LEO-1200, 90°). Case b (X=25 km): up to ~63 kHz (FR2) | FR1: ~0.14 kHz (90°); FR2: ~0.95 kHz (90°) |
| ZTE | Round-trip differential delay (case a, worst over elevation): S-band up to 1.751 ms (LEO-1200), 1.045 ms (LEO-600); Ka-band up to 0.771 ms (LEO-1200), 0.414 ms (LEO-600). Case b examples: X=25 km yields up to 0.289 ms (LEO-600/1200, both bands) | Round-trip differential delay (case a): S-band up to 10.768 ms; Ka-band up to 3.823 ms | S-band up to 8.392 kHz (LEO-600, case a), 7.252 kHz (LEO-1200, case a); Ka-band up to 50.404 kHz (LEO-600/1200, case a) | Near 0 / not evaluated (values omitted in contribution; Doppler is negligible compared to LEO) |
| Huawei, HiSilicon | Scenario 1 + case a (differential timing error): FR1 up to 2.316 ms (LEO-1200, 4 SSB, 16 footprints); FR2 up to 1.060 ms (LEO-1200, 64 SSB) | Scenario 1 + case a: up to ~7.48 ms (FR1/FR2, multi-footprint GEO) | Scenario 1 + case a (differential Doppler, ppm): FR1 up to 18.23 ppm (LEO-600, 4 SSB, 16 footprints) ≈ 36.5 kHz @2 GHz; FR2 up to 7.74 ppm (LEO-600, 64 SSB) ≈ 232 kHz @30 GHz | ≈0.03 ppm (≈60 Hz @2 GHz; ≈0.9 kHz @30 GHz) |
| Xiaomi | Up to ≈277 μs (LEO-1200, 190 km beam, 2 GHz; one-way differential delay). Typical examples: 73–131 μs (LEO-600, 50–90 km beams, 2 GHz) | Not reported | FR1 (2 GHz): up to ≈3.82 kHz (LEO-1200, 190 km, 90°); Ku (14 GHz): up to ≈26.7 kHz; FR2 (30 GHz): up to ≈57.3 kHz (LEO-1200, 190 km, 90°) | Not reported |
| CMCC | One-way differential timing error within beam (case a): FR1 S-band up to 519.6 μs (LEO-1200, 45 km radius, 30°); 288.7 μs (LEO-600, 25 km radius, 30°). FR2 Ka up to 230.9 μs (LEO-1200, 20 km radius, 30°); 115.5 μs (LEO-600, 10 km radius, 30°) | FR1 S-band up to 1627.2 μs (GEO, 125 km radius, 12.5°). FR2 Ka up to 715.96 μs (GEO, 55 km radius, 12.5°). Ku (14 GHz) up to 1041.4 μs (GEO, 80 km radius, 12.5°) | FR1 (2 GHz): up to ~2.67 kHz (LEO-600, 30°, 1500 km/h); ~2.10 kHz at 90° (low speeds). FR2 (30 GHz): up to ~37.7 kHz (LEO-600, 30°, 1500 km/h); ~12.6 kHz at 90° | FR1 (2 GHz): up to ~2.71 kHz (12.5°, 1500 km/h); near-zero at 90°. FR2 (30 GHz): up to ~40.7 kHz (12.5°, 1500 km/h); small at low speeds |
| CATT | Scenario 1: LEO-600: Max TA error up to 143 μs (L/S, 50 km beam); LEO-1200: Max TA error up to 258 μs (L/S, 90 km beam); Other bands: 55–112 μs (smaller beams); Scenario 2: Max TA error for uncertainty radius ≤5 km: 27–31 μs | GSO: Max TA error up to 812 μs (L/S, 250 km beam); other bands: 357 μs (Ku, 250 km); 58 μs (Ka, 110 km) | Scenario 1, max FO error (L/S): LEO-600: up to 2789 Hz (1500 km/h UE) LEO-1200: up to 2637 Hz GSO: up to 2711 Hz Other bands/UE speeds: up to 37.6 kHz (Ka, 1500 km/h UE) Scenario 2: Error much lower, see notes | GSO, L/S: up to 2.7 kHz (nominal UE speed); Ku/Ka: up to 40 kHz (fast UE) |
| OPPO | Differential delay within a beam (scenario 1, case a, X = beam radius): at 2 GHz LEO-600 set1 30° ≈144 μs; LEO-1200 set2 30° ≈549 μs; GEO set2 12.5° ≈1.49 ms; at 30 GHz LEO-1200 set2 30° ≈259 μs; GEO set2 12.5° ≈918 μs | At 2 GHz up to 1.49 ms (GEO set2, 12.5°); at 30 GHz up to 0.918 ms (GEO set2, 12.5°) | Differential Doppler: at 2 GHz up to 3.77 kHz (LEO-600, set2, 90°); at 30 GHz up to 31.47 kHz (LEO-600, set2, 90°) | Not evaluated (GSO Doppler assumed negligible) |
| Tejas Networks | S band, LEO 600km: - Beam diameters 50–90 km: – Max diff. delay: 0.29–0.53 ms (typical set-1/2 nadir, at 30° elev.) - For beams with cell edge at low elev. & elongated beams (worst-case): up to 1.04–2.36 ms LEO 1200km: - Beam diameters 90–190 km: – Max diff. delay: 0.52–1.1 ms (nadir/cell edge) Ka band (LEO): - Beam diam. ~20–40 km, max diff. delay 0.076–0.234 ms | S band: - Set-1: Max diff. delay 1.64 ms (250 km beam) - Set-2: 2.94 ms (450 km beam) Ka: - 0.41–0.64 ms | S band (2 GHz): - Residual Doppler (worst-case): – LEO 600km: up to 7.6–8.3 kHz – LEO 1200km: up to 7.7–8.5 kHz - At nadir, up to 4 kHz Ka band: - LEO 600/1200km: up to 35 kHz | Very low (Doppler from UE mov., e.g. 0.44 kHz or less); see note |
| Samsung | 71.8 μs (LEO-600, 2 GHz, 50 km, 3/120 km/h); 129.2 μs (LEO-1200, 2 GHz, 90 km, 3/120 km/h) | 405.8 μs (GSO 2 GHz, 250 km, 120 km/h); 398.6 μs (GSO 2 GHz, 250 km, 1500 km/h); 178.8 μs (GSO 14 GHz, 110 km, 120 km/h); 171.6 μs (GSO 14 GHz, 110 km, 1500 km/h) | 2107 Hz (LEO-600, 2 GHz, 50 km, 120 km/h); 1821 Hz (LEO-1200, 2 GHz, 90 km, 120 km/h) | 134 Hz (GSO 2 GHz, 1500 km/h); 413 Hz (GSO 14 GHz, 1500 km/h) |
| LGE | FR1 (2 GHz): LEO-600 up to 73 μs (30°), 2.39 μs (90°); LEO-1200 up to 131 μs (30°), 3.83 μs (90°) FR2 (30 GHz): LEO-600 up to 29.4 μs (30°), 0.79 μs (90°); LEO-1200 up to 58.4 μs (30°), 1.15 μs (90°) | FR1 (2 GHz): GSO up to 411.8 μs (10°), 5.3 μs (90°) FR2 (30 GHz): GSO up to 181.3 μs (10°), 1.4 μs (90°) | FR1 (2 GHz): LEO-600 up to 2,429 Hz (90°) LEO-1200 up to 2,130 Hz (90°) FR2 (30 GHz): LEO-600 up to 16,382 Hz (90°) LEO-1200 up to 15,872 Hz (90°) | FR1 (2 GHz): up to 282 Hz (90°) FR2 (30 GHz): up to 3,544 Hz (90°) |
| ETRI | LEO-600: up to 0.49 ms (164 km location uncertainty, 2 GHz, edge beam at 30°); LEO-1200: up to 0.80 ms (270 km location uncertainty, 2 GHz, edge beam at 30°); FR2 (30 GHz): up to 0.17 ms (LEO-600), 0.28 ms (LEO-1200) | Not explicitly provided (focus is LEO) | 2 GHz: up to 2.1 kHz (LEO-600), 1.81 kHz (LEO-1200); 30 GHz: up to 12.6 kHz (LEO-600), 12.1 kHz (LEO-1200) | Not evaluated in tables; focus is LEO scenarios |
| Panasonic | Scenario 1 (whole cell): S-band, LEO-600: 286 μs (cell radius 25 km, 30°) S-band, LEO-1200: 514 μs (45 km, 30°) Ka-band, LEO-600: 115 μs (10 km, 30°) Ka-band, LEO-1200: 230 μs (20 km, 30°) Scenario 2 (uncertainty radius): For S-band, LEO-600: up to 286 μs at 25 km uncertainty; LEO-1200: up to 514 μs at 45 km uncertainty; for GEO: up to 1620 μs (cell radius 125 km, 12.5°) or lower for smaller uncertainty | Scenario 1 (whole cell): S-band, GEO: 1620 μs (cell radius 125 km, 12.5°) Ka-band, GEO: 715 μs (55 km, 12.5°) Scenario 2: For S-band: up to 1620 μs at 125 km uncertainty; Ka-band: up to 715 μs at 55 km uncertainty | Scenario 1 (whole cell): S-band, LEO-600: up to 8.4 kHz S-band, LEO-1200: up to 7.3 kHz Ka-band, LEO-600: up to 44.5 kHz Ka-band, LEO-1200: up to 43 kHz Smaller for GEO | S-band, GEO: up to 0.31 kHz Ka-band, GEO: up to 2.99 kHz |
| Nokia | LEO-600 (50 km cell diameter): Max differential TA up to ±145 μs at 30° elev; ±108 μs at 50°; ±56 μs at 70°; ±3.5 μs at 90° | Not explicitly evaluated | LEO-600:Maximum differential round trip Doppler shift ±2.1 kHz at 30° elev; ±3.2 kHz at 50°; ±4.0 kHz at 70°; ±4.2 kHz at 90° | Not explicitly evaluated |
| CCU | Case a (cell/beam area): - LEO-600 (S-band): Max center-edge diff delay = 0.2638 ms - LEO-1200: 0.4982 ms Case b (uncertainty radius 25 km/45 km): - LEO-600: 0.0732 ms - LEO-1200: 0.1344 ms All at 30° elevation | Not explicitly evaluated | Case a (center-to-edge, UE speed 3 km/h): - LEO-600: 0.9712 ppm (1.157 kHz) - LEO-1200: 0.9320 ppm (1.250 kHz) Case a, 120 km/h: - LEO-600: 0.9758 ppm (1.217 kHz) - LEO-1200: 0.9370 ppm (1.302 kHz) Case b (uncertainty 25/45km, 3 km/h): LEO-600: 0.175–0.186 ppm (0.35–0.37 kHz) LEO-1200: 0.19–0.20 ppm (0.37–0.40 kHz) Case b, 120 km/h: slightly higher, up to ~0.42 kHz | Not explicitly evaluated |
| Sony | LEO-600, S-band (2 GHz), 25 km beam radius, 30°-90° elevation: Max differential delay: 71 μs (when using beam center as compensation reference) | Not explicitly evaluated | LEO-600, S-band, 25 km beam: Max differential Doppler: 2.1 kHz (with compensation to beam center) | Not explicitly evaluated |
| MediaTek | LEO-600, S-band, GNSS-based case, β = 30°: Timing error up to 43.34 μs (GNSS error 7.5 km, V_UE 1500 km/h) Timing error up to 11.55 μs (GNSS error 2 km, V_UE 1500 km/h) Timing error scales with GNSS error and velocity | Not explicitly evaluated | LEO-600, S-band: - Max freq error 0.755 ppm (1.51 kHz at 2 GHz) for position error 7.5 km and V_UE 1500 km/h, β=90° - Max freq error 0.63 ppm (1.26 kHz at 2 GHz) at V_UE 3 km/h, GNSS error 7.5 km (Table 2, β=90°) - Scales with position error, velocity, and β angle | Not explicitly evaluated |
| Toyota | LEO-600, 2GHz, cell diameter 200 km: - Single reference location: up to 292 μs (elevation 30°), 183.8 μs (60°), 27.6 μs (90°) - With 5 reference locations: reduced to 98 μs (30°), 64.9 μs (60°), 9.9 μs (90°) - At higher frequency (14 GHz, 90 km): as low as 1–26 μs with multiple references | Not explicitly evaluated | LEO-600, 2GHz, cell diameter 200 km: - Single reference location: up to 8.3 kHz (90° elev.), 6.3 kHz (60°), 1.6 kHz (30°) - With 5 reference locations: reduced to 4.2 kHz (90°), 2.2 kHz (60°), 0.58 kHz (30°) - At higher frequency (14 GHz, 90 km): as low as 1.1–4.6 kHz with multiple references | Not explicitly evaluated |
| Qualcomm | LEO, S-band, Differential RTT (timing error) within uncertainty area: - For 25 km radius at 30° elev.: 289 μs (along track) - For 25 km radius at nadir: 3.47 μs - For 25 km radius at 90° elev.: 3.47 μs - Timing error grows rapidly at low elevation and large uncertainty area (e.g., 115 μs for 10 km, 289 μs for 25 km radius, 30° elev. along track) | Not explicitly evaluated | LEO, Differential RTD (frequency error) within uncertainty area, 2 GHz: - 25 km radius at 30° elev. (cross-track): 4.2 kHz - 25 km radius at 30° elev. (along track): 1.05 kHz - 25 km radius at 90° elev.: 8.4 kHz. - PRACH Format C2 supports up to 7.5 kHz - Format 1 (unrestricted): 0.6 kHz, Format 1(B): 5.6 kHz | Not explicitly evaluated |
| Johns Hopkins University | LEO-600 km: - Max differential delay (worst-case, large beam, low elev.): up to 2.75 ms (e.g., footprint ~850 km, low elev.) - Smaller beam, high elev. (e.g., ~110–440 km diameter): differential delay ~148 μs–148 μs - TR 38.821 (Table 4.2-2): max values 3.12 ms (600 km), 3.18 ms (1200 km) | Not explicitly evaluated | LEO-600 km: - Doppler variation: up to 12 kHz (low elev., very large beam) - 9–34 kHz across a large beam (110–440 km diameter) - 0.27 ppm/s max Doppler drift (TR 38.821) | Not explicitly evaluated |
| CSCN | Differential delay: - For 25 km beam radius, LEO-600 km: • Differential delays depend on RP position, beam geometry, UE altitude. up to ~0.15 ms (for Case 3: Edge beam footprint , where RP located at the beam edge near the satellite). | Not explicitly evaluated | Differential FO: - After RP-based compensation in LEO-600 km: up to 2.1 kHz (for Case 1: Nadir beam footprint, where RP located at beam center) | Not explicitly evaluated |
| CEWiT | Reference-point TA computation (UE assumes beam/cell centre): - LEO-600 km, beam radius 25 km: Max timing error = 82.08 μs (at 10° elev.) - LEO-1200 km, beam radius 45 km: Max timing error = 147.75 μs | GSO-35786 km, beam radius 125 km: Max timing error = 410.35 μs - Max allowable UL timing error per NR spec (15 kHz SCS): 0.944 μs | Reference-point frequency error: - LEO-600 km: Max frequency error = 1.040 ppm - LEO-1200 km: Max frequency error = 0.936 ppm | GSO, 125 km radius: Max frequency error = 0.087 ppm |

## Initial proposal

### Proposal 2-1

The above summary shows substantial consensus on the physics: differential delay and Doppler scale up with satellite altitude, beam size, frequency, user velocity. The maximum differential delay is observed at lower elevation (30° for LEO or 12.2° for GSO). The maximum differential Doppler is observed at 90°elevation angle.

Most typical differential delays reported are sub-millisecond for LEO and GEO/GSO under moderate cell/beam and elevation conditions. Delays climb to the ms-range only for extreme scenarios (large beams, low elevation, high uncertainty, simple compensation).

For a more consolidated evaluation in TR 38.742, it is proposed to use a spreadsheet to collect companies’ results on differential round trip delay and differential round Doppler.

Proposal 2-1-v0:

The companion spreadsheet [*] is used to collect companies’ results on differential round trip delay and differential round Doppler. - [*] Evaluation of differential delay and Doppler v000_Moderator

Companies are encouraged to comment on Proposal 2-1-v0

| Company | Comments |
| --- | --- |
| Samsung | It is fine with the direction. But, I feel that we do not need to make an agreement to collect the result. |
| CATT | Support |
| HONOR | Fine with it. |
| Xiaomi | Support |
| Sony | Support. |
| Toyota | Agree with Samsung. Collect the evaluation results and don’t see the need for specific agreement. |
| Tejas Networks | Support |
| CEWiT | Support |
| Panasonic | We are OK to calibrate with the Excel sheet. First, confirm a common understanding of Round-Trip-Delay and Round-Trip-Doppler, though. |
| TCL | Support |
| ZTE | Generally fine with the intention. The case where beam footprint is 90 degree with respect to orbital plane need to be considered. |
| Sharp | Support |
| Qualcomm | If we agree on the previous proposal, then we have the mathematical formulation of RTT, RTD from any point. The differential RTT, RTD could be directly evaluated, no need to collect results from different companies. |
| ST Engineering iDirect | Support |
| LGE | We are generally fine with the direction, but it would be reasonable to collect the results after RAN1 has aligned its evaluation assumptions. |
| Futurewei | For the Ka band, the upper bound of X values is beyond the limit for LEO-600 and LEO-1200. Should set X = 10 km and X = 20 km for LEO-600 and LEO-1200, respectively? |
| Nokia | OK |
| Ericsson | We support the use of the spreadsheet but have some comments to it: - Elevation angles: - Consider merging the rows for elevation angle 12.5 and 30 into one, since only one of them applies for a given orbit. - Clarify if the min elevation angle case is in the orbital plane or orthogonal to it. - Discuss further cell sizes for Case a: - How to handle Ku-band - Whether Set-1 cell sizes for Ka-band (e.g. 20 km) are representative of reality - Consider Set-2 cell sizes |
| OPPO | We are generally fine with the proposal, and regarding the minimum elevation angle, the footprint within the orbital plane and 90° with respect to orbital plane should be both considered in the spreadsheet. |
| Apple | Support |
| Spreadtrum | support |
| Huawei, HiSilicon | Support |
| NTT DOCOMO | Support. |
| CMCC2 | Support. |

### Proposal 2-2

This is a preliminary observation based the results reported from different companies:

Proposal 2-2-v0:

Observation Based on evaluation results collected from different sources, it is observed that; - Typical differential delays are sub-millisecond for LEO and GSO under moderate cell/beam and elevation conditions. - At S-band/LEO, most practical/typical maximum differential Doppler values are 1–8 kHz. - At Ka-band/LEO, values can spike to 20–60 kHz - For GEO, differential Doppler is orders of magnitude lower (tens to hundreds of Hz at S-band, sub-kHz at Ka). Maximum differential delay : LEO Max Diff Delay; GSO Max Diff Delay. Sub-ms (majority, typical); 70–540 μs; 400–900 μs. ~1–3 ms (large beam/cell,; 1–2.7 ms (LEO/edge/low el); 1–3 ms (GEO/large cell/low el). Maximum differential Doppler: LEO S-band; LEO Ka; GSO S-band; GSO Ka. Typical (based on several sources); 1–8 kHz; 10–35 kHz; <0.5 kHz; <3 kHz. Extreme (minority ); 30–230 kHz; Up to 232 kHz; Up to ~40 kHz (rare, large cell/edge, fast UE); Up to 72 kHz (very rare/extreme).

Companies are encouraged to share views on Proposal 2-2-v0

| Company | Comments |
| --- | --- |
| Samsung | The range of observed value is quite large. Thus, it needs more discussion on evaluation assumption. Furthermore, it is preferable to make observation per case. |
| CATT | It needs more discussion and derives the observation based on the majority results. |
| HONOR | Support. |
| Xiaomi | These results can be consolidated after the template for timing and frequency collection is stable and fulfilled. Prefer separate tables for FR1 and FR2. |
| Sony | Agree with Samsung, Xiaomi – it is preferable to make observations per case to reduce the differential ranges. To discuss. |
| Toyota | We also think that too early to make such broad observation given the many scenarios, parameters. Agree with Samsung, Xiaomi, Sony |
| Tejas Networks | Support the proposal |
| JHUAPL | The differential Doppler for larger beam sizes is significantly more than 1-8 kHz (at S-band). For a 440 km nadir-beam scenario, the maximum differential Doppler is approximately 30 kHz (see the attached plot). |
| CEWiT | Support |
| Panasonic | Thanks to the moderator for this overview. But companies seem to have had a different understanding of how to evaluate differential delay/Doppler. Some have assumed one-way, others two way. This is all mixed up here. We think it is better to confirm a common understanding of Round-Trip-Delay and Round-Trip-Doppler, then calibrate with the Excel sheet, then return to such an observation. |
| ZTE | Can be further discussed after collection of evaluation results. |
| Sharp | Support |
| ST Engineering iDirect | Propose to get values per scenario, for the scenarios agreed in RAN1#122 (beam size, elevation angle, frequency range, UE type, UE speed) |
| LGE | We think that observations should be collected for more detailed cases. Moreover, given the current differences in evaluation assumptions, we believe observations should be collected after addressing this issue. |
| Futurewei | Are those differential delay and Doppler values one way or round trip? What is the beam diameter for the case ~1–3 ms (large beam/cell)? |
| Nokia | The values should probably be put in brackets. After all, the maximum differential delay is based on physics and equations – it should not provide a difference of a factor of 8. As long as cell size, orbital height and (max) elevation angles are assumed, the results should be aligned. Some of the differences may come different terminology when it comes to ranges (does plus/minus in the range double the reported value?). |
| Ericsson | We propose to come back to proposal 2-2 after populating the spreadsheet of proposal 2-1. Detailed comments: - Clarify if differential delays and Doppler are one-way or roundtrip - We observed 1.6 ms differential roundtrip delay for 250 km cell diameter and 12.5 degree elevation angel, which is above the 900 µs in the proposed observation. |
| OPPO | The observation can be discussed later based on the collected results in the agreed spreadsheet. |
| Apple | The observation can be made after the data collection. |
| ETRI | It needs more discussion on evaluation assumptions. |
| Spreadtrum | This observation can be postponed after collection of evaluation results. |
| Huawei, HiSilicon | It can be postpone till the methodology are aligned. In addition, we should have observation for case b at the same time. |
| CMCC2 | The observations can be discussed based on the collected results after the previous evaluation assumption and methodology become clear. |

# Topic#3 Impact on PRACH detection performance

## Background

RAN1#122 made the following agreement.

Agreement

For PRACH performance evaluation for existing PRACH formats, adopt the following methods:

- Baseline: Analytical characterization of performance based on e.g. properties of ZC sequences and values of differential Delay / Doppler.

- Optional: Link level evaluations

  - The following table is proposed for LLS parameters

| Configurations | S/L-band | Ka/Ku-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | For Ka: UL 30GHz, for Ku: UL 14 GHz |
| Channel Model | Baseline NTN TDL-C rural model in TR38.811 |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx | 1 Rx |
| Antenna Configuration at the UE | A single omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| PRACH format | To be reported by companies. | To be reported by companies |
| PRACH configuration | To be reported by companies. |  |
| Metric | PRACH detection rate, FAR (Based on the preamble pool size is not less than 64), CDF of estimation error for frequency/timing |  |
| Receiver | Companies to report the receiver for PRACH detection. |  |

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Futurewei | Observation 8: Differential round-trip delay for the GSO scenario exceeds the longest guard period and cyclic prefix duration of the legacy NR long PRACH Format 1 at elevation angle 30° and X=125 km. Observation 9: For LEO-600, LEO-1200 and GSO, none of the NR short preambles can fulfil the differential round-trip delay at low elevation angles (e.g., elevation angle 70°). Observation 10: For LEO-600, LEO-1200 and GSO, differential round-trip Doppler shift for FR1 (2 GHz) and FR2 (even at elevation angle 90°) is smaller than SCS of NR short preambles. |
| Ericsson | Observation 14 Without GNSS-based UL pre-compensation at initial access, the differential delay and Doppler shift values at the PRACH receiver will be much larger than that in legacy PRACH transmission with GNSS-based UL pre-compensation. Observation 19 The magnitude of maximum residual Doppler shift, even the current values obtained under optimistic assumptions, seems challenging to be handled by the existing PRACH. The large timing uncertainty compounds the challenge even further. Observation 20 The CP duration with long PRACH formats is about 2 to 14 times shorter than the residual delay value for LEO. The maximum CP duration in short PRACH formats is about 20 times shorter than the maximum residual delay for LEO. This implies that the PRACH receiver will need to apply multiple timing hypotheses within the PRACH receive window to detect the preamble, which will increase the receiver complexity, especially for short formats with a small CP duration. Observation 21 Both delay and frequency offset cause cyclic shift in the received ZC sequences, resulting in a composite cyclic shift from which the effect of delay cannot be separated from the effect of frequency offset. |
| Spreadtrum | Observation 1 For set-1 S-band LEO-600 and LEO-1200, PRACH format 1 can cover maximum differential delay and Doppler. Observation 2 For set 1 S-band GEO, only if elevation angle is larger than 60°, PRACH format 1 can cover maximum differential delay. Observation 3 For set-1 Ka-band LEO-600 and LEO-1200, PRACH format 1 can cover maximum differential delay but not maximum differential Doppler. Observation 4 For set-1 Ka-band GEO, only if elevation angle is larger than 30°, PRACH format 1 can cover maximum differential delay. Observation 5 For set-2 S-band LEO-600, PRACH format 1 can cover maximum differential delay. For set 2 S-band LEO-1200, only if elevation angle is larger than 50°, PRACH format 1 can cover maximum differential delay. Observation 6 For set-2 S-band GEO, only if elevation angle is larger than 80°, PRACH format 1 can cover maximum differential delay. Observation 7 For set-2 Ka-band LEO-600 and LEO-1200, PRACH format 1 can cover maximum differential delay. Observation 8 For set-2 Ka-band GEO, only if elevation angle is larger than 70°, PRACH format 1 can cover maximum differential delay. Observation 9 PRACH format 1 can’t cover the maximum differential Doppler for set 2 LEO-600 and 1200. |
| Thales | Observation 5: For LEO600 and LEO1200 based deployments, the maximum differential RTD is within the limiting delay supported with the PRACH Format-1. Observation 6: For GEO base deployment with a beam size of 250km, the maximum differential RTD is beyond the limiting delay supported with the long PRACH formats. Observation 7: In the absence of a GNSS fix, PRACH Format-1 Type B Restricted Set PRACH preamble is resilient to differential frequency offsets/Doppler in LEO and GEO based NTN access. Observation 8: In the absence of a GNSS fix, short PRACH preambles are resilient to differential frequency offsets/Doppler. Observation 9: In the absence of a GNSS fix, the maximal differential delay encountered is beyond the detection capability supported by current NR PRACH short formats. Therefore, potential solutions should be discussed to address this issue. |
| vivo | Observation 11: In FR 1, the PRACH format 1 can cope with the timing error in 50km cell for LEO 600 and 90km cell for LEO 1200, if the UE pre-compensates a proper common TA part. The PRACH format 1 cannot cope with timing error in 250km cell for GSO even the pre-compensates a common TA. Observation 12: In FR1, for PRACH format other than format 1, it is difficult to cope with the timing error in many of the evaluated scenarios if the UE pre-compensates a common TA part. Observation 13: For LEO 600 and LEO 1200, without pre-compensation using accurate GNSS information, the current NR PRACH formats may not be able to cope with the large frequency offset. Observation 14: In FR 2, PRACH formats A1~C2 cannot cope with timing error in 20km cell for LEO 600, 40km cell for LEO 1200 and 110km cell for GSO even the UE pre-compensates a common TA. Observation 15: If the timing error and FO cannot cope with the PRACH format 1 after common TA and common FO compensation, relaxed requirements may need to be defined to cope with the larger timing error and/or frequency offset. Different TA values and different FO values would exist for UEs in a cell. |
| ZTE | Proposal 4: Round trip differential delay and Doppler need to be considered when comparing with tolerable range of PRACH preamble. Proposal 5: The largest differential delay and largest differential Doppler among all potential elevation angles should be used for PRACH performance evaluation considering the satellite mobility and semi-static configuration of PRACH format. Observation 4: No long PRACH format can handle the differential delay and Doppler in LEO-600 S-band, LEO-1200 S-band and GSO S-band in case a, where the location uncertainty is the area served by beam. Observation 5: No long PRACH format can handle the differential delay and Doppler in LEO-600 S-band and LEO-1200 S-band in case b X=25km, where the location uncertainty is a circle with radius X=25km. Observation 6: No short PRACH format can handle the differential delay and Doppler in LEO-600 S-band, LEO-600 Ka-band, LEO-1200 S-band, LEO-1200 Ka-band, GSO S-band and GSO Ka-band in case a, where the location uncertainty is the area served by beam. Observation 7: No short PRACH format can handle the differential delay and Doppler in LEO-600 S-band, LEO-600 Ka-band, LEO-1200 S-band, LEO-1200 Ka-band, GSO S-band and GSO Ka-band in case b X=25km, where the location uncertainty is a circle with radius X=25km. Observation 8: No short PRACH format can handle the differential delay and Doppler in LEO-600 Ka-band, LEO-1200 Ka-band and GSO Ka-band in case b X=5km, where the location uncertainty is a circle with radius X=5km. Observation 9: For case a (the location uncertainty area is the area served by the cell or beam), existing PRACH format cannot handle the differential delay and Doppler for all evaluated cases. Observation 10: For case b (the location uncertainty area is a circle of radius X km), existing PRACH format can handle the differential delay and Doppler for moderate X values, e.g., X=5km for S-band and X=1km for Ka-band. |
| Xiaomi | Observation 1. NR defined PRACH formats cannot cover the differential delay and Doppler requirements defined in Table 2 of R1-2506987 |
| CMCC | Observation 1: For PRACH performance evaluation, the tolerable frequency error based on SCS restriction can be different depending on whether restricted sets are configured for PRACH preamble. Observation 2: The required CP lengths and SCSs (with restricted set and without restricted set) under LEO-600, LEO-1200, GEO with different beam footprint sizes are evaluated in Annex II of R1-2507012. Proposal 11: The accuracy requirements in the evaluation of GNSS-resilient NR-NTN operation should take the current PRACH preamble formats configuration into consideration. Proposal 12: 1/2 CP can be considered as a criterion to determine whether the CP of one PRACH format can be used in the satellite scenario. Proposal 13: 0.4*SCS (without restricted set) and/or 2*SCS (with restricted set) can be considered as a criterion to determine whether the SCS of one PRACH format can be used in the satellite scenario. Observation 3: Without GNSS facilitating the timing and frequency pre-compensation, under the beam footprint size-level positioning error, larger CP and subcarrier spacing are required at satellite or gNB to receive preamble correctly. Observation 4: Under current assumption of beam footprint size, the current CP design in NR cannot support the LEO-1200 and GEO in S-band. Observation 5: Current short format preamble cannot be used in the LEO-600, LEO-1200 and GEO in Ka-band, if the positioning uncertainty area is in a beam footprint size level. Proposal 15: The tolerable positioning error under current PRACH format can be studied to determine the radius of the location uncertainty area. |
| CATT | Observation 3: For preamble formats for L_RA=839 and Δf_"RA" ∈{1.25,5} kHz, maximum TA tolerance is 684.4 us, corresponding to format 1, and maximum FO tolerance is 2*SCS for restricted sets. Observation 4: For preamble formats for L_"RA" ∈{139,571,1151} and Δf_"RA" =15⋅2^μ, maximum TA tolerance is 33.3us, corresponding to format C2, and maximum FO tolerance is corresponding SCS /2. Observation 7: In scenario 1, timing tolerance of existing PRACH formats in many cases cannot meet the requirements of TA errors, but the frequency tolearance is less challenging. |
| OPPO | Observation 4: PRACH transmission without TA pre-compensation would either introduce unexpected interference to uplink transmission or waste resources. |
| Tejas Networks | Observation 9 : The maximum supported two-way differential delay for PRACH is 684.38 us for long format 1 but only with a maximum doppler support of 2.5 khz with restricted type B. For long formats, maximum supported doppler is 10khz with restricted type B for long format 3. Observation 10 : For short formats , the doppler support is typically half the SCS and limiting differential two-way delay is upto 33.08 us (format C2,15 khz SCS) Observation 11: For S band GEO scenarios and LEO 1200km Set 2 scenario , maximum differential delays clearly exceed the PRACH limiting differential two-way delay for all PRACH formats. Observation 12: For S band LEO 600km(Set 1 and Set 2 ) and LEO 1200km (Set 1) scenarios , the maximum differential delay scenarios can be handled by the PRACH long format 1 but only with a doppler support of maximum 2.5khz. Observation 13: For S band LEO scenarios , the maximum residual doppler for cases can be supported using short formats(suitable SCS) and Long Format 3 with restricted Sets . But the short formats have very less limiting differential two-way delay value hence may not be feasible for beams other than nadir beams. Observation 14: For Ka band LEO 600 km, LEO 1200 and GEO scenarios , the maximum differential delay scenarios can be handled by the PRACH long format 1 for Set 1 satellite configuration. The maximum residual doppler is supported by short formats C0/C2 with SCS 60khz or higher but the short formats have very less limiting differential two-way delay value hence may not be feasible for beams other than nadir beams. Observation 15: For S band scenarios with elongating beam diameters , maximum differential delays clearly exceed the PRACH limiting differential two-way delay for all PRACH formats. Observation 16: Although several PRACH formats can support delay and doppler separately for some of the cases but they do not support large values of delay and doppler simultaneously which is the case for many NTN scenarios. Consequently, the detection capabilities of the existing PRACH formats are inadequate for NTN scenarios that operate without GNSS assistance. Proposal 6: Enhancements are required in the existing PRACH and NR Initial Access procedures to support the large residual dopplers and differential delays in various NTN scenarios without GNSS availability. |
| NEC | Observation 1: When the GNSS information in the UE with GNSS capability is temporarily unavailable, the legacy UE timing and frequency pre-compensation cannot work, especially during the initial access. |
| LG Electronics | Observation #1: NR PRACH preamble Format 1 can tolerate TO of up to 340μs and FO of up to 2.5kHz. Observation #2: NR PRACH preamble Format C2 can tolerate TO of up to 4μs and FO of up to 60kHz when μ = 3. Observation #3: In FR1 NR-NTN, differential delay can exceed the TO tolerance of the NR PRACH format at low elevation angles in the GSO scenario. Observation #4: In FR2 NR-NTN, differential delay can exceed the TO tolerance of the NR PRACH format across almost all elevation angles in the GSO, LEO-1200, and LEO-600 scenarios. |
| Ofinno | Observation 1: Temporary unavailability of the GNSS increases errors in the pre-compensated TA applied by the UE which in turn increases the latency of RACH procedure and causes uplink interference. |
| Fraunhofer | Observation 1: An NTN UE, and in case GNSS signal was temporarily unavailable or degraded at the time of initiating a PRACH preamble, may not be able to fully pre-compensate the time offset. Observation 2: An NTN UE, and in case GNSS was temporarily unavailable or degraded at the time of initiating a PRACH preamble, may not be able to fully pre-compensate the frequency offset, which should be corrected by the network in a frequency correction closed loop mechanism. |
| Panasonic | Observation 1: Only for S-band LEO 600km, the maximum differential delay meets the delay requirement (i.e. 1/2 x CP length) with PRACH format 1 (CP length 684 us). For other scenarios (S-band LEO 1200km and GEO, Ka-band LEO 600km, 1200km and GEO) cannot meet the delay requirement of any PRACH format. Observation 2: In S-band, for LEO600km, 1200km and GEO, the maximum differential delay within uncertainty area up to 20 km radius can meet the delay requirement (i.e. 1/2 x CP length) with PRACH format 1. For uncertainty area up to around 1 km, the maximum differential delay can meet the delay requirement with PRACH format 0-3 and PRACH format A3/B4/C0/C2 with SCS 15kHz. Observation 3: In Ka-band, for LEO 600 km, 1200 km and GEO, UE location uncertainty less than 300m is required to meet the delay requirement with PRACH format C2. Observation 4: For S-band, GEO can meet the Doppler requirement with all PRACH formats. LEO 1200km can meet the Doppler requirement with short PRACH formats with SCS 15kHz but cannot meet with long PRACH format. LEO 600km cannot meet the Doppler requirement with long format and short format with SCS 15kHz. Observation 5: For Ka-band, all scenarios can meet the Doppler requirement with short format with SCS 120kHz. Observation 6: For S-band, LEO 600 km and 1200 km, UE location uncertainty up to 5 km and 10 km, respectively, can meet the Doppler requirement with long PRACH format using restricted set type B. For GEO, all scenarios can meet the Doppler requirement. Observation 7: For Ka-band, all scenarios can meet the Doppler requirement with short format with SCS 120kHz. Observation 8: Scenario 1 (case a) cannot meet either or both of the delay and Doppler requirements cannot meet for both S-band and Ka-band. Observation 9: Scenario 2 (case b) with UE uncertainty area up to 5-20 km radius (depending on the satellite types) for S-band and 300 m radius for Ka-band can meet the delay and Doppler requirements. Proposal 1: To make the following conclusion: - For Scenario 1, the differential delay and Doppler cannot meet the requirements with the existing PRACH formats. PRACH related enhancement including PRACH reception as gNB implementation (e.g. larger detection window size, multiple detection timing) is required. - For Scenario 2 with UE location uncertainty area within a certain range, the differential delay and Doppler can meet the requirements with an appropriate PRACH format. It is necessary to discuss whether and how to determine UE can use the UE location obtained in the past (i.e. the UE location uncertainty is within the acceptable range). |
| Lenovo | Observation 1: GNSS unavailability may result in network and UE’s clock offset errors and further increase the timing errors. Observation 2: Large timing and frequency uncertainties can cause the PRACH preamble to fall outside the network’s detection window, resulting in RACH failures. |
| Nokia | Observation 1: Currently defined 5G NR PRACH preamble formats are not able to cope with differential Doppler and differential TA of a 50km diameter cell for LEO-600. Observation 2: For cases where UE location uncertainty is defined within a cell area for cell diameter at 50 km and satellite at LEO-600, existing 5G NR random access preambles will not be able to meet requirements for maximum differential Doppler and maximum differential TA. Proposal 1: For ensuring successful PRACH reception at gNB, a UE is assumed to perform pre-compensation based on previous GNSS information in the case of GNSS information temporary unavailability during initial access. |
| Sony | Observation 6: Short PRACH preamble formats are unable to tolerate the differential delay when using a beam-center reference location for pre-compensation. Observation 7: Using only long PRACH Format-1 Type B Restricted Set preambles could be limiting in high-beam density scenarios. |
| MediaTek | Observation 6: With minimum angle β = 10 degrees, the maximum GNSS UE position error that can be supported is 2550 m with RACH preamble format C2 (µ=0); or 1535 m (µ=1). Observation 8: Some margin should be used for practical GNSS UE position error to avoid potential impact on RACH preamble detection performance. Proposal 3: The maximum GNSS UE position error that can be supported while avoiding physical layer channel/signal changes is 2550 m with RACH preamble formats C2 (µ=0). |
| Toyota | Observation 3: The residual Doppler and delay pre-compensation with a (common) reference location exceeds NR benchmark and PRACH capabilities. |
| Apple | Observation: The maximum differential delay will impact the PRACH coverage if no valid GNSS information is available for NTN UE. |
| Qualcomm | Observation 2: Current NR PRACH (long formats including restricted sets A/B and short formats) is not designed to operate under the time-frequency errors observed in GNSS-resilient NTN. Observation 3: Short PRACH formats cannot resolve any timing error larger than the symbol duration for 15 KHz SCS (i.e. 66.66 μs) and hence is incompatible with Scenario 1 and Scenario 2 with any reasonable size of location uncertainty area. Proposal 5: Capture the evaluation results in Table 2-2 of R1-2507719 in the TR |
| NTT DOCOMO | Observation 2: - For FR1-NTN, the long formats are feasible for FR1-NTN; while enhancements may be needed for some short formats (e.g., A1, A2, B1, B2, B3) if applied. - For FR2-NTN, the maximum timing error exceeds the maximum tolerable timing error of the short formats, enhancements are also needed. |
| CSCN | Observation 5: Due to the large differential time and frequency offset, availability of CS multiplexing would be limited. Observation 6: The differential timing and frequency offsets caused by UE location uncertainty case a with no larger than 90km radius of the beam footprint can be handled by existing PRACH formats. |
| Google | Observation 1: Considering the agreed UE speed is up to 1500 km/hr, the existing PRACH restrict set may not be feasible. |
| CEWiT | Observation 4: For successful UL transmission (e.g. PUSCH) in NTN, the maximum allowable timing error is 0.944μs. This indicates that NTN scenarios cannot be accommodated by reference-location-based TA determination. Observation 5: For Reference-Point based TA Computation, detection of short PRACH formats may not be successful in NTN scenarios. Except for long PRACH format 1, other long PRACH formats (0,2, and 3) are unlikely to be detected reliably, as their timing error limits are too low to accommodate the larger delays typical of NTN. |

## Summary of companies’ contributions

Nearly all companies observe that existing PRACH formats (both long and short) are inadequate for many NTN scenarios under study.  Differential delay (due to large beams, low elevation, and location uncertainty) and differential Doppler (due to high motion and frequency bands) frequently exceed the detection capabilities of existing PRACH. Short PRACH formats are especially problematic due to tiny cyclic prefixes (CP), limited tolerable timing and frequency errors, and small detection windows. According to some sources, only some long PRACH with custom restricted sets may work in some cases, none reliably cover all uncertainty scenarios.

| Company | PRACH tolerance exceeded ? | Special notes |
| --- | --- | --- |
| Futurewei | Yes. Exceeds NR long PRACH (GSO, large X/low elevation); All satellite types exceed NR short preamble guard at low elevation (<70°) |  |
| Ericsson | Yes. Existing PRACH formats (long/short) challenged for max Doppler/delay Legacy PRACH cyclic prefix up to 0.7 ms, much less than max delay. Frequency errors may exceed SCS/res. set; require new handling. | Edge beams yield max delay. Nadir/perpendicular (90°) beams yield max Doppler. PRACH receiver needs multiple timing/frequency hypotheses Proposes conjugated PRACH preamble repetition. Separate PRACH resources may be needed for GNSS-resilient operation. Doppler ambiguity in ZC sequences highlighted. Uplink synchronization and handover procedures impacted; RACH-less handover may not work w/o GNSS. |
| Spreadtrum, UNISOC | PRACH format 1 covers max S-band delay/Doppler in LEO-600/1200 (set-1); Does not cover max Ka-band Doppler in LEO-600/1200; In GEO, PRACH format 1 covers maximum delay only at high elevation in many cases; Set-2: max Doppler not covered for LEO-600/1200; max delay not covered for GEO unless at very high elevation. TA range in Msg3/connected mode enough for all orbits. | Recommends use of cell/beam-level common TA/Doppler for UL compensation when GNSS unavailable. Network must frequently update TA in LEO-600/1200 if GNSS lost. Closed-loop frequency adjustment recommended. Delay/Doppler drop with increasing elevation. Max Doppler at lowest elevation. TA range in Msg3/connected mode sufficient for any orbit. |
| Thales | Long PRACH Format-1: sufficient for LEO delays; not sufficient for GEO (250 km beam). Type-B restricted set (Format-1) resilient to S-band Doppler in LEO/GEO. Short PRACH formats: resilient to Doppler but not to large differential delays | Proposes open/closed-loop UL timing and frequency control (incl. frequency adjustment command), potential need for negative TA, and group-common DCI for timing control. Calls for PRACH performance study under large Doppler/RTD and solutions for short-format delay limits |
| vivo | FR1: PRACH format 1 can cover LEO (50/90 km beams) with common TA, but not GEO (250 km). Other long formats struggle; without accurate pre-comp, LEO FR1 Doppler may exceed SCS. FR2: short formats (A1–C2) cannot cover timing for LEO/GEO even with common TA | Provides validity-duration bounds: timing-based T up to >300 s (pedestrian) down to ~0.7 s (1500 km/h); frequency-based T >1000 s (pedestrian) down to ~5 s (1500 km/h). Proposes separate PRACH resources, RAR enhancements (FO command, extended/negative TA), and closed-loop FO in connected mode |
| ZTE | Concludes existing PRACH formats cannot support case a (full beam uncertainty) in any orbit/band; For case b: no support for X=25 km (all), and Ka-band not supported even at X=5 km; moderate X may be feasible (e.g., S-band X≈5 km; Ka-band X≈1 km). Long format tolerances (examples): Format 1 Type-B ~130.6 μs delay / 5 kHz; Format 3 Type-B ~32.7 μs / 20 kHz | Proposes: relax initial access requirement to allow UEs without valid GNSS; broadcast indication (e.g., SIB) when such access is allowed; add closed-loop UL frequency adjustment (MAC-CE command); indicate TA drift rate to reduce TAC overhead; GNSS status reporting; gNB-indicated UL extension time; gNB-triggered GNSS update; evaluate PRACH against largest delay (low elevation) and largest Doppler (high elevation) due to LEO mobility |
| Huawei, HiSilicon | Scenario 1 + case a differential delay/Doppler far exceed tolerance of existing PRACH (both FR1 and FR2). Scenario 2 + case b can be bounded using long PRACH format 1 (FR1) or short C2 (FR2), but within limited X/T; Huawei suggests relaxing RAN4 PRACH timing/frequency limits for GNSS-resilient operation | Proposes: relax PRACH timing/frequency requirements; prioritize coarse location via DL-TDOA; enhance DL-RSTD range; add sign (±) to Absolute TAC (incl. RAR); add timing/frequency adjustments after Msg2; introduce closed-loop UL frequency control in IA and connected; UE GNSS availability/validity reporting; separate PRACH resources for large errors; quantify UE power savings with extended GNSS periods; satellite visibility stats show multi-satellite DL-TDOA feasible |
| Xiaomi | Yes. Long PRACH: only Format 1 covers delay but fails FO in some FR1 cases; short PRACH handles Doppler but CP (≤66.7 μs) far below required delays. Conclusion: current NR PRACH cannot meet Table 2 delay/Doppler | Proposes Doppler-proof PRACH via optimized ZC roots under Ncs constraints; multi-stage frequency compensation (gNB and/or UE) during IA and connected; notes UE oscillator error (up to 10 ppm) can yield ~20 kHz at 2 GHz; suggests gNB-side FO estimation via preamble detection/scanning to simplify UE behavior |
| CMCC | With beam-size-level uncertainty, existing PRACH CP/SCS often insufficient: FR1 S-band CP cannot support LEO-1200/GEO; FR2 Ka short formats cannot cover timing error for LEO/GEO. Use criteria: timing ≤ 1/2 CP; FO ≤ 0.4·SCS (no restricted set) or ≤ 2·SCS (restricted set). In Scenario 2 (X-circle), tolerable X under current formats is limited (e.g., long format 1 up to ~29–30 km with FO 2.5 kHz; short C2 ~2.9 km at 15 kHz SCS) | No new PRACH formats/sequences or resource changes in WI; prioritize LEO S-band and define Ku-band eval parameters; beam size as baseline for uncertainty area; high mobility (1500 km/h) may need separate handling; estimate ~300 TA updates from 30°→90° over ~2 min (~2/sec) without GNSS, so study TA signaling reduction; RAT-dependent positioning can reach ~2–4 km in 4–8 s (multi-sat), ≤1 km with longer intervals |
| CATT | Scenario 1: For beam sizes and TA errors in all bands/orbits (TA up to ~800 μs), existing PRACH formats timing tolerance is NOT sufficient—timing error is larger than all allowed values. - Scenario 2: If location uncertainty X≤5 km, C2 format (limit ≈33 μs) can cover. - FO error: Existing PRACH formats (except 0/1/2) can cope with frequency errors in most scenarios; timing is the main limitation. | PRACH enhancement needed for timing focus, especially for beams >5 km (Scenario 1 & high uncertainty Scenario 2). - In LEO/GSO for L/S bands and typical beams, timing error is limiting factor (not frequency error). - Explicit recommendation: For Scenario 2, keep location uncertainty radius ≤5 km if using legacy formats. - If not, must use enhanced PRACH (similar to Scenario 1). - CATT provides explicit tables with TA/FO errors vs. orbit, band, beam size, and UE speed. |
| OPPO | Not all scenarios can meet residual TA ≤ 0.5·CP with existing PRACH CPs (e.g., GEO 12.5° exceeds max CP 684.37 μs). Residual FO can be met by selecting suitable PRACH format (per FO ranges), but both delay and FO must be jointly considered | gNB should indicate whether GNSS-resilient operation is supported. If supported, carry residual UE-specific TA and FO in RAR for Msg3. In connected mode, reuse legacy TA updates; no new UL FO compensation needed. NR positioning as alternative: SSB-based TDOA with 4 satellite positions achieved ~561 m (95% CDF). |
| Tejas Networks | PRACH long format 1 (max delay ≈ 684 μs, max Doppler ≈ 2.5 kHz w/ restricted set B): – Support only some S band LEO 600km, LEO 1200km Set 1 single beams (if Doppler ≤ 2.5 kHz). - For more severe/WORST-CASE or elongated beams, all existing PRACH formats fail (delays exceed tolerance and/or Doppler does). - Short PRACH formats always insufficient (max 33 μs delay at 15kHz SCS). - For some Ka-band LEO, short formats (C0/C2) support Doppler but not delay except tiny beams. - GEO scenarios: max delays always far exceed PRACH tolerance. | Existing PRACH cannot support combined large delay & Doppler of many realistic NTN cases. - Proposed solutions: Dual PRACH, RACH window extension, beam-specific reference points, and further procedure upgrades. |
| Samsung | In GNSS-unavailable conditions, residual delay/Doppler may exceed what PRACH preambles can correct; specialized PRACH pools/resources, and gNB-side compensation (fast TA/Doppler via DCI), are suggested; separation from legacy GNSS-dependent UEs recommended- | Proposes: study UE behavior when GNSS is (temporarily) unavailable before IA and during RRC_CONNECTED; gNB-assisted initial access/robust PRACH pools; differentiated resource allocation/scheduling for GNSS-available/unavailable UEs; explicit GNSS status reporting; fast gNB-side TA/Doppler correction via DCI; coexistence/resource separation between legacy and new UEs. LEO: max delays <130 μs, Dopplers <2.1 kHz (2 GHz); GEO: delays ~400 μs, Dopplers ~0.1–0.4 kHz. |
| LGE | FR1: Differential delay can exceed PRACH tolerance at low elevation in GSO. FR2: Differential delay exceeds PRACH tolerance at almost all elevations in GSO, LEO-1200, LEO-600. Short PRACH formats not feasible for FR2 Scenario 1. | Proposal: Scenario 1 in FR2 (30 GHz) should be deprioritized for Rel-20 NR-NTN. When uncertainty = beam radius, GNSS-resilient operation for FR2 is infeasible with current specs. Analysis confirms PRACH intolerances for differential delay in multiple scenarios. |
| ETRI | In LEO, max differential delay at edge can approach or exceed NR PRACH tolerance; delay/Doppler values much greater for wide beams; max delay and position uncertainty can be "very large" if beam mapping is considered | GNSS unavailability can last for hours in reality, so Scenario 1 (no GNSS) should be baseline for SI. Scenario 2 (degraded/aged previous GNSS) can be optional. Recommends bounding X by actual projected beam radius, not only Nadir. Provides explicit max edge values for evaluation. |
| Panasonic | S-band: For Scenario 1, only LEO-600 short format meets delay/PRACH. Others (LEO-1200, GEO, Ka-band) do not meet PRACH delay requirement for any format. Ka-band: Only for uncertainty area <300 m can meet PRACH C2. Doppler: S-band, GEO meets all; LEO-1200 short only; LEO-600 does not for long format. Ka-band: All meet Doppler for short formats. For whole cell uncertainty, existing PRACH formats not sufficient. | If uncertainty area is reduced (Scenario 2), requirements can be met as follows: - S-band: Up to 5–20 km for LEO/GEO with PRACH-1 restricted. - Ka-band: Only for <0.3 km. PRACH enhancements (gNB side): Larger detection windows, multiple timing required for Scenario 1. Summary: If UE can bound its uncertainty (small X), PRACH works. If not, enhancements needed. Proposal: For Scenario 1 (whole cell), PRACH enhancement required. For Scenario 2, only if uncertainty area is small enough. |
| Nokia | Yes; both differential delay (TA) and Doppler for 50 km cell at LEO-600 exceed NR PRACH tolerance (even restricted sets). | Proposal: For initial access, UE should always perform pre-compensation using previously acquired GNSS information. Rel-20 NR should not consider scenarios where GNSS is entirely unavailable during initial access. NR PRACH format enhancement is out-of-scope; focus is on procedural assumption of GNSS existence for timing/frequency. |
| CCU | For Case a, LEO-1200 edge to center (0.50 ms) and LEO-600 (0.26 ms) approach or exceed NR PRACH short CP formats (e.g., some formats <0.3 ms CP). For smaller uncertainty (Case b), all values <0.15 ms, most CP formats can tolerate. | Main focus LEO, 2 GHz (S-band), elevation 30°. - Full edge-to-edge values larger (e.g., up to 0.90 ms for LEO-1200). - Not separated by PRACH format in evaluation. - Provides both beam-footprint ("Case a") and uncertainty-circle ("Case b") analyses. |
| Sony | Short PRACH formats cannot tolerate the differential delay (short CP). Long PRACH (Format-1 Type B Restricted) may just cope (for LEO-600, LEO-1200) in low-density beams, but problematic in high beam-density/interfering scenarios. | Proposes research on better reference-location/time adjustment, common compensation in high-density. - High beam-count/overlap increases compensation error and PRACH failures. - Validity time of location data/compensation is important. - Study required into self-/gNB-correction, validation methods. |
| MediaTek | Not directly stated, but: For worst (7.5 km GNSS error), timing errors ~43 μs, frequency errors ~1.5 kHz could challenge short RACH preamble CP for LEO/S-band in high UE speed. Legacy RACH tolerance is a key question for RAN1 discussion. | Provides detailed error scaling with GNSS accuracy and velocity. - "Scenario 1": study what GNSS error legacy RACH formats can support (avoid PHY changes). - "Scenario 2": time duration of GNSS validity is tightly coupled to UE velocity and error growth. - RAN1 to focus on maximum GNSS position error legacy RACH can support for resilience. |
| Toyota | Yes for single reference location (delay/Doppler exceed NR PRACH tolerances in most cell locations). With multiple reference locations: significant reduction; current NR PRACH may succeed for most, but high density/very large cells may still have issues. | Proposes unified 3-step uplink time/frequency sync framework: Step 1: Multiple reference locations & DL ref signals for pre-comp. Step 2: PRACH design enhancement (format options, repetitions, power control). Step 3: Closed-loop sync. - Using more reference locations shrinks residual errors 5–8x. - NR signaling for reference location config preferred. - High-tech solutions prioritized to avoid physical layer/signal changes. |
| Qualcomm | Yes, timing/frequency error for worst-case area (low elevation, large radius) can exceed PRACH tolerances, particularly for initial access; mitigation via nominal correction necessary but not always sufficient. - Compatibility: PRACH Format 1 maximum: 683 μs; Format 1(B) 130 μs; Format C2 67 μs Short PRACH formats limited to 67 μs—fail for most NTN areas Connected Mode: Satellite RTT drift at low elevations: 43.7 μs/s, requiring up to 20 TA commands/sec for 15 kHz SCS. - For larger uncertainty, even restricted sets may fail—multiple Msg-1, Msg-3 needed. - For connected mode, drift may exceed closed-loop capabilities, requiring enhancement. | Proposes new restricted sets, multi-transmission and signaling enhancements for robust PRACH - Closed-loop TA/FA mechanism must be enhanced to cope with drift and maintain performance - Simulation: gNB issued 49 TACs and 59 FACs in 30s for one scenario. - Explicit requirement: residual timing/frequency errors after RA must be within X μs, Y ppm (FFS exact values). |
| Johns Hopkins University | Very likely exceeded (especially for initial access in large/low elevation beams): - Differential delay of up to 2.75 ms can far exceed long PRACH and legacy NR tolerance. - Short PRACH cannot operate above ~67 μs. - Drift rates require frequent closed-loop updates to maintain alignment. | Strongly recommends RAN1 study low-elevation, large-beam cases explicitly. - Closed-loop procedures for both TA and Doppler essential in connected mode. - Suggests using system timestamping (SIB9/ReferenceTimeInfo) for initial Doppler/fine TA estimation. - Provides quantitative delay/Doppler mapping for footprint and elevation. - Emphasizes time-variation of error (drift) as a major operational challenge. |
| CSCN | Up to 90 km beam radius supported by current PRACH in best format (C2, L=139). - Beam/cell radii exceeding 90 km, differential delay or frequency exceed PRACH tolerance. - For formats with L=839, max radius ~7.5 km (delay/FO). - For small/medium beams (r ≤90 km), long PRACH formats or C2 can handle (per Table 1 of their conrib.) | TA margin/advanced detection window methods proposed for overcompensation. - Network broad/assists with RP: Critical for both TA and FO compensation in worst-case. - PRACH CS (cyclic shift) availability limited in very large beams. - Proposes optimizations to PRACH and RAR reception windows. - Autonomous delay prediction at UE directly reduces frequency of TAC. - Stresses RAN1 evaluation with real NCR-NTN parameter sets. - PRACH detection rate, FAR, and CDF of timing/frequency error as key evaluation metrics. |
| CEWiT | All evaluated NTN scenarios far exceed permissible timing error for UL transmission (NR spec << actual error). - PRACH detection: – Only long PRACH format 1 (timing limit ~340 μs) potentially operable for some scenarios. – Short PRACH and other long PRACH formats fail; timing error too large. - PRACH margin/advanced detection window/negative TAC support are needed for robust initial access. | Detailed quantitative analysis for LEO/GSO altitudes, S/Ka bands. - Proposes SSB/SIB19-aided multilateration for autonomous TA estimation, minimizing position error even during GNSS outages. - TA should be quantized, include a margin, and allow negative corrections in RAR. - Closed-loop TA correction critical. - Focus on minimizing physical layer changes, maintaining backward compatibility. - Explicit compliance evaluation against NR timing error specs. |

## Initial proposal

### Proposal 3-1

The conclusion on the impact on PRACH performance may need more consolidated evaluation results/criteria (Proposal 2-1-v0 and Proposal 1-2-v0). However, as outlined above, several sources observed that both Long/Short PRACH always fail for realistic NTN uncertainty area/beam sizes. Some sources observed that only long PRACH with custom restricted sets may work in some cases, none reliably cover all uncertainty area scenarios.

With the above considerations in mind, the following working assumption is made:

Proposal 3-1-v0

Working assumption - The existing PRACH formats (both long and short) are inadequate for many NTN scenarios under the study on NR-NTN GNSS resilience. - Differential delay (due to large beams, low elevation, and location uncertainty) and differential Doppler (due to high motion and frequency bands) frequently exceed the detection capabilities of existing PRACH. - Short PRACH formats are especially problematic due to tiny cyclic prefixes (CP) and limited tolerable timing and frequency errors. - Enhancements are required to support the large differential delays/Dopplers in various NTN scenarios without GNSS availability

Companies are encouraged to share views on Proposal 3-1-v0

| Companies | Comments |
| --- | --- |
| Samsung | Without having exact observations on differential delay/Doppler and PRACH tolerance, it is premature to consider this way. |
| CATT | It is too early to make thee conclusion. |
| HONOR | It is too early to make the conclusion. |
| Xiaomi | OK for this direction |
| Sony | Agree on PRACH limitations in “many NTN scenarios”. Enhancements are required to support all cases, so we do not see this proposal as restrictive. |
| Toyota | Too early to make such working assumption |
| Tejas Networks | Support the proposal |
| JHUAPL | Support. |
| Panasonic | For scenario 1, we agree with Proposal 3-1-v0. For scenario 2, the existing PRACH can support uncertainty area of up to 5-20km radius for S-band and 300m radius for Ka-band in our evaluation. |
| TCL | We support the enhancement of PRACH for GNSS-resilience. |
| ZTE | Not support. Firstly, existing PRACH format are inadequate for case a. But for case b with moderate X, existing PRACH format can still work. However, the working assumption only considers case a and targeting to enhance for scenarios “without GNSS availability”, which is beyond the scenarios mentioned in SID (i.e., temporary unavailable, accuracy degradation, increased measurement period). Detailed observations need to be achieved before working assumption. Moreover, as mentioned in SID, the study aims to minimize physical layer procedures impact and avoids physical layer channel/signal changes. Hence, it is more reasonable to focus on scenarios where differential delays/Dopplers are tolerable by existing PRACH formats instead of enhancing for larger differential delays/Dopplers. |
| Sharp | Too early to make the conclusion. The existing PRACH formats should be kept unchanged. Some enhancements on the preamble transmission at UE and enhancement on detection at the gNB can be considered to solve the problems. |
| ST Engineering iDirect | Agree as working assumption, but the assumption needs to be confirmed at a later stage. However, we propose not to waste time on finding an agreement about this working assumption and suggest to spend time on finding an agreement on the other proposals. |
| LGE | Not support. To our understanding, there exist cases where the existing PRACH format is feasible where the size of the uncertainty area is below a certain size. We believe that cases where the existing PRACH format is feasible should be the subject of study. PRACH-related enhancements must take into account the impact on network receivers in addition to maintaining the PRACH format. |
| Futurewei | The proposal says “working assumption”, but it reads like an observation. The second bullet should be removed. |
| Nokia | Agree – and this is also the reason for our preference for having to rely on a previously acquired (or trusted) geo-location for the UE. |
| Ericsson | Support. For the second sub-bullet, "and frequency errors" could be removed since short PRACH formats are not more problematic than long in terms of tolerance to frequency errors. |
| OPPO | We should focus on the evaluation assumption and alignment among the evaluation results firstly before discussing the proposal. |
| Apple | This proposal can be starting from an observation. Whether enhancement is needed is related to whether supporting other solution(s). |
| ETRI | It is too early to make the conclusion. |
| Ofinno | Support the working assumption regarding PRACH format. This because the range of differential delays may be different for UEs which cannot rely on its GNSS for timing and frequency compensation and UEs that have previously acquired GNSS based position. |
| Huawei, HiSilicon | We do not agree with the WA. The observation is only applicable for scenario 1 + case a. If UE has coarse location from RAT-dependent positioning or in scenario 2, some PRACH formats can still work. In addition, enhancement of PRACH transmission mentioned in 2nd bullet is out of SI scope. |
| NTT DOCOMO | We agree with other companies that detailed observations on evaluation results for each case/scenario should be made before having conclusion. |
| Lenovo | Need more discussions and evaluations. |
| CMCC2 | In general we are fine with the direction. We share the similar view that current PRACH format may not applicable to all the scenarios. But as discussed in the previous section, the methodologies should be aligned and the evaluation results should be collected. Based on that, we can draw some observations and conclusions. |

# Topic#4 Impact on initial access

This section is about impacts on initial access related aspects other than PRACH detection.

## Background

TBC

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Spreadtrum | Observation 10 Current TAC in RAR is enough for LEO and GEO when SCS is 15kHz or 30kHz. |
| Huawei | Observation 14: The absolute TAC in RAR with the range of [0, 3846]*16Ts ([0, 2003.13μs]) cannot indicate a negative timing adjustment to cope with the negative timing error in GNSS resilient scenarios 1 and 2. |
| CATT | Observation 9: The maximum adjustment range of initial TAC is 2ms, which may meet the requirement for GNSS unavailable temporarily case. |
| China Telecom | Proposal 4: It needs to study the impacts on initial access in GNSS resilience scenarios. |
| LG Electronics | Proposal #6: In Rel-20 NR-NTN, RAN1 clarifies whether a new RAR payload is required to support TO/FO correction information. |

## Summary of companies’ contributions

The companies' proposals and observations focus on the sufficiency and limitations of the TAC in RAR messages for NR-NTN, under GNSS-resilient scenarios. Spreadtrum observes that the current TAC in RAR is adequate for both LEO and GEO satellites when using 15kHz or 30kHz SCS. However, Huawei points out a limitation: the present absolute TAC range cannot support negative timing adjustments, which are necessary to address negative timing errors in some GNSS-resilient situations. CATT notes that the maximum adjustment range of the current TAC (2ms) may suffice for cases of temporary GNSS unavailability. Additionally, China Telecom proposes further study on initial access impacts in GNSS-resilient scenarios, while LG suggests clarifying whether a new RAR payload is needed in Rel-20 NR-NTN to support timing and frequency offset correction information.

## Initial proposal

### Proposal 4-1

Proposal 4-1-v0

RAN1 to assess the impacts on initial access during NR NTN GNSS-resilient operation, including: - Initial Timing Advance acquisition - Impact on the Timing Advance Command (TAC) in the Random Access Response (RAR)

Companies are encouraged to share views on Proposal 4-1-v0

| Companies | Comments |
| --- | --- |
| Samsung | This can be discussed later. |
| CATT | Need more accurate proposal based on observations. |
| HONOR | Support. |
| Xiaomi | OK for discussion |
| Sony | Support. |
| Toyota | Too early to discuss this. As pointed out in our contribution, more understanding is required in Step 1 solutions in order to understand the need/value of Step 2 solutions. |
| Tejas Networks | Ok with the proposal but can be discussed later |
| Panasonic | Support. In addition to timing, frequency control should be studied. |
| TCL | Support |
| ZTE | Pre-compensation related enhancement should also be considered, e.g., pre-compensation based on past GNSS information or common pre-compensation. Otherwise, the whole RTT/RTD need to be handled instead of only differential RTT/RTD |
| Sharp | Support |
| ST Engineering iDirect | Agree with the direction of this proposal. |
| LGE | We can discuss this after we have a consensus on the observations from RAN1 and the target scenario is clear. |
| Futurewei | Ok, to discuss. |
| Nokia | Not sure that we need to investigate this right now. |
| Ericsson | The second bullet is more related to solutions and could be added in proposal 8-2 instead. |
| OPPO | Agree with Panasonic that frequency correction should also be considered, e.g., add a sub-bullet that “impact on the frequency adjustment command in the RAR” |
| Apple | Support |
| ETRI | It is too early to discuss. |
| Ofinno | Support the proposal. |
| Spreadtrum | How to handle frequency offset should also be studied. |
| NEC | Initial Timing Advance acquisition is OK. Impact on the Timing Advance Command (TAC) in the Random Access Response (RAR) is too early. |
| Huawei, HiSilicon | We are fine with the 2nd subbullet. |
| NTT DOCOMO | Support |
| Lenovo | Support |
| CMCC2 | General fine with the direction. |

# Topic#5 Impact on connected mode

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Observation 16 When UL pre-compensation is based on a reference location, to keep the TA within timing accuracy limits specified by RAN4, TA commands may need to be sent more frequently than 2-3 times per second. |
| Spreadtrum | Observation 11 Network need to send TAC MAC CE frequently to help UE maintain synchronization if GNSS is unavailable for LEO-600km and 1200km. |
| Thales | Proposal 9: The following table is proposed for LLS parameters for data transmission performance evaluation Parameters; S-band; Ka/Ku-band. Carrier frequency; 2GHz; DL 20GHz. Channel coding scheme; NR channel coding. Subcarrier spacing; 15kHz, 30kHz; 60kHz, 120kHz. Modulation order, Coding rate; To be reported by the companies. Channel estimation; Realistic estimation. Frequency offset; Residual Frequency error after DL synchronisation and compensation relative to cell reference location: FFS. Frequency drift; FFS. Frequency tracking; FFS. UE speed; 3 km/h; 0 km/h, 1000 km/h. Channel model; For GEO (optional): Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO: Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg. TRP antenna configuration; 1Tx; 1Tx. UE antenna configuration; (1, 1, 2) with omni-directional antenna element; VSAT with [60] cm and [20] cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi). Phase noise Model; FFS. Metrics; BLER, Throughput. |
| ZTE | Observation 11: UL frequency error can increase significantly if large GNSS position error is allowed, which impacts gNB decoding performance. Observation 12: The signaling overhead of TA command can increase significantly if large GNSS position error is allowed. |
| CMCC | Observation 6: As the elevation angle changes from 30o to 90o, the gNB needs to send approximately 300 TA update commands to the UE. This high rate imposes a significant signaling overhead and degrades system performance. Proposal 16: It is proposed to study the enhancements to reduce the transmission of TA commands when GNSS is not available. |
| NEC | Observation 1: When the GNSS information in the UE with GNSS capability is temporarily unavailable, the legacy UE timing and frequency pre-compensation cannot work, especially during the initial access. |
| China Telecom | Proposal 3: It needs to study the impacts on TAC adjustment for GNSS resilient UE. |
| Qualcomm | Observation 5: UE with location uncertainty cannot meet the Doppler pre-compensation assumption; the network must provide frequency adjustments to keep UE within a certain limit from the UL carrier frequency (e.g. within ±0.1 ppm) at gNB reception. Observation 6: For a 600 km LEO, a static UE experiences RTT drift due to satellite motion. The gNB may need to issue up to ~20 TA commands per second to maintain a timing alignment within the CP for 15kHz SCS. Proposal 9: RAN1 to study the impact of timing drift on closed loop TA, and potential enhancements to reduce issuance of TA commands for UEs in connected mode. |
| CSCN | Observation 2: When UE has no available GNSS information for a long time, TA adjustments via TAC in the connected mode face critical challenges of excessive signaling overhead and outdated TAC. |

## Summary of companies’ contributions

The proposals and observations from various companies highlight significant challenges and areas for enhancement in maintaining uplink timing and frequency synchronization for NR-NTN in GNSS-resilient scenarios. Ericsson notes that when uplink pre-compensation relies on a reference location, TA commands may need to be sent more frequently than current norms to meet RAN4 requirements. Spreadtrum and CMCC similarly emphasize the need for frequent TA command transmissions, potentially up to 300 updates as the satellite elevation angle changes, which could lead to significant signaling overhead and system performance degradation. Qualcomm underscores that the network must compensate for Doppler and timing uncertainties in the absence of accurate UE location, potentially issuing up to 20 TA commands per second for 600 km LEO scenarios, and recommends that RAN1 investigate enhancements to both closed loop TA and signaling efficiency. ZTE warns about the increased uplink frequency error and signaling overhead when large GNSS position errors are permitted, impacting gNB decoding and system efficiency. NEC observes that legacy timing and frequency pre-compensation methods cannot function when GNSS is temporarily unavailable, especially during initial access. China Telecom and LG propose further studies on TAC adjustment and the use of GNSS assistance information, respectively. CSCN raises the issue of excessive and outdated TAC signaling in scenarios where the UE lacks GNSS information for prolonged periods. Thales proposes a comprehensive table of LLS parameters to support realistic data transmission evaluations for different frequency bands and scenarios

## Initial proposal

### Proposal 5-1

Overall, the contributions point to a consensus on the need to study methods for reducing signaling overhead, improving TA management, and robustly supporting synchronization when GNSS signals are impaired or unavailable.

With the above in mind, the following initial proposal is made:

Proposal 5-1-v0

RAN1 to assess the impacts on connected mode during NR NTN GNSS-resilient operation, including: - Impact on signalling overhead e.g. caused by frequent TA command. RAN1 to study methods for reducing signaling overhead, improving TA management.

Companies are encouraged to share views on Proposal 5-1-v0

| Companies | Comments |
| --- | --- |
| Samsung | It can be discussed later. s |
| CATT | Can be discussed later |
| HONOR | OK |
| Xiaomi | OK for discussion |
| Sony | Ok. Study on methods for reducing signalling to be low priority until evaluations on impact complete. |
| Toyota | Discuss later. As pointed out in our contribution, these solutions are Stage 3 and without understanding Stage 1 and 2 improvements, difficult to discuss. |
| Tejas Networks | Ok with the proposal but can be discussed later |
| Panasonic | Support. In addition to timing, frequency control should be studied. |
| TCL | ok |
| ZTE | Fine to discuss |
| Sharp | Can be discussed later. |
| ST Engineering iDirect | Agree. Remark: when signaling the timing error, rather than the correction, the UE can implement higher-order loops that can cope with drift rate. A second-order loop requires less updates compared to the first-order loop for which results are presented in Fig. 3 in R1-2507719 from Qualcomm. Also, the signaling overhead to signal the reference location should be analyzed. By enhancing closed-loop timing and frequency, the reference location (and even satellite ephemeris) might not be needed in connected mode, so we can decrease the signaling overhead. |
| LGE | We can discuss this after we have a consensus on the observations from RAN1 and the target scenario is clear. |
| Futurewei | OK |
| Nokia | This proposal seems to make an assumption that closed loop TA is adopted. To our understanding there may be different methods for tracking the UE (for instance using closed loop TA/FO commands or (relative) geo-location update commands). None of these should be precluded at this stage of the study. Additionally, with respect to the proposal itself, it is worth noting that the TA control and FO control would be something that goes a bit hand-in-hand (since the satellite is moving and causing things to change as a function of time). |
| Ericsson | The last sentence is more related to solutions and could be added in proposal 9-1 instead. |
| OPPO | We can firstly discuss the mechanism to support GNSS resilient operation, and then identify whether it is necessary to study methods for overhead reduction. |
| Apple | Ok |
| ETRI | Discussed later. |
| Ofinno | Support the proposal. |
| Spreadtrum | Can be discussed later. |
| NEC | Support to discuss later. |
| Huawei, HiSilicon | Fine with the first bullet. Whether method for reducing overhead is necessary depends on the assessment. |
| NTT DOCOMO | Ok to discuss. |
| Lenovo | Support |
| CMCC2 | We are fine with the proposal. |

# Topic#6 Other Impacts

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Observation 17 RACH-less handover is not feasible to support when GNSS is unavailable. Observation 18 The GNSS position is also used in mobility-related procedures such as location/distance-based conditional handover and neighbor cell measurements, SMTC adjustment for neighbor cell SSB measurements and TN cell measurement avoidance. These procedures are out of RAN1 scope but could be studied later in RAN2 or in RAN1 based on input from RAN2. |
| Eutelsat | Observation 5 : For NGSO case the satellite switch procedure is required to prevent a layer 3 signalling storm when all users in a ‘cell’ need to change serving satellite simultaneously. Observation 6: Hard satellite switch with re-synchronization may be supported by any NTN UE with an accurate GNSS location fix, including UE with single beam directional antennas, without restriction. Observation 7: Soft satellite switch with re-synchronization may be supported by NTN UEs with an accurate GNSS location fix with a single beam directional antenna that can switch rapidly between two different satellites (source and target) and/or a by VSAT that supports more than one receive beam. Observation 8: The directional VSAT UE antenna provides spatial discrimination; this enables SAN A and SAN B to reuse the same frequency to illuminate any cell. Observation 9: Once the VSAT UE is in connected mode it already has information from timing advance, Doppler and angle of arrival from the antenna pointing vector that can help to refine its assumed location. Observation 10: If the VSAT UE location is inaccurate there may need to be an additional period of time (overlap time when SAN A and SAN B illuminate the same area) where the UE measures the SSB to find the optimal ‘peak-of-beam’ direction. Observation 11: From the observed and calculated differences the timing and frequency can be adjusted by the UE to be at least ‘approximately correct’ towards the Target SAN at the point of switchover/ handover. Observation 12: It may be possible to calculate an improved location estimate based on measurements including the old and new closed loop feedback adjustments made as a result of satellite handover switchover and for VSAT UE, relative angle of arrival. Proposal 2: RAN1 to study if open loop (measured time difference using the observed ssb-time difference and measured Doppler) and closed loop correction can be used to facilitate UE handover between SANs. |

## Summary of companies’ contributions

## Initial proposal

### Proposal 6-1

FL recommendation

The impacts of GNSS-resilient operation on mobility-related procedures fall within the scope of RAN2. This discussion is currently deprioritized and RAN2 will not be involved during the study phase. If a normative phase is conducted following completion of the study item in June, a liaison with RAN2 regarding these aspects may be established if deemed necessary.

Companies are encouraged to share views on FL recommendation

| Companies | Comments |
| --- | --- |
| CATT | Maybe in later stage one LS is needed. |
| Xiaomi | There is no RAN2 TU for this AI, right? We think firstly RAN plenary needs to discuss on the TU aspects. |
| Sony | Agree to deprioritize. |
| Panasonic | Generally support. But, items for the normative phase should be decided in RAN plenary based on RAN1 study (no need to LS to RAN2). |
| ZTE | Agree that it belongs mobility related issue should be discussed in RAN2. |
| ST Engineering iDirect | Agree that an LS to RAN2 can be sent at a later stage of the study. |
| LGE | No need to make any RAN1 agreement for this issue. Whether RAN2 needs to be involved in support of GNSS resilient operation is up to RAN plenary decision. |
| Nokia | OK – but do we really need to make agreements that something is out of scope? |
| Ericsson | Ok (although it is not up to RAN1 to decide). |
| OPPO | Agree with FL |
| Ofinno | We are fine with the proposal |
| Spreadtrum | support |
| NEC | Agree to deprioritize. |
| Huawei, HiSilicon | Agree with FL assessment |
| NTT DOCOMO | Agree with FL. |
| Lenovo | Agree |
| CMCC2 | Share the similar views that it may have some impacts to other group. But it can be done in the WI phase, when we have a clear conclusion. |

# Topic#7 Potential solutions for PRACH

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Proposal 5 Study UL pre-compensation for initial access based on a reference location, broadcast by gNB, used by the UE as its own location at preamble transmission. Proposal 6 Study conjugated preambles and other PRACH enhancements that do not require physical layer channel/signal changes to enable initial access in the absence of GNSS information at the NTN UE. Proposal 11 Study conjugated preambles and other methods to enable PDCCH order PRACH in the absence of GNSS information at the NTN UE. Proposal 12 PRACH preamble reception with large max differential delay can be handled by implementation-based techniques using a long enough PRACH processing window and multiple timing hypotheses. Proposal 13 A suitable PRACH scheme for LEO when GNSS is unavailable should be composed of two preamble transmissions: the first transmission uses a ZC sequence of an existing NR preamble format, and the second transmission uses a ZC sequence that is the complex conjugate of the first ZC sequence. Note that the complex conjugate of a ZC sequence is a ZC sequence with a root different from the original ZC sequence. Thus, the second transmission is also an existing NR preamble format. The existing Rel-18 PRACH repetition scheme can be used as baseline. |
| Spreadtrum | ## Proposal 1 Cell-level or beam-level common TA and common TA Doppler of service link can be used for UL time/frequency compensation when UE cannot rely on its GNSS for timing and frequency compensation on the service link. |
| Thales | Proposal 1: In NR NTN, when GNSS information is unavailable (ie. the UE cannot derive an accurate position using its GNSS), the UE may set its position to the reference location of the serving cell. FFS: Whether reference location is provided per cell or beam. Proposal 2: In NR NTN, when GNSS information is unavailable, for uplink transmission, the UE pre-compensates the common round trip delay and the common frequency Doppler corresponding to the link between the reference location of the serving cell and the satellite. Proposal 6: RAN1 to identify and evaluate technical solutions to address excessive differential delays which are beyond the detection capability supported by current NR PRACH short formats. |
| vivo | Proposal 1: The following should be studied to support GNSS resilient in RAN1. - How to obtain a proper common TA and common FO. - How to handle residual TA and FO to make UL transmission satisfy the RAN 4 requirement. - Enhanced RACH procedure (if feasible) to handle the timing and frequency errors. - The application condition of the acquired GNSS information. - How to handle the timing and frequency errors after the acquired GNSS information become invalid. - The backward compatibility with the legacy UE. - Closed loop timing adjustments and closed loop frequency adjustments procedure. Proposal 2: For PRACH, study how to provide the common TA and the common FO, e.g., calculated by UE based on a reference location and ephemeris broadcast by the gNB, or based on common TA and common TA drift rate which may be beneficial for unified interface design. |
| ZTE | Proposal 6: The requirement for initial access can be relaxed to support GNSS resilience initial access, e.g., allowing access of UE without valid GNSS position. |
| Huawei | Proposal 1: The timing error limit and frequency error requirement for PRACH transmission in RAN4 can be relaxed/redefined for GNSS resilient scenarios. |
| Xiaomi | Proposal 4. For Doppler proof preamble signal design, optimized roots can be chosen fitting the normalized Doppler frequency value up to a certain threshold k under given constraints on Ncs. |
| CMCC | Proposal 10: Enhanced PRACH formats and/or preamble sequences, including update of PRACH resource configuration, will not be specified in the WI phase in the Rel-20 GNSS resilient NR-NTN operation. |
| CATT | Proposal 1: In 5G-A framework, the study to address the maximum timing errors and frequency errors should be limited to parameters configuration of PRACH transmission, rather than PRACH format change. Proposal 2: Introducing TA margin and additional GT will be a candidate solution to address GNSS positioning errors, which has the least impact on the existing specification. |
| OPPO | Proposal 1: For GNSS resilience operation, PRACH is transmitted with a common TA pre-compensation which is conducted with assumption on the near-UE of a satellite beam footprint for service link. Proposal 2: For UL time synchronization during initial access procedure, UE should be indicated whether GNSS resilience operation is supported by configured PRACH format of the cell. - If supported, UL time synchronization for Msg3 PUSCH transmission can be obtained by residual UE-specific TA indication in RAR message. Proposal 4: For GNSS resilience operation, PRACH is pre-compensated/post-compensated with a common frequency offset which is conducted with assumption on the center-UE of a satellite beam footprint for service link. Proposal 5: For UL frequency synchronization during initial access procedure, UE should be indicated whether GNSS resilience operation is supported by configured PRACH format of the cell. - If supported, introduce residual UE-specific frequency offset indication in RAR message for Msg3 PUSCH transmission. |
| Tejas Networks | Proposal 2: The beam-specific reference point used for common Timing Advance (TA) calculation should be defined by the network such that it represents the location within the beam contour closest to the satellite Proposal 3: The UE should pre-compensate the common doppler with respect to the beam centre which is the reference point signalled by the network to the UE Proposal 7: The proposed Dual Format PRACH uses the existing formats as that of the legacy NR PRACH and can be configured such that two formats are separated in either time or frequency with minimal procedural changes to the existing Initial Access setup. Proposal 8: The RACH Occasion (RO) should be defined to accommodate the maximum differential delay relative to the beam-specific reference point. Although the actual signal duration for each PRACH transmission is approximately 1 ms, the overall RO duration is determined by the maximum differential delay. Proposal 9: Robust performance for preamble detection can be achieved by the proposed Dual Format PRACH in the worst-case scenarios of differential delay and residual doppler values in NTN Scenarios where GNSS is unavailable. |
| InterDigital | Proposal 7: Investigate the NTN operation whereby the UEs perform time-frequency compensation based upon a reference location and may receive UE specific adjustments from the network. Proposal 8: Investigate the impact of the time frequency residual error when a UE performs initial access based upon a reference location for the satellite beam. Proposal 9: For GNSS resilient initial access, prioritize the investigations where UEs perform time-frequency compensation based upon a reference location for the satellite beam. |
| Samsung | Proposal 2: RAN1 to study enhancements to initial access procedures to improve robustness under GNSS temporarily unavailable conditions, including gNB-assisted PRACH configuration adaptation. |
| NEC | Proposal 2: RAN1 considers SIB9 timestamp-based TTA estimation as a timing pre-compensation solution for an NTN terminal under GNSS resilient mode. |
| LG Electronics | Proposal #3: In Rel-20 NR-NTN, RAN1 studies cell- or beam-specific reference information for TO/FO pre-compensation for GNSS-resilient operation. - FFS: format of the reference information (e.g., formulation, reference point) - FFS: whether/how to align the validity duration with common TA information Proposal #5: In Rel-20 NR-NTN, RAN1 studies the feasibility of using downlink Doppler measurement for uplink FO pre-compensation. |
| Ofinno | Proposal 2: The network broadcasts a reference TA for the UEs in Scenario #1 or in Scenario # 2, to adjust their uplink timing for transmitting PRACH. Proposal 3: The network broadcasts one reference TA per beam or one reference TA per group of beams in an NTN cell with multiple beams. |
| ETRI | Proposal 8. RAN1 to define a baseline scheme for GNSS resilient operation among the following implementation-/non-3GPP-based solutions: - Option 0: NTN UL transmission is not allowed when GNSS is unavailable (Rel-19 UE behaviour) - Option 1: Dual-(multi-)GNSSs - Option 2: Dual-(multi-)frequencies - Option 3: OS-NMA - Option 4: UL time/frequency compensation via DL synchronization and signals - Option 5: LPP-based approach (note: no enhancements on LPP itself) - FFS, other options and/or a combination above Proposal 9. RAN1 to study the following specification impacts for GNSS resilient operations: - Option 1: (New) timer (like IoT-NTN, RAN2) - Option 2: Introduce new timestamp(s) in SIB19 and/or SIB9 (RAN2, RAN1) - Option 3: Reference location-based UL synchronization (RAN1, RAN2) - Option 4: Requirement relaxation (RAN4) - Option 5: Gradual timing adjustment (RAN4) - Option 6: Expansion of closed-loop compensation into frequency domain (RAN1, RAN2) - Option 7: New procedures to detect and/or report GNSS availability (RAN1, RAN2) - Option 8: New procedures to request, configure, and/or indicate GNSS resilient operation(s) (RAN1, RAN2) - FFS, other options and/or a combination above |
| Lenovo | Proposal 1: RAN1 to study the use of system time reference for internal clock offset and delay estimation in case of UE’s clock previously not synchronized with the network. Proposal 2: RAN1 to study the feasibility of providing time and frequency pre-compensation values or reference point through SIB for uplink synchronization during initial access. Proposal 3: RAN1 to study the iterative RACH procedure with updated TA and frequency compensation values after each of the failed RACH attempts. Proposal 4: RAN1 to study the use of historic data during initial access to compensate for time and frequency errors in uplink for shorter GNSS outages. |
| Nokia | Proposal 2: In case of temporary unavailable GNSS information during initial access, UE uses previously acquired UE location information for calculation of NTA,adjUE and FO pre-compensation values to perform initial access if the time from the previously acquired GNSS location is within time boundaries. |
| Sony | Proposal 3: Methods for UEs to either self-correct, or be corrected by gNBs when attempting to RACH using incorrect pre-compensation parameters should be studied. Proposal 4: Prioritize the study on reference location enhancements for time/frequency pre-compensation. Proposal 5: Study the validity time of reference locations, including validity time of any potential enhancement methods related to reference locations. Proposal 6: Twin RACH preamble approaches should be studied to improve initial access success rate and provide additional doppler shift calculation capabilities. Proposal 7: RAN1 to define performance metrics for PRACH performance assessment, such as - Mis-detection rate - Frequency estimation error CDF - Timing estimation error CDF Proposal 8: Dual RACH procedures should be investigated to independently address TA and Doppler compensation, incorporating enhanced Msg2 signaling for extended TA ranges, frequency adjustments, and position updates. |
| MediaTek | Proposal 4: Study the following options in case of UE over compensation when transmitting RACH: - Negative TAC indication in Msg2 - Margin with negative TA offset for RACH preamble transmission - Combination of options |
| Toyota ITC | Proposal 3: For uplink time and frequency pre-compensation, multiple reference locations is adopted as a candidate technique for GNSS resilient operation. Proposal 4: The multiple reference locations-based time and frequency pre-compensation is adopted as a candidate technique in the Step 1 of the proposed 3-step unified uplink synchronization framework. Proposal 5: Under GNSS resilient operation, supplementary downlink reference signals are considered. To avoid physical layer channel/signal change as per the agreed SID objective and to reduce to standardization efforts, NR supported DL reference signals such as PRS, NCD-SSB, CSI-RS, DMRS can be reused. Proposal 6: Under GNSS resilience operation of NR NTN UE, supplementary downlink reference signal configurations are indicated to the UE by adopting the NR signaling framework or can be specified in the specification. Proposal 7: The supplementary reference signals-based uplink time and frequency pre-compensation is adopted as a candidate technique in the Step 1 of the proposed 3-step unified uplink synchronization framework. Proposal 8: PRACH design options in TR 38.821 Section 6.3.3 that are within the scope of the SID are studied under the proposed 3-step unified framework for uplink time and frequency synchronization. Proposal 9: To improve the preamble detection performance, random access power control is revisited for PRACH design under Scenario 1 and 2 of the GNSS resilient operation. Proposal 10: Multiple PRACH transmissions and PRACH power control are studied under the Step 2 of the proposed 3-step uplink synchronization framework. Suitable PRACHs are chosen out of the various possible NR PRACH designs based on NR supported parameters such as sequence, format, type, repetition number, restricted sets etc. Proposal 11: Evaluate Step 2 options such as multiple PRACH transmissions and power control to identify the candidate techniques with sufficient performance and minimum standard impact. |
| Sharp | Proposal 2: For long preamble formats, the UE may transmit multiple preambles with different TA pre-compensations under a power ramping counter. Proposal 3: NTN UE can define extended RO region in consecutive RO configurations for short preamble transmissions and RAR detection. Proposal 4: Partial preamble detection in a RO region and cross RO preamble detection in consecutive ROs should be supported at the gNB. |
| Qualcomm | Proposal 6: RAN1 to study the following methods for PRACH enhancements: - Option 1: New PRACH restricted sets: - Option 1-A: New PRACH restricted sets based on maximum differential RTT-RTD within the location uncertainty area. - Option 1-B: New PRACH restricted sets based on joint distribution of differential RTT-RTD within the location uncertainty area. - Option 2: Solutions based on multiple PRACH / Msg-3 transmissions: - Option 2-A: Transmission of multiple PRACH with the same root sequence and receiving multiple responses. - Option 2-B: Transmission of a single PRACH followed by reception of multiple Msg-3 grants in the RAR with different time–frequency corrections. - Option 2-C: Transmission of two PRACH using a pre-determined root pair and receiving a single response. |
| Johns Hopkins University | Proposal 1: RAN1 should study supporting initial access to low-elevation angle scenarios where the differential delay is higher than in high-elevation angle scenarios. Proposal 3: RAN1 should study the time variations of the Doppler frequency and support a closed-loop for updating the Doppler frequency at the UE. Proposal 4: RAN1 should study using the timestamps in ReferenceTimeInfo in SIB9 to estimate the initial Doppler frequency. |
| NTT DOCOMO | Proposal 2: For GNSS resilience, - Study enhancements on UL time/frequency synchronization - Study the necessity of enhancements on initial access, including whether the existing PRACH with the enhanced UL time synchronization can work. |
| CAICT | Proposal 2: Allow differentiated levels of uplink time-frequency synchronization requirements for UEs with different GNSS capability levels. |
| CSCN | Proposal 2: For Scenario 1, the gNB may broadcast assistance information (e.g., RP position) for the UE to calculate and compensate the common TA, while the differential offset could be estimated and corrected through PRACH. |
| Google | Proposal 1: PRACH repetition should be studied for synchronization without GNSS position information. Proposal 2: For evaluating the performance of UE speed 1500 km/hr case, the baseline PRACH set includes preambles generated from individual root sequences (i.e., 0 cyclic shift). |
| CEWiT | Proposal 1: The satellite should compute reference TA using its location and the geometry of the cell/beam. The reference TA is TA between the satellite and reference point on the ground (RP2). Proposal 2: The satellite should transmit the reference TA in SIB1 for the UE’s preamble transmission. Proposal 7: The satellite should provide TA adjustments required to achieve precise uplink synchronization to the UE through the RAR message. Proposal 8: Enhancement of Timing Advance should be considered to support negative corrections in the RAR message. |

## Summary of companies’ contributions

There is strong alignment across most companies on the need for reference location-based TA/FO pre-compensation (with variations on how, where, and what level of granularity).

On potential solutions for PRACH, PRACH repetition, dual preamble design, and flexible compensation windows are also widely proposed for addressing the challenges related to GNSS-denied scenarios. Enhancements are generally preferred to avoid new physical channels/signals, favoring configuration/procedural improvements and leveraging existing NR specs.

Main proposed solutions:

- Reference Location & Common Pre-Compensation

  - Thales, vivo, OPPO, Tejas Networks, Spreadtrum, LG, InterDigital, Lenovo, Nokia, Ofinno, CEWiT, CSCN

    - Use of reference location (cell, beam, or network-provided) by UEs for initial uplink time and/or frequency pre-compensation in the absence of valid GNSS

    - UEs use reference location (specified by gNB/network) to pre-compensate timing advance (TA) and/or frequency offset (FO).

    - Studies on whether reference location is provided per cell, beam, or group, and how often it should be updated (validity duration).

    - Network may broadcast this reference info via SIB messages or RAR.

    - Multiple reference locations (Toyota ITC, Ofinno, CEWiT) suggested for improved accuracy in large beams or cells.

    - TA/FO may be provided per-beam, cell, or group.

- Doppler-proof and Dual-format/Conjugated Preambles/multiple PRACH:

  - Ericsson, Tejas Networks, Sony, Xiaomi, Thales, Lenovo, Toyota, Qualcomm, Google

    - Propose/recommend conjugated or twin PRACH preambles, dual formats, or root-sequence optimization for Doppler resilience.

    - Dual-preamble approaches (e.g., conjugated ZC sequences, multiple roots, separated in time/frequency).

    - PRACH repetition, iterative RACH procedure

    - Enhanced PRACH for robust detection under large differential delay, Doppler shift, or high-mobility.

    - User equipment (UE) may transmit multiple preambles with different compensations.

    - Multi-RO or extended RO for supporting wide delay spread.

    - Partial or cross-RO preamble detection at the gNB.

- PRACH Resource configuration:

  - CMCC, Qualcomm, Ericsson, CEWiT, Tejas Networks, CATT, Sharp

    - Push for minimal upgrade.

    - Development of new restricted sets.

    - TA margin and additional GT: CATT

- Wider Detection Windows / Multi-hypothesis Processing:

  - Ericsson, Tejas Networks, Sony, Lenovo, Sharp

    - Recommend extending PRACH processing windows to capture large delays.

    - Multiple timing hypotheses for initial access.

- Relaxation of timing/frequency requirements for PRACH transmission:

  - ZTE, Huawei, Thales, CAISCT

    - Requirement for initial access can be relaxed to support GNSS resilience initial access.

- Negative TA/TA marign:

  - MediaTek, CEWiT

    - Support for negative TA (over-compensation) to avoid missing preambles.

    - May require new signaling (negative TA in Msg2/RAR).

- Other potential solutions:

  - NEC, ETRI, Lenovo, Toyota, Johns Hopkins University

    - SIB9 timestamp-based TTA estimation

    - new timestamp(s) in SIB19 and/or SIB9

    - internal system time reference for clock

    - supplementary downlink reference signals (Toyota)

## Initial proposal

### Proposal 7-1

Proposal 7-1-v0

For the study on GNSS resilient NR-NTN operation, RAN1 to consider the following enhancements on the PRACH: - Dual PRACH transmission (with conjugated ZC sequences) - New PRACH restricted sets - Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay. - Relaxation of timing/frequency requirements for PRACH transmission - Liaison with RAN4 maybe needed - Other potential solutions are not precluded

Companies are encouraged to share views on Proposal 7-1-v0

| Company | Comments |
| --- | --- |
| Samsung | Without observations, it is premature to consider this proposal. |
| CATT | We don’t support it. It is premature. Moreover, FL doesn’t capture our solutions, that is as follows: - Define the TA margin and extended GT to handle larger timing difference. |
| HONOR | It can be discussed later. |
| Xiaomi | We think this list does not provide a full list of solutions. Our solution on optimized roots are not captured. We are Ok to further discuss this after the stabilization of Proposal 3-1-v0. |
| Sony | Agree that this list is not exhaustive and discussion needed. In particular, common pre-compensation studies. |
| Toyota | Too early to decide these options. These listed Options and other options can be considered later. As pointed out in our contribution, there are solutions to consider at Step 1 (e.g., multiple reference locations, downlink reference signals etc) and thus, without properly understanding such Step 1 solutions properly, it is premature to consider the options for Step 2. |
| Tejas Networks | No need to specifically mention “with conjugated ZC sequences)” so that all possible solutions based on the Dual PRACH transmission is captured in the option 1 of the proposal |
| JHUAPL | Prefer more discussions on this topic. |
| Panasonic | We think relaxation of the timing/frequency requirements for PRACH transmission is necessary for both scenario 1 and scenario 2. But, the other items are not necessary for scenario 2. We prefer to prioritize scenario 2 rather than studying various PRACH solutions for scenario 1. |
| TCL | Agree with Xiaomi, we can discuss proposal 3-1-v0 first. |
| ZTE | As mentioned in SID, the study aims to minimize physical layer procedures impact and avoids physical layer channel/signal changes. Hence, the enhancements in first and second bullets are out of scope. Moreover, even if dual PRACH transmission is studied, there can be multiple potential solutions and no need to be restricted to conjugated ZC sequences. Regarding third bullet, different timing and frequency error may be observed by different implementation, which makes it hard to define corresponding requirements. Regarding bullet 4, generally fine but the relaxed requirements should still be within the tolerable range of the PRACH format. |
| Sharp | This can be discussed after PRACH issues are evaluated. Existing PRACH formats and configurations should not be changed as baseline assumptions. |
| ST Engineering iDirect | We support the proposal from the FL to start defining/listing potential solutions. Agree with Xiaomi that we should not exclude solutions at this point, but this is already captured in the last bullet “other potential solutions are not precluded”. Rather than spending time on the wording of this proposal, we propose to have an agreement that all potential solutions should be defined at RAN1#123 (Dallas), and that no new (potential) solutions will be proposed at later meetings. Proposal: “Define potential PRACH enhancements for GNSS resilient NR-NTN operation at RAN1#123. Solutions not presented in Dallas will not be considered at later stages of this study.” |
| LGE | We can discuss this after the discussion on Proposal 3-1-v0 is completed. |
| Nokia | Some of these enhancements would not be feasible considering the limitations from the SID. For instance, implementation based techniques may be seen as “pushing the burden to the gNB side”, which we would strongly object to. Further restricting the PRACH may also be troublesome since PRACH capacity will become exhausted for some cells. Further, relaxation of RAN4 requirements would not be useful at all. It will simply cause UL signals to become more “unaligned”, meaning less detection probability. |
| Ericsson | Support |
| OPPO | Agree with Samsung |
| Apple | It’s too early to down select the solution without the performance evaluation. |
| ETRI | It’s too early to discuss. |
| Ofinno | We are fine with the proposal |
| Spreadtrum | It is too early to discuss enhancement before we identify issues of current PRACH format |
| Huawei, HiSilicon | Except for relaxation of RAN4 requirement, we think the others are out of SID scope |
| Lenovo | In general, we are supportive of the direction of the proposal. However, the proposal does not align with the summary of proposed solutions. We should capture either all of the proposed solutions or at least those solutions that have majority support. |
| CMCC2 | It is premature to discuss enhancements on the PRACH, and we think enhanced PRACH formats and/or preamble sequences, including update of PRACH resource configuration, are not in the scope. |

# Topic#8 Potential solutions for initial access

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Proposal 7 Study signalling enhancements in Msg2 aiming at correcting time/frequency offsets during initial access, including: • Enhanced TA command (e.g., extended range and/or negative range) • Frequency adjustment command • Position adjustment command (indicating an UE-specific reference location to the UE) |
| Spreadtrum | ## Proposal 2 Closed-loop adjustment mechanism for frequency offset can be considered both for initial access procedure and RRC connected mode. |
| Thales | Proposal 7: Following approach to support the initial TA acquisition for UL transmission can be studied in NTN with degraded or unavailable GNSS fix: - UE pre-compensates the common RTT on the service link, relative to referenceLocation - Reference location of the serving cell provided via NTN (quasi)-Earth fixed cell - movingReferenceLocation - Reference location of the serving cell of an NTN Earth-moving cell at a time reference. Time reference of this field is indicated by epochTime in ntn-Config of the serving cell. - Additional UE-specific TA indicated in RAR. FFS: The need and support of negative TAC values in RAR. |
| vivo | Proposal 1: The following should be studied to support GNSS resilient in RAN1. - How to obtain a proper common TA and common FO. - How to handle residual TA and FO to make UL transmission satisfy the RAN 4 requirement. - Enhanced RACH procedure (if feasible) to handle the timing and frequency errors. - The application condition of the acquired GNSS information. - How to handle the timing and frequency errors after the acquired GNSS information become invalid. - The backward compatibility with the legacy UE. - Closed loop timing adjustments and closed loop frequency adjustments procedure. Proposal 4: Study signal enhancement in RAR to indicate a frequency offset for msg 3 frequency offset compensation. Proposal 5: Study TA command enhancement in RAR (e.g., enlarge the maximum range of timing adjustment, or introduce negative TA). |
| ZTE | Proposal 6: The requirement for initial access can be relaxed to support GNSS resilience initial access, e.g., allowing access of UE without valid GNSS position. |
| Huawei | Proposal 4: At least a plus-minus sign should be introduced for the absolute TAC, including the TAC in random access response. Proposal 5: In the 4-step RACH, timing and frequency adjustments can be introduced after Msg2 during initial access to ensure the UL synchronization of Msg4-ACK and Msg5. Proposal 6: The closed-loop frequency adjustment should be introduced in both initial access and RRC CONNECTED. |
| Xiaomi | Observation 3. The closed loop frequency adjustment during initial access and frequency maintaining in connected mode can be performed either at gNB side or UE side. In case the value for frequency adjustment is controllable, the gNB side frequency error mitigation does not lead to much performance degradation while simplifies UE behavior and specification impact. |
| CATT | Proposal 3: A frequency adjustment control (FAC) mechanism similar to the timing adjustment in the RACH response should be introduced. |
| TCL | Proposal 3: The enhancement of the closed-loop time control can be studied, for example, TAC in RAR can indicate a negative value. Proposal 4: Frequency closed-loop adjustment can be considered for GNSS-resilient NR-NTN. |
| OPPO | Proposal 2: For UL time synchronization during initial access procedure, UE should be indicated whether GNSS resilience operation is supported by configured PRACH format of the cell. - If supported, UL time synchronization for Msg3 PUSCH transmission can be obtained by residual UE-specific TA indication in RAR message. Proposal 5: For UL frequency synchronization during initial access procedure, UE should be indicated whether GNSS resilience operation is supported by configured PRACH format of the cell. - If supported, introduce residual UE-specific frequency offset indication in RAR message for Msg3 PUSCH transmission. |
| HONOR | Proposal 4: RAN1 to study the following enhancements for initial access: - Support of enhanced TAC in RAR. - Support of frequency corrections in RAR for Msg3 transmission. |
| InterDigital | Proposal 8: Investigate the impact of the time frequency residual error when a UE performs initial access based upon a reference location for the satellite beam. Proposal 9: For GNSS resilient initial access, prioritize the investigations where UEs perform time-frequency compensation based upon a reference location for the satellite beam. |
| Samsung | Proposal 2: RAN1 to study enhancements to initial access procedures to improve robustness under GNSS temporarily unavailable conditions, including gNB-assisted PRACH configuration adaptation. |
| LG Electronics | Proposal #6: In Rel-20 NR-NTN, RAN1 clarifies whether a new RAR payload is required to support TO/FO correction information. |
| Fraunhofer | Proposal 2: RAN1 to study specifying a frequency closed-loop mechanism during initial access and connected mode. Observation 4: RAR message doesn’t have available bits to indicate the frequency offset command as part of a frequency correction closed-loop mechanism. Observation 5: The extension of a RAR message to indicate the frequency offset corrections as part of a frequency correction closed-loop mechanism may cause backward compatibility issues with pre rel-20 NTN UEs. Proposal 3: RAN1 to study solutions to indicate frequency correction commands during initial-access and RRC-Connected mode as part of a frequency correction closed-loop mechanism. |
| Lenovo | Proposal 2: RAN1 to study the feasibility of providing time and frequency pre-compensation values or reference point through SIB for uplink synchronization during initial access. Proposal 4: RAN1 to study the use of historic data during initial access to compensate for time and frequency errors in uplink for shorter GNSS outages. Proposal 6: Study the need to extend the RAR window length and update the starting position of RAR window for UEs without GNSS. Proposal 7: RAN1 to study the benefit of using a frequency correction command in Msg2 similar to TAC for finer frequency adjustments, larger TA command range and negative TA for UEs without GNSS availability. |
| Sony | Proposal 8: Dual RACH procedures should be investigated to independently address TA and Doppler compensation, incorporating enhanced Msg2 signaling for extended TA ranges, frequency adjustments, and position updates. |
| MediaTek | Proposal 4: Study the following options in case of UE over compensation when transmitting RACH: - Negative TAC indication in Msg2 - Margin with negative TA offset for RACH preamble transmission - Combination of options Proposal 5: Study closed-loop frequency adjustment during initial access and RRC connected mode. |
| Toyota ITC | Proposal 1: To accommodate the GNSS resilient operation, RAN1 to study a unified framework for uplink time/frequency synchronization which encompasses initial access and connected state procedures, configurations and parameters. |
| Sharp | Proposal 3: NTN UE can define extended RO region in consecutive RO configurations for short preamble transmissions and RAR detection. Proposal 5: Negative TA value and extended TA value range can be considered in RAR for GNSS resilience operation. |
| Apple | Proposal 1: To support initial access of a UE without valid GNSS information, consider network indicating frequency offset in Msg2. |
| Qualcomm | Proposal 7: RAN1 to study methods to signal timing and frequency adjustments in RAR for msg-3 transmission. |
| Johns Hopkins University | Proposal 1: RAN1 should study supporting initial access to low-elevation angle scenarios where the differential delay is higher than in high-elevation angle scenarios. |
| NTT DOCOMO | Proposal 3: For Rel-20 NTN GNSS resilience, regarding initial access, study the following: - Initial TA acquisition - Enhancement on TAC in RAR, e.g., increased/negative value range for TA command, etc. - Frequency adjustment mechanism |
| CSCN | Proposal 5: The following options could be considered to deal with the TA over-compensation issue. - Option 1: Introduce the TA margin - Option 1-1: Network indication, - Option 1-2: Up to UE implementation. - Option 2: Introduce an advanced preamble detection window - Option 2-1: Extend the range of TAC in RAR to support negative values, - Option 2-2: Up to the network implementation. Proposal 8: Necessary optimizations of the preamble reception and RAR windows should be introduced |
| CEWiT | Proposal 7: The satellite should provide TA adjustments required to achieve precise uplink synchronization to the UE through the RAR message. Proposal 8: Enhancement of Timing Advance should be considered to support negative corrections in the RAR message. |
| ST Engineering iDirect | Observation 3: The current specifications define a 6-bit TA MAC CE and 12-bit absolute TA MAC CE to signal timing correction from the gNB to the UE for closed-loop control. |

## Summary of companies’ contributions

Several companies propose study of enhancing TA/frequency adjustment signaling in RAR/Msg2 for GNSS-resilient initial access, including extended/negative ranges and closed-loop mechanisms.

## Initial proposal

### Proposal 8-2

Based on the above, the initial Proposal is made:

Proposal 8-2-v0

For the study on GNSS resilient NR-NTN operation, RAN1 to consider the following enhancements for initial access: - Enhanced TA command (e.g., extended range and/or negative values) - Frequency corrections in RAR for msg-3 transmission

Companies are encouraged to share views on Proposal 8-1-v0

| Company | Comments |
| --- | --- |
| CATT | It is too early. |
| HONOR | Support. |
| Xiaomi | We can have this discussion after Proposal 4-1-v0 is stabilized. |
| Sony | Support. |
| Toyota | Too early to discuss the Step 3 solutions |
| Tejas Networks | Support the proposal but can be discussed later |
| JHUAPL | Needs further discussions. |
| CEWiT | If the reference point is placed at the center of the beam, as mentioned in proposal 1.4.1, negative TA values would be required. Therefore, this depends on the agreement reached in the first proposal. Moreover, extending the range may not be necessary, since the existing TA mechanism already supports up to 300 km, corresponding to a maximum TA value of 2 ms. Therefore, this proposal can be discussed later. |
| Panasonic | Support. |
| TCL | Support |
| ZTE | Whether to consider first bullet depends on the assessment result of proposal 4-1. Hence, the first bullet can be discussed after assessment. |
| Sharp | Support |
| Qualcomm | The first bullet seems redundant if we agree on Proposal 4.1-v0. |
| ST Engineering iDirect | We support this proposal, but it might be too early to discuss. Suggest to first evaluate the impact for initial access. |
| LGE | We can discuss this after we have a consensus on the observations from RAN1 and the target scenario is clear. |
| Nokia | We find it too early to discuss such details. As we see it, it is more important that we consider the aspect of “how to minimize the control overhead in general” – and here we should keep in mind that the existing method of GNSS based operation is having zero overhead. Any solutions that are showing similar behavior (like geo-location based) should be evaluated with higher priority. |
| Ericsson | Support. Add "other enhancements are not precluded". |
| OPPO | Support |
| Apple | ok |
| ETRI | It is too early. |
| Ofinno | We are fine with the proposal |
| Spreadtrum | How to indicate frequency corrections need further study |
| Huawei, HiSilicon | Support. In addition, we also think the TAC and FC for msg4 ACK and msg5 should be considered due to the long latency from RAR. |
| NTT DOCOMO | Support. |
| Lenovo | Support |
| CMCC2 | It may be too early to discuss the solutions without drawing observation from the study. And the solutions should be listed and clarified. |

# Topic#9 Potential solutions for connected mode

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Proposal 8 Study pre-compensation for RRC_CONNECTED state based on a reference location, broadcast by gNB, used by the UE as its own location for UL transmission. Proposal 9 Study signalling enhancements (MAC CE) aiming at correcting time/frequency offsets in RRC_CONNECTED state, including: • Enhanced TA command (e.g., TA drift rate and TA drift variation rate) • Frequency adjustment command • Position adjustment command (indicating UE-specific reference location to the UE) Proposal 10 Study the GNSS operations specified for IoT-NTN in Rel-18 to see if any solutions can be adopted for GNSS-resilient operation of NR-NTN. |
| Spreadtrum | ## Proposal 2 Closed-loop adjustment mechanism for frequency offset can be considered both for initial access procedure and RRC connected mode. |
| Thales | Proposal 8: For GNSS resilient physical layer operation, study the following options: - Option 1: NTA, adjcommon should include the network-controlled common TA (as in Rel-17) and CommonTARefLoc which is the common RTD between the referenceLocation and the satellite. - Option 2: The differential TA is under network control (closed loop) via RAR and MAC CE TAC, that is included within NTA. - Option 3: The differential TA is under UE control (open loop), that is included within NTA,offset. FFS: How UE determines the timing drift Proposal 10: For UL frequency compensation in case of degraded or unavailable GNSS position fix: - Both open loop and closed loop UL frequency control can be studied - Compensation of common frequency offset at UE side - Estimation of common frequency offset and pre-compensation at UE side is conducted based on Reference location of the serving cell and satellite ephemeris. - Closed-loop method for UL frequency compensation, e.g., the gNB will signal a frequency adjustment command to UE |
| vivo | Proposal 6: Study the necessary of enhancement on timing adjustment command to maintain timing synchronization in the connected mode. Proposal 7: Study frequency offset indication for closed loop frequency adjustment to maintain frequency synchronization in the connected mode. Proposal 8: Study the TA command and/or FO command indication mechanism, and the overhead reduction should be considered. |
| ZTE | Proposal 8: To improve GNSS resilience in connected mode, indication of frequency command from network to UE can be supported. Proposal 9: To reduce overhead of TA command, indication of TA drift rate from network to UE can be supported. Proposal 11: gNB need to indicate UE the extension time for UL transmission when UE without valid GNSS position is allowed to transmit in connected mode. |
| Huawei | Observation 13: The existing close loop TA adjustment can alleviate the timing error for UL transmission other than PRACH in GNSS resilient scenarios. Observation 15: The timing advance command MAC CE can be used to reduce timing error caused by UE movement. Proposal 6: The closed-loop frequency adjustment should be introduced in both initial access and RRC CONNECTED. |
| Xiaomi | Proposal 5. Multi-stage frequency compensation can be applied for under large frequency error case. Observation 3. The closed loop frequency adjustment during initial access and frequency maintaining in connected mode can be performed either at gNB side or UE side. In case the value for frequency adjustment is controllable, the gNB side frequency error mitigation does not lead to much performance degradation while simplifies UE behavior and specification impact. |
| CMCC | Proposal 16: It is proposed to study the enhancements to reduce the transmission of TA commands when GNSS is not available. |
| CATT | Proposal 4: For RRC Connected mode, potential optimization for TAC and FAC adjustment is needed. |
| TCL | Proposal 4: Frequency closed-loop adjustment can be considered for GNSS-resilient NR-NTN. |
| OPPO | Proposal 3: For UL time synchronization in connected mode, reuse legacy close-loop TA update procedure, and study the mechanism how to make gNB be aware of temporarily unavailable of GNSS capability at UE side. Proposal 6: For UL frequency synchronization in connected mode, enhancements on compensation of residual UE-specific frequency offset are not needed. |
| HONOR | Proposal 6: RAN1 to study the enhancement in closed-loop timing adjustment during RRC connected mode for GNSS-resilient operation. Proposal 7: RAN1 to study the following enhancements for frequency compensation during RRC connected mode: - For open-loop frequency control, the UE compensates the frequency based on the measurement of the downlink reference signal. - For close-loop frequency control, the NW indicates frequency corrections to the UE. |
| InterDigital | Proposal 6: Investigate efficient acquisition of the UE’s GNSS availability at the network side to trigger closed loop time-frequency compensation operation. |
| Samsung | Proposal 5: RAN1 to study gNB-based and satellite-assisted Doppler and delay compensation mechanisms for UEs experiencing GNSS loss during connected mode, including the use of reference position information broadcast via SIB. |
| NEC | Proposal 2: RAN1 considers SIB9 timestamp-based TTA estimation as a timing pre-compensation solution for an NTN terminal under GNSS resilient mode. |
| LG Electronics | Proposal #5: In Rel-20 NR-NTN, RAN1 studies the feasibility of using downlink Doppler measurement for uplink FO pre-compensation. Proposal #8: In Rel-20 NR-NTN, RAN1 studies mechanisms to measure and compensate for UE-specific residual TO/FO errors in connected mode, including the use of uplink and/or downlink-based estimation methods. Proposal #9: In Rel-20 NR-NTN, RAN1 studies whether/how to integrate UE-specific TO/FO correction control in connected mode. Proposal #10: In Rel-20 NR-NTN, RAN1 studies procedures for uplink synchronization validity and recovery in connected mode. |
| ETRI | Proposal 9. RAN1 to study the following specification impacts for GNSS resilient operations: - Option 1: (New) timer (like IoT-NTN, RAN2) - Option 2: Introduce new timestamp(s) in SIB19 and/or SIB9 (RAN2, RAN1) - Option 3: Reference location-based UL synchronization (RAN1, RAN2) - Option 4: Requirement relaxation (RAN4) - Option 5: Gradual timing adjustment (RAN4) - Option 6: Expansion of closed-loop compensation into frequency domain (RAN1, RAN2) - Option 7: New procedures to detect and/or report GNSS availability (RAN1, RAN2) - Option 8: New procedures to request, configure, and/or indicate GNSS resilient operation(s) (RAN1, RAN2) - FFS, other options and/or a combination above |
| Fraunhofer | Proposal 1: RAN1 to study enhancements to timing correction closed loop mechanism to support an NTN operation in which GNSS signal is temporarily unavailable or performance degraded. |
| Lenovo | Proposal 8: RAN1 to study the use of historic data in connected state to predict the continuous change in propagation delay and Doppler errors over a shorter GNSS outages duration. Proposal 10: RAN1 to check the feasibility of configuring frequent timing and frequency correction commands by the network, when a UE experience a GNSS outages in connected state. Proposal 11: RAN1 to study disabling TA reporting for GNSS resilient UE. |
| Nokia | Proposal 3: In case of temporary unavailable GNSS information during RRC_CONNECTED mode, a UE uses the latest measured GNSS location for calculation of NTA,adjUE and FO pre-compensation values for UL transmissions at least until instructed otherwise by the network. Proposal 4: RAN1 to investigate various options for implementing closed loop commands for UE maintaining UL TA and FO synchronization. Such options may consider at least (a) geo-location based tracking commands or (b) direct TA/FO commands. |
| MediaTek | Proposal 5: Study closed-loop frequency adjustment during initial access and RRC connected mode. |
| Toyota ITC | Proposal 1: To accommodate the GNSS resilient operation, RAN1 to study a unified framework for uplink time/frequency synchronization which encompasses initial access and connected state procedures, configurations and parameters. Proposal 12: For maintaining uplink synchronization, existing NR supported closed-loop/connected state techniques are studied under the Step 3 of the proposed 3-step uplink synchronization framework. |
| Apple | Proposal 3: For a UE in RRC connected mode, RAN1 to study TA calculation formula in case the UE does not have valid GNSS information. Proposal 4: For a UE in RRC connected mode, RAN1 to study whether TA command scheme needs to be enhanced in case the UE does not have valid GNSS information. Proposal 5: For a UE in RRC connected mode, RAN1 to consider the frequency offset indication from network in case the UE does not have valid GNSS information. Proposal 6: For a UE in RRC connected mode, consider both “legacy open and closed loop TA mode” and “enhanced TA mode”, as well as mode synchronization between UE and network. |
| Qualcomm | Proposal 8: RAN1 to study procedures to provide closed-loop frequency adjustments in connected mode. Proposal 9: RAN1 to study the impact of timing drift on closed loop TA, and potential enhancements to reduce issuance of TA commands for UEs in connected mode. |
| Johns Hopkins University | Proposal 2: The rate of change of delay is significant at low elevation angle scenarios. RAN1 should study the time variations of the delay to support the update rate required in the connected mode. Proposal 3: RAN1 should study the time variations of the Doppler frequency and support a closed-loop for updating the Doppler frequency at the UE. |
| NTT DOCOMO | Proposal 5: For Rel-20 NTN GNSS resilience, regarding Connected mode, support at least the following: - Enhancement on UL time synchronization - Study enhancements on TA indication, e.g., NW estimation/indication of UE specific TA, increased value range for TA command, etc. - Enhancement on UL frequency synchronization Study frequency adjustment mechanism, e.g., NW-assisted frequency compensation. |
| CSCN | Proposal 7: The closed-loop frequency synchronization mechanism should be introduced when using the RP indication-based common FO compensation scheme. |
| ST Engineering iDirect | Proposal 3: Add a MAC CE to signal estimated timing errors. Proposal 4: Add a MAC CE to signal estimated frequency errors from the gNB to the UE. Observation 4: Enhancing closed-loop frequency control will also solve existing deployment challenges that are related to (unsignaled) station-keeping and frequency offsets from low-cost radios or unslaved BUCs/LNBs. Observation 5: The proposed frequency and timing error MAC CEs do not require any knowledge of the UE or gNB/satellite position, and work for any orbit. Observation 6: The presented solution can be combined with existing pre-compensation methods and will address UE mobility and TLE errors. Observation 7: A first order closed loop is not sufficient to have the time and frequency errors within the required accuracy to avoid inter-symbol and inter-carrier interference. |

## Summary of companies’ contributions

In connected mode for GNSS-resilient NR-NTN, companies broadly propose a combination of network-assisted and UE-based uplink synchronization enhancements. Most advocate supporting both open-loop (UE estimates using reference location, historic data, and/or latest GNSS fix) and closed-loop (network signaling explicit correction via MAC CEs) operation. Key proposals stress enhancing TA command to cover drift rates, larger/negative ranges, and reducing signaling overhead, alongside introducing or optimizing frequency adjustment commands for fine frequency offset control. Frequent or adaptive network signaling is recommended when GNSS is unavailable, and some companies suggest allowing the network to detect/report on GNSS status at the UE.

Some companies push for leveraging downlink reference signals or satellite ephemeris to estimate and pre-compensate for timing/frequency errors, with additional methods such as DL measurements for UL FO estimation, as well as the adoption of multi-stage or gradual compensation approaches.

Procedures to allow more flexible, less frequent, or adaptive TA/FO updates are suggested to limit overhead while maintaining accuracy.

## Initial proposal

### Proposal 9-1

Proposal 9-1-v0

For the study on GNSS resilient NR-NTN operation, RAN1 to consider the following enhancements for connected mode: - Enhanced TA command (e.g., TA drift rate and TA drift variation rate) - Frequency adjustment command in MAC CE i.e. closed-loop frequency control in connected mode Overhead reduction should be considered

Companies are encouraged to share views on Proposal 9-1-v0

| Company | Comments |
| --- | --- |
| CATT | It is too early. |
| HONOR | Support. |
| Xiaomi | We can have this discussion after Proposal 4-1-v0 is stabilized. |
| Sony | Support. |
| Toyota | Agree with CATT, it is too early. As pointed out in our contribution, these solutions are Step 3 and without understanding Steps 1 and 2 improvements, difficult to discuss. |
| Panasonic | Support. |
| TCL | We agree with this proposal. Enhanced TA maintance due to frequent TA adjustment and frequency adjustment in connected mode should be considered. |
| ZTE | Fine with the proposal. |
| Sharp | Too early to discuss. |
| ST Engineering iDirect | Agree with this proposal. To address the TA drift, we propose in R1-2507937 to signal the timing error, rather than the correction. With higher-order time tracking loops, the TA drift rate and drift variation rate can be corrected, and less TA correction messages will be required. Furthermore, the signaling of the satellite ephemeris (and even signaling of the reference location) can be decreased. This will also allow higher accelerations. With higher-order loops, we can get the timing and frequency errors within the existing limits for the following UE mobility profile (with a speed up to 100 m/s) connecting to a GEO satellite. (a) UE mobility profile (b) Tracked delay and delay error (c) Frequency error |
| LGE | We can discuss this after we have a consensus on the observations from RAN1 and the target scenario is clear. |
| Nokia | We find it too early to discuss such details. As we see it, it is more important that we consider the aspect of “how to minimize the control overhead in general” – and here we should keep in mind that the existing method of GNSS based operation is having zero overhead. Any solutions that are showing similar behavior (like geo-location based) should be evaluated with higher priority. Additionally, this all comes down to the “tracking of the UE” – how this is going to be implemented. And if companies are targeting connected mode tracking, we should be aware that we need signals/control messages implemented in both UL (for the reference signals to allow the gNB to estimate the compensation information) and DL (for providing the compensation information). |
| Ericsson | Support. Add "other enhancements are not precluded". |
| Apple | Support |
| ETRI | It is too early. |
| Ofinno | Fine with the proposal |
| Spreadtrum | How to indicated frequency adjustment need further discussion |
| NEC | Too early to discuss. |
| Huawei, HiSilicon | We think the existing delta TAC is sufficient. The only thing missing is frequency correction. Overhead reduction should be deferred after assessment. |
| NTT DOCOMO | Support. |
| Lenovo | Support |
| CMCC2 | Too early to discuss. The impact of timing error and frequency on connected mode need to be clarified first to identify whether the potential enhancements on timing adjustment or frequency adjustment are necessary. |

### Proposal 9-2

Proposal 9-2-v0

Study the GNSS operations specified for IoT-NTN in Rel-18 to see if any solutions can be adopted for GNSS-resilient operation of NR-NTN

Companies are encouraged to share views on Proposal 9-1-v0

| Company | Comments |
| --- | --- |
| Xiaomi | OK for discussion |
| Panasonic | “GNSS operations specified for IoT-NTN in Rel-18“ is too vague, it should be described more specifically. |
| TCL | Ok |
| LGE | Support. |
| Nokia | It may be OK to study this, but again, care should be taken to ensure the robustness of NR over NTN in the first place. |
| Ericsson | Support. |
| OPPO | Support |
| Apple | ok |
| ETRI | OK |
| Spreadtrum | Support |
| Huawei, Hisilicon | It is out of scope of SID |
| NTT DOCOMO | Ok to discuss. |
| CMCC2 | Fine to discuss. Share similar view as Panasonic that the solutions should be more specific or elaborated clearly. |

# Topic#10 RAT dependent positioning method

Positioning based method is another solution discussed in companies contributions. This method is used to acquire a coarse UE location.

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Huawei | Observation 9: In LEO-600 and LEO-1200, UE can see at least 2 satellites in most of serving time. The larger constellation size, the more satellites UE can see. The higher latitude UE locates, the more satellite UE can see. Observation 10: For LEO-1200 constellation (264 satellites in total), UE can achieve with 90% probability the location uncertainty of 5.45km or 20.24km by measuring DL-TDOA from 3 or 2 satellites respectively. Observation 11: Under LEO600 constellation with 800 satellites, if a UE can observe 4 satellites, the location uncertainty of 232m for 90% of UEs can be achieved; Under LEO600 constellation with 1400 satellites, if a UE can observe 5 satellites, an accuracy of 130m for 90% of UEs can be achieved. Observation 12: When employing the DL-TDOA positioning, the differential Doppler and delay can be greatly reduced compared to values in scenario 1 + case a. Proposal 3: To acquire UE coarse location by measuring the PRSs from satellite(s), the measurement of DL-RSTD can be enhanced to accommodate larger propagation delay difference from multiple satellites. |
| CMCC | Observation 7: Single satellite with 4 measurement occasions and 3 RSTDs can achieve 4.23km and 2.02km positioning accuracy (or positioning uncertainties) in 24s and 36s. Observation 8: Multiple satellites with at least two measurement occasions can achieve 4.34km and 2.23 km positioning accuracy in 4s and 8s. And with larger time intervales, the positioning errors would be even lower to 1km and less than 1km. Proposal 17: It is proposed to study the RAT-dependent positioning to improve the GNSS resilience. |
| TCL | Observation 1: RAT-dependent positioning can be considered for NTN positioning. Observation 2: Considering the positioning accuracy, Time-based positioning methods can be considered for UE-based positioning to access the network. Observation 3: If single-satellite positioning is adopted for UE-based positioning, the position accuracy and access latency should be considered. Observation 4: The mapping between satellites and PRS ID should be clarified under the multiple-satellite case. Proposal 1: For multiple-satellite positioning, each satellite can be considered as a TRP. Proposal 2: For multiple-satellite positioning, the number of satellites for positioning should be decided. |
| OPPO | Proposal 7: Study autonomous maintenance at UE with UE location acquired by NR positioning for GNSS resilience operation. |
| InterDigital | Observation 2: If the UEs are able to acquire and maintain its location through NR based signals, the legacy procedures, e.g., time and frequency compensation, timing adjustments, etc., may be applied similar to the legacy NR NTN operation. Observation 3: The UEs need to acquire the configuration of positioning reference signals from multiple satellites to determine UE location based upon NR signals. Observation 4: The UEs being able to receive and estimate the signals from multiple satellites for location estimation will result in significant processing complexity at the UE side. Observation 5: NR NTN satellites transmitting RS to enable UE location determination will result in significant overhead and will lead to poor spectrum efficiency for the system. Observation 6: UE location estimation based upon NTN satellites transmitting PRS will require dense satellite constellations designed so that the UEs in different locations may receive and measure signals from multiple satellites. Observation 10: If the UE is able to acquire its location through NR based signals prior to initiating initial access, the legacy initial access procedures, e.g., time and frequency compensation, RACH window placement, etc., may be applied similar to initial access in legacy NR NTN operation. Observation 11: UEs determining their location based upon NR based positioning has serious implications and complexity on network and UE sides. |
| NEC | Observation 6: The legacy propagation delay compensation (PDC) could provide service link RTT information when the GNSS is temporarily unavailable during the connected mode. Proposal 1: RAN1 considers legacy PDC, e.g. the UE-side RTT-based propagation delay compensation (PDC) and the gNB-side RTT-based PDC, to estimate the TTA for an NTN terminal under GNSS resilient mode. |
| Qualcomm | Observation 7: Enabling DL-only UE-based positioning solutions to reduce time-frequency uncertainty using measurements from multiple satellites depends on satellite constellation characteristics. Observation 8: Enabling DL-only UE-based positioning solutions to reduce time-frequency uncertainty using measurements from single satellites is not feasible because: - The UE requires RSTD measurements over long time window (e.g. >24 seconds). This will introduce significant latency before random access to NTN cell for GNSS-resilient UEs. - Even with larger time windows, whether the UE will be able to reduce the time-frequency uncertainty to the range that can be handled by the existing PRACH formats depends on UE local oscillator errors. |
| ESA | Observation 4: It is important to clarify that coarse localization determined with DL-PRS signal from multiple satellites is not impacting the existing initial access procedure, which will remain based on UE location and satellite ephemeris. Observation 5: With a 20° elevation mask, a UE always sees ≥3 satellites globally, and ≥4 satellites at mid- and high latitudes, ensuring strong positioning availability. Observation 6: With a 30° elevation mask, global 3D availability with ≥4 satellites cannot be guaranteed; only mid- and high latitudes achieve reliable coverage. Observation 7: The barometric pressure sensor method, already specified in LPP (TS 37.355), enables 3D positioning even when only three satellites are visible. Observation 8 - When considering minimum synchronization error, NTN positioning can fully substitute GNSS for initial NTN access, since: - For user masking angle of 20° the horizontal positioning error is always below 50m at any user latitude. - For user masking angle of 30°, this performance is also fulfilled, except for 20° latitude. - When considering 50ns synchronization error, NTN positioning can serve as enabler for relaxed GNSS resilient NTN access solutions, since: - Horizontal positioning errors approximately below 1km are found for all cases. Observation 9: RAT-dependent localization complemented by barometric pressure sensor could shrink location uncertainty area in Case b (typically, better than few hundreds of meters according to Table 6 and Table 7) and reduces the probability of Case a, though not eliminating it as availability for 3 or more satellites cannot be guaranteed everywhere in the world if minimum elevation angle is 30 degrees (according to Table 4 and Table 5). Proposal 1: Given the severity and occurrence of GNSS denial attacks (Scenario 1), it is proposed to adopt a toolbox approach (i.e., multiple solutions not just one) to increase the resilience of NTN operations when GNSS is not available. Proposal 2: As part of the toolbox approach for GNSS-resilient NTN operation, investigate coarse localization based on RAT-dependent positioning methods in Rel-20 SI. Note 1: Positioning should be considered complementary to (i.e., not precluding) other approaches already under discussion. Note 2: Among positioning methods, DL-TDOA should be prioritized for NTN as it can reduces the probability of large uncertainty regions assumed in current Case a/b evaluations. |
| ISSDU | Observation 1: The already standardized PRS and SIB19 can enable UE-centric positioning when GNSS information is unavailable. Observation 2: Among all RAT-dependent positioning methods, only DL-TDOA remains feasible for both initial access and connected mode in NR-NTN scenario. Proposal 1: PRS with SIB19 should be adopted for GNSS-resilient NR-NTN, offering operators a cost-effective way to improve reliability and meet requirements. Proposal 2: DL-TDOA is proposed to be prioritized for GNSS-resilient NR-NTN, ensuring reliable positioning and helping operators optimize performance, cost, and support IoT and Public Safety. Proposal 3: Incorporating PRS configuration into SIB19 allows UEs in both RRC_IDLE and RRC_CONNECTED to synchronize, helping operators cut signaling load, improve access, and ensure service continuity. |
| CAICT | Proposal 4: To assess whether the current positioning accuracy of UE is comparable with the accuracy of GNSS for the various cases. |
| Google | Observation 2: Although there is a note in the SID stating that “This study does not include any enhancement on positioning for the above scenario”, but reusing existing RRC signalling such as reference location (e.g., mobility enhancement for quasi-earth fixed cell) of a cell or beam should be allowed. Proposal 3: Study the case where UE acquires a coarse position information from satellite system information (e.g., reference location for mobility enhancement) or RRC signalling for RRC connected state. |
| CEWiT | Proposal 3: The UE should be able to estimate the location using SSB and SIB19 when GNSS temporarily unavailable. UE should estimate TDoA using SSB from multiple satellites to estimate its location. UE specific timing advance estimated using UE location and the ephemeris information from the SIB19. The TDoA estimation at the UE is implementation specific, and it will not use positioning framework of NR. Proposal 4: The UE should employ multilateration techniques to determine its location based on measurements from multiple satellites. The determined location is used to calculate the corresponding TA. Proposal 5: In order to avoid TA overestimation, the UE should quantize the estimated TA using the predefined step size and subsequently subtract the TA margin from the quantized value. Proposal 6: If the step size is not signalled by the satellite, the UE should subtract the TA margin from the estimated TA. |

## Summary of companies’ contributions

Several companies prefer to prioritize multi-satellite NR RAT-dependent positioning as a potential solution for NR NTN GNSS resilience, with the DL-TDOA method, standardized PRS/SSB, and system information support. According these companies the approach is feasible, scalable, and sufficient for supporting access-related timing/frequency estimations, though complexity and overhead tradeoffs must be managed.

On the feasibility and gains of RAT-dependent positioning:

- Huawei and ESA, Airbus and Thales show that in large LEO constellations, UEs can frequently see 2–5 satellites, especially at higher latitudes, enabling multilateration.

- Huawei demonstrates that using 3–5 satellites, horizontal positioning errors of 130m to ~20km are possible (the more satellites, the better).

- ESA and co-sourcing companies confirm global 3-satellite visibility with a 20° mask and possible sub-50 m accuracy; even with realistic sync errors, errors are typically <1 km.

- Qualcomm: Clearly observes that multi-satellite RAT-dependent positioning is feasible depending on satellite constellation characteristics. However, single-satellite solutions are not feasible for initial access because they require RSTD measurements over a long time window (>24s), which introduces high access latency and may not even bring residual time-frequency uncertainty within PRACH tolerances.

- Several companies CMCC, TCL, OPPO, InterDigital, NEC, ESA, AIRBUS, Thales, ISSDU, CAICT, Google, CEWiT propose to study the RAT-dependent positioning to improve the GNSS resilience.

On the proposed methods:

- Primary methods and proposals, including DL-TDOA, PRS/SIB19, propagation delay compensation, and signaling options, remain unchanged.

| Company | Key observations | Proposals/Key points |
| --- | --- | --- |
| Huawei | Multi-sat visibility, RAT-based pos improves delay/Doppler, high potential accuracy | DL-TDOA or enhanced PRS methods; measurement protocol adaptations needed |
| CMCC | RAT-based pos achieves km-level accuracy, even faster with more satellites | Proposes in-depth study of RAT-dependent pos for GNSS resilience |
| TCL | Multi-sat advantageous; single-sat less accurate, higher latency | Clarify mapping/IDs; decide # of satellites/PRS/TRP for pos |
| OPPO | - | Study UE-based maintenance via NR positioning for resilience |
| InterDigital | NR-based pos can substitute legacy pre-comp if config available, but with system tradeoffs | Configuration/overhead must be evaluated; especially with dense constellations |
| NEC | Legacy PDC can offer fallback timing (RTT) | Both UE-side & gNB-side RTT-based PDC for TA when GNSS unavailable |
| Qualcomm | Multi-satellite positioning is feasible, single-satellite is not (high latency, error) | Multi-sat approaches are practical, single-sat not suited for initial access |
| ESA | Global multi-sat visibility, high accuracy, barometric sensors can help | Toolbox/portfolio approach; DL-TDOA preferred, complemented by barometric |
| ISSDU | PRS/SIB19 allows for GNSS-free UE positioning | Adopt PRS in SIB19; DL-TDOA prioritized |
| CAICT | Compare RAT-based accuracy to GNSS | Detailed evaluation and standardization needed |
| Google | Can allow access to reference location via signaling | Use RRC, SIB19, or similar messages for fallback coarse positioning |
| CEWiT | SSB/SIB19 enable position-based TA/FO | TDoA/multilateration, proper TA quantization/margins, implementation-specific |

## Initial proposal

### Proposal 10-1

Proposal 10-1-v0

For GNSS-resilient NTN operation, study DL-only UE-based positioning method, using DL-TDOA. - This should include assessing the feasibility, evaluating the achievable UE position accuracy, and identifying any potential impacts on existing specifications. - Evaluation assumptions and parameters should be defined during this meeting.

Companies are encouraged to share views on Proposal 10-1-v0

| Companies | Comments |
| --- | --- |
| CATT. | Support it. We don’t only rely on PRACH enhancement. |
| Xiaomi | We don’t think the second bullet is needed given this study should anyway be optional - Evaluation assumptions and parameters should be defined during this meeting. |
| Sony | Support. Highlight that multi-satellite localization assumptions need to be studied during feasibility as we disagree with some contributions on satellite visibility. |
| Toyota | Need to understand SID scope limitations with positioning based solutions |
| CEWiT | Support |
| TCL | Support. We think access latency and number of satellites for position should also be considered. |
| ZTE | Regarding DL-TDOA, an important issue is that how UE can measure RS from multiple satellites. Both the UE behaviour and potential deployment should be considered in the study of feasibility and spec impacts. |
| ST Engineering iDirect | Agree with observations 7 and 8 in R1-2507719 (Qualcomm) that - DL-only UE-based positioning using signaling from a single satellite is not possible because of the long time window that is required - DL-only UE-based positioning using signaling using multiple satellite depends on the constellation characteristics Note that DL-TDOA based positioning will only be possible with NGSO constellations with a large number of satellites. It will not work for GSO, and in the Ku/Ka band, directive antennas may limit the number of satellites that are in view. It might also significantly increase signaling, and satellite operators might not want to implement PNT. As the allocated time for this study item is too limited, it is proposed to postpone the PNT study to 6G. |
| ISSDU | Support. It is further suggested to consider hybrid scenarios involving both TN and NTN, where TN base stations are included in the evaluation framework. |
| Nokia | DL-TDOA based methods will mainly have to rely on (a) time stability of the UE between measurements, and (b) the UE being able to observe multiple satellites at the same time (deployment aspect) and (c) actually receive signals from these multiple satellites with short separation in time. Due to needing to serve all deployment scenarios, we do not think it is feasible to explore this approach. |
| Ericsson | Based on the SID, solutions for positioning should not have specification impact. This should preferably be stated in the first sentence (e.g., "without specification impact"), instead of the "identifying any potential impacts on existing specifications" in the first bullet. |
| OPPO | Support to study the feasibility of DL-TDOA for NTN positioning |
| Airbus | Support. RAT-dependent positioning method can be used as a complementary solution, by providing coarse UE location in case of GNSS unavailability. The assumptions and parameters to enable DL-only UE-based positioning method are proposed to be defined during this meeting, in order to assess feasibility, achievable performance and identification of applicable existing specifications. |
| ESA | - We support the proposal. We think NTN-based coarse localization is a promising option for Scenario 1 and both case a (reduces the percentage of time when the location uncertainty is as poor as the area of the beam) and case b (can shrink the uncertainty <1 km). Furthermore, coarse localization for initial access can be obtained based on existing reference signals and signalling already specified for UE-based DL-TDOA positioning. - Existing reference signals: SSB (always ON) and PRS - Signalling: SIB19 for ephemeris of serving and neighbour satellites, DL-TDOA specific posSIB which are carried in RRC System Information messages as defined in TS 38.331 and TS 37.355. Coarse localization is also complementary to the other options discussed above and we think it should be part of the toolbox of solutions for the problem addressed in the SID. 2. For bullet number 2, the achievable performance could be evaluated based on the methodology agreed in Rel18 for Network Verified Location: (R1-2208321). |
| ETRI | Same view with Xiaomi |
| Huawei, HiSilicon | support |
| Lenovo | Support, but we need to be careful with SID scope limitations |
| CMCC2 | Support to study. And in our contribution we also provided the evaluation results for both single satellite and multiple satellite based positioning performance. |

# Topic#11 GNSS Status reporting and control

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Proposal 1 Study network-assisted and UE-assisted methods to detect a loss of GNSS information in a cell or UE. |
| Thales | Proposal 3: RAN1 to discuss whether GNSS information unavailability applies to the entire cell coverage area or only to a specific UE. |
| vivo | Proposal 9: Study the mechanism to support GNSS mode transition (e.g., from a GNSS based mode to GNSS resilient mode) for a UE. Proposal 10: Study the GNSS status report to help NW to decide whether to transmit TA command and FO command considering signalling overhead reduction. |
| ZTE | Proposal 7: Network supporting relaxed requirement need to indicate that UE without valid GNSS position is allowed to access. Proposal 10: Status report for GNSS should be supported when UE without valid GNSS position is allowed to transmit in connected mode. Proposal 11: gNB need to indicate UE the extension time for UL transmission when UE without valid GNSS position is allowed to transmit in connected mode. Proposal 12: gNB triggered GNSS update should be supported when UE without valid GNSS position is allowed to transmit in connected mode. |
| Huawei | Proposal 7: UE should report the availability and validity of GNSS position information in GNSS resilient operation. |
| TCL | Proposal 5: Whether UE can acquire GNNS information can be reported implicitly via the PRACH resources, such as RO or preamble. |
| HONOR | Proposal 8: RAN1 to study signalling solutions to enable the network to be aware of the UE’s GNSS status, e.g., the UE may need to report its GNSS status when the latest GNSS based UE location accuracy has degraded or become invalid. |
| InterDigital | Proposal 1: Study how the UE may detect being in scenario 1, namely the conditions under which it cannot rely on GNSS for timing and frequency compensation. Proposal 2: Clarify the intentions for UE behavior subsequent to UE detected being in scenario 1. Proposal 3: Study how the UE may estimate its position error based upon DL measurements. Proposal 4: Study the conditions under which the UE can use its last determined outdated location to perform time frequency compensation. |
| Samsung | Proposal 1: RAN1 to study UE behavior for GNSS temporarily unavailable conditions before initial access and during RRC_CONNECTED. Proposal 4: RAN1 to study explicit GNSS availability status reporting mechanisms and define operational categories of “temporarily unavailable” GNSS. |
| China Telecom | Proposal 1: Support reporting UE GNSS status to the network. Proposal 2: It needs to discuss judgment methods or criteria for GNSS accuracy degradation, and discuss when to trigger GNSS status report from UE side, if supported. |
| LG Electronics | Proposal #11: In Rel-20 NR-NTN, RAN1 studies whether/how to determine GNSS positioning accuracy degradation. Proposal #12: In Rel-20 NR-NTN, RAN1 studies potential impacts from switching between GNSS-based operation and GNSS-resilient operation. |
| Ofinno | Proposal 4: The GNSS reception can indicate information such as: whether the UE is currently in scenario #1 or in scenario #, time when the UE last acquired GNSS, time when the UE moved to scenario #1 or scenario #2, etc. Proposal 5: The UE indicates its GNSS reception status to the network during RACH procedure or when the UE transitions to the RRC connected state. |
| Fraunhofer | Proposal 4: RAN 1 to study mechanisms to assist an NTN UE evaluating the reliability of an available GNSS signal. |
| Lenovo | Proposal 9: RAN1 to study the mechanism for indication of GNSS connectivity loss in connected mode. |
| Nokia | Proposal 5: RAN1 to discuss whether a limit on the temporary unavailability is necessary, and whether there should be conditions associated with such limits. Proposal 7: RAN1 to clarify and conclude whether a UE can be assumed to be aware of the degraded accuracy of GNSS information. |
| Qualcomm | Proposal 11: RAN1 to study signaling/procedures for handling the loss/gain of GNSS-based location during connected mode. |
| NTT DOCOMO | Proposal 6: Study how to determine whether the GNSS-resilient operation or GNSS-based operation is applied, e.g., - Based on NW indication - Based on UE measurement, and - E.g., request or report to NW with or without confirmation response - E.g., UE decision with conditions |

## Summary of companies’ contributions

Several companies have provided proposals and observations regarding detection and reporting of GNSS status or loss. Ericsson and Thales observe that a temporary GNSS loss can be either cell-specific or UE-specific and suggests the need to study both network- and UE-assisted methods to detect such events. Similarly, Huawei, ZTE, vivo, Honor recommend that UEs report both the availability and validity of GNSS position information during GNSS-resilient operation. TCL proposes that UEs can report their GNSS acquisition status implicitly through PRACH resources, while HONOR calls for a study of signaling solutions to enable the network to become aware of the UE’s GNSS status, particularly when location accuracy degrades or becomes invalid. Samsung highlights the importance of establishing explicit reporting mechanisms for GNSS availability and encourages the definition of operational categories for temporary unavailability. China Telecom, pointing out that current specifications require valid GNSS position before transmission, suggests supporting GNSS status reporting at the UE as well as developing judgment criteria and triggers for when this reporting should occur. Ofinno agrees on the benefits for the network in knowing the UE’s GNSS status and proposes that such status be indicated during connection procedures. Lenovo also advocates for mechanisms to indicate GNSS connectivity loss, while Qualcomm urges the study of signaling and procedures for handling the loss or restoration of GNSS-based location, especially during connected mode.

The transition between GNSS-based and GNSS-resilient modes is another key area. vivo recommends studying mechanisms to support such transitions at the UE, including the facilitation of network decisions related to timing advance and frequency offset commands based on GNSS status reports. NTT DOCOMO proposes to investigate the methods for determining whether GNSS-based or resilient operation should be used, suggesting criteria could be based on network indications, UE measurements, explicit requests or reports, or predefined UE decision logic. LG calls for a study into the determination of GNSS positioning accuracy degradation and the implications of switching between GNSS-based and resilient operating modes. ZTE highlights the need for the network to signal relaxation of requirements, permit continued access or transmission for UEs without valid GNSS, and trigger GNSS status reports or updates from the gNB in these scenarios. Handling of UEs without valid GNSS information brings additional complexity. ZTE details several proposals on network signaling for relaxed requirements, extension of uplink transmission time, and procedures for GNSS status reporting or updates triggered by the gNB.

Regarding the scope of GNSS unavailability, whether it applies network-wide or is strictly a UE-level concern. Thales proposes clarifying the domain of GNSS information unavailability, and Nokia believes it is important to discuss limits on temporary GNSS unavailability and UEs’ ability to detect degraded GNSS accuracy. Ericsson’s observation aligns with this, distinguishing between cell and UE-specific loss scenarios.

## Initial proposal

### Proposal 11-1

Based on the above discussion, the following initial proposal is made:

Proposal 11-1-v0

RAN1 to study network/UE methods for detecting GNSS loss, including reporting of GNSS status and mechanisms for transitioning operational modes e.g., from a GNSS based mode to GNSS resilient mode.

Companies are encouraged to share views on Proposal 11-1-v0

| Companies | Comments |
| --- | --- |
| CATT | Need more discussion |
| HONOR | Support. |
| Xiaomi | OK for discussion |
| Sony | Support discussion. |
| Toyota | Agree with CATT. Need more discussion. |
| Tejas Networks | Agree with the proposal |
| CEWiT | Support |
| Panasonic | Support. |
| ZTE | As GNSS module is equipped on UE, we think it is more reasonable to study how UE detect GNSS loss and reporting the status. Network cannot justify the status of a UE module better than UE. |
| Sharp | Support |
| ST Engineering iDirect | Agree to discuss. |
| Nokia | OK to study further this aspect. |
| Ericsson | Support. |
| OPPO | The operational modes switch between GNSS based mode and GNSS resilient mode, i.e., from GNSS based mode to GNSS resilient mode and from GNSS resilient mode to GNSS based mode, should be both considered |
| Apple | OK |
| ETRI | Support. |
| Spreadtrum | Operational mode from GNSS resilient mode to GNSS based mode should also be studied |
| Huawei, HiSilicon | Detecting GNSS loss is up to UE implementation and out of SID scope. what need to be specified in 3GPP is to define a mechanism to let UE report GNSS status in connected mode, similar as the validity duration report in IoT NTN. |
| NTT DOCOMO | OK |
| Lenovo | Support |
| CMCC2 | General fine with the direction. Whether and how can UE determine its losing of GNSS should be discussed and clarifed. |

# Topic#12 Coexistence with Legacy UEs

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Observation 2 An NTN cell may need to simultaneously support UEs with and without temporary loss of GNSS information. Observation 15 Separate PRACH resources may be needed to support initial access for UEs with and without GNSS information in a cell. |
| Huawei | Proposal 8: To support backward compatibility, separate PRACH resources is needed for preamble transmissions with large timing and frequency error. |
| TCL | Observation 6: If the closed-loop control can be enhanced, UE cannot distinguish whether the indication applies to a UE without GNSS or a GNSS-capable UE. |
| OPPO | Proposal 8: Separate PRACH configurations including PRACH formats and PRACH occasions for GNSS capable and GNSS unavailable UEs. |
| HONOR | Proposal 5: RAN1 to study coexistence strategies between legacy (GNSS-dependent) and GNSS-unavailable UEs and consider the following options: - Separate PRACH resources are configured during initial access for UEs with and without valid GNSS based UE location in a cell. - The UE transmits PRACH only within the GNSS based location validity duration; otherwise, PRACH transmission is not permitted. |
| Samsung | Proposal 6: RAN1 to study coexistence strategies between legacy GNSS-dependent UEs and GNSS-unavailable UEs, including optional resource separation and scheduling isolation. |
| LG Electronics | Proposal #4: In Rel-20 NR-NTN, RAN1 studies the separation of ROs according to TO/FO pre-compensation accuracy. |
| Ofinno | Proposal 1: Introduce separate PRACH resource sets: - a first set (Set 1) for UEs whose GNSS is available, - a second set (Set 2) for UEs in Scenario # 1 (i.e., which cannot rely on GNSS), and - a third set (Set 3) for UEs in Scenario # 2 (i.e., which didn’t receive GNSS since time duration T). |
| Lenovo | Observation 4: Use of same RACH resources for UEs with and without GNSS availability would cause unnecessary delays for UEs with GNSS. |
| Apple | Proposal 2: Consider different RACH resources are used for UEs without valid GNSS information and for UEs with valid GNSS information. |
| NTT DOCOMO | Proposal 4: For Rel-20 NTN GNSS resilience, regarding initial access, study the enhancement for PRACH resource configuration for UE with and without GNSS. |

## Summary of companies’ contributions

The main focus of company proposals and observations revolves around efficiently supporting both GNSS-capable and GNSS-unavailable UEs in NTN.

Separation of PRACH resources:
Several companies, including Ericsson, Huawei, OPPO, HONOR, Samsung, LG Electronics, Ofinno, Apple, and NTT DOCOMO, propose or observe the need to configure separate PRACH resources or configurations for UEs with valid GNSS information versus those without it. The primary motivation is to maintain backward compatibility, accommodate large timing and frequency errors in GNSS-unavailable UEs, and reduce access delays for GNSS-capable UEs. Ericsson and Lenovo highlight that a shared PRACH resource could cause unnecessary delays for GNSS-capable UEs, while LG specifically suggests separating ROs based on pre-compensation accuracy. Ofinno provides a more granular approach by suggesting three separate PRACH resource sets, catering to UEs with current GNSS, those with no GNSS in “Scenario #1,” and UEs that haven’t received GNSS updates in “Scenario #2.” OPPO also suggests distinct PRACH formats and occasions tailored for each group. HONOR additionally suggests operational constraints, such as permitting PRACH transmission only within the window of GNSS-based location validity. TCL observes that closed-loop control enhancements could blur the distinction between GNSS-capable and non-GNSS UEs if the indication mechanism is not sufficiently explicit.

Coexistence and scheduling strategies:
HONOR and Samsung recommend studying coexistence and scheduling strategies to ensure that both legacy (GNSS-dependent) and GNSS-unavailable UEs can be efficiently served within the same cell. This includes exploring resource separation as well as scheduling isolation to minimize interference and optimize network operation.

## Initial proposal

### Proposal 12-1

Proposal 12-1-v0

Study coexistence strategies between legacy (GNSS-dependent) and GNSS-unavailable UEs, including the separation of PRACH resources.

Companies are encouraged to share views on Proposal 12-1-v0

| Companies | Comments |
| --- | --- |
| CATT | It depends on above discussion for PRACH configuration. |
| HONOR | Support. |
| Xiaomi | OK for discussion |
| Sony | Support. |
| CEWiT | Support |
| Panasonic | Support. |
| TCL | Support |
| ZTE | Fine to further discuss. |
| Sharp | Support |
| ST Engineering iDirect | Support. |
| Nokia | In principle OK to study. |
| Ericsson | Support. |
| OPPO | Support. |
| Apple | Support |
| Ofinno | Support the proposal. We support studying the scenarios/cases. This is because the range of differential delays may be different for UEs which cannot rely on its GNSS for timing and frequency compensation and UEs that have a previously acquired GNSS based position. |
| Huawei, Hisilicon | Support |
| NTT DOCOMO | Support. |
| Lenovo | Support |
| CMCC2 | Fine with the proposal. |

# Compilation of companies’ proposals

The compilation of companies’ proposals regarding NR-NTN resilience to GNSS unavailability and degradation is available in document R1-2507952. This document gathers contributions from various companies, presenting their recommendations and technical solutions to enhance NR-NTN performance in scenarios where GNSS signals are unavailable or degraded.

# Proposals for Tuesday offline session

## Proposal 1-1

Proposal 1-1-v1

For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions:

- [For scenario 1,] it is assumed that the UE performs timing and frequency compensation based on a reference location inside the uncertainty area.

  - The reference location (RP) is placed at the center of the beam.

    - Other positions of the RP (e.g.  with the shortest distance to the satellite which may be relevant in case of moving cell/beam) can be considered and reported.

  - The following time/frequency differences should be evaluated:

    - The differential one-way/round trip delay (in µs) corresponding to the difference between UE-Satellite RTT and the RP-Satellite  as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area.

      - the uncertainty area as defined for case a previous agreement has a fixed size within the satellite coverage footprint.

    - The differential one-way/ round trip Doppler/frequency offset (in kHz) corresponding to the difference between UE-Satellite round-trip Doppler and the RP-Satellite round-trip Doppler. within the uncertainty area. Calculated between the largest UE-Sat RTT and smalltest UE sat RTT within the uncertainty area.

    - The differential round trip time is claculted/reported for minimum elevation angle.

    - The differential one-way Dopple is claculted/reported for 90°  elevation angle

  - At least maximum values of differential RTT and one-way  Doppler be reported

  - Other methods (e.g. based on frequency and timing error ) can be used and reported

- For Scenario 2: it is assumed that the UE performs timing and frequency compensation based on a the previously acquired GNSS based position.

  - The residual time and frequency pre-compensation errors are evaluated, by considering the following:

    - Time duration T from the last acquired GNSS position and hence

    - UE Speed

    - FFS: value of T

    - FFS: GNSS based UE location validity duration

    - FFS: GNSS accuracy

## Proposal 1-2

Proposal 1-2-v1

For PRACH performance evaluation for existing PRACH formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, GP, Sequence duration×maxNcsL) -2Te ≤ Differential RTT, where;

    - Ncp is cyclic prefix duration

    - GP: Guard period

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error to be reported.

    - Note: the delay spread is neglected

  - Or  Differential Round Trip Doppler+2fefc≥ 1+γΔfRA

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

    - fe=0.1 ppm

    - fc is the carrier frequency

    - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively (corresponding to γ=1 and γ=3).

## Proposal 1-4

As Samsung proposed in R1-2507251, the following geometry model and the equations can be considered for the calculation of differential one-way delay and differential Doppler in the scenarios/cases under study.

Proposal 1-4-v0

For the calculation of differential one-way delay and differential Doppler, the following geometry model and the equations (1) through (11) are considered and will be captured in the TR 38.742. For the variables in the equations, refer to Figure below. d=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα; (12). φ1=φ-RcRE; (13). dref=RE2+RE+hs2-2RERE+hscosφ1; (14). φ=sin-1dRE+hscosα; (15). β=π2-α+φ; (16). β1=sin-1REdrefsinφ1; (17). γ=cos-1RE+hsdrefsinφ1; (18). fdest=vssinβ+vuecosαcfc; (19). fdref=vssinβ1cfc; (20). ∆t=d-drefc; (21). ∆fd=fdref-fdest; (22). In equations above, fdest is Doppler frequency if the UE has the perfect knowledge of both its location and speed. fdref is Doppler frequency calculated by the UE assuming the reference point and zero speed for the UE. ∆t is differential delay and ∆fd is differential Doppler Geometry model for the evaluations

Companies are encouraged to provide comments on Proposal 1-4-v0:

| Company | Comments |
| --- | --- |
| Samsung | Support as different companies have different assumption. Thus, it is better to have the common understanding on analytical analysis. |
| CATT | Support it |
| HONOR | Support it. |
| Xiaomi | We support capturing some common analytical analysis, but we believe the methodology listed in our paper for Differential Doppler computation as copied below can also be considered, we are open to further discussion. fd=fcc∙vradψ,λ=fcc∙v∙RsinψcosλR2+rs2-2Rrssinψ Figure A 2 Illustration of the elevation angle and central angle |
| Sony | Agree to defining unified geometry. Can discuss. |
| Toyota | Companies can report their analytical framework used for calculations. Can discuss if needed to agree on a common approach. |
| Tejas Networks | Support the proposal |
| JHUAPL | This is valid only when the LEO velocity vector is in the same plane as the center of the Earth, locations of UE and the satellite. This assumption should be stated clearly. |
| Panasonic | This calculation shows differential one-way delay/Doppler. But we are interested in “Differential Round Trip Delay” and “Differential Round Trip Doppler”. In our understanding, there can be a factor 4 between one-way and Round Trip values. We’d propose to add equations for computing Round Trip values. |
| ZTE | The model seems only consider the case where footprint is within orbital plane. The case where footprint is 90 degree with respect to orbital plane is not considered and hence not support the proposal. |
| Sharp | Support it. |
| Qualcomm | UE may not be located at the orbital plane of satellite. Agree with Xiaomi's formulation, we need to consider location uncertainty area centered around a point ψ,λ. As we have indicated in our paper, max differential RTD is observed for the uncertainty areas located at λ=90o. Also, the formula, if at all required, needs to be defined for a given location. No need to mention “reference location” since the definition of differential RTT, RTD does not depend on reference location. |
| ST Engineering iDirect | Support |
| LGE | We are supportive to have aligned evaluation assumptions especially for the analysis of differential delay and Doppler. In this sense, this proposal is one of examples as a method of calculating differential delay and Doppler. However, it can be applied for limited cases (e.g., along track). We are open to discuss its details or defining other methods. |
| Futurewei | Could the set of Doppler equations apply to a beam at 90° with respect to orbital plane and UE moving in any direction? The set of equations assume the beam center as a reference location? |
| Nokia | We do not think this is very beneficial compared to using the general models from TR38.821. |
| Ericsson | The equations and figure are to our understanding correct and we support their inclusion in the TR. A few comments: - Regarding the definition of differential delay, our view is that it is the max delay difference between any two points in a cell, not the difference between the reference location and a location at the cell edge (and similarly for differential Doppler). Could different terms be used for ∆t and ∆fd? - The reference location is assumed to be on the Earth surface, which is not a necessary restriction. - The use of φ and ϕ should be aligned (φ is used in the equations while ϕ is used in the figure). |
| Apple | Support. It’s important to align the understandings on differential delay and differential Doppler among the companies. |
| ETRI | The above equations are fine, as they just capture a geometry model. However, we think the following assumptions on the beam size of non-nadir beams should be further clarified. In this regard, we propose to add the following assumption: β-β1=3dBbeamwidth2 . Determining the beam size of non-nadir beams based on a fixed beam angle is more practical than using a fixed beam size (from our understanding, there are no commercial deployment examples for this fixed beam size for all satellite beams..). Because the evaluation results are primarily affected by the beam shape and size, the assumptions of the geometry model should be clearly defined. |
| NEC | Support and update the model with the capability to describe a UE which is not located at the orbital plane of satellite. |
| Huawei, Hisilicon | We think the differential delay and Doppler should be calculated in round trip |
| NTT DOCOMO | Support. |
| CMCC2 | Currently, there are certain discrepancies in scenario modeling among different companies, leading to variations in simulation results. These differences may impact the assessment of issues such as the usability of the current PRACH format. Therefore, it is necessary to establish a simulation modeling approach that is agreed upon by all companies. The current modeling approach only considers the two-dimensional plane. The goal of our simulation is to determine the range, or the maximum value, of time and frequency differential. Thus, modeling UE mobility speed as currently done is acceptable (velocity components in other directions relative to the UE-satellite vector would only be smaller). For three-dimensional scenarios, such as a beam at 90° with respect to the orbital plane, companies can first perform simple calculations to check whether the time and frequency offset range in such cases exceeds that of the current modeling approach. If not, there is no need to complicate the modeling. The current calculation of time and frequency offsets only considers the one-way direction. Regarding the question raised by some companies about whether round-trip effects need to be considered, we believe this can be addressed by applying a factor to the calculation results. Therefore, the current one-way modeling is sufficient. |

# Proposals for Wednesday offline session

## Proposal 1-2

Proposal 1-2-v2

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration×maxNcsL) -2Te ≤ Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or  (scaling factor*(Differential Doppler) +2* fe×fc) Differential Round Trip Doppler+2fefc≥ 1+γΔfRA

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively (corresponding to γ=1 and γ=3).

      - Othe values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

## Proposal 1-4

Proposal 1-4-v0

Working assumption: For the calculation of differential one-way delay and differential Doppler, the following geometry model and the equations (1) through (11) are considered and will be captured in the TR 38.742. For the variables in the equations, refer to Figure below. d=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα; (23). φ1=φ-RcRE; (24). dref=RE2+RE+hs2-2RERE+hscosφ1; (25). φ=sin-1dRE+hscosα; (26). β=π2-α+φ; (27). β1=sin-1REdrefsinφ1; (28). γ=cos-1RE+hsdrefsinφ1; (29). fdest=vssinβ+vuecosαcfc; (30). fdref=vssinβ1cfc; (31). ∆t=d-drefc; (32). ∆fd=fdref-fdest; (33). In equations above, fdest is Doppler frequency if the UE has the perfect knowledge of both its location and speed. fdref is Doppler frequency calculated by the UE assuming the reference point and zero speed for the UE. ∆t is differential delay and ∆fd is differential Doppler Geometry model for the evaluations

## Proposal 10-1

Proposal 10-1-v1

For GNSS-resilient NTN operation, study DL-only UE-based positioning method, using DL-TDOA. - This should include assessing the feasibility, evaluating the achievable UE position accuracy, and identifying any potential impacts on existing specifications.

## Proposal 1-3

Proposal 1-3-v0

For the evaluation of frequency and timing adjustment methods, RAN1 will consider the following evaluation methodology: - Step 1: Drop the UE at a location where the satellite is visible at an elevation angle θ - Step 2: Propagate the satellite during the simulation time T (assuming a circular trajectory around the center of Earth) - Step 3: The UE applies an initial timing and frequency offset (randomly distributed within Te and fe, which are the residual timing and frequency errors after successful completion of random access) and updates the timing and frequency adjustments at time t based on a correction method - Step 4: The network evaluates the timing and frequency error after every TUL ms, assuming an uplink transmission every TUL ms - Step 5: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively - Using the steps mentioned above, RAN1 to evaluate timing and frequency control using the following metric: Number of TA and FA commands issued within T.

# Proposals for Wednesday online session

## Proposal 1-2

Proposal 1-2-v2

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration×maxNcsL) -2Te ≤ Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or  (scaling factor*(Differential Doppler) +2* fe×fc) Differential Round Trip Doppler+2fefc≥ 1+γΔfRA

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively (corresponding to γ=1 and γ=3).

      - Othe values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

## Proposal 10-1

Proposal 10-1-v1

For GNSS-resilient NTN operation, study DL-only UE-based positioning method, using DL-TDOA.

This should include assessing feasibility, evaluating the achievable UE position accuracy, and identifying any potential impacts on existing specifications.

## Proposal 1-3

Proposal 1-3-v0

For the evaluation of frequency and timing adjustment methods, RAN1 will consider the following evaluation methodology:

- Step 1: Drop the UE at a location where the satellite is visible at an elevation angle θ

- Step 2: Propagate the satellite during the simulation time T (assuming a circular trajectory around the center of Earth)

- Step 3: The UE applies an initial timing and frequency offset (randomly distributed within Te and fe, which are the residual timing and frequency errors after successful completion of random access) and updates the timing and frequency adjustments at time t based on a correction method

- Step 4: The network evaluates the timing and frequency error after every TUL ms, assuming an uplink transmission every TUL ms

- Step 5: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively

- Using the steps mentioned above, RAN1 to evaluate timing and frequency control using the following metric: Number of TA and FA commands issued within T.

## Proposal 4-1

Proposal 4-1-v0

RAN1 to assess the impacts on initial access during NR NTN GNSS-resilient operation, including:

- Initial Timing Advance acquisition

- Impact on the Timing Advance Command (TAC) in the Random Access Response (RAR)

## Proposal 5-1

Proposal 5-1-v0

RAN1 to assess the impacts on connected mode during NR NTN GNSS-resilient operation, including:

- Impact on signalling overhead e.g. caused by frequent TA command.

RAN1 to study methods for reducing signaling overhead, improving TA management.

## Proposal 7-1

Proposal 7-1-v0

For the study on GNSS resilient NR-NTN operation, for the cases where RAN1 concludes that current PRACH tolerance is exceeded, RAN1 to consider the following solutions for initial access:

- Option 1: Solutions based on modified PRACH procedures:

  - Option 1A: Multiple PRACH transmissions (e.g. with different roots or different formats)

  - Option 1B: New PRACH restricted sets.

  - Option 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR

- Option 2: Solutions based on reducing time-frequency uncertainty

  - Option 2A: multi-satellite DL-TDOA based on current specifications.

  - Option 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

  - Option 2C: Solutions based on broadcasting DL timestamp(s)

- Option 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

- Other options are not precluded

- For all the options above:

  - common service link time/frequency pre-compensation can be considered (e.g. based on reference point or based on explicit indication of common time/frequency pre-compensation).

  - RAN4 requirements on initial time/frequency accuracy may need to be revisited

    - Liaison with RAN4 may be needed

on the PRACH:

- Dual PRACH transmission (with conjugated ZC sequences)

- New PRACH restricted sets

- Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

- Define the TA margin and extended GT to handle larger timing difference

- Relaxation of timing/frequency requirements for PRACH transmission

  - Liaison with RAN4 maybe needed

Other potential solutions are not precluded

## Proposal 9-1

Proposal 9-1-v0

For the study on GNSS resilient NR-NTN operation, RAN1 to consider the following enhancements for connected mode: - Enhanced TA command (e.g., TA drift rate and TA drift variation rate) - Frequency adjustment command in MAC CE i.e. closed-loop frequency control in connected mode Overhead reduction should be considered

## Proposal 9-2

Proposal 9-2-v0

Study the GNSS operations specified for IoT-NTN in Rel-18 to see if any solutions can be adopted for GNSS-resilient operation of NR-NTN

## Proposal 1-4

Proposal 1-4-v0

For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model and the equations (1) through (11) are considered and will be captured in the TR 38.742. For the variables in the equations, refer to Figure below. d=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα; (34). φ1=φ-RcRE; (35). dref=RE2+RE+hs2-2RERE+hscosφ1; (36). φ=sin-1dRE+hscosα; (37). β=π2-α+φ; (38). β1=sin-1REdrefsinφ1; (39). γ=cos-1RE+hsdrefsinφ1; (40). fdest=vssinβ+vuecosαcfc; (41). fdref=vssinβ1cfc; (42). ∆t=d-drefc; (43). ∆fd=fdref-fdest; (44). In equations above, fdest is Doppler frequency if the UE has the perfect knowledge of both its location and speed. fdref is Doppler frequency calculated by the UE assuming the reference point and zero speed for the UE. ∆t is differential delay and ∆fd is differential Doppler Geometry model for the evaluations

## Proposal 10-2

Proposal 10-2-v0

For the evaluation of the DL-only UE-based positioning the following parameters are considered:

| Parameter | Description/Value |
| --- | --- |
| Scenarios | Rural, LOS |
| Satellite Orbit | 600km, 1200km |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1/2 of TR38.821 |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1-NTN: 2GHz, S-band . FR2-NTN: Ku/Ka |
| BW | To be reported by companies |
| Subcarrier spacing | 15 kHz for FR1-NTN, 120 kHz for FR2-NTN |
| Number of satellites in view | At least 3 satellites |
| UE type | Handheld terminal, VSAT |
| Positioning signals | PRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL: Comb-2 |
| Number of symbols used per occasion | 6 |
| Time window for measurement collection | To be reported |
| UE speed | 3km/h |
| Performance metrics | UE 2D as baseline. |

# Second round Proposals

## Proposal 1-4

On the geometry model and the equations for the calculation of differential one-way delay and differential one-way Doppler:

From moderator perspective, such equations are discussed for the first time and many companies may need time to further check and provide necessary corrections if any. It is moderator suggestion to take it as working assumption that we will revisit in the next meeting.

The proposal is updated as follows:

The read text, with FFS are added to address some concerns raised during the online session.

Proposal 1-4-v1

Working assumption: When the serving beam/ uncertainty area is within the orbital plane (along track), for the calculation of one-way differential delay and one-way differential Doppler, the following geometry model and the equations (1) through (11) are considered and will be captured in the TR 38.742. For the variables in the equations, refer to Figure below. d=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα; (1). φ1=φ-RcRE; (2). dref=RE2+RE+hs2-2RERE+hscosφ1; (3). φ=sin-1dRE+hscosα; (45). β=π2-α+φ; (5). β1=sin-1REdrefsinφ1; (6). γ=cos-1RE+hsdrefsinφ1; (7). fdest=vssinβ+vuecosαcfc; (8). fdref=vssinβ1cfc; (9). ∆t=d-drefc; (10). ∆fd=fdref-fdest; (11). In equations above, fdest is Doppler frequency if the UE has the perfect knowledge of both its location and speed. fdref is Doppler frequency calculated by the UE with respect to the center of the beam or the uncertainty area, and zero speed for the UE. ∆t is differential delay and ∆fd is differential Doppler Geometry model for the evaluations FFS: How the differential one-way delay and the differential one-way Doppler/frequency offset agreed in this meeting are derived from ∆t and ∆fd. FFS: The equations for serving beam/ uncertainty area for other azimuts (e.g. at 90°)

Companies are encouraged to provide comments on Proposal 1-4-v1

| Companies | Comments |
| --- | --- |
| NEC | In the provided figure, the hue is consistently depicted as a positive quantity. For cases where the UE is situated in a basin, it is necessary to assign a negative value to the hue to maintain the validity of Equation (1). We should add the following note. Note: The hue is defined as positive when the UE is above the horizontal plane and negative when it is below. |
| JHUAPL | We are fine as long as it is stated that this is a simplified model that neglects Earth’s rotation. Although these impacts could be neglected in the initial access mode, such effects cannot be neglected in the evaluations in the connected mode, where the tolerable timing error could be significantly lower. |
| Futurewei | Both the set of equations and the figure need to be updated, reflecting the RAN1 agreements. For example, the reference point at the beam center should be removed in the figure. Those Doppler equations should be modified to produce the largest UE-sat Doppler/frequency offset and the smallest UE-sat Doppler/frequency offset within an uncertainty area. It is worth noting that the smallest UE-sat Doppler/frequency offset can be a zero or negative value, depending on the elevation angle and UE trajectory with respect to the satellite trajectory. |
| LGE | Equations that fit the differential delay and differential Doppler definitions we agreed on should be discussed (i.e., the difference between the largest and smallest UE-sat delay/Doppler within the uncertainty region). We suggest the following the modifications: For the calculation of one-way differential delay (∆t and one-way differential Doppler (∆f, the following equations are considered and will be captured in the TR 38.742. ∆t=maxP∈UdP-minP∈UdPc ∆f=fc∙maxP∈UvP-minP∈UvPc where dP=S-P vP=vSat-vUE∙S-PS-P S: satellite position vector P: UE position vector (including UE height) vSat: satellite velocity vector vUE: UE velocity vector U: uncertainty area fc: center frequency |
| ZTE | It is preferred to directly have formula regarding max-min calculation to obtain the differential one-way delay and the differential one-way Doppler/frequency offset instead of further study unknown transformation. For example, replace UE point as farthest point and replace reference point as nearest point, and assume they have same height hue, as shown in the example figure below. Then, the formula can be updated to calculate the max-min difference for non-nadir beam. For nadir beam, new set of formula can be introduced considering that the nearest point is beam center and differential Doppler is no longer the difference between nearest and farthest points. For non-nadir beam dfar=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα. φ1=φ-2RcRE. drefnear=RE+hue2+RE+hs2-2RE+hueRE+hscosφ1. φ=sin-1dfarRE+hscosα. β=π2-α+φ. β1=sin-1RE+huedrefnearsinφ1. γ=cos-1RE+hsdrefnearsinφ1. fdestfar=vssinβ+vuecosαcfc. fdrefnear=vssinβ1+vuecosγcfc. ∆t=d-drefdfar-dnearc. ∆fd=fdref-fdestfdfar-fdnear For nadir beam dnear=hs-hue φ=RcRE dfar=RE+hue2+RE+hs2-2RE+hueRE+hscosφ β=sin-1RE+huedfarsinφ α=π2-β+φ fdfar=vssinβ+vuecosαcfc ∆t=dfar-dnearc ∆fd=2fdfar. |
| Toyota | We suggest the following update: When the serving cell/beam/ uncertainty area is within the orbital plane (along track), <Text Intentionally Omitted> In equations above, fdest is Doppler frequency if the UE has the perfect knowledge of both its location and speed. fdref is Doppler frequency calculated by the UE with respect to a single location the center of the beam or within the uncertainty area, and zero speed for the UE. Note: Companies to report the single location considered for the calculations. The single location can be a centre of the beam/cell, edge of the beam/cell or any other location. |
| Ericsson | Agree with other companies that it is better to have equations that directly give the differential delay and the differential Doppler/frequency offset. In these equations, UE altitude uncertainty should also be taken into account. |
| Qualcomm | Figure-X To capture (1) the definition of differential delay and doppler in agreements, and (2) UE azimuth angle with respect to the orbital plane, we suggest the following: Working assumption: For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered and will be captured in the TR 38.742. For the variables in the equations, refer to Figure-X. Considering satellite location at s=0,0,RE+h One-way delay between a point u and satellite: done-wayu=¦u-s¦2c One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along k=cosα,sinα,0 with speed vue: dopplerone-wayu=vcfcu-s,x>done-wayu+vuecfcs-u,k>done-wayu Let R be the location uncertainty area. Differential one-way delay = maxu∈Rdone-wayu-m∈u∈Rdone-wayu Differential one-way doppler = maxu∈Rdopplerone-wayu-m∈u∈Rdopplerone-wayu For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ϕ from the orbital plane (x-y plane). Note: Assume satellite and UE moves parallel to the x-y plane. ⋅,⋅> is the vector inner product, ∨.∨2 is the vector magnitude. |
| Tejas Networks | On the proposed equations, Based on RAN1 agreements , the differential doppler and differential delay is defined based on the nearest point in the beam from the satellite and farthest point in the beam from the satellite. So the reference point used in these equations can be modified accordingly For Nadir beams this reference point can be the centre of the beam as it is the nearest point to the satellite. Also agree with Qualcomm proposal as a starting point as the geometry model captures all relevant parameters required for the evaluation |
| Samsung | Based on the discussions made yesterday we make the following modifications. Basically, it’s similar to ZTE’s modifications. However, for f_d_min (or as ZTE name it f_d_near) we have a different understanding, and we think for the minimum case we should assume that the UE is moving in the same direction the Satellite d=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα φ1=φ-2RcRE dmin=RE+hue2+RE+hs2-2RE+hueRE+hscosφ1 φ=sin-1dRE+hscosα β=π2-α+φ β1=sin-1REdrefsinφ1 γ=cos-1RE+hsdrefsinφ1 fdmax=vssinβ+vuecosαcfc fdmin= ∆t=d-dminc ∆fd=fdmax-fdmin |
| CEWiT | We support the proposal with the modification suggested by ZTE. |

## Proposal 7-1

Proposal 7-1-v1

For the study on GNSS resilient NR-NTN operation, for the cases where RAN1 concludes that current PRACH tolerance is exceeded, RAN1 to consider the following solutions for initial access:

- Option 1: Solutions based on modified PRACH procedures:

  - Option 1A: Multiple PRACH transmissions (e.g. with different roots or different formats)

  - Option 1B: New PRACH restricted sets.

  - Option 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR

- Option 2: Solutions based on reducing time-frequency uncertainty

  - Option 2A: multi-satellite DL-TDOA based on current specifications.

  - Option 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

  - Option 2C: Solutions based on broadcasting DL timestamp(s)

- Option 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

- Other options are not precluded

- For all the options above:

  - common service link time/frequency pre-compensation can be considered (e.g. based on reference point or based on explicit indication of common time/frequency pre-compensation).

  - RAN4 requirements on initial time/frequency accuracy may need to be revisited

    - Liaison with RAN4 may be needed

Companies are encouraged to provide comments on Proposal 7-1-v1

| Companies | Comments |
| --- | --- |
| NEC | Fine with the above list. |
| JHUAPL | Support. |
| Futurewei | For understanding, could the proponents of Option 1 (the sub-options therein) elaborate on what changes need to be made to the legacy PRACH procedures? Does Option 2C require new signalling? |
| CMCC3 | The list seems not complete. In our contribution, we also provide single satellite based positioning to narrow down UE’s location uncertainty. Option 2 can be updated as below. - Option 2: Solutions based on reducing time-frequency uncertainty - Option 2A: single/multi-satellite DL-TDOA based on current specifications. - Option 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area) - Option 2C: Solutions based on broadcasting DL timestamp(s) - Option 2D: UE side time/frequency pre-compensation based on reference location. |
| LGE | Not support. RAN1 should first establish consensus observations regarding the problem before deciding whether enhancements to specific features are necessary. Since such consensus has not yet been reached, this proposal is premature for discussion. Furthermore, the proposal combines multiple issues into a single proposal, including common pre-compensation, PRACH timing tolerance, PRACH frequency tolerance, RAR procedures, Msg3 handling, and RAN4 requirements. Mixing these topics makes it difficult to have a focused and productive discussion. We would like to emphasize that even without having this kind of agreement, it is enough to capture the relevant information in FL summary, and companies can refer to this when preparing their contributions. |
| ZTE | We think the following updates should be considered. Firstly the solutions in option 1 is actually to “improve the tolerance to time-frequency uncertainty”, which corresponds to “reducing time-frequency uncertainty” in option 2. It is preferred to align the dimension of expression. And the examples can be removed since too much details are not needed at current stage. Moreover, regarding option 1C, it is preferred to generally state enhancement on RAR as there are also other proposed enhancement such as modified TAC range. Regarding option 2, the solution to use last acquired GNSS position to perform pre-compensation is also a potential solution, since the time-frequency uncertainty can be reduced when GNSS error not larger than beam size. And for the last bullet, “common” should be removed for in “common service link time/frequency pre-compensation”, since UE specific pre-compensation may be possible at least for option 2 like solutions. With above considerations, the proposal can be updated as shown below: Proposal 7-1-v1 For the study on GNSS resilient NR-NTN operation, for the cases where RAN1 concludes that current PRACH tolerance is exceeded, RAN1 to consider the following solutions for initial access: - Option 1: Solutions based on modified PRACH procedures improving tolerance to time-frequency uncertainty: - Option 1A: Multiple PRACH transmissions (e.g. with different roots or different formats) - Option 1B: New PRACH restricted sets. - Option 1C: Single PRACH transmission with multiple Enhancement on TAC/FAC commands in RAR - Option 2: Solutions based on reducing time-frequency uncertainty - Option 2A: multi-satellite DL-TDOA based on current specifications. - Option 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area) - Option 2C: Solutions based on broadcasting DL timestamp(s) - Option 2D: service link time/frequency pre-compensation based on last acquired GNSS position - Option 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay. - Other options are not precluded - For all the options above: - common service link time/frequency pre-compensation can be considered (e.g. based on reference point or based on explicit indication of common time/frequency pre-compensation). - RAN4 requirements on initial time/frequency accuracy may need to be revisited - Liaison with RAN4 may be needed |
| Toyota | We suggest updating the Option 1 and 2 as follows: - Option 1: Solutions based on modified PRACH procedures: - Option 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) - Option 1B: New PRACH restricted sets. - Option 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR - Option 2: Solutions based on reducing time-frequency uncertainty - Option 2A: multi-satellite DL-TDOA based on current specifications. - Option 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on using multiple reference points locations within the uncertainty area) - Option 2C: Solutions based on broadcasting DL timestamp(s) We prefer following approach for describing the solutions as the logical flow is the steps a UE usually takes in usual operation. These steps are described in detail in our contribution R1-2507642 (#122-bis). Figure 1: 3-step unified framework for uplink time and frequency synchronization Options for Step 1 (time and frequency compensation prior to PRACH transmission, i.e., pre-compensation stage): - Option 1A: <Descriptions for Option 1A> - Option 1B: <Descriptions for Option 1B> - Option 1C: <Descriptions for Option 1C> - … Options for Step 2 (Solutions during PRACH transmissions): - Option 2A: <Descriptions for Option 2A> - Option 2B: <Descriptions for Option 2B> - Option 2C: <Descriptions for Option 2C> - … Options for Step 3 (Solutions after PRACH transmissions): - Option 3A: <Descriptions for Option 3A> - Option 3B: <Descriptions for Option 3B> - Option 3C: <Descriptions for Option 3C> - … We prefer observations first and categorization based on common understanding next. |
| Ericsson | Add Option 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command) |
| Tejas Networks | Support the above list |
| ESA | Support the above list |
| Ofinno | Support the above list |
| CEWiT | Support |
| Airbus | Support. |

## Proposal 2-1

Proposal 2-1-v1:

The companion spreadsheet [*] is used to collect companies’ results on differential round trip delay and differential round Doppler. - [*] Evaluation of differential delay and Doppler v000_Moderator

Companies are encouraged to further comment on the spreadsheet

| Companies | Comments |
| --- | --- |
| NEC | As agreed in last meeting, the UE altitude error must be taken into account and reported. The current spreadsheet needs to include a dedicated row for this metric. For case-a: Orbit. Band/Carrier Frequency. Beam diamter (km). Elevation angle (°). Altitude Error (km). For case-b: Orbit. Carrier Frequency. X (km). Elevation angle (°). Altitude Error (km). |
| Futurewei | For “Differential RTT-case b”, the range of X values is the same for the S band and the Ka band. The range of X values (including the upper limit) should be defined according to the beam radius of the S band and Ka band for Set 1 and Set 2 for LEO-600, LEO-1200 and GSO. (Set 2 is low priority according the previous RAN1 agreements). The same for “Differential round Doppler-case b. |
| LGE | In accordance with the agreements, evaluation results collection for one-way differential delay and one-way differential Doppler should proceed. |
| ZTE | According to following note in previous agreement, both “within orbital plane” and “90° with respect to orbital plane” are considered. Note 4: Footprint of the beam could be: case 1: within the orbital plane, case 2: at 90° with respect to orbital plane. Companies to report the elevation angle of the beam. The table may need to be updated to reflect the two cases at least for Doppler evaluation, e.g., split the row for 30 degree to two rows for 30 degree case 1 and 30 degree case 2. |
| Toyota | We support to collect the results through an excel sheet. However, we don’t see the need to report Scenario 1 and 2 separately as we are calculating the maximum Differential doppler and RTT within an area uncertain to the UE. We have following suggestions. Comment 1: If FL see the absolute need for collecting results for Scenarios 1 and 2 separately, we would like to report the results for moderately high and large T values/UE speeds. For example, we can add x=45 km, x=90km for 2GHz, LEO600 (Set 2). We can keep the workload same by spacing the x km values with higher separation between them. Comment 2: We strongly support the merge table where representative maximum Differential doppler and RTT are reported by companies. |
| Ericsson | Support with minor comment: In the proposal text: fix typo "differential roundtrip Doppler" |
| Tejas Networks | Support the proposal |
| Ofinno | Support the proposal |
| CEWiT | Support |

## Proposal 1-3

Proposal 1-3-v0

For the evaluation of frequency and timing adjustment methods, RAN1 will consider the following evaluation methodology:

- Step 1: Drop the UE at a location where the satellite is visible at an elevation angle θ

- Step 2: Propagate the satellite during the simulation time T (assuming a circular trajectory around the center of Earth)

- Step 3: The UE applies an initial timing and frequency offset (randomly distributed within Te and fe, which are the residual timing and frequency errors after successful completion of random access) and updates the timing and frequency adjustments at time t based on a correction method

- Step 4: The network evaluates the timing and frequency error after every TUL ms, assuming an uplink transmission every TUL ms

- Step 5: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively

- Using the steps mentioned above, RAN1 to evaluate timing and frequency control using the following metric: Number of TA and FA commands issued within T.

| Companies | Comments |
| --- | --- |
| CMCC3 | One clarification is need for Ttol and Ftol in step 5. Should tolerance limits for timing and frequency based on RAN4 requirements for uplink transmission ? Or the timing and frequency tolerance would be a value as certain scaled CP value or scaled SCS ? |
| Ericsson | Support |
| Tejas Networks | Support |

# Proposals for Thursday offline

## Proposal 7-1

Proposal 7-1-v2

For the study on GNSS resilient NR-NTN operation, for the cases where RAN1 concludes that current PRACH tolerance is exceeded, RAN1 to consider the following solutions for initial access:

- Solution 1: To improve robustness:

  - Solutions based on modified random access procedures, using existing PRACH formats:

    - Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area)

    - Solution 1B: New PRACH restricted sets.

    - Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR

    - Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command)

    - Solution 1E: Optimized PRACH root

    - Solution 1F: Using TA margin and RO masking.

    - Other solutions are not precluded

- Solution 2: Solutions based on reducing time-frequency uncertainty and applying a new service link time/frequency pre-compensation

  - Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

  - Solution 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

  - Solution 2C: Solutions based on broadcasting DL timestamp(s)

  - Solution 2D: UE side time/frequency pre-compensation based on reference location.]

  - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position]

  - Other solutions are not precluded

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

- Other Solutions are not precluded

- For some solutions above the following could be considered:

  - Service link time/frequency pre-compensation can be considered (e.g. based on reference point or based on explicit indication of common time/frequency pre-compensation).

- For all the solutions above:

  - RAN4 requirements on initial time/frequency accuracy may need to be revisited

    - Liaison with RAN4 may be needed

  - Increase the tolerance of PRACH

## Proposal 12-1

Proposal 12-1-v0

Study coexistence strategies between legacy (GNSS-dependent) and GNSS-unavailable UEs, including the separation of PRACH resources.

## Proposal 1-4

Proposal 1-4-v3

Working assumption: For the calculation of the differential one-way delay and the differential one-way Doppler/frequency offset the following Alt could be considered: geometry model and equations could be considered. For the variables in the equations, refer to Figure below. For non-nadir beam dfar=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα. φ1=φ-2RcRE. drefnear=RE+hue2+RE+hs2-2RE+hueRE+hscosφ1. φ=sin-1dfarRE+hscosα. β=π2-α+φ. β1=sin-1RE+huedrefnearsinφ1. γ=cos-1RE+hsdrefnearsinφ1. fdestfar=vssinβ+vuecosαcfc. fdrefnear=vssinβ1+vuecosγcfc. ∆t=d-drefdfar-dnearc. ∆fd=fdref-fdestfdfar-fdnear For nadir beam dnear=hs-hue φ=RcRE dfar=RE+hue2+RE+hs2-2RE+hueRE+hscosφ β=sin-1RE+huedfarsinφ α=π2-β+φ fdfar=vssinβ+vuecosαcfc ∆t=dfar-dnearc ∆fd=2fdfar. Note 1: UE is assumed to be at farthest point and the reference point at nearest point, and UE is at same height hue as the reference point. Note 2: the minimum elevation is at farthest w.r.t satellite point of the uncertainty area Note 3: For the purpose of evaluation, for Scenario 1 , fixed size of uncertainty area is used within the whole satellite coverage footprint FFS: How the take into account the UE altitude uncertainty.

Alt 2: Working assumption: For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered and will be captured in the TR 38.742. For the variables in the equations, refer to Figure-X. Considering satellite location at s=0,0,RE+h One-way delay between a point u and satellite: done-wayu=¦u-s¦2c One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along k=cosα,sinα,0 with speed vue: dopplerone-wayu=vcfcu-s,x>done-wayu+vuecfcs-u,k>done-wayu Let R be the location uncertainty area. Differential one-way delay = maxu∈Rdone-wayu-m∈u∈Rdone-wayu Differential one-way doppler = maxu∈Rdopplerone-wayu-m∈u∈Rdopplerone-wayu For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ϕ from the orbital plane (x-y plane). Note: Assume satellite and UE moves parallel to the x-y plane. ⋅,⋅> is the vector inner product, ∨.∨2 is the vector magnitude.

## Proposal 1-3

Proposal 1-3-v0

For the evaluation of frequency and timing adjustment methods, RAN1 will consider the following evaluation methodology:

- Step 1: Drop the UE at a location where the satellite is visible at an elevation angle θ

- Step 2: Propagate the satellite during the simulation time TSim (assuming a circular trajectory around the center of Earth)

- Step 3: The UE applies an initial timing and frequency offset (randomly distributed within Te and fe, which are the residual timing and frequency errors after successful completion of random access) and updates the timing and frequency adjustments at time t based on a correction method

- Step 4: The network evaluates the timing and frequency error after every TUL ms, assuming an uplink transmission every TUL ms

- Step 5: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively

- Using the steps mentioned above, RAN1 to evaluate timing and frequency control using the following metric: Number of TA and FA commands issued within TSim.

## Proposal 2-1

Proposal 2-1-v1:

The companion spreadsheet [*] is used to collect companies’ results on differential round trip delay and differential round Doppler. - [*] Evaluation of differential delay and Doppler v000_Moderator

# Conclusion

# FL Recommendations

For the upcoming RAN1 meeting, to facilitate the collection of companies’ results on differential one-way delay and differential one-way Doppler evaluations, companies are encouraged to use the companion spreadsheet “Evaluation of Differential Delay and Doppler v1.00”.

Regarding the calculation equations/methodology agreed on Thursday’s online session, companies may consider the following clean text format for Alternative 1 (Alt1).

For non-nadir beam dfar=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα. φ1=φ-2RcRE. dnear=RE+hue2+RE+hs2-2RE+hueRE+hscosφ1. φ=sin-1dfarRE+hscosα. β=π2-α+φ. β1=sin-1RE+huednearsinφ1. γ=cos-1RE+hsdnearsinφ1. fdfar=vssinβ+vuecosαcfc. fdnear=vssinβ1+vuecosγcfc. ∆t=dfar-dnearc. ∆fd=fdfar-fdnear For nadir beam dnear=hs-hue φ=RcRE dfar=RE+hue2+RE+hs2-2RE+hueRE+hscosφ β=sin-1RE+huedfarsinφ α=π2-β+φ fdfar=vssinβ+vuecosαcfc ∆t=dfar-dnearc ∆fd=2fdfar.

# RAN1#122bis agreements

RAN1#122bis made the following agreements and observations:

Agreement:

For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions:

- For scenario 1 and 2

  - The following time/frequency differences should be evaluated:

    - The differential one-way delay (in µs) as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area.

      - For UL performance evaluation 2 times differential one-way delay should be considered.

    - The differential one-way Doppler/frequency offset (in ppm) is calculated between the largest UE-sat Doppler/frequency offset and smallest UE-sat Doppler/frequency offset within the uncertainty area.

      - For UL performance evaluation, scaled one-way differential Doppler/frequency offset to determine UL differential Doppler/frequency offset should be considered. Scaling factor should be reported.

Agreement:

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration×maxNcsL) - 2 Te ≤ Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or  (scaling factor*(Differential Doppler) +2* fe×fc) ≥ (1+γ)ΔfRA

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively.

      - Othe values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

Agreement:

RAN1 to assess the impacts on connected mode during NR NTN GNSS-resilient operation, including at least:

- Impact on signalling overhead.

Observation:

For the study on GNSS resilient NR-NTN operation, the following solutions for initial access to increase PRACH tolerance and reducing time/frequency uncertainty are listed based on inputs/discussions in RAN1#122bis:

- Solution 1: To improve robustness:

  - Solutions based on modified random access procedures, using existing PRACH formats:

    - Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

    - Solution 1B: New PRACH restricted sets.

    - Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR

    - Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command)

    - Solution 1E: Selection of a set of existing PRACH root

    - Solution 1F: Using TA margin and RO masking.

    - Other solutions are not precluded

- Solution 2: Solutions based on reducing time-frequency uncertainty

  - Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

  - Solution 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

  - Solution 2C: Solutions based on broadcasting DL timestamp(s)

  - Solution 2D: UE side time/frequency pre-compensation based on reference location.

  - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position

  - Solution 2F: UE side time/frequency pre-compensation based on reference signal.

  - Other solutions are not precluded

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

- Other Solutions are not precluded

Agreement:

In RAN1#123, based on the following alternatives representing two calculation methodologies, select a calculation methodology with potential adjustments, including correction of equations:

Alt 1:

For the calculation of the differential one-way delay and the differential one-way Doppler/frequency offset the following could be considered:

Geometry model and equations could be considered. For the variables in the equations, refer to Figure below.

For non-nadir beam

|  | dfar=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα |
| --- | --- |
|  | φ1=φ-2RcRE |
|  | drefnear=RE+hue2+RE+hs2-2RE+hueRE+hscosφ1 |
|  | φ=sin-1dfarRE+hscosα |
|  | β=π2-α+φ |
|  | β1=sin-1RE+huedrefnearsinφ1 |
|  | γ=cos-1RE+hsdrefnearsinφ1 |
|  | fdestfar=vssinβ+vuecosαcfc |
|  | fdrefnear=vssinβ1+vuecosγcfc |
|  | ∆t=d-drefdfar-dnearc |
|  | ∆fd=fdref-fdestfdfar-fdnear For nadir beam dnear=hs-hue φ=RcRE dfar=RE+hue2+RE+hs2-2RE+hueRE+hscosφ β=sin-1RE+huedfarsinφ α=π2-β+φ fdfar=vssinβ+vuecosαcfc ∆t=dfar-dnearc ∆fd=2fdfar |

Note 1: UE is at same height hue as the reference point.

Note 2: For the purpose of evaluation, fixed size of uncertainty area is used within the whole satellite coverage footprint is a baseline. Other size of uncertainty area is not precluded.

FFS : Whether the minimum elevation is at farthest w.r.t satellite point of the uncertainty area  or at the center of the uncertainty area

FFS: How to take into account the UE altitude uncertainty.

Alt 2:

For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered. For the variables in the equations, refer to Figure-X.

Considering satellite location at s=0,0,RE+h

One-way delay between a point u and satellite:
done-wayu=|u-s|2c

One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along k=cosα,sinα,0 with speed vue:

dopplerone-wayu=vcfc  <u-s,x>done-wayu+vuecfc<s-u,k>done-wayu

Let R be the location uncertainty area.

Differential one-way delay = maxu∈R done-wayu-minu∈R done-wayu

Differential one-way doppler = maxu∈R dopplerone-wayu-minu∈R dopplerone-way(u)

For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ϕ from the orbital plane (x-y plane).

Note: Assume satellite and UE moves parallel to the x-y plane. <⋅,⋅> is the vector inner product, ||.|​2 is the vector magnitude.

# RAN1#122 agreements

RAN1#122 made the following agreements.

Agreement

For the study on GNSS resilient operation, scenarios should be considered where:

- Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link.

- Scenario 2: There is a previously acquired GNSS based position

  - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded

    - FFS: value of T

    - FFS: GNSS based UE location validity duration

    - FFS: GNSS accuracy

- Note: Two scenarios above belong to description as in SID.

Agreement

For the study on GNSS resilient operation, the following scenarios are considered for evaluation purposes:

| Satellite orbit | GSO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Satellite scenario parameters | At least Table 6.1.1.1-1 in TR 38.821, Parameters in Table 6.1.1.1-2 in TR 38.821could be considered. |  |  |
| Beam size (note 3) | At least values captured in Table 6.1.1.1-1/2 are considered, |  |  |
| Minimum elevation angle | 30° (LEO), 12.5° (GSO) |  |  |
| Frequency ranges/bands | FR1 NTN (2GHz/Ku: 14 GHz) and FR2 NTN (30 GHz) |  |  |
| UE type | Handheld (L/S band), VSAT (Ku/Ka) |  |  |
| UE speed (note 1) | L/S band: 3 km/h, 120km/h, 1500 km/h, Ku/Ka: 3 km/h , 120 km/h, 1500 km/h, (note 2) |  |  |
| Beam/cell type (note 4) | (Quasi)-Earth-fixed beams/cells and Earth-moving |  |  |
| Note 1: UE altitude is considered Note 2: 10 km altitude in case of aircraft scenario. 1500 km/h is for aircraft scenario. Note 3: Same values can be reused for other elevation angle than nadir. Beam size of the edge beam can be reported by companies. Note 4: Footprint of the beam could be: case 1: within the orbital plane, case 2: at 90° with respect to orbital plane. Companies to report the elevation angle of the beam. Note 5: Other parameters can be reported by companies. |  |  |  |

Note: The cyan highlighted parameters are prioritized for evaluation purposes.

Agreement

For PRACH performance evaluation for existing PRACH formats, adopt the following methods:

- Baseline: Analytical characterization of performance based on e.g. properties of ZC sequences and values of differential Delay / Doppler.

- Optional: Link level evaluations

  - The following table is proposed for LLS parameters

| Configurations | S/L-band | Ka/Ku-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | For Ka: UL 30GHz, for Ku: UL 14 GHz |
| Channel Model | Baseline NTN TDL-C rural model in TR38.811 |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx | 1 Rx |
| Antenna Configuration at the UE | A single omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| PRACH format | To be reported by companies. | To be reported by companies |
| PRACH configuration | To be reported by companies. |  |
| Metric | PRACH detection rate, FAR (Based on the preamble pool size is not less than 64), CDF of estimation error for frequency/timing |  |
| Receiver | Companies to report the receiver for PRACH detection. |  |

Agreement

For evaluation regarding GNSS-resilient operation, based on GNSS information or the lack of it, it is assumed that the UE has a horizontal location uncertainty within an area

- Case a: the location uncertainty area is the area served by the cell or beam.

- Case b: the location uncertainty area is a circle of radius X km

  - X to be reported

UE altitude error should be taken into account.

- UE altitude error to be reported.

For both Case a and Case b above, RAN1 to evaluate at least:

- Differential delay values

- Differential frequency offset/Doppler values

# References

| - RP-251933 | Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation | Thales (Rapporteur) |
| --- | --- | --- |
| - R1-2506749 | Discussion on Rel-20 GNSS resilient NR NTN operation | FUTUREWEI |
| - R1-2506785 | On NR-NTN GNSS resilience | Ericsson |
| - R1-2506812 | Discussion on NR NTN GNSS resilience | Spreadtrum, UNISOC |
| - R1-2506842 | Considerations on NR-NTN Resilience to GNSS Unavailability and Degradation | THALES |
| - R1-2506896 | Discussion on NR-NTN GNSS resilience | vivo |
| - R1-2506917 | Discussion on NR-NTN GNSS resilience | ZTE Corporation, Sanechips |
| - R1-2506941 | GNSS resilience for NR-NTN | Huawei, HiSilicon |
| - R1-2506987 | Discussion on NR-NTN GNSS resilience | Xiaomi |
| - R1-2507012 | Discussion on NR-NTN GNSS resilience | CMCC |
| - R1-2507117 | Discussion on NR-NTN GNSS resilience | CATT |
| - R1-2507129 | Discussion on the GNSS resilient NR-NTN operation | TCL |
| - R1-2507130 | Discussion on NR-NTN GNSS resilience | SageRAN |
| - R1-2507174 | Discussion on NR-NTN GNSS resilience | OPPO |
| - R1-2507202 | GNSS Resilient NR-NTN Operation -NR-NTN Phase 4 | Tejas Network Limited |
| - R1-2507211 | Discussion on NR-NTN GNSS resilience | HONOR |
| - R1-2507217 | NR-NTN GNSS resilience | InterDigital, Inc. |
| - R1-2507251 | Discussion on NR-NTN GNSS resilience | Samsung |
| - R1-2507321 | Discussion on NR-NTN GNSS resilience | NEC |
| - R1-2507333 | Discussion on NR-NTN GNSS resilience | China Telecom |
| - R1-2507379 | Discussion on GNSS resilient NR-NTN operation | LG Electronics |
| - R1-2507429 | Ku band parameters and inter-SAN handover with GNSS unavailable | Eutelsat Group |
| - R1-2507465 | Discussion on NR-NTN GNSS resilience | Ofinno |
| - R1-2507504 | Discussion on NR-NTN GNSS resilient operations | ETRI |
| - R1-2507541 | Discussion on NR-NTN GNSS resilience | Fraunhofer IIS, Fraunhofer HHI |
| - R1-2507550 | Discussion on GNSS resilient NR-NTN | Panasonic |
| - R1-2507557 | Discussion on NR-NTN GNSS resilience | Lenovo |
| - R1-2507560 | Discussion on GNSS resilient operation for NR over NTN | Nokia |
| - R1-2507586 | Discussion on NR-NTN GNSS resilience | CCU |
| - R1-2507594 | Discussion on GNSS resilient NR-NTN operation | Sony |
| - R1-2507624 | GNSS resilient operations in NR NTN | MediaTek Inc. |
| - R1-2507642 | Discussion on GNSS resilient NR-NTN operation | Toyota ITC |
| - R1-2507643 | Preamble transmission and detection in NR NTN GNSS resilient operation | Sharp |
| - R1-2507675 | Considerations of NR-NTN GNSS Resilient Operations | Apple |
| - R1-2507719 | NR-NTN GNSS resilience | Qualcomm Incorporated |
| - R1-2507761 | Discussion on Rel-20 GNSS resilient NR NTN operation | ESA, Airbus, Thales |
| - R1-2507782 | Discussion on GNSS-resilient NR-NTN operation | ISSDU, NYCU |
| - R1-2507784 | Discussion of NR-NTN GNSS Resilience | Johns Hopkins University APL |
| - R1-2507811 | Discussion on NR-NTN GNSS resilience | NTT DOCOMO, INC. |
| - R1-2507845 | Discussion on NR-NTN GNSS resilience | CAICT |
| - R1-2507850 | Discussion on GNSS resilience for NR-NTN | CSCN |
| - R1-2507857 | Discussion on NR-NTN GNSS resilience | Google Korea LLC |
| - R1-2507901 | Discussion on GNSS Resilient Operation for NR NTN | CEWiT |
| - R1-2507937 | Considerations for GNSS-resilient NR-NTN operation | ST Engineering iDirect |