3GPP TSG RAN WG1 Meeting #123	R1- 2508987

Dallas, USA, Nov 17th – 21st , 2025

Agenda Item:	10.1.2

Source:	HONOR

Title:	Inter-vendor training collaboration for two-sided CSI compression

Document for:	Discussion and Decision

- Introduction

In RAN#108, it is concluded that CSI compression moves to normative phase. Following objectives are captured in updated WID [1] as below:

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) levering defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In this contribution, we provide our views on the inter-vendor collaboration aspects for sub-options 4-1 of Direction A.

- Discussion

  - Target CSI and CSI feedback format

The following agreements have achieved regarding target CSI and CSI feedback formats.

Agreement in RAN1#122 - For Option 4-1 under Direction A in AI/ML based CSI compression, support at least target CSI and CSI feedback in the exchanged dataset(s). - FFS: Target CSI type and format - FFS: CSI feedback type and format - FFS: Association between Target CSI and CSI feedback, including mapping for different number of Tx port, number of sub bands, and CSI payload size. Agreement in RAN1#122 For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, down select one of the following options: - Option 1: The exchanged CSI feedback is the latent message before quantization. - Option 2: The exchanged CSI feedback is the binary sequence at the output of quantization. - Note: Quantization codebook is exchanged in addition if not specified in the spec. - Note: Whether or not using the Quantization codebook for model training, is up to UE implementation.

For the target CSI type, Section 10.1.1.1 has decided to support a port–subband domain-based precoding matrix. For the target CSI format, Section 10.1.1.2 has not yet reached a conclusion. For the exchanged dataset in Option 4-1 under Direction A, we recommend adopting the same target CSI type and format across data collection, inference, and monitoring to ensure consistency and reduce standardization effort.

Proposal 1: Support using the same target CSI type and format for data collection, monitoring, and dataset exchange.

For CSI feedback type and format in Option 4-1 under Direction A of AI/ML-based CSI compression, two options were discussed in RAN1#122. We support Option 1. Some companies have expressed concerns that exchanging CSI feedback before quantization may lead to mismatches in the implementation of the quantization codebook. We believe that exchanging CSI feedback before quantization together with the corresponding quantization codebook can avoid this issue. In this approach, the CSI feedback before quantization is scalar-quantized using Float32. For example, one CSI feedback set may have N quantization codebooks, since a single encoder can employ multiple quantization schemes to achieve different quantization resolution. The performance target is defined for each quantization codebook.

Proposal 2: For Option 4-1 under Direction A in AI/ML-based CSI compression, support exchanging CSI feedback before quantization.

  - Association between target CSI and CSI feedback

Considering the association between target CSI and CSI feedback, the following agreements are reached at the RAN1#122bis meeting.

Agreement in RAN1#122bis For Option 4-1 under Direction A in AI/ML based CSI compression, for the association between target CSI and CSI feedback in the exchanged dataset. - Support 1: M mapping between one target CSI sample/set of samples to M CSI feedback samples/sets of samples associated with different CSI payload size configurations. - FFS: Association between Target CSI and CSI feedback, including scalability related information for different number of Tx port, sub bands related information. - FFS: Whether/how Target CSI samples/set of samples within an exchanged dataset, with different Tx port and sub-band related information are organized. - FFS: Whether the Target CSI is per layer or per rank when Target CSI is precoder matrix. - FFS: Whether and how to support CSI feedback structure with different payload scalability options - FFS: Other details

Considering the first bullet regarding the association between target CSI and CSI feedback, we would like to further clarify that the associated CSI feedback set may configured with a subset of the target CSI’s ports and subbands.

Proposal 3: For Option 4-1 under Direction A in AI/ML-based CSI compression, support 1: M mapping between one target CSI sample/set of samples to M CSI feedback samples/sets of samples associated with different CSI payload size configurations where each CSI feedback set can adopt partial ports and subbands of the target CSI set.

Regarding the fourth FFS items, we support target CSI is per rank when target CSI is precoder matrix, it is feasible for all two-sided model setups. To be specific, if rank-specific model is adopted for the exchanged dataset, but the target CSI is organized per layer, the rank value information is missing. In such cases, the UE may train an encoder that is rank-common, which can lead to a mismatch with the NW-side decoder. By organizing the target CSI per rank, this mismatch between UE and NW training conditions can be avoided. For example,

- if a rank-common or layer-common model is used, the shared target CSI with a single layer is feasible for all ranks and layers. Define a common performance target for all rank values or all layer values is sufficient for UE-sided offline training.

- if a rank-specific and/or layer-specific model is used, the network can additionally indicate the rank and layer index along with the target CSI set. Defining a performance target for each layer for a certain rank is sufficient for UE-sided offline training.

In summary, regardless of whether a rank-common, layer-common, or rank-/layer-specific model is adopted for inter-vendor training collaboration, per-rank-based target CSI sharing remains feasible for UE-side offline training.

Proposal 4: Support target CSI is per rank when target CSI is precoder matrix.

  - Performance Target

Considering performance target, the following agreements are reached at the last two RAN1 meetings.

Agreement in RAN1#122 For Option 4-1 under Direction A in AI/ML based CSI compression, support at least target CSI and CSI feedback in the exchanged dataset(s). - FFS: Target CSI type and format - FFS: CSI feedback type and format - FFS: Association between Target CSI and CSI feedback, including mapping for different number of Tx port, number of sub bands, and CSI payload size. Agreement in RAN1#122 For Option 4-1 under Direction A in AI/ML based CSI compression, consider the following methods for performance target in the exchanged dataset, - SGCS - FFS: Average SGCS - FFS: SGCS values at X-percentiles - NMSE: - FFS: When the exchanged CSI feedback is the floating-point values at the input of quantization - FFS: When the exchanged CSI feedback is the binary bit sequence at the output of quantization, the binary sequence will be mapped back to the floating-point values via quantization codebook - FFS: Multiple performance targets for different layer when the target CSI type is precoding matrix, different configurations such as antenna ports, subband configuration and payload size configuration Agreement in RAN1#122bis • For Option 4-1 under Direction A in AI/ML based CSI compression, both average SGCS and average NMSE as performance target in the exchanged dataset are supported. Both average SGCS and average NMSE are provided. • NW side can optionally send NMSE and/or SGCS values at X-percentiles.

For the final FFS item, we propose supporting multiple performance targets for different layers when the target CSI type is a precoding matrix. This is because, given a specific rank value and payload size configuration, the upper-bound performance target for Layer 3 and Layer 4 is typically lower than that for Layer 1 and Layer 2. Defining a layer-specific performance target CSI allows us to better capture the performance target differences across layers.

In addition, under varying antenna port and subband configurations, the performance target may differ depending on whether the network trains separate models for each antenna port and subband configuration. Furthermore, for a given antenna port and subband configuration, the payload size configuration of CSI feedback can be flexibly selected to support different reporting resolutions and channel conditions.

Proposal 5: For Option 4-1 under Direction A, support multiple performance targets

- for different layer when target CSI type is precoding matrix.

- for different antenna port and subband configurations.

- for different payload size configurations.

In the inference process, the reported CSI feedback may experience UCI omission when UCI bits are insufficient. The performance target defined under the assumption of complete CSI feedback and target CSI pairs may not be accurate or feasible when CSI feedback is incomplete. Therefore, we suggest defining the performance target for different numbers of omitted UCI groups.

Proposal 6: For Option 4-1 under Direction A in AI/ML-based CSI compression, support performance target calculation with UCI omission.

Furthermore, if CSI feedback can omit UCI groups, the target CSI can also omit UCI groups. In this case, the performance target can be defined for target CSI and corresponding CSI feedback with the same omitted UCI groups, to guide performance evaluation under UCI omission conditions. For the third bullet, we have the following proposal:

Proposal 7: For Option 4-1 under Direction A in AI/ML-based CSI compression, support performance target calculation per target CSI and CSI feedback omission configuration.

  - model fine-tuning

In the RAN1#122 meeting, the following agreement related to model fine-tuning was made.

Agreement in RAN1#122 For Option 4-1 under Direction A in AI/ML based CSI compression, support exchange of single pairing ID along with dataset exchange. - One pairing ID is assigned to one dataset. - The dataset can have different number of Tx port, number of subbands, and CSI payload size configurations (including different quantization codebooks if needed). - From RAN1 perspective, the uniqueness of the pairing ID needs further studied. - FFS: the impact on pairing ID(s) when additional data samples are added to an exchanged dataset, if supported.

The first question is whether the same pairing ID can be used when additional data samples are added to an exchanged dataset? In our views, reusing the original pair ID can reduce signaling overhead, particularly if the initial dataset (e.g., dataset A) and quantization codebook remain unchanged. From a network perspective, introducing new pair IDs in model fine-tuning will greatly increase the complexity of managing pair IDs.

However, using the same pair ID may introduce other issues. When an additional dataset (e.g., dataset B) is sent with the same pair ID, the UE behavior must clarify whether UE should use only dataset B or both dataset A and dataset B for model retraining. Explicit indicator may be required, such as an additional value tag.

Furthermore, additional datasets may affect the performance target of the initial dataset. Two potential solutions can be considered:

- Option1: Network indicates performance target#1 for dataset A and performance target#2 for dataset B.

- Option2: For the same inference configuration, a single performance target can be applied to both dataset A and dataset B.

Proposal 8: For model fine-tuning, support reuse of the original pairing ID, and an additional value tag is required to explicitly indicate whether the dataset is supplemental.

Proposal 9: With the introduction of additional training datasets, the performance target of the initial dataset must also be updated accordingly.

Conclusion

In this paper we make the following observations and proposals:

Proposal 1: Support using the same target CSI type and format for data collection, monitoring, and dataset exchange.

Proposal 2: For Option 4-1 under Direction A in AI/ML-based CSI compression, support exchanging CSI feedback before quantization.

Proposal 3: For Option 4-1 under Direction A in AI/ML-based CSI compression, support 1: M mapping between one target CSI sample/set of samples to M CSI feedback samples/sets of samples associated with different CSI payload size configurations where each CSI feedback set can adopt partial ports and subbands of the target CSI set.

Proposal 4: Support target CSI is per rank when target CSI is precoder matrix.

Proposal 5: For Option 4-1 under Direction A, support multiple performance targets

- for different layer when target CSI type is precoding matrix.

- for different antenna port and subband configurations.

- for different payload size configurations.

Proposal 6: For Option 4-1 under Direction A in AI/ML-based CSI compression, support performance target calculation with UCI omission.

Proposal 7: For Option 4-1 under Direction A in AI/ML-based CSI compression, support performance target calculation per target CSI and CSI feedback omission configuration.

Proposal 8: For model fine-tuning, support reuse of the original pairing ID, and an additional value tag is required to explicitly indicate whether the dataset is supplemental.

Proposal 9: With the introduction of additional training datasets, the performance target of the initial dataset must also be updated accordingly.

- Reference

- RP-251870, “New WID on Aritificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface enhancements,” 3GPP TSG RAN Meeting #108, Qualcomm (Moderator), Prague, Czech, June 9-13, 2025.