3GPP TSG RAN WG1 Meeting #123              	     		R1-2509457

Dallas, USA, November 17-21, 2025

_____________________________________________________________________Agenda item: 10.5.1

Source: LG Electronics

Title: Discussion on evaluation assumptions and performance evaluation for NR ISAC

Document for: Discussion and decision

- Introduction

In this contribution, we discuss the issues related to the performance evaluation in NR-based ISAC systems, focusing on monostatic gNB-based sensing scenarios.

- Discussions

  - Measurement Level and Reporting Quantities

    - Measurement Level Selection Based on Functional Roles

To meet the defined requirements, it is essential to specify the necessary measurements. To this end, various measurement levels were discussed and proposed during the last meeting, as follows:

[FL5][H] Proposal 7.1-1-rev3 - The following measurements are identified in the study of NR ISAC. - Level 1: raw data, which is the time-domain samples or subcarrier-domain samples of the channel response from each antenna port, or anything equivalent - Level 2: (partial raw data) - Amplitude and phase profile of delay, and/or Doppler, and/or angle - E.g., Amplitude and phase profile of delay-angle - Level 3 per path (subset of Level 2): - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles - A path can correspond to one or multiple detected points - A path can correspond to one or multiple detected objects - Level 4 per point: - Option 1: Range, velocity, angle, and power per detected point for a given time stamp - Definition: A point is defined by one range/delay, one Doppler and one angle - Option 2: Position, velocity, and power per detected point for a given time stamp - Definition: A point is defined by one location and one velocity - Level 5: e.g., {position, velocity, no target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported - Level 6: e.g., {position, velocity, target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported - SINR, RSRP from perspective of sensing - Time stamp

Prior to down-selection, it is essential to clarify the capabilities and functionalities of both the gNB and the sensing function (SF) to determine the appropriate measurement level from the RAN1 perspective. In particular, the gNB functionalities—such as its processing capability for generating sensing results and its storage capability for maintaining past (background) sensing results—need to be clearly identified.

According to TS 38.305, which outlines the functionalities of elements involved in UE positioning within the NG-RAN, the roles of the gNB and the Location Management Function (LMF) are described as follows [1]:

5.4 Functional Description of Elements Related to UE Positioning in NG-RAN <omitted text> 5.4.2 gNB The gNB is a network element of NG-RAN that may provide measurement information for a target UE and communicates this information to an LMF. To support NR RAT-Dependent positioning, the gNB may make measurements of radio signals for a target UE, and provide measurement results for position estimation. A gNB may serve several TRPs, including for example remote radio heads, and UL-SRS only RPs and DL-PRS-only TPs. For NTN, a TRP may be located on board the satellite. A gNB may host AI/ML models to infer the measurements from radio signals transmitted by a target UE. The AI/ML model that is used for measurement inference by the gNB may have been trained by the gNB. A gNB may broadcast assistance data information, received from an LMF, in positioning System Information messages. <omitted text> 5.4.4 Location Management Function (LMF) The LMF manages the support of different location services for target UEs, including positioning of UEs and delivery of assistance data to UEs. The LMF may interact with the serving gNB or serving ng-eNB for a target UE in order to obtain position measurements for the UE, including uplink measurements made by an NG-RAN and downlink measurements made by the UE that were provided to an NG-RAN as part of other functions such as for support of handover. The LMF may interact with a target UE in order to deliver assistance data if requested for a particular location service, or to obtain a location estimate if that was requested. The LMF may interact with multiple NG-RAN nodes to provide assistance data information for broadcasting. The assistance data information for broadcast may optionally be segmented and/or ciphered by the LMF. The LMF may also interact with AMFs to provide ciphering key data information to the AMF as described in greater detail in TS 23.273 [35]. For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, gNB positioning capabilities and ng-eNB positioning capabilities. The LMF then invokes these positioning methods in the UE, serving gNB and/or serving ng-eNB. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The LMF may combine all the received results and determine a single location estimate for the target UE (hybrid positioning). Additional information like accuracy of the location estimate and velocity may also be determined. The LMF may interact with the AMF to provide (updated) UE Positioning Capability to AMF and to receive stored UE Positioning Capability from AMF as described in TS 23.273 [35]. <omitted text>

As highlighted, the current gNB is limited to performing signal measurements and providing the results for position estimation. It does not have the capability to select positioning methods or compute the final location. These responsibilities lie with the LMF.

Observation 1: In the current NG-RAN positioning system, since there is no gNB-based positioning functionality defined, assigning position and velocity calculation tasks to the gNB for ISAC may introduce additional processing burden due to its functional limitations:

  - gNB only provides measurement results from radio signals,

  - LMF determines the positioning methods and calculates a single location estimate.

Similar to the positioning systems described above, generating sensing results at a single TRP may be insufficient for accurately estimating position and velocity, particularly in the context of 5G-A ISAC. Generating sensing results—such as position and velocity estimates—based solely on angle and range estimation at a single TRP may not meet accuracy requirements, potentially leading to incorrect target detection and identification. Furthermore, velocity estimation derived from consecutive position estimates can degrade as positioning errors accumulate. In dynamic and densely populated environments, determining similarity across continuous measurements for tracking purposes may also result in a high likelihood of misjudgment. Therefore, the gNB’s capabilities—such as the precision of its sensing signal processing and its ability to detect and identify objects—must be clearly defined to identify the measurements reporting from gNB side.

Observation 2: Generating sensing results at TRP may be insufficient for accurate position and velocity estimation, particularly in dynamic 5G-A ISAC environments.

At the same time, the role of SF should be clarified, as it is responsible for deriving the final sensing results. The SF can aggregate sensing data from multiple entities and generate highly accurate results by leveraging extensive contextual information. By providing intermediate sensing results, which can be measurable results from radio signal such as delay/Doppler/angle, to SF from gNB, it is advisable to delegate determination of sensing results to the SF, provided it is equipped with sufficient auxiliary information and signal processing capabilities.

Observation 3: SF can take responsibility for deriving final sensing results by leveraging contextual information and advanced processing capabilities to enhance accuracy.

To ensure the accuracy of sensing results based on a single TRP, it may be necessary for the sensing function (SF) to provide a substantial amount of auxiliary information, which can result in significant data volume and resource overhead—especially for high-resolution processing. Examples of such auxiliary data include local or global coordinates, historical or contextual background, target-specific features, and other relevant parameters. This not only increases the burden on data transmission but also requires sufficient storage capacity to accommodate the additional information.

Observation 4. Ensuring accurate sensing results from TRP requires extensive auxiliary information from the SF, leading to increased data transmission and storage overhead.

Given these observations, it is proposed that the gNB’s role be clearly defined to focus on generating and reporting intermediate measurable results—such as delay, Doppler, and angle. The SF, on the other hand, should be responsible for deriving final sensing results by aggregating these measurements from multiple gNBs. Accurate sensing require sufficient auxiliary information and computational resources. With its broader contextual awareness and advanced processing capabilities, the SF is better suited to ensure reliable and precise sensing outcomes. In contrast, a single gNB may be inadequate for accurate position and velocity estimation, especially for 5G-A ISAC. Therefore, from the RAN1 perspective, it is essential to explicitly define the roles and limitations of both the gNB and SF to support consistent and high-performance sensing across implementations.

Proposal 1. Prior to discussion on measurements, clarify capability and functionality of gNB and SF.

  - gNB: Responsible for generating and reporting intermediate measurable results.

  - SF: Responsible for deriving final sensing results, potentially by aggregating measurements from multiple gNBs.

To identify the measurement level described in the above FL proposal, key definitions and terminologies—such as "path" and "point"—should be clearly defined. In Level 3, a path can be defined as a set of indistinguishable propagation rays sharing a specific tap delay in the delay profile. Notably, a single path may correspond to multiple points with varying angles and/or Doppler values, and can be further decomposed into individual points through post-processing. Conversely, a point in Level 4 can be defined as a resolution grid in the 3D data cube formed by delay, angle, and Doppler dimensions, and should be associated with a specific delay, angle, and Doppler value. The points may represent scattering points located at different positions, exhibiting different Doppler values. From this perspective, if each path can be distinguished by having a unique association with one delay, one Doppler, and one angle, then the concepts of Level 3 and Level 4 may effectively represent the same measurement.

Proposal 2. Clarify key definitions of ‘path’ and ‘point’:

  - Path: defined as a certain tap delay in delay profile.

  - Point: defined as a resolution grid within a 3D data space composed of delay, angle, and Doppler dimensions.

Specifically, unlike Level 5/6, which abstract away individual points into object-level representations, Level 4 retains the physical characteristics (e.g., range, velocity, angle) of each sensing point, enabling more flexible post-processing and fusion with external data sources. Compared to Level 1/2 (raw data), Level 4 significantly reduces data volume while maintaining sufficient detail for downstream tasks such as localization and trajectory estimation. From these perspectives such as preservation of physical meaning and efficiency, level 4 ensures a balance between sensing performance and practical deployment constraints in 5G-A sensing systems.

Proposal 3. Support Level 4 Option 1 as a measurement level for 5G-A ISAC.

    - Measurement List for 5G-A ISAC

Beyond functional considerations, reporting raw measurement data at the receiver side can lead to substantial overhead, both in terms of signaling and processing. Moreover, deriving final sensing results often involves high computational complexity, which may be impractical for receivers with limited processing capabilities. To streamline efforts during the 5G-Advanced ISAC evaluation phase, it is recommended to reuse the measurement definitions already specified in TR 38.215 [2]. These definitions offer a well-structured and comprehensive framework for sensing-related parameters—such as delay and angle—that are essential for tasks like ranging and positioning. Leveraging these existing formats helps avoid redundant specification work and promotes alignment across related standards. Therefore, a more practical and efficient approach would be to report intermediate results, such as delay, Doppler, and/or angle per detected path or point, rather than raw data or final object-level outputs.

Proposal 4: Range, Doppler, and/or angle per point are reported as measurement results.

One of the key challenges in defining sensing-related measurements is determining the appropriate reporting quantity and structure. While TS 38.215 specifies measurements for positioning—such as Rx-Tx time difference—to derive location estimates, further study is required to define the measurement parameters suitable for sensing. In particular, the existing gNB Rx-Tx time difference, which calculates round-trip time (RTT) between TRP and UE at the subframe level based on the first path, is not directly applicable to TRP-target ranging. Therefore, RTT measurement definitions should be refined from a TRP-based monostatic sensing perspective. For example, a TRP may measure reception time using the same time unit index (e.g., frame, subframe, slot, or symbol) as the transmission time index. This measurement can be performed for each detected path and/or target, where each detected path may be represented as a point characterized by a specific delay, angle, and Doppler, as previously described. The TRP can then report the measured monostatic delay along with the corresponding timestamp to the sensing function. Furthermore, to reduce the reporting overhead, the interesting delay range for measurements can be (pre-)configured by a higher layer. Within this range, the monostatic delay of the m-th detected path may be measured and reported as a relative value with respect to the first detected path within the range. Based on this, the definition of the measurement quantity for ranging can be considered as follows:

The gNB monostatic delay of m-th detected path (or point) is defined as TgNB-RX,m – TgNB-TX Where: TgNB-RX,m of the m-th detected path (or point) (m>=1) is the Transmission and Reception Point (TRP)/Reception Point (RP) received timing of downlink/uplink/flexible/sensing symbol #k of subframe #i, defined by the m-th detected path (or point) in time. The first detected path (or point) (m=1) is a detected path having smallest gNB monostatic delay among the detected paths (or points), where the range of gNB monostatic delays of the detected paths (or point) is (pre-)configured by a higher layer or by network. TgNB-TX is the TRP/Transmission Point(TP) transmit timing of downlink/uplink/flexible/sensing symbol #k of subframe #i that corresponds to the resource used for measuring TgNB-RX,m.

Proposal 5: Refine the definition of Rx-Tx time difference for TRP-target ranging to enable transmission and reception time measurements using the same time index, per path (or point) and/or target.

Moreover, Doppler-related measurements to attain the target’s velocity and power-related measurements to evaluate false alarm and miss detection performance must be newly defined. Regarding the Doppler-related measurement, it can be obtained by fast Fourier transform (FFT). For instance, Doppler can be estimated as an average over a given long-time interval at fixed frequency resource elements, or derived from the instantaneous channel impulse response across multiple symbols. The resulting Doppler estimates cab be reported along with their corresponding timestamp, which are aligned with specific temporal reference point within the CPI (Coherent Processing Interval).

Proposal 6: Doppler measurements can be obtained via FFT over a long-time scale and reported alongside the corresponding timestamp reflecting the reference time of estimation.

Power-related measurements, such as received power, are essential for monitoring the sensing link between the gNB and the target object. Existing radio link monitoring metrics defined in TS 38.215, including RSRP and RSRPP, can serve as a basis to ensure consistency with current 5G systems. However, from the perspective of false alarms and missed detections, measuring received power at the resource level may be insufficient for accurately monitoring the sensing link between the gNB and the object. To support sensing-based operations, it is essential to define new L1-level measurements that can effectively represent the quality of a target or detected path.

For example, it can be measured as the ratio between the received power of the target (or detected) path and that of non-target signals, including background channel and/or noise averaged over measurement resources. In cases with multiple target paths, the measurement may be based on the average or minimum received power of those paths. These values can be reported as a difference between the received power of target and non-target signals, providing insight into signal dominance and interference conditions. To identify target paths, the measurement range in delay, Doppler, or angle domains can be (pre-)configured by higher layers. Within this range, sensing-specific SINR can be measured and reported based on detectable points.

Proposal 7: Sensing-specific SINR can be measured as the ratio or difference between received power of target point(s) and non-target signals.

To support the mobility of the target object, the sensing BS/TRP can no longer be proper for monostatic sensing, e.g. because of the long distance from the target object. In this case, it would be better to switch the sensing BS/TRP that is closer to the target object. For this mobility support operation, BS/TRP can report the estimated RCS value of the detected/tracked target object, so that SF can determine whether or not to switch the sensing entity to other BS/TRP. In addition, the change (or the ratio) of the currently measured delay, Doppler, angle, and/or RCS values from the previously measure values can be reported for that purpose.

Proposal 8: The change between current and previous measurement values can be reported to support sensing/tracking moving object.

    - Quantization of Reporting Measurement

In the context of 5G-A ISAC, in line with measurement level and list considering the existing capability and measurements, the quantization of measurement parameters—such as delay, Doppler, and angle—can be determined based on the target performance requirements. Rather than introducing entirely new quantization schemes, it is more efficient to leverage legacy formats already defined in current 3GPP specifications. For instance, TS 38.455 specifies bit sizes for reporting measurement results, while TS 38.133 provides mapping tables that define measurement ranges, resolutions, and corresponding quantized values [3, 4]. Specifically, the reporting range for RSTD measurement reporting is defined by resolution step based on the granularity parameter (k) and basic time unit (Tc), where the reported value corresponds to the region in which the measurement value falls. These existing frameworks offer a practical baseline for sensing measurement quantization, ensuring consistency and compatibility across implementations. Since they were designed with performance requirements and signaling overhead in mind, using them as a starting point is both practical and efficient.

Proposal 9: The measurement quantization can be determined by target performance requirements, using legacy (e.g., format and report mapping table in TS 38.455 and 38.133) as a starting point.

    - Assistance Information for Sensing

Regarding the measurement reporting, it may not always be necessary to report all the sensing results that Rx module can measure. The extent of range, speed or the direction of interest can be provided by the sensing service consumer from the beginning. Reducing the amount of the reported measurements also helps to save the unnecessary power consumption, otherwise always be reported. In this sense, the extent of the delay(range), Doppler(speed) and/or direction(angle) to be reported can be (pre-)configured by SF (sensing function). In addition to such extent for reporting, the minimum value of the received sensing signal power, RCS, and Doppler frequency level can also be (pre-)configured. Only measurement values larger than the (pre-)configured values can be reported so as not to report a noisy component as a meaningful sensing result.

Furthermore, a specific measurement range and/or time-frequency grid with defined granularity can be configured to help the receiver identify the target measurement region. Given such (pre-)configuration above, there can be additional power saving by reporting only the interested measurement of delay, Doppler, and/or angle values. There can be (pre-)configured prioritized extents of such measurements, so that BS/TRP can report the relevant measurements step-by-step according to the prioritization. In such case, BS/TRP can report e.g. the delay measurement per each interested angle extent, or the angle measurement per each interested delay extent, and so forth.

Proposal 10: The range of the measurement values to be reported is (pre-)configured.

In addition to the measurement reports, BS/TRP can report to SF the information related to the measurement. For example, BS/TRP can report the sensing signal Tx power so that the reported measurement on the received signal power can be properly interpreted by SF to identify the detected objects. Moreover, for BS/TRP monostatic sensing, the transmitted sensing signal can be directly received by Rx antenna almost without path loss, which may significantly degrade the detection performance based on the relatively very low reflected sensing signal. This self-interference level needs to be minimized, and is very critical for sensing performance. Sensing function can request the level of self-interference by (pre-)configuring the required self-interference level for BS/TRP monostatic sensing. As BS/TRP may have different capability in terms of the self-interference cancellation, BS/TRP can report the self-interference level, which was measured at the Rx antenna side.

BS/TRP can report the information about the Tx/Rx parameters that are used for the sensing operation, such as follows:

- Sensing signal bandwidth,

- Type of the sensing signal waveform,

- Sensing signal Tx resource (set),

- Sensing signal Tx power,

- Time duration of the sensing signal,

- Time stamp associated with the reported measurement,

- Number of repetitions of the sensing signal,

- Interval between the sensing signals (or periodicity),

- Polarization of the sensing signal,

- Antenna configuration used for sensing signal Tx/Rx,

- Self-interference level between Tx and Rx antenna,

- Self-interference canceling capacity,

- BS/TRP ID (e.g. cell ID), the location of BS/TRP,

- Target sensing area (TSA)

- Whether the detected entity is 3GPP entity or not,

- Tx/Rx beamwidth used for sensing signal Tx/Rx.

Proposal 11: Study what assistance information related to the measurement needs to be additionally reported.

  - Remaining issues

    - Performance Metric

With respect to key performance indicators, sensing service latency and refresh rate are defined as target requirements in TS 22.137. However, these aspects are primarily influenced by other working groups, rather than being directly addressed by RAN1. While CPI duration, which impacts velocity estimation performance, may be implementation-dependent like resource configuration, latency and refresh rate should not be considered as performance metrics for 5G-A ISAC; instead, CPI should be reported as previously agreed.

Proposal 12: Latency and refresh rate should not be considered as a performance metric for 5G-A ISAC.

    - Evaluation Assumption

One unresolved issue is the method for STX/SRX selection, as described below:

Proposal STX/SRX selection is to select one or more STX/SRX used to detect a specific target: - If a company uses STX/SRX selection in the evaluation, the detailed STX/SRX selection method should be reported - The details of the STX/SRX selection will be captured along with the corresponding evaluation results in the Rel-20 TR - If a company does not use STX/SRX selection in the evaluation, all STX/SRX are used to detect all targets - Companies should report what measurements are used when more than one STX/SRX is used to detect a target

To enable TRP selection for monostatic sensing, the sensing function requires a priori information about the Target Sensing Area (TSA). Based on this, it can select multiple TRPs, such as the N nearest TRPs, where N can be configurable. Given this, a rule for the final single TRP selection, e.g. based on distance or RSRP/RSSI can be applied. However, without TSA information, sensing would need to be performed across all TRPs, which is highly inefficient and impractical. There is no need to constrain TRP selection methods during performance evaluation, as this is one of the sensing approaches that can be determined based on company choices.

Proposal 13: TSA-related information should be considered as a prior information for STX/SRX selection, and company should report the specific STX/SRX selection method used.

  - Initial Evaluation Results

    - Simulation Assumptions

CPI analysis

For a given carrier frequency, the CPI is known to determine the (radial) velocity resolution

| ∆v=λ2TCPI | (1) |
| --- | --- |

where TCPI is the length of CPI.

As follows from equation (1), having the longer CPI is beneficial as the velocity resolution is improved. This observation is illustrated by in Table 1 presenting the velocity resolution vs. TCPI for the carrier frequencies of interest.

Table 1 Velocity resolution vs CPI length

| Fc [GHz] | TCPI [ms] | 5 | 10 | 20 | 40 | 100 | 200 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | Δv [km/h] | 27.0 | 13.5 | 6.7 | 3.4 | 1.35 | 0.68 |
| 4.9 |  | 22.0 | 11.0 | 5.5 | 2.76 | 1.1 | 0.55 |
| 28 |  | 1.07 | 0.54 | 0.27 | 0.13 | 0.05 | 0.03 |
| Travelled distance [m] |  | 0.25 | 0.5 | 1.0 | 2.0 | 5.0 | 10.0 |

It is straightforward to see that having TCPI = 200 ms provides significant benefit compared to TCPI = 5 ms, especially for the lower carrier frequencies. This may be of crucial importance for scenarios when a slow-moving target is to be detected on the background of strong scattering from a static object. Given the target range and that of the object are close to each other, the non-zero velocity may be the only feature that distinguishes the target from the background.

Such a case is depicted by Figure 1. The true target parameters are R = 120 m and v = 6 km/h (highlighted with the red circle). The static object is composed by the set of ‘bright’ points so that it spans the ranges from 100 to 118 m. The left and right parts of Figure 1 correspond to the OFDM waveform consisting of Nsym = 128 (TCPI = 4.6 ms) and Nsym = 512 (TCPI = 18.3 ms) OFDM symbols, accordingly. All the available subcarriers, i.e. Nsub = 3276 are used for sensing in both cases. It is easy to see that the longer CPI provides much more details in velocity domain.

It also should be highlighted that whereas both range and velocity resolutions are not considered as the metrics of sensing performance, they may affect such metrics as the missed detection and false alarm probabilities. In particular, the definitions of these metrics include the notion of “association”. One can expect if the resolution in not sufficient, two or more targets or environment objects can be perceived as a whole. In turn, this may affect the results of association of the detected objects with the true targets. Beyond that, the quality of resolution may affect the performance of sophisticated localization algorithms. Thus, it is preferable to get so high resolution as possible.

Figure 1. Target in the presence of strong scattering from a static object for TCPI = 4.6 ms (left) and TCPI = 18.3 ms (right)

One can conclude that in contrast to the bandwidth, the CPI has no strict limits and so has the velocity resolution. Nevertheless, OFDM-based sensing (like any other type of sensing) uses the fundamental assumption that the measurement time is short enough to one can neglect the target displacement during this time interval. As a result, in practice the length of CPI is always restricted.

The distance travelled by a target moving with the speed of 180 km/h in time interval TCPI is shown in Table 1. One can see in the case TCPI = 200 ms, the target displacement is 10 m. If the target range to be measured is 100 m, the target displacement is much less than the target range and can be neglected. However, according to the agreement, the minimum target range to be measured (minimum BS-target 3D distance) is 10 m. In such a case, the target displacement of 10 m is equal to the measured target range and cannot be neglected. To overcome this issue, one should pose a restriction on the travelled distance and, in turn, a restriction on the allowed CPI length. Based on the results presented in Table 1, to provide the travelled distance less than 2 m, one should have TCPI ≤ 40 ms.

Observation 5: The maximum target displacement during the CPI must be much less than the measured target range. Provided the minimum target range to be measures is equal to 20 m, the allowable target displacement is about 2 m. Thus, it is reasonable to restrict the considered length of CPI so that TCPI ≤ 40 ms.

For the performance evaluation, simulation assumptions are summarized in the table below:

Table 2. Simulation parameters

| Parameter | Value |
| --- | --- |
| Sensing mode | Monostatic TRP, continuous, one-shot measurement |
| Scenario | UMa-AV, 500m ISD |
| Carrier frequency / SCS | 4.9 GHz / 30kHz |
| System BW | 100 MHz |
| Total BS Tx power | 37 dBm |
| Antenna configurations | 1) [12,16,2,1,1], [2,16] ports for RX, [1,1] ports for TX 2) [1 1 2 1 1;1,1], 1 port for TX am RX |
| Antenna elements | Basic 8dBi sector antenna / special case - omni |
| Tx/Rx Isolation | 65 dB |
| Rx Noise (with residual SI power) | Modelled as Tx thermal noise leaked to Rx N0 = - 89 dBm/100 MHz (~5 dB over thermal) |
| BS noise figure | 5 dB |
| Background modeling | Fully static |
| Sensing waveform | OFDM, DL PRS, comb 2, 2 symbols per slot |
| Sensing resource ratio | Option1 = Option2 = 1/7 (≈14%) |
| CPI | 32 ms (64 slots, 128 observation points in time) |
| Target distributions: | 5 targets in the center-area |
| Target RCS | Model 1, Small UAV, A = - 12.81 dBsm, B2 = 3.74 |
| Min. distances | 10m TRP-Target, 10m Target-Target |
| Signal processing method | Cell-average CFAR with post-processing / 2D-FFT |
| Positioning mode | Multi-RTT on the TRP subsets |

For the initial assessment, one-shot measurement is considered, all the detection and positioning are done on the base of the single sensing signals sequence (32 ms). No trajectory tracking is assumed at this time.

To demonstrate the possibilities of the AI/ML ISAC processing, the beamforming aspect of the sensing is omitted and sensing data gathered from the single antenna element at each TRP. This will demonstrate potential of ISAC performance using the minimal hardware in older deployments. Targets data in the form of Range-Velocity pairs (and corresponding expected accuracies) for each detected point are reported to the centralized SF and used for target association and de-duplication via clustering.

Presented multi-TRP approach demonstrates the potential of ISAC performance using the minimal hardware in existing deployments.

    - Simulation Results

The following performance results were obtained for the simulation assumptions described above. It should be specially noted that the reasonable one-shot performance can be obtained even for single antenna, leaving the possibilities for ISAC technology application with the minimal hardware requirements.

Table 3. Initial simulation results for UMa-AV scenario

| Scenario | Single TRP measurement |  | Multi-TRP processing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Range error @90% [m] | Radial velocity error @90% [m/s] | Horizontal position error @90% [m] | Vertical position error @90 [m] | 3D velocity error @90%, [m/s] | PRD Right detection probability | Pfa1 False alarm probability (Type1) | Pfa2 False alarm probability (Type2) |
| UMa-AV 12 TRPs [12;16;1,1,1;2,16] | 0.803 | 0.823 | 0.091 | 0.228 | 0.127 | 86.2% | 0.0% | 2.8% |
| UMa-AV 12 TRPs [1,1,2,1,1; 1 1] | 0.827 | 0.796 | 0.119 | 0.349 | 0.167 | 80.0% | 0.0% | 2.0% |
| UMa-AV 7 omni TRP,no sectors [1,1,2,1,1; 1 1] | 0.84 | 0.82 | 0.34 | 1.01 | 0.87 | 77.0% | 0.0% | 2.1% |

Notes:

- False alarm probability type 1 appears to be zero, since target FA was set to 0.5e-6 in the CFAR algorithm and not a single FA type 1 case was observed.

- Here we present range and radial velocity estimation using a single TRP, and full 3D position and velocity estimation at the SF by integrating measurements from multiple TRPs.

- It can be seen that multi-TRP cooperation demonstrate great accuracy in the final target positioning

Figure 2. CDFs of the positioning and velocity estimation errors for UMa-AV ISD 500m case.

Observation 6: Cooperative sensing of multi-TRP enables accurate 3D position and velocity estimation by aggregating multiple measurements at the sensing function, which can be helpful for further target tracking and trajectory identification.

- Conclusions

In this contribution, the issues related to the ISAC channel modeling were discussed. The following observations and proposals were made as conclusions.

Observation 1: In the current NG-RAN positioning system, since there is no gNB-based positioning functionality defined, assigning position and velocity calculation tasks to the gNB for ISAC may introduce additional processing burden due to its functional limitations:

  - gNB only provides measurement results from radio signals,

  - LMF determines the positioning methods and calculates a single location estimate.

Observation 2: Generating sensing results at TRP may be insufficient for accurate position and velocity estimation, particularly in dynamic 5G-A ISAC environments.

Observation 3: SF can take responsibility for deriving final sensing results by leveraging contextual information and advanced processing capabilities to enhance accuracy.

observation 4. Ensuring accurate sensing results from TRP requires extensive auxiliary information from the SF, leading to increased data transmission and storage overhead.

Proposal 1. Prior to discussion on measurements, clarify capability and functionality of gNB and SF.

  - gNB: Responsible for generating and reporting intermediate measurable results.

  - SF: Responsible for deriving final sensing results, potentially by aggregating measurements from multiple gNBs.

Proposal 2. Clarify key definitions of ‘path’ and ‘point’:

  - Path: defined as a certain tap delay in delay profile,

  - Point: defined as a resolution grid within a 3D space composed of delay and angle dimensions,

Proposal 3. Support Level 4 Option 1 as a measurement level for 5G-A ISAC.

Proposal 4: Range, Doppler, and/or angle per point are reported as measurement results.

Proposal 5: Refine the definition of Rx-Tx time difference for TRP-target ranging to enable transmission and reception time measurements using the same time index, per path (point) and/or target.

Proposal 6: Doppler measurements can be obtained via FFT over a long-time scale and reported alongside the corresponding timestamp reflecting the reference time of estimation.

Proposal 7: Sensing-specific SINR can be measured as the ratio or difference between received power of target point(s) and non-target signals.

Proposal 8: The change between current and previous measurement values can be reported to support sensing/tracking moving object.

Proposal 9: The measurement quantization can be determined by target performance requirements, using legacy (e.g., format and report mapping table in TS 38.455 and 38.133) as a starting point.

Proposal 10: The range of the measurement values to be reported is (pre-)configured.

Proposal 11: Study what assistance information related to the measurement needs to be additionally reported.

Proposal 12: Latency and refresh rate should not be considered as a performance metric for 5G-A ISAC.

Proposal 13: For STX/SRX selection, TSA-related information should be considered as a prior information, while company should report the specific selection method used.

Observation 5: The maximum target displacement during the CPI must be much less than the measured target range. Provided the minimum target range to be measures is equal to 20 m, the allowable target displacement is about 2 m. Thus, it is reasonable to restrict the considered length of CPI so that TCPI ≤ 40 ms.

Observation 6: Cooperative sensing of multiple TRPs enables accurate 3D position and velocity estimation by aggregating multiple measurements at the sensing function, which can be helpful for further target tracking and trajectory identification.

- References

- NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN, 3GPP, TS 38.305

- 5G NR Physical layer measurements, 3GPP TS 38.215.

- NG-RAN; NR Positioning Protocol A (NRPPa), 3GPP TS 38.455

- NR Requirements for support of radio resource management, 3GPP TS 38.133