3GPP TSG RAN WG1 #124                                                                 R1-2600247

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	5

Source:	Spreadtrum, UNISOC

Title:	Discussion on LS on bandwidth aggregation

Document for:	Discussion and decision

Introduction

In the LS from RAN2 [1], RAN2 respectfully asks RAN1 to take it into account and make necessary update if any.

RAN2 has discussed the UE behaviour if it receives common DCI format 2_9 for SSB adaptation for a deactivated SCell and made the following agreements: Confirm that the UE can receive common DCI format 2_9 for SSB adaptation for a deactivated SCell but the UE shall ignore it for the deactivated SCell. Capture this clarification in stage 2 spec. Send LS to RAN1 (Cc: RAN4) to inform this RAN2 agreement..

In this contribution, we provide our views on this issue.

Discussion

According to the description of 38.321, when an SCell is deactivated, the physical layer resources of the cell (including PDSCH, PDCCH, PRACH, SSB, etc.) are all released.

…… 1> if the SCG associated with the activated SCell is deactivated: 2> deactivate the SCell according to the timing defined in TS 38.213 [6]; 2> stop the sCellDeactivationTimer associated with the SCell; 2> stop the bwp-InactivityTimer associated with the SCell; 2> deactivate any active BWP associated with the SCell; 2> clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively; 2> clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell; 2> suspend any configured uplink grant Type 1 associated with the SCell; 2> flush all HARQ buffers associated with the SCell; 2> cancel, if any, triggered consistent LBT failure for the SCell. ……

If UE receives common DCI format 2_9 for SSB adaptation for a deactivated SCell, the SSB adaptation indication for the deactivated SCell is meaningless based on the description of 38.321. In our view, RAN1 specifications do not require any clarification.

Observation 1: RAN1 specifications do not require any clarification on the UE behaviour if it receives common DCI format 2_9 for SSB adaptation for a deactivated SCell.

Conclusion

In this contribution, we have the following observation:

Observation 1: RAN1 specifications do not require any clarification on the UE behaviour if it receives common DCI format 2_9 for SSB adaptation for a deactivated SCell.

Reference

- R1-2600007, “LS on DCI 2_9 for SSB adaptation for deactivated SCells”, RAN2.