3GPP TSG RAN WG1 #124	R1-2600282

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	5

Source:	CATT

Title:	         Discussion on LS reply on Linear polarization orientation RRC signalling for NR NTN

Document for:	Discussion and Decision

Introduction

RAN1 has received a LS from RAN2 about the signaling indication for linear polarization [1]. Within this LS, RAN2 respectfully asked RAN4 and Cc to RAN1 to check the feedback from RAN4. The LS content is as follows:

1. Overall Description:

Some companies in RAN2 see the need to introduce in Rel-19 the support of signalling by the network to the UE for two orthogonal (horizontal / vertical) linear polarization orientations for NR NTN.

RAN2 would like to ask RAN4 whether the signalling of the linear polarization orientation by the network to the UE can be useful for NR NTN and to provide any related feedback.

2. Actions:

To RAN WG4:

ACTION: 	RAN2 would like to ask RAN4 whether the signalling of the linear polarization orientation by the network to the UE can be useful for NR NTN and to provide any related feedback.

In this contribution, we analyzed the feasibility on RRC signalling of linear polarization indication for NR NTN.

Discussion

If recalling the discussion in RAN1 R17 NTN, the RRC signaling for antenna polarization of satellites had been specified. In TS38.331, one signaling indication with three candidates is shown as follows:

CellsToAddModExt-v1710 ::=          SEQUENCE {

ntn-PolarizationDL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL,  -- Need R

ntn-PolarizationUL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL   -- Need R

}

With this signaling, DL and UL polarization information can be informed to UE. Regarding further linear polarization, horizontal polarization and vertical polarization have been utilized in TN network very popularly, and it seems no necessity to indicate the UE. For NTN channel, it doesn’t show any specialty to get advantages from the indication of horizontal or vertical polarization. Otherwise, this indication should be introduced in TN network in previous release, however, it didn’t.

In this context, we don’t see any benefit to have this signaling indication. Therefore, we suggest RAN2 to stop this signaling discussion.

Proposal 1: From RAN1 perspective, the RRC signaling on linear polarization indication for NR NTN is not necessary.

Conclusion

In this contribution, we analyzed the feasibility of signaling on linear polarization indication for NR NTN, and concluded with the following proposal:

Proposal 1: From RAN1 perspective, the RRC signaling on linear polarization indication for NR NTN is not necessary.

References

[1] R2-2509373, LS on Linear polarization orientation RRC signalling for NR NTN