3GPP TSG RAN WG1 #124                          	     				             R1-2600344

Gothenburg Metropolitan Area , SE, 9th – 13th February 2026

Source	: Tejas Networks

Title	: Views on NR coverage enhancements phase 3

Agenda Item	: 9.4.1

Document for	: Discussion and Decision

- Introduction

Additional improvements on NR coverage enhancements phase 3 have been discussed in the RAN plenary #107 [2],where several objectives had strong support. In this contribution we provide our views on the proposed objectives of the Rel-20 WI on the NR coverage enhancements Phase 3. In RAN WG1 meeting #123 the following agreements were made with respect to the coverage enhancement objectives :

The following agreement was reached regarding the PUSCH repetition scheduled by DCI format 0_0 with C-RNTI : Agreement @123 Support [request or capability report] of PUSCH repetition scheduled by DCI 0_0 with C-RNTI before receiving RRCReconfiguration in Msg3 Agreement @123 Support PUSCH repetitions scheduled by DCI format 0_0 with C-RNTI both before and after receiving RRCReconfiguration. - Note: Strive for a single mechanism for PUSCH repetition scheduled by DCI 0_0 with C-RNTI Agreement @123 Down-select one of the following options to indicate the number of repetitions of PUSCH scheduled by DCI 0_0 with C-RNTI both before and after receiving RRCReconfiguration - Option 1: Using at most 2 MSB of MCS field in DCI format 0_0 with CRC scrambled by C-RNTI - FFS: details - Option 2: Based on TDRA - FFS: details The following agreements were reached in last meeting on the extension of pi/2-BPSK to more MCS entries : Agreement @122bis For extension of pi/2-BPSK to more MCS entries, down-select one of the following Options: - Option 1：Pi/2-BPSK is extended to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than N = 0.8770. - gNB can configure the maximum MCS up to which pi/2 BPSK is applicable. - Details on configuration to be discussed further - FFS: UE signaling to the gNB for this configuration - FFS: whether/how to provide assistance information and what is the assistance information - Option 2：Support extending Pi/2-BPSK to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than X. - Where X the fixed value select from: - a) X = 0.8770 - b) X = 0.6016 - Extension is captured in the modified MCS table. - gNB enables/disables this feature via RRC signalling.

This document provides extended views on these aspects, aiming to highlight their implications for future system design and to build upon the ongoing discussions in both WG1 and Plenary meetings.

- Discussion

For the Rel-20 5G-Advanced scope, coverage enhancement is a key focus area driven by strong market demand and its strategic importance in network deployments. While notable progress has been achieved in uplink (UL) coverage especially through enhancements to physical channels such as PUSCH, PUCCH, and Msg3 identified in the Rel-17, further improvements are still required. Coverage remains a fundamental concern for operators, particularly in challenging scenarios like cell-edge performance, deep indoor penetration, and rural deployments.

Despite the gains made, several physical channels continue to exhibit limitations that hinder optimal coverage. These include control channels and initial access procedures, which are critical for reliable connectivity. Therefore, Rel-20 should aim to address these bottlenecks to ensure robust and uniform NR coverage across diverse deployment environments.

  - PUSCH repetition scheduled by DCI 0_0 with C-RNTI (Msg5 PUSCH repetition)

Until Rel-18, repetition-based coverage enhancements have been specified for several uplink channels including PRACH, Msg3 PUSCH, PUCCH. However, Msg5 PUSCH carrying the RRC message RRCSetupComplete, which occurs after Msg3 and before the reception of dedicated PUSCH configuration does not currently support repetition, making it a significant coverage bottleneck, especially in challenging scenarios such as NTN (Non-Terrestrial Networks).

From a specification standpoint, PUSCH is scheduled using DCI format 0_0 with CRC scrambled by C-RNTI (Msg5) and is included under the category of PUSCH scheduled by this format. Despite the large payload size of Msg5 (often exceeding 100 bytes, which is typically larger than Msg3), no repetition mechanism is defined, which severely limits its transmission reliability during initial access. Significant performance gap between Msg3 repetitions and Msg5 retransmissions was presented by many companies at RAN Plenary #107 [3].

Field tests have confirmed that Msg5 is a critical bottleneck due to:

- Large payload size.

- Lack of repetition support.

- Potential congestion when Msg3 and Msg5 retransmissions overlap.

To address this, study is required in Rel-20 to enable PUSCH repetition for transmissions scheduled by DCI format 0_0 with C-RNTI, specifically targeting Msg5 (before RRCReconfiguration) . This would significantly improve coverage and reliability during initial access. HARQ-based retransmissions are not always viable in such cases, particularly for low latency applications where retransmission delays are unacceptable.

- Request of Msg5 repetition :

The request for Msg3 repetition (initial transmission) is handled through PRACH resource partitioning, by allocating separate PRACH resources/occasions. However, dedicating additional PRACH resources solely for indicating repetitions of other PUSCH transmissions can be challenging.

To support repetitions for initial Msg5 transmissions, an additional indication will be necessary to signal the UE's capability for repetitions. We propose conveying this indication via Msg3-based signalling for UE requested Msg5 PUSCH repetition with gNB indicating the number of repetitions, using the following options:

- Option 1: Inherently assume repetitions for Msg5 initial transmission by default, whenever repetition for initial transmission of Msg3 is requested/indicated by the UE.

- Option 2: Indicate/request repetitions for Msg5 initial transmission via DM-RS configurations in Msg3. Following are the examples of DMRS configurations that can be altered to indicate/request repetition

  - DMRS sequence partitioning

Partitioning of the DMRS sequences to differentiate between UEs requesting for Msg5 repetition and UEs not requesting for Msg5 repetition. Upon detection of these reserved DMRS sequences in Msg3 initial transmission, the gNB can effectively distinguish between coverage-limited UEs and legacy UEs.

  - DMRS resource muting pattern

Resource muting/puncturing is applied to selected resource elements within the first DMRS symbol of the initial Msg3 transmission to signal a request for Msg5 repetition.

  - DMRS symbol position

The first DMRS symbol position can be configured to predefined symbol number to indicate repetition for initial transmission of Msg5.

Proposal 1: Study on the default assumption that the UE requests repetitions for the initial transmission of PUSCH (Msg5), scheduled via DCI format 0_0 with CRC scrambled by C-RNTI, whenever repetition was requested for Msg3.

Proposal 2: Study on the request mechanism for repetition of initial transmission of PUSCH (Msg5), scheduled via DCI format 0_0 with CRC scrambled by C-RNTI, using Msg3-based signalling via DM-RS configurations as per following options :

- DMRS sequence partitioning

- DMRS resource muting pattern

- DMRS symbol position

- Mechanisms of Msg5 repetition :

Once the repetition request is identified for initial transmission of Msg5, the UE may utilize the repetition information/parameters provided in the UL grant, originally intended for Msg3, to also configure Msg5 repetition. This reuse may include the number of repetitions, redundancy version (RV) for repetition, and frequency hopping configurations. The repetition parameters might also be configured through DCI for retransmissions of Msg5.

We prefer to support Option 1, as it allows reuse of a design similar to the existing Msg3 PUSCH repetition mechanism, thereby minimizing further discussion and alignment effort. Specifically, for Msg3 PUSCH repetition, during the initial transmission, the UE derives the repetition number from the MCS information field (4 bits) in the Msg2 PDCCH. In this process, (i) the two most significant bits (MSBs) select one repetition number from a set of four candidate values configured via SIB1, and (ii) the two least significant bits (LSBs) select one MCS index from a set of four candidate MCS indices configured via SIB1.

For retransmissions, the UE determines the repetition number based on the MCS information field (5 bits) in DCI 0_0, with the CRC scrambled by the TC‑RNTI, which schedules the Msg3 PUSCH repetition. In this case, (i) the two MSBs select one repetition number from the same set of four SIB1‑configured candidate values, and (ii) the three LSBs select one MCS index from a set of eight candidate MCS indices configured via SIB1. The first four indices of this eight‑MCS set are used for the initial PUSCH transmission.

Proposal 3: Enable the gNB to signal the Msg5 PUSCH repetition number to the UE by using up to the two most significant bits of the MCS field in DCI format 0_0 with CRC scrambled by the C‑RNTI.

- Indication of Msg5 repetition :

The Msg3 repetition count is indicated by the two most significant bits (MSB) of the MCS field in the RAR UL grant or in DCI format 0_0 with TC-RNTI. This value is selected from the set provided by numberOfMsg3-RepetitionsList, or from {1, 2, 3, 4} if the list is not available. Similarly, the number of Msg5 repetitions can be determined using the similar approach via DCI format 0_0 with CRC scrambled by C-RNTI.

Proposal 4: Study of dynamic indication of the number of repetitions for PUSCH (Msg5) retransmission via DCI format 0_0 with C-RNTI

  - Improvement of PUSCH coverage for high UL data rate

This objective aims to extend the MCS entries for pi/2-BPSK modulation in the MCS table defined in TS 38.214 for PUSCH with transform precoding enabled. Currently, MCS Table 6.1.4.1-1 'MCS index table for PUSCH with transform precoding and 64QAM' includes the first two entries that support π/2-BPSK (where modulation order q = 1 for pi/2-BPSK and q = 2 for QPSK). These entries correspond to code rates of 240/1024 and 314/1024, respectively.

The PAPR difference between pi/2-BPSK and QPSK is approximately 1–2 dB. This margin can be leveraged by extending the MCS entries for π/2-BPSK to enhance PUSCH coverage, particularly for high uplink data rates. A detailed study is required to assess the impact of such MCS extension on spectral efficiency at higher data rates.

- General considerations on extension of pi/2-BPSK to MCS entries :

When extending pi/2-BPSK to more MCS entries in MCS tables for higher code rate, the code rate should be selected accordingly to keep spectral efficiency the same. The Pi/2-BPSK MCS entries shall be extended to more entries in MCS tables with spectrum efficiency no larger than N, where N can be up to 0.6016 to avoid any performance degradation compared to QPSK.

The PAPR margin of pi/2 BPSK over QPSK, allows the power amplifier in the user equipment (UE) to operate more efficiently, enabling higher effective transmit power without introducing nonlinear distortion. This is particularly beneficial for PC3 devices, which are typically limited to a maximum transmit power of 23 dBm, but can temporarily exceed this limit under controlled conditions to improve link reliability. According to Table 6.2.2-1 (TS 38.101-1) , Maximum power reduction (MPR) for power class 3, the UE indicates support for UE capability powerBoosting-pi2BPSK, where the reference power is boosted by at most 1dB.

The maximum output power for a power class 3 UE with pi/2 BPSK modulation can be 25.8 dBm which is the reference power of 0 dB MPR (26dBm) minus 0.2 dB MPR. Since QPSK modulation does not involve power boosting, the maximum output power for a power class 3 UE remains at 23 dBm. In contrast, pi/2-BPSK can achieve an additional gain of 2.8 dB over QPSK.

Observation 1: Power class 3 UE can achieve up to 2.8 dB power gain for pi/2 BPSK over QPSK.

Proposal 5: Support extension of pi/2 BPSK for additional MCS indices with spectral efficiency no larger than 0.6016 for MCS table 6.1.4.1-1 of TS 38.214

- MCS entries which are supported for the extension of pi/2-BPSK :

As per the general considerations discussed it was agreed to maintain the code rate the in order to keep spectral efficiency the same. When extending pi/2-BPSK modulation scheme to additional MCS entries, the trade-off between increased code rate and power boosting gain should be carefully considered. Additionally, to improve coverage for UEs experiencing low signal strength, additional MCS entries with lower code rates can also be introduced.

The MCS entries for pi/2 BPSK of the Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM, can be further extended up to MCS index 4 as shown in Table 1. To improve coverage for UEs experiencing low signal strength, additional MCS entries with lower code rates can also be introduced as per example entry shown in Table 2. This allows the network to maintain ubiquitous connectivity and service quality even when the UE is operating at the edge of the cell or in challenging environments. Extending the table’s lower MCS entries for π/2-BPSK, enables the scheduler to apply stronger forward error correction without needing to switch to the Low SE table, thereby avoiding reconfiguration overhead and preserving reliable fallback behaviour in mixed-traffic environments.

The MCS entries are suggested to be extended for Table 6.1.4.1-1 (TS 38.214), for the following :

- Option 1: Enabling the support of pi/2 BPSK modulation for higher code rate entries for MCS index up to 4 (Table 1)

- Option 2 : Adding new MCS entries to support further low code rate entries with pi/2 BPSK to ensure connectivity in challenging environments, along with extension of pi/2 BPSK MCS index up to 4 (Table 2)

Table 1: Potential enhancement of MCS index table for PUSCH with transform precoding and 64QAM

| MCS index | Modulation order | Code rate (R×1024) | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | Q | 240/q | 0.2344 |
| 1 | Q | 314/q | 0.3066 |
| 2 | q_m | 386/ q_m | 0.3770 |
| 3 | q_m | 502/ q_m | 0.4902 |
| 4 | q_m | 616/ q_m | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| …<unchanged>… |  |  |  |

Table 2: Potential enhancement of MCS index table for PUSCH with transform precoding and 64QAM

| MCS index | Modulation order | Code rate (R×1024) | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 1 | 157 | 0.1533 |
| 1 | 1 | 193 | 0.1884 |
| 2 | Q | 240/q | 0.2344 |
| 3 | Q | 314/q | 0.3066 |
| 4 | q_m | 386/ q_m | 0.3770 |
| 5 | q_m | 502/ q_m | 0.4902 |
| 6 | q_m | 616/ q_m | 0.6016 |
| 7 | 2 | 379 | 0.7402 |
| …<unchanged>… |  |  |  |

Observation 2: Considering the power gain that can be achieved with pi/2 BPSK, it is beneficial to extend the MCS indices up to MCS index 4 for Table 6.1.4.1-1 (TS 38.214).

Observation 3: The extension of the MCS indices for pi/2 BPSK should also be considered for lower code rates as well to extend the application of pi/2 BPSK for coverage limited scenarios

Proposal 6: Support extension of pi/2 BPSK for additional MCS indices up to MCS index 4 as indicated in table below, maintaining the legacy spectral efficiency for MCS table 6.1.4.1-1 of TS 38.214

| MCS index | Modulation order | Code rate (R×1024) | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | Q | 240/q | 0.2344 |
| 1 | Q | 314/q | 0.3066 |
| 2 | q_m | 386/ q_m | 0.3770 |
| 3 | q_m | 502/ q_m | 0.4902 |
| 4 | q_m | 616/ q_m | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| …<unchanged>… |  |  |  |

Proposal 7: Discuss whether to support additional MCS entries of pi/2 BPSK at lower code rates (e.g. 193/1024 and below) to improve connectivity in coverage-limited environments and support low-data-rate applications .

| MCS index | Modulation order | Code rate (R×1024) | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 1 | 157 | 0.1533 |
| 1 | 1 | 193 | 0.1884 |
| 2 | Q | 240/q | 0.2344 |
| 3 | Q | 314/q | 0.3066 |
| 4 | q_m | 386/ q_m | 0.3770 |
| 5 | q_m | 502/ q_m | 0.4902 |
| 6 | q_m | 616/ q_m | 0.6016 |
| 7 | 2 | 379 | 0.7402 |
| …<unchanged>… |  |  |  |

- LLS methodology and results

For comparative analysis of extended higher MCS indices in the MCS index table, the target code rate must be increased to maintain the same spectral efficiency (SE). In this case as we are comparing the performance of pi/2 BPSK (q = 1) with QPSK (q = 2), the code rate of the existing MCS entries is doubled to ensure same SE.

While a higher code rate can negatively impact transmission performance due to reduced error protection, the inherent power boosting gain associated with pi/2-BPSK can help offset this degradation. Therefore, when deciding the extension of MCS entries with pi/2-BPSK, it is essential to carefully evaluate the trade-off between the increased code rate and the compensatory benefits of power boosting to ensure optimal link performance.

The following table compares the performance of pi/2-BPSK and QPSK, observing the SNR for which BLER not is exceeding 10 %. The evaluation is done with 4 receive antennas, 24 PRBs and 2 DMRS symbols per slot, with the rest of the simulation parameters are provided in Table 5 in Appendix section.

Table 3 : Performance evaluation of new MCS entries for pi/2 BPSK in comparison with QPSK (existing MCS entries)

| MCS index | Code rate (R×1024) | SNR required for QPSK (dB) | SNR required for pi/2 BPSK (dB) | SNR required for pi/2 BPSK with 2.6 dB gain (dB) | SNR required for pi/2 BPSK with 2.8 dB gain (dB) |
| --- | --- | --- | --- | --- | --- |
| 0 | 157 | - | -9.2 | -11.9 | -12.2 |
| 1 | 193 | - | -8.3 | -10.5 | -11.2 |
| 2 | 240/q | -8.2 | -7.7 | -10 | -10.5 |
| 3 | 314/q | -7.4 | -6.6 | -9.2 | -9.5 |
| 4 | 386/q | -6.4 | -5.7 | -8 | -8.5 |
| 5 | 502/q | -5.5 | -4.3 | -6.7 | -7.1 |
| 6 | 616/q | -4.4 | -3.2 | -5.6 | -5.9 |
| 7 | 758/q | -3.5 | -1.7 | -4.3 | -4.5 |
| 8 | 898/q | -2.5 | 0.8 | -1.7 | -2 |

The throughput gain due to extension of pi/2 BPSK MCS entries for higher code rates over QPSK (for pi/2 BPSK with 2.6 dB gain) is as shown in Figure 1.

Figure 1 Throughput gain in % of pi/2 BPSK (with 2.8dB gain) over QPSK for each MCS index in Proposed Table 2

The following table (Table 5) compares the performance of pi/2-BPSK and QPSK, observing the SNR for which BLER not is exceeding 1 %. The evaluation is done with 4 receive antennas, 24 PRBs and 2 DMRS symbols per slot, with the rest of the simulation parameters are provided Table 6 in Appendix section.

Table 4: Performance evaluation of new MCS entries for pi/2 BPSK in comparison with QPSK (existing MCS entries)

| MCS index | Code rate (R×1024) | SNR required for QPSK (dB) | SNR required for pi/2 BPSK (dB) | SNR required for pi/2 BPSK with 2.6 dB gain (dB) | SNR required for pi/2 BPSK with 2.8 dB gain (dB) |
| --- | --- | --- | --- | --- | --- |
| 0 | 157 | - | -8.2 | -11 | -11.2 |
| 1 | 193 | - | -7.6 | -10 | -10.3 |
| 2 | 240/q | -7.3 | -6.6 | -9.3 | -9.6 |
| 3 | 314/q | -6.3 | -5.6 | -8.2 | -8.6 |
| 4 | 386/q | -5.5 | -4.6 | -7.2 | -7.6 |
| 5 | 502/q | -4.3 | -3.2 | -6 | -6.3 |
| 6 | 616/q | -3.5 | -2.4 | -4.6 | -4.9 |
| 7 | 758/q | -2.6 | -0.9 | -3.1 | -3.5 |
| 8 | 898/q | -1.5 | 1.9 | -0.8 | -1 |

The observation here is that as the MCS index increases, the performance degradation of pi/2-BPSK compared to QPSK becomes more pronounced. Specifically, at MCS index 5 (as per Table 6.1.4.1-1), pi/2-BPSK exhibits a performance loss of ~ 1.7 – 1.9dB. This loss increases to around 3.4 dB when extended to MCS index 6 (as per Table 6.1.4.1-1).

Given an additional power‑boosting gain of about 2.6 dB for pi/2‑BPSK relative to QPSK, the performance improvement at MCS index 5 (per Table 6.1.4.1‑1) is marginal. At MCS index 6, pi/2‑BPSK shows a performance degradation of approximately 0.7–0.8 dB compared to QPSK. Therefore, extending the MCS entries only up to MCS index 4 in Table 1 is recommended, as the observed gain for higher MCS entries does not exceed 1 dB.

Similarly, with an additional power‑boosting gain of about 2.8 dB for pi/2‑BPSK over QPSK, the performance gain at MCS index 5 is still less than 1 dB, and at MCS index 6, pi/2‑BPSK experiences a performance loss of roughly 0.4–0.5 dB relative to QPSK. Hence, it is again suggested that the MCS entries be extended only up to MCS index 4 in Table 1, where the maximum gain observed is around 2 dB.

By introducing MCS entries with lower code rates, we can achieve data rates in the range of approximately 1 to 1.5 Mbps. This is well-suited for Low-power IoT devices or enhanced Machine-Type Communication (eMTC), which are designed for low data rate applications with extended coverage.

Observation 4: The extension of the MCS entries is only reasonable up to MCS index 4 (as per Table 6.1.4.1-1) due to the performance trade-off between gain due to additional power boost and degradation due to increased code rate.

Observation 5: The performance improvement for MCS index 5 (as per Table 6.1.4.1-1) is minimal, while MCS index 6 (as per Table 6.1.4.1-1) shows a performance degradation even with a power increase of up to 2.8 dB.

- Signaling design

From signalling aspect, support for pi/2-BPSK has been defined as an optional UE capability since 3GPP Release 15, controlled via the RRC parameter tp-pi2BPSK (TS 38.331). To enable configuration of extended MCS entries utilizing pi/2-BPSK modulation, a new RRC parameter (e.g., tp-pi2BPSK-R20) should be introduced only applicable for power class 3 UEs, indicating a new feature for Release 20 (Figure 2). However, a prerequisite for this configuration is that the gNB must first activate the tp-pi2BPSK feature, ensuring that the UE supports this modulation scheme before assigning the extended MCS entries.

Proposal 8: Supporting new RRC parameter indicating the extension of MCS entries based on conditionally based on existing RRC signalling not to impact the legacy UEs.

- Conclusion

In this contribution, we propose the following ideas to be considered for UL coverage enhancements

Proposal 1: Study on the default assumption that the UE requests repetitions for the initial transmission of PUSCH (Msg5), scheduled via DCI format 0_0 with CRC scrambled by C-RNTI, whenever repetition was requested for Msg3.

Proposal 2: Study on the request mechanism for repetition of initial transmission of PUSCH (Msg5), scheduled via DCI format 0_0 with CRC scrambled by C-RNTI, using Msg3-based signalling via DM-RS configurations as per following options :

Proposal 3: Enable the gNB to signal the Msg5 PUSCH repetition number to the UE by using up to the two most significant bits of the MCS field in DCI format 0_0 with CRC scrambled by the C‑RNTI.

Proposal 4: Study of dynamic indication of the number of repetitions for PUSCH (Msg5) retransmission via DCI format 0_0 with C-RNTI

Observation 1: Power class 3 UE can achieve up to 2.8 dB power gain for pi/2 BPSK over QPSK.

Proposal 5: Support extension of pi/2 BPSK for additional MCS indices with spectral efficiency no larger than 0.6016 for MCS table 6.1.4.1-1 of TS 38.214

Observation 2: Considering the power gain that can be achieved with pi/2 BPSK, it is beneficial to extend the MCS indices up to MCS index 4 for Table 6.1.4.1-1 (TS 38.214).

Observation 3: The extension of the MCS indices for pi/2 BPSK should also be considered for lower code rates as well to extend the application of pi/2 BPSK for coverage limited scenarios

Proposal 6: Support extension of pi/2 BPSK for additional MCS indices up to MCS index 4 as indicated in table below, maintaining the legacy spectral efficiency for MCS table 6.1.4.1-1 of TS 38.214

| MCS index | Modulation order | Code rate (R×1024) | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | Q | 240/q | 0.2344 |
| 1 | Q | 314/q | 0.3066 |
| 2 | q_m | 386/ q_m | 0.3770 |
| 3 | q_m | 502/ q_m | 0.4902 |
| 4 | q_m | 616/ q_m | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| …<unchanged>… |  |  |  |

Proposal 7: Discuss whether to support additional MCS entries of pi/2 BPSK at lower code rates (e.g. 193/1024 and below) to improve connectivity in coverage-limited environments and support low-data-rate applications .

| MCS index | Modulation order | Code rate (R×1024) | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 1 | 157 | 0.1533 |
| 1 | 1 | 193 | 0.1884 |
| 2 | Q | 240/q | 0.2344 |
| 3 | Q | 314/q | 0.3066 |
| 4 | q_m | 386/ q_m | 0.3770 |
| 5 | q_m | 502/ q_m | 0.4902 |
| 6 | q_m | 616/ q_m | 0.6016 |
| 7 | 2 | 379 | 0.7402 |
| …<unchanged>… |  |  |  |

Observation 4: The extension of the MCS entries is only reasonable up to MCS index 4 (as per Table 6.1.4.1-1) due to the performance trade-off between gain due to additional power boost and degradation due to increased code rate.

Observation 5: The performance improvement for MCS index 5 (as per Table 6.1.4.1-1) is minimal, while MCS index 6 (as per Table 6.1.4.1-1) shows a performance degradation even with a power increase of up to 2.8 dB.

Proposal 8: Supporting new RRC parameter indicating the extension of MCS entries based on conditionally based on existing RRC signalling not to impact the legacy UEs.

- References

[1] RP-251827, “NR coverage enhancements Phase 3- MODERATOR SUMMARY AND PROPOSAL”, RAN#108, June 2025.

[2] RP-250802, “Moderator's summary for RAN1 led REL-20 topics”, RAN1 Chair (Samsung), 3GPP TSG  RAN#107, March 2025.

[3] RP-250554, “Views on UL Coverage Enhancements for 5G-A in Rel-20”, ZTE Corporation, Sanechips, 3GPP TSG RAN #107, Incheon, Korea, March 12th – 14th, 2025.

- Appendix

Table 5: Parameters for UL link level simulations for performance evaluation for simulation results with 10% BLER

| Link level simulation parameters |  |
| --- | --- |
| Metric | UL BLER |
| Subcarrier spacing | 15 kHz |
| Channel model | TDL-B with 100 ns delay spread, 400 Hz Doppler |
| Frequency hopping | Disabled |
| Modulation | Transform precoding with fixed MCS |
| Channel estimation | Practical DMRS based |
| Receiver | MMSE |

Table 5: Parameters for UL link level simulations for performance evaluation for simulation results with 1% BLER

| Link level simulation parameters |  |
| --- | --- |
| Metric | UL BLER |
| Subcarrier spacing | 15 kHz |
| Channel model | TDL-C with 300 ns delay spread, 11 Hz Doppler |
| Frequency hopping | Disabled |
| Modulation | Transform precoding with fixed MCS |
| Channel estimation | Practical DMRS based |
| Receiver | MMSE |