3GPP TSG RAN WG1 #124			                            R1-2600380

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Source: 	CMCC

Title:	Discussion on other procedures

Agenda item:	9.3.2.3

Document for:	Discussion/Decision

- Introduction

RAN#108 has agreed R20 SI to study enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices. RAN1 related objectives are as followings[1],

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage.

In this contribution, we will discussion the necessary and feasible procedure changes to the Rel-19 air interface and related required information that is different from R19, in order to support traffic types DO-DTT, DT, DO-A in D4T1 scenario for Device 2b and Device C.

- Discussion on initial frequency/broadcast information acquisition

2.1 Reader ID differentiation

During last RAN1 meeting, the following agreements were made for initial reader identification[3].

Agreement Initial frequency acquisition procedure is necessary for Device 2b and for Device C, at least for flexible deployment of A-IoT frequency location(s) for outdoor scenario. Agreement Reader identification/differentiation is necessary for Device 2b and for Device C, at least in outdoor scenario to differentiate R2D among different readers. Agreement For reader identification/differentiation at least in outdoor scenario to differentiate R2D among different readers, a device acquires the reader ID-related information by at least one of the following options: - Option 1: Reader ID-related information is contained in the broadcast information. - Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s). - Option 3: Reader ID-related information is carried by L1 R2D control information. - Option 4: Reader ID-related information is contained in the higher-layer information in the paging-like message.

It has been agreed that Reader identification/differentiation is necessary for outdoor scenario. On one hand, the number of Reader IDs should be able to differentiate the Readers each device sees, on the other hand, the number Reader IDs plays a important role on network-planning complexity. Both have to be considered. Reader ID quantities used in NB-IoT (e.g., 504) or NR can be starting point.

Proposal 1: The number of Reader IDs should consider Reader differentiation and network-planning complexity.
Among the options to convey Reader ID-related information, we think it should be identified by the device as early as possible, so that device can realize Reader selection/reselection without reading more information. So we think option 1, option 2 and combination of option 1 and option 2 is better.

- Option 1: Reader ID-related information is contained in the broadcast information.
This relaxes synchronization signal detection complexity, but since the device cannot identify the cell before decoding the channel for broadcast, cell-specific parameters can not be applied to physical channel carrying broadcast information, so inter-Reader interference randomization can not be used.

- Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s).
This is similar as NR. But detecting different sequences during initial cell search increases device energy consumption. So the number of IDs should be limited for this option.

- Combination of Option 1 and Option 2.
This option offers a trade-off between synchronization signal detection complexity and subsequent broadcast decoding performance.

- Finial determination may depends on the number of Reader IDs and sequence design of synchronization signal. If only a few IDs can be carried by sync signal sequence(s), the remaining can be contained in broadcast information.

Proposal 2: For the options by which a device acquires the reader ID-related information, the following is better for early identification,

- Option 1: Reader ID-related information is contained in the broadcast information.

- Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s).

- Combination of Option 1 and Option 2.

- 2.2 Broadcast information

During last RAN1 meeting, the following agreements about broadcast information were agreed[3],

Agreement - The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception - On how A-IoT devices acquire the broadcast information, study the following two cases: - Case B1: Single-step acquisition of the broadcast information - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information) - On whether to use PRDCH to carry the broadcast information, - For Case B1, PRDCH is used to carry the broadcast information. - For Case B2 - PRDCH is used to carry the SIB1-like broadcast information. - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information

In this section, we first provide our consideration on what information to be broadcast and how to transmit broadcast information.

The following broadcast information needs to be considered for subsequent transmission and reception.

- Timing information

- According to agreement in evaluation agenda reached at last RAN1 meeting, the calibrated SFO for Device 2b can be 100~1000 ppm and for Device C can be 10 ppm. Assuming 10~100 ppm SFO after calibration, the timing error is only 1~10 µs for a time duration of 100 ms. Hence it is feasible for Device 2b/C to maintain time synchronization. Therefore the broadcast information should include timing information that help devices determining frame number/slot number, based on which devices can determine the RACH occasions.

- Resource allocation for the first D2R transmission for DOA

- According to the agreements[2], there are two options for resource allocation for the first D2R message of DO-A. In our understanding, supporting DOA means devices can trigger D2R reporting autonomously, which means periodic random access resources should be provided, i.e. option 1. The random access resource configuration can follow a method similar to that used in NR, i.e. the configuration indicates the resource period, time offset, and which slots within each period are available for access, so that devices can derive the resources to transmit Msg1. And when FDMA is supported for D2R Msg1 and Msg3, the candidate frequency resources to support FDM transmission can also be provided by broadcast information.

Agreement For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility: - Option 1: Periodic D2R resource allocation - Option 2: Aperiodic D2R resource allocation

For option 2 aperiodic D2R resource allocation, the intention is to assign random access resources on demand. However, periodic scheduling request resources before the aperiodic D2R resource allocation are still required, and how the Reader decides the amount of access resources to allocate after receiving the device’s SR remains to be solved, increasing standard complexity. On the other hand, when device has DOA traffic to send, it would first send an SR, then receive the aperiodic resource configuration, and finally go through the random access procedure, causing a lot of power consumption for an active device and possibly leaving insufficient energy for the subsequent data reporting. Therefore we recommend using Option 1.

Observation 1：Option 2: Aperiodic D2R resource allocation for the first D2R transmission for DO-A requires more standard effort and consumes more energy before device can send DOA traffic.

Proposal 3: For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, support Option 1: Periodic D2R resource allocation.

Proposal 4: The random access configuration includes periodicity, time offset, and available resource within each period, candidate frequency resources for FDM transmission if supported.

Another issue to be discussed is how to provide the necessary information for first D2R transmission, and four options are agreed to study in last meeting[3].

Agreement For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, following options are considered feasible. - Option 1: necessary information provided by broadcast information - Option 2: necessary information provided by a paging-like R2D message - Option 3: necessary information derived from a synchronization signal and provided by broadcast information For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study following option considering feasibility and applicability - Option 4: necessary information provided by device-specific R2D transmission

For Option 1, as long as device acquires cell synchronization and reads broadcast information, it can get the access resource allocation and initiate random access any time it has D2R traffic to report, this option is simple and more straightforward.

For Option 2, to our understanding, the “paging-like” message is sent only after the device has transmitted a scheduling request. As above analysis, this is unfriendly to energy-constrained Device-2b/C types. Compared with transmitting Msg1 directly with periodic resource allocation, they must first send an SR and then wait for the paging message, then send Msg1. Additional SR and paging monitoring may leave them without enough energy for the subsequent data transmission.

For Option 3, we note that the synchronization signal itself is not able to carry the resource allocation configuration,  the configuration still has to be provided by the broadcast information. Therefore Option 3 is essentially covered by Option 1, while any association between the configuration and the synchronization signal is a matter for more detailed design.

For option 4, the device has to finish RACH procedure, then be provided the necessary information for later DOA data transmission for Device 2b and for Device C, we think this is not the first D2R transmission for DOA.

Observation 2: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C,

- “Option2: necessary information provided by a paging-like R2D message” means more energy consumption before DOA traffic report than option 1, and has a risk of no enough energy for traffic report;

- Option 3 is still based on option 1, and details can be further studied.

- Option 4 is based on RACH procedure ahead, and is not the first D2R transmission for DOA.

- Proposal 5: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study Option 1: necessary information provided by broadcast information.

- Paging-message configuration

- Unlike device 1/2a, device 2b/C are expected to work outdoors under macro-coverage, so the received signal level will be lower. Then R19 SIP-based detection can no longer applicable for start indication of R2D channel, which means detecting arriving of R2D channel will consume more energy. So it will be impractical for device to continuously monitor possible paging message due to its limited stored energy. DRX cycle for paging monitoring should be studied that allows the device to wake up only at paging occasions. The broadcast information shall therefore provide the DRX parameters for paging (e.g. cycle, offset) so that the device can decide exactly when to detect paging.
Observation 3: The R19 SIP-based paging detection mechanism is no longer applicable for inventory use case and a DRX-based solution is required.

- Configurations of cell/reader-common L1 R2D control information

- According to discussions in previous meeting, in Rel-20 A-IoT system, there exists cases that a physical channel carriers L1 R2D control information only, and hence, a more efficient solution is to carry L1 R2D control information in an independent PRDCH. In this sense, configurations of cell/reader-common L1 R2D control information should be provided by the broadcast information.

- Cell-identity information

- Following discussion in section 2.1, the broadcast message may additionally carry part of the cell ID.

Proposal 6：Study the following information to be included in broadcast information,

- Timing information, e.g. SFN-like info.

- Periodic random access resource configuration, e.g. periodicity, offset, available resource in each periodicity, and frequency resources for FDM transmission.

- DRX configuration for paging monitoring.

- Configurations of cell/reader-common L1 R2D control information.

- Cell ID related information

- 2.3 How to transmit broadcast information

- During last RAN1 meeting, the following agreements on how to transmit broadcast information was agreed[3].

Agreement - The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception - On how A-IoT devices acquire the broadcast information, study the following two cases: - Case B1: Single-step acquisition of the broadcast information - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information) - On whether to use PRDCH to carry the broadcast information, - For Case B1, PRDCH is used to carry the broadcast information. - For Case B2 - PRDCH is used to carry the SIB1-like broadcast information. - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information

In NR, MIB and RMSI (i.e., SIB1) are carried on separate channels. According to discussion above, the payload to be carried in the broadcast information is expected to be large, to reduce the overhead and improve the efficiency, it is more reasonable to split the broadcast information into two parts, i.e., Case B2 in above agreement, MIB-like and then SIB1-like broadcast information. For the MIB-like broadcast information, it can carry timing information, cell ID related information, and also necessary information for receiving SIB1-like part. The other necessary information can then be carried in the SIB1-like information.

On whether to use PRDCH to carry the broadcast information, for case B2, both a separate PRDCH or a new physical channel dedicated to MIB-like are fine to carry the MIB-like broadcast information. For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetion number can be predefined. While for SIB1-like part, the PRDCH parameters can be scheduled by L1 control information.

Observation 4: The payload to be carried in the broadcast information is expected to be large, it is more reasonable to split the broadcast information into two parts, i.e., a MIB-like part and a SIB1-like part.

Proposal 7：Support Case B2: Two-step acquisition of the broadcast information, and MIB-like broadcast information can be carried by a separate PRDCH or a new physical channel dedicated to MIB-like information.

- Discussion on random access

In this section, we discuss the possible enhancement for active devices to support DOA service and inventory/command procedure.

3.1 Support of DOA

Random access procedure

In R19 the random-access procedure assumes that Msg1 already carries a 16-bit Random ID and that Msg2 resolves any collision by echoing this ID. In other words Msg1 simultaneously announces “a device wants access” and “which device wants access”. The 16-bit Random ID is sufficient for passive device under indoor coverage, and collision probability is low.

For R20 active devices, however, the target is outdoor macro-coverage with a 500m cell radius. Using density of 20 devices / 100 m² in TR38.848, a single cell may serve about 130000 devices. With a 1RB system design, a 15 minutes DOA reporting interval, and 10 % of the D2R resources reserved for Msg1, the R19 preamble + Random-ID + CRC format yields a collision probability above 70 %. If we adopt an NR-like longer Random ID (e.g. 39 bits) to improve contention resolution, Msg1 becomes longer and the collision probability rises to more than 90 % (detailed evaluation is provided in our companion contribution [4]). Hence the R19 mechanism with contention resolution by Msg1 + Msg2 to is no longer viable.

Observation 5: Reusing the R19 Msg1+Msg2 collision resolution procedure for outdoor active device random access leads to high collision probability and excessive D2R resource consumption.

So for R20, the random access procedure should be updated,

- Introduce a preamble based Msg1 that simply informs the reader “some device requests access”, and no ID is carried. This can shorten the Msg1 length and reduce the collision probability.

- The device identity is carried in Msg3.

- Contention is resolved by Msg4.

- Additionally, supporting CDM for Msg1 further lowers the preamble collision probability.

Proposal 8: Study for R20 a random access procedure update, including 1)sequence-based Msg1 is used and carries no Random ID, 2) contention is resolved via Msg3 and Msg4.

Proposal 9: Study CDM for Msg1 to reduce random access collision.

First D2R transmission for DOA

During last RAN1 meeting, the following agreement was agreed, and here we provide our views on this aspects[3].

Agreement For the 3 options for the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following aspects: - Recommended scenarios (event-triggered or periodic) - Subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A - Other aspects

The analysis is provided in the following table.

Table.1. Comparison of three first D2R transmission options

| First D2R transmission for DO-A | Recommended scenarios (event-triggered or periodic) | Subsequent procedures after the first D2R transmission | Other aspects |
| --- | --- | --- | --- |
| Option 1 Msg1-like content | Periodic/Event-triggered | Random access procedure, e.g.Msg.2/3/4, and DOA data transmission | Pre-step: Periodic random access resource is configured by broadcast information This option is anyway needed either for option3 after paging message or for option2 before periodic D2R resource is configured |
| Option 2 DO-A data payload | Periodic | Possible feedback and re-transmission | Pre-step: Periodic D2R resource is configured to device. If device common D2R resource is configured, collision may happen for DOA reporting; If device dedicated D2R resource is configured, the resource overhead is large. For this case, the device has to keep a connection state to maintain D2R synchronization, which means high power consumption for long interval, e.g. 5~15 minutes traffic interval in TR22.840. |
| Option 3 DO-A scheduling request | Periodic/Event-triggered | Paging-like R2D transmission to allocate random access resource, random access procedure,e.g.Msg.2/3/4, and DOA data transmission | Pre-step: Periodic scheduling request resource is configured by broadcast information If SR resource is device common, it is hard for Reader to determine how many RO resource to be allocated by paging. If the scheduling request rate is high for devices, following paging overhead will be large, the overhead reduction benefit compared to option1 will be reduced. The delay is larger than option1. |

Based on analysis on above table, we think option 1 is more efficient, since,

- It is part of the procedure of option3, and a pre-procedure for option2, which means it has to be supported;

- The overhead of ROs can be adjust by Reader based on traffic load and number of devices. The collision handling is simple.

- It applies to both periodic and event-triggered service. And for high event-triggered rate, the overhead can be similar or smaller than option 3, which require frequent paging transmission. For low event-triggered rate, the RO resources can be sparse to reduce overhead.

- Observation 6: Option 1:Msg1-like content as the first D2R transmission for DO-A for Device 2b and for Device C, is more efficient, due to the following aspects,

- It is part of the procedure of option3, and a pre-procedure for option2, which means it has to be supported;

- The overhead of ROs can be adjust by Reader based on traffic load and number of devices. The collision handling is simple.

- It applies to both periodic and event-triggered service. And for high event-triggered rate, the overhead can be similar or smaller than option 3, which require frequent paging transmission. For low event-triggered rate, the RO resources can be sparse to reduce overhead.

- Proposal 10: Support Option 1:Msg1-like content as the first D2R transmission for DO-A for Device 2b and for Device C

- Proposal 11: Comparison of three first D2R transmission options is in the Table 1.

Table.1. Comparison of three first D2R transmission options

| First D2R transmission for DO-A | Recommended scenarios (event-triggered or periodic) | Subsequent procedures after the first D2R transmission | Other aspects |
| --- | --- | --- | --- |
| Option 1 Msg1-like content | Periodic/Event-triggered | Random access procedure, e.g.Msg.2/3/4, and DOA data transmission | Pre-step: Periodic random access resource is configured by broadcast information This option is anyway needed either for option3 after paging message or for option2 before periodic D2R resource is configured |
| Option 2 DO-A data payload | Periodic | Possible feedback and re-transmission | Pre-step: Periodic D2R resource is configured to device. If device common D2R resource is configured, collision may happen for DOA reporting; If device dedicated D2R resource is configured, the resource overhead is large. For this case, the device has to keep a connection state to maintain D2R synchronization, which means high power consumption for long interval, e.g. 5~15 minutes traffic interval in TR22.840. |
| Option 3 DO-A scheduling request | Periodic/Event-triggered | Paging-like R2D transmission to allocate random access resource, and random access procedure,e.g.Msg.2/3/4, and DOA data transmission | Pre-step: Periodic random access resource is configured by broadcast information If SR resource is device common, it is hard for Reader to determine how many RO resource to be allocated by paging. If the scheduling request rate is high for devices, following paging overhead will be large, the overhead reduction benefit compared to option1 will be reduced. The delay is larger than option1. |

3.2 Support of inventory and command

In addition, Devices 2b/C also continue to support inventory and command scenarios that have been developed for device 1. For active devices, two design alternatives are under consideration:

- Alt.1: Reuse the Release 19 inventory procedure with minimal changes, but replacing the continuous-wave (CW) with an actively generated carrier in the D2R link. Some modification are needed for active devices.

- Alt.2: Design new inventory procedure for active devices. For example, by eliminating the access-trigger message and instead relying on the pre-configured random-access occasions for Msg1 transmission. This option , being a new design, can ensure compatibility with the DOA design .

Proposal 12: For active device to support inventory, the following two alternatives need to be studied,

- Alt 1: Reuse Release 19 inventory procedure modification

- Alt 2: Design new inventory procedure for active devices

For Alt1, the following Figure.1 shows the random-access flow for R19. Enhancements for active device can be investigated at the four marked points ①–④.

Figure.1 R19 Random access procedure

① Msg1 transmission

Thanks to better clock accuracy, the time gap between adjacent Msg1 occasions caused by SFO is now only ~0.01 % level of the Msg1 duration (100 ppm). This gap is far smaller than the Msg1 duration, so the value of X1 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message) can be increased to reduce Access trigger commands and reduce inventory time. Then Toffset for the Msg1 occasions corresponding to X1>1 should be defined.

② Msg2 transmission

Currently in R19, one Msg2 can at most response to 8 devices, since no TDM is supported for Msg3. And when TDM is introduced for Msg3 due to smaller SFO, whether to increase this number can be studied. Scheduling information in Msg2 need to update for active D2R frequencies other than smaller frequency shift factor.

③ Msg3 transmission

For the same reason as supporting larger X1 value for Msg1, TDM of Msg3 is possible, i.e. X3 > 1 (X3 = number of time-domain resources for Msg3 indicated in the corresponding R2D message). When X3 > 1, there will be Msg3 occasions that do not immediately follow Msg2, so Msg2 has to carry an explicit time offset indication for Msg3, or a Toffset is defined similar to Toffset2 used for X = 2 Msg1 occasions.

④ NACK feedback message

R19 only support indication of NACK for Msg3, while Msg3 re-transmission is not supported. For active devices, we think for both DOA and DT/DO-DTT traffic, support of Msg3 restransmission can improve efficiency.

Proposal 13: If R19 random access procedure is reused for R20 active devices, the following modification can be studied,

- Support X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message), and the related Toffset values for occasions with X1>2.

- Increase the maximum echo random IDs in Msg2, and the frequency resource indication in Msg2 for Msg3 scheduling needs update.

- Support X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message), and the related Toffset values for occasions with X3>1

- Support Msg3 retransmission.

It can be seen that if larger TDMA occasions is supported for Msg1 and Msg3, there will be more modifications to be specify. And since Msg1 and Msg3 transmission are based on Toffset values, it is hard for Reader to fully avoid overlap between Msg1/Msg3 transmission of inventory procedure and Msg1/Msg3 transmission of DOA services. Handling of this is needed.

For Alt 2, after receiving a paging message the device picks an access occasion according to the configured random access resources, transmits Msg1 on the chosen occasion, then it monitors Msg2, and derives the time-frequency location of Msg3 from the scheduling information in Msg2. The timing can be based on the R2D indication. Different from Alt.1, Msg3 transmission can base on scheduling info in Msg2, the overlap with Msg3 of DOA services can be avoided.

Observation 7: If random access procedure of active devices is based on R19 random access procedure, the handling of time collision for Msg1/Msg3 between inventory service and autonomous DOA traffic is complex

Observation 8: If new inventory procedure for active devices is designed, the R2D and D2R timing can base on R2D control indication, overlapping of Msg1/Msg3 between inventory service and autonomous DOA traffic can be avoided based on Reader indication.

3.3 General change for both DOA and DT/DO-DTT traffic

- Msg2 monitoring

For Msg2 monitoring, R19 supports Msg2 monitoring only after all X Msg1s, as shown in Figure.2 left side, since RAN4 agreed that CW transmission and A-loT BS downlink data transmission are non-concurrent. But for active devices, no CW exists since active devices can generate carrier frequency themselves. Therefore, reader can transmit and receive simultaneously. As shown in following Figure, with simultaneous Msg2 reception and Msg1 reception at Reader side, Both Msg2 and Msg3 can be scheduled earlier, shortening the overall inventory duration.

Figure.2 Msg2 monitoring after Msg1

And since the peak power consumption is higher for device 2b and device C, it is more critical for them to save power, so Msg2 monitoring window can be introduce. As soon as the switch time is satisfied after device transmits Msg1, device can start monitoring Msg2, or the starting time should be aligned with slot boundary if R2D slot alignment is required, if no Msg2 is detected during the window, it stops monitoring Msg2.

Proposal 14: For Device 2b/C design, Reader can transmit and receive simultaneously in FDD band, for example, detecting Msg1 and transmitting Msg2 at the same time.

Proposal 15: Monitoring window is needed for Msg2 monitoring to save device energy consumption.

- Msg3 retransmission

Not only for DT and DO-DTT traffic as we discussed above, but also for DOA traffic, we need to decide whether to introduce re-transmission for Msg3, which would improve the single-round inventory success rate and reduce total inventory delay, for example, a physical control channel such as PRDCCH can be used to schedule such Msg3 restransimision.

Proposal 16: Study whether to support Msg3 restransmission for both DOA and DT/DO-DTT traffic of device 2b/C, to improve efficiency.

- Timing between R2D transmission and D2R transmission

For the time offset during random access between a R2D transmission and following D2R transmission, we think it is more simple and flexible to indicate the time offset by R2D, since the device can maintain synchronization with smaller SFO. And according to above analysis, it can make the collision Msg1/Msg3 transmission easier between inventory service and autonomous DOA traffic.

Proposal 17: For R20 timing design between R2D transmission and following D2R transmission, support time offset indication by R2D message.

- Conclusion

In this contribution, we discuss the necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios and the following observation and proposals are made:

Proposal 1: The number of Reader IDs should consider Reader differentiation and network-planning complexity.

Proposal 2: For the options by which a device acquires the reader ID-related information, the following is better for early identification,

- Option 1: Reader ID-related information is contained in the broadcast information.

- Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s).

- Combination of Option 1 and Option 2.

Observation 1：Option 2: Aperiodic D2R resource allocation for the first D2R transmission for DO-A requires more standard effort and consumes more energy before device can send DOA traffic.

Proposal 3: For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, support Option 1: Periodic D2R resource allocation.

Proposal 4: The random access configuration includes periodicity, time offset, and available resource within each period, candidate frequency resources for FDM transmission if supported.

Observation 2: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C,

- “Option2: necessary information provided by a paging-like R2D message” means more energy consumption before DOA traffic report than option 1, and has a risk of no enough energy for traffic report;

- Option 3 is still based on option 1, and details can be further studied.

- Option 4 is based on RACH procedure ahead, and is not the first D2R transmission for DOA.

Proposal 5: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study Option 1: necessary information provided by broadcast information.

Observation 3: The R19 SIP-based paging detection mechanism is no longer applicable for inventory use case and a DRX-based solution is required.

Proposal 6：Study the following information to be included in broadcast information,

- Timing information, e.g. SFN-like info.

- Periodic random access resource configuration, e.g. periodicity, offset, available resource in each periodicity, and frequency resources for FDM transmission.

- DRX configuration for paging monitoring.

- Configurations of cell/reader-common L1 R2D control information.

- Cell ID related information

Observation 4: The payload to be carried in the broadcast information is expected to be large, it is more reasonable to split the broadcast information into two parts, i.e., a MIB-like part and a SIB1-like part.

Proposal 7：Support Case B2: Two-step acquisition of the broadcast information, and MIB-like broadcast information can be carried by a separate PRDCH or a new physical channel dedicated to MIB-like information.

Observation 5: Reusing the R19 Msg1+Msg2 collision resolution procedure for outdoor active device random access leads to high collision probability and excessive D2R resource consumption.

Proposal 8: Study for R20 a random access procedure update, including 1)sequence-based Msg1 is used and carries no Random ID, 2) contention is resolved via Msg3 and Msg4.

Proposal 9: Study CDM for Msg1 to reduce random access collision.

Observation 6: Option 1:Msg1-like content as the first D2R transmission for DO-A for Device 2b and for Device C, is more efficient, due to the following aspects,

- It is part of the procedure of option3, and a pre-procedure for option2, which means it has to be supported;

- The overhead of ROs can be adjust by Reader based on traffic load and number of devices. The collision handling is simple.

- It applies to both periodic and event-triggered service. And for high event-triggered rate, the overhead can be similar or smaller than option 3, which require frequent paging transmission. For low event-triggered rate, the RO resources can be sparse to reduce overhead.

Proposal 10: Support Option 1:Msg1-like content as the first D2R transmission for DO-A for Device 2b and for Device C

Proposal 11: Comparison of three first D2R transmission options is in the Table 1.

Table.1. Comparison of three first D2R transmission options

| First D2R transmission for DO-A | Recommended scenarios (event-triggered or periodic) | Subsequent procedures after the first D2R transmission | Other aspects |
| --- | --- | --- | --- |
| Option 1 Msg1-like content | Periodic/Event-triggered | Random access procedure, e.g.Msg.2/3/4, and DOA data transmission | Pre-step: Periodic random access resource is configured by broadcast information This option is anyway needed either for option3 after paging message or for option2 before periodic D2R resource is configured |
| Option 2 DO-A data payload | Periodic | Possible feedback and re-transmission | Pre-step: Periodic D2R resource is configured to device. If device common D2R resource is configured, collision may happen for DOA reporting; If device dedicated D2R resource is configured, the resource overhead is large. For this case, the device has to keep a connection state to maintain D2R synchronization, which means high power consumption for long interval, e.g. 5~15 minutes traffic interval in TR22.840. |
| Option 3 DO-A scheduling request | Periodic/Event-triggered | Paging-like R2D transmission to allocate random access resource, and random access procedure,e.g.Msg.2/3/4, and DOA data transmission | Pre-step: Periodic random access resource is configured by broadcast information If SR resource is device common, it is hard for Reader to determine how many RO resource to be allocated by paging. If the scheduling request rate is high for devices, following paging overhead will be large, the overhead reduction benefit compared to option1 will be reduced. The delay is larger than option1. |

Proposal 12: For active device to support inventory, the following two alternatives need to be studied,

- Alt 1: Reuse Release 19 inventory procedure modification

- Alt 2: Design new inventory procedure for active devices

Proposal 13: If R19 random access procedure is reused for R20 active devices, the following modification can be studied,

- Support X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message), and the related Toffset values for occasions with X1>2.

- Increase the maximum echo random IDs in Msg2, and the frequency resource indication in Msg2 for Msg3 scheduling needs update.

- Support X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message), and the related Toffset values for occasions with X3>1

- Support Msg3 retransmission.

Observation 7: If random access procedure of active devices is based on R19 random access procedure, the handling of time collision for Msg1/Msg3 between inventory service and autonomous DOA traffic is complex

Observation 8: If new inventory procedure for active devices is designed, the R2D and D2R timing can base on R2D control indication, overlapping of Msg1/Msg3 between inventory service and autonomous DOA traffic can be avoided based on Reader indication.

Proposal 14: For Device 2b/C design, Reader can transmit and receive simultaneously in FDD band, for example, detecting Msg1 and transmitting Msg2 at the same time.

Proposal 15: Monitoring window is needed for Msg2 monitoring to save device energy consumption.

Proposal 16: Study whether to support Msg3 restransmission for both DOA and DT/DO-DTT traffic of device 2b/C, to improve efficiency.

Proposal 17: For R20 timing design between R2D transmission and following D2R transmission, support time offset indication by R2D message.

- Reference

- RP-251884,New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices.

- Minutes_report_RAN1#122bis_v104.

- Chair notes RAN1#123-eom3.

- R1-2600379, Discussion on D2R signals, channels, waveform and procedures.