3GPP TSG RAN WG1 #123                                                     R1-2600484

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Source:	vivo

Title:	Discussion on enhancing DL CSI acquisition

Agenda Item:	9.2.2

Document for:	Discussion and Decision

Introduction

In RAN #108 meeting, the following Rel-20 WIDs have been approved for enhancing DL CSI acquisition, including early SRS/CSI/CSI-RS triggering and CSI-RS density reduction.

On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

In this contribution, further details on early SRS/CSI/CSI-RS triggering are discussed, including the configuration of CSI resource and reporting, the mechanism of UE capability reporting based on Msg3, the detailed design of Msg4 for triggering early SRS/CSI/CSI-RS, PUSCH resource for CSI reporting and QCL/TA/PC assumption for SRS and CSI reporting. Besides, details on CSI-RS density reduction are also discussed, including issues on subband sizes for new densities.

Early SRS/CSI/CSI-RS triggering

Transition from idle/inactive mode to connected mode

General procedure for early CSI acquisition

For early SRS/CSI-RS/TRS/CSI triggering in Rel-20, these early triggering cases can be categorized as following:

- Case1: TRS measurement

- Case2: PMI-free CSI reporting based on SRS-AS and CSI-RS

- Case3: PMI-based CSI reporting based on CSI-RS

The procedure for early CSI triggering while UE transitioning from idle/inactive mode to connected mode is shown in Figure 1.

Procedure for early triggering in idle/inactive mode

Early SRS/CSI/CSI-RS/TRS triggering can be mainly categorized into three triggering cases as following.

- Case1: TRS measurement

- Case2: PMI-free CSI reporting based on SRS-AS and CSI-RS

- Case3: PMI-based CSI reporting based on CSI-RS

Resource/reporting configuration in SIBx

Agreement For early triggering of SRS-AS when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple SRS configurations based on one or multiple ‘xTyR’ UE capability assumptions. - At least one SRS configuration(s) for SRS-AS is provided for a same ‘xTyR’ - FFS: Information to be provided in SRS configuration(s) - FFS: Whether Configuration information could be SRS configuration or the CSI report configuration for PMI-free report. - FFS: Whether SRS configuration can be associated with CSI report configuration for PMI-free report. - In Step-2, UE reports through MSG3 which/whether the SRS configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the SRS configuration(s) provided in SIBx is/are supported. Agreement For early triggering of aperiodic CSI reporting and the associated CSI-RS for CSI when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple CSI report configurations based on one or multiple UE capability assumptions - Each CSI reporting configuration is associated with a CSI resource configuration for channel measurement. - The CSI-RS resource set(s) in the CSI resource configuration for channel measurement is provided with a same number of CSI-RS ports - Each CSI reporting configuration is associated with a CSI resource configuration for CSI-IM based interference measurement. - FFS: Support of NZP CSI-RS based interference measurement - FFS: Information to be provided in a CSI report/resource configuration - In Step-2, UE reports through MSG3 which/whether the CSI report configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the CSI report configuration(s) provided in SIBx is/are supported. Agreement For early triggering of aperiodic TRS when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-1, i.e., - In Step-1, SIBx provides one CSI resource configuration for aperiodic TRS - FFS: Whether to support more than one TRS resource sets provided in one CSI resource configuration (e.g., TRS resource sets can be provided with different triggering offsets, numbers of TRS bursts and/or gap between TRS bursts, etc) - In Step-2, UE reports through MSG3 whether the CSI resource configuration for aperiodic TRS provided in SIBx is supported. - FFS: Whether/How UE additionally reports via MSG3 the number of TRS bursts that the UE needs Agreement For UE transition from IDLE/INACTIVE to CONNECTED mode, support the followings for aperiodic CSI reporting triggered by MSG4: - For PMI-based reporting, support the report quantity ‘CRI-RI-PMI-CQI’ - FFS: ‘CRI-RI-LI-PMI-CQI’ - For PMI-free reporting, support the report quantity ‘CRI-RI-CQI’

For an idle/inactive UE, CSI resource/reporting configuration is pre-configured by SIB as described in WID. Therefore, one key point is which SIB is used to broadcast the CSI resource/reporting configuration(s). There are two potential solutions as following.

- Alt1: Based on SIB1

- Alt2: Based on a new SIB for early SRS/CSI/CSI-RS triggering

For Alt1, SIB1 should be extended to support resource/reporting configuration of SRS/CSI-RS/CSI/TRS. In this case, although no new SIB is introduced in Rel-20, it would cause a huge SIB1 size, due to so many resource/reporting configurations to be broadcast to UEs with different UE capabilities of early SRS/CSI-RS/CSI/TRS. Considering SIB1 is also received by legacy UEs, it would also increase the complexity for legacy UEs to receive the SIB1.

However, for Alt2, introducing a new SIB can avoid the issue of large SIB1 size, which is more friendly to legacy UEs. Only Rel-20 UEs can receive the new SIB on corresponding SIB occasion. Meanwhile, legacy UEs don’t need to monitor this new SIB.

CSI resource/reporting configuration based on SIB1 would cause a huge SIB1 size, which is not friendly to legacy UEs.

Support to specify a new SIB for CSI resource/reporting configuration for UE in idle/inactive mode.

Regarding the details of resource/reporting configuration in SIB, how to design the resource configuration for SRS/CSI-RS/TRS, and CSI reporting configuration should be further discussed. For TRS triggering, it has been agreed that only TRS resource configuration is provided in SIBx without associated CSI report configuration. For PMI-based CSI triggering and PMI-free CSI triggering, it has been agreed that a CSI resource configuration for channel measurement and a CSI resource configuration for channel measurement for CSI-IM based interference measurement can be associated with a CSI reporting configuration.

However, for PMI-free CSI triggering, there is a remaining issue that whether SRS-AS resource configuration can be associated with CSI report configuration for PMI-free CSI reporting. In the RRC connected mode, SRS-AS resource configuration is not directly associated with a CSI report configuration, since it is gNB’s implementation for how/when to use the estimation results based on SRS-AS to precode CSI-RS. However, for early PMI-free CSI triggering, SRS-AS, CSI-RS and PMI-free CSI reporting need to be triggered by Msg4 simultaneously. Thus, separately triggering SRS-AS without PMI-free CSI reporting makes no sense, since there is only one instance of transition from idle/inactive to connected mode.

Separately triggering SRS-AS makes no sense without triggering PMI-free CSI reporting together.

Moreover, there are two key benefits for SRS-AS resource configuration being associated with a PMI-free CSI report configuration as following.

- The overhead of UE capability reporting in Msg3 can be reduced

- The overhead of triggering in Msg4 can be reduced

Conversely, if the SRS-AS resource configuration and PMI-free CSI reporting configuration broadcast in SIB are separately reported in Msg3 by UE, then the overall overhead of UE capability reporting would be large, and leads to the high complexity of signaling design in Msg3. Meanwhile, if the SRS-AS resource configuration and PMI-free CSI reporting configuration are separately triggered in Msg4, the overall overhead of early triggering would be also large.

There are two key benefits for SRS-AS resource configuration being associated with a PMI-free CSI report configuration as following.

- Reduce the overhead of UE capability reporting in Msg3

- Reduce the overhead of triggering in Msg4

Support SRS-AS resource configuration to be associated with a PMI-free CSI report configuration.

Moreover, another remaining issue on the report quantity of CSI reporting should be further discussed. For early CSI reporting for FR1, it is enough to support one basic report quantity for each case to save the signaling overhead, i.e., 'cri-RI-PMI-CQI' for PMI-based CSI reporting, and 'cri-RI-CQI' for PMI-free CSI reporting. Regarding 'cri-RI-LI-PMI-CQI', since the use case of early CSI is for FR1 as captured in the WID, reporting ‘LI’ for PTRS port determination is not needed.

Only support the basic report quantity for two early CSI reporting cases in Rel-20, i.e.,

- Case1: 'cri-RI-CQI' for PMI-free CSI reporting

- Case2: 'cri-RI-PMI-CQI' for PMI-based CSI reporting

UE capability reporting on Msg3

Agreement For early triggering of SRS-AS when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple SRS configurations based on one or multiple ‘xTyR’ UE capability assumptions. - At least one SRS configuration(s) for SRS-AS is provided for a same ‘xTyR’ - FFS: Information to be provided in SRS configuration(s) - FFS: Whether Configuration information could be SRS configuration or the CSI report configuration for PMI-free report. - FFS: Whether SRS configuration can be associated with CSI report configuration for PMI-free report. - In Step-2, UE reports through MSG3 which/whether the SRS configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the SRS configuration(s) provided in SIBx is/are supported. Agreement For early triggering of aperiodic CSI reporting and the associated CSI-RS for CSI when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple CSI report configurations based on one or multiple UE capability assumptions - Each CSI reporting configuration is associated with a CSI resource configuration for channel measurement. - The CSI-RS resource set(s) in the CSI resource configuration for channel measurement is provided with a same number of CSI-RS ports - Each CSI reporting configuration is associated with a CSI resource configuration for CSI-IM based interference measurement. - FFS: Support of NZP CSI-RS based interference measurement - FFS: Information to be provided in a CSI report/resource configuration - In Step-2, UE reports through MSG3 which/whether the CSI report configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the CSI report configuration(s) provided in SIBx is/are supported.

After UE receives the CSI resource configuration and CSI reporting configuration in SIB, an idle/inactive UE reports its UE capability to notify the network whether it supports the broadcast configuration(s) in SIB. However, when an SRS-AS resource configuration is associated with a PMI-free CSI report configuration in SIB, there is no need to report the support of an SRS-AS resource configuration and a PMI-free CSI reporting configuration separately. Instead, reporting either the support of SRS-AS resource configuration or the support of PMI-free CSI reporting configuration can be considered. However, considering unified design with PMI-based CSI reporting, to report the support of PMI-free CSI reporting configuration in Msg3 is preferred for PMI-free CSI early triggering case.

For early triggering of PMI-free CSI, support to only report the capability of PMI-free CSI reporting configuration in Msg3 by UE.

Another key issue is the signaling design for UE capability reporting in Msg3. In the current spec, UE would use Msg3 to transmit UL-CCCH message including RRCSetupRequest message, RRCResumeRequest message, RRCReestablishmentRequest message, and RRCSystemInfoRequest message for different purposes. For an idle UE, Msg3 would carry RRCSetupRequest message to set up RRC connection.

There are three potential solutions for consideration for UE capability reporting of early SRS/CSI-RS/TRS/CSI as following.

- Alt1: Reusing spare fields in the current RRCSetupRequest message

- Alt2: Introducing a new RRCSetupRequest message in UL-CCCH message

- Alt3: Introducing new values of LCID or eLCID for UL-SCH

Regarding Alt1, there is one spare bit in RRCSetupRequest-IEs, leading to very limited UE capability indication. Besides, other five spare fields in EstablishmentCause can also be reused. Since the original purpose for these spare fields is to indicate establishment causes to the network, using them to indicate UE capabilities should be defined to associate with a certain establishment cause.

Limited spare fields in the current RRCSetupRequest message can be reused to indicate UE capabilities of early SRS/CSI-RS/TRS/CSI.

Regarding Alt2, due to the limited spare fields in RRCSetupRequest message, another potential way is to define a new RRCSetupRequest message in UL-CCCH message. In this way, sufficient number of bits are available for indicating different combinations of UE capability of early SRS/CSI-RS/TRS/CSI.

Introducing a new RRCSetupRequest message can provide sufficient number of bits to indicate UE capabilities of early SRS/CSI-RS/TRS/CSI.

Regarding Alt3, specific values of LCID for UL-SCH has been specified to report UE capability of RedCap UE. Therefore, similar principle can be reused to indicate UE capability of early SRS/CSI-RS/TRS/CSI. Some new values of LCID or eLCID can be defined for different UE capabilities. For instance, a new value#1 of LCID or eLCID can be defined for UE supporting early SRS with xTyR, or a new value#2 of LCID or eLCID can be defined for UE supporting early TRS, or a new value#3 of LCID or eLCID can be defined for UE supporting early PMI-based CSI reporting. Since there are many reserved values of LCID or eLCID in the current spec, it is possible to indicate many different UE capabilities or UE capability combinations of early SRS/CSI-RS/TRS/CSI in Rel-20.

Specifying new values of LCID or eLCID for UL-SCH can be a viable way to indicate UE capabilities of early SRS/CSI-RS/TRS/CSI, which is similar to the indication of UE capability of RedCap.

Regarding indication of UE capabilities of early triggering, following three schemes can be considered in Rel-20.

- Reusing spare fields in the current RRCSetupRequest message

- Specifying a new RRCSetupRequest message in UL-CCCH message

- Specifying new values of LCID or eLCID for UL-SCH

Early triggering based on Msg4

Agreement For UE transition from IDLE/INACTIVE to CONNECTED mode, support at least aperiodic SRS-AS transmission triggered via MSG4 of 4-Step RACH. Conclusion For UE transition from IDLE/INACTIVE to CONNECTED mode, there is no RAN1 consensus to support the following cases triggered by MSG4 of 4-Step RACH: - Periodic and semi-persistent SRS-AS - Periodic and semi-persistent CSI-RS for CSI associated with aperiodic CSI reporting Agreement: For early TRS reception when UE transition from IDLE/INACTIVE to CONNECTED mode, at least support aperiodic TRS with up to Nmax TRS burst(s) triggered by a MAC-CE in MSG4 of 4-Step RACH. - FFS: The value of Nmax - FFS: Whether to further support periodic TRS triggered by MSG4

After the network obtains the UE capability of early SRS/CSI-RS/TRS/CSI, the network can further trigger at least one of early SRS/CSI-RS/TRS/CSI. One remaining issue is that whether periodic TRS is supported to be triggered by Msg4, and the number of aperiodic TRS burst to be supported.

For early TRS, the motivation of early TRS is used to improve the downlink reception in the early phase of RRC connected mode. Considering the time duration from Msg4 to the reception of first downlink transmission after the transition to RRC connected mode would not be long, one-slot early aperiodic TRS measurement triggered by Msg4 before transition to RRC connected mode is enough to provide accurate T/F tracking. Periodic TRS measurement in multiple occasions would increase the UE complexity without significant improvement of tracking gain, compared with aperiodic TRS.

It is enough to only support aperiodic TRS to be triggered by Msg4.

To further improve the performance of T/F tracking, triggering multiple bursts of aperiodic TRS can be considered. A potential solution is that up to two TRS bursts can be supported. Additionally, a gap between two TRS bursts can be introduced to control the overall time span of two TRS bursts to enhance the TRS measurement performance.

Support aperiodic TRS with up to Nmax=2 bursts with a configurable gap of between TRS bursts to be triggered by Msg4.

For early PMI-free CSI triggering, if SRS-AS resource configuration is associated with PMI-free CSI reporting, then separately triggering SRS-AS and PMI-free CSI reporting in different codepoints can be avoided. Thus, to save the Msg4 triggering overhead and simplify the design of Msg4, SRS-AS and PMI-free CSI reporting can be triggered in the same codepoint in Msg4.

For early PMI-free CSI triggering, support SRS-AS and PMI-free CSI reporting to be triggered in the same codepoint in Msg4.

Moreover, it should be mentioned that whether all three cases (i.e., PMI-free CSI reporting, PMI-based CSI reporting, and TRS measurement) are triggered or only some of them are triggered depends on UE’s capability and network’s scheduling. For example, TRS can be triggered together with one of PMI-based CSI reporting and PMI-free CSI reporting. Regarding PMI-based CSI reporting and PMI-free CSI reporting, triggering one of them is sufficient. Besides, SRS-AS can be triggered together with CSI-RS and PMI-free CSI reporting.

Support multiple early SRS/CSI-RS/TRS/CSI to be triggered simultaneously as following.

- CSI-RS + PMI-based CSI reporting

- SRS-AS + CSI-RS + PMI-free CSI reporting

- CSI-RS + PMI-based CSI reporting + TRS

- SRS-AS + CSI-RS + PMI-free CSI reporting + TRS

PUSCH for CSI reporting

Agreement: On PUSCH allocation for MSG4-triggered aperiodic CSI reporting associated with aperiodic CSI-RS for CSI, when UE transition from IDLE/INACTIVE to CONNECTED mode, support Alt-1 from Agreement in RAN1#122bis, i.e., the PUSCH is scheduled by MAC CE in MSG4 along with the aperiodic CSI report triggering. - Strive to reuse legacy UL grant design in RAR - FFS: Whether/How to handle the re-transmission of MSG4 PDSCH with the aperiodic CSI report triggering - FFS: Whether HARQ-ACK for PDSCH Msg4 is transmitted or not.

In the previous meeting, it has been agreed that the PUSCH is scheduled by MAC CE in MSG4 along with the aperiodic CSI report triggering. The remaining issue is whether/how to handle the re-transmission of MSG4 PDSCH with the aperiodic CSI report triggering, and whether HARQ-ACK for PDSCH Msg4 is transmitted or not.

Firstly, HARQ-ACK for PDSCH Msg4 is necessary, since there is a UE identification ID in Msg4 for RACH collision resolution besides early triggering information. gNB should know whether UE receive the Msg4 successfully for subsequent scheduling. Moreover, without HARQ-ACK for PDSCH Msg4, gNB can only perform the blind detection on the resource of the PUSCH, which would increase the gNB complexity and obstruct faster retransmission of Mgs4.

HARQ-ACK for PDSCH Msg4 should be still transmitted, following the legacy principle.

Secondly, when PUSCH for CSI reporting is scheduled by MAC CE in Msg4, after UE decodes the PDSCH of MAC CE, UE would transmit the HARQ-ACK information to gNB, and perform the measurement of CSI-RS associated with the CSI reporting and calculation of CSI. However, unfortunately, if gNB doesn’t receive the HARQ-ACK information successfully, the re-transmission of Msg4 PDSCH would be scheduled. In this case, when UE decodes the PDSCH of Msg4 re-transmission, what is the UE’s behavior should be determined. Two potential solutions can be considered.

- Alt1: UE performs the measurement of CSI-RS associated and calculation of CSI again

- Alt2: Introduce a restriction for re-transmission of Msg4 that UE doesn’t perform new measurement of CSI-RS associated and calculation of CSI

Regarding Alt1, UE would measure a new AP-CSI-RS transmitted by gNB, and get an updated CSI with less latency. Conversely, regarding Alt2, although it can reduce the UE complexity, it can only provide a relatively outdated CSI, which degrades the performance of early CSI.

For re-transmission of Msg4 PDSCH, support UE to perform CSI measurement and CSI calculation again, considering the CSI latency.

QCL/TA/PC assumption

In the current NR system, for a UE CSI-RS/TRS and SRS are transmitted/received after RRC connection is completed, and the parameters for receiving or transmitting the signals, such as TCI state, TA or power control, are configured or indicated by the dedicated RRC signaling or DCI signaling. For early SRS/CSI-RS/TRS/CSI triggering, some of the above-mentioned parameters should be configured in SIB or indicated during RACH procedure, or determined by the default behavior.

Currently TCI states used for AP CSI-RS/TRS are configured by qcl_info in CSI-AssociatedReportConfigInfo, or the indicated TCI states are applied if qcl_info is not configured. However, without TCI state configuration, UE can assume the early CSI-RS/TRS are QCL-ed with the SSB used for PRACH association, which is same as the QCL assumption of the DMRS of PDCCH and PDSCH for receiving Msg2 and Msg4. Besides, regarding the TA, the TA of Msg3 can be reused for early SRS and PUSCH for CSI reporting.

Power control of PUSCH for Msg3 corresponding to a RAR UL grant has been specified in the current spec. Open-loop power control parameters such as P0_PRE, ΔPREAMBLE_Msg3 are provided by SIB1, and closed-loop power control parameters such as δmsg2,b,f,c are indicated by the TPC command in Msg2. The UE uses the same RS resource index as for a corresponding PRACH transmission to estimate the pathloss. For the newly introduced PUSCH for CSI report, a simple way to determine the PUSCH transmission power can be fully reusing that of PUSCH for Msg3. However, more accurate power control specific to PUSCH for CSI report can be designed, e.g., configuring open-loop power control parameters in the new SIB, indicating a closed-loop power control command in Msg4. Thus, detailed design should be further discussed.

Regarding SRS which is newly introduced to random access procedure, the power control parameters should also be defined. Like PUSCH, open-loop power control parameters can be configured in SIB, and a closed-loop power control parameter can be indicated in Msg4 or following that of PUSCH for Msg3.

The early CSI-RS/TRS are QCL-ed with the SSB used for PRACH association for UE.

The TA for the SRS and PUSCH for CSI report is same as that of PUSCH for Msg3.

For the power control of SRS and PUSCH for CSI report,

- Open-loop power control parameters can be configured in SIB, and a closed-loop power control parameter can be indicated in Msg4

- UE uses the same RS resource index as for a corresponding PRACH transmission to calculate the pathloss

SCell activation

In Rel-15, SCell activation/deactivation is based on MAC CE signaling. For power saving purpose, SCell dormancy based on BWP switching by DCI is introduced in Rel-16, which allows fast switching between dormant BWP and non-dormant BWP. In Rel-17, AP-TRS is introduced to further reduce the Tactivation_time for SCell activation, i.e., replacing SSB with AP-TRS to shorten the process time of AGC setting and T/F tracking. However, the processing time for CSI reporting TCSI_Reporting is not optimized, since CSI-RS measurement and CSI reporting should be indicated/configured after UE finishes T/F tracking. Therefore, faster CSI acquisition for SCell activation is a new breakthrough to further reduce the activation time for SCell, including two cases of SCell activation based on MAC CE and switching out of SCell dormancy based on DCI.

SCell activation based on MAC CE

Agreement For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) - FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command - FFS: Whether/what new information is provided in SCell activation command? Agreement For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell. - The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) - FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command? - FFS: Whether/what new information is provided in SCell activation command? Agreement On triggering mechanism for early aperiodic SRS-AS transmission on a SCell and early aperiodic CSI reporting and associated aperiodic CSI-RS for CSI for a SCell, based on the legacy SCell activation activating the SCell, support Alt-2, i.e., - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to an indication in SCell activation command. - For early aperiodic CSI reporting and associated aperiodic CSI-RS for CSI, the CSI report configuration(s) triggered for the SCell is determined according to an indication in SCell activation command.

In the current spec, if a SCell is not activated by MAC CE, there is no SRS-AS transmission and CSI report on the SCell. In Rel-20, directly triggering SRS/CSI-RS/CSI in the MAC CE for SCell activation is discussed to accelerate the process of CSI measurement and reporting, which is similar with AP-TRS being triggered simultaneously in the MAC CE for SCell activation in Rel-17 as shown in Figure2.

An example of timeline for SCell activation with fast CSI acquisition

In the previous meeting, how to indicate SRS-AS, CSI-RS and CSI reporting has been agreed, but how to determine the timeline for early SRS/CSI-RS/CSI triggering in the MAC CE needs further discussion. Regarding the timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS measurement, and aperiodic CSI reporting, one key point is when UE is ready to perform these behaviors. After early triggered by MAC CE, UE might not obtain the necessary information based on SSB for subsequent SRS-AS transmission, CSI-RS measurement, and CSI reporting, e.g., the information of AGC, T/F tracking, TA and pathloss, until UE acquires them based on monitoring SSB. Otherwise, it would have significant impact on the CSI acquisition for the fast SCell activation.

Moreover, RAN4 is also discussing the timeline issues for Rel-20 early SRS/CSI-RS/CSI triggering in MAC-CE for SCell activation, including the impact of BWP switching. Therefore, how to determine the timeline for early SRS/CSI-RS/CSI triggering should be discussed in RAN4 firstly, then RAN1 can further discuss related details, about how to trigger and acquire early CSI for SCell.

Send a LS to RAN4 for clarifying the timeline of early SRS/CSI-RS/CSI triggering for SCell activation based on MAC CE.

Switching out of SCell dormancy based on DCI

Agreement For early triggering of SRS-AS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic SRS-AS transmission on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) Agreement For early triggering of CSI/CSI-RS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic CSI reporting for the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - The aperiodic CSI reporting is associated with aperiodic CSI-RS for CSI on the non-dormant BWP - Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4)

Regarding UE behavior when entering dormant BWP in the current spec, it has been specified that UE can perform periodic or semi-persistent CSI measurement for the dormant BWP, if configured. Besides, no SRS would be transmitted on the dormant BWP. Therefore, when UE switches out of the dormant BWP to non-dormant BWP by DCI, early SRS/CSI-RS/CSI can also be triggered for faster CSI acquisition simultaneously.

Different than MAC CE which can be used for up to 31 SCells activation, only limited bits can be used in DCI for switching between dormant BWP and non-dormant BWP. For DCI format 1_1 and 0_1 with data scheduling, only up to 5 SCell groups can be activated simultaneously. Besides, for DCI format 1_1 without data scheduling, up to 16 SCells can be activated simultaneously by repurposing other DCI fields. In other words, there are not enough bits in DCI to trigger early CSI acquisition per SCell as compared to MAC CE based triggering, especially for DCI format 1_1 and 0_1 with data scheduling.

Therefore, one potential solution is to reuse ‘SCell dormancy indication’ field for early CSI acquisition triggering per SCell with pre-configured CSI resource/reporting configuration(s) by RRC. In this case, a unified mechanism of early CSI acquisition triggering by DCI format 0_1/0_3/1_1/1_3/2_6 can be specified, and there is no need to design the mechanism of early CSI acquisition triggering for different DCI format, especially for DCI format 2_6. When one SCell is activated, the associated CSI resource/reporting configuration is triggered simultaneously. In this way, no additional DCI bits are necessary for early CSI acquisition triggering.

When CSI resource/reporting configuration for each SCell is pre-configured by RRC, no additional DCI bits are needed for early triggering for switching out of SCell dormancy.

There is no additional restriction on ‘SCell dormancy indication’ field to be applied for early triggering in DCI format 0_1/0_3/1_1/1_3/2_6, compared with CSI/SRS request field which doesn’t exist in all DCI formats among DCI format 0_1/0_3/1_1/1_3/2_6.

Support to reuse ‘SCell dormancy indication’ field in DCI format 0_1/0_3/1_1/1_3/2_6 for early triggering with pre-configured early CSI resource/reporting configuration for each SCell.

Moreover, when a SCell is activated by MAC CE, whether the SCell is actually activated or not depends on the configuration of firstAtiveDownlinkBWP-Id. If firstAtiveDownlinkBWP-Id = dormantBWP-Id, although the SCell is activated by MAC CE, the SCell continues to stay in the dormant BWP, until switching out of SCell dormancy based on DCI. However, if firstAtiveDownlinkBWP-Id ≠ dormantBWP-Id, then the SCell is directly activated as shown in Figure 3.

SCell dormancy indication

Whether early CSI acquisition is triggered depends on the configuration of firstAtiveDownlinkBWP-Id.

- If firstAtiveDownlinkBWP-Id = dormantBWP-Id, no early CSI acquisition should be triggered by MAC CE

- If firstAtiveDownlinkBWP-Id ≠ dormantBWP-Id, early CSI acquisition can be triggered with SCell activation together

Thus, as for early CSI acquisition triggering, if firstAtiveDownlinkBWP-Id is configured as dormantBWP-Id by RRC, then use legacy MAC CE for SCell activation only, without related CSI reporting indication. If firstAtiveDownlinkBWP-Id is not configured as dormantBWP-Id, Rel-20 MAC CE for SCell activation with early CSI acquisition triggering can be used.

If firstAtiveDownlinkBWP-Id = dormantBWP-Id, use legacy MAC CE for SCell activation only, without early CSI acquisition triggering.

CSI-RS density reduction

CSI-RS with lower density in subband

Conclusion No enhancement to the following legacy specification restriction on frequency density of CSI-RS port in a CSI subband for the frequency-domain densities ρ = 1/4 and 1/8, i.e., the following from Clause 5.2.1.4 in TS 38.214 shall be applied: - A UE is not expected to be configured with csi-ReportingBand which contains a subband where a CSI-RS resource linked to the CSI Report setting has the frequency density of each CSI-RS port per PRB in the subband less than the configured density of the CSI-RS resource. FFS: Whether to apply the legacy specification restriction to the frequency-domain densities ρ = 1/3 and 1/6

Since new densities of 1/3, 1/4, 1/6, and 1/8 are introduced, whether these new densities match with current subband sizes should be further discussed. However, for new densities of 1/3 and 1/6, the subband size defined in current spec is not divisible by 3 and 6, which could lead to some issues on CSI measurement. Some examples are given below to show the problem of mismatch between the new density and the subband size in Figure 4. It is observed that there are three unexpected cases, where the REs occupied by CSI-RS resources/ports are different in subbands.

Relationship between new densities and subband sizes

In the current TS 38.214, there is a restriction for avoiding the unexpected cases as following.

The reportFreqConfiguration contained in a CSI-ReportConfig indicates the frequency granularity of the CSI Report. A CSI Reporting Setting configuration defines a CSI reporting band as a subset of subbands of the bandwidth part, where the reportFreqConfiguration indicates: - the csi-ReportingBand as a contiguous or non-contiguous subset of subbands in the bandwidth part for which CSI shall be reported. - A UE is not expected to be configured with csi-ReportingBand which contains a subband where a CSI-RS resource linked to the CSI Report setting has the frequency density of each CSI-RS port per PRB in the subband less than the configured density of the CSI-RS resource. - If a CSI-IM resource is linked to the CSI Report Setting, a UE is not expected to be configured with csi-ReportingBand which contains a subband where not all PRBs in the subband have the CSI-IM REs present.

In other words, the average frequency density of each CSI-RS port per PRB in a subband should be larger than or equal to the configured density to guarantee the performance of CSI measurement. However, for the new densities of 1/3 and 1/6, there are no excepted cases as shown in figure 4, if new subband sizes are not introduced. But introducing new subband sizes would bring other related issues such as the value of new subband sizes for different BWP sizes, and new PRG size. Therefore, for new densities of 1/3 and 1/6, a potential solution is not to restrict average frequency density of each CSI-RS port per PRB in a subband. In this case, it is up to UE implementation for CSI-RS estimation and CSI measurement.

For the new densities of 1/3 and 1/6, support the following solution to handle the subband issue as following.

- No restriction of average frequency density of each CSI-RS port per PRB in a subband is required based on UE implementation

- No enhancement on subband size

- No enhancement on PRG size

Conclusion

In summary, the following observations and proposals are made for Rel-20 CSI acquisition.

- Early SRS/CSI/CSI-RS/TRS triggering can be mainly categorized into three triggering cases as following.

- Case1: TRS measurement

- Case2: PMI-free CSI reporting based on SRS-AS and CSI-RS

- Case3: PMI-based CSI reporting based on CSI-RS

- CSI resource/reporting configuration based on SIB1 would cause a huge SIB1 size, which is not friendly to legacy UEs.

- Separately triggering SRS-AS makes no sense without triggering PMI-free CSI reporting together.

There are two key benefits for SRS-AS resource configuration being associated with a PMI-free CSI report configuration as following.

- Reduce the overhead of UE capability reporting in Msg3

- Reduce the overhead of triggering in Msg4

Limited spare fields in the current RRCSetupRequest message can be reused to indicate UE capabilities of early SRS/CSI-RS/TRS/CSI.

Introducing a new RRCSetupRequest message can provide sufficient number of bits to indicate UE capabilities of early SRS/CSI-RS/TRS/CSI.

Specifying new values of LCID or eLCID for UL-SCH can be a viable way to indicate UE capabilities of early SRS/CSI-RS/TRS/CSI, which is similar to the indication of UE capability of RedCap.

When CSI resource/reporting configuration for each SCell is pre-configured by RRC, no additional DCI bits are needed for early triggering for switching out of SCell dormancy.

There is no additional restriction on ‘SCell dormancy indication’ field to be applied for early triggering in DCI format 0_1/0_3/1_1/1_3/2_6, compared with CSI/SRS request field which doesn’t exist in all DCI formats among DCI format 0_1/0_3/1_1/1_3/2_6.

- Whether early CSI acquisition is triggered depends on the configuration of firstAtiveDownlinkBWP-Id

- If firstAtiveDownlinkBWP-Id = dormantBWP-Id, no early CSI acquisition should be triggered by MAC CE

- If firstAtiveDownlinkBWP-Id ≠ dormantBWP-Id, early CSI acquisition can be triggered with SCell activation together

- Support to specify a new SIB for CSI resource/reporting configuration for UE in idle/inactive mode.

Support SRS-AS resource configuration to be associated with a PMI-free CSI report configuration.

Only support the basic report quantity for two early CSI reporting cases in Rel-20, i.e.,

- Case1: 'cri-RI-CQI' for PMI-free CSI reporting

- Case2: 'cri-RI-PMI-CQI' for PMI-based CSI reporting

For early triggering of PMI-free CSI, support to only report the capability of PMI-free CSI reporting configuration in Msg3 by UE.

Regarding indication of UE capabilities of early triggering, following three schemes can be considered in Rel-20.

- Reusing spare fields in the current RRCSetupRequest message

- Specifying a new RRCSetupRequest message in UL-CCCH message

- Specifying new values of LCID or eLCID for UL-SCH

- It is enough to only support aperiodic TRS to be triggered by Msg4.

- Support aperiodic TRS with up to Nmax=2 bursts with a configurable gap of between TRS bursts to be triggered by Msg4.

- For early PMI-free CSI triggering, support SRS-AS and PMI-free CSI reporting to be triggered in the same codepoint in Msg4.

Support multiple early SRS/CSI-RS/TRS/CSI to be triggered simultaneously as following.

- CSI-RS + PMI-free CSI reporting

- SRS-AS + CSI-RS + PMI-free CSI reporting

- CSI-RS + PMI-based CSI reporting + TRS

- SRS-AS + CSI-RS + PMI-based CSI reporting + TRS

- HARQ-ACK for PDSCH Msg4 should be still transmitted, following the legacy principle.

For re-transmission of Msg4 PDSCH, support UE to perform CSI measurement and CSI calculation again, considering the CSI latency.

The early CSI-RS/TRS are QCL-ed with the SSB used for PRACH association for UE.

The TA for the SRS and PUSCH for CSI report is same as that of PUSCH for Msg3.

For the power control of SRS and PUSCH for CSI report,

- Open-loop power control parameters can be configured in SIB, and a closed-loop power control parameter can be indicated in Msg4

- UE uses the same RS resource index as for a corresponding PRACH transmission to calculate the pathloss

- Send a LS to RAN4 for clarifying the timeline of early SRS/CSI-RS/CSI triggering for SCell activation based on MAC CE.

- Support to reuse ‘SCell dormancy indication’ field in DCI format 0_1/0_3/1_1/1_3/2_6 for early triggering with pre-configured early CSI resource/reporting configuration for each SCell.

- If firstAtiveDownlinkBWP-Id = dormantBWP-Id, use legacy MAC CE for SCell activation only without early CSI acquisition triggering.

For the new densities of 1/3 and 1/6, support the following solution to handle the subband issue as following.

- No restriction of average frequency density of each CSI-RS port per PRB in a subband is required based on UE implementation

- No enhancement on subband size

- No enhancement on PRG size