3GPP TSG RAN WG1 #124			R1-2600704

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda item:	10.7.1

Source: 	China Telecom

Title:	Discussion on NTN specific requirements and design for GNSS based operation

Document for:	Discussion

- Introduction

As an essential part of future 6G infrastructure, Non-Terrestrial Networks (NTN) can effectively meet the diverse demands of vertical industries and consumer expectations with broader coverage, stronger network resilience, and higher sustainability. To achieve truly ubiquitous connectivity and enable high accuracy and reliable location services, the integration of NTN with TN necessitates unified architecture and deep convergence. As we discussed in RAN1#122bis meeting [1], the following aspects should be considered in 6G NTN:

Agreement

- The aspects to consider for supporting NTN include, but not limited to

  - Initial access, including cell search and SSB periodicity

  - Coverage

  - Duplexing

  - Capacity

  - Signalling overhead

  - GNSS-less/resilient/based operation

  - Large/varying doppler and propagation delay

  - Beamforming / beam management / beam hopping

Note:

- High-level aspects to consider to enable lower CAPEX/OPEX with respect to current networks include, but not limited to

  - UE/NW implementation complexity

  - UE/NW energy efficiency

  - MRSS

Spectrum efficiency

In this contribution, we analyze the existing implementation solutions for 5G NTN and discuss how they can be improved.

- Discussion

2.1 Timing and Synchronization

The propagation delays in terrestrial mobile systems are usually less than 1 ms. In contrast, the propagation delays in NTN are much longer, ranging from several milliseconds to hundreds of milliseconds, depending on the altitudes of the spaceborne or airborne platforms and payload type in NTN.

Figure 1. Illustration of timing relationship (for collocated gNB and NTN Gateway)

As specified in TS 38.211 and TS 38.213, DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset , and several timing relationships are enhanced by a Common Timing Advance (Common TA) and two offsets Koffset and Kmac, where Koffset is a configured scheduling offset that needs to be larger or equal to the sum of the service link RTT and the Common TA and Kmac is a configured offset that is apWproximately equal to the RTT between the RP and the gNB.

Specifically, Koffset  is used in the following timing relationship:

- The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH)

- The transmission timing of RAR grant scheduled PUSCH

- The transmission timing of HARQ-ACK on PUCCH

- PRACH occasion for PRACH transmission triggered by PDCCH order

- CSI reference resource timing

- The transmission timing of aperiodic SRS

Kmac is used in the following timing relationship:

- Monitor start time of DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI if UE is provided a configuration for PRACH transmission by PRACH-ResourceDedicatedBFR

- Starting time of RS resource updated by MAC CE

- time of the DCI detection window

- time of MAC CE activation command for one of the TCI states

4.2	Transmission timing adjustments

A UE can be provided a value NTA,offset of a timing advance offset for a serving cell by n-TimingAdvanceOffset for the serving cell. If for a serving cell the UE is provided two coresetPoolIndex values 0 and 1 for first and second CORESETs, or is not provided coresetPoolIndex value for first CORESETs and is provided coresetPoolIndex value of 1 for second CORESETs, the UE can be provided first and second NTA,offset values by n-TimingAdvanceOffset and n-TimingAdvanceOffset2 for transmissions with first and second spatial filters associated with first and second TCI states for the first and second CORESETs, respectively.

If a UE operates with two TAGs on a serving cell and is not provided coresetPoolIndex, the UE can be provided a value NTA,offset by n-TimingAdvanceOffset for transmissions with first and second spatial filters associated with first and second joint TCI states provided by dl-OrJointTCI-StateList or with first and second UL TCI states provided by ul-TCI-State-List, respectively.

…

For a timing advance command received on uplink slot n, except for a timing advance command received in a cell switch command, and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant as described in clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in clause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot n+k+1+2μ∙Koffset where k=Nslotsubframe,  μ∙NT,1+NT,2+NTA,max+0.5Tsf, NT,1 is a time duration in msec of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2 is a time duration in msec of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214], NTA,max is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits, Nslotsubframe,  μ is the number of slots per subframe, Tsf is the subframe duration of 1 msec, and Koffset=Kcell,offset-KUE,offset, where Kcell,offset is provided by cellSpecificKoffset and KUE,offset is provided by a Differential Koffset MAC CE command [11, TS 38.321]; otherwise, if not respectively provided, Kcell,offset=0 or KUE,offset=0. N1 and N2 are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers. For μ=0, the UE assumes N1,0=14 [6, TS 38.214]. Slot n and Nslotsubframe,  μ are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG. NTA,max is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and for all configured initial UL BWPs provided by initialUplinkBWP. The uplink slot n is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211].

…

## 8.2	Random access response - Type-1 random access procedure

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321] if the PRACH transmission is not triggered by a PDCCH order that includes a Cell Indicator field with non-zero value; otherwise, the UE does not attempt to detect the DCI format 1_0. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

…

To sum up, 5G NTN introduces two additional offsets on top of TN to meet timing relationship requirements. In 6G, the goal is to maximize unified design between TN and NTN. Therefore, in our view, on Day 1 of 6G, NTN timing relationships should adopt the same design as TN, rather than incorporating extra offsets.

Proposal 1：Support unified timing and synchronization design between TN and NTN.

2.2 HARQ Enhancements

HARQ Round Trip Time in NR is of the order of several milliseconds. The propagation delays in NTN are much longer, ranging from several milliseconds to hundreds of milliseconds, depending on the satellite orbit. The HARQ RTT can be much longer in NTN. As specified in TS 38.300 and TS 38.213, to mitigate the impact of HARQ stalling in NTN, the HARQ feedback can be disabled in the presence of ARQ re-transmissions at the RLC layer, and/or the number of HARQ processes for re-transmissions at the MAC layer can be increased to 32.

The network may configure the HARQ operation as follows:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process (as specified in clause 5.3.2.2 and clause 5.7 of TS 38.321 [6]). Disabling HARQ feedback allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

-	For uplink, HARQ mode (i.e. HARQ mode A or HARQ mode B) can be configured per HARQ process (as specified in clause 5.4.3.1 and clause 5.7 of TS 38.321 [6]). HARQ mode B allows scheduling a HARQ process before one HARQ RTT has elapsed since last scheduled.

NOTE:	For the HARQ processes configured with HARQ feedback enabled/disabled, it is up to network implementation to ensure a proper configuration of HARQ feedback (e.g., either all enabled or all disabled) for HARQ processes used by an SPS configuration. For the HARQ processes configured with HARQ mode, it is up to network implementation to ensure a proper configuration of HARQ mode (e.g., either all HARQ mode A or all HARQ mode B) for HARQ processes used by a CG configuration.

In this first RAN1 meeting, we propose to support disabling HARQ and the number of HARQ processes in NR as a starting point. Additionally, considering the RTT of LEO and GEO satellites, HARQ buffer size, RLC feedback, and RLC ARQ buffer size of 6G UE, whether extended HARQ processes should be applied in GEO scenarios, and whether the number of processes needs to be further expanded, remains a topic for further study.

Proposal 2：Support HARQ disabling from 6G day-1.

Proposal 3：Support the number of HARQ processes in NR as a starting point, whether expanding the number requires further study.

2.3 Duplexing Mode

For TDD mode, the TDD pattern with a longer periodicity and a guard interval with a longer duration is required to avoid simultaneous transmission and reception at the UE. This guard time is directly determined by the propagation delay between the UE and gNB, and will directly impact the useful throughput and hence the spectral efficiency. Due to the long RTT between satellites and UEs, the required guard period for TDD in NTN scenarios ranges from approximately 14 ms for LEO to 540 ms for GEO systems. Additionally, the dynamic variation and uncertainty of propagation delay in LEO satellites present further challenges for TDD implementation. As a result, the efficiency of TDD mode requires further analysis and optimization under such conditions before standardization.

Besides, considering that most frequency bands allocated for satellite services are paired, though some unpaired bands are available, and given the benefits of unpaired spectrum allocation, HD-FDD operation should also be considered for NTN. Accordingly, it is recommended that both FDD and HD-FDD modes be prioritized in NTN design.

Proposal 4：FDD and HD-FDD operation in 6G NTN should be prioritized.

2.4 PAPR Requirements/Enhancements for NTN

PAPR (peak-to-average power ratio) is a very important metric when it comes to operating transmission equipment as efficiently as possible. High PAPR of transmitted signals introduces significant nonlinear distortions and leads to increased power consumption in NTN systems, particularly impacting satellite downlink coverage and power amplifier efficiency.

The primary issue is to assess the impact of PAPR and whether existing technologies can address the high PAPR problem. Besides, to address this challenge, the adoption of low-PAPR waveform/modulation becomes essential. Specifically, applying DFT-s-OFDM in the downlink direction, or along with other PAPR-efficient modulation schemes such as APSK, etc., presents a promising solution set for 6G NTN. Consequently, the evaluation and specification of low-PAPR waveform/modulation for both downlink and uplink should be considered as an important design aspect in 6G NTN.

Proposal 5: Evaluate the impact of PAPR and optimize low-PAPR solutions.

2.5 Coverage & Beam Hopping

In 5G NR NTN, an extended periodicity of 160 ms (apart from the existing 20ms value) for the half frames with SS/PBCH blocks is assumed by UE during initial cell selection to extend downlink coverage. However, based on the observation from multiple companies, the coverage ratio is about only 48.4% when the periodicity of SSB is 160ms in Set 1-2. In 6G NTN, the total number of beam footprints may further increase, and the existing SSB period may not meet coverage requirements. Therefore, the period value for the half frames with SS/PBCH blocks should be further studied.

Proposal 6：Further study/extend the default periodicity of SSB.

- Conclusions

In this contribution, we discussed existing implementation solutions for 5G NTN and how they can be improved. The following proposals were made.

Proposal 1：Support unified timing and synchronization design between TN and NTN.

Proposal 2：Support HARQ disabling from 6G day-1.

Proposal 3：Support the number of HARQ processes in NR as a starting point, whether expanding the number requires further study.

Proposal 4：FDD and HD-FDD operation in 6G NTN should be prioritized.

Proposal 5: Evaluate the impact of PAPR and optimize low-PAPR solutions.

Proposal 6：Further study/extend the period value for the half frames with SS/PBCH blocks.

- References

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #122bis, Prague, Czech, Oct 13th – 17th, 2025.