3GPP TSG RAN WG1 #124		R1-2600730

Gothenburg, Sweden, Feb 9th – 13th, 2026

Agenda item:	8.6

Source:	Samsung

Title:	Maintenance on low-power wake-up signal and receiver for NR

Document for:	Discussion and Decision

# Introduction

This contribution includes discussions on the remaining issues for low-power wake-up signal and receiver for NR.

# LP-WUS and LP-SS design

## Editorial changes for alignment

RAN2 specification has finalized the RRC parameters for Rel-19, and RAN1 specifications shall be updated to align the structures and names of RRC parameters accordingly.

Proposal 1: For LP-WUS and LP-SS design:

- Adopt TP#1 as part of an alignment CR for TS 38.211.

- Adopt TP#2 as part of an alignment CR for TS 38.212.

# LP-WUS operation in IDLE/INACTIVE modes

## Editorial changes for alignment

RAN2 specification has finalized the RRC parameters for Rel-19, and RAN1 specifications shall be updated to align the names of RRC parameters accordingly.

Proposal 2: For LP-WUS operation in IDLE/INACTIVE modes:

- Adopt TP#3 as part of an alignment CR for TS 38.213.

# LP-WUS operation in CONNECTED modes

## LP-WUS monitoring in Option 1-2

For Option 1-2, the UE behavior on LP-WUS monitoring after the reception of the wake-up indication needs to be further clarified. For example, as shown in Figure 1, after a UE receives a wake-up indication (e.g., in the blue MO in the figure), the UE may start the timer given by lpwus-PDCCH-MonitoringTimer after the time offset given by lpwus-TimeOffset1-2 wherein, within the lpwus-TimeOffset1-2, there can be additional MOs with different period (e.g., the red MO in the figure) associated with other new timer. If the UE continues to monitor the MOs within the lpwus-TimeOffset1-2, the UE may need to restart the timer given by lpwus-PDCCH-MonitoringTimer if receiving another wake-up indication, which is a new UE behavior not specified in RAN2. To clarify the UE behavior, two alternatives are discussed in the last meeting:

- Alt 1: After the reception of wake-up indication, LP-WUS monitoring in the MOs within the time lpwus-TimeOffset1-2 should stop.

- Alt 2: After the reception of wake-up indication, it is up to UE implementation whether to continue to monitor LP-WUS in the MOs within the time period given by lpwus-TimeOffset1-2. The UE restarts the timer given by lpwus-PDCCH-MonitoringTimer, if receiving another wake-up indication during the time lpwus-TimeOffset1-2.

Comparing the two alternatives, Alt 1 is simpler for implementation by both UE and gNB, and hence is preferred.

Figure 1 Illustration of LP-WUS monitoring in Option 1-2.

Proposal 3: For Option 1-2, support Alt 1 and adopt TP#5 for TS 38.213.

- Alt 1: after the reception of wake-up indication, a UE does not need to monitor LP-WUS in the remaining MOs within the time period given by lpwus-TimeOffset1-2.

- Alt 2: RAN1 concludes that it is up to UE implementation whether to continue to monitor LP-WUS in the remaining MOs within the time period given by lpwus-TimeOffset1-2 after the reception of wake-up indication, and it will be up to RAN2 to decide whether to specify the UE behavior on restarting the timer given by lpwus-PDCCH-MonitoringTimer.

## Editorial changes for alignment

RAN2 specification has finalized the RRC parameters for Rel-19, and RAN1 specifications shall be updated to align the names of RRC parameters accordingly.

Proposal 4: For LP-WUS operation in CONNECTED modes:

- Adopt TP#4 as part of an alignment CR for TS 38.213.

# Conclusion

The proposals from this contribution are summarized below:

Proposal 1: For LP-WUS and LP-SS design:

- Adopt TP#1 as part of an alignment CR for TS 38.211.

- Adopt TP#2 as part of an alignment CR for TS 38.212.

Proposal 2: For LP-WUS operation in IDLE/INACTIVE modes:

- Adopt TP#3 as part of an alignment CR for TS 38.213.

Proposal 3: For Option 1-2, support Alt 1 and adopt TP#5 for TS 38.213.

- Alt 1: after the reception of wake-up indication, a UE does not need to monitor LP-WUS in the remaining MOs within the time period given by lpwus-TimeOffset1-2.

- Alt 2: RAN1 concludes that it is up to UE implementation whether to continue to monitor LP-WUS in the remaining MOs within the time period given by lpwus-TimeOffset1-2 after the reception of wake-up indication, and it will be up to RAN2 to decide whether to specify the UE behavior on restarting the timer given by lpwus-PDCCH-MonitoringTimer.

Proposal 4: For LP-WUS operation in CONNECTED modes:

- Adopt TP#4 as part of an alignment CR for TS 38.213.

# Appendix: Collection of TPs

## TP#1

=================================== TP #1 for TS 38.211 ===================================

Reason for change: RRC parameter names are not aligned with TS 38.331 and typos exist in TS 38.211.

Summary of changes: Align RRC parameter names with TS 38.331 and fix typos.

Consequence if not approved: RRC parameter names are not aligned with TS 38.331 and typos exist in TS 38.211.

=================================== Start of TP #1 =======================================

7.4.4.1.1	Generation of rZC,m(n)

=============================== Unchanged Text Omitted ===================================

The root sequence number q∈1, …, NZC-1 is configured by the higher-layer parameter root1 or root2 in lpwus-OverlaidSeqRoot, where root1 is used for the root sequence number when cmP=0, and root2 is used otherwise.The cyclic shift ncs is given by

ncs=cm mod PNZCP

P=NseqNroot

where

-	Nseq is the number of sequences configured by the higher-layer parameter lpwus-OverlaidSeqNum or lpwus-OverlaidSeqNum-SCS-60kHz or lpwus-OverlaidSeqNum-SCS-120kHz

-	Nroot=2 if root2 in lpwus-OverlaidSeqRoot is configured, otherwise Nroot=1

=============================== Unchanged Text Omitted ===================================

7.4.4.1.2	Generation of rWUS(n)

The block of complex-valued symbols rWUS(0),…,rWUS(MbitMZC-1) is defined by

rWUSlNscWUS+k=1NscWUSi=0NscWUS-1rWUSlNscWUS+ie-j2πi(k-NscWUS2)NscWUS

k=0,1,…,NscWUS-1

l=0,1,…,MbitMWUS-1

NscWUS=11NscRB

where

rWUSmMZC+n=b(m)rZC,m(n)

m=m2

m=0,1,…,Mbit-1

n=0,1,…,MZC-1

The quantity MWUS∈1, 2, 4 is given by the higher-layer parameter lpwus-MvalueAndSeqConfigFR1or lpwus-MvalueAndSeqConfigFR2.

The bit sequence b0, …, b(Mbit-1) and the number of bits Mbit corresponds to g00,g01,…,g0G0-1 and G0, respectively, in clause 7.4.3 of [4, 38.212].

=============================== Unchanged Text Omitted ===================================

7.4.5.1.1	Generation of rOOK(n)

The sequence rOOK0,…,rOOK(NOOK-1) is defined by Tables 7.4.5.1.1-1 to 7.4.5.1.1-3 with the quantity MLPSS given by the higher-layer parameter lpss-MvalueAndSeqConfig, the sequence length by the higher-layer parameter lpss-BinarySeqLen, and the configuration index by the higher-layer parameter lpss-BinarySeqIndex.

=============================== Unchanged Text Omitted ===================================

7.4.5.1.2	Generation of rZC(n)

If the quantity qϵ1,…,NZC-1 is configured by the higher-layer parameter lpss-OverlaidSeqRoot, the sequence rZC(n) is defined by

rZC(n)=xqn mod NZC

xqi=e-jπqi(i+1)NZC

n=0,1,…,MZC-1

=============================== Unchanged Text Omitted ===================================

7.4.5.2	Mapping to physical resources

The UE shall assume the block of complex-valued symbols rLPSS(0),…,rLPSS(NOOKMZC-1) is scaled by a factor βLPSS and mapped to resource elements k,lp,μ used for LPSS transmission in increasing order of first k and then l.

==================================== End of TP #1 =======================================

## TP#2

=================================== TP #2 for TS 38.212 ===================================

Reason for change: RRC parameter names are not aligned with TS 38.331.

Summary of changes: Align RRC parameter names with TS 38.331.

Consequence if not approved: RRC parameter names are not aligned with TS 38.331.

=================================== Start of TP #2 =======================================

7.4	Wake-up information

The wake-up information is carried by a wake-up signal as defined in clause 7.4.4 of [4, TS 38.211].

-	For a UE configured with higher layer parameter lpwus-LPSS-StartRB and operating in the RRC_IDLE or RRC_INACTIVE state, the wake-up information bit sequence c0,c1,…,cK-1 is the binary sequence of the codepoint as defined by Clause 10.4C of [5, TS38.213], where c0 is the most significant bit and K is

-	maxlog2NPO, 1  if NsubgroupPO=1, where NPO is configured by higher layer parameter lpwus-PO-NumPerLO and NsubgroupPO is configured by higher layer parameter lp-SubgroupsNumPerPO;

-	log2NPONsubgroupPO+1 if NsubgroupPO>1, where NsubgroupPO is configured by higher layer parameter lp-SubgroupsNumPerPO;

-	For a UE configured with higher layer parameter lpwus-StartRB and operating in the RRC_CONNECTED state, the wake-up information bit sequence c0,c1,…,cK-1 is the binary sequence of the codepoint as defined by Clause 10.4D of [5, TS38.213], where c0 is the most significant bit, K is provided by the higher layer parameter lpwus-NumOfBits.

=============================== Unchanged Text Omitted ===================================

7.4.2.1	Rate matching for OOK modulation

The input bit sequence to rate matching is d00,d01,…,d0N0-1.

Rate matching is performed according to Clause 5.4.3 by setting the rate matching output sequence length E=E0, where E0=EWUS, EWUS=NOS×MLP/2, where

-	for a UE configured with higher layer parameter lpwus-LPSS-StartRB and operating in the RRC_IDLE or RRC_INACTIVE state, NOS is configured by higher layer parameter lpwus-ActualDuration in LowPowerConfig and MLP is configured by higher layer parameter lpwus-MvalueAndSeqConfigFR1 or lpwus-MvalueAndSeqConfigFR2 in LowPowerConfig;

-	for a UE configured with higher layer parameter lpwus-StartRB and operating in the RRC_CONNECTED state, NOS is configured by higher layer parameter lpwus-ActualDuration in LPWUS-Config and MLP is configured by higher layer parameter lpwus-MvalueAndSeqConfigFR1 or lpwus-MvalueAndSeqConfigFR2 in LPWUS-Config.

The output bit sequence after rate matching is denoted as f00,f01,f02,…,f0E0-1.

7.4.2.2	Rate matching for sequence modulation

Information bits for the second bit block are delivered to the rate matching block. They are denoted by c0,c1,…,cK-1, where K is the number of bits and K≤5.

If the number of sequences configured by higher layer parameter lpwus-OverlaidSeqNum or lpwus-OverlaidSeqNum-SCS-60kHz or lpwus-OverlaidSeqNum-SCS-120kHz, denoted as L1, is larger than one, padding is performed and the bits after padding are denoted by d10,d11,…,d1N1-1, where N1=K+L, L=-K  mod log2L1. The relation between ck and d1k is:

d1k=0 for k=0,1,…, L-1

d1k=ck-L for k=L,L+1,…, N1-1.

=============================== Unchanged Text Omitted ===================================

==================================== End of TP#2 =======================================

## TP#3

=================================== TP #3 for TS 38.213 ===================================

Reason for change: RRC parameter names are not aligned with TS 38.331.

Summary of changes: Align RRC parameter names with TS 38.331.

Consequence if not approved: RRC parameter names are not aligned with TS 38.331.

=================================== Start of TP #3 =======================================

10.4C	PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE

A UE configured with DRX mode operation and operating in the RRC_IDLE or RRC_INACTIVE state can be provided for LPSS/WUS reception

-	a number of OOK symbols per OFDM symbol, a first RB by lpwus-LPSS-StartRB, and an overlaid OFDM sequence per OOK symbol for LPSS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211],

-	a number of OOK symbols per OFDM symbol, the first RB, and one or more overlaid OFDM sequences per OOK symbol for WUS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211], and

A UE determines to receive LPSS/WUS based on procedures defined in [17, TS 38.304].

A UE assumes that an SCS configuration for LPSS/WUS receptions is same as an SCS of the initial DL BWP and an SCS configuration of an SS/PBCH block the UE used to obtain SIB1.

A UE receives an LPSS in consecutive symbols within a slot. The UE can be provided one or two first symbols for respective one or two LPSS reception occasions in the slot by lpss-StartSymbol. The UE determines slots for LPSS reception occasions based on a periodicity and a time offset, relative to the start of a system frame with SFN 0, provided by lpss-PeriodicityAndOffset. Within a period of LPSS reception occasions, LPSS reception occasions are in a set of KL consecutive slots that have all symbols indicated as downlink by tdd-UL-DL-ConfigurationCommon, if provided, and start from the first slot provided by the time offset in the period, where K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1 and L is the number of LPSS reception occasions in a slot.

LPSS reception occasions are indexed sequentially in time. An LPSS reception at the k-th LPSS reception occasion is quasi co-located with the k-th transmitted SS/PBCH block, with respect to quasi co-location 'typeC' or 'typeD' properties when applicable, where 1≤k≤K.

If a UE is provided lpwus-LPSS-BeamSubset, the UE receives LPSS/WUS based on the quasi co-location properties of transmitted SS/PBCH blocks indicated by lpwus-LPSS-BeamSubset [12, TS 38.331]; otherwise, the UE receives LPSS/WUS based on the quasi co-location properties for transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1. A WUS occasion includes K⋅M WUS monitoring occasions that are indexed sequentially in time, where

-	K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1, M is a number of WUS monitoring occasions associated with each of the K transmitted SS/PBCH blocks provided by lpwus-MO-NumPerLO, and

-	a WUS monitoring occasion with index k-1⋅M+m-1, where 1≤m≤M and 1≤k≤K, is quasi co-located with the k-th transmitted SS/PBCH block with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable

A UE can be provided, by lpwus-AvailableSlot in LowPowerConfig, a bitmap that corresponds to a set of time units that repeats continuously and indicates a subset of time units from the set of time units that is available for the UE to monitor WUS [12, TS 38.331]. A time unit includes one slot or two slots. A set of time units includes a total of either 10, or 20, or 40 time units. A duration P, in msec, of the set of time units has maximum value of 40 msec. The first symbol of the set of time units every 40 msec/P periods is a first symbol in frame nf mod 4 = 0. The UE can be additionally provided, by lpwus-AvailableSymbol in LowPowerConfig, an indication of symbols in each time unit from the subset of time units that is available for the UE to monitor WUS. If the UE is not provided lpwus-AvailableSlot, the UE assumes that all time units are available for the UE to monitor WUS. If the UE is not provided lpwus-AvailableSymbol, the UE assumes a time unit of one slot and, for a time unit that is available for the UE to monitor WUS, all symbols in the time unit are available for the UE to monitor WUS. The UE assumes that a symbol is not available to monitor WUS when

-	the symbol is indicated as uplink, by tdd-UL-DL-configurationCommon

-	the symbol is indicated for an SS/PBCH block transmission, by ssb-PositionsInBurst in SIB1, and the SS/PBCH block transmission would overlap in frequency with the WUS transmission

-	the symbol is indicated for PDCCH transmissions, by pdcch-ConfigSIB1, and CORESET 0 for the PDCCH transmissions would overlap in frequency with the WUS transmission

A WUS monitoring occasion is over a first number of symbols, provided by lpwus-NominalMoDuration in LowPowerConfig. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion is smaller than a second number of symbols, provided by lpwus-ActualDuration in LowPowerConfig, the UE does not monitor WUS in the WUS monitoring occasion. The UE monitors WUS in a WUS monitoring occasion over the earliest available lpwus-ActualDuration symbols in the WUS monitoring occasion. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion includes a symbol for LPSS reception, the UE does not monitor WUS in the WUS monitoring occasion.

A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on lpwus-PO-NumPerLO. A first frame of a WUS occasion starts a number of frames prior to the first of a number of paging frames associated with the WUS occasion [17, TS 38.304]. The first WUS monitoring occasion of a WUS occasion starts at an offset provided by lpwus-OffsetFirstMoWithinLo relative to the start of the first frame.

A paging occasion associated with a WUS occasion has index iPO=UE_ID mod N⋅NS+i_s mod NPOWO where NPOWO is a number of paging occasions associated with a WUS occasion, N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. If a number of NSGPO subgroups per paging occasion, provided by lp-SubgroupsNumPerPO, is NSGPO>1, the codepoint for the subgroup index iSG in a PO iPO is iPO*NSGPO+1+iSG, and the codepoint for all subgroups in the PO is (iPO+1)*NSGPO+1-1; otherwise, the codepoint for the PO iPO is iPO.

If, in a WUS monitoring occasion, a UE determines a codepoint associated with the UE [17, TS 38.304], the UE performs PDCCH monitoring according to Type2-PDCCH CSS sets for the paging occasion associated with the WUS monitoring occasion; otherwise, the UE is not required to perform the PDCCH monitoring. The UE may also perform PDCCH monitoring for Type2A-PDCCH CSS sets for DCI format 2_7, if provided.

==================================== End of TP #3 =======================================

## TP#4

=================================== TP #4 for TS 38.213 ===================================

Reason for change: RRC parameter names and structures are not aligned with TS 38.331.

Summary of changes: Align RRC parameter names and structures with TS 38.331.

Consequence if not approved: RRC parameter names and structures are not aligned with TS 38.331.

=================================== Start of TP #4 =======================================

10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

A UE configured with DRX mode operation and operating in the RRC_CONNECTED state can be provided for WUS reception on the primary cell of a cell group

-	a number of OOK symbols per OFDM symbol, a first RB by lpwus-StartRB, and overlaid OFDM sequences per OOK symbol for WUS reception [4, TS 38.211], and

-	a number of codepoints provided for the UE by the WUS [6, TS 38.212], by lpwus-Codepoint

A UE assumes that a WUS is quasi co-located with an SS/PBCH block or a CSI-RS with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable.

If a UE is provided dl-OrJointTCI-StateList, the UE receives WUS based on the quasi co-location information of the TCI states provided by dl-OrJointTCI-StateList, indicated by a most recent DCI format or MAC CE, after a respective application time in clause 5.1.5 [6, TS 38.214]; otherwise, the UE receives WUS based on the quasi co-location information of the TCI states for a CORESET with controlResourceSetId value that is same as the one indicated by lpwus-TCI-States.

A UE assumes that an SCS configuration for WUS receptions is same as an SCS configuration for the active DL BWP.

A UE does not monitor a WUS during Active Time [11, TS 38.321].

A UE does not monitor WUS during DTX inactive period for the primary cell.

When lpwus-PDCCH-MonitoringTimer is not provided, a UE can be provided by lpwus-MO-1-1 a periodicity and a time offset, relative to the start of a system frame with SFN 0, for the UE to determine WUS monitoring occasions. The UE starts to monitor WUS in a first WUS monitoring occasion that is not earlier than a first slot that is prior to a second slot where the drx-onDurationTimer would start by a time provided by lpwus-TimeOffset1-1, and monitors WUS for a number of monitoring occasions provided by lpwus-NumOfMO-1-1. The UE reports a number of milliseconds [18, TS 38.306] where the UE is not required to monitor WUS prior to the slot where the drx-onDurationTimer would start. The UE is not required to monitor WUS within the reported number of slots prior to the slot where the drx-onDurationTimer would start. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts the drx-onDurationTimer [11, TS 38.321].

When lpwus-PDCCH-MonitoringTimer is provided, a UE can be provided by lpwus-MO-1-2 a periodicity and a time offset, relative to the start of a system frame with SFN 0, for the UE to determine first WUS monitoring occasions from a number of WUS monitoring occasions per periodicity, provided by lpwus-NumOfMO-1-2. The UE reports a number of milliseconds [18, TS 38.306] and expects that a time gap, from a last WUS monitoring occasion from the number of WUS monitoring occasions per periodicity to the slot where the lpwus-PDCCH-MonitoringTimer would start, is no smaller than the reported number of milliseconds If the UE determines to monitor PDCCH based on a detected WUS, the UE starts lpwus-PDCCH-MonitoringTimer [11, TS 38.321] after a time, provided by lpwus-TimeOffset1-2, with respect to the start of the first WUS monitoring occasion from the number of WUS monitoring occasions per periodicity.

A UE can be provided, by lpwus-AvailableSlot in LPWUS-Config, a bitmap that corresponds to a set of time units that repeats continuously and indicates a subset of time units from the set of time units that is available for the UE to monitor WUS [12, TS 38.331]. A time unit includes one slot or two slots. A set of time units includes a total of either 10, or 20, or 40 time units. A duration P, in msec, of the set of time units has maximum value of 40 msec. The first symbol of the set of time units every 40 msec/P periods is a first symbol in frame nf mod 4 = 0. The UE can be additionally provided, by lpwus-AvailableSymbol in LPWUS-Config, an indication of symbols in each time unit from the subset of time units that is available for the UE to monitor WUS. If the UE is not provided lpwus-AvailableSlot, the UE assumes that all time units are available for the UE to monitor WUS. If the UE is not provided lpwus-AvailableSymbol, the UE assumes a time unit of one slot and, for a time unit that is available for the UE to monitor WUS, all symbols in the time unit are available for the UE to monitor WUS. The UE assumes that a symbol is not available to monitor WUS when

-	the symbol is indicated as uplink, by tdd-UL-DL-configurationCommon or tdd-UL-DL-ConfigurationDedicated

-	the symbol is indicated for transmission of SS/PBCH blocks, by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon

A WUS monitoring occasion is over a first number of symbols, provided by lpwus-NominalMoDuration in LPWUS-Config. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion is smaller than a second number of symbols, provided by lpwus-ActualDuration in LPWUS-Config, the UE does not monitor WUS in the WUS monitoring occasion. The UE monitors WUS in a WUS monitoring occasion over the earliest available lpwus-ActualDuration symbols in the WUS monitoring occasion.

If a UE detects a codepoint in a WUS reception, from the number of codepoints, on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group.

==================================== End of TP #4 =======================================

## TP#5

=================================== TP #5 for TS 38.213 ===================================

Reason for change: The UE behavior for WUS monitoring after receiving waking-up indication is ambiguous.

Summary of changes: Add the UE behavior for WUS monitoring after receiving waking-up indication.

Consequence if not approved: The UE behavior for WUS monitoring after receiving waking-up indication is not specified and unclear.

=================================== Start of TP#5 =======================================

10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

=============================== Unchanged Text Omitted ===================================

If a UE detects a codepoint in a WUS reception, from the number of codepoints, on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group. If lpwus-PDCCH-MonitoringTimer is provided, after detecting the codepoint, the UE does not need to monitor WUS in the remaining WUS monitoring occasions within the associated time provided by lpwus-TimeOffset1-2.

==================================== End of TP#5 =======================================