3GPP TSG RAN WG1 #124                                                                   R1- 2600837

Gothenburg, Sweden, Feb 9th – 13th, 2026

Agenda Item:	10.5.3.3

Source:	Apple Inc.

Title:	On other aspects of CSI acquisition

Document for:	Discussion/Decision

# Introduction

6GR starts discussion of other aspects of CSI acquisition for 6GR. Based on the latest draft agenda [1], it should cover "CSI acquisition and report jointly considering both downlink and uplink, other reference signal(s) design and transmission, e.g., for tracking, etc.”. In this contribution, we provide our initial thoughts on CSI/SRS fusion, time/frequency tracking and phase noise tracking.

# Discussion on CSI and SRS fusion

The motivation for fusing Sounding Reference Signals (SRS) with Channel State Information (CSI) feedback lies in balancing the trade-off between spectral granularity, temporal freshness, and signal-to-noise ratio (SNR). SRS provides high-resolution, per-Resource Block (RB) measurements essential for precise beamforming. However, because SRS often employs frequency hopping to cover a wide bandwidth, it takes multiple SRS hopping periodicities to complete a full channel observation, leading to significant channel aging where the earlier hop is outdated by the time of transmission. Furthermore, SRS involves a critical power trade-off: while a wideband SRS captures the whole channel in one shot, the User Equipment's (UE) transmit power is spread across the entire bandwidth, resulting in poor uplink SNR for cell-edge users. Frequency hopping improves this SNR by concentrating power into a narrower band at each hop, but it exacerbates the aging issue.

In contrast, CSI feedback provides a wideband snapshot in a single reporting instance. Although it is coarser at a subband level (e.g., 16 RBs for a 100MHz carrier), it offers a more immediate, low-latency observation that is less susceptible to mobility-induced aging. By fusing these two sources, we can leverage the high-precision, high-SNR narrowband measurements from SRS while using the timely, wideband CSI reports to "anchor" and update the aging frequency hops.

Evaluation assumption and preliminary results:

To evaluate this fusion, previous R18 AI-based CSI compression, where subband SGCS is defined as the intermediate metric, cannot accurately reflect the frequency granularity impact. We propose using per-RB or per-Resource Block Group (RBG) Squared Generalized Cosine Similarity (SGCS) as an intermediate metric:

- Per RB SGCS: the SGCS between the precoding matrix and eigen-vector of each RB’s transmission covariance matrix.

- Per RBG SGCS: the SGCS between the precoding matrix and eigen-vector of transmission covariance matrix averaged per RBG.

- Per subband SGCS: the SGCS between the precoding matrix and eigen-vector of transmission covariance matrix averaged per subband.

We focus on wide width carrier with 100MHz BW. The antenna parameters for both the gNB and UE follow assumptions similar to those used in MIMO evaluations. Table 1 lists simulation assumptions for CSI/SRS fusion.

Table 1: SLS assumption for CSI and SRS fusion

| Parameter | Description |
| --- | --- |
| Scenario | Dense Urban (Macro only) |
| Frequency Range | FR1: 3.5GHz |
| Deployment | FR1: 7 site, 21 cells, 24 UEs per cell. 80% UE indoor, 20% UE outdoor. Spatial consistency: OFF |
| Channel model | According to TR 38.901 |
| Antenna setup and port layouts at gNB | FR1: 64 ports: (12,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ for model input |
| Antenna setup and port layouts at UE | FR1: 4Rx: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BW | 100MHz frequency, with 30KHz SCS. |
| Subband number and size | 16 PRB. Total of 16 subbands within 100MHz channel |
| SRS hopping periodicity and BW | 16 hops, one subband per hop 4 hops, four subbands per hop Wideband transmission over entire 16 subband. SRS periodicity is 40ms per hop. |
| Benchmark | For CSI feedback, e-type 2 Config 3. |
| Performance metric | Per RB and per RBG SGCS, where per RBG or per subband precoding is used at gNB. |
| Noise modelling | DL: based on CSI-RS measurement full power transmission (51 dBm). One CSI-RS RE per port per RB. UL: based on SRS sounding assuming full power transmission (23 dBm), 6 SRS REs per SRS port per RB. |

For comparison, we evaluate e-type-2 PMI feedback and pure SRS-based sounding under different hopping and SNR trade-off configurations. Performance is assessed separately with and without downlink (DL) and uplink (UL) noise modeling. Table 2 presents the results without noise, whereas Table 3 reports the performance when DL and UL noise are taken into account. For UL noise modeling, the SRS transmit power is assumed to be 23 dBm. When comparing 16-hop and 4-hop SRS configurations, the 4-hop SRS achieves a 6 dB lower UL SNR, while also experiencing reduced channel aging effects.

Table 2: Per RB/RBG SGCS comparison without noise.

|  | Per RB SGCS | Per RBG SGCS | Per subband SGCS |
| --- | --- | --- | --- |
| e-type 2 config 3 | 0.6661 | 0.7358 | 0.8631 |
| Pure ML Precoder Compression Model | 0.6380 | 0.7027 | 0.8121 |
| SRS sounding with 16 hops | 0.3738 | 0.4027 | 0.4549 |
| SRS sounding with 4 hops | 0.6158 | 0.6501 | 0.7078 |
| Fusion of ML model with SRS sounding with 16 hops | 0.6624 | 0.7193 | 0.8119 |
| Fusion of ML model with SRS sounding with 4 hops | 0.7205 | 0.7535 | 0.8102 |

Table 3: Per RB/RBG SGCS comparison with noise impact.

|  | Per RB SGCS | Per RBG SGCS | Per subband SGCS |
| --- | --- | --- | --- |
| e-type 2 config 3 | 0.6003 | 0.7167 | 0.8354 |
| Pure ML Precoder Compression Model | 0.6349 | 0.6990 | 0.8064 |
| SRS sounding with 16 hops | 0.3403 | 0.3912 | 0.4510 |
| SRS sounding with 4 hops | 0.4557 | 0.5799 | 0.6809 |
| Fusion of ML model with SRS sounding with 16 hops | 0.6587 | 0.7148 | 0.8082 |
| Fusion of ML model with SRS sounding with 4 hops | 0.7015 | 0.7343 | 0.7958 |

As shown in Table 2 and Table 3, the per-RB and per-RBG SGCS, used as intermediate metrics, enable a granular evaluation of how effectively the fused estimate tracks the true channel eigenvectors across the frequency domain, capturing both fine-grained multipath components and the trade-off between rapid temporal variations and noise.

Proposal 1: For CSI feedback and SRS fusion, reuse the MIMO evaluation methodology as a starting point. Use the per-RB SGCS and/or per-RBG SGCS, instead of per-sub band SGCS as the intermediate KPI.

Different CSI and SRS fusion use cases with UE RF calibration error

Since CSI feedback and received SRS represent different observations of the same underlying physical channel, the exact form of the CSI feedback may vary while still enabling the fusion network to operate effectively. An example is presented in [2] for an FDD system, where SRS transmitted in the uplink (UL) band is leveraged to improve downlink (DL) CSI feedback obtained from CSI-RS measurements in the DL band of the FDD spectrum.

Other variations are also possible. For instance, when the UE is equipped with a large number of DL receive antennas, sounding all antennas using SRS can incur significant UL resource overhead. If only a subset of DL receive antennas is sounded, some DL performance degradation is expected. In this case, as illustrated in Figure 1, SRS can be used to sound a subset of the receive antennas, while CSI feedback is used to convey information for the remaining antennas.

Another example involves the fusion of precoding matrix feedback with received SRS. Alternatively, the UE may feedback a pre-coded channel matrix. Specifically, given the DL channel measurement H=UΣVH, the UE feeds back UHH which corresponds to the precoding matrix with power scaling, as shown in Figure 2. The anti-aliasing concept introduced in [2] can be directly applied to fuse SRS with either the precoding matrix or the pre-coded channel matrix.

Figure 1, Example of SRS fusion from different antennas between CSI feedback and SRS sounding

Figure 2, Example of CSI feedback and SRS fusion where CSI feedback is the precoding matrix, or pre-coded channel matrix

Another example of using SRS fusion to improve downlink (DL) CSI acquisition is in frequency-division duplex (FDD) systems, as shown in [2]. In FDD systems, uplink (UL) and DL channel reciprocity does not hold due to the frequency separation between UL and DL bands. However, it has been shown that large-scale channel parameters, such as path loss and angular characteristics, remain correlated across the two links and can therefore be exploited to enhance DL CSI acquisition.

When the UE transmit (Tx) and receive (Rx) RF paths are not calibrated, channel reciprocity does not hold because the effective uplink and downlink channels include different RF impairments. Although the physical propagation channel is reciprocal in time-division duplex (TDD) systems, the overall baseband-equivalent channel is distorted by the UE’s RF chains, such as mismatched gains, phases, and frequency responses between the Tx and Rx paths. For all SRS and CSI feedback fusion evaluation, it is important to model un-calibrated UE RF impact.

Proposal 2: For CSI feedback and SRS fusion in TDD system, study fusion of different CSI feedback candidate with SRS selection:

- CSI feedback is channel with matching antenna element as SRS sounded antennas

- CSI feedback is channel from different antenna elements compare to SRS sounded antennas.

- CSI feedback is the precoding matrix.

- CSI feedback is the transformed matrix using UE side precoding vector U matrix.

Proposal 3: For CSI feedback and SRS fusion in TDD system, SRS and CSI feedback fusion study should assume un-calibrated UE Tx/Rx chain.

Proposal 4: Study CSI feedback and SRS fusion in FDD system.

# Time/frequency tracking

For 6GR, maintaining accurate time/frequency synchronization is still crucial for a good demodulation performance. From UE’s perspective, the reference signals UE relies on for time/frequency tracking should support diverse mobility scenarios. It may be beneficial for energy saving’s purposes to keep overhead low for such reference signals. The TRS designed in NR has provided a proven framework to achieve those purposes. It is dedicated for time/frequency tracking with a periodicity configurable from 10ms to 80ms. Other alternatives one might come up with is SSB-only tracking or even DMRS-only tracking. The motivation might be to save more energy for transmitting TRS by the NW. However, performance for those alternatives might not be sufficient, which in the end may consume more NW energy, e.g., spend more energy to transmit actual traffic due to poor demodulation performance. Figure 3 compares the throughput of TRS-only tracking and SSB-only tracking at periodicities of 5, 10, 20, 40, 80 and 160 milliseconds. More simulation parameters/values can be found in Table 4 in the appendix.

Figure 3, Throughput comparison, TRS-only tracking VS. SSB-only tracking

From the throughput comparison, it can be observed that TRS-only tracking consistently outperforms SSB-only tracking for all the periodicities. As periodicities increase, SSB-only tracking degrades much faster than TRS-only tracking. For TRS-only tracking, there is almost no performance degradation when periodicity increases from 5ms to 10ms. When periodicity increases from 10ms to 20ms, noticeable performance degradation is observed. When periodicity further increases to >40ms, TRS-only tracking performance is significantly degraded.

Observation 1: TRS-only tracking outperforms SSB-only tracking in throughput comparison for periodicities 5, 10, 20, 40, 80 and 160 milliseconds:

- Performance of TRS-only tracking also degrades significantly for periodicities 40 and above.

Figure 4 compares Doppler estimation performance of TRS-only tracking and SSB-only tracking with 20ms periodicity.

Figure 4, Doppler estimation comparison, TRS-only tracking VS SSB-only tracking

From the comparison of Doppler estimation performance, it can be observed that TRS-only tracking significantly outperforms SSB-only tracking. This is due to that for SSB-only tracking, Doppler estimation can only be performed using measurement from PBCH DMRS within a slot. However, for TRS-only tracking, Doppler estimation can be performed using measurements from TRS available both within a slot and across slots.

Observation 2: TRS-only tracking outperforms SSB-only tracking in Doppler estimation using periodicity 20ms:

- Doppler estimation based on SSB-only tracking has large errors.

Proposal 5: For 6GR, maintain NR TR TRS framework for fine frequency/time tracking for connected mode:

- Further study the periodicity range, TRS pattern considering high speed scenario, high MCS, etc.

# Phase noise tracking

It is well known that phase noise is a characteristic for FR2. For FR3 from 7.0GHz to 24.25GHz which is between FR1 and FR2, whether the phase noise chrematistics are similar to FR1 or FR2 needs further evaluation. In the past, phase noise is modelled based on TR 38.803 [3] for FR2. We probably need to first study whether a new phase noise model is needed for FR3 or we can simply extrapolate the phase noise model of FR2 for FR3. After we have an aligned model of phase noise for FR3, RAN1 can perform link level simulation for high MCS, e.g., 256QAM, to test whether phase noise should be compensated with the assistance of PT-RS. Figure 5 presents our evaluation results comparing BLER performance of MCS 20 without phase noise and with phase noise (without compensation) using an extrapolated phase noise model based on the example model defined in [3].

Figure 5, BLER of MCS 20 for no PN vs. PN without compensation

From the simulation results of BLER performance, it can be observed that there is noticeable performance degradation when phase noise is modelled using the extrapolated model at 7GHz carrier frequency for the considered MCS (256QAM, 0.67 coding rate).

Proposal 6: As a starting point, uses an extrapolated phase noise model from the example model in 3GPP TR 38.803 for 7GHz:

- Study whether additional phase noise model is needed for 7GHz.

Observation 3: With an extrapolated phase noise model for 7GHz, there is noticeable degradation if phase noise is not compensated for MCS 20 (256QAM, 0.67 coding rate).

Proposal 5: For 7GHz, study support of PT-RS:

- FFS whether reuse NR like design or any enhancement is needed.

Proposal 7: For FR2, support PT-RS using NR-like design as a starting point:

- FFS whether enhancement is needed considering overhead, simplification, ICI estimation, etc.

# Conclusion

In this contribution, we discussed our initial thoughts on CSI feedback/SRS fusion, time/frequency tracking and phase noise compensation.  Based on the discussion, we propose:

Proposal 1: For CSI feedback and SRS fusion, reuse the MIMO evaluation methodology as a starting point. Use the per-RB SGCS and/or per-RBG SGCS, instead of per-sub band SGCS as the intermediate KPI.

Proposal 2: For CSI feedback and SRS fusion in TDD system, study fusion of different CSI feedback candidate with SRS selection:

- CSI feedback is channel with matching antenna element as SRS sounded antennas

- CSI feedback is channel from different antenna elements compare to SRS sounded antennas.

- CSI feedback is the precoding matrix.

- CSI feedback is the transformed matrix using UE side precoding vector U matrix.

Proposal 3: For CSI feedback and SRS fusion in TDD system, SRS and CSI feedback fusion study should assume un-calibrated UE Tx/Rx chain.

Proposal 4: Study CSI feedback and SRS fusion in FDD system.

Observation 1: TRS-only tracking outperforms SSB-only tracking in throughput comparison for periodicities 5, 10, 20, 40, 80 and 160 milliseconds:

- Performance of TRS-only tracking also degrades significantly for periodicities 40 and above.

Observation 2: TRS-only tracking outperforms SSB-only tracking in Doppler estimation using periodicity 20ms:

- Doppler estimation based on SSB-only tracking has large errors.

Proposal 5: For 6GR, maintain NR TR TRS framework for fine frequency/time tracking for connected mode:

- Further study the periodicity range, TRS pattern considering high speed scenario, high MCS, etc.

Proposal 6: As a starting point, uses an extrapolated phase noise model from the example model in 3GPP TR 38.803 for 7GHz:

- Study whether additional phase noise model is needed for 7GHz.

Observation 3: With an extrapolated phase noise model for 7GHz, there is noticeable degradation if phase noise is not compensated for MCS 20 (256QAM, 0.67 coding rate).

Proposal 5: For 7GHz, study support of PT-RS:

- FFS whether reuse NR like design or any enhancement is needed.

Proposal 7: For FR2, support PT-RS using NR-like design as a starting point:

- FFS whether enhancement is needed considering overhead, simplification, ICI estimation, etc.

# Reference

- RP-251881, New SID: Study on 6G Radio, Prague, Czech Republic, June 9-13, 2025

- “Physically-inspired deep learning anti-aliasing framework in efficient channel state feedback”, Yu-Chien Lin, Yan Xin, Ta-Sung Lee, Charlie Zhang, Zhi Ding, IEEE Trans. On wireless communications, Vol 24, No 2, Feb 2025.

- 3GPP TR 38.803, V14.4.0

# Appendix

Table 4: Simulation assumptions for TRS

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5GHz |
| Subcarrier Spacing | 30 kHz |
| Simulation Bandwidth | 52 PRBs |
| Number of Tx antennas | 1 |
| Number of layers | 1 |
| MCS | MCS15, Table 2 |
| Channel Model | TDL-A |
| Delay spread | 30ns |
| UE velocity | 3 km/h |
| TRS configuration |  |
| Slots per burst | 2 |
| OFDM symbols | 4, 8 |
| Comb size | 4 |
| Comb offset | 0 |
| Bandwidth | Full bandwidth |
| Periodicities | 5, 10, 20, 40, 80, 160 ms |
| SSB configuration |  |
| SSB bandwidth | 20 PRBs |
| Tracking signal | PBCH-DMRS |
| Periodicities | 5, 10, 20, 40, 80, 160 ms |
| Impairment Configuration |  |
| CFO drift rate | 0.16 ppm |
| TO drift rate | 0.16 ppm |