3GPP TSG RAN WG1 #124                                                                     R1- 2600840

Gothenburg, SE, Feb. 9th – 13th, 2026

Agenda Item:	10.5.5

Source:	Apple Inc.

Title:	On other physical layer signals, channels and procedure

Document for:	Discussion/Decision

# Introduction

This contribution provides our views on other physical layer signals, channels and procedures in 6GR. We specifically review uplink Tx switching in NR and discuss why/how this very important feature needs to be simplified in 6GR.

# Uplink Tx switching

Uplink Tx switching aims to provide a trade-off among different design parameters like, uplink coverage, UL throughput, load balancing, implementation cost, etc. In NR, uplink Tx switching between two uplink carriers was first specified in Rel-16, where the Tx chain would switch from one carrier to the other carrier when uplink switching is triggered. Uplink switch triggering is a grant-based procedure. More precisely, UE keeps current Tx state unless an uplink transmission requires UE to switch to a new Tx state. The uplink transmission that triggers uplink Tx switch may be an uplink grant associated with a DCI, or it may be a configured grant. This grant-based Tx switching, also called memory-based Tx switching, is not implementation friendly as both UE and NW must keep track of current Tx state based on which the next Tx state is determined. This design gets even more complicated when Tx switching is triggered by DCI.

## Dynamic Tx switching

There are some issues with dynamic Tx switching. First, uplink processing timeline at the UE side must be extended to budget for Tx switching. This extension is needed because the exact switching instance is indicated by DCI and before such indication, UE is not supposed to switch. For example, if a DG-PUSCH triggers Tx switching, PUSCH preparation and transmission timeline Tproc,2 should now include not only N2 but also Tswitch. This is shown in Fig. 1. In this figure, UE is scheduled by an UL-DCI to transmit a DG-PUSCH in slot 2n+4 at TDD numerology, which requires UE to switch from FDD to TDD. Here, assuming single PDCCH monitoring occasion per slot, and Tswitch = 140us, the UL DCI triggering the Tx switch cannot be sent earlier than 2 slots before DG-PUSCH starts. This is because although N2 is like 12 symbols at 30kHz SCS for capability 1, but Tswitch is almost 4 symbols which together with N2 goes beyond one slot.

Figure 1: required timeline to complete dynamic Tx switching vs semi-static Tx switching.

On the other hand, if Tx switch was based on a semi-static pattern, then Tswitch could be excluded from the processing timeline. Going back to the above example, let’s assume in slot 2n+4 which is UL slot in TDD, UE will switch all its Tx chains to TDD carrier, regardless of there is an uplink transmission in TDD or not. As shown in the figure, in this case, UL DCI can schedule a DG-PUSCH in a slot before DG-PUSCH starts.

Dynamic Tx switching may also result in ambiguity between UE and NW, as mentioned before. For instance, if UE is configured with UL skipping, or UL-DMRS in new band is not detected, NW has to go through different hypothesis whether UE has finished UL Tx switching or it has missed the triggering DCI. Based on the above discussion, we have the following proposal for 6GR:

Proposal 1: Support UL Tx switching based on a semi-static pattern in 6GR.

# Conclusion

In this contribution, we provided some views on uplink Tx switching in 6GR. We have the following proposal:

Proposal 1: Support UL Tx switching based on a semi-static pattern in 6GR.