3GPP TSG RAN WG1 #124			R1-2600848

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	10.5.3.1

Source:	Lenovo

Title:	Aspects of downlink-based CSI acquisition

Document for:	Discussion and decision

# Introduction

MIMO is a critical technology for achieving a high-performing physical layer in terms of coverage, reliability and spectral efficiency. Compared to LTE, 5G-NR took MIMO technology into a whole different level, where almost all 5G-NR key technologies are designed considering massive MIMO. In 6GR, MIMO is positioned to play an even bigger role, where FR3 band (7~15 GHz) will also be utilized by 6GR systems, alongside the existing 5G-NR mid-bands. For that, larger MIMO array dimensions (e.g., 4 or 8 times more antenna elements than 5G-NR mid-band) and larger number of CSI-RS ports are essential to compensate for higher pathloss and to achieve comparable coverage to 5G-NR mid-band.

Proposal 1: Support increasing the number of CSI-RS ports to at least 256.

However, although more antenna elements/ports can provide great performance benefits, they also lead to more energy consumption and higher complexity, which require careful considerations. Therefore, 6GR MIMO shall fundamentally integrate energy-saving techniques into MIMO operations from Day 1.

Proposal 2: 6GR MIMO shall fundamentally integrate energy-saving techniques into MIMO operations from Day 1.

On the other hand, MIMO performance strongly depends on the CSI accuracy, where high-precision CSI acquisition with limited overhead should be a key optimization target in 6GR. In 5G-NR MIMO, DL CSI acquisition mainly relies on CSI-RS and CSI feedback framework. In this contribution, we share our view on 6GR CSI-RS design, DL CSI acquisition and report mechanism, and DL CSI repot transmission mechanism considering both non-AI and AI based schemes.

# CSI-RS Design

In 5G-NR, CSI-RSs are not only used for DL CSI acquisition, but also for beam management (BM), interference measurement, mobility, tracking and RRM measurements. 5G-NR CSI-RS framework has flexible configurations in both time and RB-level frequency domains, allowing some-level of trade-off between measurement accuracy and resource efficiency. Similarly, 6GR should strive for a single CSI-RS framework to support diverse functionalities including BM, CSI acquisition, interference measurement, tracking, and RRM measurements.

Proposal 3: In 6GR, RAN1 should aim for a single CSI-RS framework to support diverse functionalities including BM, CSI acquisition, interference measurement, tracking, and RRM measurements.

In 5G-NR, the number of CSI-RS ports in a CSI-RS resource is at most 32, where 2 to 4 CSI-RS resources can be aggregated to get 48, 64, or 128 CSI-RS ports. As the number of CSI-RS ports are envisioned to be increased in 6GR MIMO, e.g., to at least 256 CSI-RS ports, the question is how to define CSI-RS RE patterns for larger CSI-RS ports. One option would be reusing the CSI-RS RE mapping patterns design principle applied in 5G NR, e.g., a CSI-RS resource with 256 ports is obtained by aggregating 8 32-Port CSI-RS resources. However, this approach might unnecessarily complicate the CSI-RS resource configurations and increase the configuration overhead. Another approach is to redesign CSI-RS REs patterns for all possible or envision number CSI-RS ports with overall consideration such that a 256-Port CSI-RS resource is configured in single CSI-RS resource. The latter approach has potential of making the CSI-RS resource configuration simpler with less configuration overhead.

Proposal 4: RAN1 to discuss which design option for CSI-RS RE mapping patterns is supported in 6GR MIMO

- Option 1: Reuse 5G-NR design principle

- Option 2: A new design approach for all possible number CSI-RS ports.

As mentioned above, 6GR MIMO is expected to employ 4x to 8x larger antenna arrays and larger number of CSI-RS ports. While this increase generally improves the beamforming gain, it leads not only to higher energy consumption, but also to higher CSI-RS overhead, which would become a limiting factor that significantly degrades DL performance. In Table 1, we show the CSI-RS overhead ratio defined as the number of REs used by CSI-RS divided by the total number of REs in one period. From the table, we can observe that, for a given periodicity, CSI-RS overhead increases linearly with the increasing number of CSI-RS ports, while, for a given number of CSI-RS ports, CSI-RS overhead increases linearly with the decreasing periodicity of CSI-RS. This shows that with the increasing antenna array size, the importance of CSI-RS overhead reduction increases to maintain scalable and sustainable MIMO performance.

Observation 1: As antenna array size increases, reducing CSI-RS overhead becomes essential to maintain scalable and sustainable MIMO performance.

Table 1: Resource occupation ratio by CSI-RS with different number of CSI-RS ports and CSI-RS periodicity (assuming frequency density equal 1)

| Number of CSI-RS Ports \ CSI-RS periodicity (in slots) | 5 | 10 | 20 | 40 |
| --- | --- | --- | --- | --- |
| 8 | 0.95% | 0.48% | 0.24% | 0.12% |
| 16 | 1.90% | 0.95% | 0.48% | 0.24% |
| 32 | 3.81% | 1.90% | 0.95% | 0.48% |
| 48 | 5.71% | 2.86% | 1.43% | 0.71% |
| 64 | 7.62% | 3.81% | 1.90% | 0.95% |
| 128 | 15.24% | 7.62% | 3.81% | 1.90% |
| 256 | 30.48% | 15.24% | 7.62% | 3.81% |
| 512 | 60.95% | 30.48% | 15.24% | 7.62% |

Therefore, an essential design objective in 6GR MIMO is to strike a good balance between CSI-RS overhead and CSI precision to sustain high MIMO performance while maintaining efficient resource utilization. A straightforward approach to limit/reduce CSI-RS overhead is to reduce CSI-RS density in the frequency domain and/or to increase the CSI-RS time domain periodicity. However, this may lead to CSI estimation accuracy degradation, especially in high channel selectivity and UE mobility. Both these approaches are supported in 5G-NR CSI-RS, where in Rel-20, CSI-RS overhead reduction has been included in the objectives of MIMO evolution, with the main objective of further reducing the CSI-RS frequency domain density, i.e., enabling support for lower frequency density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port. In 6GR, in addition to frequency and time domain density reduction, RAN1 shall study additional schemes to further reduce CSI-RS overhead. The challenge would be maintaining similar or at least comparable CSI accuracy, while pushing further for CSI-RS overhead reduction.

As one approach, CSI measurements and reporting based on PDSCH/PDCCH DMRS can be studied, which may be used to facilitate and complement time-domain based CSI-RS overhead reduction by increasing the time periodicity of CSI-RS transmissions. Moreover, CSI-RS with non-orthogonal CSI-RS ports can also be studied in 6GR to reduce the CSI-RS overhead by exploiting advancements in compressed sensing (CS) based pilot design and channel/CSI estimation, e.g., different sequences can be used in different CSI-RS ports using the same REs. On this latter approach, i.e., non-orthogonal CSI-RS design, some preliminary evaluation results are provided in Figure 1, where the simulation assumptions are provided in Table 2 in the Appendix. In the simulation, {2/4}ports/RE means that the number of CSI-RS ports mapped to the same REs is {2/4} times of that in NR, and for the throughput performance, the saved resources are used for data transmission. From Figure 1, we can observe that non-orthogonal CSI-RS transmission schemes can maintain a comparable CSI accuracy that translates to higher throughput.

Figure 1: Performance comparison on orthogonal CSI-RS ports and non-orthogonal CSI-RS ports

In addition, in 6GR we may study adaptive CSI-RS transmission intervals mechanisms for CSI-RS overhead reduction, e.g., based on the necessity for updating a CSI measurement. For example, when the UE is engaged in active DL channel or signal receptions, the gNB, based on some feedback from the UE, might assess whether the current CSI remains valid or has become outdated. If the CSI remains accurate, the gNB may withhold further CSI-RS transmissions; conversely, if the information has expired, the gNB may initiate a CSI reporting procedure.

In 6GR, therefore, CSI-RS design should study and integrate CSI-RS-overhead reduction schemes into CSI-RS design aiming for enhanced configuration flexibility, where CSI-RS-overhead reduction schemes include RE-level/non-orthogonal CSI-RS design flexibility, in addition to RB-level and time domain flexibilities.

Proposal 5: In 6GR, CSI-RS design should study and integrate CSI-RS-overhead reduction into CSI-RS design aiming for enhanced configuration flexibility.

Proposal 6: 6GR MIMO to consider the following in the CSI-RS overhead reduction study

- DMRS-based CSI measurements and reporting

- Non-orthogonal CSI-RS sequence design

- Dynamical adjustments of CSI-RS transmission intervals based on CSI validity

On the other hand, increasing the number of CSI-RS ports might also increase the PAPR of the DL signal, leading to degradation in the energy efficiency of gNB. In 5G-NR, different CSI-RS ports use the same pseudo-random sequence, and it is left to gNB’s implementation to select a suitable precoder to avoid high PAPR. In 6GR, the CSI-RS sequence design based on the pseudo-random sequence can be taken as a starting point. However, for the large number of CSI-RS ports, it may be difficult to fully depend on implementation. Therefore, CSI-RS sequence design with low PAPR shall be studied in 6GR.

Proposal 7: In 6GR, study low PAPR CSI-RS sequence design.

Regarding the time behavior, periodic, semi-persistent and aperiodic CSI-RS transmissions are supported in 5G-NR, where semi-persistent and periodic CSI-RS mainly differ on the activation mechanism. Periodic CSI-RS transmissions offer essential measurements in the expense of high overhead, while aperiodic CSI-RS has low overhead and offers quick CSI measurements based on the needs. In 6GR, at least periodic and aperiodic CSI-RS shall be supported to provide accurate CSI measurements with manageable overhead.

Proposal 8: In 6GR, at least support periodic and aperiodic CSI-RS transmissions.

# DL CSI Codebook

Since Rel. 15, 5G-NR has repeatedly expanded the DL CSI Type I and Type II codebooks in every release to accommodate an increasing variety of MIMO operation modes. Despite these continuous enhancements, commercial deployments today largely rely on the Rel. 15 Type-I single-panel codebook, highlighting a clear gap between standard complexity and practical adoption. Such incremental CSI design approach places a growing complexity and inconsistency on both standardization and UE implementation, while failing to deliver similar gains in real networks.

Observation 2: 5G-NR approach of iterative enhancement of CSI codebooks has not delivered the intended effectiveness.

In 6GR, RAN1 should avoid repeating this approach. Instead, a unified, flexible, and forward-compatible DL CSI codebook should be defined from Day-1, aiming to enable efficient support of both current and future MIMO schemes, reduce implementation and specification complexities, and eliminate the need for repeated CSI-related UE capability expansions. Moreover, the unified DL CSI codebook should be designed considering trade-off between performance/CSI accuracy, complexity, and reporting overhead. In our view, we believe that Type II codebook family can be used as a starting point, since it has a better common structure and can provide more refined CSI to achieve MU-MIMO gain. Meanwhile, Type I codebook family can be designed/merged based on the common structure. For example, DL CSI codebook can be designed with a common structure as W=W1W2(Wf⨂Wd)H, where W1, Wf,Wd are the basis in spatial domain, frequency domain, and Doppler domain, respectively, and W2 is the linear combination coefficients matrix. With such common codebook structure, different kinds of codebooks can be generated by flexible parameter configuration for spatial basis, frequency basis, Doppler basis, including the number of selected basis vectors.

Proposal 9: In 6GR, a unified, flexible, and forward-compatible DL CSI codebook should be designed from Day-1, aiming to enable efficient support of both current and future MIMO schemes, reduce implementation and specification complexities, better tradeoff between CSI accuracy, complexity, and reporting overhead, and eliminate need for repeated CSI-related UE capability expansions.

# CSI acquisition and reporting

## Non-AI/ML based CSI acquisition and reporting

In 5G-NR, DL CSI reporting framework is highly flexible in terms of measurement resource (e.g., CSI-RS, SSB), report settings/configurations (e.g., multiple (sub)configurations), CSI report time behavior (e.g., AP/P/SP), report activation/triggering mechanisms (e.g., DCI/MAC-CE), report content (e.g., CQI/PMI/RI/CRI, etc.), and report container (i.e., PUSCH/PUCCH). These multiple flexibility options, in fact, made 5G-NR CSI reporting framework unnecessarily more complicated than it should be. In 6GR, RAN1 should strive for a more simplified and unified CSI reporting framework, aiming to support essential features that have better tradeoff between CSI accuracy and reporting overhead. One direction for simplification is to limit CSI reporting container only over PUSCH and to simplify triggering/activation mechanism. Moreover, the CSI reporting framework should support multiple transmission scenarios (e.g., SU-MIMO, MU- MIMO, multi-TRP/panel, CJT, etc.) and transmission hypothesis including those for NES from 6GR Day-1. The actual transmission hypothesis for CSI report may be configured by gNB or selected at UE’s side from some configured candidate hypothesis. To support multiple transmission hypothesis, the measurement resources including channel measurement resources and interference measurement resources can be configured based on each transmission hypothesis. To save overhead, measurement resources may be considered with sharing for some transmission hypothesis.

Proposal 10: In 6GR, RAN1 to study simplified and a unified CSI reporting framework, aiming to support essential features that have better tradeoff between CSI accuracy and reporting overhead.

Proposal 11: RAN1 to consider limiting CSI reporting container to only over PUSCH with simplified triggering/activation mechanism.

### CSI in near field

In 6GR, the number of antenna elements and array size at the base station are expected to grow substantially compared to 56-NR (e.g., 4x or 8x), due to higher demand for capacity, spectrum efficiency, and coverage. However, as the antenna array size increases, the near field region increases as well, wherein near field region is usually defined by the Rayleigh distance, i.e., dRayleigh=2D2λ, where D is the dimension of the antenna array and λ is the wavelength. For many of the scenarios agreed for evaluation, the dRayleigh has become so large relative to the cell radius that it cannot be simply ignored. In Table 3, dRayleigh is calculated for some outdoor and indoor scenarios and shown as a fraction of the cell radius (ISD = 20m for Indoor and 200m for Dense Urban). The near field region grows more prominent with the carrier frequency, especially for 7 GHz (indoor and outdoor) and 30 GHz (indoor), where the Rayleigh distance reaches up to 55% of the cell radius. Consequently, the near-field region may include a large portion of UEs and should be studied, including channel estimation and CSI feedback.

Proposal 12: In 6GR, RAN1 to study the CSI reporting considering near field effect for high frequency bands.

Table 3: Rayleigh distance under multiple scenarios

| fC (GHz) | Scenario | (M, N, P, Mg , Ng; Mp, Np) | Antenna spacing (λ) | dRayleigh (m) | dRayleighrcell=ISD/2 |
| --- | --- | --- | --- | --- | --- |
| 7 | Indoor | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.5) | 5.49 | 55% |
|  |  | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5) | 5.49 | 55% |
|  | Outdoor | (32, 32, 2, 1, 1, 8, 16) | (0.5, 0.5) | 21.94 | 22% |
|  |  | (64, 16, 2, 1, 1; 16, 16) | (0.5, 0.5) | 87.77 | 88% |
| 15 | Outdoor | (64,16,2,1,1,16,16) | (0.5,0.8) | 40.96 | 41% |
|  |  | (16, 8, 2, 4, 2; 1, 1) | (0.5, 0.8) | 40.96 | 41% |
| 30 | Indoor | (16, 8, 2, 2, 2; 1, 1) | (0.5, 0.5) | 5.12 | 51% |
|  | Outdoor | (16, 8, 2, 4, 4; 1, 1) | (0.5, 0.5) | 20.48 | 20% |

### Cell-free MIMO

User centric cell-free network architecture defines a cooperative serving cluster of spatially distributed transmitters specifically for each user. It is a practical method to deploy cell-free communications together with MIMO, termed MIMO (CF-MIMO), on high-capacity scenarios such as hotspot, campus, and factory. Figure 2 shows an example of a CF-MIMO system, where multiple APs connected with a centric processor unit (CPU) for joint processing and each AP has a small number of antennas, e.g., up to 4. In practice, each UE is connected and served by a cluster of APs to account for power efficiency and joint process capability. For example, Cluster 2 for UE 2 consists of AP1, AP2 and AP3, which make joint transmission/reception for UE2. CF-MIMO brings several benefits, including improved cell-edge UE throughput since it can increase the received signal quality and LoS probability by reducing the minimum distance between UE and serving APs, higher diversity due to distinct channel conditions from multiple serving APs, inter-cell interference suppression/reduction by joint coordination between APs within one cluster; and reduced handover and interruption time.

For CF-MIMO system, joint transmission can be made based on UE centric cluster, where the potential AP number for CJT transmission may be large, e.g. more than 4, and the cluster size may be smaller. To provide time/frequency/phase synchronization between APs in the cluster and guarantee CJT performance, calibration between APs in the cluster is needed. In 5G system, CJT based calibration report is specified in Rel. 19, where the report content may include delay offset (i.e. DO), frequency offset (i.e. FO), phase offset (i.e. PO) or combination of two types, such as DO+FO. For 6GR, similar functionality is needed. However, more study can be made for calibration report to match with UE specific cluster based CJT transmission in case of more than 4 potential AP number for CJT and/or denser cluster with smaller region size.

Proposal 13: Study calibration report for clustered based CJT transmission.

Figure 2: Example of a cell free massive MIMO system with UE specific cluster

### CSI reporting with large BW

In 5G-NR, UE can support maximum 100MHz bandwidth. In 6GR, larger bandwidth may be used, where 400MHz maximum channel bandwidth is assumed at network side in case of 30kHz SCS around 7GHz, and maximum channel bandwidth of 800MHz or 400MHz is assumed at network side for FR2-1. For UE with different capability, it may support different maximum channel bandwidth. For maximum bandwidth, e.g. 400M for 7GHz, different realization may be used where one or multiple of baseband processors, FFTs, RF chains may be used and related details can refer to the 5 options listed in the agreement of RAN1#123.

With increasing bandwidth, the RB number and CSI reporting subband number are also increased. For example, with 400MHz, there are around 1100 RBs and 69 CSI subbands (assuming 16 RBs in one CSI subband), which is around 4 times larger than 5G-NR system with 100MHz and 30K SCS. If full CSI for all subbands is reported, the CSI overhead is increased remarkably, e.g. around 4X overhead relative to 5G CSI with 100MHz and 30KHz. Thus, CSI overhead reduction can be considered for CSI reporting schemes, especially for UE with smaller data overhead for transmission. Moreover, different UEs may have different capability (e.g. 1 or 2 RF, 1 or 2 FFT, 1 or 2 baseband processor) to support large bandwidth. For example, some UEs can only support 100M bandwidth with 1 RF, 1 FFT and 1 baseband processor. Moreover, large bandwidth is supported by aggregating multiple 100M bandwidth with using more RF, FFT and baseband processor. Some UEs can support larger bandwidth with 1 RF and 1 baseband processor. The CSI report, including the configuration e.g. CSI-reportingband, subbandSize, cqi-FormatIndicator, cqi-FormatIndicator, may be designed with matching UE capability for supporting large bandwidth.

- Proposal 14: Study CSI report for large bandwidth, e.g. 400MHz at 7GHz.

### CSI reporting and PMI selection

In 5G-NR, PMI selection primarily involves evaluating candidate precoding matrices based on a configured codebook and choosing a PMI that optimizes the link performance. This can be accomplished by computing metrics such as the highest achievable MCS, post-precoding SINR, or mutual information. These approaches are conceptually aligned with SVD-based PMI selection, wherein the eigenvectors corresponding to the largest eigenvalues are reported as PMIs. By transmitting along these directions, the gNB can ensure that multiple layers remain largely interference-free at the UE while utilizing the most efficient channel directions. However, this benefit relies on accurate channel estimation and equalization at the UE, which otherwise may necessitate higher DMRS overhead. The necessity for improved channel estimation and reduced inter-layer interference become increasingly significant with the anticipated expansion in supported layers for future 6GR systems. Based on these considerations, it is recommended to further investigate an adaptive PMI selection criterion based on a desired performance goal, e.g., maximizing throughput, minimizing interference, etc., while considering low-density DMRS patterns or DMRS-less transmission scenarios.

Proposal 15: Study mechanisms for supporting PMI selection that mitigate inter-layer interference in low-density DMRS and DMRS-free transmission scenarios.

### CSI for joint DL and UL

CSI acquisition and report for DL and UL may be jointly designed to reduce CSI overhead or improve CSI feedback accuracy. For DL CSI acquisition of FDD system, the general method is that UE measures the downlink reference signal (e.g. CSI-RS), and feeds back the measurement results (e.g. RI/PMI/CQI) to BS. However, the CSI feedback overhead is larger to provide accurate CSI, where elaborate design for multiple kinds of codebooks are needed to provide tradeoff between feedback overhead, CSI accuracy and UE realization complexity. Moreover, the CSI-RS overhead is increasing too much with increasing port numbers, e.g. 128/256 ports. From another view, SRS can be transmitted for uplink CSI measurement at gNb’s side. Based on SRS measurement, some downlink CSI in angular and delay domain can be obtained when there exists partial reciprocity in FDD system. To determine DL CSI by exploiting partial reciprocity, beamformed CSI-RS can be firstly transmitted, where beamforming can be determined based on SRS measurement. Then, the DL CSI can be reported with lower overhead by exploiting the partial reciprocity, where special codebook for partial reciprocity channel may be required, e.g. feType2 port selection codebook. As for the details of report schemes, it can be further studied.

For DL CSI acquisition of TDD system, the general method is that BS determines the downlink CSI based on uplink transmission (e.g. SRS) through the DL-UL reciprocity. However, beamforming may not be used during SRS receiving and SRS is power limited for cell edge area, especially for 7GHz. Thus, the performance of channel estimation may not be good and CSI at gNB’s side is not accurate. This will degrade the system’s performance. Moreover, the reciprocity property may not be accurate when different paths are used for DL and UL transmission because of UE’s movement. To solve the mentioned issues, CSI-RS can be transmitted to assist to obtain DL CSI when UE is located at the cell edge region or channel reciprocity is not accurate. With the reported CSI, gNB can receive SRS with the desirable beam and this will further improve UL CSI accuracy. As for the details of report schemes, it can be further studied.

Proposal 16: Study enhanced CSI acquisition and report mechanism for joint downlink and uplink based on reciprocity property.

### CSI feedback and energy saving

Energy efficiency will be an important performance target in 6GR, considering the increase in the number of antenna elements/ports and RF chains at the base station. One means of saving energy is via spatial-domain adaptation of the transmission at the network side, i.e., the network activates only a subset of its antennas/ports based on an energy efficient metric. In NR Rel. 18, spatial domain adaptation techniques for downlink transmission have been specified, where the UE is configured with multiple spatial-domain adaptation patterns, where each pattern is configured separately. CSI measurement and reporting for each adaptation pattern is then pursued separately. While this approach enables achieving some energy savings at the network side, it moves the burden to the UE side which needs to compute CSI for multiple adaptation patterns, thus increasing computational complexity and feedback overhead. In 6GR, methods should be studied to reduce this complexity and overhead while still enabling the network to maximize energy efficiency with multiple CSI options.

Proposal 17: Study network-side energy saving with multiple CSI reports, while minimizing the induced UE-side complexity and feedback overhead.

## AI/ML based CSI acquisition and report

AI/ML enabled CSI feedback including temporal domain CSI prediction based on UE-sided model and CSI compression based on two-sided model were comprehensively evaluated in 5G-NR. The evaluation results demonstrate that employing AI/ML in wireless air interface is feasible and, in certain scenarios, can achieve better system performance compared to non-AI/ML counterpart solutions. Given that, AI/ML based CSI prediction was specified in 5G-NR Rel. 19 by reusing non-AI/ML based Doppler codebook. CSI feedback with compression by using two-sided model is being specified in 5G-NR Rel. 20. The normative work includes data collection for training, inference, and performance monitoring. For two-sided model, inter-vendor collaborative is also encompassed.

According to the IMT-2030(6G) vision recommendation defined by ITU-R, intelligent AI/ML, compute & communication is one of the 6GR usage scenario. AI/ML based solutions should be supported in 6GR Day 1. In this section, we provided our views and preliminary evaluation results for some AI/ML based CSI report schemes.

### CSI prediction in frequency and/or spatial domain with lower overhead

Thanks to the powerful reasoning and prediction capabilities of AI/ML, CSI-RS overhead can be significantly reduced compared to conventional non-AI/ML based CSI acquisition. For instance, the CSI-RS overhead can be reduced in the spatial, frequency, and/or time domains, while the CSI corresponding to resources without CSI-RS mapping are obtained by using AI/ML inference. In this section, we consider three sub-use cases for low overhead CSI-RS design with AI/ML inference

- CSI-RS Case#1: Low CSI-RS overhead in frequency domain (FD). CSI-RS is transmitted only on part of the RBs in the sounding bandwidth, and the AI/ML model is used to predict channel information across the whole sounding bandwidth.

- CSI-RS Case#2: Low CSI-RS overhead in spatial domain (SD): CSI-RS is transmitted over part of the antenna ports for CSI reporting, and the AI/ML model is used to predict channel information over all the antenna ports for CSI reporting.

- CSI-RS Case#3: Low CSI-RS overhead in both FD and SD: CSI-RS is transmitted over part of the antenna ports and only on part of sounding bandwidth, and the AI/ML model is used to predict channel information over all the antenna ports in the whole sounding bandwidth.

#### Evaluation results

Simulation assumptions for data collection and AI/ML model training are shown in Table 4. We consider system level simulation with UMa channel model for obtaining the downlink channel dataset for model training. 57k training samples and 14.4k testing samples are derived from different UEs over a duration in the system level simulation. SGCS between the eigenvectors of label channel matrix and the eigenvectors of predicted channel matrix is used as the performance metric.

Table 4: Simulation assumptions for data collection and AI/ML model training for lower overhead CSI-RS

| Parameter | Value |
| --- | --- |
| Channel modelling | UMa |
| Bandwidth | 52 RBs |
| Antenna configuration | 32 Tx port: (M, N, P, Mp, Np) = (8, 8, 2, 2, 8) 128 Tx port: (M, N, P, Mp, Np) = (16, 16, 2, 4, 16) 4 Rx port: (M, N, P, Mp, Np) = (2, 1, 2, 2, 1) |
| Training samples | 57k |
| Testing samples | 14.4k |
| Performance metric | SGCS |

The acquisition of model input data for the training data set involves two distinct strategies, each with its own implementation tradeoffs:

- Down-sampling of full measurements: This method first obtains the full-dimension (all ports and all RBs) CSI measurements, which are also used to generate the ground truth. A down-sampling process is then applied in the spatial and/or frequency domain to create the corresponding partial-observation input data.

- Measurement of independent CSI-RS resources: In this more systematic approach, dedicated and sparser CSI-RS resources are configured and transmitted. The UE directly measures these sparse CSI-RS which combines with measurement of a full density and full port CSI-RS resource for a training data sample. The CSI-RS for input-data acquisition is not restricted by the CSI-RS for label acquisition. This facilitates vendors developing diverse model, e.g., different antenna virtualization across CSI-RS for input and CSI-RS for output. This makes it the preferred and more flexible solution for commercial deployment.

For CSI-RS Case#1 (FD only), the number of CSI-RS ports is set to 32 Tx ports. The RBs with CSI-RS are evenly selected over the sounding bandwidth. In details, we consider the following cases:

- Case 1: Only 1/2 RBs has CSI-RS (i.e., every 2 RBs has a CSI-RS mapping with 32 ports). Equivalent to a 1/2=50% overhead reduction.

- Case 2: only 1/4 RBs has CSI-RS (i.e., every 4 RBs has a CSI-RS mapping with 32 ports). Equivalent to a 3/4=75% overhead reduction.

- Case 3: only 1/8 RBs has CSI-RS (i.e., every 8 RBs has a CSI-RS mapping with 32 ports). Equivalent to a 45/52=87% overhead reduction.

- Case 4: only 1/15 RBs has CSI-RS (i.e., every 15 RBs has a CSI-RS mapping with 32 ports). Equivalent to a 48/52=92% overhead reduction.

The SGCG performance is shown in Figure 3. With 50% RB reduction, there is almost no loss of channel information, i.e., 0.99 SGCS for both layer 1 and layer 2. Even with 90.3% RB reduction, i.e., reducing 48 RBs of the 52 RBs, AI/ML-based scheme can still achieve 0.94 SGCS for layer 1 and 0.91 SGCS for layer 2.

- Observation 3: A 90% CSI-RS overhead reduction in FD can still achieve SGCS performance of at least 0.9.

Figure 3:  SGCS performance for CSI-RS Case#1.

For CSI-RS Case#2 (SD only), 128 Tx ports and 32 Tx ports are considered as prediction target in two respective cases. Only part of the Tx ports is selected for actual channel measurement (i.e., with actual CSI-RS transmission). To provide diverse channel information for the AI/ML model, the selected Tx ports cover two polarization directions and as much as antenna space position. The illustration of port selection pattern for 32 Tx ports case is shown in Figure 4.

Figure 4: Iillustration of selected antenna port pattens for 32 Tx port case.

Simulation results for the 32 Tx ports and 128 Tx port with three Tx port reduction cases are shown in Figure 5. Overall, for all three Tx port reduction scheme, the desired channel estimation accuracy can be achieved with significant CSI-RS overhead reduction. With the continued reduction of CSI-RS overhead, the SGCS performance achieved by AI/ML-based scheme decreased slightly. In detail, for both 32 Tx ports and 128 Tx ports, the AI/ML based scheme can achieve larger than 0.91 SGCS with 50% CSI-RS overhead reduction for all the layers. With further reduce the CSI-RS port, the SGCS is slightly reduced but is still larger than 0.81 SGCS with 87.5% Tx port reduction for both layer 1 and layer 2.

- Observation 4: A CSI-RS overhead reduction of 87% in spatial-domain achieves SGCS performance of at least 0.8.

Figure 5: SGCS performance over spatial domain CSI-RS overhead reduction.

For CSI-RS Case#3 (joint FD and SD), the CSI-RS overhead is reduced in both frequency and spatial domain. The schemes for reducing overhead in the frequency and spatial domains follow Case 1 and Case 2, respectively. In the evaluation, we only consider the 32Tx ports case. The SGCS performance is provided in Figure 6. It can be observed that AI/ML based scheme can achieve larger than 0.98 SGCS with combination of 1/2 RB reduction and 1/2 Tx port reduction, i.e., 3/4 CSI-RS overhead reduction in total, for both layer 1 and layer 2. With combination of 45/52 RB reduction and 7/8 Tx port reduction, i.e., 98.3% CSI-RS overhead reduction in total, 0.81 SGCS and 0.75 SGCS can still be obtained for layer 1 and layer 2, respectively.

Figure 6: SGCS performance over joint frequency domain and spatial domain CSI-RS overhead reduction.

- Observation 5: A CSI-RS overhead reduction of 98% in both FD and SD achieves a SGCS of at least 0.75.

In this contribution, we only provide the preliminary simulation results on SGCG. These simulation results indicate that spatial- and frequency-domain CSI prediction is feasible, and the resulting reduction in CSI-RS overhead is significant. The system-level performance, such as throughput and generalization capability, will be provided in a future contribution.

- Proposal 18: Study and evaluate separate and joint FD and SD CSI prediction with UE-sided AI model in 6GR.

### CSI compression and feedback with AI/ML

CSI compression based on two-sided models have been studied and evaluated in 5G-NR over two releases and is under standardization in Rel. 20. Looking forward to CSI compression in 6GR, some potential use cases have been identified as shown in Table 5

Table 5: CSI compression and feedback with AI

| Sub-use case | Sub-case A: CSI compression with JSCC | Sub-case B: CSI compression with JSCM | Sub-case C: CSI feedback with DLable codebook | Sub-case D: Explicit CSI feedback based on projection matrix |
| --- | --- | --- | --- | --- |
| Model input of decoder (or model output of encoder) | UE-part output: Compressed CSI bits NW-part input: Demodulated LLRs after equalization and de-mapper. | UE-part output Compressed CSI complex values NW-part input: Received compressed CSI complex values after equalization | UE-part output: Compressed CSI bits via a downloaded codebook NW-part input: Amplitudes and phases obtained by a look up table based on feedback CSI bits and the selected basis vectors | UE-part output: Compressed CSI complex values via a projection matrix NW-part input: Received compressed CSI complex values after equalization |
| Model output of decoder (or model input of encoder) | NW-part output: Reconstructed precoding matrix UE-part input: Precoding matrix (eigenvectors) | NW-part output: Reconstructed precoding matrix UE-part input: Precoding matrix (eigenvectors) | NW-part output: Reconstructed precoding matrix UE-part input: Precoding matrix (eigenvectors) | NW-part output: Reconstructed explicit channel matrix UE-part input: Explicit channel matrix |
| Label | Eigenvectors | Eigenvectors | Eigenvectors | Explicit channel matrix |
| Training types | Offline training | Offline training | Offline training | Offline training |
| KPI | SGCS | SGCS | SGCS | SGCS |
| Benchmark | - eType II - NR Rel-19 two sided model based CSI compression with polar coding | - eType II - NR Rel-19 two sided model based CSI compression with polar coding | - eType II - NR Rel-19 two sided model based CSI compression with polar coding | - eType II - NR Rel-19 two sided model based CSI compression with polar coding |
| Model location for inference | Two-sided model | Two-sided model | NW-sided model | NW-sided model |

Detailed descriptions of different use cases are provided below.

- Sub-case A: CSI compression with JSCC: The JSCC encoder part includes an encoder and quantizer. The precoding matrix is first compressed by the encoder, then quantized into transmit bitstream. The output bitstream is directly mapped to constellation points for resource mapping and OFDM modulation. The receiver obtains LLR values from the demodulator after channel equalization, which are then processed by a JSCC decoder to reconstruct CSI data, e.g., precoding matrix.

- Sub-case B: CSI compression with JSCM: The JSCM framework integrates modulation functionality into the encoder, further reducing quantization errors compared with JSCC. CSI data (e.g., precoding matrix) is first compressed by the JSCM encoder, which outputs complex-valued symbols directly. The symbols directly mapping to the subcarriers, followed by the same subsequent chain of signal transmission. Layer mapping and precoding may be further applied when in case of MIMO scheme is used for PUSCH transmission. The receiver obtains complex symbols from the equalizer, which are then processed by a JSCM decoder to reconstruct the original CSI data.

- Sub-case C: CSI feedback with DLable (Downloadable) codebook: In this use case, the UE obtains a codebook including a set of basis vectors dii=0LCB-1 for the UE for the CSI quantization. Each of the basis vector has a demission PN3×1, i.e., cover all the antenna ports P across all N3 subbands. The precoding matrix corresponding to all the SBs is W=w0,w1,…,wN3-1T. Each layer of the precoding matrix is projected directly by the UE into the codebook matrix space. Then the UE selects L out of the LCB basis vectors with strongest coefficients to construct the precoding vector of w based on linear combination approximating. The reported information includes the information of the selected  L basis vectors and the corresponding coefficients. The codebook basis is trained by AI method and the NW may deploy a AI model to reconstruct each of the precoding vectors.

- Sub-case D: Explicit CSI feedback based on projection matrix: For this case, a projection matrix A, is provided for the UE for explicit channel matrix feedback. The UE performs Y=AH, where H=h0, h1,…,hNSB-1T.  hn is the estimated DL channel matrix for the n-th subband and has a dimension of NTx×NRx. NTx is the number of transmit antenna ports at the NW side and NRx is the number of receive antenna ports at the UE side. The elements of Y are directly mapped onto PUSCH layers. Other necessary steps including precoding, resource mapping and OFDM modulation can proceed with the routine PUSCH transmission processing. An AI model is adopted at the NW side, the received CSI information after channel equalizer is input to the AI model at the NW side to recover the expected channel matrix.

#### Evaluation results

For the above use case, a preliminary performance evaluation was conducted, with simulation assumption provided in Table 6.

Table 6: Simulation assumptions for AI based CSI feedback

| Parameter | Value |
| --- | --- |
| Waveform | CP-OFDM for both DL and UL |
| DL channel for CSI acquisition | Uma, 32T4R |
| DL CSI-RS bandwidth | 52RBs/13Subbands |
| UL bandwidth for CSI feedback | 1T32R AWGN/CDL-C |
| FFT size | 512 |
| Resource for PUSCH | 4 RBs and 2 Symbols |
| Modulation for PUSCH | QPSK |
| eType II CSI bits | 67/92/120/174/231/250/285/335bis |
| Channel coding for non-AI CSI feedback | Polar with code rate 0.25/0.5 |
| JSCC encoder output | 128bis |
| JSCM encoder output | 64 complex number |
| DLable CB parameter | 832×80 |
| DLable CB output | 10/20/40/60/80 |
| Projection based feedback | 64 complex number |

For Sub-case A/B, Figure 7 presents SGCS performance comparison between JSCC/JSCM based CSI feedback schemes and Rel-19 AI CSI compression scheme with NR Rel-15 PUCCH scheme.  It can be observed that AI-based joint source and channel coding scheme demonstrates significant advantages in low-SNR conditions where traditional polar code exhibit cliff effects, integrating the crucial capabilities of CSI compression and dynamic channel adaptation within a single, efficient framework. Furthermore, JSCM scheme can further enhance performance by effectively eliminating quantization errors.

Figure 7. Performance of JSCC/JSCM for CSI feedback

For Sub-case C, Figure 8 presents SGCS performance comparison between DLable codebbok and eType II based CSI feedback and JSCC/JSCM based CSI feedback schemes. It can be observed that the SGCS performance of DLable codebook closely approximates eType II with similar CSI feedback overhead, while significantly reducing computational requirements on UE. Furthermore, the AI-based approaches have considerable potential for further performance enhancement.

Figure 8. Performance of DLable codebook for CSI feedback.

For Sub-case D, Figure 9 presents SGCS performance comparison between explicit CSI feedback based on projection matrix. It can be observed that the performance of CSI with projection matrix slightly underperforms the two-sided JSCM, but it still significantly outperforms both JSCC and the CSI feedback with polar code. Compared to the two-sided JSCM, this method reduces the complexity associated with the two-sided collaborative model training, eliminate the need for deploying complex models on the UE side, making it more practical for explicit channel feedback.

Figure 9. Performance of explicit CSl feedback based on projection matrix.

Given the above observations, we propose that all those schemes should be further study and evaluations in 6GR by considering the performance gain and the UE-side complexity.

- Proposal 19: Study and evaluate the following AI/ML enable CSI feedback schemes in 6GR:

- Sub-case A: CSI compression with JSCC

- Sub-case B: CSI compression with JSCM

- Sub-case C: CSI feedback with DLable codebook

- Sub-case D: Explicit CSI feedback based on projection matrix

# Conclusion

In this contribution, we have the following observations and proposals for 6GR studies.

Observation 1: As antenna array size increases, reducing CSI-RS overhead becomes essential to maintain scalable and sustainable MIMO performance.

Observation 2: 5G-NR approach of iterative enhancement of CSI codebooks has not delivered the intended effectiveness.

- Observation 3: A 90% CSI-RS overhead reduction in FD can still achieve SGCS performance of at least 0.9.

- Observation 4: A CSI-RS overhead reduction of 87% in SD achieves SGCS performance of at least 0.8.

- Observation 5: A CSI-RS overhead reduction of 98% in both FD and SD achieves a SGCS of at least 0.75.

Proposal 1: Support increasing the number of CSI-RS ports to at least 256.

Proposal 2: 6GR MIMO shall fundamentally integrate energy-saving techniques into MIMO operations from Day 1.

Proposal 3: In 6GR, RAN1 should aim for a single CSI-RS framework to support diverse functionalities including BM, CSI acquisition, interference measurement, tracking, and RRM measurements.

Proposal 4: RAN1 to discuss which design option for CSI-RS RE mapping patterns is supported in 6GR MIMO

- Option 1: Reuse 5G-NR design principle

- Option 2: A new design approach for all possible number CSI-RS ports.

Proposal 5: In 6GR, CSI-RS design should study and integrate CSI-RS-overhead reduction into CSI-RS design aiming for enhanced configuration flexibility.

Proposal 6: 6GR MIMO to consider the following in the CSI-RS overhead reduction study

- DMRS-based CSI measurements and reporting

- Non-orthogonal CSI-RS sequence design

- Dynamical adjustments of CSI-RS transmission intervals based on CSI validity

Proposal 7: In 6GR, study low PAPR CSI-RS sequence design.

Proposal 8: In 6GR, at least support periodic and aperiodic CSI-RS transmissions.

Proposal 9: In 6GR, a unified, flexible, and forward-compatible DL CSI codebook should be designed from Day-1, aiming to enable efficient support of both current and future MIMO schemes, reduce implementation and specification complexities, better tradeoff between CSI accuracy, complexity, and reporting overhead, and eliminate need for repeated CSI-related UE capability expansions.

Proposal 10: In 6GR, RAN1 to study simplified and a unified CSI reporting framework, aiming to support essential features that have better tradeoff between CSI accuracy and reporting overhead.

Proposal 11: RAN1 to consider limiting CSI reporting container to only over PUSCH with simplified triggering/activation mechanism.

Proposal 12: In 6GR, RAN1 to study the CSI reporting considering near field effect for high frequency bands.

Proposal 13: Study calibration report for clustered based CJT transmission.

Proposal 14: Study CSI report for large bandwidth, e.g. 400MHz at 7GHz.

Proposal 15: Study mechanisms for supporting PMI selection that mitigate inter-layer interference in low-density DMRS and DMRS-free transmission scenarios.

Proposal 16: Study enhanced CSI acquisition and report mechanism for joint downlink and uplink based on reciprocity property.

Proposal 17: Study network-side energy saving with multiple CSI reports, while minimizing the induced UE-side complexity and feedback overhead.

Proposal 18: Study and evaluate separate and joint FD and SD CSI prediction with UE-sided AI model in 6GR.

- Proposal 19: Study and evaluate the following AI/ML enable CSI feedback schemes in 6GR:

- Sub-case A: CSI compression with JSCC

- Sub-case B: CSI compression with JSCM

- Sub-case C: CSI feedback with DLable codebook

- Sub-case D: Explicit CSI feedback based on projection matrix

# Appendix

Table 2: LLS evaluation assumptions for non-orthogonal CSI-RS ports

| Parameters | Assumptions |
| --- | --- |
| PDSCH Bandwidth | 52RB |
| Subcarrier spacing | 15kHz |
| Carrier frequency | 2GHz |
| MCS | QPSK, coding rate=120/1024 |
| Antenna config. | 32x4 |
| rank | 1 |
| precoder | Feedback |
| Channel model | CDL-C |
| Channel estimation | 2D MMSE |
| Delay spread | 100ns |
| Velocity | 3km/h |
| PDSCH transmission | 14 OFDM symbols with 2 OFDM symbols for DMRS |