3GPP TSG RAN WG1 #124	R1-2600869

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda item:	10.5.0

Source: 	Fujitsu

Title:	Discussion on 6GR duplex types and frame structure

Document for:		Discussion and decision

# Introduction

This contribution provides our views on 6GR duplex types and frame structure.

# Discussion

For 6GR duplex types, the following agreement was reached in RAN1#122 meeting.

Agreement - Study and identify the lessons learned from NR duplex modes - On 6GR duplexing study, RAN1 considers at least following duplex types - FD-FDD - Semi-static TDD - gNB semi-static SBFD - HD-FDD on UE side - Dynamic TDD - Study whether to consider following duplexing types - gNB dynamic SBFD - UE SBFD - gNB FD - Note: Other duplex modes are not precluded

Among the duplex types listed in the agreement, FD-FDD, semi-static TDD and dynamic TDD are supported in the first release of NR, and gNB semi-static SBFD and HD-FDD on UE side are introduced in later releases.

In general, we think 6GR should at least support the duplex types which have been supported in NR, with consideration on some lessons learned from NR. For example:

- For dynamic TDD in NR, the rules of link direction determination are complicated largely because separate rules need to be defined for various cases caused by support of dynamic SFI. On the other hand, the benefit of dynamic SFI is not clear given that the dynamic link direction can already be supported by DCI-based scheduling. With that said, it is not preferred to support dynamic SFI for 6GR.

- In NR, for some duplex modes introduced after the first release, the design is complicated due to constraints of backward compatibility. For example, for gNB semi-static SBFD in NR, it is very challenging to deal with the relation between new symbol types with legacy symbol types, esp. flexible symbols, because they are overlapped in time domain. For 6GR, it is preferable to support it in the first release with a simple unified design.

- Similar to NR, 6GR may also introduce new duplex modes (e.g. gNB dynamic SBFD, UE SBFD etc.) after the first release. Good forward compatibility is expected.

Proposal 1: For 6GR, it is recommended to support the following duplex types in the first release while maintaining a forward-compatible design to accommodate new duplex types in future releases.

- FD-FDD

- Semi-static TDD

- gNB semi-static SBFD

- HD-FDD on UE side

- Dynamic TDD without dynamic SFI

For 6GR frame structure, the following agreement was reached in RAN1#122bis meeting.

Agreement For communication, 6GR considers NR frame structure used as a starting point for the study item, - Resource defined by one subcarrier and one symbol is called as resource element (RE). - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12 - Radio Frame length is 10ms - Each radio frame is split into 10 subframes, each with a duration of 1 ms - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design. - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings.

To support the multiple duplex types, MRSS, and multiplexing between communication and sensing, and for the sake of forward compatibility, the following symbol types should be supported in the first release of 6GR.

- Downlink symbols: Only downlink signals/channels are allowed.

- Uplink symbols: Only uplink signals/channels are allowed.

- Sub-band Full-Duplex (SBFD) symbols: Simultaneous downlink and uplink signals/channels in non-overlapping sub-bands are allowed.

- Flexible symbols: Both downlink and uplink signals/channels are allowed, but only downlink or uplink signals/channels can be actually received or transmitted in a given symbol.

- Reserved symbols: Both downlink and uplink signals/channels are not allowed.

Proposal 2: For 6GR, it is recommended to support the following symbol types in the first release.

- Downlink symbols: Only downlink signals/channels are allowed.

- Uplink symbols: Only uplink signals/channels are allowed.

- Sub-band Full-Duplex (SBFD) symbols: Simultaneous downlink and uplink signals/channels in non-overlapping sub-bands are allowed.

- Flexible symbols: Both downlink and uplink signals/channels are allowed, but only downlink or uplink signals/channels can be actually received or transmitted in the same symbol.

- Reserved symbols: Both downlink and uplink signals/channels are not allowed.

# Conclusion

According to the discussions above, we have the following proposals.

Proposal 1: For 6GR, it is recommended to support the following duplex types in the first release while maintaining a forward-compatible design to accommodate new duplex types in future releases.

- FD-FDD

- Semi-static TDD

- gNB semi-static SBFD

- HD-FDD on UE side

- Dynamic TDD without dynamic SFI

Proposal 2: For 6GR, it is recommended to support the following symbol types in the first release.

- Downlink symbols: Only downlink signals/channels are allowed.

- Uplink symbols: Only uplink signals/channels are allowed.

- Sub-band Full-Duplex (SBFD) symbols: Simultaneous downlink and uplink signals/channels in non-overlapping sub-bands are allowed.

- Flexible symbols: Both downlink and uplink signals/channels are allowed, but only downlink or uplink signals/channels can be actually received or transmitted in the same symbol.

- Reserved symbols: Both downlink and uplink signals/channels are not allowed.