3GPP TSG-RAN WG1 #124	R1-2600934

Gothenburg, Sweden, February 9th - 13th, 2026

Agenda Item:	10.5.2.2

Source:	Panasonic

Title:	Downlink transmission scheme(s) for downlink shared channels

Document for:	Discussion

# Introduction

In RAN plenary #108, a study item description (SID) on 6G radio (6GR) with the following objectives for physical layer structure [1].

Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1

Based on that, chairman provides the following guidance to discuss some aspects for downlink transmission scheme(s) for downlink shared channels in the following.

Note 1: Including downlink transmission scheme for downlink channel, e.g., PDSCH, associated downlink DMRS design (if any), scrambling sequence, mechanism for layer mapping and modulation may consider different evaluation assumptions, as well as mechanism for supporting MRSS.

For DL shared channels, 5G NR specified many aspects (e.g., scrambling sequence, modulation, VRB-to-PRB mapping, TDRA, FDRA, etc.), wherein some of them can be applicable to 6G. Hence, it is reasonable to study to reuse 5G NR aspects as much as possible to save discussion effort. On the other hands, we would like to discuss potentially different aspects for 6G in the following.

# Discussion

## DL transmission scheme for DL shared channel

In 5G NR, for single TRP operation, a single transmission scheme using DMRS is defined for the PDSCH and is applied to all PDSCH transmissions. To maintain a single PDSCH transmission scheme for 6GR would provide continuity with NR and reduce specification complexity, especially considering of possible support of MRSS. Hence, for simplicity of 6GR, we propose to continue 1 transmission scheme for PDSCH, as similar to that of NR.

For the number of supported ranks and corresponding codewords, RAN plenary would determine the maximum number of antennas including around 7 GHz and around 15 GHz. If more than NR number are agreed, the enhancement of layer mapping and respective codeword mapping should be discussed.

Proposal 1: For single TRP, support to continue to use a single transmission scheme.

Proposal 2: Subject to RAN plenary conclusions on the supported number of antennas, the enhancement of layer mapping and respective codeword mapping should be discussed.

For multiple TRP operation, to leverage 5G design, at least following NR schemes should be reviewed for 6G.

- Scheme 1: Non-coherent joint transmission (NCJT)

  - DL data is transmitted from different TRPs based on single DCI-based or multi-DCI-based operations

- Scheme 2: DL multi-TRP for PDSCH

  - DL data from different TRPs can be sent to UE based on frequency multiplexing or time multiplexing

- Scheme 3: UL multi-TRP for PUSCH

  - UL data from a UE is transmitted to different TRPs

Proposal 3: Support to review applicability of at least the following NR schemes for 6G

- Scheme 1: Non-coherent joint transmission (NCJT) based on single DCI-based or multi-DCI-based operations

- Scheme 2: DL multi-TRP for PDSCH

- Scheme 3: UL multi-TRP for PUSCH

## DMRS pattern for PDSCH

The DMRS design in NR provides significant flexibility to accommodate diverse deployment scenarios and performance requirements. Key characteristics of the NR PDSCH DMRS include:

- Flexible design across use cases: The DMRS structure supports a wide range of deployment scenarios, including low-mobility, high-mobility, and high-frequency operation.

- Frequency- and time-domain multiplexing: Two DMRS resource-element mapping types based on Comb and orthogonal complementary code (OCC) structure are defined to support multi-layer transmission and multi-user multiplexing.

- Time-domain flexibility: The number of DMRS symbols can be configured depending on the PDSCH duration, with support for up to 4 DMRS symbols.

Although AI/ML may reduce the required number of DMRS, as described in SID, the system should be operatable without AI/ML. Therefore, the required amount of DMRS for conventional receiver architectures should be supported. The NR DMRS design can serve as a suitable starting point for defining such a baseline.

For the possibility to increase MU-MIMO capacity or support more SU-MIMO layers, more DMRS ports (such as 48 or 64 DMRS ports) can be required, as compared to NR's DMRS ports. This aspect should be further discussed with also the possibility to use AI/ML. The usage of AI/ML based receiver also can be considered to reduce DMRS density given the same channel condition.

Proposal 4: A baseline PDSCH DMRS pattern, which does not rely on AI/ML-based receiver processing, should be specified.

Proposal 5: In order to increase MU MIMO capacity or support more SU-MIMO layers or to reduce DMRS density further given the same channel condition, AI/ML based receiver processing should be considered.

## AI/ML-based receiver

For DMRS overhead reduction, it has been identified the following sub-use case for AI/ML-based receivers [2].

- Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain

- Sub-case B: Superimposed pilot

- Sub-case C: DMRS free

These approaches aim to reduce DMRS overhead while maintaining or improving channel estimation performance through AI/ML-based processing.

Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain

For sparse DMRS configurations in both time and frequency domains, the following summarizes possible directions considered in:

- Sparse orthogonal DMRS in the time domain, such as symbol-level or slot-level DMRS patterns.

  - DMRS symbols are sparser than that of the baseline DMRS pattern in symbol-level/slot-level

- Sparse orthogonal DMRS in the frequency domain, such as RB-level sparse patterns or DMRS-bundle-level patterns.

  - DMRS symbols are sparser than that of the baseline DMRS pattern in RB-level sparse patterns or DMRS-bundle-level patterns.

Potential specification impact could include DMRS pattern design, demodulation requirement in RAN4, as well as signalling/ procedure related to life-cycle management (LCM) for UE-sided model and/or NW-sided model.

Sub-case B: Superimposed pilot

In this sub-use case, the DMRS resource elements (REs) for a given DMRS pattern may be either overlapped or non-overlapped with data REs within the scheduled PDSCH resources. The number of DMRS REs may be equal to or smaller than the number of data REs.

When full overlap is applied, both DMRS and data are transmitted on the same REs by employing different power allocations. This approach enables all REs to carry data while embedding DMRS information through power-domain multiplexing, thereby eliminating explicit DMRS overhead.

When partial overlap or non-overlap is applied, it is expected that DMRS pattern should be explicitly provided or specified to minimize complexity of channel estimation. The operation may be partially similar to sparse DMRS.

Potential specification impact could include DMRS pattern design, power allocation between DMRS and data, demodulation requirement in RAN4, as well as signalling/ procedure related to life-cycle management (LCM) for UE-sided model or NW-sided model, or two-sided model (including inter-vendor calibration), when applicable.

Sub-case C: DMRS free

In 5G NR, the UE relies on DMRS to estimate the downlink channel for coherent demodulation. In this case, the AI/ML-based receiver uses trained models to infer the channel from received signal on data REs, which give the trained model partial information about the channel and the transmitted data. The model may output estimated channel at data REs or directly decode data, without relying on explicit DMRS. This approach enables significant DMRS overhead reduction.

Potential specification impact could include constellation design and related signalling/procedure, demodulation requirement in RAN4, as well as signalling/ procedure related to life-cycle management (LCM) for UE-sided model or NW-sided model, or two-sided model (including inter-vendor calibration), when applicable.

Proposal 6: Support to study following sub-use case for AI/ML-based receivers.

- Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain

- Sub-case B: Superimposed pilot

- Sub-case C: DMRS free

# Conclusion

In this document, we have discussed some aspects for PDSCH. We have the following proposals:

Proposal 1: For single TRP, support to continue to use a single transmission scheme.

Proposal 2: Subject to RAN plenary conclusions on the supported number of antennas, the enhancement of layer mapping and respective codeword mapping should be discussed.

Proposal 3: Support to review applicability of at least the following NR schemes for 6G

- Scheme 1: Non-coherent joint transmission (NCJT) based on single DCI-based or multi-DCI-based operations

- Scheme 2: DL multi-TRP for PDSCH

- Scheme 3: UL multi-TRP for PUSCH

Proposal 4: A baseline PDSCH DMRS pattern, which does not rely on AI/ML-based receiver processing, should be specified.

Proposal 5: In order to increase MU MIMO capacity or support more SU-MIMO layers or to reduce DMRS density further given the same channel condition, AI/ML based receiver processing should be considered.

Proposal 6: Support to study following sub-use case for AI/ML-based receivers.

- Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain

- Sub-case B: Superimposed pilot

- Sub-case C: DMRS free

# Reference

[1]. RP-251881, “New SID: Study on 6G Radio”, 3GPP, RAN#108.

[2]. Chair notes RAN1#123 meeting, Dallas, US, Nov 2025.