3GPP TSG RAN WG1 #124		   		                 R1-2601004

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Source: 	ETRI

Title:	Discussion on 6GR PRACH and RACH procedure

Agenda Item:	10.5.1.2

Document for:	Discussion

- Introduction

According to the updated SID [1], the overview inlcuding rough details are studied from this meeting. In this contribution, we discuss our view regarding 6GR frame structure, focusing on the Chair’s guidance and highlights from the last TSG meeting [2]. Our general view is included in our companion tdoc [3].

- Initial access and mobility Note 1: Targeting unified design under Sing-TRP and Multi-TRP for all candidate duplexing types, for spectrum utilization, for aggregation, for TN&NTN, scalability related aspects, maximum bandwidth at network side and UE side, and for targeting coverage, if applicable. - PRACH and RACH procedure Note 1: Including design of PRACH and transmission, and RACH procedure

- Discussion

In E-UTRA LTE and 5G NR, the random access procedure was introduced not only for initial access, but also to support camping and mobility in connected mode, while keeping a common framework. In this framework, a UE derives a set of ROs and then selects an RO depending on the event and its conditions. Since many different cases share the same RA procedure, the framework can be efficient for reuse, but it can also become inefficient for some specific cases.

A basic and most common case is RRC connection setup with contention resolution. This has worked well for a long time and has solid theoretical basis. Therefore, the 4-step random access can remain as the baseline for contention-based access.

Proposal 1: 4-step random access procedure is the baseline for CBRA.

In 5G NR, it is not always required to use 4-step RA. For CBRA, 2-step random access was specified, where MsgA (PRACH and PUSCH) and MsgB (PDSCH) can support small data transmission together with contention control. In our view, 2-step RA can also be useful for UEs in RRC non-connected to transmit small data without keeping RRC connection or without repeating full connection control procedures too often. However, this may also make it difficult for the gNB to stay in deep sleep for a long time, since MsgA occasions need to exist and may not be fully aligned with the legacy RO set used for 4-step RA.

Proposal 2: Study scenarios applicable to 2-step random access.

From the latest release, network energy saving became more important. For example, Rel-19 discussed reducing the energy consumption for periodic SIB1 in non-connected mode. A typical approach is on-demand SIB1, where the UE transmits an UL WUS based on PRACH, then monitors for RAR, and receives SIB1 within a defined time window. Also, backhaul signalling can be introduced to exchange SIB1 request configuration across cells, so that the UE can request SIB1 of a target cell. In this context, it is reasonable to discuss whether additional procedure can be used before a typical 4-step RA under a standalone assumption.

SI request has a similar aspect to CFRA and may also reuse CBRA resources. However, if a standalone on-demand SIB1 operation is considered, SI request may become different from legacy NR SI request, because the UE still needs minimum information to transmit UL WUS. To keep NES gain, it is also important that the gNB is not forced to receive UL WUS too frequently, equivalently, arbitrary UE may not transmit UL WUS repeatedly with power boost, so some specification impact are necessary to avoid uncontrolled request behavior. Such information block design whether to define a new SIB or to partition SIB1 is mainly subject to RAN2 scope, but RAN1 may need to discuss L1 impacts, thus the baseline should remain flexible at this stage.

Proposal 3: Study additional 2-step ahead of 4-step random access procedure under standalone assumption.

For CFRA, the full 4-step procedure is not necessary since contention resolution is not the main purpose. However, for signalling and resource reuse, it is still reasonable to reuse CBRA resources and keep an NR CFRA structure. If CBRA resources are reused, Msg2 RAR reception may still be required even for CFRA.

Proposal 4: Support the CFRA procedure, taking the NR CFRA procedure as baseline.

  - Random access triggering conditions

Random access (RA) is mainly used for RRC connection setup and reestablishment, and it is important to reserve RA radio resources so that the UE and gNB can start communication with minimum information. We expect that this high-level approach will also be reused in 6GR, and it is useful to review under which conditions the UE performs CBRA/CFRA/SI request (or does not perform them).

A UE may not be allowed to perform RA, and this can be determined by cell barring related information. Rel-15 UAC and its subsequent extensions are mainly for higher-layer connection control and RA attempt control. In addition, cell barring functions introduced in Rel-17/Rel-18 are related to whether camping is allowed, and together with MIB-based cell barring, they can become conditions for deciding whether the UE can attempt Msg1 (PRACH). In our view, rather than having separate (use-case specific) barring rules for Rel-17 (e.g., NTN) and Rel-18 (e.g., NES, ATG, etc.), it is desirable to introduce a unified framework by selecting meaningful use cases, so that barring/allowing information is expressed efficiently. Such information may also be broadcast together with UL WUS related information. Since the condition for allowing (or not allowing) RA while camping is also connected to the condition for UL WUS transmission from the NES perspective, the relation between them should be discussed.

Proposal 5: Support the unified framework for allowing Msg1 transmission, and study potential L1 impacts.

Once the UE confirms that RA is allowed, it can select RACH resources and proceed. In Rel-15 NR, RA can be viewed as operating under relatively basic conditions. In Rel-17, RA was also used as a method to provide early indication of UE capability or relevant service/operation aspects, to handle more complex environments. For example, although SIB reception is common, Msg1-based preamble grouping was introduced to enable early differentiation of feature combinations (e.g., for operation in a different BWP right after RRC connection). This approach improves extensibility, but overly fine-grained featurePriorities can increase specification and implementation complexity. Therefore, for 6GR, it is reasonable to reuse the NR framework as a baseline and then select which use cases are needed in 6GR.

Proposal 6: Support the early indication of UE capability or relevant service, taking the NR framework as baseline.

In 6GR, RACH (Random Access) would be extended to a general UL trigger mechanism, which can be triggered directly or indirectly by various events. In particular, RACH-based operation can include both L3 events where the RRC state changes, and fast events at L1/L2 where the RRC state is kept, generally RRC connected assumption. These two types have different layer objectives, but they may still share the same Msg1 transmission. Therefore, it becomes important to keep in a consistent way of triggering the Msg1 transmission.

L3 events can be understood as events that involve RRC message exchange, and in some cases, RRC state transition. For example, RRC setup, RRC resume, and RLM are L3 procedures, and they can be coupled with PRACH procedures. Also, for mobility support, various L3 events such as A3 are used. In such a case, the UE performs SSB-based L3 measurements, and if the event condition is satisfied, then the UE transmits PRACH to the target gNB.

On the other hand, L1/L2 events can be seen as events mainly in RRC Connected, to recover or switch link/beam/cell state quickly without changing the RRC state. BFR in Rel-15 is a typical example of an L1/L2 event, where the UE transmits BFRQ on PRACH for the PCell. Also, it is possible to use SSB-based L1 measurements to decide an event condition and then trigger RACH. For example, a RACH-based LTM cell switch involves an indication received by MAC CE, which in turn triggers CFRA. Thus it might be described as an indirect RACH event.

Another dimension is that RACH can be related to initial beam acquisition and beam refinement, and in this sense L1/L2 events can be defined. For example, PRACH sweeping in Rel-20 performs UL beam refinement in a single-TRP scenario, and it can be further applied to other scenarios such as inter-cell beam management, LTM, and multiple-TRP scenarios, together with Rel-18/-19 LTM. Those are based on multiple preamble transmissions per RACH attempt, and it can be an example of L1/L2 events.

If we extend this view to on-demand items from the latest release, OD-SIB1 request can be classified as an L1/L2 event since it may not change the RRC state, and OD-SSB request can also be classified as an L1 event.

Thus, L1/L2 events, L3 events, and on-demand activation events can all trigger RACH directly or indirectly. In our view, these are currently fragmented, and while a unified management framework may be largely discussed in RAN2, there is still a need to study the consistency of L1/L2 events in RAN1.

If RACH events are simplified under a unified view, then measurement object or measurement set design should be discussed as well. Also, depending on the RACH event, it may be necessary to discuss 4-step RA, 2-step RA, or even a RAR-less approach. In such a case, it should be clarified which RA procedure type is suitable per event, and what are the related L1 impacts such as RAR contents, UL grant assumptions, timing constraints, and repetition/beam mapping.

Proposal 7: Support the unified framework for Msg1 transmission, and study L1/L2-driven RA events if necessary.

  - RA resource

RA resources derivation and selection seems complicated in NR, but its structure is flexible to satisfy diverse requirements. In 5G NR, the PRACH configuration index captures much information. An index in the table provides a set of ROs in the time domain, i.e., the preamble format and radio frame number/slot number and the structure of RACH slots. Then UE knows the location of a set of ROs, and chooses an appropriate SSB.

In our view, the 5G NR framework can be reused for legacy IMT spectrum, and discuss how to support FR3 RA index table, for unpaired spectrum. UL opportunities can be more limited due to potential UL-DL switching period and the PRACH association period based on the NR framework may or may not have sufficient ROs for diverse deployment scenarios.

On the other hand, for FR1 or MRSS operation and FR2-1, it is desirable to maintain compatibility with NR as much as possible, since reuse of the existing index and mapping rules reduces specification work.

Proposal 9: Discuss a PRACH configuration index table for FR3 unpaired spectrum, while reusing the existing NR configuration index for FR1 and FR2-1 as much as possible.

Regarding the frequency resources of an RO, the NR parameters can be reused as a baseline, e.g., keeping a constant number of ROs in the RACH slot. If SBFD is considered, we believe the Rel-19 approach including option 1 and option 2 can be reused for defining RO resources and constraints but we are open to discuss.

SSB–RO mapping needs further discussion, i.e., whether NR mapping can be reused, or whether a modified mapping is needed to support additional objectives. Related motivation appeared since PRACH repetition discussion in Rel-18. For example, RO grouping can support a RACH attempt, but when a TDD slot pattern is considered, it can require a long time period (time period X) to complete a grouping operation, which increases buffering time at the gNB side and can also increase access latency. Similar motivations arouse PRACH beam refinement (or related discussions) in Rel-20, where beam-related behavior and RO/SSB association can become a limiting factor.

In 6GR, more diverse scenarios may be considered. For example, the UE may receive SSBs from multiple TRPs and 1T2R TRP, and an RA event may be triggered based on such reception. A key question is whether such an event requires beam report or beam indication in RRC idle mode, and whether the benefit is large enough compared to a single TRP baseline. If multi-TRP-based initial access does not provide a clear advantage over the NR baseline, which enters RRC connection setup using a single TRP first and then expanding to multi-TRP operation, it may be more reasonable to keep the baseline as single-TRP initial access. While these RA events can be viewed as a joint RAN1/RAN2 topic, the L1 impacts should be discussed in RAN1, especially resource mapping and field design in L1 channel.

Proposal 10: Study whether/how to support multiple-TRP reception from the RRC idle state, and identify potential L1 impacts.

  - Msg1 enhancement

PRACH preamble design can support various sequences and formats, and for FR3 cellular deployment we do not see a strong need to introduce a new preamble specifically optimized for FR3. In particular, considering MRSS operation, reusing an NR-aligned preamble design can be beneficial for gNB implementation and interoperability. At the same time, for NTN aspects, extreme coverage may be required and/or a short format may be preferred depending on dwell time and timing constraints. Also, for low-tier devices, using a long format can be a practical option to extend coverage. In such cases, the required sequence multiplexing capacity may increase (e.g., larger preamble set to support more UEs and/or more diverse deployments). If new preamble sequences are discussed, they may still be based on ZC sequences, but can also be alternative sequences, or even linear combinations of known sequences if they provide useful correlation properties.

Proposal 8: Support NR PRACH preamble sequences as a baseline for both long and short formats, and identify any limitations of the legacy sequence design under diverse deployment scenarios and device types.

In addition, LTE introduced restricted sets to address Doppler effects, and NR also introduced multiple types of restricted sets for PRACH. Since 6GR day-one is expected to consider NTN LEO operation, it can be necessary to revisit whether additional restricted-set options are needed to better handle high Doppler and related impairments. In addition, to realize energy-efficient NTN operation in the early deployment stages of 6GR, the introduction of a robust PRACH format tailored for GNSS-resilient scenarios is indispensable. In particular, we maintain that supporting at least one long PRACH format is required to overcome the significant propagation delays and Doppler effects inherent in non-terrestrial environments. This plays a critical role in increasing the network entry success rate by expanding the tolerance for initial synchronization. [4]

Proposal 9: Study whether PRACH needs additional types of restricted sets and preamble format to support NTN scenarios, especially LEO with high Doppler, larger RTT and differential delay for GNSS resilient operation.

Regarding PRACH transmission power, we consider the NR framework with full pathloss compensation as a reasonable baseline. In NR, the target power is driven by a ramping counter configured/updated via RAR, which provides a clear rule for retransmission power increase.

Proposal 10: Support the NR PRACH power control framework as a baseline.

  - Msg2/4 enhancement

Although the content of Msg2 PDSCH is mainly a MAC-layer topic, some issues are still relevant to RAN1 since they may impact L1 procedures and signalling. If Msg1 is transmitted under different random access (RA) event conditions, the gNB may need to respond differently, and the detailed information can be delivered on DL-SCH. Therefore, it is useful to discuss potential L1 impact related to Msg2/RAR, while keeping the baseline procedure aligned with NR.

In 5G NR, a framework for receiving the RAR is defined and it works well with limited overhead. The RAR is addressed by RA-RNTI on a Type-1 PDCCH CSS set, and the UE monitors the RAR within the RA window after finishing a RACH attempt. If the UE finds a valid RAR, it continues with the remaining random access procedure; otherwise, it may consider reattempt depending on the configuration. For NTN scenarios, the round-trip time (RTT) is large, so a different approach from typical terrestrial operation may be needed, and RTT-related constants can be revised.

The RAR UL grant dynamically schedules Msg3 PUSCH, and several enhancements have been discussed in NR-Advanced. For example, Rel-17 introduced Msg3 repetition, and for Rel-18 PRACH repetition, MCS field partition was proposed. In addition, Rel-20 topics such as PRACH Tx refinement (or Msg3 beam indication) also discuss related issues. Interest of mTRP operation in idle mode is increasing, with the motivation of rapid connection establishment.In 6GR, new RA events from new deployment scenarios or services may be introduced as well, and the associated RAR may need to be revisited from the L1 perspective.

Proposal 11: Study whether the RAR requires new or modified fields in response to 6GR RA events, while taking the NR RAR as a baseline.

The HARQ-ACK for Msg4 PDSCH is typically the first PUCCH transmission from the UE during initial access. In NR, the PUCCH resource set is determined from an index obtained via SIB1, and one resource is derived based on the Msg4 scheduling DCI. In Rel-18 NTN, PUCCH repetition was introduced (configured via NTN-related system information such as SIB19). For 6GR cellular deployment, if coverage extension is required, it can be considered to support HARQ-ACK repetition and configure it in the SIB1 so that the UE can apply repetition for the initial PUCCH transmission.

Proposal 12: Support PUCCH repetition for HARQ-ACK during 6GR initial access.

  - Msg3 enhancement

Initial access coverage is largely determined by Msg3, and Msg3 can be seen as a bottleneck channel. For this reason, improvement has been continuously requested since Rel-17. In Rel-17, Msg3 repetition was introduced, and in Rel-20, Msg3 beam indication is being actively discussed. In FR3, to secure coverage, it is desirable to consider both repetition and sweeping.

However, the beam/TCI state (or spatial relation) of Msg3 PUSCH is closely related to Msg1. For Msg3 repetition, the spatial filter used for Msg1 can be reused. On the other hand, if Msg3 beam indication is applied, the gNB can indicate one beam among the spatial filters of Msg1 to the UE. Then, this indicated beam can be used as a beam reference for Msg3 transmission, and it may mean that Msg3 can be transmitted with better SNR, possibly with a single transmission (i.e., reduced repetition).

If needed, we can also discuss a method to combine repetition and sweeping, because the UE may not always find the PRACH Tx beam accurately only based on SSB. In such a case, limited sweeping (or refinement) may help to select a better beam, and repetition can be applied with that beam to increase the success probability in FR3 initial access.

Proposal 13: Support the NR-based Msg3 transmission scheme as the baseline, and study whether any further enhancements are beneficial

- Conclusion

In this section, we address our proposals below.

Proposal 1: 4-step random access procedure is the baseline for CBRA.

Proposal 2: Study scenarios applicable to 2-step random access.

Proposal 3: Study additional 2-step ahead of 4-step random access procedure under standalone assumption.

Proposal 4: Support the CFRA procedure, taking the NR CFRA procedure as baseline.

Proposal 5: Support the unified framework for allowing Msg1 transmission, and study potential L1 impacts.

Proposal 6: Support the early indication of UE capability or relevant service, taking the NR framework as baseline.

Proposal 7: Support the unified framework for Msg1 transmission, and study L1/L2-driven RA events if necessary.

Proposal 8: Support NR PRACH preamble sequences as a baseline for both long and short formats, and identify any limitations of the legacy sequence design under diverse deployment scenarios and device types.

Proposal 9: Study whether PRACH needs additional types of restricted sets and preamble format to support NTN scenarios, especially LEO with high Doppler, larger RTT and differential delay for GNSS resilient operation.

Proposal 10: Support the NR PRACH power control framework as a baseline.

Proposal 11: Study whether the RAR requires new or modified fields in response to 6GR RA events, while taking the NR RAR as a baseline.

Proposal 12: Support PUCCH repetition for HARQ-ACK during 6GR initial access.

Proposal 13: Support the NR-based Msg3 transmission scheme as the baseline, and study whether any further enhancements are beneficial

- References

[1] RP-253876	Revised SID: Study on 6G Radio	NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R1-2600002	Highlights from RAN#110	RAN1 Chair

[3] R1-2601002	Discussion on general aspects and frameworks for 6GR	ETRI

[4] R1-2601018   On the GNSS-based NTN operation for 6GR   ETRI