3GPP TSG-RAN WG1#124     						R1-2601148

Gothenburg, Sweden, February 9 – 13, 2026

Agenda item:	10.6.1.1

Source: 	Kyocera

Title: 	Discussion on Design of WUS with OFDM based sequence

Document for:	Discussion

Introduction

At the RAN#108 meeting[1], the 6G SID was approved. As part of its objectives, the following was stated.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. […] - Target scalable and forward compatible design for diverse device types. […] - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. […] - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Physical Layer structure for 6GR, […] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] […] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] […] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1

Furthermore, further improvement of energy efficiency and UE power consumption is identified as one of the design targets within the 6G SID.

At RAN1#122bis[2], the following agreements were reached regarding 6G WUS:

Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities

At RAN#110[3], the following proposals were endorsed:

Proposal 2: - Diverse device types as in 6G SIDs are expected to include supporting at least MBB (as highest priority), FWA, and Massive IoT services - Companies are encouraged to further contribute at RAN plenary regarding the analysis of necessary aspects to help progress RAN WG technical design and evaluations. - The detailed discussion for device types (how many, parameters, whether or not to define device types, whether or not to introduce minimum mandatory set of features, etc.) is on hold in RAN plenary and to resume from RAN#113 (September 2026). Discussions on device types are not to be discussed in WGs until further update from RAN plenary.

For 6GR, it was agreed to support at least MBB, FWA, and Massive IoT. 
Based on this, we discuss the design of 6GR WUS.

Discussion on Design of 6GR WUS

In the 6G SID[1], further improvement of energy efficiency and UE power consumption is clearly identified as one of the key objectives in the 6GR design. Furthermore, it was agreed that 6GR will support various device types, including FWA and Massive IoT, within a single radio technology framework, while prioritizing MBB.

Under these design principles, key technical challenges include how to efficiently wake up UEs in low power states and how to maintain synchronization during extended sleep periods. For power-constrained devices, NR has envisioned use cases requiring long-term sleep to reduce power consumption, while simultaneously necessitating communication setup within a predefined latency for emergency events. Consequently, balancing low power consumption with low latency has been recognized as a fundamental challenge[4]. These issues are expected to remain critical in 6GR, which supports diverse device types including Massive IoT.

Considering the large number of Massive IoT and power-constrained devices, the challenges of low power consumption and low latency seen in NR may remain significant in 6GR.

This contribution provides an overview of the technical background and the achieved benefits of WUS and LP-SS, which were designed and agreed upon in Rel-18 and Rel-19 for 5G NR. Based on this review, we discuss design of 6GR WUS.

WUS and LP-SS Design in 5G NR

WUS Design

In 5G NR, it has become evident that conventional power-saving control using eDRX faces challenges in simultaneously satisfying long battery life and low latency requirements. This tendency is particularly prominent in use cases such as Massive IoT, sensors, and wearable devices, where extremely long standby times are required alongside immediate responsiveness upon event occurrence.

To address these challenges, the Low Power Wake-Up Signal (LP-WUS) was introduced. The key features of the LP-WUS design in NR are as follows[5]:

- Assumes reception by a Low Power Wake-Up Receiver (LP-WUR) that operates independently of the Main Radio (MR).

- Enables continuous monitoring while the MR is in a complete power-off or deep sleep state.

- Wakes up the MR only when there is a specific need for the UE to be paged or active.

For the waveform, OOK-1 and OOK-4 are adopted, which enable simplified receiver processing and reduced power consumption. Furthermore, by overlaying Zadoff-Chu (ZC) sequence-based OFDM sequences on the OOK 'ON' symbols, performance can be enhanced through correlation detection, and information transmission is achieved via sequence selection [6].

LP-WUS is used as a trigger for paging monitoring in RRC IDLE/INACTIVE mode and as a trigger for PDCCH monitoring in RRC CONNECTED mode, providing a common design framework that is applicable across different RRC states[4].

LP-SS Design

LP-SS (Low Power Synchronization Signal) was newly designed in NR as a low-power synchronization signal that operates in coordination with LP-WUS. In conventional synchronization methods using SSB, the MR must be activated for every synchronization event, which poses a significant challenge for UEs in long-term sleep from a power consumption perspective. To address this issue, LP-SS is designed with the following objectives[4]：

- Low-power synchronization assuming reception by an LP-WUR.

- Performance of time synchronization and/or RRM measurements for the serving cell without activating the MR.

The design is based on the same principles as LP-WUS, and the following configurations are being considered[6]:

- Use of OOK-1 and/or OOK-4 waveforms.

- Support for both cases with and without overlaid OFDM sequences.

LP-SS is transmitted periodically, with assumed periodicities of 320 ms or 160 ms. This periodic transmission allows UEs to suppress time drift and ensure sufficient synchronization accuracy while maintaining long-term sleep states. [7]

Benefits and effectiveness achieved by NR Design

The benefits and effectiveness of the WUS and LP-SS designs in NR have been reported through evaluations in Rel-18 and Rel-19.

- Reduction of UE power consumption

- In RRC IDLE/INACTIVE modes, introducing a mechanism that sufficiently relaxes MR measurement requirements and triggers paging monitoring via LP-WUS/WUR can reduce power consumption by more than 90% compared to legacy I-DRX. Furthermore, compared to eDRX, it has been shown that operations independent of the existing Paging Time Window (PTW) can significantly improve communication latency while maintaining power-saving effects [4]. These NR designs, which enable substantial UE power reduction, are particularly effective for Massive IoT and other applications with long standby requirements.

- Coverage performance

- Coverage performance is a critical metric in the design of LP-WUS and LP-SS[2]. Specifically, NR targets are established based on the reception level of PUSCH Message 3. Simulation results have confirmed that by overlaying ZC-based OFDM sequences on OOK waveforms, sufficient coverage can be ensured using practical resource amounts while maintaining detection performance comparable or superior to existing channels [5]. This enables reliable wake-up control even for UEs located at the cell edge.

- Improvement of paging latency

- Even with long eDRX cycles, the use of LP-WUS enables improved responsiveness in paging monitoring while minimizing power consumption. Furthermore, the LP-WUS in NR serves as more than a simple wake-up signal; it can transmit up to 5 bits of information. This supports detailed wake-up triggers using subgroups and up to 32 codepoints, which effectively suppresses unnecessary wake-up for UE[7].

- Maintaining synchronization and performing RRM measurements with low power consumption using LP-SS

- LP-SS enables UEs to estimate time and frequency synchronization without activating the MR through periodic transmissions, such as every 320 ms or 160 ms[7]. By aligning its design with LP-WUS (e.g., adopting OOK-1 / OOK-4 waveforms), it reduces the receiver's processing load while supporting time drift suppression and RRM measurements for the serving cell.

- Simplification of design and RRC states

- LP-WUS and LP-SS are configured to share the same starting PRB index or offset from the carrier boundary, which simplifies the NR signal structure and the UE's reception process[7]. The design of LP-WUS and LP-SS is applicable not only to RRC IDLE/INACTIVE modes but also to CONNECTED mode, providing a consistent power-saving mechanism across all RRC states.

The design of WUS and LP-SS in NR provides benefits from the following perspectives:
・ Reduction of UE power consumption
・ Coverage performance
・ Improvement of paging latency
・ Maintaining synchronization and performing RRM measurements with 
   low power consumption using LP-SS
・ Simplification of design and RRC states

Design of 6G WUS

Even in the 6G era, considering the existence of numerous Massive IoT devices and battery-constrained UEs, the challenges of low power consumption and low latency identified in NR remain critical. In this context, introducing entirely new mechanisms without leveraging the established WUS and LP-SS designs from NR could lead to increased design complexity and an unnecessary proliferation of options.

On the other hand, the NR WUS and LP-SS designs exhibit high alignment with the 6G SID objectives in the following aspects

- Minimization of power consumption and enhancement of energy efficiency.

- Applicability of diverse device types.

- Simplification of design and assurance of forward compatibility

Taking these factors into account, positioning the NR WUS and LP-SS designs as the baseline for 6G is both technically and effective for the initial discussions in RAN1.

RAN1 should study the NR WUS designs as the baseline for 6GR WUS.

In NR WUS designs, the UE operating under extended DRX does not maintain fine time/frequency tracking and cannot rely on the relatively infrequent SSB burst periodicity to re-establish synchronization prior to WUS reception. To address this, LP-SS was introduced as a low-complexity synchronization construct that provides coarse OFDM symbol timing, residual CFO estimation, and essential resource-grid alignment with substantially reduced UE processing relative to full PSS/SSS/PBCH acquisition.

This enables robust WUS detection at low SNR with minimal RF/baseband activation time, while allowing WUS transmission to be scheduled independently of SSB occasions. Consequently, LP-SS satisfies key RAN1 objectives for low-power wake-up operation, reduced UE implementation burden, and greater design flexibility for WUS and paging without impacting legacy synchronization procedures.

RAN1 should study the NR LP-SS designs as the baseline to enable WUS synchronization and serving cell RRM in 6GR.

Conclusion

In the previous sections we made the following observations and proposals for 6GR WUS designs :

Observation 1	Considering the large number of Massive IoT and power-constrained devices, the challenges of low power consumption and low latency seen in NR may remain significant in 6GR.

Observation 2	The design of WUS and LP-SS in NR provides benefits from the following perspectives: 
・ Reduction of UE power consumption
・ Coverage performance 
・ Improvement of paging latency 
・ Maintaining synchronization and performing RRM measurements with low power 
   consumption using LP-SS 
・ Simplification of design and RRC states

Proposal 1	RAN1 should study the NR WUS designs as the baseline for 6GR WUS.

Proposal 2	RAN1 should study the NR LP-SS designs as the baseline to enable WUS synchronization and serving cell RRM in 6GR.

References

- RP-251881, New SID: Study on 6G Radio, NTTDOCOMO, RAN#108, Prague, Czech Republic, June 2025

- Chair notes RAN1#122bis, RAN1 Chair, RAN1#122bis, Prague, Czech, October 2025.

- RP-253856, Moderator's summary for 6G SI: operational requirements: diverse device types, Apple, RAN#110, Baltimore, USA, December 2025

- RP-251200, " Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)", vivo, NTT DOCOMO, RAN#108, Prague, Czech, June 2025

- TR38. 869, V18.00 (2023-12), Study on low-power wake-up signal and receiver for NR

- TR38.211, V19.2.0 (2025-12), Physical channels and modulation

- TR38.311, V19.0.0 (2025-09), Radio Resource Control (RRC) protocol specification