3GPP TSG RAN WG1 #124	R1-2601161

Gothenburg, Sweden, Feb. 9th – 13th, 2026

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on R19 UE features

Agenda Item:	8.9

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss UE features for NTN_Ph3 and TEI19 with [Common_PDCCH_rep_TN]. Based on the agreements reached at the last RAN1 meeting, we have the following FGs list for NTN_Ph3 and TEI19 with [Common_PDCCH_rep_TN] [1].

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65. NR_NTN_Ph3 | 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection |  | see Note | N/A | UE does not support an additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional with capability signaling Note: The capability signalling is introduced to allow the NW to collect the statistics about the percentage of UEs that support this feature FFS: A UE that supports Rel-19 NR-NTN must support this FG |
| 65. NR_NTN_Ph3 | 65-1-2 | PDCCH repetition for Type0 PDCCH CSS and SIB1 PDSCH repetition within 20ms duration | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1 2. Support reception of SIB1 PDSCH repetition within 20 ms duration |  | YES | N/A | PDCCH repetition for Type0 PDCCH CSS and SIB1 PDSCH repetition within 20ms duration are not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |
| 65. NR_NTN_Ph3 | 65-1-3 | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS | 1. Support reception of PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS 2. Required number of BDs for the two PDCCH candidates in RRC CONNECTED mode | 65-1-2 | Yes | N/A | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Candidate values for component 2: {1, 2} | Conditionally mandatory with capability signaling for UE supporting FG 65-1-2 |
| 65. NR_NTN_Ph3 | 65-1-5 | Msg4 PDSCH repetition | 1. Support reception of Msg4 PDSCH repetition 2. Support of repetition factors 2 and 4 | [FFS] | YES | N/A | Msg4 PDSCH repetition is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |
| 65. NR_NTN_Ph3 | 65-2-1 | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE | 1. Support handling collision Case 3 2. Support handling collision Case 4 | 26-1, 28-3 | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission Note: this FG is for FR1 FDD only Note: collision Case 3 handling is for RRC CONNECTED and INACTIVE states, and collision Case 4 is for RRC CONNECTED state only | Optional with capability signaling |
| 65. NR_NTN_Ph3 | 65-3-1 | PUSCH with inter-slot OCC | 1. Support inter-slot OCC for DFT-s-OFDM PUSCH transmissions with repetition Type A 2. Support RV cycling across OCC groups 3. Support collision handling between PUCCH and PUSCH with OCC 4. Support of keeping phase continuity and power consistency across one OCC group as per requirements defined by RAN4 5.Supported OCC length 6. Supported satellite orbit for PUSCH with OCC | 26-1, at least one of {5-14, 5-16, 5-17} | YES | N/A | UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Component 5 candidate values: {(2), (2 and 4)} Component 6 candidate values: {(GSO), (NGSO), (GSO and NGSO)} | Optional with capability signalling |

| 67. TEI19 [Common_PDCCH_rep_TN] | 67-9 | PDCCH repetition for Type0 PDCCH CSS in TN | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1 in TN |  | YES | N/A | PDCCH repetition for Type0 PDCCH CSS in TN is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for FR1 TN | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 67. TEI19 [Common_PDCCH_rep_TN] | 67-10 | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS for TN | 1. Support reception of PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS for TN 2. Required number of BDs for the two PDCCH candidates in RRC CONNECTED mode | 67-9 | Yes | N/A | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS for TN is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for FR1 TN Candidate values for component 2: {1, 2} | Conditionally mandatory with capability signaling for UE supporting FG 67-9 |

Besides, at the last RAN plenary meeting, the following agreement was reached for the above TEI [2].

RP-253829 On support of SIB1 repetition for TN Qualcomm Incorporated Replaces RP-253363 conclusion: proposal is agreed Proposal: SIB1 repetition is supported for TN, following the exact same implementation as for NTN. - Update RAN1 UE features accordingly to align with the current specifications..

- Discussion on NTN_Ph3 / TEI19 with [Common_PDCCH_rep_TN]

  - DL coverage enhancement

    - FG 65-1-1 (SSB periodicity extension)

- Mandatory/optional: Based on discussion so far, removing the whole FFS is the reasonable choice as e.g., the extended periodicity is not necessary in NTN with higher satellite capability, or e.g., mandating the support for (e)RedCap UE could not be a good way.

Proposal 1: Update FG 65-1-1 as follows.

| 65. NR_NTN_Ph3 | 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection |  | see Note | N/A | UE does not support an additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional with capability signaling Note: The capability signalling is introduced to allow the NW to collect the statistics about the percentage of UEs that support this feature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - FG 67-9 (TEI FG for PDCCH repetition for Type0 PDCCH CSS)

Based on the agreement at the last RAN plenary meeting, the component of SIB1 PDSCH repetition is copied from FG 65-1-2, and the FG name/Consequence if… columns are updated accordingly.

Proposal 2: Update FG 67-9 as follows.

| 67. TEI19 [Common_PDCCH_rep_TN] | 67-9 | PDCCH repetition for Type0 PDCCH CSS SIB1 PDSCH repetition within 20ms duration in TN | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1 in TN 2. Support reception of SIB1 PDSCH repetition within 20 ms duration in TN |  | YES | N/A | PDCCH repetition for Type0 PDCCH CSS and SIB1 PDSCH repetition within 20ms duration in TN are not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for FR1 TN | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - FG for Msg4 PDSCH repetition

Conclusion It can be discussed in UE feature session whether a dedicated PDSCH repetition capability (e.g. FG5-17a and FG16-2b-5) is a pre-requisite UE feature for Msg4 PDSCH repetition.

- Prerequisite: At the previous meeting, it was concluded whether a dedicated PDSCH repetition capability is a prerequisite for this FG or not can be discussed in UE feature discussion as above. The motivation is that PDSCH right after RRC connection establishment (called “Msg6”) is performed with repetition.

  - If this prerequisite relationship is not supported, then UE capability of PDSCH repetition based on dedicated PDSCH configurations is unclear at NW side at the timing of UE contention resolution/RRC connection establishment and thus PDSCH repetition configurations for dedicated PDSCH cannot be provided via Msg4 PDSCH.

  - If this prerequisite relationship is supported, then PDSCH repetition configurations for dedicated PDSCH can be provided via Msg4 PDSCH and Msg6 can be performed with repetition.

Based on the above, we believe that the prerequisite relationship should be defined in this FG. Regarding which FG should be included, FG 5-17a (semi-static) and FG 16-2b-5 (dynamic) are straightforward.

Proposal 3: Update FG 65-1-5 as follows.

| 65. NR_NTN_Ph3 | 65-1-5 | Msg4 PDSCH repetition | 1. Support reception of Msg4 PDSCH repetition 2. Support of repetition factors 2 and 4 | At least one of {5-17a, 16-2b-5} | YES | N/A | Msg4 PDSCH repetition is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - RedCap/eRedCap support

No remaining issue is found for UE feature of (e)RedCap support.

  - UL capacity/throughput enhancement

No remaining issue is found for UE feature of UL OCC.

- Conclusion

In this contribution, we presented our views regarding FGs for NTN_Ph3 and TEI19 with [Common_PDCCH_rep_TN]. Based on the discussion in this contribution, the following proposals were made.

Proposal 1: Update FG 65-1-1 as follows.

| 65. NR_NTN_Ph3 | 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection |  | see Note | N/A | UE does not support an additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional with capability signaling Note: The capability signalling is introduced to allow the NW to collect the statistics about the percentage of UEs that support this feature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Update FG 67-9 as follows.

| 67. TEI19 [Common_PDCCH_rep_TN] | 67-9 | PDCCH repetition for Type0 PDCCH CSS SIB1 PDSCH repetition within 20ms duration in TN | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1 in TN 2. Support reception of SIB1 PDSCH repetition within 20 ms duration in TN |  | YES | N/A | PDCCH repetition for Type0 PDCCH CSS and SIB1 PDSCH repetition within 20ms duration in TN are not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for FR1 TN | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Update FG 65-1-5 as follows.

| 65. NR_NTN_Ph3 | 65-1-5 | Msg4 PDSCH repetition | 1. Support reception of Msg4 PDSCH repetition 2. Support of repetition factors 2 and 4 | At least one of {5-17a, 16-2b-5} | YES | N/A | Msg4 PDSCH repetition is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Reference

[1]	R1-2509584	Updated RAN1 UE features list for Rel-19 NR after RAN1 #123 EOM		Moderators (AT&T, NTT DOCOMO, INC.)

[2]	RAN#110 meeting report