3GPP TSG RAN WG1 #124			R1-2601170

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Source:	NTT DOCOMO, INC.

Title:	Discussions on coverage enhancement

Agenda Item:	9.4.1

Document for: 	Discussion and Decision

- Introduction

At RAN#108 meeting, WID on Coverage enhancements for NR Phase 3 was approved with the objective as follows [1]:

The detailed objectives for coverage enhancement of this work item include: - Specify following PRACH coverage enhancements [RAN1, RAN2] - Multiple PRACH transmissions with different Tx beams for 4-step RACH procedure. - UE may receive UL beam information after transmission of MSG1 - UL beam information is to assist the UE decision on Msg3 beam selection. - Note 1: “different Tx beams” is for the purpose of future RAN1 discussions - Note 2: The enhancements of PRACH are targeting for FR2 and can also apply to FR1 when applicable. - Note 3: The enhancements of PRACH are targeting short PRACH formats and can also apply to other formats when applicable. - Note 4: The PRACH repetitions are transmitted over ROs associated with the same SSB. - Note 5: The procedure for repetitions of a PRACH transmission is as in Rel-18 - Specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI [RAN1, RAN2] - Specify enhancements to improve PUSCH coverage for higher uplink data rate by extending pi/2-BPSK to more MCS entries in MCS tables [RAN1]

In this contribution, coverage enhancements for PRACH and PUSCH are discussed.

- Discussions

2.1 Multiple PRACH transmissions with different beams

PRACH resource selection

At last meeting, it was agreed that separate PRACH resource is supported for multiple PRACH transmissions with different TX beams.

Agreement Support to use separate ROs or separate preambles on shared RO to differentiate multiple PRACH transmissions with same Tx beam and different Tx beams. - FFS: details

Now there are three types of PRACH resources. PRACH resource selection rule needs to be clarified.

- #1 (R15/16/17): single-PRACH transmission

- #2 (R18/19): multiple PRACH repetitions with same beam

- #3 (R20): multiple PRACH transmissions with different beams

Considering the main motivation is to identify best UL beam, whether UL beam sweeping is needed or not should be based on UE capability of beam correspondence with or without TX beam sweeping.

In Rel-15, a mandatory UE capability FG 2-20 for beam correspondence is defined. UE reports whether UE shall meet a beam correspondence requirement with or without additional TX beam sweeping. In Rel-16, optional capabilities FG 8-2/FG 8-3 are introduced for SSB/CSI-RS based beam correspondence. UE also reports whether with or without TX beam sweeping for corresponding beam correspondence.

If UE can support at least one beam correspondence without TX beam sweeping out of Rel-15/16 beam correspondence capabilities, no TX beam sweeping is needed for UE. UE selects between #1 and #2 based on SSB RSRP threshold for Msg 1 repetition determination, as in Rel-18.

If UE supports none of Rel-15/16 beam correspondence without TX beam sweeping, TX beam sweeping is needed.  #3 is applied.

Proposal 1: UE determines whether to select PRACH resource for multiple PRACH transmissions with different beams based on UE capability of beam correspondence with or without TX beam sweeping.

- If UE supports none of Rel-15/16 beam correspondence without TX beam sweeping, UE selects PRACH resource for multiple PRACH transmissions with different beams.

- If UE supports at least one beam correspondence without TX beam sweeping out of Rel-15/16 beam correspondence capabilities, UE selects PRACH resource for single PRACH transmission or PRACH resource for PRACH repetitions with same beam, based on RSRP threshold for Msg 1 repetition.

Whether gNB configures UE to use same TX beam in part of PRACH transmissions

At the RAN1#122bis meeting, determination of TX beam utilization was discussed with following agreement.  Whether/how gNB can configure UE to use same Tx beams in part of the PRACH transmissions needs further discussion.

Agreement For the determination of Tx beam utilization for multiple PRACH transmissions with different Tx beams, - It is up to UE implementation to determine which Tx beams to use, subject to any necessary restriction - FFS: any necessary restriction - FFS: whether/how gNB can configure UE to use same Tx beams in part of the PRACH transmissions

If gNB configuration of same TX beam in part of PRACH transmissions is supported, there would be larger specification effort, e.g. design of sub-RO groups in one RO group, determination of number of total ROs and number of sub-RO-groups in one RACH attempt, etc. Considering the limited TU budget for Rel-20 Coverage Enhancement WI, it is prefer to not pursue this direction.

Proposal 2: It is not supported for gNB to configure UE to use same Tx beams in part of the PRACH transmissions.

Determination of number of PRACH transmissions with different beams

Following agreement was made at the RAN1#123 meeting.

Agreement At least for initial RACH attempt, the determination of total number of PRACH transmissions per RACH attempt is down-selected based on the following options - Option 1: A single RSRP threshold is introduced to determine whether multiple PRACH transmissions with different Tx beams can be applied - Option 2: each of the values of multiple RPSP thresholds corresponding to each of the values of the total number of multiple PRACH transmissions - Option 3: each of the values of multiple RSRP thresholds corresponding to each of the values of the number of repetitions of a RACH transmission per Tx beam. - Option 4: UE determines total number of RACH transmission based on the number of beams it supports. - Other options are not precluded. - Combination of options not precluded.

The motivation of multiple PRACH transmissions with different TX beams is to identify the best UL beam if UE requires TX beam sweeping to achieve beam correspondence capability. Considering there are UEs with different TX beam sweeping capabilities in network, one or multiple values can be configured by SIB/RRC, and UE determines one value based on corresponding TX beam sweeping capability, e.g. the number of supported TX beams. The principle can be applicable for initial RACH attempt and RACH re-attempts.

Proposal 3: For initial RACH attempt and RACH re-attempts, UE determines total number of PRACH transmission in the attempt based on the number of beams it supports.

RAR and Msg 3 beam indication

For Rel-18 multiple PRACH repetitions with same beam, UE monitors only one RAR in RAR window which starts after the last RO. It can be reused for multiple PRACH transmissions with different beams.

Proposal 4: UE monitors single RAR in RAR window which starts after the last RO.

Regarding indication of TX beam for Msg 3 PUSCH, following options were agreed for further study.

Agreement At least for initial Msg3 transmission, support indicating the UL beam information via one of multiple options from following options, for down-selection: - Option 1: based on implicit indication by RA-RNTI - Option 3: based on explicit indication by repurposing field(s) in UL Grant in RAR - Option 4: based on explicit indication by introducing new field in MAC RAR

Option 1 doesn’t need change on RAR fields, but it would result in more UE complexity on RAR monitoring, especially when the number of PRACH transmissions with different TX beams is large, e.g. 8. The UE blind detection complexity would be very high.

For option 4, it looks like a simple solution by adding new field in MAC RAR. It should be based on enhanced MAC subPDU format or a new MAC subPDU format with different subPDU size from legacy format. Since the enhancement is only for Rel-20 CovEnh UEs, legacy UEs can’t recognize such a new/enhanced subPDU format. Considering the MAC PDU of Msg 2 includes MAC RAR of multiple UEs, it may cause understanding ambiguity on subPDUs in Msg 2. If Msg 2 for legacy UEs and Rel-20 UEs are separated, e.g. RAR for multiple PRACH transmissions with different TX beams are put in one Msg 2, and RAR for other PRACH schemes (e.g. single PRACH transmission, or multiple PRACH transmissions with same TX beam) are put in another Msg 2, it is fine. However, it seems not practical assumption for gNB scheduling restriction. Therefore, option 4 is not preferred.

For option 3, no additional field is required. The size of UL grant in RAR or size of MAC RAR is not changed. There is no co-existence issue for legacy UE and Rel-20 CovEnh UE. For a Rel-20 UE who transmitted multiple PRACH transmissions with different beams in the RACH attempt, field(s) in RAR UL grant is/are repurposed for UL beam indication. For a legacy UE or for a Rel-20 UE who is not applying the feature, RAR UL grant is interpreted same as legacy.

Proposal 5: For initial Msg3 transmission, support indicating the UL beam information based on explicit indication by repurposing field(s) in UL Grant in RAR.

The key challenge is selection of field(s) in RAR UL grant for repurposing.

Before discussing specific fields in RAR UL grant, how many bits are needed for UL beam indication should be clarified first. The field length for the UL beam indication can be determined based on the selected number of PRACH transmission beams, or determined based on the NW configured max number of PRACH transmission beams. For the former case, the field length may be different for UEs with different number of TX beams swept during PRACH transmissions. For example, if UE transmitted 2 PRACH transmissions with 2 different TX beams, only 1 bit is needed for repurposing. If UE transmitted 8 PRACH transmissions with 8 different TX beams, 3 bits are needed for repurposing. It can avoid unnecessary bits for repurposing, e.g. remaining bits in the field are used for legacy usage, which can minimize the impact on legacy usage. And it doesn’t cause any ambiguity or coexistence issue.

Moreover, when more than 1 bit for repurposing is needed, it can be further discussed whether the bits for repurposing are from one field (e.g. FDRA, or TDRA, or MCS), or based on a combination of more than one field. For example, for 2 bits for repurposing, 1 bit could be repurposed from FDRA field, and the other 1 bit could be from TPC command field.

Proposal 6: For UL beam indication by repurposing field(s) in RAR UL grant, the number of bits for repurposing is determined based on the selected number of PRACH transmission beams.

- Remaining bits not used for repurposing are interpretated for legacy usage.

- FFS whether one field or multiple fields are selected for repurposing, when the number of bits for repurposing is larger than 1.

Power ramping

At the RAN1#123 meeting, power ramping for multiple PRACH transmissions with different beams was discussed with following agreements.

Agreement For the power ramping between different RACH attempt for multiple PRACH transmission with different Tx beams, down-select from the following options: - Option 1: when any of the Tx beams used in the next attempt is changed, Layer 1 (may) notifies higher layer to suspend the power ramping counter. - Option 2: when all the Tx beams used in the next attempt are changed, Layer 1 notifies higher layer to suspend the power ramping counter - Option 3: when M>1 UL Tx beams used in the next attempt are changed, Layer 1 (may) notifies higher layer to suspend the power ramping counter, FFS the value of M. - Option 4: multiple power ramping counters are used for each PRACH transmission in the RO group.

In legacy, if prior to a PRACH retransmission, a UE changes the spatial domain transmission filter, Layer 1 notifies higher layers to suspend the power ramping counter. For multiple PRACH transmissions with different TX beams, multiple TX beams are used in one RACH attempt. It is not clear when/how to notify higher layer to suspend the power ramping counter.

The principle of legacy rule is that, if same TX beam is used, power ramping is necessary because it is assumed that power is not enough for the direction. If TX beam is changed, the failure in prior attempt doesn’t mean that power for the new beam is not enough, thus no power ramping for this case.

If following similar principle, one straightforward method can be whether power ramping or not is dependent on whether all or only part of TX beams are changed. For example, if all TX beams for next RACH attempts are changed, the failure of previous RACH attempt doesn’t mean the power is not enough for new beams, and hence power ramping may not be necessary. L1 should notify higher layer to suspend the power ramping counter.

Proposal 7: For the power ramping between different RACH attempt for multiple PRACH transmission with different Tx beams, option 2 is supported.

- Option 2: when all the Tx beams used in the next attempt are changed, Layer 1 notifies higher layer to suspend the power ramping counter

2.2 PUSCH repetition scheduled by DCI 0_0 with C-RNTI

At the last RAN1 meeting, the following agreements were reached:

Agreement Support PUSCH repetitions scheduled by DCI format 0_0 with C-RNTI both before and after receiving RRCReconfiguration. Note: Strive for a single mechanism for PUSCH repetition scheduled by DCI 0_0 with C-RNTI Agreement: Support [request or capability report] of PUSCH repetition scheduled by DCI 0_0 with C-RNTI before receiving RRCReconfiguration in Msg3.

As agreed in the first one, irrespective of the status for UE on whether it is before or after receiving RRCReconfiguration, this Msg5 PUSCH repetition is supported. Also, it is noted that we should strive for a single mechanism for the whole feature.

Per the second agreement, at least before RRCReconfiguration, [request or capability report] of PUSCH repetition scheduled by DCI 0_0 with C-RNTI is supported. One remaining issue would be whether to extend this agreement even after RRCReconfiguration. For this issue, we think it is simpler, from specification building point of view, to support this [request or capability report] after RRCReconfiguration. Although we sympathize with the argument that the [request or capability report] will mainly be used for UE transmitting RRC based UE capability signalling (so generally the [request or capability report] will be used before RRCReconfiguration), we still think it isn’t easy to correctly specify this limitation within RAN1 specifications. Hence, our suggestion is not to limit the occurrence of the [request or capability report] itself, and let UE implementation decides whether to send it back to NW. From NW point of view, anyway we need to rely on UE request/report to enable any optional features, so this suggestion is essentially equivalent to how we handle optional features in general, with minimized specification efforts. Note that the RAN2 TU is rather small, which also motivates us not to go much further to the direction bringing specification development needs.

Proposal 8: Support [request or capability report] of PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled by C-RNTI even after receiving RRCReconfiguration in Msg3

- Note: It is up to UE implementation, irrespective of pre- or post-RRCReconfiguration, whether to have the [request or capability report] in Msg3

There was also another progress in the last RAN1 meeting as follows, by which we could narrow down the alternatives for indication of the number of repetitions of PUSCH scheduled by DCI 0_0 with C-RNTI:

Agreement Down-select one of the following options to indicate the number of repetitions of PUSCH scheduled by DCI 0_0 with C-RNTI both before and after receiving RRCReconfiguration - Option 1: Using at most 2 MSB of MCS field in DCI format 0_0 with CRC scrambled by C-RNTI - FFS: details - Option 2: Based on TDRA - FFS: details

Among the remaining two options on the table, we are in general open for either one. Our assessment of the two options can be described as follows:

- For option 1, the high-level concept is quite similar to Rel-17 Msg3 repetition. With the sacrifice of available MCS indexes, a repetition factor is indicated without any other changes. Since the main scenario for this feature will be similar to Rel-17 Msg3 repetition, we think this direction is to some extent reasonable.

- For option 2, since the target PUSCH is transmitted only when at least cell-specific RRC configuration is available for the UE, our understanding is that for almost all the usages of this feature, TDRA doesn’t rely on default table, rather we can expect that cell-specific PUSCH configuration is available. Therefore, we believe this option also makes sense given its impact is likely to be limited to RRC parameter introduction. Meanwhile, if it is deemed difficult to ensure this Msg5 repetition occurs when and only when cell-specific PUSCH configuration is available (i.e., need to consider some impacts on default TDRA table), the level of interest toward this option will disappear.

At this stage, our slight preference goes to Option 1. It is because in the scenario in which PUSCH repetition scheduled by DCI 0_0 with C-RNTI is needed, the UE is expected to be located around cell edge. Hence, it is likely not required to indicate higher order modulation scheme for the PUSCH transmission. Also, this impact is still UE-specific. Although option 2 is also reasonable with small specification impact, Option 2 will require some impacts on cell-specific RRC parameters, which may be a bit too huge at this late stage of NR operation.

Proposal 9: Support Option 1: Using at most 2 MSB of MCS field in DCI format 0_0 with CRC scrambled by C-RNTI, to indicate the number of repetitions for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI

Also, candidate number of repetitions should also be decided as it impacts the needed bitwidth for indication. We believe the main intention of this objective would be to unlock PUSCH repetition (which is already available for a lot of UEs for other types of PUSCH) for the one scheduled by DCI format 0_0 with CRC scrambled by C-RNTI. Hence, for repetition factor, we believe it would be the starting point to consider {2, 4, 8}, which is analogous to Rel-15 PUSCH repetition. Especially for TN scenario, we view these factors can be enough. Meanwhile, for NTN scenario, we may need additional larger value, e.g., 16. The additional one value isn’t really a problem in our view.

Proposal 10: Regarding PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled by C-RNTI,

- Support repetition factors {2, 4, 8}

  - For NTN scenario, additional larger value can be considered, e.g., 16

2.3 Extension of pi/2-BPSK to more MCS entries

Given the good progress and thanks to companies’ effort with evaluation, the progress of this issue seems very good. Based on that, it seems RAN1 already identified that SE wise, it is beneficial to support this feature for MCS indexes with SE equal to N=0.8770 or smaller in legacy basic MCS table. It is also observed that for MCS indexes with SE smaller than 0.8770, a visible gain would be there, and hence RAN1 agreed to support these cases for this feature. A remaining issue is whether/how to support this feature for MCS with SE N=0.8770, as agreed as follows:

Agreement For extension of pi/2-BPSK to more MCS entries, down-select one of the following Options: - Option 1：Pi/2-BPSK is extended to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than N = 0.8770. - gNB can configure the maximum MCS up to which pi/2 BPSK is applicable. - Details on configuration to be discussed further - FFS: UE signaling to the gNB for this configuration - FFS: whether/how to provide assistance information and what is the assistance information - Option 2：Support extending Pi/2-BPSK to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than X. - Where X the fixed value select from: a) X = 0.8770 b) X = 0.6016 - Extension is captured in the modified MCS table. - gNB enables/disables this feature via RRC signalling.

Although we are fine with either option, we would like to emphasize that this feature itself should not cause too many UE capability signallings, due to the simplicity of the whole feature. In that sense, our slight preference would be to go with Option 1 as it seems companies are interested in having multiple UE capabilities in case of Option 2 (e.g., a capability supporting up to N=0.6016, and the other one for N=0.8770 additionally). If Option 2 like configuration is deemed necessary, we prefer not to fragment the resulted UE capabilities.

For the enhanced MCS tables (i.e., whether to support URLLC table), we have no strong opinion either. Although we do not see the strong need to extend this feature for URLLC table, given the additional specification impact is expected to be quite small, we are fine to support URLLC table as well.

Proposal 11: For extension of pi/2-BPSK to more MCS entries,

- Slightly prefer Option 1 in the following agreement:

Agreement For extension of pi/2-BPSK to more MCS entries, down-select one of the following Options: - Option 1：Pi/2-BPSK is extended to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than N = 0.8770. - gNB can configure the maximum MCS up to which pi/2 BPSK is applicable. - Details on configuration to be discussed further - FFS: UE signaling to the gNB for this configuration - FFS: whether/how to provide assistance information and what is the assistance information - Option 2：Support extending Pi/2-BPSK to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than X. - Where X the fixed value select from: a) X = 0.8770 b) X = 0.6016 - Extension is captured in the modified MCS table. - gNB enables/disables this feature via RRC signalling.

- On whether to support Table 6.1.4.1-2 for the extension, slightly prefer not to support Table 6.1.4.1-2 given the smaller need, while open to discuss

- Conclusion

In this contribution, we discussed further coverage enhancements for PRACH and PUSCH. We have the following proposals:

Proposal 1: UE determines whether to select PRACH resource for multiple PRACH transmissions with different beams based on UE capability of beam correspondence with or without TX beam sweeping.

- If UE supports none of Rel-15/16 beam correspondence without TX beam sweeping, UE selects PRACH resource for multiple PRACH transmissions with different beams.

- If UE supports at least one beam correspondence without TX beam sweeping out of Rel-15/16 beam correspondence capabilities, UE selects PRACH resource for single PRACH transmission or PRACH resource for PRACH repetitions with same beam, based on RSRP threshold for Msg 1 repetition.

Proposal 2: It is not supported for gNB to configure UE to use same Tx beams in part of the PRACH transmissions.

Proposal 3: For initial RACH attempt and RACH re-attempts, UE determines total number of PRACH transmission in the attempt based on the number of beams it supports.

Proposal 4: UE monitors single RAR in RAR window which starts after the last RO.

Proposal 5: For initial Msg3 transmission, support indicating the UL beam information based on explicit indication by repurposing field(s) in UL Grant in RAR.

Proposal 6: For UL beam indication by repurposing field(s) in RAR UL grant, the number of bits for repurposing is determined based on the selected number of PRACH transmission beams.

- Remaining bits not used for repurposing are interpretated for legacy usage.

- FFS whether one field or multiple fields are selected for repurposing, when the number of bits for repurposing is larger than 1.

Proposal 7: For the power ramping between different RACH attempt for multiple PRACH transmission with different Tx beams, option 2 is supported.

- Option 2: when all the Tx beams used in the next attempt are changed, Layer 1 notifies higher layer to suspend the power ramping counter

Proposal 8: Support [request or capability report] of PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled by C-RNTI even after receiving RRCReconfiguration in Msg3

- Note: It is up to UE implementation, irrespective of pre- or post-RRCReconfiguration, whether to have the [request or capability report] in Msg3

Proposal 9: Support Option 1: Using at most 2 MSB of MCS field in DCI format 0_0 with CRC scrambled by C-RNTI, to indicate the number of repetitions for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI

Proposal 10: Regarding PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled by C-RNTI,

- Support repetition factors {2, 4, 8}

  - For NTN scenario, additional larger value can be considered, e.g., 16

Proposal 11: For extension of pi/2-BPSK to more MCS entries,

- Slightly prefer Option 1 in the following agreement:

Agreement For extension of pi/2-BPSK to more MCS entries, down-select one of the following Options: - Option 1：Pi/2-BPSK is extended to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than N = 0.8770. - gNB can configure the maximum MCS up to which pi/2 BPSK is applicable. - Details on configuration to be discussed further - FFS: UE signaling to the gNB for this configuration - FFS: whether/how to provide assistance information and what is the assistance information - Option 2：Support extending Pi/2-BPSK to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than X. - Where X the fixed value select from: a) X = 0.8770 b) X = 0.6016 - Extension is captured in the modified MCS table. - gNB enables/disables this feature via RRC signalling.

- On whether to support Table 6.1.4.1-2 for the extension, slightly prefer not to support Table 6.1.4.1-2 given the smaller need, while open to discuss

References

- RP-251862, New WID: Coverage enhancements for NR Phase 3

- RAN1#123 Chairman’s notes, Nov, 2025