3GPP TSG RAN WG1 #124			R1-2601176

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda item:	10.2.1

Title:	Discussion on Waveform

Source:	NTT DOCOMO, INC.

Document for:	Discussion and Decision

- Introduction

In the previous RAN1#123, the evaluation assumptions for both UL low PAPR proposals and multi-layer PUSCH based on DFT-s-OFDM were discussed and reached agreements [1]. Including,

- UL low PAPR proposals: Details are shown in Appendix A.

  - Six agreements for link-level evaluation assumptions, including evaluation metric, general evaluation assumptions, single user evaluation assumption for MCS and subcarriers UL low-PAPR proposals with spectrum extension (SE), spectrum truncation (ST), and tone reservation (TR), the occupied BW, as well as the link-level multi-user evaluation assumption for UL low-PAPR proposals.

  - One agreement for the table template to characterize each proposal.

- Multi-layer PUSCH based on DFT-s-OFDM: Details are shown in Appendix B.

  - Two agreements for both the link-level and system-level evaluation assumptions for multi-layer PUSCH based on DFT-s-OFDM.

In this contribution, we first discuss the low PAPR proposals for UL coverage enhancement, including DFT-s-OFDM with SE and DFT-s-OFDM with ST for pi/2-BPSK in Section 2. Multi-layer PUSCH based on DFT-s-OFDM is discussed in Section 3. Afterwards, we show our views on DL DFT-s-OFDM and other waveforms, e.g., OTFS, in Section 4. Lastly, this contribution is summarized in Section 5.

- Low PAPR proposals for UL coverage enhancement

  - Motivation

Now that mobile communication has become an important infrastructure for a lot of people, MNO’s responsibility is getting more and more important to ensure connectivity in wider areas. To achieve this, we have deployed a bunch number of BSs, spending a lot of cost and effort. It would always be important to reduce the sources of such a cost and effort. Extension of site coverage is a promising approach since per-area BSs can be reduced without having a coverage hole. Note that, to practically achieve this, it is essential to make sure that such an extended coverage is supported from the perspective of any UE, not only NW.

A typical approach to extend coverage is repetition of the UL signal/channel, which has been implemented and operated in real fields already. While we believe this is still promising in 6GR (and so we are proposing to extend the specification support of repetition from 6G Day 1), there is indeed a drawback, e.g., UL resource efficiency. Especially for TDD band, since it isn’t easy (even not practical) to increase the UL ratio, this drawback might be a significant bottleneck for the whole UL performance.

To satisfy the following 6G requirements as listed in 6GR SID [2]:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage.

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

- Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

Low PAPR enhancement based on NR waveforms with better coverage may be necessary for 6GR, especially for UL. Meanwhile, we believe it should be carefully discussed together with other perspectives, such as the achievable net gains, link budget, and system-wise performance.

  - Candidate schemes for DFT-s-OFDM with SE

DFT-s-OFDM is adopted in the 5G NR UL when coverage is limited due to the low PAPR performance. DFT-s-OFDM can be considered as a scheme that upsamples the modulated symbols by a factor equal to the ratio of the inverse fast Fourier transform (IFFT) and DFT block sizes and applies a circular-shaped pulse before the CP insertion. The circular-shaped pulse of DFT-s-OFDM is the Dirichlet sinc function. However, the heavily overlapped pulse-sidelobes of the Dirichlet sinc pulses can create large peaks in the time domain when combined coherently, which also results in poor PAPR performance.

DFT-s-OFDM with frequency domain spectrum shaping (FDSS) can reduce the PAPR by introducing a well-designed low-complexity frequency-domain filter to change the original time-domain Dirichlet sinc function of DFT-s-OFDM. FDSS combined with the low PAPR modulation pi/2 phase-rotated binary phase-shift keying (pi/2-BPSK) for DFT-s-OFDM has been adopted in NR since R15 for uplink coverage-limited scenarios. In R18, FDSS combined with the quadrature PSK (QPSK) for DFT-s-OFDM has also been supported for NR uplink coverage enhancement. Although DFT-s-OFDM with FDSS can reduce the PAPR, the resulting inter-symbol interference (ISI) will impact the quality of the received signal.

DFT-s-OFDM-based enhancements with SE can increase the time separation between adjacent time-domain pulse shapes, thereby reducing ISI. Furthermore, the PAPR is further reduced by the SE operation. Both symmetric SE and asymmetric SE can be considered.

    - FDSS-SE

FDSS-SE refers to the DFT-s-OFDM enhancement with symmetric SE,  a scheme also proposed in  [4], whose transmitter block diagram is shown inn Figure 2-1.

Figure 2-1.  FDSS-SE transmitter block diagram

The symmetric SE is achieved by introducing the SE module that follows the DFT spreading module.  Within the allocated M frequency-domain subcarriers, there are only Q≤M subcarriers carrying data. The other M-Q subcarriers are the excess band to realize symmetric SE, and α=MQ  refers to the SE factor.

For FDSS-SE, the time separation between time-domain shaped pulses of DFT-s-OFDM increases based on the symmetric SE, thereby reducing ISI. Additionally, the PAPR is further reduced as the adjacent pulses have larger time separation. However, an equivalent ejπα-1m phase rotation is also introduced for the m-th time-domain shaped-pulses by FDSS-SE, which will affect the PAPR performance. Moreover, when an odd number of RB is allocated for SE, FDSS-SE imposes a fractional-RB extension on each side.  Another potential issue for FDSS-SE could be that the double-sided SE increases implementation complexity.

Table 2-1. Characterization of FDSS-SE

|  | Description |
| --- | --- |
| Name of the proposal | FDSS-SE |
| Motivation of the proposal | TN, NTN |
| Applicable link direction | UL |
| Enhancement to CP-OFDM? | No |
| Enhancement to DFT-s-OFDM? | Yes |
| Additional OFDM-compatible waveform? | No |
| Target channel(s)/signal(s) | PUSCH |
| Target modulation | QPSK/QAM |
| Motivation/use case | Improved uplink coverage |
| Key Metric / KPI | Net gain |
| Key spec impact foreseen | RAN1 definition of the spectrum extension RAN4 requirements characterizing the filter |
| MRSS compatibility | The same as the NR baseline waveform DFT-s-OFDM. |
| Multiplexing/coexistence with other waveforms | Can easily multiplex/coexist with the baseline waveforms |
| Multi-user multiplexing | The same as the NR baseline waveform DFT-s-OFDM |
| MIMO compatibility | The same as the NR baseline waveform DFT-s-OFDM |

    - FDSS-CE with CS

FDSS-CE with cyclic shift (CS) is an enhanced scheme of FDSS-CE [5] by introducing CS and frequency-domain cyclic extension (FD-CE) modules after symmetric DFT spreading. The former applies a frequency-domain cyclic shift according to a specified CS value, and the latter performs an asymmetric SE. FDSS-CE with CS can achieve flexible S, its transmitter block diagram is shown in Figure 2-2.

Figure 2-2. FDSS-CE with CS transmitter block diagram

The detailed procedure for FDSS-CE with CS is provided below,

- DFT spreading: Apply Q-point DFT on Q modulated symbols xn to get Xk

Xk=0Q-1xne-j2πnkQ,  k=0,…, Q-1

- CS: Apply L-point CS to Xk according to

XCSk=Xk+L mod Q,  k=0,…, Q-1

- FD-CE: Applying cyclic extension to extend Q-length XCSk to M-length XCEkaccording to

XCEk=XCSk mod Q,  k=0,…, M-1

- Then, FDSS, subcarrier mapping, IFFT, and CP are implemented to obtain the final time-domain signal.

Like FDSS-SE, the time separation between time-domain shaped pulses of DFT-s-OFDM increases based on the asymmetric SE, thereby reducing ISI. Additionally, the PAPR is further reduced as the adjacent pulses have larger time separation [6].

Based on FDSS-CE with CS, asymmetric SE, symmetric SE, as well as SE combining with QPSK rotation can be achieved by configuring the suitable CS value L. For example,

- Asymmetric SE with L=0: This is equivalent to the FDSS-CE in [7].

- Symmetric SE with  L=1-32αM: This is equivalent to the FDSS-SE.

  - α=M-QM represents the SE factor

- Asymmetric SE combining with QPSK rotation with  L=-5-α8M-12: This is equivalent to QPSK rotation as defined in [7].

Compared to FDSS-SE, which only supports symmetric SE, FDSS-CE with CS achieves flexible SE and equivalent modulation rotation with better performance.

Table 2-2. Characterization of FDSS-CE with CS

|  | Description |
| --- | --- |
| Name of the proposal | FDSS-CE with CS |
| Motivation of the proposal | TN, NTN |
| Applicable link direction | UL |
| Enhancement to CP-OFDM? | No |
| Enhancement to DFT-s-OFDM? | Yes |
| Additional OFDM-compatible waveform? | No |
| Target channel(s)/signal(s) | PUSCH |
| Target modulation | QPSK/QAM |
| Motivation/use case | Improved uplink coverage |
| Key Metric / KPI | Net gain |
| Key spec impact foreseen | RAN1 definition of the spectrum extension RAN4 requirements characterizing the filter |
| MRSS compatibility | The same as the NR baseline waveform DFT-s-OFDM. |
| Multiplexing/coexistence with other waveforms | Can easily multiplex/coexist with the baseline waveforms |
| Multi-user multiplexing | The same as the NR baseline waveform DFT-s-OFDM |
| MIMO compatibility | The same as the NR baseline waveform DFT-s-OFDM |

    - Evaluation results for net gain

Net Gain, defined as follows, has been agreed as the link-level performance evaluation criterion in RAN1#122bis.

Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER.

However, some details for calculating the Tx power gain and coding rate for different proposals still need further clarification.

- Tx power gain calculation

The lowest output back off (OBO) or the maximum power reduction (MPR) that satisfies the RF requirements provides a more comprehensive assessment for power efficiency, which jointly considers both the nonlinear PA impact and the PAPR distribution.

The OBO or MPR is calculated as

MPRdB=PsatdBm-PoutdBm,

where Psat and Pout are the PA saturation power and actual mean output power. The OBO/MPR gain, i.e., ∆MPR between the low PAPR proposal and the baseline, should be used as the TX power gain.

Based on the agreement in RAN1#123, the following three UE RF requirements should be considered for the OBO/MPR evaluation, whose definitions can also be found in [3].

- Error vector magnitude (EVM): The RMS average of the difference between the reference waveform and the measured waveform.

- Adjacent channel leakage ratio (ACLR): The ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

- In-band emission (IBE):  The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

- Coding rate calculation

To keep the similar spectral efficiency under the same occupied bandwidth for DFT-s-OFDM w/ and w/o SE, the coding rate under the same modulation order is calculated as

CRSE =CRDFT1-α,

where CRDFT represents the coding rate for DFT-s-OFDM, and CRSE represents the coding rate when SE is considered.

Under the link-level evaluation assumptions detailed in Table 2-3 and Table 2-4.

Table 2-3. Link-level evaluation assumptions for Low PAPR proposals

| Parameter | Values |
| --- | --- |
| Carrier frequency and scenario | 4GHz |
| Channel BW | 100MHz |
| RB allocation | Outer/Inner RB allocation |
| SCS | 30 kHz for 4GHz |
| Channel model | TDL-C 300ns for 4GHz |
| UE speed | 3km/h |
| Number of Tx antennas for TDL channel | 1 |
| Number of Rx antennas for TDL channel | 1 and 4 for 4GHz |
| Number of DMRS symbols/slot (location as defined in NR) | 2 |
| Number of PUSCH data symbols/slot | 12 |
| HARQ configuration | No retransmissions |
| Frequency hopping | Disabled |
| RF requirements for Tx power gain | ACLR, EVM, IBE |
| PA model for Tx power gain | Polynomial PA model as shown in [R1-166004] |

Table 2-4. Evaluation assumptions for occupied BW, SE factor, and CR for DFT-s-OFDM with SE proposals

| Waveform | FDSS type | SE factor α=M-QM | Occupied BW (RB) | Modulation | CR |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | NONE | 1 | 8,32,128,256 | QPSK | 120/1024 |
| DFT-s-OFDM with FDSS | TRRC with (0.5, 0.1667) | 1 | 8,32,128,256 | QPSK | 120/1024 |
| SE-Scheme 1 SE-Scheme 2 | 2 types of FDSS: - 3-tap [-0.335 1 -0.335] - TRRC with (0.5, 0.1667) | 1/6 | 8,32,128,256 | QPSK | 144/1024 |
|  |  | 1/4 | 8,32,128,256 | QPSK | 160/1024 |
|  |  | 3/8 | 8,32,128,256 | QPSK | 192/1024 |
|  |  | 7/16 | 8,32,128,256 | QPSK | 214/1024 |
|  |  | 1/2 | 8,32,128,256 | QPSK | 240/1024 |

Two SE schemes are considered in our evaluation,

- SE scheme 1: Symmetric SE, which can be achieved based on FDSS-CE with CS ( L=1-32αM) or FDSS-SE

- SE scheme 2: Asymmetric SE, which can be achieved based on FDSS-CE with CS ( L=0).

Two FDSS filter types are evaluated for the two SE schemes: a less aggressive TRRC filter and a more aggressive 3-tap filter [-0.335, 1, -0.335]. The net gains presented below are the maximum values achieved by either filter type for each scheme.

Case 1: Net gain under different SE factors

The net gain of the two SE schemes relative to the NR baseline DFT-s-OFDM is evaluated under different SE factors, based on the assumptions of equal occupied bandwidth and outer RB allocation. Figure 2-3 (a) and Figure 2-3 (b) present the net gain performance for 1Rx and 4Rx antenna configurations, respectively.

It can be found that both DFT-s-OFDM with FDSS and the proposed SE schemes (FDSS-SE and FDSS-CE with CS) achieve a net gain over the NR baseline DFT-s-OFDM. While DFT-s-OFDM with FDSS alone only provides a limited gain of less than 0.15 dB, the introduction of SE leads to significantly greater improvements. The net gain of the SE schemes initially increases with the SE factor before declining, peaking at 1.05 dB for 1Rx and 1.28 dB for 4Rx when the SE factor alpha = 3/8. Furthermore, depending on the SE factor, the maximum gain is attained by either symmetric or asymmetric SE.

| - 1Rx | - 4Rx |
| --- | --- |

Figure 2-3. Net gain of two SE schemes for different SE factors under 32RB

Case 2: Net gain under different occupied bandwidths

The net gain of two SE schemes relative to the NR baseline DFT-s-OFDM is evaluated under different occupied bandwidths based on the assumptions of the same SE factor and outer RB allocation.  Figure 2-4 (a) and Figure 2-4 (b) show the net gain performance under 1Rx antenna and 4Rx antennas, respectively.

It can be found that DFT-s-OFDM with FDSS does not always yield net gains—for instance, no net gain is observed under 8RB bandwidth for 4Rx. In contrast, the SE schemes (FDSS-SE and FDSS-CE with CS) consistently achieve net gains across all bandwidths. The net gain of the SE schemes increases with the occupied bandwidth, reaching up to 0.83 dB for 1Rx and 1.13 dB for 4Rx when the occupied bandwidth is 256RB. Furthermore, depending on the occupied bandwidth, the maximum gain is achieved by either symmetric or asymmetric SSE.

| - 1Rx | - 4Rx |
| --- | --- |

Figure 2-4. Net gain of two SE schemes for different occupied BWs under alpha =1/6

Case 3: Net gain under different RB allocations

The net gain of the two SE schemes relative to the NR baseline DFT-s-OFDM is evaluated under different RB allocation based on the assumption of equal occupied bandwidth. Figure 2-5 and Figure 2-6 present the net gain performance for 1Rx and 4Rx antenna configurations, respectively.

It can be found that DFT-s-OFDM with FDSS does not always produce net gains—for example, no net gain is observed under inner RB allocation for 1Rx. In contrast, the SE schemes (FDSS-SE and FDSS-CE with CS) consistently achieve net gains across both outer and inner RB allocations. Specifically, when the SE factor is relatively small (e.g., less than 1/4), inner RB allocation yields a larger net gain than outer RB allocation; conversely, with a larger SE factor (e.g., greater than 1/4), outer RB allocation results in a higher net gain than inner RB allocation.

| - 1Rx, Outer RB allocation | - 1Rx, Inner RB allocation |
| --- | --- |

Figure 2-5. Net gain of two SE schemes for different RB allocations under 32RB, 1Rx

| - 4Rx, Outer RB allocation | - 4Rx, Inner RB allocation |
| --- | --- |

Figure 2-6. Net gain of two SE schemes for different RB allocations under 32RB, 4Rx

Observation 2-1: The transparent scheme, DFT-s-OFDM with FDSS, does not always produce net gains.

Observation 2-2: DFT-s-OFDM enhanced with SE, such as FDSS-SE and FDSS-CE with CS, can achieve net performance gains consistently, regardless of the SE factor, occupied bandwidth, or RB allocation.

Observation 2-3: DFT-s-OFDM enhanced with SE can achieve much larger net gains than DFT-s-OFDM with FDSS.

Observation 2-4: The maximum net gain is achieved by either symmetric or asymmetric SE, depending on the SE factor, occupied bandwidth, and RB allocation.

Case 4: Net gain comparison between FDSS-SE and FDSS-CE with CS

Figure 2-7 and Figure 2-8 illustrate the net gain performance of FDSS-SE and FDSS-CE with CS under 1Rx and 4Rx antenna configurations, respectively. The results indicate that, owing to the flexible symmetric/asymmetric SE capability, FDSS-CE with CS consistently achieves a net gain that is no less than that of FDSS-SE under all evaluated assumptions.

| - Different SE factors, Outer RB allocation | - Different SE factors, Inner RB allocation | - Different bandwidths, Outer RB allocation |
| --- | --- | --- |

Figure 2-7. Net gain comparison between FDSS-SE and FDSS-CE with CS, 1R

| - Different SE factors, Outer RB allocation | - Different SE factors, Inner RB allocation | - Different bandwidths, Outer RB allocation |
| --- | --- | --- |

Figure 2-8. Net gain comparison between FDSS-SE and FDSS-CE with CS, 4Rx

Observation 2-5: FDSS-CE with CS offers flexible (symmetric/asymmetric) SE, which can achieve better net gain than FDSS-SE.

Proposal 2-1

- Support the low PAPR proposals based on DFT-s-OFDM with SE, including FDSS-CE with CS and FDSS-SE, to improve uplink coverage.

  - Candidate schemes for DFT-s-OFDM with ST under pi/2-BPSK

DFT-s-OFDM based on pi/2-BPSK can achieve better coverage than based on QPSK due to the lower PAPR. However, the spectrum efficiency for pi/2-BPSK is lower. The spectrum efficiency of pi/2-BPSK can be improved based on frequency-domain ST. This is thanks to a certain frequency domain symmetry property of DFT-s-OFDM waveforms with pi/2-BPSK, which can be expressed as:

- For the BPSK modulation based on the assumption of sBPSKm∈1, -1, there is

Sk =SL2-k*,                 0≤k≤L2S3L2-k*,L2+1≤k≤L-1

- For the BPSK modulation based on the assumption of sBPSKm∈121+j, -1-j, there is

Sk =e-jπ2SL2-k*,            0≤k≤L2e-jπ2S3L2-k*, L2+1≤k≤L-1

Where Sk are the frequency domain samples and L is the DFT size.

So within in L DFT output frequency domain samples, up to L2-1 DFT samples can be dropped based on frequency-domain ST to increase the spectrum efficiency of pi/2-BPSK without any loss of information. However, the PAPR performance loss is introduced due to the frequency-domain ST. The larger the frequency-domain ST size/ratio, the larger the PAPR performance loss [9]. Therefore, frequency-domain ST size/ratio should be designed carefully to achieve a tunable trade-off between PAPR and spectrum efficiency. There are two methods to achieve frequency-domain ST, as described as follows.

    - DFT-s-OFDM with truncated mapping

DFT-s-OFDM with truncated mapping realizes frequency-domain ST based on the frequency-domain truncated mapping after asymmetric DFT spreading, as shown in Figure 2-7. The symmetric DFT spreading requires higher computational complexity because L-to-L point DFT spreading is required.

Figure 2-9. DFT-s-OFDM with truncated mapping

Table 2-5. Characterization of DFT-s-OFDM with truncated mapping

|  | Description |
| --- | --- |
| Name of the proposal | DFT-s-OFDM with truncated mapping |
| Motivation of the proposal | TN, NTN |
| Applicable link direction | UL |
| Enhancement to CP-OFDM? | No |
| Enhancement to DFT-s-OFDM? | Yes |
| Additional OFDM-compatible waveform? | No |
| Target channel(s)/signal(s) | PUSCH |
| Target modulation | pi/2-BPSK |
| Motivation/use case | Improved uplink coverage |
| Key Metric / KPI | Net gain |
| Key spec impact foreseen | RAN1 definition of the spectrum truncation |
| MRSS compatibility | The same as the NR baseline waveform DFT-s-OFDM. |
| Multiplexing/coexistence with other waveforms | Can easily multiplex/coexist with the baseline waveforms |
| Multi-user multiplexing | The same as the NR baseline waveform DFT-s-OFDM |
| MIMO compatibility | The same as the NR baseline waveform DFT-s-OFDM |

    - Asymmetric DFT-s-OFDM

Asymmetric DFT-s-OFDM realizes frequency-domain ST by replacing the symmetric DFT spreading with asymmetric DFT spreading whose output size K is smaller than the input size L, as shown in Figure 2-8.

The element fk,l in the k-th row and l-th column of the asymmetric DFT matrix can be represented as

fk,l=e-j2πlk+EL, 0≤k≤K-1, ≤l≤L-1

Where E is the ST size at the head of frequency-domain samples. To avoid information loss, the value of E should satisfy E≤L4. Thanks to the L-to-K point asymmetric DFT spreading, the computational complexity is lower than that of DFT-s-OFDM with truncated mapping.

Figure 2-10. Asymmetric DFT-s-OFDM

Table 2-6. Characterization of Asymmetric DFT-s-OFDM

|  | Description |
| --- | --- |
| Name of the proposal | Asymmetric DFT-s-OFDM |
| Motivation of the proposal | TN, NTN |
| Applicable link direction | UL |
| Enhancement to CP-OFDM? | No |
| Enhancement to DFT-s-OFDM? | Yes |
| Additional OFDM-compatible waveform? | No |
| Target channel(s)/signal(s) | PUSCH |
| Target modulation | pi/2-BPSK |
| Motivation/use case | Improved uplink coverage |
| Key Metric / KPI | Net gain |
| Key spec impact foreseen | RAN1 definition of the spectrum truncation |
| MRSS compatibility | The same as the NR baseline waveform DFT-s-OFDM. |
| Multiplexing/coexistence with other waveforms | Can easily multiplex/coexist with the baseline waveforms |
| Multi-user multiplexing | The same as the NR baseline waveform DFT-s-OFDM |
| MIMO compatibility | The same as the NR baseline waveform DFT-s-OFDM |

Observation 2-6: DFT-s-OFDM with ST based on truncated mapping and/or asymmetric DFT spreading can achieve a tunable trade-off between PAPR and spectrum efficiency.

Proposal 2-2

- Support the low PAPR proposals based on DFT-s-OFDM with ST, including  DFT-s-OFDM with truncated mapping and asymmetric DFT-s-OFDM, to improve uplink coverage.

  - Discussions on other UL low PAPR proposals

As outlined in [9], the I/Q-offset DFT-s-OFDM scheme reduces PAPR for QPSK/QAM by applying a one-symbol delay between in-phase and quadrature components, producing sequences equivalent to pi/2-BPSK. This approach, however, increases complexity: frequency-domain implementations require extra processing to separate real and imaginary parts, while time-domain methods may demand larger DFT spreading sizes.

In [10], I-modulation DFT-s-OFDM was introduced to lower PAPR by smoothing phase transitions between pi/2-BPSK or QAM symbols, using either time-domain linear interpolation or a corresponding frequency-domain FDSS. A key limitation of the time-domain interpolation is the need for a larger DFT spreading size, which raises complexity, while the frequency-domain non-transparent FDSS requires additional signalling overhead.

DFT-s-OFDM with time-domain spreading, proposed in [11], follows a similar principle to I-modulation. In fact, I-pi/2-BPSK can be considered a special case of time-domain spreading with a specific spreading filter. Consequently, this method also suffers from increased complexity due to the larger DFT spreading size. Furthermore, the two-fold spectrum extension leads to reduced spectral efficiency.

- Multi-layer PUSCH based on DFT-s-OFDM

In 5G NR, DFT-s-OFDM supports only single-layer transmission in the uplink, while CP-OFDM is used for multi-layer transmission. Compared to CP-OFDM, DFT-s-OFDM offers higher transmit power capability due to its lower PAPR, which helps achieve better coverage and throughput. In 6GR, the NR DFT-s-OFDM waveform is agreed as the basis waveform for UL. Considering that 6G demands even higher capacity and more extensive coverage than 5G NR, it is necessary to study multi-layer UL MIMO transmission based on DFT-s-OFDM.

To evaluate the performance gains of uplink multi-layer DFT-s-OFDM compared to CP-OFDM, we first compare the two waveforms in uplink multi-layer transmission scenarios through link-level evaluations.

  - Link-level evaluation performance

Under the link-level evaluation assumptions detailed in Table 3-1, the BLER performance of both multi-layer DFT‑s‑OFDM and multi‑layer CP‑OFDM is evaluated, along with the net performance gains of multi-layer DFT‑s‑OFDM relative to CP‑OFDM.

Table 3-1. Link-level evaluation assumptions for UL multi-layer DFT-s-OFDM

| Parameters | Values (agreed in RAN1#123) | Note (Our assumption) |
| --- | --- | --- |
| Carrier frequency​​ | 4 GHz |  |
| Subcarrier spacing​​ | 30 kHz​​ |  |
| UE antenna ports | 2, 4 | 2-port prioritized |
| BS antenna ports | 64, port reduction is not precluded |  |
| FDRA | 4, 8, 16, 32, 64​​ | 4 and 64 RB prioritized |
| Waveform​​s | CP-OFDM DFT-s-OFDM​​ |  |
| MCS/modulation​​ | NR UL MCS table with 256 QAM | MCS 4(QPSK),10(16QAM),9(64QAM),27(64QAM) |
| Number of layers​ | 2 layers​​, 4 layers | 2 layers prioritized |
| Channel model​​ | CDL-A30​​, TDL-C300, CDL-C300, | CDL-A30 prioritized |
| UE speed​​ | 3km/h​​, 30 km/h | 3km/h prioritized |
| Channel est.​​ | Practical​ |  |
| SRS periodicity | To be reported by the company |  |
| Receiver | LMMSE |  |
| HARQ retransmission​ | Disabled |  |
| DMRS configuration | Configuration type 1 2 DMRS symbols per slot |  |
| Number of PUSCH data | 12 symbols |  |
| Waveform and MIMO configuration | 5G codebook, to be reported by the company |  |
| BLER target | 10% |  |
| Frequency hopping | Disabled |  |
| Power class and power mode | Option 1: PC2, total power limited to 26 dBm. - MaxRank 2: Each PA is limited to 23 dBm - MaxRank 4: Each PA is limited to 20 dBm Option2: PC3, total power limited to 23 dBm. - MaxRank 2: Each PA is limited to 20 dBm - MaxRank 4: Each PA is limited to 17 dBm | Option 1 and 2 are considered for net gain calculation of 2-layer. Option 2 is considered for net gain calculation of 4-layer. |

The BLER evaluation results for 2-layer and 4-layer UL MIMO using non-coherent precoders with a diagonal matrix are presented in Figure 3-1 and Figure 3-2, respectively.

| - BLER performance under 4PRB | - BLER performance under 64PRB |
| --- | --- |

Figure 3-1. BLER performance for 2-layer DFT-s-OFDM and 2-layer CP-OFDM under different MCS

| - BLER performance under 4PRB | - BLER performance under 64PRB |
| --- | --- |

Figure 3-2. BLER performance for 4-layer DFT-s-OFDM and 4-layer CP-OFDM under different MCS

As shown in the BLER performance in Figure 3-1(a)(b) and Figure 3-2(a)(b), the BLER performance of DFT-s-OFDM is no worse than that of CP-OFDM across all modulation orders/types and bandwidths for both 2-layer and 4-layer cases.

Observation 3-1: With a non-coherent precoder, DFT-s-OFDM achieves at least the same performance as CP-OFDM in terms of BLER for both 2-layer and 4-layer scenarios.

| - PC2 UEs with 2Tx | - PC3 UEs with 2Tx |
| --- | --- |

Figure 3-3. Tx power gain between 2-layer DFT-s-OFDM and 2-layer CP-OFDM

The net gain metric, defined for single-layer low PAPR proposals, is also applicable to the multi-layer scenario.

From Figure 3-1(a)(b) and Figure 3-2(a)(b), we can find that there is no SNR degradation for DFT-s OFDM compared to CP OFDM for both 2-layer and 4-layer scenarios. Therefore, the net gain equals the Tx power gain.

The Tx power gain between 2-layer DFT-s-OFDM and 2-layer CP-OFDM is shown in  Figure 3-3. Figure 3-3(a) is derived from the MPR values listed in Table 6.2D.2-1 of [3]. Detailed results are provided in Appendix C. Figure 3-3(b) is derived from the MPR values listed in Table 6.2.2-1 of [3], targeting a PC3 UE with 2Tx. Detailed results are provided in Appendix D. It can be confirmed that compared to 2-layer CP-OFDM, 2-layer DFT-s-OFDM achieves 0.5 dB to 2.5 dB net gain for PC2 UEs and 1.0 dB to 2.0 dB net gain for PC3 UEs.

Observation 3-2: UL 2-layer DFT-s-OFDM can provide a net gain compared to UL 2-layer CP-OFDM.

Figure 3-4. Tx power gain between 4-layer DFT-s-OFDM and 4-layer CP-OFDM

The Tx power gain between 4-layer DFT-s-OFDM and 4-layer CP-OFDM is shown in Figure 3-4, which is derived from the MPR values in Table 6.2.2-1 of [3] for a PC3 UE with 4Tx, with detailed results provided in Appendix D. It can be found that 4-layer DFT-s-OFDM can achieve 1.0dB to 2.0dB net gain over 4-layer CP-OFDM.

Observation 3-3: UL 4-layer DFT-s-OFDM can provide a net gain compared to UL 4-layer CP-OFDM

Proposal 3-1

- Support multi-layer UL MIMO transmission based on DFT-s-OFDM.

- Discussions on other waveform candidates

  - Discussion on DL DFT-s-OFDM

DFT-s-OFDM for downlink coverage enhancement and energy saving is also proposed for the 6GR waveform. However, DL DFT-s-OFDM may have significant impacts on actual DL operation, e.g., potential scheduler limitation caused by the FDRA limitation, potential MU possibility limitation, and MU-MIMO performance degradation caused by low PAPR DMRS pattern and sequence. Moreover, the practical gain (or the case where we can enjoy the gain) and specification impact should be clearly justified.

Observation 4-1: DL DFT-s-OFDM may have a significant impact on DL operation, so the practical gain (or the case where we can enjoy the gain) and specification impact should be clearly justified.

  - Discussion on OTFS

OTFS-based waveforms for high-speed scenarios (e.g., NTN) and ISAC are another research direction for the 6GR waveform. OTFS-based waveforms improve the Doppler robustness by transforming the time-frequency domain modulated symbols into the delay-Doppler domain using a new specific transform, e.g., symplectic finite Fourier transform (SFFT), Zak Transform, etc.  However, their processing latency may be increased due to the block operation for the new specific transform. Moreover, the introduction of a new specific transform may cause high transceiver complexity and large specification impacts.

Observation 4-2: OTFS-based waveforms cause additional complexity and cost due to low compatibility with OFDM-based waveforms.

- Conclusion

In this contribution, we present our general views on the waveform design for the 6GR. This includes comprehensive theoretical analysis, numerical evaluations, and comparisons of the low PAPR proposals for UL coverage enhancement and multi-layer DFT-s-OFDM. The following observations and proposals are made based on the discussion.

Observation 2-1: The transparent scheme, DFT-s-OFDM with FDSS, does not always produce net gains.

Observation 2-2: DFT-s-OFDM enhanced with SE, such as FDSS-SE and FDSS-CE with CS, can achieve net performance gains consistently, regardless of the SE factor, occupied bandwidth, or RB allocation.

Observation 2-3: DFT-s-OFDM enhanced with SE can achieve much larger net gains than DFT-s-OFDM with FDSS.

Observation 2-4: The maximum net gain is achieved by either symmetric or asymmetric SE, depending on the SE factor, occupied bandwidth, and RB allocation.

Observation 2-5: FDSS-CE with CS offers flexible (symmetric/asymmetric) SE, which can achieve better net gain than FDSS-SE.

Observation 2-6: DFT-s-OFDM with ST based on truncated mapping and/or asymmetric DFT spreading can achieve a tunable trade-off between PAPR and spectrum efficiency.

Observation 3-1: With a non-coherent precoder, DFT-s-OFDM achieves at least the same performance as CP-OFDM in terms of BLER for both 2-layer and 4-layer scenarios.

Observation 3-2: UL 2-layer DFT-s-OFDM can provide a net gain compared to UL 2-layer CP-OFDM.

Observation 3-3: UL 4-layer DFT-s-OFDM can provide a net gain compared to UL 4-layer CP-OFDM

Observation 4-1: DL DFT-s-OFDM may have a significant impact on DL operation, so the practical gain (or the case where we can enjoy the gain) and specification impact should be clearly justified.

Observation 4-2: OTFS-based waveforms cause additional complexity and cost due to low compatibility with OFDM-based waveforms.

Proposal 2-1

- Support the low PAPR proposals based on DFT-s-OFDM with SE, including FDSS-CE with CS and FDSS-SE, to improve uplink coverage.

Proposal 2-2

- Support the low PAPR proposals based on DFT-s-OFDM with ST, including DFT-s-OFDM with truncated mapping and asymmetric DFT-s-OFDM, to improve uplink coverage.

Proposal 3-1

- Support multi-layer UL MIMO transmission based on DFT-s-OFDM.

- References

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting#123, Dallas, USA, Nov. 2025.

- 3GPP, RP-252912, NTT DOCOMO, INC, “Revised SID: Study on 6G Radio”, Jun. 2025.

- TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 19)”, V19.2.0, June 2025.

- 3GPP, R1-2508336, Nokia, “Waveform for 6G Radio Air Interface”, GPP TSG RAN WG1 Meeting#123, Dallas, USA, Nov. 2025.

- J. Liu, X. Hou, W. Liu and L. Chen, "Low-PAPR and Low-Complexity DFT-s-OFDM Enhancement for 6G," 2025 30th Asia-Pacific Conference on Communications (APCC), Osaka, Japan, 2025, pp. 1-6.

- J. Liu, X. Hou, W. Liu, C. Li, and L. Chen, “Low-PAPR 6G Waveform and Modulation Design with 5G NR Backward Compatibility,” IEICE Trans. on Commun., vol. E109-B, no. 2, Feb. 2026.

- 3GPP, R1-2509282, NTT DOCOMO, INC, “Discussion on Waveform”, 3GPP TSG RAN WG1 Meeting#123, Dallas, USA, Nov. 2025.

- 3GPP, R1-2506218, Qualcomm Incorporated, “Waveforms for 6GR”, 3GPP TSG-RAN WG1 #122, 25th – 29th August 2025, Bengaluru, India.

- 3GPP, R1-2508735, Huawei, HiSilicon, “Waveform for 6GR air interface”, GPP TSG RAN WG1 Meeting#123, Dallas, USA, Nov. 2025.

- 3GPP, R1-2508856, ZTE Corporation, Sanechips, “Views on the waveform for 6G”, GPP TSG RAN WG1 Meeting#123, Dallas, USA, Nov. 2025.

- 3GPP, R1-2509231, Qualcomm Incorporated, “Waveform for 6GR air interface”, GPP TSG RAN WG1 Meeting#123, Dallas, USA, Nov. 2025.

- Appendix

Appendix A: Agreements for UL low PAPR proposals in RAN1#123

Agreement (Monday) Add the following metrics for UL PAPR reduction to the existing agreement (made in RAN1#122b) - Net gain@10% BLER assuming similar spectral efficiency and same occupied bandwidth for each compared method - ACLR, EVM, IBE Agreement General evaluation assumptions for UL low-PAPR proposals 3GPP 6GR. Carrier frequency and scenario; 4GHz. Channel BW; At least 100MHz for Urban (4GHz). Occupied BW; To be discussed with detailed simulation assumptions. SCS; 30 kHz for 4GHz. Channel model; TDL-C 300ns for 4GHz. UE speed; 3km/h. Number of Tx antennas for TDL channel; 1. Number of Rx antennas for TDL channel; 1 and 4 for 4GHz. Number of DMRS symbols/slot (location as defined in NR); 2. Number of PUSCH data symbols/slot; 12. HARQ configuration; No retransmissions. Frequency hopping; Disabled. Agreement For single user evaluation assumption for MCS and subcarriers UL low-PAPR proposals with spectrum extension No Spectrum Extension; With Spectrum Extension. MCS; #subcarriers; #SCs before extension (A); Occupied BW: #SCs after extension (B); Spectrum extension Extension: α=B-AB. NR MCS; B; (1-α)B; B; α∈16, 14,27,13,38,25,716,12. For single user evaluation assumption for MCS and subcarriers UL low-PAPR proposals with spectrum truncation No Spectrum Truncation; With Spectrum Truncation. MCS; #subcarriers; #SCs before truncation (A); Occupied BW: #SCs after truncation (B); Spectrum truncation factor Truncation: α=A-BA. NR MCS; B; A; 𝐵; α∈110, 210,310,410. Agreement For both CP-OFDM and DFT-S-OFDM, for single user evaluation assumption for MCS and subcarriers UL low-PAPR proposals with tone reservation. MCS; Occupied BW: #subcarriers; #reserved tones (inband/sideband); Ratio of reserve. NR MCS; B; A; A/𝐵. Agreement For UL PAPR reduction, values for occupied BW B: - {2, 4, 8, 16, 24, 30, 32, 64, 128, 240, 256} PRBs. - Other PRB allocations are not precluded. - Edge, outer and inner PRB allocations as defined in TS 38.101 should be considered. Agreement Table: Link level multi-user evaluation assumption for UL low-PAPR proposals Parameters; Values. User #; 2 users. RB allocation; X RBs per UE, with Y RB overlap. I.e. 2X-Y RBs as the total occupied BW. Per user configuration; As in the single user sim assumptions agreed earlier.. Multi-user overlap RBs; Reported by companies. Baseline scheme; No spectrum extension/truncation. (2X-Y)/2 RB as the occupied BW for each simulated UE.. Per UE MCS; Chosen to align with the spectral efficiency. Receiver type; The receiver type/algorithm to be reported by the proponent. Agreement Table is endorsed to characterize each proposal as a potential RAN1 observation. Characterization of each waveform proposal Description. Name of the proposal. Motivation of the proposal; E.g. TN, NTN, ISAC, etc…. Applicable link direction; DL/UL/both. Enhancement to CP-OFDM?; No/Yes. Enhancement to DFT-s-OFDM?; No/Yes. Additional OFDM-compatible waveform?; No/Yes. Target channel(s)/signal(s); PDCCH/PDSCH/PUCCH/PUSCH/xxx. Target modulation. Motivation / use case; Improved spectral efficiency, …. Key Metric / KPI; Spectral efficiency, …. Key spec impact foreseen. MRSS compatibility; Please explain. Multiplexing/coexistence with other waveforms; Please explain. Multi-user multiplexing; Please explain. MIMO compatibility; Please explain.

Appendix B: Agreements on evaluation assumptions for multi-layer PUSCH based on DFT-s-OFDM in RAN1#123

Agreement - Performance benefit to be evaluated using both link level and system level simulation. - FFS: metrics - Link level configuration for multi-layer UL waveforms study. Parameters; Values. Carrier frequency​​; 4 GHz. Subcarrier spacing​​; 30 kHz​​. UE antenna ports; 2, 4. BS antenna ports; 64, port reduction is not precluded. FDRA; 4, 8, 16, 32, 64​​. Waveform​​s; CP-OFDM ; DFT-s-OFDM​​. MCS/modulation​​; NR UL MCS table with 256 QAM. Number of layers​; 2 layers​​, 4 layers. Channel model​​; CDL-A30​​, TDL-C300, CDL-C300,. UE speed​​; 3km/h​​, 30 km/h. Channel est.​​; Practical​. SRS periodicity; To be reported by the company. Receiver; LMMSE. HARQ retransmission​; Disabled. DMRS configuration; Configuration type 1; 2 DMRS symbols per slot. Number of PUSCH data; 12 symbols. Waveform and MIMO configuration; 5G codebook, to be reported by the company. BLER target; 10%. Frequency hopping; Disabled. Power class and power mode; Option 1: PC2, total power limited to 26 dBm. - MaxRank 2: Each PA is limited to 23 dBm - MaxRank 4: Each PA is limited to 20 dBm Option2: PC3, total power limited to 23 dBm. - MaxRank 2: Each PA is limited to 20 dBm - MaxRank 4: Each PA is limited to 17 dBm. Agreement Table: System-level configuration for multi-layer UL CP-OFDM/DFT-s-OFDM study: Parameters; Values. System configuration; UMa, 21 cells, 10, 30 or 50 UEs/cell - Detailed parameters not listed in this table according to UMa as defined in AI 11.2. Traffic model; FTP model 3, - 0.5 Mbyte packet sizes - Network load up to companies to report Full buffer (optional, for calibration). Carrier frequency; 4 GHz. Channel bandwidth/ Subcarrier spacing; At least 100 MHz, 30KHz. Frame structure; TDD (DDDSU), S-slot D:G:U as 10:2:2. UE antenna ports; Same as link level. BS antenna ports; Outdoor Combination 2 for UMa as agreed in 11.2. UE antenna model; The UE handheld model as agreed in AI 11.2, companies to report the used model.. ISD; 500 m. Maximum Number of layers (maxRank) per UE; Adaptive rank, maxRank 2 or 4 SU-MIMO. Receiver; Same as link level, port reduction mapping to be reported. Waveform and MIMO configuration; Same as link level. Power class and power mode; Same as link level. UL Maximum power; MPR based Pcmax according to modulation/FDRA/waveform/coherency/etc. - Option 1: 38.101-1 section 6.2D definition - Option 2: Based on a realistic PA model. UL Power Control Settings; P0 and Alpha to be reported by the company. SRS periodicity; To be reported by the company.

Appendix C: MPR for 2-layer UL MIMO for PC2 with 2Tx based on Table 6.2D.2-1 in [3]

Table 6.2D.2-1: Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.52 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |
| NOTE 1: Applicable for a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostPi2BPSKRel18] is set to 1. The reference power is increased by ΔPPowerBoost - ΔPPowerClass NOTE 2: Applicable for a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostQPSKRel18] is set to 1. The reference power is increased by ΔPPowerBoost - ΔPPowerClass |  |  |  |  |

Appendix D: MPR for 2/4-layer UL MIMO for PC3 with 2/4Tx based on Table 6.2.2-1 in [3]

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 |  | 05 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB NOTE 4: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 5: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |