3GPP TSG RAN WG1 #124	                                	R1-2601265

Gothenburg, Sweden, February 9th – 13th, 2026

Agenda item:	9.6.1

Source: 	Qualcomm Incorporated

Title: 	NR NTN GNSS resilience

Document for:	Discussion and Decision

# Background

### 4.1	Objective of Core part WI

The study will be carried assuming that

- The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose

  - This study will initially focus on 1/ (where GNSS information is temporarily unavailable)

  - Note: This study does not include any enhancement on positioning for the above scenario

Study GNSS resilient NR-NTN operation [RAN1, RAN4]

- Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications

  - Aim to minimize physical layer procedures impact

  - Avoid physical layer channel/signal changes

  - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented

  - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites

- Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

In RAN#108, a study item on GNSS-resilient NR-NTN operation was agreed [1], including the following objective:

In RAN1#122 [2], the following was agreed:

Agreement

For the study on GNSS resilient operation, scenarios should be considered where:

- Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link.

- Scenario 2: There is a previously acquired GNSS based position

  - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded

    - FFS: value of T

    - FFS: GNSS based UE location validity duration

    - FFS: GNSS accuracy

- Note: Two scenarios above belong to description as in SID.

# Main challenges of GNSS resilient operation

Up to Rel-19 NR-NTN, the UE is assumed to have valid GNSS position and ephemeris, which are used to autonomously pre-compensate both timing and frequency for UL transmissions. This applies from the start of the connection (initial access), and continues in connected mode, during which the UE is required to continuously update the timing advance and frequency pre-compensation according to the ephemeris and UE location. This operation allowed to alleviate the main impairments present in satellite communications – large Doppler and round-trip time.

There are two main aspects of GNSS-resilient operation:

- Random Access

- Time-Frequency Adjustments in Connected Mode

During GNSS resilient operation, the RAN should provide mechanisms for the UE to access the NTN cell and keep uplink time/frequency synchronization without relying on the UE location or assuming that the UE location may be inaccurate. Some of these techniques were documented in the Rel-16 study item [3].

The main aspects to study by RAN1 to enable GNSS resilient NTN operation are as follows:

- PRACH / Initial access: The initial transmission by the UE would suffer from increased time and frequency errors due to lack of GNSS availability. Although some methods may partially alleviate this issue (e.g. signalling a common frequency / time correction for the entire cell), the performance of initial access should be thoroughly studied.

- Closed loop time and frequency corrections: A single NTN cell may serve legacy UEs with GNSS and UEs requiring GNSS-resilient access. Therefore, it is desirable to align the time and frequency of the GNSS resilient UEs with those having GNSS.

Agreement:

For evaluation regarding GNSS-resilient operation, based on GNSS information or the lack of it, it is assumed that the UE has a horizontal location uncertainty within an area

- Case a: the location uncertainty area is the area served by the cell or beam.

- Case b: the location uncertainty area is a circle of radius X km

  - X to be reported

UE altitude error should be taken into account.

- UE altitude error to be reported.

For both Case a and Case b above, RAN1 to evaluate at least:

- Differential delay values

- Differential frequency offset/Doppler values

For evaluation of the problems of timing and frequency synchronization in GNSS-resilient NTN, RAN1 (RAN1-122 [2]) made the following agreement.

Agreement

For the study of GNSS resilient operation, for evaluation purposes, it is assumed that the UE pre-compensates RTT and Doppler on the service link for UL transmission. This may include, but not limited to:

- At least for Scenario 1, based on assistance information provided by the Network.

- For Scenario 2, based on at least the last known UE location.

Since GNSS is temporarily unavailable, the UE will apply timing and frequency pre-compensation based on a “proxy” location. In RAN1-123 [3], the following was agreed.

Even after the UE applies a timing and frequency correction described as above, the gNb will observe large uplink timing and frequency errors caused by the differential delay and doppler within the location uncertainty area. The differential delay and doppler of a location uncertainty area can be expressed as a function of (1) the minimum elevation angle and (2) azimuth angle of the point in the uncertainty area with minimum elevation angle with respect the satellite orbital plane. For instance, depending on whether the area is “along track” (i.e. assuming the uncertainty area as a circle, the center is located on satellite’s ground track which is the trace of sub-satellite points on ground) or “cross track” (i.e. the center of the area is located at an azimuth angle of 90° with respect to the sub-satellite point), see Fig. 1. In Table 1, we present the differential one-way delay and Doppler observed for case a.

Figure 1: Figure 1: Illustration of geometry of location uncertainty areas: Along Track and Cross Track areas

x

y

ψ

θ

Along track area (satellite elevation angle at center = θ)

Cross track area (satellite elevation angle at center = θ)

Uncertainty area at Nadir

Sub-satellite point

Satellite moves along +x

h

Rb

Therefore, in RAN1#122bis [4], the following was agreed.

Agreement:

For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions:

- For scenario 1 and 2

  - The following time/frequency differences should be evaluated:

    - The differential one-way delay (in µs) as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area.

      - For UL performance evaluation 2 times differential one-way delay should be considered.

    - The differential one-way Doppler/frequency offset (in ppm) is calculated between the largest UE-sat Doppler/frequency offset and smallest UE-sat Doppler/frequency offset within the uncertainty area.

      - For UL performance evaluation, scaled one-way differential Doppler/frequency offset to determine UL differential Doppler/frequency offset should be considered. Scaling factor should be reported.

For computing the one way differential doppler value, in RAN1-123, following agreement was made.

Agreement:

For the calculation of one-way differential delay and one-way differential Doppler, adopt Alt2 in RAN1#112bis agreement with the following correction on the one-way Doppler:

dopplerone-way(u)=vc2fcu-s,xdone-way(u)+vc2fcu-s,kdone-way(u)

Note: Earth’s rotational motion is neglected for LEO.

We noticed a typographic error in the above formula which we propose to correct in the following way.

Proposal 1: Correct the typographic error in RAN1-123 Agreement as follows:

For the calculation of one-way differential delay and one-way differential Doppler, adopt Alt2 in RAN1#112bis agreement with the following correction on the one-way Doppler:

dopplerone-way(u)=vsatc2fcu-s,xdone-way(u)+vUEc2fcu-s,kdone-way(u)
where vsat is the speed of satellite and vUE is the speed of the UE.

Table 1: Differential one-way delay and Doppler values for different beam diameters (case a)

|  |  |  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Orbit |  | LEO600 |  | LEO1200 |  | GEO |  | LEO600 |  | LEO1200 |  | GEO |  |  |  |
|  |  | Band/Carrier Frequency |  | S/2GHz |  | S/2GHz |  | S/2GHz |  | Ka/30GHz |  | Ka/30GHz |  | Ka/30GHz |  |  |  |
|  |  | Beam diameter (km) |  | 50 |  | 90 |  | 250 |  | 20 |  | 40 |  | 110 |  |  |  |
| Elevation angle (°) |  | 12.5 |  |  |  |  |  | 814.085 |  |  |  |  |  | 358.218 |  |  |  |
|  |  | 30 |  | 144.424 |  | 259.959 |  |  |  | 57.774 |  | 115.547 |  |  |  |  |  |
|  |  | 90 |  | 1.9 |  | 3.343 |  | 4.818 |  | 0.304 |  | 0.661 |  | 0.933 |  |  |  |
| Case a: the location uncertainty area is the area served by the cell or beam ¦ case 1: within the orbital plane |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Orbit |  | LEO600 |  | LEO1200 |  | GEO |  | LEO600 |  | LEO1200 |  | GEO |  |  |  |  |  |
|  | Band/Carrier Frequency |  | S/2GHz |  | S/2GHz |  | S/2GHz |  | Ka/30GHz |  | Ka/30GHz |  | Ka/30GHz |  |  |  |  |  |
|  | Beam diameter (km) |  | 50 |  | 90 |  | 250 |  | 20 |  | 40 |  | 110 |  |  |  |  |  |
| Elevation angle (°) | 12.5 |  |  |  |  |  | 814.085 |  |  |  |  |  | 358.218 |  |  |  |  |  |
|  | 30 |  | 144.424 |  | 259.959 |  |  |  | 57.774 |  | 115.547 |  |  |  |  |  |  |  |
|  | 90 |  | 1.9 |  | 3.343 |  |  |  | 0.304 |  | 0.661 |  |  |  |  |  |  |  |
| Case a: the location uncertainty area is the area served by the cell or beam ¦ case 2: at 90° with respect to orbital plane |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |  |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |  |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |  |
| Elevation angle (°) | 12.5 |  |  | 0 |  |  | 0 |  |
|  | 30 | 0.359 | 0.373 |  | 0.143 | 0.166 |  |  |
|  | 90 | 2.1 | 1.814 | 0 | 0.841 | 0.807 | 0 |  |
| Case a: the location uncertainty area is the area served by the cell or beam ¦ case 1: within the orbital plane ¦ |  |  |  |  |  |  |  |  |

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |  |  |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |  |  |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |  |  |
| Elevation angle (°) | 12.5 |  |  | 0 |  |  | 0 |  |  |
|  | 30 | 1.173 | 1.09 |  | 0.469 | 0.484 |  |  |  |
|  | 90 | 2.1 | 1.814 | 0 | 0.841 | 0.807 | 0 |  |  |
| Case a: the location uncertainty area is the area served by the cell or beam ¦ case 2: at 90° with respect to orbital plane |  |  |  |  |  |  |  |  |  |

# Random Access

Random access is a critical component of GNSS-resilient operation. In the absence of any UE-specific time and frequency corrections, the UE can only rely on common/broadcast information as described earlier. Consequently, the timing and frequency offsets are expected to be the largest during initial access, and should then be reduced by gNB-driven corrections.

The key functionality and requirements for random access in GNSS-resilient NTN comprises the following two aspects:

• Robustness to large differential round-trip Doppler (RTD) and round-trip time (RTT), consistent with the agreed evaluation scenarios. Without sufficient robustness, a GNSS-resilient UE may fail to establish a connection.

• Upon completion of the random access procedure, the UE should be provided with time and frequency corrections that significantly reduce the initial errors.

Agreement:

The inequation for the limit on differential roundtrip delay in the RAN#122bis Agreement (copied below), is not applicable for unrestricted set.

For unrestricted set PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration) - 2 Te ≥ Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or  (scaling factor*(Differential Doppler) +2* fe×fc) ≥ 1+γΔfRA

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively.

      - Othe values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

Agreement:

For PRACH performance evaluation,  Te  used for the limit on differential roundtrip delay determination is to be set according to TS 38.133, Table 7.1.2-1.

Working assumption:

For UL performance evaluation, UL differential Doppler/frequency offset is calculated based on scaled DL one-way differential Doppler/frequency offset with the following scaling factor:

scaling factor=1+fc,ul/fc,dl

Scaling factor = 2*fc,ul / fc,dl

fc,ul and fc,dl  are the uplink and downlink carrier frequency respectively.

In addition to the differential RTT/RTD within the uncertainty area, the UE may introduce additional timing and frequency errors during PRACH transmission due to e.g. errors in timing / frequency estimation and RF calibration inaccuracies. For analytical PRACH performance evaluation, RAN1 (RAN1-123 [3]) made the following agreement:

Under current random access procedure specifications in 38.213, for each random access attempt, the UE transmits a PRACH sequence in a random access occasion (RO), where the PRACH sequence is generated according to 38.211, Section 6.1 [5].

Observation 1: According to 38.213 [6], for a single random access procedure performed by layer 1, the UE transmits a single PRACH sequence based on a PRACH Format across one or more ROs, where the PRACH sequence per RO is generated according to 38.211.

The RACH sequences can be generated according to the following PRACH formats:

- Long PRACH format (formats 0, 1, 2, 3).

- Short PRACH format (formats A0, A1, A2, B1, B2, B3, B4, C0, C2) [5].

For each format, we can now evaluate the maximum differential round trip delay and doppler limits that can be tolerated. The evaluation results are captured in the following working assumptions of RAN-123 [3].

Working assumption

Differential roundtrip delay limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | L | Ncp [µs] | Max Ncs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Unrestricted | 839 | 103,13 | 419 | 0,39 | 800 | 102,34 |
| 0 | type A | 839 | 103,13 | 237 | 0,39 | 800 | 102,34 |
| 0 | type B | 839 | 103,13 | 137 | 0,39 | 800 | 102,34 |
| 1 | unrestricted | 839 | 684,38 | 419 | 0,39 | 800 | 683,59 |
| 1 | type A | 839 | 684,38 | 237 | 0,39 | 800 | 225,20 |
| 1 | type B | 839 | 684,38 | 137 | 0,39 | 800 | 129,85 |
| 2 | unrestricted | 839 | 152,60 | 419 | 0,39 | 800 | 151,82 |
| 2 | type A | 839 | 152,60 | 237 | 0,39 | 800 | 151,82 |
| 2 | type B | 839 | 152,60 | 137 | 0,39 | 800 | 129,85 |
| 3 | unrestricted | 839 | 103,13 | 419 | 0,39 | 200 | 102,34 |
| 3 | type A | 839 | 103,13 | 237 | 0,39 | 200 | 55,71 |
| 3 | type B | 839 | 103,13 | 137 | 0,39 | 200 | 31,88 |

Differential roundtrip delay limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 15 | 139 | 9.38 | 69 | 0.39 | 66.67 | 8.59 |
| A1 | 30 | 139 | 4.69 | 69 | 0.33 | 33.33 | 4.04 |
| A1 | 60 | 139 | 2.34 | 69 | 0.11 | 16.67 | 2.12 |
| A1 | 120 | 139 | 1.17 | 69 | 0.11 | 8.33 | 0.94 |
| A2 | 15 | 139 | 18.75 | 69 | 0.39 | 66.67 | 17.97 |
| A2 | 30 | 139 | 9.38 | 69 | 0.33 | 33.33 | 8.72 |
| A2 | 60 | 139 | 4.69 | 69 | 0.11 | 16.67 | 4.46 |
| A2 | 120 | 139 | 2.34 | 69 | 0.11 | 8.33 | 2.12 |
| A3 | 15 | 139 | 28.13 | 69 | 0.39 | 66.67 | 27.34 |
| A3 | 30 | 139 | 14.06 | 69 | 0.33 | 33.33 | 13.41 |
| A3 | 60 | 139 | 7.03 | 69 | 0.11 | 16.67 | 6.80 |
| A3 | 120 | 139 | 3.52 | 69 | 0.11 | 8.33 | 3.29 |
| B1 | 15 | 139 | 7.03 | 69 | 0.39 | 66.67 | 6.25 |
| B1 | 30 | 139 | 3.52 | 69 | 0.33 | 33.33 | 2.86 |
| B1 | 60 | 139 | 1.76 | 69 | 0.11 | 16.67 | 1.53 |
| B1 | 120 | 139 | 0.88 | 69 | 0.11 | 8.33 | 0.65 |
| B2 | 15 | 139 | 11.72 | 69 | 0.39 | 66.67 | 10.94 |
| B2 | 30 | 139 | 5.86 | 69 | 0.33 | 33.33 | 5.21 |
| B2 | 60 | 139 | 2.93 | 69 | 0.11 | 16.67 | 2.70 |
| B2 | 120 | 139 | 1.46 | 69 | 0.11 | 8.33 | 1.24 |
| B3 | 15 | 139 | 16.41 | 69 | 0.39 | 66.67 | 15.63 |
| B3 | 30 | 139 | 8.20 | 69 | 0.33 | 33.33 | 7.55 |
| B3 | 60 | 139 | 4.10 | 69 | 0.11 | 16.67 | 3.87 |
| B3 | 120 | 139 | 2.05 | 69 | 0.11 | 8.33 | 1.82 |
| B4 | 15 | 139 | 30.47 | 69 | 0.39 | 66.67 | 29.69 |
| B4 | 30 | 139 | 15.23 | 69 | 0.33 | 33.33 | 14.58 |
| B4 | 60 | 139 | 7.62 | 69 | 0.11 | 16.67 | 7.39 |
| B4 | 120 | 139 | 3.81 | 69 | 0.11 | 8.33 | 3.58 |
| C0 | 15 | 139 | 40.36 | 69 | 0.39 | 66.67 | 39.58 |
| C0 | 30 | 139 | 20.18 | 69 | 0.33 | 33.33 | 19.52 |
| C0 | 60 | 139 | 10.09 | 69 | 0.11 | 16.67 | 9.87 |
| C0 | 120 | 139 | 5.05 | 69 | 0.11 | 8.33 | 4.82 |
| C2 | 15 | 139 | 66.67 | 69 | 0.39 | 66.67 | 65.89 |
| C2 | 30 | 139 | 33.33 | 69 | 0.33 | 33.33 | 32.67 |
| C2 | 60 | 139 | 16.67 | 69 | 0.11 | 16.67 | 16.45 |
| C2 | 120 | 139 | 8.33 | 69 | 0.11 | 8.33 | 8.11 |

Working assumption

Theoretical differential Doppler limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0,1,2 | unrestricted | 1250 | 0 | 0.1 | 2 | 2 | 2.0 | 0.21 | 0.85 |
| 0,1,2 | type A | 1250 | 1 | 0.1 | 2 | 2 | 2.0 | 0.53 | 2.1 |
| 0,1,2 | type A | 1250 | 2 | 0.1 | 2 | 2 | 2.0 | 0.84 | 3.35 |
| 0,1,2 | type B | 1250 | 3 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 0,1,2 | type B | 1250 | 4 | 0.1 | 2 | 2 | 2.0 | 1.46 | 5.85 |
| 3 | unrestricted | 5000 | 0 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 3 | type A | 5000 | 1 | 0.1 | 2 | 2 | 2.0 | 2.40 | 9.6 |
| 3 | type A | 5000 | 2 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
| 3 | type B | 5000 | 3 | 0.1 | 2 | 2 | 2.0 | 4.90 | 19.6 |
| 3 | type B | 5000 | 4 | 0.1 | 2 | 2 | 2.0 | 6.15 | 24.6 |

Theoretical differential Doppler limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all short formats | 15 | 15000 | 0 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
|  | 30 | 30000 | 0 | 0.1 | 2 | 2 | 2.0 | 7.40 | 29.6 |
|  | 60 | 60000 | 0 | 0.1 | 30 | 20 | 2.5 3 | 1.08 0.9 | 54 |
|  | 120 | 120000 | 0 | 0.1 | 30 | 20 | 2.5 3 | 2.28 1.9 | 114 |

Based on the analytical conditions, we evaluate whether the PRACH tolerance exceeds the differential round trip delay and doppler observed in a location uncertainty area (few examples are captured in Table 1) in the attached spreadsheet.

Proposal 2: Capture the attached spreadsheet the TR.

In the working assumption for frequency tolerance of all short PRACH format, we noticed a typographic error as indicated above. However, this error does not propagate into any analysis derived from this table as the right most column is the factor that ultimately limits determines whether the PRACH format can tolerate the frequency error observed in a location uncertainty area.

Proposal 3: Capture the corrections in highlighted text on the working assumptions of RAN1-123 in the TR.

From the above discussion, we make the following observation:

Observation 2: For scenario 1, there is no PRACH format with the existing PRACH procedure which can tolerate the timing and frequency error observed in the following uncertainty areas:

Table 2: List of location uncertainty areas in scenario 1 for which no PRACH formats are compatible

| Orbit | Band | UA diameter (km) | Elevation angle (a=along, c=cross track) |
| --- | --- | --- | --- |
| LEO1200 | Ka | 40 | 30a |
| LEO1200 | Ka | 40 | 30c |
| LEO1200 | Ka | 90 | 30a |
| LEO1200 | Ka | 90 | 30c |
| LEO1200 | S | 90 | 30a |
| LEO1200 | S | 90 | 30c |
| LEO1200 | S | 190 | 30a |
| LEO1200 | S | 190 | 30c |
| LEO600 | Ka | 20 | 30a |
| LEO600 | Ka | 20 | 30c |
| LEO600 | Ka | 50 | 30a |
| LEO600 | Ka | 50 | 30c |
| LEO600 | Ka | 50 | 90 |
| LEO600 | S | 50 | 30a |
| LEO600 | S | 50 | 30c |
| LEO600 | S | 90 | 30a |
| LEO600 | S | 90 | 30c |

Proposal 4: RAN1 to conclude that with the current random access procedure with existing PRACH formats:

- gNB cannot resolve the UL timing and frequency errors observed in the majority of the location uncertainty areas at least in Scenario 1.

To reduce the time and frequency errors during initial access of GNSS-resilient NTN, in RAN1-123 [3] the following was agreed.

Agreement:

For the evaluation of GNSS resilient NR-NTN operation, consider the following solutions at least for initial access procedure at least to increase PRACH tolerance and/or reducing time/frequency uncertainty:

- Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

- Solution 1B: New PRACH restricted sets.

- Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in single/multiple RAR MAC PDU scheduling multiple msg3 with different TA/FA.

- Solution 1D: Signalling enhancements for Msg2/Msg4 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command).

- Solution 1E: configuration/selection of a set of existing PRACH root for a Single PRACH transmission.

- Solution 1F: PRACH transmission using TA margin indicated by Network and RO masking.

- Solution 1G: Adaptation of PRACH configuration.

- Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

- Solution 2B: Multiple random access attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

- Solution 2C: Solutions based on broadcasting DL timestamp(s).

- Solution 2D: UE side time/frequency pre-compensation based on reference location or TA/Doppler compensation information provided by gNB.

- Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position

- Solution 2F: UE side time/frequency pre-compensation based on reference signal.

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

Other Solutions are not precluded.

As part of the evaluation of the solutions above, at least the following aspects should be assessed:

- specification impact when relevant (including whether the solutions remain within the scope of the SID)

- performance,

- applicability to different scenarios,

- complexity,

- Coexistence with legacy UEs

- the signalling overhead.

We will now evaluate the solutions listed above in Table 3.

Table 3: Evaluation of Solutions for initial access

| Solution code | Solution | Evaluation |
| --- | --- | --- |
| 1A | Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats | Requires random access procedure changes No change in random access channel/signal, within scope of SID (see Section 3.1) |
| 1B | New PRACH restricted sets. | Requires change in random access channel (38.211), limited applicability |
| 1C | Single PRACH transmission with multiple TAC/FAC commands in single/multiple RAR MAC PDU scheduling multiple msg3 with different TA/FA. | Requires random access procedure changes, no change in random access channel/signal, within scope of SID. Msg3 Resource wastage and latency impacts when timing and frequency uncertainties are large |
| 1D | Signalling enhancements for Msg2/Msg4 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command). | Requires to be combined with other solutions e.g. 1A, 1B, 1C, 2A, or 3. |
| 1E | configuration/selection of a set of existing PRACH root for a Single PRACH transmission | From previous discussion, single transmission of PRACH formats cannot tolerate the timing and frequency uncertainty. |
| 1F | PRACH transmission using TA margin indicated by Network and RO masking. | Solution is not clear |
| 1G | Adaptation of PRACH configuration | Solution is not clear |
| 2A | Single/multi-satellite DL-TDOA based on current specifications | Not feasible without specification change, out of scope of SI (see section 4) Depends on UEs discovery of PRS from multiple satellites within the search window Requires constellation planning and network overhead to deliver assistance info to UE |
| 2B | Multiple random access attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area) | Not feasible since it will lead to PRACH collision |
| 2C | Solutions based on broadcasting DL timestamp(s). | Requires the UE to have an accurate clock, which is not the case typically. The clock drift of the local oscillator of the UE can be arbitrarily large. |
| 2D | UE side time/frequency pre-compensation based on reference location or TA/Doppler compensation information provided by gNB. | Requires to be combined with other solutions e.g. 1A, 1B, 1C |
| 2E | service link time/frequency pre-compensation based on last acquired GNSS position | Feasible, but only under limited scenario. Depends on when accuracy of the last acquired GNSS position. |
| 2F | UE side time/frequency pre-compensation based on reference signal. | It is not possible to compute timing and frequency pre-compensation based on DL measurements since UE does not have an ideal clock. For example, if the UE is synchronized to DL of serving satellite, the Doppler measurements will be zero. |
| 3 | Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay. | It is possible to implement advanced PRACH processing at gNB (e.g. with search across a large time frequency window). This solution could be combined with 2D, 1D |

Using the toolbox of solutions mentioned above, we present a composite solution as follows.

## Solution 2D-1A-1D

This solution is a combination of 3 of the solutions above which, in our view, provide a complete toolkit to solve the random access problem. It comprises the following steps:

- Step 1 (solution 2D): UE applies initial timing and frequency pre-compensation based on assistance information from network (e.g. a reference location) or last known UE location

- Step 2 (solution 1A): UE transmits two PRACH sequences generated from the existing PRACH format with different root indices at two ROs

- Step 3 (solution 1D): UE receives timing and frequency corrections in RAR

Step 1 (solution 2D) reduces the UL time frequency uncertainty to the differential round trip delay and doppler within the uncertainty area, eliminating common timing and frequency errors. Solution 1D allows for correction of the remaining time and frequency errors before msg3 transmission. Solution 1A provides a robust, computationally simply solution with minimum specification impact to the problem of detecting a PRACH and estimating the remaining time and frequency errors during PRACH reception. We motivate next the solution 1A.

### Motivation for solution 1A

PRACH uses a Zadoff–Chu (ZC) sequence of prime length L. PRACH format 1 uses a ZC sequence with L=839 and ΔfRA=1.25 kHz, occupying 6 PRBs. The sequence is generated with a root u∈ZL, where ZL denotes integers modulo L [5, TS 38.211, Clause 6.3.3]. When L is prime, ZL forms a finite field and every non-zero element has a multiplicative inverse.

After the UE applies nominal timing and frequency adjustments for Msg-1 (as proposed earlier), the gNB estimates the differential timing and frequency offset from the PRACH reception by locating the correlation peak. A known property (originating from ZC sequence structure) is a time–frequency ambiguity whereby a frequency offset on the order of one or more ΔfRA induces a shift equivalent to a propagation delay. Under ideal conditions, the observed peak location can be expressed as (τ+k u-1) mod L, where:

- τ is the integer timing offset (rounded by the sample duration),

- k is the integer frequency offset (rounded by ΔfRA),

- u-1 is the multiplicative inverse of u modulo L.

According to the specification, a UE generates multiple PRACH preambles by applying cyclic shifts Cv to the same root; 64 preambles are obtained by applying this procedure across multiple roots. For preambles derived from the same root, selected cyclic shifts must lie outside the zero-correlation zone to avoid overlap of correlation peaks used for estimating timing/frequency offsets.

An uncertainty area can be mapped to a set W(τ,k)⊂ZL×ZL of integer time–frequency offsets. For a given root u, a restricted set can be constructed by searching cyclic shifts Cv such that the peak locations (Cv+τ+k u-1) mod L for all (τ,k)∈W(τ,k) remain non-overlapping (see Appendix A). Handling time–frequency errors beyond generalized restricted sets would require multiple Msg-1/Msg-3 transmissions to resolve the ambiguity set {(τ,k)} generated by W(τ,k). One of the solutions is to transmit two Msg-1 using different root sequences. The gNB uses the two observed peak patterns to resolve (τ,k) unambiguously by solving the following equations:

p1=τ+u1-1k mod L

p2=τ+u2-1k mod L

k=u1-1-u2-1-1p1-p2 mod L

τ =p1-u1-1k mod L

Here p1, p2 are the observed peaks. For the simplicity of exposition, we assume no fractional timing and frequency offset in this section. However, we have considered fraction frequency and timing offset resolution in our LLS. The gNB can resolve the fractional timing offset by using a sampling rate larger than ΔfRA and fractional frequency offset by performing additional frequency hypothesis per RO.

The UE transmits Msg-3 with the indicated correction signaled in RAR.

### Evaluation Results

In this section, we present link level simulation results for solution 1A. The simulation setup is explained in Table 5. The misdetection probability vs SNR plot is provided in Fig. 2. Note that the detection event is defined as (1) the PRACH is detected above detection threshold, (2) the timing error is less than timing error tolerance, and (3) the frequency error is less than the frequency error tolerance. As shown in Table 3, the timing and frequency errors include only the differential components, with the common error being removed by technique 2D. Similarly, as a result of the correct detection of time and frequency errors, solution 1D can be applied to ensure that msg3 and later transmission have reduced the time-frequency errors to those required for PUSCH reception. The misdetection probability is averaged across 10 different root pairs chosen from low to high values of u1-u2 mod L.

Table 3: LLS parameters for the evaluation of PRACH detection performance using two root sequences

| Parameter | Value |
| --- | --- |
| PRACH Format | Format 1 |
| RO Duration | 3 ms |
| PRACH SCS (ΔfRA) | 1.25 kHz |
| Frequency | 2 GHz |
| PRACH Format | Format 1 |
| # of ROs | 2 |
| # of Root Pairs (u1,u2) used for averaging | 10 (u1≠u2) |
| Timing Error | Uniformly distributed within (0, CP) |
| Frequency Error | Uniformly distributed within (-10ΔfRA,10 ΔfRA) |
| Channel | NTN TDL-C |
| Delay Spread | 10 ns |
| Doppler Spread | 5 Hz |
| Channel Dimension | 2Tx-1Rx |
| Timing Error Tolerance | 2.55 μs |
| Frequency Error Tolerance | 0.1 ppm |

Figure 2: PRACH Misdetection Probability vs SNR averaged across 10 arbitrary root pairs

### Applicability to Different Scenarios

Based on the above evaluation, we observe that the solution 2D-1A-1D is applicable to all the scenarios as long as the differential round trip delay of the location ambiguity region is within the timing error tolerance limit of the PRACH format being used.  Although we used frequency error within ±12.5 kHz, the theoretical tolerance of UL time and frequency error achievable by this solution can be expressed as follows:

Uplink absolute timing error≤ minNcp, Tseq

and

Uplink absolute frequency error≤ K ΔfRA,

where Uplink absolute timing error=Differential round trip delay+Te and Uplink absolute frequency error=(Differential round trip doppler +2×0.1[ppm])× fc,ul. Here K≤L2 can be set arbitrarily large allowing tolerable degradation of misdetection probability. The reason of this degradation is that the gNB will not be able to observe the full sequence due to large frequency shift and hence the correlation peak will drop. However, based on our evaluations, this degradation is negligible for K=10 which covers the worst case UL frequency error in scenario 1 (with 1.25 KHz SCS).

### Impact of solution 1A gNB Implementation

For estimation of the timing and frequency offsets from two ROs, we do not anticipate any significant impact on gNB processing complexity.

- Peak Detection Per RO: gNB performs peak detection per RO independently (existing implementation). To minimize the estimation error, gNB should use the multiple fractional CFO hypothesis.

- Time And Frequency offset Estimation: gNB performs a joint estimation of  τ,k, and fractional components of delay and doppler from the estimated peaks from two RO, which is an simple step of solving a system of linear equations in integer domain and requires no additional signal processing.

Figure 3: gNB Processing for timing and frequency offset estimation for Solution 2D-1A-1D

A schematic diagram of the gNB implementation is provided in Fig. 3.

### Specification Impact

The transmission of multiple root sequences over consecutive ROs could be achieved without having any impact on the existing random access channel with minimum changes in the physical layer procedures. We provide a high-level analysis of the potential specification impact while remaining in the scope of the SI.

| Reference to Specifications | Aspects of Random Access Channel/Procedure | Potential Specification Impact |
| --- | --- | --- |
| 38.211, Section 6.1.1 | Random access channel: defines what is transmitted per RO, where UE receives prach-RootSequenceIndex from higher layer | No change |
| 38.213, Section 8 | Random access procedure: for a PRACH transmission with Npreamblerep preamble reptations, the UE uses Npreamblerep valid ROs consecutively in time | Need procedural changes which allows UE to change the values of prach-RootSequenceIndex within these set of Npreamblerep Ros, which is within the scope of the SI |

Based on the above discussion, we make the following proposals.

Proposal 5: For initial access in GNSS resilient NTN, RAN1 should study the solution which is a combination of solution 2D-1A-1D, i.e.:

Step 1: UE applies initial timing and frequency pre-compensation based on assistance information from network (e.g. a reference location) or last known UE location

Step 2: UE transmits two PRACH sequences generated from the existing PRACH format with different root indices at two ROs

Step 3: UE receives timing and frequency corrections in RAR

Observation 3: Assuming that the gNB performs a single FFT (i.e. correlation) based detection per frequency hypothesis with at least two frequency hypotheses per RO (to resolve the fractional CFO), this solution is robust for any uplink timing error and frequency error as long as:

Uplink absolute timing error≤ minNcp, Tseq and Uplink absolute frequency error≤ K ΔfRA,

where Uplink absolute timing error=Differential round trip delay+Te and Uplink absolute frequency error =(Differential round trip doppler +2×0.1[ppm])× fc,ul, and K can be large enough (K<L) to allow tolerable degradation of misdetection probability.

Proposal 6: Capture the evaluation results and specification impact analysis of solution 2D-1A-1D in the TR.

## Enabling Long PRACH Formats in FR2 NTN

According to the agreements reached in RAN1-122, one of the key use cases for GNSS-resilient NTN is the operation of VSAT user equipment (UE) in Ku and Ka frequency bands. Under current specifications, Ku band supports both FR1 and FR2 numerologies, while Ka band supports only FR2 numerologies. In FR2 numerology, we are not allowed to use the PRACH formats 0,1,2,3 which resolves large timing and frequency ambiguity in GNSS-resilient NTN (see Table 6.3.3.2-4 in 38.211).

As observed before, short PRACH formats are unable to accommodate the uplink timing and frequency errors encountered by the gNB, assuming a conventional PRACH detection procedure (i.e., single correlation at the gNB). To this end, enabling long PRACH transmission in FR2 NTN will allow us to leverage:

- Large time and frequency resolution from of msg-1 using long PRACH (using solution 2D-1A-1D)

- For reasonable UL SNR, the gNB may be able to obtain the finer timing resolution required for FR2 numerology (e.g. 0.59 μs for 120 kHz). Otherwise, the network may send initial corrections of timing and frequency and trigger additional msg-1 transmissions (possibly using different numerology for acquiring finer timing corrections) may be required.

The key enabler for this is to allow the transmission of long PRACH formats in FR2 NTN, which we propose next.

Proposal 7: RAN1 should study procedures to enable the existing PRACH formats available for FR1 (i.e. formats 0,1,2,3) to be used in FR2 NTN.

A fundamental consideration in evaluating the performance of long PRACH formats in FR2 NTN is the impact of phase noise. Transmission of long PRACH formats requires coherent transmission over few slots (e.g. 3 ms for format 1) per RO. Since ZC is a phase sequence, present of phase noise may impact the detection performance. In this section, we study the impact of phase noise on the detection of PRACH format 1.

Table 4: Link-level Simulation Parameters

| Parameter | Value |
| --- | --- |
| Carrier Frequency (fc) | 30 GHz |
| PRACH Format | Format 1 (i.e. LRA=839, ΔfRA =1.25 KHz) |
| PRACH Configuration | Unrestricted set, NCS=0 |
| Phase Noise Model | Model-1, UE in (TR 38.803, Section 6.1.11.2 [7]) |
| Timing Error | Uniform random variable in (0, CP duration) |
| Frequency Error | Uniform random variable in (-0.5ΔfRA,0.5ΔfRA) |
| Channel Model | NTN TDL-C |
| Delay Spread | 10 ns |
| Doppler Spread | 5 Hz |
| Detection Threshold | Set to achieve 0.1% False alarm rate |
| Timing Error Tolerance | 2.55 μs |
| Frequency Error Tolerance | 0.1 ppm |

For the evaluation, we did not consider integer frequency offset since a non-zero integer offset will result in an additional shift of the correlation peak which will be solved by the techniques described in previous sections.

Based on the evaluation results in Fig. 4, we observe that the PRACH misdetection probability with phase noise goes below 1% for SNR>-16.5 dB. This SNR is significantly lower than the expected operating SNR in FR2-NTN (e.g. that required for supporting a reasonable minimum data rate in PUSCH such as 1 Mbps in FR2 NTN).

Figure 4: PRACH misdetection with and without phase noise

| (a) PSD of proposed phase noise model at UE | (b) SNR vs misdetection probability |
| --- | --- |

Observation 4: Based on link level simulation, long PRACH formats in FR2 NTN offer enough link margin even with the addition of phase noise.

Proposal 8: RAN1 to study the impact of phase noise on the detection of the existing long PRACH formats in FR2 NTN.

In the following figure, we present the timing error CDFs obtained by 10x oversampling at the gNB where the base sampling rate is 7.68 MHz. We observe that the timing error is small enough that the gNB can issue timing corrections required for UL data transmission 120 kHz SCS (i.e. 0.59 μs).

Figure 5: Timing error CDF for PRACH detection with 10x oversampling over base sampling rate of 7.6 MHz

# Feasibility of NR-Positioning to reduce Time-Frequency Uncertainty (Solution 2A)

In this section, we analyze the feasibility of any solution that includes Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

## Multi satellite-based positioning

the major challenge in NTN based UE location verification is the difficulty to have multiple available satellites for UE positioning. Even without considering the waste of resources for verifying UE reported location using multiple satellites, several other issues still severely limit the multi-satellite based operation.

…

In addition, different frequency are usually deployed for different satellites to avoid intra-frequency interference considering the timing on the different satellites is difficult to be aligned for a given UE. Therefore, inter-frequency measurement would be needed if the UE location verification is based on multiple satellite assumption. This would introduce extra-complexity on UE.

First, we recall a similar discussion on network verified UE location in release 18. In R1-2205859 [8], it was observed:

Quite obviously, these observations are valid for this scenario where we are furthermore limited to use DL-only positioning methods. The challenges for using DL-only positioning before initial access are listed as follows:

- A multi-satellite based positioning solution is dependent on the satellite constellation density: the satellite constellation density must be high enough for a UE to see multiple satellites above the minimum elevation angle

- A multi-satellite based positioning solution is dependent on beam-planning and additional resource/operational overhead:

- Even if multiple satellites are above the minimum elevation angle, they all need to serve the same region for the UE to receive DL reference signals (e.g. SSB and/or PRS) with sufficient power to perform DL RSTD measurements. This would require additional DL resources per beam.

- Either the PRS from different satellites are transmitted in different frequencies requiring inter frequency measurement at the UE or the signals from multiple satellites will interfere with each other leading to not just poor positioning accuracy but also lead to interference to other DL signals from the serving satellite

- Requires tight coordination across multiple satellites which is challenging given that the satellites are moving relative to each other and the serving area

- A multi-satellite based positioning solution needs additional standards work in 3GPP and is outside the purview of the work item description. For instance, the PRS muting framework to enable PRS measurements from non-serving satellites needs to be investigated.

Observation 5: Enabling solution 2A for initial access for GNSS-resilient NTN has the following problems:

- the availability of a positioning constellation which requires significant resource overhead to transmit PRS over additional beams, maintaining scheduling and resources.

- Significant increase in UE complexity to search PRS from multiple satellites offset by large timing and frequency uncertainty.

- In FR2 NTN, for VSAT UEs, the UE requires Rx beam sweeping to receive DL-PRS from different directions.

### UE Procedure for receiving DL PRS based on Current Specifications

The UE may be indicated by the network that DL PRS resource(s) can be used as the reference for the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo may include a dl-PRS-ID, a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of DL PRS resource IDs [17, TS 37.355]. The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference.

According to 38.214, Section 5.1.6.5 [9],

The UE expects to be configured with higher layer parameter nr-DL-PRS-ExpectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS, and nr-DL-PRS-ExpectedRSTD-Uncertainty, which defines a search window around the nr-DL-PRS-ExpectedRSTD.

Based on the above text, the search window of the UE for receiving PRS from a neighboring TRP is dimensioned as nr-DL-PRS-ExpectedRSTD-Uncertainty. See Fig. 3 for illustration. Additionally, under current specifications, the UE does not perform any frequency search for the PRS. In other words, if the UE receives a DL PRS from a neighbouring TRP which is offset by more than SCS/2, the UE is not going to detect the PRS. Since the current specification does not support an equivalent expected frequency shift signalling, the PRS from a neighbouring TRP can be multiple SCS away because of the relative DL doppler between the reference TRP and the neighbouring TRP.

Figure 6: UE PRS search window according to existing specifications.

nr-DL-PRS-ExpectedRSTD-Uncertainty

This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device.

The resolution R is

-	Ts if all DL-PRS Resources are in frequency range 2,

-	4Ts otherwise,

with Ts=1/(15000*2048) seconds.

The target device may assume that the beginning of the subframe for the DL-PRS of this TRP is received within the search window of size

-	[-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts,

where TREF is the reception time of the beginning of the subframe for the DL-PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on

-	nr-DL-PRS-SFN0-Offset

-	dl-PRS-Periodicity-and-ResourceSetSlotOffset

-	dl-PRS-ResourceSlotOffset.

The maximum value of the DL-PRS-ExpectedRSTD-Uncertainty is ±32 μs for FR1 according to 37.355 [10]:

Furthermore, for UE-based positioning, the UE will require the TRP location information. Based on the existing LPP specifications designed for terrestrial positioning, the UE can be only signaled static position of the TRPs. For LEO constellation, the TRP locations need to be either constantly updated, requiring significant signaling overhead, or the UE to propagate the TRP location over time, which requires additional signaling.

Proposal 9: For evaluation of solution 2A, the following aspects of DL-TDOA based positioning based on current specifications need to be considered:

- UE is not expected to receive DL PRS from neighboring TRP outside a search window of ±32 μs centered at a deterministic RSTD offset from the DL symbol boundary of the reference TRP.

  - For initial access in GNSS-resilient NTN using Solution 2A, this RSTD offset should be a common configuration for all the UEs within a location uncertainty area.

- DL PRS is not detectable if the frequency offset is more than ±SCS2 with respect to the reference DL frequency.

- The UE receives static TRP locations for UE based positioning and needs either (1) additional information to propagate the TRP location over time accurately, (2) frequent update of the TRP location information in assistance information.

### Evaluation Assumptions

In RAN1-123, the following was agreed:

Agreement:

With respect solution 2A, for the evaluation of DL-only UE-based positioning method, using DL-TDOA, consider the following evaluation assumptions and parameters:

[see Table 5]

Table 5: Table from RAN1-123 Agreement

| Parameter | Description/Value |
| --- | --- |
| Satellite Orbit | As per RAN1#122 agreement |
| Satellite parameters | As per RAN1#122 agreement |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | As per RAN1#122 agreement |
| BW | To be reported by companies. Baseline: 10 MHz for FR1 and 50 MHz for FR2 |
| Subcarrier spacing | 15 kHz for FR1-NTN, 120 kHz for FR2-NTN |
| Number of satellites in view | Case 1: 1 Satellite Case 2: At least 2 Satellites |
| UE type | As per RAN1#122 agreement |
| Positioning signals | DL-PRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL: Comb-2 |
| Number of symbols used per occasion | 6 |
| Number of DL-PRS occasions | To be reported. |
| Time window for measurement collection | To be reported. |
| UE speed | As per RAN1#122 agreement |
| UE minimum elevation angle | For Case 1, as per RAN1#122 agreement. For Case 2, to be reported. |
| Ue clock drift/error | To be reported |
| Performance metrics | UE 2D as baseline. 3D can be reported by companies. Residual UL timing and frequency error after applying pre-compensation based on positioning |
| UE position estimation algorithme | To be reported. |

It is to be noted that the evaluation assumptions above do not directly specify what is the search window that the UE should assume for searching for the DL PRS from a neighbour TRP (satellite) with respect to the reference DL timing and frequency- implicitly assuming an infinite search window over time and frequency for the detecting DL-PRS from the neighbour satellites. Such assumption is:

- impractical from an implementation perspective and puts unnecessary UE complexity just for the purpose of initial access in GNSS-resilient NTN

and

(b) is not compliant with the DL-PRS processing procedures as discussed above.

As we will demonstrate later, such simulation assumptions will lead to near-perfect positioning accuracy while ignoring the fundamental limitations on PRS reception based on 38.214. To align the evaluations of solution 2A with existing specifications, we propose the following amendments to this agreement.

Observation 6: The agreement in RAN1-123 on the evaluation assumptions of DL-only UE based positioning method does not put any limitation on the search window for measuring DL-PRS from neighbor satellite (TRP) which is not in agreement with the DL-PRS reception procedures in 38.214.

Proposal 10: To align the evaluations of solution 2A with existing specifications, consider adding the following rows in the evaluation assumptions:

| UE PRS Search Window in time domain (with respect to DL timing of Ref TRP) | RSTD offset ± 32 μs |
| --- | --- |
| UE PRS Seach Window in frequency domain (with respect to DL frequency of Ref TRP) | ±SCS/2 |

where RSTD offset between the reference and neighboring TRP should be common for all the UEs in the location uncertainty area.

Table 6: Parameters for evaluation of solution 2A

| Parameter | Description/Value |
| --- | --- |
| Satellite Orbit | LEO 1200km |
| Satellite Constellation | Combination of 2 layers Layer 1: Walker Star 120/15/1 with 120 satellites in 12 orbital planes at 89 deg inclination Layer 2: Walker Delta 143/11/1 with 143 satellites in 11 orbital planes at 55 deg inclination |
| Satellite Parameters | Set-1 Satellite parameters in table 6.1.1.1-1/2 of TR 38.821 |
| Channel model | NTN-Rural based on section 6.7.2 of TR 38.811 |
| Carrier Frequency | FR1-NTN: 2 GHz S-band |
| Bandwidth | 10MHz |
| Subcarrier Spacing | 15kHz |
| UE Type | Handheld |
| Positioning Signals | DL-PRS |
| Number of symbols used per occasion | 6 |
| Number of DL-PRS occasions | 1 |
| UE Minimum Elevation Angle | 20 degrees |
| UE clock drift/error | Assumed to be 0 |
| Atmospheric and Ionospheric Errors | Assumed to be 0 |
| Satellite clock error | Assumed to be 0 |
| Satellite ephemeris error | Assumed to be 0 |
| Performance metrics | UE 2D positioning error with known UE altitude |
| Positioning method | DL-TDOA |

### Evaluation Results

To evaluate the satellite visibility and UE positioning performance, we drop 1000 UEs within a circular beam of diameter 90 km (beam diameter for LEO1200 in Set-1) centered at different latitudes. We obtain the metrics at 1000 random time instances within a 24 hour period. Our simulation assumptions are presented in Table 6 and Proposal 10.

#### Satellite Visibility

A satellite to be visible if the elevation angle of the satellite at UE location and the elevation angle of the satellite at beam center are both above the minimum elevation angle of 20 degrees. In Table 7-9, we present the satellite visibility statistics at different latitudes.

Table 7: Satellite Visibility Statistics: Infinite PRS search window in time-frequency

| Number of visible satellites | Latitude |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 |
| 0 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| ≥3 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% |
| ≥4 | 82.45% | 82.65% | 90.49% | 94.61% | 100.00% | 100.00% | 100.00% | 100.00% |
| ≥5 | 43.44% | 44.82% | 62.43% | 82.03% | 100.00% | 100.00% | 99.90% | 99.99% |

Table 8: Satellite Visibility Statistics (With search window of size 64 μs in time adjusted by offset per satellite, infinite in frequency)

| Number of visible satellites | Latitude |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 |
| 0 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| ≥3 | 38.16% | 36.20% | 42.58% | 46.23% | 66.39% | 69.72% | 71.12% | 63.26% |
| ≥4 | 11.40% | 9.73% | 14.33% | 18.62% | 41.10% | 39.85% | 43.55% | 40.80% |
| ≥5 | 3.24% | 2.42% | 5.07% | 6.41% | 22.10% | 21.88% | 23.48% | 20.13% |

Table 9: Satellite Visibility Statistics (With search window of size 64 μs in time adjusted by offset per satellite, SCS in frequency with zero offset)

| Number of visible satellites | Latitude |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 |
| 0 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| ≥3 | 0.81% | 6.27% | 1.73% | 4.93% | 18.93% | 4.40% | 6.71% | 19.32% |
| ≥4 | 0.00% | 0.41% | 0.00% | 0.44% | 4.82% | 0.38% | 0.58% | 2.34% |
| ≥5 | 0.00% | 0.00% | 0.00% | 0.00% | 1.06% | 0.01% | 0.01% | 0.12% |

From the above table, we observe the following:

- Infinite PRS search window in time-frequency:

  - 100% of the time UEs see at least 3 satellites. Measuring PRS from 3 satellites provides 2 DL-TDOA measurements which allow us to perform 2D positioning assuming the altitude is known

  - >80% of the time UEs see at least 4 satellites. Measuring PRS from 4 satellites provides 3 DL-TDOA measurements which allow us to perform 3D positioning even when altitude is known

- Satellite visibility starts to drop significantly once we apply the additional constraints due to finite search window in time frequency. To put the importance of considering finite search window into perspective, we have presented the results of satellite visibility only considering search window in time and no constraints on searching over frequency in Table 8. Even with this assumption, the number of visible satellites drop significantly, e.g. by 64% for visibility of more than 4 satellites at equator.

- While Table 8 is presented to highlight the importance of considering finite search window in one dimension (time), what we should be looking at is the visibility statistics with a search window in both time and frequency. As mentioned before, it is implicitly assumed that the UE does not expect any frequency uncertainty of DL-PRS from the reference frequency of the DL TRP. If the DL Doppler difference from reference TRP and neighboring TRP is more than SCS/2, the TRP (i.e. the satellite) is no longer visible. With this constraint, in Table 9 we observe that satellite visibility drops even further.

#### Horizontal Positioning Accuracy

The drop in satellite visibility created by the limited search window in time and frequency domain affects the horizontal positioning errors. We present the 68 and 95th percentile of error in the following Tables for the different cases above. It can be observed that the positioning error with infinite PRS search window is <10m which is expected given 10 MHz PRS BW. However, with the search windows as discussed above, the positioning accuracy drops significantly due to the loss of satellite visibility. The positioning errors are in the same as the location uncertainty area sizes considered in scenario 1, for which we observed that the existing random access procedures will fail.

Table 10: Horizontal Positioning Error: Infinite PRS search window in time-frequency

| Horizontal Positioning Error | Latitude |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 |
| 95 percentile | 9.15m | 8.74m | 5.82m | 4.75m | 1.76m | 1.73m | 1.73m | 1.63m |

Table 11: Horizontal Positioning Error: With search window of size 64 μs in time adjusted by offset per satellite, infinite in frequency

| Horizontal Positioning Error | Latitude |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 |
| 68 percentile | 35.4km | 35.2km | 33.3km | 33.8km | 24.5km | 0.043km | 0.093km | 27.5km |
| 95 percentile | 43.8km | 43.6km | 43.3km | 43.6km | 43.2km | 43km | 42.9km | 42.9km |

Table 12: Horizontal Positioning Error: With search window of size 64 μs in time adjusted by offset per satellite, SCS in frequency with zero offset

| Horizontal Positioning Error | Latitude |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 |
| 68 percentile | 37.5km | 36.9km | 36.1km | 37.1km | 35.7km | 36.5km | 36.7km | 35.9km |
| 95 percentile | 43.9km | 43.6km | 43.6km | 43.9km | 43.8km | 43.7km | 43.7km | 43.9km |

Observation 7: Based on correct assumptions on the finite search window for measuring DL-PRS as outlined in 38.214, the 95-percentile horizontal positioning error is more than 25 km which is same as the location uncertainty observed in Scenario 1.

## Single satellite-based positioning

Another possible solution is to rely on DL measurements from a single satellite. Due to the high relative velocity of the satellites in LEO, a UE can measure the arrival times of PRS from the same satellite at different times and obtain a coarse location estimate and reduce the location uncertainty sufficiently.

As the UE clock is not synchronized to that at the satellite, the difference in the arrival time measured by the UE with the transmission time is not an accurate estimate of the one-way delay and cannot be directly used. Instead, the time difference between the arrival times of the PRS can be used. For example, for obtaining 4 RSTD measurements, the UE needs to measure PRS in 5 PRS occasions from the same satellite.

If PRS measurements are conducted within a brief time interval (for example, several tens or hundreds of milliseconds), the satellite’s position does not change significantly. This results in poor geometry and does not contribute to reducing location uncertainty. However, this problem can be solved, at least in theory, by increasing the duration of measurement window to few tens of seconds.

As observed in R1-2505223 [11], for LEO-1200 even after measuring 5 PRS separated by 6s resulting in a total measurement duration of 24s, the location uncertainty has not reduced sufficiently for existing PRACH configurations to handle the time and frequency error.

A higher latency (>24s) can bring the location uncertainty down within limits. Even if such a high latency is tolerable, this only works assuming an ideal clock at the UE with no error in oscillator frequency.

With an oscillator with frequency error within [-0.1,0.1] ppm and 6s gap between PRS measurements, we observe an additional error of [-0.6,0.6] μs to every RSTD measurement resulting in a very large location uncertainty.

Hence a single satellite-based position method is not viable when considering realistic UE clock models.

Observation 8: Solution 2A using TDOA measurements from single satellites is not feasible because:

- The UE requires RSTD measurements over long time window (e.g. >24 seconds). This will introduce significant latency before random access to NTN cell for GNSS-resilient UEs.

- Even with larger time windows, whether the UE will be able to reduce the time-frequency uncertainty to the range that can be handled by the existing PRACH formats depends on UE local oscillator errors.

# Frequency and Timing Adjustment in Connected mode

Based on the principles outlined in the previous section, a UE will have reduced its time and frequency errors after completion of the random access procedure. The next step is to guarantee that these errors remain within the acceptable limits during an RRC connection.

According to [12, TS 38.101-1, Clause 6.4], frequency synchronization requirement in NR is the following: the UE modulated carrier frequency shall be accurate to within ±0.1 ppm over 1 ms compared to the carrier frequency received from the gNB.

When the UE connects to an NTN cell, according to [5, TS 38.300, Section 16.14.2.2]:

- UE computes service-link Doppler shift from GNSS and ephemeris, pre-compensates in UL.

- If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained.

In connected mode, under current specifications, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation based on its GNSS and ephemeris information.

For GNSS-resilient NTN access, the UE cannot completely autonomously adjust for RTD during an RRC connection. Using a nominal frequency correction, while reducing the error, will still result in unacceptable performance in connected mode. Therefore, and in line with the study conclusions in [2], network-driven corrections are required to minimize the differential frequency error.

Observation 9: UE with location uncertainty cannot meet the Doppler pre-compensation assumption; the network must provide frequency adjustments to keep UE within a certain limit from the UL carrier frequency (e.g. within ±0.1 ppm) at gNB reception.

Proposal 11: RAN1 to study procedures to provide closed-loop frequency adjustments in connected mode.

For maintenance of timing during connected mode, current NTN specifications already include the basic mechanisms of providing closed loop corrections via timing advance commands. However, the feasibility of relying just on timing advanced commands should be assessed during the study.

In GNSS resilient operation, we observe that the RTT drift due to the satellite motion will lead to frequent transmission of TA commands. For a satellite moving at speed vsat, a static UE that observes the satellite at elevation angle θ and is located at beam azimuth ψ will experience a RTT rate  RTT=2vsatccosθcosψ. Substituting vsat=7.56 km/s for LEO-600 km,  RTT=50.45cosθcosψ μs/s. In Table 5, we summarize the RTT drift for along track beams and the corresponding number of TA commands issued per second required to keep a timing error of ±CP2, where CP=4.76 μs.

Table 5: RTT Drift and TA Command Rate for a Static UE (computed for LEO-600 km, normal CP at 15 KHz SCS)

| Satellite Elevation angle | RTT Drift (μs/s) | TA commands per second |
| --- | --- | --- |
| 900 | 0 | 0 |
| 750 | 13.06 | 5.57 |
| 600 | 25.22 | 10.76 |
| 450 | 35.67 | 15.21 |
| 300 | 43.69 | 18.63 |

We observe that, in many cases, the number of TA commands per second is unacceptable, reaching almost 20 TA commands per second (once every 50ms) for lower elevation angles. These numbers will be even worse when considering numerologies with a shorter cyclic prefix (~8 times worse for 120kHz SCS).

Observation 10: For a 600 km LEO, a static UE experiences RTT drift due to satellite motion. The gNB may need to issue up to ~20 TA commands per second to maintain a timing alignment within the CP for 15kHz SCS.

RAN1 should study the impact of timing drift on TA procedures. In particular, RAN1 should assess whether the current TAC mechanism is sufficient, or if enhancements are needed to reduce the frequency of TA commands.

Proposal 12: RAN1 to study the impact of timing drift on closed loop TA, and potential enhancements to reduce issuance of TA commands for UEs in connected mode.

To evaluate the number of TA commands issued at a given time and study methods to reduce the number of TAs (such as UE autonomous TA adjustments), it is our view that any link-level simulation should be avoided until absolutely required. To count the number of TA commands issued in a given time frame, we can drop the satellite and UE at specific positions and propagate the satellite to simulate the evolution of delay and Doppler over the simulation time. Using this framework, RAN1 can study different methods for maintaining timing and frequency adjustments in the presence of UE location uncertainty. The mechanisms for maintaining TA (FA) can be evaluated by counting the number of TACs (FACs) issued during the simulation time. Using this method, multiple TA (FA) maintenance approaches can be studied, including:

- Accumulation of TA (FA) commands issued by the network (as described above)

- Any additional UE autonomous timing adjustments

Agreement:

To assess the impacts on connected mode during NR-NTN GNSS-resilient operation, evaluate the signalling overhead for a UE in RRC Connected mode under the following assumptions:

For the evaluation of overhead associated with frequency and timing adjustment methods, RAN1-123 has agreed an evaluation methodology.

| Parameter | Description/Value |
| --- | --- |
| Satellite Orbit | As per RAN1#122 agreement |
| Satellite parameters | As per RAN1#122 agreement |
| Cell type | Quasi-Earth Fixed cell - Moving cell may be considered and reported. |
| Range of elevation angle | Baseline: From 90 degree to minimum elevation angle. Other values outside orbital plane to be reported. In this case, starting point is the highest elevation angle, and ending at the minimum elevation angle. |
| UE pre-compensation of RTT/Doppler on the service link | Case 1: At least for Scenario 1, based on assistance information provided by the Network Case 2: For Scenario 2, based on at least the last known UE location Details to be reported. Other pre-compensation methods can be reported by companies. |
| Duty cycle | Baseline: Continuous UL transmission every slot Other approaches if used, to be reported. |
| UE speed | Baseline: 3 km/h To be reported |
| Timing tolerance when connected TtolTUL (Note 1) | To be reported |
| Frequency tolerance when connected Ftol (Note 1) | To be reported |
| Initial timing error | Baseline: 0 To be reported |
| Initial frequency error | Baseline: 0 To be reported |
| Other | To be reported Note: Details of deriving UL timing and frequency error at gNB to be reported. |
| Performance metrics | Minimum period between two consecutive adjustment commands (in seconds) Maximum period between two consecutive adjustment commands (in seconds) Average number of adjustment commands per second Average number of times frequency error exceeds tolerance limit per second Average number of times timing error exceeds tolerance limit per second Other metrics may be considered and reported. |
| Note 1: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively. |  |

We provide our evaluation results as follows.

Table 13: Evaluation assumptions for connected mode simulations

| Method | # of TACs | # of FACs |
| --- | --- | --- |
| TA, FA computed by accumulating network issued TACs, FACs | 49 | 59 |

| - Trace of residual timing offset | - Trace of residual CFO |
| --- | --- |

Figure : Residual timing and frequency offset evolution. The dotted lines indicate the instance at which TAC/FAC were issued.

# Other Aspects

As per the study item description, GNSS-resilient NTN operation should be backward-compatible with GNSS-based NTN. A UE that initially accesses a cell with GNSS may temporarily lose GNSS coverage and may need to operate in GNSS-resilient mode. During this time, the UE can remain connected by using the mechanisms described in the previous sections (e.g., receiving frequency adjustment commands). Conversely, when GNSS-based position is regained, the UE should be moved back to legacy NTN mode of operation (i.e. termination of network controlled frequency adjustment and revert to time and frequency adjustments based on GNSS-based position). In order to enable this operation, RAN1 should study the associated signaling and procedures.

Proposal 13: RAN1 to study signaling/procedures for handling the loss/gain of GNSS-based location during connected mode.

# Summary

Proposal 1: Correct the typographic error in RAN1-123 Agreement as follows:

For the calculation of one-way differential delay and one-way differential Doppler, adopt Alt2 in RAN1#112bis agreement with the following correction on the one-way Doppler:

dopplerone-way(u)=vsatc2fcu-s,xdone-way(u)+vUEc2fcu-s,kdone-way(u)
where vsat is the speed of satellite and vUE is the speed of the UE.

Observation 1: According to 38.213 [6], for a single random access procedure performed by layer 1, the UE transmits a single PRACH sequence based on a PRACH Format across one or more ROs, where the PRACH sequence per RO is generated according to 38.211.

Proposal 2: Capture the attached spreadsheet the TR.

Proposal 3: Capture the corrections in highlighted text on the working assumptions of RAN1-123 in the TR.

| Preamble format | SCS | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all short formats | 15 | 15000 | 0 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
|  | 30 | 30000 | 0 | 0.1 | 2 | 2 | 2.0 | 7.40 | 29.6 |
|  | 60 | 60000 | 0 | 0.1 | 30 | 20 | 2.5 3 | 1.08 0.9 | 54 |
|  | 120 | 120000 | 0 | 0.1 | 30 | 20 | 2.5 3 | 2.28 1.9 | 114 |

Observation 2: For scenario 1, there is no PRACH format with the existing PRACH procedure which can tolerate the timing and frequency error observed in the following uncertainty areas:

Table 2: List of location uncertainty areas in scenario 1 for which no PRACH formats are compatible

| Orbit | Band | UA diameter (km) | Elevation angle (a=along, c=cross track) |
| --- | --- | --- | --- |
| LEO1200 | Ka | 40 | 30a |
| LEO1200 | Ka | 40 | 30c |
| LEO1200 | Ka | 90 | 30a |
| LEO1200 | Ka | 90 | 30c |
| LEO1200 | S | 90 | 30a |
| LEO1200 | S | 90 | 30c |
| LEO1200 | S | 190 | 30a |
| LEO1200 | S | 190 | 30c |
| LEO600 | Ka | 20 | 30a |
| LEO600 | Ka | 20 | 30c |
| LEO600 | Ka | 50 | 30a |
| LEO600 | Ka | 50 | 30c |
| LEO600 | Ka | 50 | 90 |
| LEO600 | S | 50 | 30a |
| LEO600 | S | 50 | 30c |
| LEO600 | S | 90 | 30a |
| LEO600 | S | 90 | 30c |

Proposal 4: RAN1 to conclude that with the current random access procedure with existing PRACH formats:

- gNB cannot resolve the UL timing and frequency errors observed in the majority of the location uncertainty areas at least in Scenario 1.

Proposal 5: For initial access in GNSS resilient NTN, RAN1 should study the solution which is a combination of solution 2D-1A-1D, i.e.:

- Step 1: UE applies initial timing and frequency pre-compensation based on assistance information from network (e.g. a reference location) or last known UE location

- Step 2: UE transmits two PRACH sequences generated from the existing PRACH format with different root indices at two ROs

- Step 3: UE receives timing and frequency corrections in RAR

Observation 3: Assuming that the gNB performs a single FFT (i.e. correlation) based detection per frequency hypothesis with at least two frequency hypotheses per RO (to resolve the fractional CFO), this solution is robust for any uplink timing error and frequency error as long as:

  - Uplink absolute timing error≤ minNcp, Tseq and Uplink absolute frequency error≤ K ΔfRA,

  - where Uplink absolute timing error=Differential round trip delay+Te and Uplink absolute frequency error =(Differential round trip doppler +2×0.1[ppm])× fc,ul, and K can be large enough (K<L) to allow tolerable degradation of misdetection probability.

Proposal 6: Capture the evaluation results and specification impact analysis of solution 2D-1A-1D in the TR.

Proposal 7: RAN1 should study the procedures to enable the existing PRACH formats available for FR1 (i.e. formats 0,1,2,3) to be used in FR2 NTN.

Observation 4: Based on link level simulation, long PRACH formats in FR2 NTN offer enough link margin even with the addition of phase noise.

Proposal 8: RAN1 to study the impact of phase noise on the detection of the existing long PRACH formats in FR2 NTN.

Observation 5: Enabling solution 2A for initial access for GNSS-resilient NTN has the following problems:

- the availability of a positioning constellation which requires significant resource overhead to transmit PRS over additional beams, maintaining scheduling and resources.

- Significant increase in UE complexity to search PRS from multiple satellites offset by large timing and frequency uncertainty.

- In FR2 NTN, for VSAT UEs, the UE requires Rx beam sweeping to receive DL-PRS from different directions.

Proposal 9: For evaluation of solution 2A, the following aspects of DL-TDOA based positioning based on current specifications need to be considered:

- UE is not expected to receive DL PRS from neighboring TRP outside a search window of ±32 μs centered at a deterministic RSTD offset from the DL symbol boundary of the reference TRP.

  - For initial access in GNSS-resilient NTN using Solution 2A, this RSTD offset should be a common configuration for all the UEs within a location uncertainty area.

- DL PRS is not detectable if the frequency offset is more than ±SCS2 with respect to the reference DL frequency.

- The UE receives static TRP locations for UE based positioning and needs either (1) additional information to propagate the TRP location over time accurately, (2) frequent update of the TRP location information in assistance information.

Observation 6: The agreement in RAN1-123 on the evaluation assumptions of DL-only UE based positioning method does not put any limitation on the search window for measuring DL-PRS from neighbor satellite (TRP) which is not in agreement with the DL-PRS reception procedures in 38.214.

Proposal 10: To align the evaluations of solution 2A with existing specifications, consider adding the following rows in the evaluation assumptions:

| UE PRS Search Window in time domain (with respect to DL timing of Ref TRP) | RSTD offset ± 32 μs |
| --- | --- |
| UE PRS Seach Window in frequency domain (with respect to DL frequency of Ref TRP) | ±SCS/2 |

where RSTD offset between the reference and neighboring TRP should be common for all the UEs in the location uncertainty area.

Observation 7: Based on correct assumptions on the finite search window for measuring DL-PRS as outlined in 38.214, the 95-percentile horizontal positioning error is more than 25 km which is same as the location uncertainty observed in Scenario 1.

Observation 8: Solution 2A using TDOA measurements from single satellites is not feasible because:

- The UE requires RSTD measurements over long time window (e.g. >24 seconds). This will introduce significant latency before random access to NTN cell for GNSS-resilient UEs.

- Even with larger time windows, whether the UE will be able to reduce the time-frequency uncertainty to the range that can be handled by the existing PRACH formats depends on UE local oscillator errors.

Observation 9: UE with location uncertainty cannot meet the Doppler pre-compensation assumption; the network must provide frequency adjustments to keep UE within a certain limit from the UL carrier frequency (e.g. within ±0.1 ppm) at gNB reception.

Proposal 11: RAN1 to study procedures to provide closed-loop frequency adjustments in connected mode.

Observation 10: For a 600 km LEO, a static UE experiences RTT drift due to satellite motion. The gNB may need to issue up to ~20 TA commands per second to maintain a timing alignment within the CP for 15kHz SCS.

Proposal 12: RAN1 to study the impact of timing drift on closed loop TA, and potential enhancements to reduce issuance of TA commands for UEs in connected mode.

Proposal 13: RAN1 to study signaling/procedures for handling the loss/gain of GNSS-based location during connected mode.

# References

[1] RP-251863, WID: Study on GNSS Resilient NR-NTN Operation, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

[2] Chair notes RAN1#122, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #122, Bangalore, India, Aug. 22–26, 2025.

[3] Chair notes RAN1#123, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #123, Dallas, USA, November 17–21, 2025.

[4] Chair notes RAN1#122b, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #122b, Prague, Czech Republic, Oct. 13–17, 2025.

[5] 3GPP TS 38.211, NR; Physical Channels and Modulation.

[6] 3GPP TS 38.213, NR; “NR; Physical layer procedures for control.

[7] 3GPP TR 38.803, NR; Study on new radio access technology: Radio Frequency (RF) and co-existence aspects

[8] R1-2205859, Discussion on network-verified UE location for NR NTN (3GPP TSG-RAN WG1 Meeting #110). Toulouse, France. Agenda Item: 9.12. Huawei, HiSilicon

[9] 3GPP TS 38.214, NR; Physical layer procedures for data

[10] 3GPP TS 37.355, LTE Positioning Protocol (LPP)

[11] R1‑2505223, GNSS resilience for NR-NTN (3GPP TSG-RAN WG1 Meeting #122), Bengaluru, India. Agenda Item: 10.6.1. Huawei, HiSilicon