3GPP TSG RAN WG1 #124	R1-2601289

Gothenburg, Sweden, Feburary 9th – 13st, 2026

Source:	Sharp

Title:	Enhancing DL CSI acquisition

Agenda Item:	9.2.2

Document for:	Discussion and Decision

Introduction

In RAN#108, the work item on MIMO Phase 6 was agreed as below [1]. In this contribution, we discuss the enhancements for early SRS/CSI/CSI-RS triggering and CSI-RS overhead reduction.

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

Discussions

  - Early SRS/CSI/CSI-RS triggering via RACH

Agreement: in RAN1#123 On PUSCH allocation for MSG4-triggered aperiodic CSI reporting associated with aperiodic CSI-RS for CSI, when UE transition from IDLE/INACTIVE to CONNECTED mode, support Alt-1 from Agreement in RAN1#122bis, i.e., the PUSCH is scheduled by MAC CE in MSG4 along with the aperiodic CSI report triggering. - Strive to reuse legacy UL grant design in RAR - FFS: Whether/How to handle the re-transmission of MSG4 PDSCH with the aperiodic CSI report triggering - FFS: Whether HARQ-ACK for PDSCH Msg4 is transmitted or not.

Handling the re-transmission of MSG4 PDSCH

In the previous meeting, MAC CE was agreed to convey PUSCH resource for early CSI report. The FFS point is whether/how to handle the re-transmission of MSG4 PDSCH. In the end, we consider this issue is dependent on the gNB implementation. When the gNB receives NACK, it can choose whether transmitting new data or retransmitting old data. In the transmitting new data case, the HARQ functionality cannot be utilized. However, gNB can choose to abandon the early triggering, so its behaviour remains align with the legacy specification. On the other hand, in the retransmitting old data case, it can be regarded as having determined that using the same resource as in the old uplink grant poses no issue.

Proposal 1: On re-transmission of MSG4 PDSCH with the aperiodic CSI report triggering, whether/how to handle it is up to gNB implementation.

(FL’s Note in RAN1#123) Question 1: How to determine the PL RS of SRS and QCL source of CSI-RS for CSI and TRS? - Option-1: PDCCH DMRS (corresponding to MSG2 and/or MSG4) - Option-2: The SSB the UE identified during the initial access procedure - Option-3: The TRS transmitted before MSG4-triggered CSI-RS for CSI or SRS

QCL assumption

Regarding to the QCL assumption, Option-2 and Option-3 are remaining and we support Option-2. For Option-3, to align with legacy QCL assumptions, it is reasonable. However, between early TRS and early CSI-RS, additional association or restriction on timeline will become necessary. We think each signal should be triggered separately, such association or restriction is not needed.

On the other hand, in Option-2, no such association or timeline restriction are required. Although this approach introduces QCL assumption for early triggering, we consider this to be acceptable requirement.

Proposal 2: For QCL source of CSI-RS for CSI and TRS, we support Option-2, i.e., the SSB the UE identified during the initial access procedure

Association between CSI and SRS and TRS

Regarding the association between CSI and SRS for PMI-free reporting case, we do not support it. Whether to apply precoder based on SRS is fully up to NW even though PMI-free reporting case. And in our view, there is one possible situation that UE transmit SRS via RACH for dedicated CSI reporting which report quantity is “cri-RI-CQI” after RRC setup complete. Therefore, we do not support the association between CSI and SRS.

And also, we do not support the association between CSI and TRS. In legacy, CSI-RS is always QCLed with TRS, but with above QCL assumptions, TRS is no longer needed for CSI-RS on early triggering. Furthermore, in our view, the main purpose of early TRS is to fill the gap between the beginning of the measurement for dedicated periodic TRS. Therefore, we consider early TRS should also be able to operate independently.

Proposal 3: We do not support association among CSI, SRS and TRS on early triggering.

Agreement: in RAN1#122-bis For early triggering of aperiodic CSI reporting and the associated CSI-RS for CSI when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple CSI report configurations based on one or multiple UE capability assumptions - Each CSI reporting configuration is associated with a CSI resource configuration for channel measurement. - The CSI-RS resource set(s) in the CSI resource configuration for channel measurement is provided with a same number of CSI-RS ports - Each CSI reporting configuration is associated with a CSI resource configuration for CSI-IM based interference measurement. - FFS: Support of NZP CSI-RS based interference measurement - FFS: Information to be provided in a CSI report/resource configuration - In Step-2, UE reports through MSG3 which/whether the CSI report configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the CSI report configuration(s) provided in SIBx is/are supported.

Configuration for CQI calculation

When calculating CQI, the UE assumes several conditions, e.g., the number of PDSCH and DMRS symbols, bandwidth part subcarrier spacing, and more. However, among those assumed conditions, the DMRS additional position (dmrs-AdditionalPosition) and the number of front-loaded symbols (maxLength) are not provided during the random access procedure because they are based on dedicated parameters.

There are several ways to address this issue, such as providing the parameter via SIBx, or indicated in MAC CE. However, in this case, we believe that such flexibility is not needed, and it is sufficient that the UE assumes the most commonly used settings for the DMRS additional position and max length, specifically Pos2 and 1.

Proposal 4: For CQI calculation on early CSI triggering, the UE assumes the following conditions:

- dmrs-AdditionalPosition = Pos2

- maxLength = 1

Conclusion

In this contribution, we have the following proposal:

Proposal 1: On re-transmission of MSG4 PDSCH with the aperiodic CSI report triggering, whether/how to handle it is up to gNB implementation.

Proposal 2: For QCL source of CSI-RS for CSI and TRS, we support Option-2, i.e., the SSB the UE identified during the initial access procedure

Proposal 3: We do not support association among CSI, SRS and TRS on early triggering.

Proposal 4: For CQI calculation on early CSI triggering, the UE assumes the following conditions:

- dmrs-AdditionalPosition = Pos2

- maxLength = 1

References

- [1]	RP- 251856, New WID: NR MIMO Phase 6, June, 2025.