3GPP TSG-RAN WG1 Meeting #124	R1-2601356

Gothenburg, Sweden, February 09th – 13th, 2026

Agenda Item:	10.1

Source:	Tejas Networks, CEWiT, Indian Institute of Technology Madras (IITM)

Title:	Evaluation assumptions for 6GR Air Interface

Document for:	Discussion and Decision

# Introduction

This contribution provides the common 6GR evaluation assumptions for rural deployment scenario as defined in the latest TR 38.914 [1] and the new deployment scenario “single cell with large coverage” proposed in [3, 4] which is under discussion [2].

# Discussion

In this section, we propose a new deployment scenario for rural geographies, called Single Cell with Large Coverage. It is an isolated macro cell deployment with, clustered, low population density where communication is through terrestrial technologies only.

Figure 1: Isolated Macro Cell with cell radius up to 6 km.

Proposal 1: Study Isolated Macro cell with cluster UE drop as one of the deployment scenarios for Rural in 6G.

## Single Cell with Large Coverage

The motivation for the single cell with large coverage is to cater basic digital services such as Internet based banking, remote health and digital education for the Indian villages with low population density. These services would have different data rate and latency requirements.

The sub-1 GHz band provides better coverage compared to sub-6 GHz and FR2 as per the physical properties of EM waves. The 700 MHz band is generally considered as the proxy for sub-1 GHz. However, due to the spectrum availability and the imminent auction of spectrum around 600 MHz coupled with the extra coverage that 600 MHz offers, we propose to study 600 MHz (both 600 MHz and 700 MHz if needed) in the sub-1 GHz range for this deployment scenario.

To achieve the requirements of telecom services for the single cell deployment scenario mentioned earlier in this section, we propose to use to following 2 evaluation assumptions.

- 600 MHz (Around 700 MHz) with up to 32 BS antenna elements, 2-4 UE antenna elements (1.5m Handheld UE)

- 7GHz with FWA with up to 1024 BS antenna elements, 8-16 UE antenna elements (5m CPE)

Proposal 2: For 6GR evaluation, study 600 MHz and 7 GHz as carrier frequencies for handheld UE and FWA respectively, for the rural isolated macro cell deployment. Consider Table 2.1-1 and Table 2.1-2 as the evaluation parameters for rural isolated macro cell deployments for large coverage.

Other attributes related to these 2 options are provided in the following tables.

Table 2.1-1: Evaluation parameters for eMBB Extreme Rural (Single Cell with Large Coverage)

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 600 MHz (Around 700 MHz) |
| Base station Antenna elements | 32 (Baseline), 4 TXRU, (M, N, P, Mg, Ng; Mp, Np) = (8, 2, 2, 1, 1; 1, 2), (dH,dV)= (0.5, 0.5)λ 64 (Optional), 8 TXRU, (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; x, y), (dH,dV)= (0.5, 0.5)λ |
| Aggregated BW | Up to 60 MHz |
| Simulation BW | 20 MHz (FDD) 60 MHz (FDD) |
| Layout | Single cell layout [Isolated Macro cell] |
| Cell radius | 2km to 6km |
| User density and UE speed | 10 users per cell (distributed along cell edge) 20% Outdoor pedestrian: 3km/h 10% Outdoor in cars: 30km/h or 60km/h 70% Indoor: 0-3km/h |
| UE height | 1.5m |
| Traffic model | Full buffer |
| KPI | DL Coverage (Note 1), UL Coverage (Note 1), 5th percentile spectral efficiency, Average spectral efficiency |

Table 2.1-2: Evaluation parameters for FWA Extreme Rural (Single Cell with Large Coverage)

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 7GHz |
| Base station Antenna | 1024, 256 TXRU, (M, N, P, Mg , Ng; Mp, Np) =(32, 16, 2, 1, 1; 8, 16), (dH,dV)= (0.5, 0.8)λ 2048, 256 TXRU, (M, N, P, Mg , Ng; Mp, Np) =(32, 32, 2, 1, 1; 8, 16), (dH,dV)= (0.5, 0.5)λ 2048, 512 TXRU, (M, N, P, Mg , Ng; Mp, Np) =(32, 32, 2, 1, 1; 8, 16), (dH,dV)= (0.5, 0.5)λ |
| Aggregated BW | Upto 400 MHz |
| System Bandwidth | 20 MHz (TDD) 100 MHz (TDD) 200 MHz (TDD) 400 MHz (TDD) |
| Layout | Single cell layout [Isolated Macro cell] |
| Cell radius | 2km to 6km |
| User density and UE speed | 10 users per cell (distributed along cell edge) 100% Outdoor stationary: 0km/h |
| UE height | 5m |
| Traffic model | Full buffer |
| KPI | DL Coverage (Note 1), UL Coverage (Note 1), 5th percentile spectral efficiency, Average spectral efficiency |

Note 1: Coverage is expressed as the Maximum Link Distance with target data rate x Mbps. The actual value of x for download and uplink data channels can be defined.

## Link Budget Templates

In the recent RAN1 WG meeting (RAN1#122 bis) (1), two candidate templates are agreed to perform link budget analysis. Candidate 1 is link budget template from TR 38.830 (NR Coverage enhancements) and Candidate 2 is for Maximum coupling loss (MCL) provided in Table 7.10.1-1 of TR 38.913.

In this contribution, we propose some additions to Candidate 1 template to compute available path loss and maximum range. We would like to incorporate target data rate with a specified BLER (19bis) for data channel and link it with the required SNR (19) of the table. Our proposed changes (highlighted text) are added to the Candidate 1 template below

| (19) Required SNR (dB) = SNR for (19bis) | SNR required to achieve target data rate of X Mbps |
| --- | --- |
| (19bis) Target data rate for the allocated bandwidth (3c) (Downlink and uplink data channels) with Y% BLER (e.g, Y=10) | X Mbps |
| (20) Receiver implementation margin (dB) |  |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS |  |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) |  |
| (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) |  |
| (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12) |  |
| Calculation of available pathloss |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29) Available path loss = (23) – (25) + (26) – (27) + (28) (dB) |  |
| Range/coverage efficiency calculation |  |
| FFS: (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) |  |

## NTN

Related to NTN, some initial evaluation parameters on frequency bands are agreed in the last RAN1 WG meeting along with evaluation parameter template in [5]. We propose considering payload types including LEO, GEO and HAPS to be considered under the NTN deployment scenario.

HAPS based IMT networks are essential for resilience and ubiquitous coverage perspective. IMT 2030 requirement document [6] agreed resilience and extended connectivity as one of the technical requirements as below:

## 4.17 Resilience and Extended Connectivity This requirement includes the following capabilities: Resilience: It refers to capabilities of the networks and systems to continue operating correctly during and after a service disruption, such as the loss of primary source of power, etc. The RIT/SRIT should support functionalities over the radio interface that enable continuous operation or rapid temporary restoration during and after disturbance to radio infrastructure. Extended Connectivity: It refers to the capability to provide connectivity in areas that are currently uncovered or scarcely covered due to challenges arising from geographical conditions, such as the lack of ground infrastructure (e.g., fiber cables and power supply), where users desire access to communication services. The RIT/SRIT should have functionalities that support providing geographically extended connectivity in these areas. Functionalities related to resilience and extended connectivity may include, but are not limited to, support for connecting to one or more of the following advanced network infrastructures within the terrestrial component of IMT: - HIBS - Relays and repeaters - Shared network architectures to provide redundancy and backup - Base station onboard a vehicle, airship and/or vessel - or other mechanisms for these capabilities using network infrastructures within the terrestrial component of IMT This requirement is defined for the purpose of evaluation in the Ubiquitous Connectivity usage scenario and may also be applicable to other usage scenarios.

Based on this HIBS/HAPS is important scenario to be studied in 3GPP. We propose the following table on evaluation parameters for HAPS considering the agreed template “Tab2 SLS for NT”.

Table 2.4-1: Evaluation parameters for HAPS under NTN scenarios

|  | Parameters | HAPS |
| --- | --- | --- |
| Deployment parameters | Satellite orbit | NA |
|  | HAPS altitude | 20 km |
|  | SLS Methodology (Single or Multiple HAPS ) | Single/Multiple HAPS |
|  | Cell/Beam Type (Earth-fixed, Quasi-earth fixed, Earth moving) | Earth Fixed |
|  | Cell Deployment (One beam per cell, Multiple beams per cell) | One beam per cell |
|  | Minimum User Elevation Angle | 30 (Maximum elevation can go up to 90) |
| Payload parameters | Payload Type | Regenerative, Transparent |
|  | DL carrier frequency | 700 MHz, 2 GHz |
|  | Equivalent satellite antenna aperture |  |
|  | Payload Total DL power level (dBm) | 43 dBm |
|  | Aggregated Satellite EIRP (Total) (dBm) | 59.8 dBm |
|  | Carrier bandwidth | 20 MHz |
|  | HAPS EIRP density/beam (dBm/MHz) | 46.8 dBm/MHz |
|  | HAPS Tx max Gain | 16.8 dBi |
|  | HAPS 3 dB beam width (HPBW) | 65 |
|  | HAPS beam diameter | 30Km |
|  | Total number of simultaneously active beams | 7 |
|  | UL carrier frequency | 2 GHz |
|  | Payload G/T | TBD |
|  | HAPS Rx max Gain | 16.8 dBi |
| UE parameters | Rx Gain | 0 dBi |
|  | Noise Figure | 9 dB |
|  | Antenna temperature | TBD |
|  | UE G/T | NA |
|  | Tx transmit power | 23 dBm |
|  | Tx antenna gain | 0 dBi |
| Reference constellation parameters | Constellation details (orbital planes, satellites per orbital plane) | NA |
|  | Beam layout | Seven-cell structure |
|  | Total number of beam footprints | 7 |
|  | Satellite position update frequency (e.g., simulation sampling rate) | NA |
|  | Inter-beam distance (IBD) | 26Km |
|  | Inter-Beam-Interference Modelling | NA |
|  | Beam footprint-to-satellite mapping (for multi-satellite) | NA |
| Others | Frequency re-use factor | 1 |
|  | Polarization re-use | NA |
|  | Propagation conditions | NLoS |
|  | UE distribution | 10 UEs per Cell |
|  | UE orientation | Random |

Proposal 5: For 6GR evaluation, consider HAPS as one of the payloads under NTN deployment scenarios. Consider Table 2.4-1 as the Evaluation parameters for HAPS payload.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Proposal 1: Study Isolated Macro cell with cluster UE drop as one of the deployment scenarios for Rural in 6G.

Proposal 2: For 6GR evaluation, study 600 MHz and 7 GHz as carrier frequencies for handheld UE and FWA respectively, for the rural isolated macro cell deployment. Consider Table 2.1-1 and Table 2.1-2 as the evaluation parameters for rural isolated macro cell deployments for large coverage.

Proposal 3: The link budget template should specify the target data rate to obtain required SNR.

Proposal 4: For 6GR evaluation, consider HAPS as one of the payloads under NTN deployment scenarios. Consider Table 2.4-1 as the Evaluation parameters for HAPS payload.

- References

- RP-253750, TR 38.914 v 0.3.0 Study on 6G Scenarios and Requirements, December, 2025.

- RP-253802, Moderator’s summary for 6G SI: Deployment Scenarios, RAN #110, December 2025.

- RP-252822 (Revision of RP-252118), Discussion on 6G Deployment Scenarios, Tejas Networks, CEWiT, IIT Madras, September 2025.

- R1-2507895, Evaluation Assumptions for 6GR, Tejas Networks, CEWiT, IIT Madras, October 2025.

- R1-2507956, The evaluation assumptions template for NTN_v04, RAN1#122bis meeting

- Chairs Report Chapter 5 - Annex 5.6 - Preliminary draft new Report ITU-R M.[IMT-2030.TECH PERF REQ] - Minimum requirements related to technical performance for IMT-2030 radio interface(s), WP5D