3GPP TSG RAN WG1 #124                                 R1-2601411

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda item:	10.5.1.1

Source: 	CSCN

Title:	Discussion on synchronization acquisition and beam measurement

Document for:	Discussion & Decision

- Introduction

In this contribution, we discuss the synchronization acquisition and beam measurement.

- Discussion

  - 2.1 Synchronization acquisition

6GR is designed for supporting diverse device types with ubiquitous connectivity, which requires wide-area, continuous, and seamless network coverage. NTN inherently possesses global coverage characteristics, making it a key technology for achieving this goal. Consequently, 6GR devices should natively support NTN access or have the capability to extend their functionality to support NTN access. Accordingly, as the first step for UE access, the initial access configurations and procedures should be maximumly harmonized between TN and NTN. In other words, the initial access design, including the design of sync signal/channel, physical random access channel (PRACH), random access procedure, and sync acquisition procedure, should inherently accommodate the NTN propagation characteristics and coverage performance. This design approach also aligns with the core 6G principle of harmonization between NTN and TN.

In the agenda item for overview of 6GR air interface, several aspects were agreed to be considered for supporting NTN, and most of which are highly relevant to the synchronization design [1].

Agreement - The aspects to consider for supporting NTN include, but not limited to - Initial access, including cell search and SSB periodicity - Coverage - Duplexing - Capacity - Signalling overhead - GNSS-less/resilient/based operation - Large/varying doppler and propagation delay - Beamforming / beam management / beam hopping

Proposal 1: The design of sync signal/channel, PRACH, random access procedure, and sync acquisition procedure, should inherently accommodate the NTN propagation characteristics and coverage performance.

Proposal 2: The aspects for supporting NTN as agreed in RAN1#122-bis should be considered in synchronization acquisition.

### 2.1.1 Synchronization signal/channel design

In frequency domain, the minimum spacing between Global Synchronization Channel Number (GSCN) points should exceed twice the maximum Doppler shift, sparser sync raster is needed for NTN considering the larger doppler compared with TN. For example, assuming a 40 degrees elevation, LEO-600km orbit, and the satellite velocity of 7.55 km/s, the maximum frequency offset between the satellite and the UE is calculated to be 52.8kHz for 3GHz carrier frequency, doubling of which already exceeds the existing 100kHz GSCN spacing for frequencies below 3 GHz. Therefore, if sub‑3 GHz bands are deployed for NTN, sparser sync raster is required. Moreover, this sparse raster can also be applicable to TN where longer SSB periodicity might be adopted for energy saving, thereby lowering the UE access latency. Furthermore, a unified sync raster design for TN and NTN is recommended, as it would avoid the complexity and redundancy of multiple/separate designs, especially considering the potential reuse of TN bands in NTN.

Proposal 3: Sparser sync raster should be considered in the sync signal/channel design.

Then, in time domain, the sync signal periodicity and structure should be designed to accommodate both TN and NTN characteristics. Specifically, coverage and beam hopping are essential aspects to be considered for supporting NTN. To achieve better coverage performance in NTN with beam hopping scheme and/or reduce the signaling overhead, at least the 160ms SSB periodicity already supported in NR should be adopted as the default SSB periodicity during initial access in 6GR. Besides, the SSB periodicity can currently be configured up to a maximum of 160ms. Considering the energy efficiency in both TN and NTN, as well as the coverage performance of potential sparse constellation in the early stage of NTN deployment, larger SSB periodicity beyond 160ms, e.g., 320ms or 640ms, could be further considered in 6GR.

Proposal 4: The existing SSB periodicities already supported in NR should be retained for 6GR, while the introduction of longer periodicities should be considered.

- At least the 160ms SSB periodicity currently supported in NR should be adopted as the default periodicity during initial access in 6GR.

- The maximum configurable SSB periodicity shall be extended beyond 160ms in 6GR.

### 2.1.2 Other design for facilitating synchronization acquisition

After achieving downlink synchronization via SSB reception and MIB decoding, the UE should receive necessary information for initiating random access (RA) via SIB. In NR, such essential information reception procedure differs between TN and NTN. To be specific, while TN-camped UEs acquire RA-related configurations solely via SIB1, which refers to the remaining minimum system information (RMSI), NTN UEs must additionally receive SIB19 to obtain satellite ephemeris for timing and frequency offset pre-compensation. Such additional procedure prolongs the access latency, while causing performance degradation in NTN. Besides, in 6GR, NTN-specific designs should be minimized to promote harmonization between TN and NTN, especially during initial access procedures. Moreover, NTN is a key technology for emergency scenarios where TN services are not or poorly available (e.g. where disasters disrupt TN or damage undersea communication cables). To ensure the wireless communication in such scenarios, essential NTN-related information for initial access should be accessible to diverse devices, regardless of whether NTN access is enabled. Based on the above analysis, essential information for initial access in GNSS-based/less/resilient NTN systems (e.g., ephemeris data, common TA parameters, timing synchronization parameters, and polarization information carried in SIB19, as well as the network assistance information might be introduced in R20 5G-A GNSS-resilient study) should be included in MSI messages to reduce UE synchronization complexity and shorten access latency. Meanwhile, since MSI is received before the UE initiates random access, its size must be considered to ensure the coverage performance. Specifically, information essential for access in both TN and NTN can be included in the MSI, while other information that has less impact on the execution of access can be transmitted later.

Proposal 5: The 6G SIB design should consider the harmonized integration of TN and NTN, with essential NTN-related access information included in the Minimum SI.

Besides, there exists a large scheduling gap between SSB and SIBs carrying essential TN/NTN access-related information, especially for SIB19, during which the UE must maintain downlink synchronization through some time-frequency tracking algorithms. Additionally, due to the higher dynamics of NTN compared to TN, its delay and Doppler shift vary rapidly and significantly over time. This poses a significant challenge to maintaining accurate downlink synchronization during the relatively long scheduling gap, more complicated tracking algorithms might be required, leading to increased UE complexity. Furthermore, the delay and Doppler shift during the gap can degrade decoding performance and potentially lead to SIB acquisition failure, preventing the UE from accessing the network. In summary, to ensure the effectiveness of time-frequency tracking and guarantee the correct SIB reception, SIBs carrying essential TN/NTN access-related information should be scheduled closer to SSB, and a finer-grained scheduling interval can be considered.

Proposal 6: The SIBs carrying essential TN/NTN access-related information should be scheduled closer to SSB.

  - 2.2 Beam measurement

Based on the harmonized design principle for TN and NTN, a unified beam measurement framework should be adopted, while taking NTN-specific characteristics into account. For example, given the high predictability of satellite ephemeris and propagation conditions in NTN, simplified/sparser reference signal configurations for beam measurement can be introduced. Besides, under the long-delay and wide-coverage conditions of NTN, the reference signal-based mobility management introduces excessive signaling overhead and handover latency, for addressing this, more adaptable mobility management in NTN should be considered.

Proposal 7: A unified beam measurement framework should be adopted, while taking NTN characteristics into account, thereby supporting more adaptable CSI acquisition and mobility management in NTN.

- Conclusions

In this contribution, we provide some views on synchronization acquisition and beam measurement. The proposals are summarized as following.

Proposal 1: The design of sync signal/channel, PRACH, random access procedure, and sync acquisition procedure, should inherently accommodate the NTN propagation characteristics and coverage performance.

Proposal 2: The aspects for supporting NTN as agreed in RAN1#122-bis should be considered in synchronization acquisition.

Proposal 3: Sparser sync raster should be considered in the sync signal/channel design.

Proposal 4: The existing SSB periodicities already supported in NR should be retained for 6GR, while the introduction of longer periodicities should be considered.

- At least the 160ms SSB periodicity currently supported in NR should be adopted as the default periodicity during initial access in 6GR.

- The maximum configurable SSB periodicity shall be extended beyond 160ms in 6GR.

Proposal 5: The 6G SIB design should consider the harmonized integration of TN and NTN, with essential NTN-related access information included in the Minimum SI.

Proposal 6: The SIBs carrying essential TN/NTN access-related information should be scheduled closer to SSB.

Proposal 7: A unified beam measurement framework should be adopted, while taking NTN characteristics into account, thereby supporting more adaptable CSI acquisition and mobility management in NTN.

- References

- Chairman's Notes RAN1#122-bis, Oct. 13th-17th, 2025.