3GPP TSG-RAN WG1 #76BIS	R1-141447

31st March – April 4th, 2014

Shenzhen, China

Agenda item:   7.2.7.1.1

Source:            Qualcomm Incorporated

Title:                  Link level simulation results for D2D broadcast communication

Document for: Discussion/Decision

# 1	Introduction

In this contribution, we present link level results along with techniques for improvement for VoIP for D2D broadcast communication.

We look at design targets of -107 dBm and -112 dBm as per [1]. We note that for either design target, retransmissions as well as diversity techniques are needed to improve link level performance for VoIP.

In particular, we look at the impact of:

- Impact of number of retransmissions

- Impact of TTI Bundling

- Impact of time interleaving to harness time diversity

- Impact of frequency interleaving to harness frequency diversity

Based on the simulation results, we propose:

- To take -107 dBm as the design target for D2D VoIP communication.

- To take frequency diversity and TTI bundling as mechanisms for improving link level performance

# 2  	Simulation Assumptions

| Parameters | Assumptions |
| --- | --- |
| Carrier Frequency, System Bandwidth | 700 MHz, 10 MHz |
| VoIP Packet Size | 44 Bytes (including CRC) |
| Channel Model | ITU UMi NLOS (reduced variability) |
| Number of RX antennas | 2 |
| Coding/Modulation | Turbo/QPSK |
| HARQ Transmissions | Chase Combining |
| Channel Estimation Algorithm | Quadratic Interpolation |
| Transmission Bandwidth | 2 PRBs |

We note that -112dBm link budget requirement translates to an SNR of –3 dB over 2 PRBs, and -107 dBm corresponds to SNR of 2 dB. Thus the target rate is less than 2% BLER at SNR of – 3 dB or 2 dB – these are highlighted in the figures as reference points for comparison.

We study various schemes to increase performance at a link level, as shown in in Figure 1 below. These schemes can be summarized as:

- Basic – where a VoIP packet is re-transmitted on the same frequency at every scheduling opportunity.  The scheme is parameterized by number of retransmissions (n retransmission = n+1 transmissions of the packet).

- TTI Bundling – here we assume that each transmission instead of a single SF (sub-frame) could be multiple consecutive sub-frames. This enables joint channel estimation across multiple sub-frames and hence improves performance.

- Time diversity – here, we make use of the 200ms delay target for the VoIP packet to spread the packet out over 200ms.

- Frequency diversity – here, we exploit frequency diversity by hopping to a different PRB pair for each HARQ transmission. We implement randomized hopping where each PRB pair is selected uniformly randomly for each HARQ transmission.

Figure 1 Schemes to improve link budget

# 3  Simulation Results

In this section, we present link level simulation results for VoIP packet transmissions for D2D broadcast. Below, we simulate the BLER result for each improved transmission scheme and evaluate the performance gain by comparing the SNR to achieve the target BLER of 2% against a corresponding basic scheme. All the results are also provided in the Appendix A for both 700 MHz and 2 GHz carrier frequencies.

## 3.1 Understanding channel behavior

We first study the behavior of the ITU UMi NLOS fading channel. Below, we plot the sample paths of channel gains in different PRBs during a simulation period of 2000ms (at 700 MHz). Note that the channel gains are averaged over two receiver antennas.

Figure 2 Channel variations in time (sample paths)

We note that the channel coherence time is around 100ms. Thus, diversity gain can be achieved by time interleaving retransmissions over the VoIP delay budget (200ms).

We next plot the sample paths of channel gains across different PRBs. The results are averaged over two receiver antennas.

Figure 3 Channel variations in frequency (sample paths)

It can be observed that the channel coherence bandwidth is around 10-15 PRBs. Thus, diversity gain can be achieved by interleaving retransmissions in frequency across the system bandwidth (50 PRBs).

## 3.2 Impact of Retransmissions

To improve the link level BLER performance, we next investigate the impact of retransmissions. Below, we compare BLER results of the basic schemes with different number of retransmissions.

-107 dBm

-112 dBm

Figure 4 BLER results with different retransmissions

Increasing the number of retransmissions from 3 to 7 improves the SNR by around 3dB for the target BLER of 2% which is inline with the expected gain due to more accumulated energy per packet.

## 3.3 Impact of TTI Bundling

Next, we investigate the impact of TTI bundling. In below, we plot BLER results with different TTI bundling schemes.

-112 dBm

-107 dBm

Figure 5 BLER results with different TTI bundling schemes

Introducing TTI bundling (2 TTI Bundling, 3 ReTx) over the basic scheme (Basic, 7 ReTx) improves the SNR by around 2dB for the target BLER of 2%.

We make the following observation:

Observation 1: TTI Bundling provides an additional gain of 2dB over retransmissions.

## 3.4 Impact of Frequency Diversity

Next, we look at the impact of frequency diversity. The following figure shows the BLER results with frequency interleaving for different transmission schemes.

-112 dBm

-107 dBm

Figure 6 BLER results with frequency interleaving for different transmission schemes

The frequency interleaving scheme (Basic, 3 ReTx, and Freq Diversity) improves the SNR by around 4dB over the base scheme (Basic, 3 ReTx) for the target BLER of 2%.

We make the following observation:

Observation 2: performance gain of 4dB can be achieved by frequency interleaving.

## 3.5 Impact of Time Diversity

Finally, we investigate the impact of time diversity. The following figure shows the BLER results with time interleaving for different transmission schemes.

-112 dBm

-107 dBm

Figure 7 BLER results with time interleaving for different transmission schemes

The time interleaving scheme (Basic, 3 ReTx, Time Diversity) improves the SNR by around 4dB over the base scheme (Basic, 3 ReTx) for the target BLER of 2%.

We make the following observation:

Observation 3: performance gain of 4 dB can be achieved by time interleaving.

# 4 	Design implications

First, we note that to meet -112 dBm is a stringent requirement as a total of 8 transmissions per packet are needed. Further, we note that TETRA DMO has a reference sensitivity of -103 dBm for fading channels [2]. Based on this, we observe and propose:

Observation 4: 8 transmissions/packet are needed to meet -112 dBm design target

Observation 5: TETRA DMO has a reference sensitivity of -103 dBm

Proposal 1: -107 dBm should be the design target for D2D VoIP communication.

Further, to meet -107 dBm design target, we summarize the gains of various schemes below:

Table 1 Summary of simulation results

| Scheme | Gain achieved |
| --- | --- |
| Retransmissions | Up to 7 dB (energy gain) |
| Retransmissions with frequency diversity | 4 dB + energy gain |
| Retransmissions with time diversity | 4 dB + energy gain |
| Retransmissions with TTI bundling | 2 dB + energy gain |

Based on this, we propose to take frequency diversity and TTI bundling as the two schemes for  link level improvements – we note that time diversity techniques incur a delay and additional increase UE complexity by requiring UEs to store LLRs for longer period of time. Given that the performance gain of time diversity is comparable to performance gains obtained via frequency diversity, we propose to use frequency diversity as the diversity technique. We note that the gains by combining time and frequency diversity techniques are not significant over using them individually – these results are included in the Appendix.

Proposal 2: TTI bundling and frequency diversity schemes should be used to improve link level performance.

We further note that TTI bundling techniques are already defined in the context of improving UL coverage, and hence should be reused.

Proposal 3: existing TTI bundling techniques should be reused with support for different sized TTI bundling if needed.

# 5 	Conclusion

In this contribution, we make the following observations:

Observation 1: TTI Bundling provides an additional gain of 2dB over retransmissions.

Observation 2: performance gain of 4dB can be achieved by frequency interleaving.

Observation 3: performance gain of 4dB can be achieved by time interleaving.

Observation 4: 8 transmissions/packet are needed to meet -112 dBm design target

Observation 5: TETRA DMO has a reference sensitivity of -103 dBm

Based on these observations, we make the following proposal:

Proposal 1: -107 dBm should be the design target for D2D VoIP communication.

Proposal 2: TTI bundling and frequency diversity schemes should be used to improve VoIP performance.

Proposal 3: existing TTI bundling techniques should be reused with support for different sized TTI bundling if needed.

# References

- 3GPP TR 36.843

- ETSI EN 300 396-2 V1.3.1,  “Terrestrial Trunked Radio (TETRA); Technical requirements for Direct Mode Operation (DMO); Part 2: Radio aspects”

# Appendix

## A. BLER Results (700MHz)

Here, we present BLER results of all transmission schemes with carrier frequency of 700MHz.

Figure 8 BLER results of all transmission schemes for carrier frequency of 700MHz

## B. BLER Results (2GHz)

Here, we present BLER results of all transmission schemes with carrier frequency of 2GHz.

Figure 9 BLER results of all transmission schemes for carrier frequency of 2000MHz