3GPP TSG RAN WG1 Meeting #79                                      	     R1-144869

San Francisco, USA, 17th – 21st November 2014

Agenda item:	6.3.3.2

Source:	            NEC

Title:               Initial performance evaluation

Document for:	   Discussion and Decision

- Introduction

In the study item of Rel-13 [1], the potential enhancement for implementing elevation beamforming or full dimension MIMO will be evaluated in order to identify the antenna configurations and the simulation scenarios. In the first step, the performance of Re.12 downlink MIMO using 3D-UMa and 3D-UMi channel models should be evaluated as the baseline. In the RAN1#78bis meeting, there is an agreement of simulation assumptions for Homogeneous scenarios based on [2][3] as following

Agree R1-14444 with following updates

- In Page 3, Carrier frequency: Mandatory: 2GHz for 3D UMi and 3D UMa with 200 m and 500 m ISD, 3.5 GHz for 3D UMi, Optional: 3.5 GHz only for 3D UMa with 200 m ISD

- In Page 6, CSI-RS, CRS: CSI-RS 1-1 mapping to TXRU, only CRS port 0 is modeled for UE attachment, CRS port 0 is associated with the first column with +45 degree pol, CRS port 0 to TXRU mapping is ideal and given by [1, 0, 0, 0, 0, 0, 0, 0]

- In Page 5, Transmission scheme:

  - TM10, single CSI process, dynamic SU/MU-MIMO with rank adaptation

  - Single CSI process is used for phase 1 simulation only and the number of CSI processes allowed will be discussed in phase 2 simulation

- In page 6, Downtilt

  - Email discussion until 16th Oct. to confirm downtilting angle

And some further details were determined in the subsequent email discussion [78bis-18] as

- 3D-UMa with ISD=500m, 2 GHz: 100deg

- 3D-UMa with ISD=200m, (2 GHz mandatory, 3.5 GHz optional): 104 deg

- 3D-UMi with ISD=200m, (2 GHz, 3.5 GHz – both mandatory): 100 deg

In this contribution, we provide our preliminary simulation results with FTP model 1 for homogeneous scenarios. The detailed simulation assumption can be found in Appendix.

- Discussion

In this contribution, all simulation results are based on SU-MIMO, and 5%, 50% and 95% user packet throughput are all presented. For comparing the performance of different scenarios, the same user arrival rates for different scenarios, i.e. lambda = 2, 3, 3.5 are adopted in our simulation. Table 1 is for UMa scenario where centre frequency is 2 GHz, cell ISD is 500m, the fixed down tilt is 100 degree and the transmit power is 46dBm. Table 2 is for UMa scenario where cell ISD is 200m, the fixed down tilt is 104 degree, the transmit power is 41 dBm and centre frequency is 2GHz. Table 3 is for UMi scenario where centre frequency is 2GHz, cell ISD is 200m, the fixed down tilt is 100 degree and the transmit power is 41 dBm. 3.5GHz centre frequency is adopted in table 4 compared with table 3.

As shown in table 1, table 2 and table 3, we can find that the performance of the UMa with 500m ISD scenarios is larger than that of UMi scenario for high traffic load. Since the antenna height in UMi scenario is only 10m but the users can locate higher than antennas in UMi Scenario, some users with higher locations cannot achieve the good antenna gain with the fixed down tilt 100 degree. Compared with UMi scenario, UEs cannot locate higher than the antenna in UMa scenario, a good coverage in vertical space can be achieved with a reasonable down tilt. In addition, the performance of UMa scenario with 500m ISD is larger than that in UMa scenario with 200m ISD for lambda =3 and 3.5 because of smaller transmit power and cell radius in latter scenario.

Observation1: The performance of UMa with 500m ISD is larger than that in UMi and UMa with 200m ISD.

As described in table 3 and table 4, the obvious performance gap can be found between 2GHz and 3.5GHz centre frequency. Since the path loss in 3.5 GHz centre frequency is larger than that in 2GHz, and the large scale fading play an important role in low interference cases, lower performance results is reasonable in 3.5GHz scenario.

Observation 2: The performance of UMi with 2GHz is larger than that with 3.5GHz.

Table 1 UMa with ISD 500m

| User arrival rate | RU | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) |
| --- | --- | --- | --- | --- |
| 2 | 0.24 | 8.68 | 28.57 | 52.63 |
| 3 | 0.43 | 5.04 | 18.69 | 52.63 |
| 3.5 | 0.64 | 1.54 | 10.55 | 49.38 |

Table 2 UMa with ISD 200m

| User arrival rate | RU | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) |
| --- | --- | --- | --- | --- |
| 2 | 0.22 | 10.08 | 30.08 | 52.63 |
| 3 | 0.5 | 2.64 | 14.98 | 50.63 |
| 3.5 | 0.67 | 1.31 | 8.93 | 43.96 |

Table 3 UMi with 2GHz

| User arrival rate | RU | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) |
| --- | --- | --- | --- | --- |
| 2 | 0.24 | 8.60 | 29.2 | 52.63 |
| 3 | 0.52 | 2.82 | 14.76 | 51.28 |
| 3.5 | 0.67 | 1.18 | 8.85 | 48.78 |

Table 4 UMi with 3.5GHz

| User arrival rate | RU | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) |
| --- | --- | --- | --- | --- |
| 2 | 0.26 | 7.52 | 27.21 | 52.63 |
| 3 | 0.58 | 1.81 | 11.73 | 50 |
| 3.5 | 0.7 | 1.05 | 7.33 | 44.94 |

- Conclusion

In this contribution, the baseline performance results are provided, and we observe that

Observation1: The performance of UMa with 500m ISD is larger than that in UMi and UMa with 200m ISD.

Observation 2: The performance of UMi with 2GHz is larger than that with 3.5GHz.

- References

[1]RP-141644, "New SID Proposal: Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE", Samsung, Nokia Networks, 3GPP TSG RAN#65, Edinburgh, Scotland, 09th – 12th September 2014.

[2] Chairman’s Notes 3GPP TSG RAN1#78bis, Ljubljana, Slovenia, 6th – 10th October 2014.

[3] R1-144444, "WF on simulation assumption", 3GPP TSG RAN1#78bis, Ljubljana, Slovenia, 6th – 10th October2014.

- Appendix

Table 5 Simulation assumption

| Duplex | FDD |
| --- | --- |
| Traffic model | FTP model 1 |
| Wrapping method | Geographical distance based |
| Channel estimation | Ideal |
| Feedback | PUSCH 3-2 |
| Number of UE receive antennas | 2 |
| RAN | SU-MIMO Rank 1/2 adaption |
| Others | Based on [2][3] |