3GPP TSG RAN WG1 Meeting #80		R1-150683

Athens, Greece, 9th – 13th February 2015

Agenda item:	7.2.4.2.1

Source:	InterDigital

Title:	Phase 1 evaluation results with Rel-12 DL-MIMO

Document for:	Discussion

# 1	Introduction

In this contribution, we provide evaluation results of Rel-12 DL MIMO schemes based on the agreed evaluation assumptions in the 3D channel model.

# 2	Evaluation Results for Rel-12 MIMO schemes

In this section, we evaluate the Rel-12 downlink MIMO performance in 3D-UMa and 3D-UMi channels. The details of the simulation assumptions are listed in Table A in Annex. Tables 1 and 2 show the performance of DL MIMO schemes with dynamic SU and MU-MIMO switching with full buffer traffic model and non-full buffer traffic model, respectively. In addition, no UE transparent elevation beamforming technique (e.g. vertical sectorization) is used.

Table 1: Full-buffer results (bps/Hz)

| Scenario, ISD, CF, downtilt | Cell-avg | Cell-edge |
| --- | --- | --- |
| 3D-UMa, 500m, 2 GHz, 100 deg | 2.45 | 0.09 |
| 3D-UMa, 200m, 2 GHz, 104 deg | 2.33 | 0.08 |
| 3D-UMi, 200m, 2 GHz, 100 deg | 2.40 | 0.09 |
| 3D-UMi, 200m, 3.5 GHz, 100 deg | 2.38 | 0.08 |

Table 2: Non-full buffer results (Mbps)

| Scenario, ISD, CF, downtilt | Low RU (~20%) |  |  | Medium RU (~50%) |  |  | High RU (~70%) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean | 50% | 5% | Mean | 50% | 5% | Mean | 50% | 5% |
| 3D-UMa, 500m, 2 GHz, 100 deg | 31.68 | 28.13 | 9.36 | 17.46 | 14.96 | 2.87 | 13.94 | 10.64 | 2.37 |
| 3D-UMa, 200m, 2 GHz, 104 deg | 32.1 | 29.25 | 10.70 | 18.01 | 15.00 | 3.58 | 14.0 | 9.7 | 2.56 |
| 3D-UMi, 200m, 2 GHz , 100 deg | 30.03 | 26.77 | 8.37 | 18.50 | 15.60 | 3.60 | 15.2 | 10.87 | 2.44 |
| 3D-UMi, 200m, 3.5 GHz, 100 deg | 28.50 | 24.73 | 7.36 | 18.06 | 15.33 | 3.32 | 13.61 | 9.65 | 1.73 |

# 3	Summary

In this contribution, we provided phase 1 evaluation results of Rel-12 DL MIMO scheme using dynamic SU and MU-MIMO switching in 3D-UMa and 3D-UMi.

# References

- RP-141644, “New SID Proposal: Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”, Samsung, Nokia Networks.

- 3GPP TR 36.873, “Study on 3D channel model for LTE,” V2.0.0, Release 12, Mar. 2014.

- R1-144444, “WF on simulation assumptions”, Nokia Networks, Nokia Corporation, CMCC, Samsung

- R1-144437, “WF on Antenna modeling”, Samsung, Nokia Networks, Nokia Corporation, CMCC, Huawei, HiSilicon, Interdigital, LGE, Motorola Mobility, Qualcomm, CHTTL

# Annex

Table A. System Level Simulation Assumptions

| Parameter | Assumption |
| --- | --- |
| Network layout | 7-site 21-cell wraparound |
| Channel model | 3D Urban Macro (3D-UMa) 3D Urban Micro (3D-UMi) [2] |
| eNB antenna configuration | (M,N,P,Q)=(8,4,2,8), 0.5 λ spacing in H, 0.8 λ spacing in V, cross-polarization (X-pol) [3,4] |
| UE antenna configuration | nR=2, cross-polarization, 0o/90o |
| UE attachment | RSRP on CRS port 0 [2] |
| UE distribution | uniformly dropped according to Table 6-1 [2] |
| Electrical Downtilt | θtilt = 100o , 104o |
| Number of UEs per cell | 10 |
| Traffic model | full buffer non-full buffer FTP model 1, packet size 0.5M bytes |
| Scheduler | proportional fair (PF) |
| Transmission scheme | SU/MU-MIMO dynamic switching with SU-MIMO feedback, Non-transparent MU-MIMO |
| Codebook | Rel-10 8Tx codebook |
| Link adaptation | AMC with OLLA, 10% BLER target |
| Receiver | MMSE-IRC, ideal channel estimation, ideal interference modelling |
| Feedback | PUSCH 3-1, CQI and PMI reporting triggered per 5ms |
|  | feedback delay is 5 ms |
| Maximum number of HARQ retransmission | 4 |