3GPP TSG RAN WG1 Meeting #80bis	R1-151648

# Belgrade, Serbia, 20th – 24th April 2015

Agenda item:	7.2.5.2.2

Source: 	Samsung

Title: 	Performance Evaluation of FD-MIMO with DMRS Enhancement

Document for:	Discussion and Decision

- Introduction

DMRS enhancement for high order multi-user spatial multiplexing is one of SIs for FD-MIMO [1]. In the RAN1#80 meeting, following alternatives for the enhanced DMRS structure were discussed and their performance evaluations in FTP traffic model were encouraged [2]:

- Alternative 1: 12 DM-RS REs with OCC = 4 for up to total 4 layers per scrambling sequence

- Alternative 2: 24 DM-RS REs with OCC = 2 for up to total 4 layers per scrambling sequence

- Alternative 3: 24 DM-RS REs with OCC = 4 for up to total 8 layers per scrambling sequence

This contribution presents Samsung’s views on DMRS enhancement and provides evaluation results by considering DMRS overhead and DMRS channel estimation accuracy for above alternatives.

- DMRS enhancement

Multi-user spatial multiplexing is an important requirement of FD-MIMO’s performance enhancement. In order to achieve this, channel estimation performance on DMRS should be sufficiently guaranteed. The current specification provides 2 orthogonal DMRS ports and 2 scrambling sequences for MU-MIMO support. As a result, for multi-user transmissions whose composite rank is above 2, it is not possible to maintain orthogonality between DMRS ports assigned for different UEs. One might argue that since FD-MIMO systems have larger number of TXRU’s compared to legacy LTE/LTE-A systems, better signal separation would be possible and further enhancement on DMRS is not necessary. However, given the fact that DMRS channel estimation performance is the bottleneck for downlink receiver performance, this aspect should be investigated in the course of this SI.

Larger number of TXRUs combined with adequate DMRS enhancement could enhance performance of FD-MIMO well beyond what is possible in conventional LTE systems. In order to do so, the following three alternatives depicted in Figure 1 have been discussed in the last meeting.

- Alternative 1: 12 DM-RS REs with OCC = 4 for up to total 4 layers per scrambling sequence

- Alternative 2: 24 DM-RS REs with OCC = 2 for up to total 4 layers per scrambling sequence

- Alternative 3: 24 DM-RS REs with OCC = 4 for up to total 8 layers per scrambling sequence

For alternative 1, 12 REs are used for DMRS in order to support up to 4 orthogonal ports. As shown in Figure 1(a), ports 7, 8, 11, and 13 are mapped to the blue REs with OCC=4. In terms of resources, alternative 1 utilizes 12 REs. Compared to alternative 2 and 3, alternative 1 is more efficient in terms of resource utilization but requires 3dB less transmission power to work with. Also, alternative 1 might be more susceptible to high Doppler scenarios due to the utilization of OCC length=4 instead of OCC length=2. Note that this might not be an issue if such UEs are configured with a transmission scheme that does not utilize DMRS and is more suitable for high Doppler (e.g. transmit diversity).

On the other hand, for alternative 2, 24 REs are used for DMRS in order to support up to 4 orthogonal ports. As shown in Figure 1(b), ports 7 and 8 are mapped to the blue REs with OCC=2 and ports 9 and 10 are mapped into the red REs with OCC=2. In terms of resources, alternative 2 utilizes 24 REs regardless the number of co-scheduled UEs. Such an approach may not be efficient in terms of resource utilization but will incur less overhead on DCI. A modified version of alternative 2 would be to allow dynamic switching between 12 REs and 24 REs depending on the sum rank of multi-user spatial multiplexing.

Lastly, alternative 3 is combination of alternative 1 and 2 where 24 REs and OCC=4 are used for DMRS in order to support up to 8 orthogonal ports. Specifically, as shown in Figure 1(c), ports 7 and 8 are mapped to the blue REs with OCC=4 and ports 9 and 10 are mapped into the red REs with OCC=4. Compared to alternative 1 and 2, alternative 3 is capable of handling more co-scheduled UEs at the expense of additional DMRS overhead.

| - Alternative 1 | - Alternative 2 | - Alternative 3 |
| --- | --- | --- |

Figure 1. Possible alternatives for supporting more than 4 orthogonal DMRS ports

For high order multi-user spatial multiplexing, both alternatives 1 and 2 provide DMRS orthogonality of up to 4 UEs and alternative 3 provides DMRS orthogonality of up to 8. Note that such alternatives have not been introduced in the current LTE specification due to the fact that with the existing antenna configurations, it is highly unlikely to schedule multi-user spatial multiplexing for more than 2 UEs. But above alternatives are applicable for FD-MIMO which has more TXRU’s to work with and allows more UEs to be co-scheduled.

- Performance evaluation

In order to study DMRS enhancements for multi-user spatial multiplexing, it is important to study the performance of FD-MIMO as a function of the number of TXRUs and different parameters of the enhanced DMRS design which have different number of orthogonal DMRS ports, overhead and channel estimation accuracy. For performance evaluation, the baseline is the DMRS configuration in Rel-12 and Alternatives 1 and 2 in section 2 are compared:

- Baseline: 12 DM-RS REs with OCC = 2 for up to total 2 layers per scrambling sequence

- Alternative 1: 12 DM-RS REs with OCC = 4 for up to total 4 layers per scrambling sequence

- Alternative 2: 24 DM-RS REs with OCC = 2 for up to total 8 layers per scrambling sequence

For Baseline, with two scrambling sequences, total 4 quasi-orthogonal DMRS transmission layers are considered. On the other hands, for Alternatives 1 and 2, without scrambling sequences, total 4 orthogonal DMRS transmission layers are considered. In this evaluation, channel estimation accuracy is modelled by using channel estimation mean square error (MSE). The estimated channel coefficient can be expressed as

,

where  is the ideal channel and  is the channel estimation error term which follows complex normal distribution. Using link level simulations, MSE curve was obtained and used to generate the magnitude of the error term, . For evaluating the performance gain of two alternatives over baseline, we consider the antenna configuration as M=8, N=4, P=2 and Q=16. Note that antenna array structure for each column is cross-polarized (P=2) where M is the number of antenna elements with the same polarization in each column, N is the number of columns and Q is the number of TXRUs. Further detailed evaluation assumptions are provided in Annex.

The average cell throughput and 5% user throughput in 3D-UMi with ISD 200m are given in Table 1.

Table 1. Relative average cell throughput and 5% edge UE throughput for DMRS enhancement over baseline (3D-UMi with ISD 200m)

| Alternative | Baseline |  | Alt.1 |  | Alt.2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Mobility | Avg. cell throughput | 5% UE throughput | Avg. cell throughput | 5% UE throughput | Avg. cell throughput | 5% UE throughput |
| 3km/h | 100% | 100% | 137% | 123% | 122% | 116% |
| 30km/h | 100% | 100% | 138% | 110% | 119% | 160% |

Based on the evaluation results summarized in Table 1, following observations can be made:

Observations:

- Compared to baseline, for UE speed of 3km/h, Alternative 1 provides more performance gain than Alternative 2 in both average cell throughput and 5% user throughput.

- Compared to baseline, for UE speed of 30km/h, Alternative 2 shows better performance gain over Alternative 1 in 5% user throughput. However, Alternative 1 provides more performance gain than Alternative 2 in average cell throughput.

Note that the evaluation provided in this contribution considers only up to 4 co-scheduled layers for an eNB with 16 TXRUs. In that aspect, Alternative 3 which extends the number of orthogonal ports to 8 was not considered. If additional TXRUs and larger number of co-scheduled layers are considered, Alternative 3 might have gains over Alternative 1 and 2.

- Conclusions

This contribution has presented evaluation results for FD-MIMO with DMRS enhancement. The observations in this contribution are summarized as follows:

Observations:

- Compared to baseline, for UE speed of 3km/h, Alternative 1 provides more performance gain than Alternative 2 in both average cell throughput and 5% user throughput.

- Compared to baseline, for UE speed of 30km/h, Alternative 2 shows better performance gain over Alternative 1 in 5% user throughput. However, Alternative 1 provides more performance gain than Alternative 2 in average cell throughput.

References

RP-141644, Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE, Samsung, Nokia Networks.

Chairman’s note 3GPP RAN1 WG #80

Annex (System simulation parameters)

| Parameters | Value |
| --- | --- |
| Homogeneous scenarios | 3D-UMi ISD 200m |
| Polarized antenna modeling | Model -2 from 36.873 |
| Traffic model | Full buffer model |
| Wrapping method | Geographical distance based |
| Handover margin | 3dB |
| Metrics | Mean, 5% UPT |
| System bandwidth | 10MHz (50 PRBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Carrier Frequency | 2GHz |
| Network synchronization | Synchronized |
| Number of UEs per cell | 15 |
| UE Speed | 3km/h, 30km/h |
| UE distribution | according to 36.873 |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Receiver | Non-ideal channel estimation and interference modelling |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions |
| UE Rx configuration | 2 Rx x-polar (+90/0) |
| Feedback | PUSCH 3-2 for non-reciprocity operation |
|  | CQI, PMI and RI reporting triggered per 5ms |
|  | Feedback delay is 5 ms |
|  | Kronecker product based 2D codebook with Rel-10 8Tx codebook in horizontal dimension and 2Tx codebook in vertical |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 or 24 REs per PRB depending on the alternative enhancement scheme |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |
| CSI-RS, CRS | CSI-RS, CRS: CSI-RS 1-1 mapping to TXRU, only CRS port 0 is modeled for UE attachment, CRS port 0 is associated with the first column with +45 degree pol, CRS port 0 to TXRU mapping is ideal and given by [1, 0, 0, 0, 0, 0, 0, 0] |
| Downtilt | 3D-UMi ISD 200m and θetilt = [100] degree for 3D-UMa ISD 200m |
| CSI-RS/SRS periodicity | 5 msec |