3GPP TSG RAN WG1 Meeting #80bis	                                                               R1- 152158

Belgrade, Serbia, 20th - 24th April 2015

# Source: 	ZTE

Title:	Discussion on Aperiodic and Beamformed CSI-RS schemes

Agenda Item:	7. 2.5.2.1

Document for:	Discussion and Decision

- Introduction

In RAN#65, a new MIMO SID [1] is approved. Elevation Beamforming (E-BF) and Full Dimension (FD) MIMO are two promising technologies for meeting the future spectrum efficiency demands.  In [2], we discuss potential enhancements targeting 2D antenna array for E-BF/FD-MIMO. In [5], tremendous gain is observed from 2D antenna array system with up to 64TXRUs using channel covariance feedback scheme.  However, overhead on CSI feedback and reference signal for channel measurement are issues to the system design.  Among the potential enhancements discussed in [2], aperiodic CSI-RS and precoded/beamformed RS are the candidates to reduce CSI-RS overhead. Similar proposals on precoded CSI-RS[3][4] have been made in RAN1#79.  In this contribution we further discuss about the aperiodic CSI-RS and beamformed CS-RS schemes.

- Potential Enhancements on CSI-RS

2.1 Beamformed CSI-RS

CSI-RS can be virtualized with elevation beams.  In [3], DFT beams are used for beamformed CSI-RS.  Here we evaluate the performance of 4 beams and 8 beams and compared to the baseline case of 8TXRU with fixed tilting angle.  Full buffer and FTP simulation results with Rel-10 8Tx codebook based feedback are shown in following tables.   Simulation assumptions are list in appendix.

Table 1. Performance of beamformed CSI-RS with FTP 0.5MBytes, λ=4.5, offered load=18Mbps

| Scenario/ Offered Load | Antenna Configurations (M,N,K) | Resource Utilization | Mean UPT (Mbps) | 5% UPT (Mbps) | 50% UPT (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 3D-UMi with Offered Load=18Mbps (λ=4.5) | Case 1 - 8TXRU (8,4,8) | 0.726 | 22.02 | 2.75 | 17.17 |
|  | Case 2 - 64TXRU with 4 DFT Beams (8,4,1) | 0.611 | 26.76(21.53%) | 4.56(65.82%) | 22.75 (32.50%) |
|  | Case 3 - 64TXRU with 8 DFT Beams (8,4,1) | 0.602 | 27.45(24.66%) | 4.58(66.55%) | 23.98(39.66%) |

Table 2. Performance of beamformed CSI-RS with full buffer

| Antenna array type (M,N,K) | 3D-UMi 200m ISD |  |
| --- | --- | --- |
|  | Average cell S.E.(bps/Hz) | 5% cell edge S.E. (bps/Hz) |
| Case 1 - 8TXRU (8,4,8) | 3.31 | 0.0685 |
| Case 2 - 64TXRU with 4 Beams (8,4,1) | 3.44(3.9%) | 0.0781(14%) |
| Case 3 - 64TXRU with 8 Beams (8,4,1) | 3.62(9.4%) | 0.0877(28%) |

It is observed that significant gain can be obtained in 3D-UMi scenario.  8 beams gives larger gain especially in full buffer cases.  Note that this result does not take into account the CSI-RS overhead.   Since different UEs need different beamformed CSI-RS, the overhead increases significantly with number of beams.   e.g. if the existing UEs in the cell need all 8 beams, the overhead of 8 beams would be 64REs in the CSI-RS transmission subframe.    If periodic CSI-RS with 5ms periodicity is configured for each beam, it consumes more than 10% overhead of the total resources.   Therefore, reducing CSI-RS overhead is important.

2.2  Aperiodic CSI-RS

Although UE specific CSI-RS configuration signaling is supported in LTE-A specification, CSI-RS resource is often considered to be cell/TP-specific.  For shared CSI-RS, the power, frequency density/location, port number and period should satisfy the maximum requirement of a large number of UEs which share the CSI-RS resource. Since CSI-RS is semi-statically configured and periodically transmitted, it may potentially cause resource waste if the current CSI-RS resource is used in UE or UE-group specific manner.  More importantly, shared CSI-RS means the same port virtualization method should be adopted for each UE, it strongly limits the CSI measurement flexibility and hence eNodeB cannot support adaptive transmission very well.  For the proposal like beamformed CSI-RS, it is hard to share beamformed CSI-RS for many UEs especially for the case of large number of beams.

Aperiodic CSI-RS is a candidate for overhead reduction which should be studied in Rel-13 in order to achieve flexible UE or UE group specific virtualization.  Since it is generated only when needed, overhead can be reduced.  The time/Frequency density/location, port number, transmission power of aperiodic CSI-RS can be configured more flexibly than current periodic CSI-RS.  As shown in figure 1, eNodeB can allocate RS resource in terms of port, RB and TTI which can be different for different UEs.     This means the density of CSI-RS seen by different UE can be different.  Different aperiodic beamformed CSI-RS can be triggered for different UE.  Therefore, the eNodeB can configure proper CSI-RS for each UE according to its requirements e.g. CSI accuracy, overhead and virtualization.

Figure 1 Configuration of aperiodic CSI-RS in different domains

Number of ports can be flexibly configured for aperiodic CSI-RS.  Therefore, virtualization mapping to the CSI-RS ports can also be done more dynamically and more specifically for one UE or a group of UEs.  Multiple CSI processes framework can be used so that the UE can measure multiple aperiodic CSI-RS resources and legacy periodic CSI-RS resources at the same time.

In addition, precoded RS is an important method for reducing CSI-RS port number and feedback dimension as we point out in [2]. Separate elevation/azimuth CSI-RS/feedback can also be easily supported if precoded CSI-RS is introduced.  In addition we can utilize precoded RS for higher CQI accuracy especially for TDD system.

Aperiodic CSI-RS/Precoded CSI-RS can work well together with current CSI-RS as well:

- Aperiodic CSI-RS can be used to extend the current 8-port CSI-RS to more than 8 ports.

- The current periodic CSI-RS can be used to decide how precoding is done on aperiodic precoded CSI-RS.

- Conclusion

In this contribution,  scheme using beamformed CSI-RS is discussed and evaluated.  Potential gain is observed but CSI-RS overhead needs to be reduced in order to exploit the gain.  Aperiodic CSI-RS is an effective method to reduce the CSI-RS overhead which should be considered as one of the CSI-RS enhancements in Rel-13.

- References

[1] RP-141644, “New SID Proposal: Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”, Samsung, Nokia Networks,RAN#65, Edinburgh, Scotland, 9 - 12 September 2014

[2] R1-144813, "Initial consideration on enhancements targeting 2D antenna array for EB/FD-MIMO", 3GPP TSG RAN WG1 Meeting #79 ,ZTE

[3] R1-144631, "Evaluation of FD MIMO with beamformed CSI-RS with 16-64 TXRU", 3GPP TSG RAN WG1 Meeting #79, CATT

[4] R1-144912, "Discussion on RS design enhancements ",  3GPP TSG RAN WG1 Meeting #79, LG Electronics

[5] R1-144810, “Initial Evaluation Results of  2D Array System with up to 64 TXRUs”, 3GPP TSG RAN WG1 Meeting #79 ,ZTE

- Appendix A

Table A.1 Simulation parameters for Macro cell Scenario

| Parameters | Assumptions |
| --- | --- |
| Cellular Layout | Hexagonal grid, 7 sites, 3 Macro cells per site, geographical based wrap-around |
| Number of UEs dropped within each macro geographical area | 30 |
| Channel Model | 3D UMi ISD 200 |
| Operating bandwidth (BW) | 10 MHz |
| Tx Power | 3D UMI ISD 200: 41 dbm |
| UE Speed | 3km/h |
| Antenna configuration | Transmitter: Config0~Config6 [see table 1] Receiver: 2Rx cross-polarized antenna at UE |
| Downtilt | θetilt = 100 deg |
| Antenna element spacing | (dV,dH)=( 0.8λ, 0.5λ,) |
| Element Number of each port | [see table 1] |
| CQI/PMI reporting interval and frequency granularity | 5ms for CSI, 6RB |
| Feedback scheme | Rel-12 enhanced CSI feedback, PUSCH mode 3-2, ideal channel covariance R |
| Delay for scheduling and AMC | 6ms |
| Scheduler | Proportional Fair |
| Receiver | MMSE-IRC With non-ideal interference covariance matrix estimation by using complex Wishart distribution with 12 degrees of freedom (Model in TR36.829 with DMRS based sample covariance matrix) |
| HARQ Scheme | Chase Combining |
| Maximum number of retransmissions | 4 |
| Traffic model | Full buffer model/FTP 1 with 0.5MByte file size |
| Feedback Assumption | Non-ideal modeling of channel estimation error modeling is used, based on DMRS for data demodulation, based on IMR for interference measurement |
| Transmission scheme | SU/MU dynamic switching |
| Wrapping method | Geographical distance based |