3GPP TSG RAN WG1 #81	R1-152805

May 25 – 29, 2015

Fukuoka, Japan

Agenda item:	6.2.7.2

Source: 	Qualcomm Incorporated

Title: 	Evaluation Methodology

Document for:	Discussion and Decision

# 1	Introduction

In [1], we propose to include UMa channel model for performance evaluation of NOMA. In this document, we propose the evaluation methodology for NOMA.

# 2	Evaluation Methodology

When submitting the evaluated performance of the system with NOMA, the following algorithms will need to be reported (include description), though not covered by the evaluation methodology.

- CQI reporting algorithm – What is reported, and how frequent

- eNB scheduling algorithm – Who to pair, which MCS is used

- Receiver architecture – interference cancellation base

Proposal:

- Also report CQI design, eNB scheduling design, and receiver architecture when reporting NOMA system evaluation results

## 2.1	Baseline for the Study

As a baseline for the study, for a fair comparison, we would like to use a transmission mode with both SU-MIMO and MU-MIMO support as reference. We propose to use MU-MIMO with orthogonal DM-RS serving up to 4 layers at a time. We note that for 4x2 and 4x4 case, it was already agreed on to use SU/MU-MIMO for baseline. We propose to use SU/MU-MIMO for baseline for 2x2 as well.

Proposal:

- Use SU/MU-MIMO with existing TM9 CSI feedback as baseline for 2x2. We also propose to use MU-MIMO with orthogonal DM-RS serving up to 4 layers at a time for the 4Tx cases

We note that proposal 2 assumes a standards change in the baseline that is not part of the SI objectives. Also note that the assumed standard change is rather trivial, especially when compared with NOMA. So NOMA should offer significant improvement over much simpler alternatives as a condition for its adoption.

## 2.2	Link to System Mapping

In the link to system mapping, we need to find a mapping from given channel, operation mode (spatial direction of serving signal and interference signal, interference cancellation used or not, etc), and MCS selected for either stream, to the probability of decoding failure. For single UE operation, this mapping is generated by the link level simulation in the form of a short term link curve, where the channel is converted to the effective SINR and a BLER vs effective SINR curve is generated for each MCS. In this section, we will generalized this idea to NOMA case.

We will use the following signal model

## y=HP0x0+HP1x1+n

Where H is the channel. x0 is the target signal, and P0 is the corresponding precoding matrix used. x1 is the interference signal, and P1 is the corresponding precoding matrix used. n is the out-of-cell interference seen. This model covers both NOMA, where P0=P1, and MU-MIMO, where P0⊥P1. The power split between x0 and x1 is captured in Ex0 and Ex1.

The link to system mapping depends on the receiver. We will consider the following

- Enhancement layer UE with MLM receiver

- Enhancement layer UE with symbol level interference cancellation receiver

- Enhancement layer UE with decoding based interference cancellation receiver

- Base layer UE

### 2.2.1	Enhancement Layer UE with Joint Demodulation Receiver

Receiver operation can be described as follows:

- For the target spatial layer, form MMSE for the combined signal x=x0+x1

- Run joint demodulation to compute LLR for bits in x0 (bits in x1 can be computed as well, but will be disgarded)

To model the above, we will need the following link curve:

- Given MCS of x0 (constellation of x0 and code rate) and combined constellation of x, i.e., the constellation of x1 and power split

- The short term MCS (of x0) vs effective SINR (of x) curve

Note the code rate of x1 does not matter, only modulation order/constellation matters.

### 2.2.2	Enhancement Layer UE with Symbol Level Interference Cancellation Receiver

Receiver operation can be described as follows:

- For the target spatial layer, form MMSE for the combined signal x=x0+x1

- First perform LLR computation for bits in x1, treating x0 as noise

- Cancel x1

- Perform LLR computation for bits in x0 and decode

To model the above, we will need the following link curve:

- Given modulation order of of x1 and SINR of x1, find a cancellation efficiency for x1 with SLIC

- Assume x1 is cancelled with some residual, form MMSE again and compute effective SINR for x0

Note this link curve is just a normal single user link curve, with effective SINR computation captures the cancellation efficiency

### 2.2.3	Enhancement Layer UE with Code Word Level Interference Cancellation Receiver

Receiver operation can be described as follows:

- For the target spatial layer, form MMSE for the combined signal x=x0+x1

- Compute LLR for bits in x1, treating x0 as interference

- Decode x1 and cancel, and there will be cancellation residual error

- Form MMSE for x0 again, with x1 cancelled

- Compute LLR for bits in x0 and decode

To model the above, we will need two link curves

The link curve needed for the first decoding

- Given MCS of x1 (constellation of x1 and code rate)

- The short term MCS (of x1) vs effective SINR (of x1) curve

Note this is the legacy link curve, just treating x0 as another interference

The link curve needed for the 2nd decoding will depend on the result of the first decoding

- If the first decoding failed, can directly declare the 2nd decoding will fail

- If the first decoding passed, need to model a cancellation efficiency of the first decoding output, which is mainly a function of the channel estimation quality

- After cancellation, the second link curves is also a traditional type link curve, with MCS of x0, and effective SINR of x0, except that the new effective SINR considers the cancellation error

### 2.2.4	Base Layer UE

Receiver operation can be described as follows:

- For the target spatial layer, form MMSE for the combined signal x=x0+x1

- Compute LLR for x0, treating x1 as interference and decode

Link curve needed for the decoding

- Given MCS of x0

- Use the short term MCS (of x0) vs effective SINR (of x0) curve

Note this is the legacy link curve, just treating x1 as another interference

Proposal:

- Adopt the described link to system mapping methodology for NAIC system evaluations.

## 2.3	Metrics to Log

As the output of the system simulation, the following needs to be logged for comparison:

- UE perceived throughput distribution at least 5%, median, and 95% points of the cdf.

- Histogram of different NOMA operations modes.

Proposal:

- For system simulation output, log UE perceived throughput and NOMA operation modes histogram.

# 3			Conclusions

This document discusses the evaluation methodology for NOMA study. We propose the framework of link to system mapping for various receiver structures. We also proposed the logging required for comparison.

Proposal:

- Also report CQI design, eNB scheduling design, and receiver architecture when reporting NOMA system evaluation results

- Use SU/MU-MIMO with existing TM9 CSI feedback as baseline for 2x2. We also propose to use MU-MIMO with orthogonal DM-RS serving up to 4 layers at a time for the 4Tx cases.

- For system simulation output, log UE perceived throughput and NOMA operation modes histogram.

- Adopt the described link to system mapping methodology for NOMA system evaluations.

# 4			References

[1] R1-151423, “Target Deployment Scenario” Qualcomm