3GPP TSG RAN WG1 #81	R1-152806

May 25 – 29, 2015

Fukuoka, Japan

Agenda item:	6.2.7.3

Source: 	Qualcomm Incorporated

Title: 	Multiuser superposition schemes

Document for:	Discussion and Decision

# 1	Introduction

In the NOMA study item, the scope has been decided to consider the case that the UEs paired will share the same precoding matrix.

In this document, we propose a signal constellation and mapping design for NOMA under the above restriction.

# 2	Designs

## 2.1	NOMA Constellation Design

Under the condition that the same precoding matrix is used for both UEs, the signal can be written as follows:

yi=HiPα0x0+α1x1+ni

Where i=0,1 is the UE index, αi is the power split for UE i and α0+α1=1, xi is the signal targeting UE i, Hi is the channel to UE i, P is the common precoding matrix used, and ni is the noise seen at UE i. Without loss of generality, we can assume UE 0 is weaker than UE 1. For constellation design, let’s assume both x0 and x1 are scalars.

In this document, we consider the following problems

- How to select the power split α0,1-α0,

- How to pick the constellation for x0 and x1

The above two choices will decide the shape of combined constellation of x=α0x0+1-α0x1.

Figure 1. Normalized Capacity Region of a SIMO System As a Function of SNR Pair

In Figure 1, we shown the normalized capacity region for a SIMO with different SNR pairs. The UE0 SNR is fixed to be 0dB, and UE1 SNR varies. The capacity region is normalized in both x and y axis by the single user rate. Thus the maximum normalized rate is 1. As we can see, the capacity region is convex and becomes larger when the SNR different is larger. In the figure, we also show the rate pair locations correspond to α0=0.7, 0.8 or 0.9 for each SNR pair.

In general, any power split α0,1-α0 can work, and xi, i=0,1, can use any uniform constellation such as QPSK, 16QAM, 64QAM, and 256QAM, that the SNR that layer can support. We can even apply a rotation to the EL constellation relative to the base layer constellation.

As can be seen in Figure 1, different α0 translate to different rate tradeoff between the two UEs along the capacity line.

(a)                                                                                   (b)

Figure 2. PFS Metric Maximizing α0, (a) SNR0=0dB, (b) SNR0=10dB

However, not all α0 are interesting, when fairness is considered. When we consider a proportionally fair scheduler for the SIMO problem, and assume the UE average rate is the single user rate, the multi-user PFS metric is actually the sum of normalized rate for the two UEs. The multi-user PFS is maximized at the point on the capacity curve in Figure 1 with slope -1. The PFS metric maximizing α0 is as shown in Figure 2. Here are some observations:

- The value is SNR dependent and is typically in the range of 0.6 to 0.95.

- The optimum α0 increases with larger SNR gap

- The optimum α0 increases with when both SNR increases

From Figure 1, we can see the slope of the capacity curve does not change fast, which seems to imply the PFS metric is not very sensitive to α0 choice. Therefore, in Figure 2, we also show the range of α0 when we relax the PFS metric gain over single user scheduling by 10%. More precisely, if the maximum PFS metric is ρ>1, the metric gain over single user scheduling is ρ-1. If we relax the gain by 10%, we will have a PFS metrix gain of ρ-1×0.9, and the corresponding PFS metric is

ρ'=1+ρ-1×0.9

The 90% α0 range is the α0 such that the PFS metric of ρ' is achieved. As we can see, the band is quite wide. This implies that, given SNR pair, it is enough to have one power split, instead of having many choices, which would complicate the design.

If we start from a uniform constellation, and split it into a base layer constellation for weaker UE and an enhancement layer constellation for stronger UE, it turns out the α0 naturally falls into the range with good PFS metric. We don’t need non-uniform combined constellation to provide finer power split control.

- 16QAM=QPSK(BL) x QPSK(EL): α0=0.8

- 64QAM=QPSK(BL) x 16QAM(EL): α0=0.762

- 64QAM=16QAM(BL) x QPSK(EL): α0=0.952

- 256QAM=QPSK(BL) x 64QAM(EL): α0=0.753

- 256QAM=16QAM(BL) x 16QAM(EL): α0=0.941

- 256QAM=64QAM(BL) x QPSK (EL): α0=0.988

Another benefit of using a uniform combined constellation is that we can leverage existing functionality in the transmit and receive chains.

The above discussion is assuming rank 1 transmission. The same arguments apply to rank 2 transmission, either treating the other spatial layer as interference, or decode and cancel the other spatial layer.

Proposal:

- Use uniform 16QAM, 64QAM, and 256QAM as combined constellation for NOMA.

- Consider 6 ways to split the combined constellation into base layer and enhancement layer constellations: 16QAM=QPSK(BL) x QPSK(EL), 64QAM=QPSK(BL) x 16QAM(EL), 64QAM=16QAM(BL) x QPSK(EL), 256QAM=QPSK(BL) x 64QAM(EL), 256QAM=16QAM(BL) x 16QAM(EL), and 256QAM=64QAM(BL) x QPSK (EL)

## 2.2	Constellation Mapping Design

In the previous section, we proposed to use existing uniform 16QAM, 64QAM, and 256QAM as the combined constellation for NOMA. Given the constellation design, we still need to define the bits to constellation mapping as defined in Section 7.1 of [1].

Different bits to constellation mapping will affects the LLR computation in the receiver. One straight-forward design is simply use the mapping for the component constellations and directly combine signal together. For example, if we need to use QPSK (BL) x QPSK (EL), we use the bits to constellation mapping defined for QPSK in Section 7.1.2 of [1] for both BL and EL, and directly combine them. The problem with this approach is the LLR computation block in the receiver needs to be modified at the enhancement layer UE, and the combined constellation is not Gray mapping anymore.

Here we propose to still use the original mapping defined for 16QAM, 64QAM, and 256QAM defined in Section 7.1 of [1], just assigning different bits to different UEs. Still use the 16QAM as example. Each modulation symbol is mapped to 4 bits, bi, bi+1, bi+2, b(i+3). We will assign bi and  b(i+1) to base layer UE, and assign b(i+2) and bi+3 to enhancement layer UE.

Proposal:

- Use the existing bits to modulation symbol mapping for combined constellation for NOMA

# 3			Conclusions

This document proposes a design for constellation of NOMA and bits to modulation symbol mapping.

Proposal:

- Use uniform 16QAM, 64QAM, and 256QAM as combined constellation for NOMA.

- Consider 5 ways to split the combined constellation into base layer and enhancement layer constellations: 16QAM=QPSK(BL) x QPSK(EL), 64QAM=QPSK(BL) x 16QAM(EL), 64QAM=16QAM(BL) x QPSK(EL), 256QAM=QPSK(BL) x 64QAM(EL), and 256QAM=16QAM(BL) x 16QAM(EL).

- Use the existing bits to modulation symbol mapping for combined constellation for NOMA

# 4			References

[1] 3GPP TS 36.211