3GPP TSG RAN WG1 Meeting #81                                                                                 	R1-153643

Fukuoka, Japan, 25th - 29th May 2015

Source: 	Ericsson, Huawei

Title:	Notes from Offline Sessions for LAA

Agenda Item:	6.2.4

Document for:	Discussion and Decision

# Notes from offline session on Wednesday

Potential agreement (revision of R1-153578):

A single idle sensing interval allows the start of a DL transmission burst (which may not start with the DRS) containing DRS without PDSCH within the DMTC

- Total sensing period may be greater than one sensing interval referred to in the above

- FFS: Whether the above can be used for the case where transmission burst does not contain PDSCH but contains DRS, and any other reference signals or channels

- FFS: Whether the above can be used for DL transmission bursts longer than 1+x ms, x << 1 ms

- Further discuss the length of the sensing interval, and the energy detection threshold, or their possible ranges

The ECCA counter used for LBT category 4 for the PDSCH is frozen during DL transmission burst containing DRS without PDSCH

Discussed R1-153578:

- Agree on Alt2 for DRS transmissions:

  - Subject to LBT, allow the DRS to be transmitted in at least one of different time positions within the configured DMTC

    - More than one candidate time position can be configured within the configured DMTC

    - More than one candidate time position can be configured as part of the DRS configuration

Discussed R1-153573:

    - It is beneficial for LAA to support fast carrier switching at the UE. The standards impacts include:

      - Scell configuration and activation/deactivation enhancement:

        - Allows #CCs configured/activated to a UE ≥ #CCs for simultaneous reception/transmission/monitoring for data/control channels

      - L1 indication to the UE to start and possibly end monitoring a carrier, which is selected from the configured/activated carriers by the eNB

      - RRM measurement enhancement:

        - FFS: Allows the UE to derive RSRP and time/frequency synchronization of one carrier from another (co-located intra-band) carrier

Discussed R1-153549

- For LAA PUSCH transmission, when eNB schedules a UE to transmit PUSCH in subframe n, the UE can transmit corresponding PUSCH only in subframe n.

Discussed R1-153548

- Individual LBT procedure per carrier can be used when operating on multiple carriers

  - Note: It is up to eNB implementation whether/how to coordinate the start of the DL transmission burst from carriers in an eNB

Needs further discussion:

- The following additional options are identified for performing multi-carrier DL LBT at an eNB including

  - Sensing on multiple carriers together in a common LBT procedure

    - Other options are not precluded

    - Considering the above in addition to individual LBT procedure per carrier

    - Option 2: A LBT procedure performed over a bandwidth aggregated over a subset of carriers in an eNB

      - Each carrier should belong to at least one subset

      - FFS: CCA threshold to be used for sensing on aggregated carriers

    - Option 3: The aggregated LBT procedure in Option 2 in addition to individual LBT procedure is used to determine the LBT outcome for each carrier

    - Other options are not precluded

- Mechanisms to coordinate the start of the UL transmission burst from carriers in an eNB should be studied further

# Notes from offline session on Thursday

Thursday discussion

Draft TR Update:

Potential agreement of draft TR 36.889 v0.4.3 taking into account the following notes:

Note: In Section 8.3.1.3, page number 60, some corrections on the numbers are needed in one sentence as requested by Broadcom

Note: Revert (Undo) change of “asynchronous UL HARQ” to “CA” in Section 7.2.1.5

Note: Change It is recommended that LAA supports uplink LBT at the UE in Section 7.2.1.6.

Study item conclusions TP:

Discussion about IDC and whether additional text from RAN2 TP is needed to be captured in the conclusions. Can revisit after the final RAN2 TP is received.