3GPP TSG RAN WG1 Meeting #82                                             R1-154057

Beijing, China, 24th–28th August, 2015

Source:	ZTE

Title:	Further Discussion and Analysis on the Simulation Results for DL MUST

Agenda Item:	7.2.7.2

Document for: 	Discussion and Decision

# Introduction

At the RAN#81 meeting, further details of evaluation methodology for downlink multiuser superposition transmission were agreed [1]. In this contribution, we describe the design details of evaluation methodology of the system level simulation for MUST. And very preliminary simulation results are presented. Further discussion is conducted on the simulation results.

# CSI Feedback for MUST

In this section, we describe CSI and UE SINR computation methods at eNB that are relevant to downlink MUST.

In MUST, eNB needs to be informed about the CSIs of all UEs in order to select appropriate UE pairs. CSI includes RI, PMI, CQI or MCS. In the case of 2Tx and 2Rx, the feedback overhead of RI, PMI and CQI are 1bit, 2bit and 4bit, respectively. It is generally believed that two paired UEs have similar precoding matrix, but very CQIs.

In closed-loop system, the SINR of all UE cannot be fed back to eNB directly. UE SINR should be derived by MCS fed back by UE. UE SINR and MCS are mapped based on BLER curves.

# Scheduling Methodology for System Level Simulation of MUST

- Basic processing flow in MUST scheduler

A key aspect of scheduler implementation for MUST is the selection algorithm for pairing users. Basic steps of the scheduler for wideband operation are described below:

- For each cell, any two UEs served by the BS may form a candidate pair of UEs. If the number of UE served by an eNB is n, there would be nn-12 user pairs.

- Among nn-12 user pairs, a few candidates of UE pairs are selected, according to the rule to be described shortly.

- Proper transmission powers are selected for these UE pair candidates to maximize the throughput metric.

- The optimal UE pair is selected from the above UE pair candidates, given proper transmission powers.

- The optimal UE pair recommended by MUST scheduler and the optimal UE recommended by SU-MIMO scheduler are compared based on their metrics. The comparison gives the multiplexing type and the scheduled UEs.

- The selected UEs in pair are scheduled in wideband.

- Rule for UE pairing

In order to reduce computation cost, those UE pairs that are obviously not suitable for MUST would be rejected at the beginning. Let us use UE1 and UE2 to represent two UEs in a pair. If the precoding matrices of UE1 and UE2 are significantly different, UE1 and UE2 would not be multiplexed. Furthermore, if the geometries (i.e., downlink wideband SINR) of UE1 and UE2 are very close, they would not be considered for pairing.

Precoding matrixes of UE1 and UE2 are denoted as W1 and W2. The rank of UE1 and UE2 are labeled as r1 and r2.

W1=v1,1v1,2⋯v1,r1                                               (1)

W2=v2,1v2,2⋯v2,r2                                                  (2)

vi,j=vi,jvi,j*vi,j                                                        (3)

Vector vi,j is the unitization of vector vi,j. i=1,2, j=1,2,⋯,ri.

The SINR of UE1 and UE2 are labeled as s1 and s2, in dBs. s2-s1 is the absolute value of the difference of  s1 and s2.

The multiplexing conditions of UE pair are listed below:

- At least one vector of set v1,1,v1,2,⋯,v1,r1 and one vector of set v2,1,v2,2,⋯,v2,r1 are same.

The number of the same vectors in the two sets is the multiplexing rank of the two UE, which is labeled as rM.

- s2-s1>sT. sT is the preconfigured threshold value, such as 10 dB, 15 dB, etc.

The above condition about SINR tries to ensure that a central UE and an edge UE, rather than two central UEs or two edge UEs, should be multiplexed in MUST. If s1>s2, UE1 is central UE and UE2 is edge UE. If not, UE1 is edge UE and UE2 is central UE.

- Selection of transmission power for UEs

For a UE pair candidate (UE1 and UE2) that meets the above condition, proper transmission power p1 and p2 should be set in order to maximize the throughput metric of the UE pair. The metric of a UE pair is the metric of the performance estimation of UE1 and UE2 multiplexing in power domain. Normally, the greater the metric, the greater the multiplexing performance of the two UEs is. p1 and p2 are subject to the constraint

p1+p2=1                                                              (4)

Suppose UE1 is central UE and UE2 is edge UE, The candidates of p1,p2 are 0.5-∆p,0.5+∆p,0.5-2∆p,0.5+2∆p,⋯,pmin,pmax. Here ∆p,pmin,pmax are subject to the constraints:

0<∆p<0.5                                                              (5)

pmin+pmax=1                                                            (6)

0<pmin<0.5                                                              (7)

0.5<pmax<1                                                           (8)

It is needed to conduct exhaustive search on the candidate set of p1,p2 and extract the best value pair p1,b,p2,b by eNB. Value p1,b,p2,b can maximize the metric of the current UE pair.

- Estimation of scheduling SINR of multiplexed UEs

When two UEs are assumed to multiplex in power domain, the scheduling SINR of the two UEs should be estimated. Suppose UE1 is central UE and UE2 is edge UE among the two UEs, the scheduling SINR s1 of UE1 and the scheduling SINR s2 of UE2 is calculated by these formulas below.

s1=s1+10log10p1,b                                                     (9)

s2=s2s1+110log10p2,b                                         (10)

Here, s1 and s2 are respectively the reported SINR of UE1 and UE2. Value p1,b,p2,b is the proper transmission power of UE1 and UE2. And p1,b,p2,b is subject to the constraint below.

p1,b+p2,b=1                                                 (11)

- Calculation of multi-user PF metric

The metric of a UE-pair not only considers the contribution of UE-pair on the system performance gain but also takes the history of scheduling of UE1 and UE2 into account.  The metric tends to give UEs with less scheduled times more chances to be scheduled. So the metric used by the scheduling algorithm is the metric with proportional fairness (PF). We used the scheduling metric with proportional fair described by [1].

Mk=RktTkt                                                        (12)

Here, Mk is the metric of k-th UE at the time t. Rkt is the “requested rates” of k-th UE at the time t. Tkt is the average throughputs of k-th UE until the time t.

Tkt=1-1∆tTkt-1+1∆tRkt                                   (13)

Here, ∆t is the length of the exponentially weighted window. Tkt-1 is the average throughputs of k-th UE until  time t-1.

# Preliminary System Simulation Results

In this section, preliminary system level simulation on FTP is provided. 2x2 antenna is assumed. Only wideband scheduling is considered. Detailed simulation assumptions are described in Table A1.

UEs use SIC receiver to remove the interference of the edge UE to the central UE. Nevertheless, the interference of the central UE over the edge UE can hardly be eliminated.

Performance comparison of SU-MIMO and MUST on FTP is shown as Table 1. The gain of the percent 5 UE throughput of MUST than SU-MIMO is 14.59%. For FTP, the benefit of MUST to cell edge users of MUST is significant. According to the 95-percentile UE throughput described by Table 1, MUST also results in some gains in the average spectral efficiency.

Table 1 Performance comparison of SU-MIMO and MUST on FTP

|  | RU | Mean UPT (Mbps) | 5% UPT (Mbps) | 50% UPT (Mbps) |
| --- | --- | --- | --- | --- |
| SU-MIMO | 0.8175 | 9.3040 | 1.3750 | 8.1823 |
| MUST | 0.7988 | 9.8742 | 1.5756 | 8.6733 |
| Gain |  | +6.13% | +14.59% | +6.00% |

It should be noted that the simulation here is very preliminary, meaning that there is quite a lot of room for further optimization of scheduler to fit MUST, so that MUST system performance can be further improved.

# Conclusion

This contribution describes the scheduling algorithm of system level simulation for MUST. Some preliminary simulation results for FTP traffic are presented. Compared with SU-MIMO, MUST can significantly improve the system performance. Further optimization of scheduler implementation is expected to boost MUST performance further.

# References

- R1-153442. “Way forward on further details of evaluation methodology for MUST.” MediaTek, CHTTL, III, ITRI, NTT DoCoMo, ZTE.

- David Tse, Pramod Viswanath. Fundamentals of Wireless Communication. Posts and Telecom Press. 2009, 8.

# Annex

Table A1:  System-level simulation assumptions of  DL MUST

| Cellular Layout | Hexagonal grid, 3 sectors per site, 7 macro sites |
| --- | --- |
| Inter-site distance | 500 m |
| Minimum distance between BS and UE | 25m |
| Channel Model | UMa |
| Channel Estimation | ideal |
| Channel Measurement | ideal |
| UE speed | 3 km/h |
| Carrier Frequency | 2.0 GHz |
| System bandwidth | 10 MHz |
| Number of BS antennas | 2 |
| Number of UE antennas | 2 |
| Number of UEs per cell | 10 |
| Maximum number of multiplexed UE | 2 |
| Delay time of scheduling | 6ms |
| Number of subbands | 1 (wideband scheduling) |
| Traffic model | FTP3 |
| Total BS TX power (total per carrier) | 46 dBm |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| Antenna Height | 25 m |
| UE antenna Height | 1.5 m |
| Codebook | LTE Rel. 8 |
| OLLA | Yes |
| Receiver | MMSE with IRC and SIC |
| Packet Size | 0.5 M |
| Resource Utilization | 80% |