3GPP TSG RAN WG1 Meeting #82	R1-154060

Beijing, China, August 24th – 28th, 2015

Source:	Sharp

Title:	Evaluation results of PUCCH payload size for CA beyond 5 carriers

Agenda Item:	7.2.2.1.1

Document for:	Discussion and Decision

Introduction

In the RAN1#81 meeting, the following agreements were made for the CRC of HARQ-ACK transmission [1].

Agreements: - For a UE that transmits more than 22 HARQ-ACK/SR bits in a subframe in a CG on either PUCCH or PUSCH, - X-bit CRC is included in the HARQ-ACK transmission, X >= 8 - Baseline X for evaluation purpose only: X=8 - Rel-8 TBCC and rate matching is used - FFS for a UE that transmits less than 23 HARQ-ACK/SR bits in a subframe in a CG on either PUCCH or PUSCH

Moreover, the following agreements were also made for the maximum HARQ-ACK codebook size.

Agreements: - The maximum HARQ-ACK codebook size in the uplink by one UE in one subframe for DL CA of up to 32 CCs is at least 128 bits - In case of FDD PUCCH cell, the maximum HARQ-ACK codebook size is 64 bits

In this contribution, we share our evaluation results on PUCCH payload size.

Discussions

Figure 1 shows the PUCCH structure candidates for the new PUCCH format for HARQ-ACK transmission for evaluation.

(a) PUCCH format (PF3) like structure

(b) PUSCH like structure

Figure 1. Candidates of new PUCCH structure

Figure 2 shows the required SINR which satisfies ACK miss detection probability < 1% and NACK-ACK error probability < 0.1%. In this evaluation, we assume 8-bit CRC is attached. When eNB fails CRC, the eNB assume all bits as “NACK”. Other detailed evaluation assumptions and bit error rate performances are listed in Table 3 and Figures 4 – 8 in the Annex.

| - EPA 3km/h | - ETU 3km/h |
| --- | --- |

Figure 2. Required SINR performances (8 bits CRC)

Observation 1:

- When we assume the PUCCH payload size is more than around 50 bits, PUSCH structure is better performance.

Figure 3 shows the C.D.F. of PUCCH SINR in case of target SNR = 10 dB and 20 dB. Evaluation assumptions are listed in Table 4 in Annex.

| (a) Case 1: All UEs transmit PUCCH on macro frequency |  |
| --- | --- |
| Macro UEs | SC UEs |
| - Case 2: All UEs transmit PUCCH either on macro carrier or small cell carrier, based on RSRP/RSRQ |  |

Figure 3. C.D.F. of PUCCH SINR

By using the evaluation results in Figure 3, average error probability in SCE2a scenario is shown in Table 1 and 2 according to [2].

Table 1. Average error probability in Case 1

|  |  | ACK miss detection probability (< 0.01) |  |
| --- | --- | --- | --- |
|  |  | Target SNR = 10 dB | Target SNR = 20 dB |
| 22 bits (EPA) | PF3 | 0.0164 | 0.0611 |
|  | PUSCH | 0.0196 | 0.0660 |
| 32 bits (EPA) | PF3 | 0.0267 | 0.0766 |
|  | PUSCH | 0.0307 | 0.0815 |
| 64 bits (EPA) | PF3 | 0.0842 | 0.1314 |
|  | PUSCH | 0.0795 | 0.1283 |
| 128 bits (EPA) | PF3 | 0.3038 | 0.2677 |
|  | PUSCH | 0.2470 | 0.2343 |
| 192 bits (EPA) | PF3 | 0.6369 | 0.4715 |
|  | PUSCH | 0.2470 | 0.2343 |

Table 2. Average error probability in Case 2

- Macro UEs

|  |  | ACK miss detection probability (< 0.01) |  |
| --- | --- | --- | --- |
|  |  | Target SNR = 10 dB | Target SNR = 20 dB |
| 22 bits (EPA) | PF3 | 0.00227 | 0.0107 |
|  | PUSCH | 0.00291 | 0.0121 |
| 32 bits (EPA) | PF3 | 0.00431 | 0.0150 |
|  | PUSCH | 0.00524 | 0.0165 |
| 64 bits (EPA) | PF3 | 0.0199 | 0.0337 |
|  | PUSCH | 0.0181 | 0.0323 |
| 128 bits (EPA) | PF3 | 0.1240 | 0.1020 |
|  | PUSCH | 0.0827 | 0.0810 |
| 192 bits (EPA) | PF3 | 0.5060 | 0.3105 |
|  | PUSCH | 0.2519 | 0.2343 |

- SC UEs

|  |  | ACK miss detection probability (< 0.01) |  |
| --- | --- | --- | --- |
|  |  | Target SNR = 10 dB | Target SNR = 20 dB |
| 22 bits (EPA) | PF3 | 0.00273 | 0.00219 |
|  | PUSCH | 0.00351 | 0.00276 |
| 32 bits (EPA) | PF3 | 0.00524 | 0.00404 |
|  | PUSCH | 0.00636 | 0.00485 |
| 64 bits (EPA) | PF3 | 0.0239 | 0.0173 |
|  | PUSCH | 0.0219 | 0.0159 |
| 128 bits (EPA) | PF3 | 0.1349 | 0.0934 |
|  | PUSCH | 0.0985 | 0.0689 |
| 192 bits (EPA) | PF3 | 0.4873 | 0.3244 |
|  | PUSCH | 0.2581 | 0.1752 |

In the current specification, 22 bits payload can be supported by PUCCH format 3, so the reference performance is set to average error probability 0.07 (i.e., target average error in system-level performance is 7%) which is error rate of 22 bits in PF3-like structure. In this case, it is preferable that the maximum PUCCH payload size is 64 bits when the PUCCH format 3 like structure is applied or 128 bits when the PUSCH like structure is applied.

Moreover, we can see that many cases cannot satisfy 1% ACK miss detection probability. Therefore, further investigation is needed, e.g. multiple-PRB transmission, TPC enhancements, and so on.

Proposal:

- The following payload size for new PUCCH format should be considered

  - The maximum PUCCH payload size is 64 bits when PUCCH format 3 like structure and 1 PRB are applied

  - The maximum PUCCH payload size is 128 bits when PUSCH like structure and 1 PRB are applied

- Further investigation is needed to improve PUCCH performance

Conclusion

In this contribution, we propose:

Proposal:

- The following payload size for new PUCCH format should be considered

  - The maximum PUCCH payload size is 64 bits when PUCCH format 3 like structure and 1 PRB are applied

  - The maximum PUCCH payload size is 128 bits when PUSCH like structure and 1 PRB are applied

- Further investigation is needed to improve PUCCH performance

References

- “Chairman’s notes RAN1 #81,” May 2015.

- Email discussion [80-02]

- Annex

Table 3 and Table 4 show the evaluation assumptions for link and system level evaluations, respectively.

Table 3. Link level simulation assumptions

| Parameters | Assumptions / Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz |
| Channel model | EPA, ETU |
| Velocity | 3 km/h |
| Frequency hopping | At the slot boundary |
| Channel coding | LTE TBCC (Mother coding rate is r = 1/3) |
| Channel estimation | Practical, non-ideal |
| Receiver | Frequency domain MMSE |
| PUCCH payload size | 22, 32, 64, 128, 192 |
| The number of PRBs | 1 |
| The number of CRC bits | 8 |
| Performance metric | ACK miss detection probability & NACK-to-ACK error probability |

Table 4. Link level simulation assumptions

| Parameters | Assumptions / Values |
| --- | --- |
| Scenario | SCE 2a |
| Carrier frequency | 2 GHz for macro cell, 3.5 GHz for small cell |
| The number of macro cells | 57 (Hexagonal grid layout, 19 cell sites with 3 sectored cells |
| The number of small cell clusters | 1 |
| The number of small cells within a cluster | 4 |
| Indoor probability | 80% |
| The number of UEs within macro area | 60 |
| Tx power of eNBs | 46 dBm for macro eNBs, 30 dBm for small cell eNBs |
| PO_PUCCH | Determined so that received SNR is 10 dB or 20 dB |
| F, TxD, g(i), h | 0 dB is assumed for all parameters |
| The number of interferer | 1 UE from each cell |

Figure 4 – 8 show the error probability performances in case of PUCCH payload size.

| (a) EPA 3km/h | (b) ETU 3km/h |
| --- | --- |
| Figure 4. Error probability in 22-bit payload size |  |
| (a) EPA 3km/h | (a) ETU 3km/h |
| Figure 5. Error probability in 32-bit payload size |  |
| (a) EPA 3 km/h | (b) ETU 3 km/h |
| Figure 6. Error probability in 64-bit payload size |  |
| (a) EPA 3km/h | (a) ETU 3km/h |
| Figure 7. Error probability in 128-bit payload size |  |
| (a) EPA 3 km/h | (b) ETU 3 km/h |

Figure 7. Error probability in 192-bit payload size