3GPP TSG RAN WG1 Meeting #82bis	R1-155545

Malmö, Sweden, 5th – 9th Oct 2015

Source:	Ericsson

Title:	TBS design for DL transmission modes with 4/8 CSI-RS antenna ports

Agenda Item:	7.1

Document for:	Discussion and Decision

# Introduction

In RAN1 #82, it was agreed to introduce new maximum TBSs for at least Rel-12 UEs configured with TMs 9/10 since the overhead can be high with 4/8 CSI-RS antenna ports under such configurations.

Agreements:

- Introduce new maximum TBSs assuming fewer number of available REs, e.g., 120 REs  per PRB, for at least Rel-12 UEs configured with TMs 9/10

  - Note that both 64QAM and 256QAM to be considered

- Companies are encouraged to provide the exact maximum TBS values and solutions to RAN1#82bis

In this contribution, we provide the TBS design in accordance to the agreements. Three design options are discussed and one is proposed.

# Rel-12 TBS table design

Up to Rel-12, the maximum TBSs for 64QAM and 256QAM are designed based on lower overhead assumption than other TBS rows, i.e., 136 REs/RB is assumed for last TBS rows to achieve higher peak data rate. As an example, the MCS entries for 256QAM in Rel-12 are shown in Table 1.

Table 1: MCS entries for 256QAM

| MCS | Mod | TBS | Target r×1024 | Spectral Efficiency |
| --- | --- | --- | --- | --- |
| 21 | 8 | 27 | 711 | 5.5547 |
| 22 | 8 | 28 | 754 | 5.8906 |
| 23 | 8 | 29 | 797 | 6.2266 |
| 24 | 8 | 30 | 841 | 6.5703 |
| 25 | 8 | 31 | 885 | 6.9141 |
| 26 | 8 | 32 | 916.5 | 7.1602 |
| 27 | 8 | 33 | 948* | 7.4063* |

* based on 136 available REs.

Following TBS table is agreed to be supported for 256QAM in Rel-12 for single-layer to multi-layer mapping.

As specified in TS 36.213, the Rel-12 UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.931. The higher code rate threshold of 0.931 is designed to enable higher code rates in multi-layer transmissions. TBS design for single-layer transmissions should still target below 0.930 to ensure no new problems for multi-layer transmissions.

# New TBS table design for TM9/10 Option 1

In [1], it is proposed:

- Target coding rate

  - Under the above overhead assumption, the coding rate should be below 1. On the other hand, it is not so important for the highest TBS values to satisfy the coding rate of lower than 0.93 since the highest TBS is chosen targeting mainly the peak data rate rather than the practical usage case. Thus, the TBS values that achieve the coding rate very close to 1 could be considered.

The designs in this section follow the above proposal.

- No new TBS row is designed for 64QAM.

- One new TBS row is designed for 256QAM.

- No new TBS is introduced.

## New TBS table design for 64QAM

It is noted that 711*8/6 = 948. That is, the TBS for TM9/10 with 64QAM and target code rate of 948/1024 are identical to those for 256QAM with target code rate of 711/1024. Therefore, these TBS can be re-used and there is no need to introduce a new TBS row for TM9/10 with 64QAM, i.e. TBS row 27 can be re-used as the last row of 64QAM with the assumption of 120 REs/RB.

Table 2 TBS table design for 64QAM option 1

| MCS | Mod | TBS | Target r×1024 | Spectral Efficiency |
| --- | --- | --- | --- | --- |
| 28A | 6 | 27 | 948 | 7.4063 |

## New TBS table design for 256QAM

With the assumption of 120 REs/RB, a new TBS row is designed for 256QAM without introducing any new TBS as given in Table 4.

Table 3 MCS table design for 256QAM option 1

| MCS | Mod | TBS | Target r×1024 | Spectral Efficiency |
| --- | --- | --- | --- | --- |
| 27A | 8 | 34 | 948 | 7.4063 |

Table 4 TBS row design for 256QAM option 1

| 872 | 1736 | 2664 | 3496 | 4392 | 5352 | 6200 | 6968 | 7992 | 8760 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9528 | 10680 | 11448 | 12216 | 12960 | 14112 | 15264 | 15840 | 16992 | 17568 |
| 18336 | 19080 | 20616 | 21384 | 22152 | 22920 | 23688 | 24496 | 25456 | 26416 |
| 27376 | 28336 | 29296 | 30576 | 30576 | 31704 | 32856 | 34008 | 34008 | 35160 |
| 36696 | 36696 | 37888 | 39232 | 39232 | 40576 | 42368 | 42368 | 43816 | 43816 |
| 45352 | 45352 | 46888 | 46888 | 48936 | 48936 | 51024 | 51024 | 52752 | 52752 |
| 55056 | 55056 | 55056 | 57336 | 57336 | 59256 | 59256 | 59256 | 61664 | 61664 |
| 63776 | 63776 | 63776 | 66592 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 |
| 71112 | 73712 | 73712 | 73712 | 75376 | 76208 | 76208 | 78704 | 78704 | 78704 |
| 81176 | 81176 | 81176 | 84760 | 84760 | 84760 | 84760 | 87936 | 87936 | 87936 |
| 90816 | 90816 | 90816 | 90816 | 93800 | 93800 | 93800 | 93800 | 97896 | 97896 |

## Design performance

As shown in Figure 1, the actual code rates of these TBS are within ±2% of the target code rate aligned with the design principle since Rel-8. Some of code rates can exceed 0.930. Note further that, for the case of NPRB=100, the actual code rates for 64QAM and 256QAM are 0.9289 and 0.9200, respectively. That is, 64QAM and 256QAM peak rates of TM9/10 can be achieved using TBS=66592 and TBS=87936, respectively.

Figure 1 Comparison of actual code rate and code rate inaccuracy in percentage.

# New TBS table design for TM9/10 Option 2

In [1], it is proposed:

- Target coding rate

  - Under the above overhead assumption, the coding rate should be below 1. On the other hand, it is not so important for the highest TBS values to satisfy the coding rate of lower than 0.93 since the highest TBS is chosen targeting mainly the peak data rate rather than the practical usage case. Thus, the TBS values that achieve the coding rate very close to 1 could be considered.

For the 64QAM TBS design, this proposal is followed, which allows reuse of an existing TBS row. For the 256QAM case, a new TBS row is designed with the actual code rates controlled to be below 0.930.

- No new TBS row is designed for 64QAM.

- One new TBS row is designed for 256QAM.

- No new TBS is introduced.

## New TBS table design for 64QAM

Same as option 1, it is noted that the TBS for TM9/10 with 64QAM and target code rate of 948/1024 are identical to those for 256QAM with target code rate of 711/1024. Therefore, these TBS can be re-used and there is no need to introduce a new TBS row for TM9/10 with 64QAM, i.e. TBS row 27 can be re-used as the last row of 64QAM with the assumption of 120 REs/RB.

Table 5 TBS table design for 64QAM option 2

| MCS | Mod | TBS | Target r×1024 | Spectral Efficiency |
| --- | --- | --- | --- | --- |
| 28A | 6 | 27 | 948 | 7.4063 |

## New TBS table design for 256QAM

With the assumption of 120 REs/RB and code rate below 0.930, a new TBS row is designed for 256QAM without introducing any new TBS as given in Table 7.

Table 6 MCS table design for 256QAM option 2

| MCS | Mod | TBS | Target r×1024 | Spectral Efficiency |
| --- | --- | --- | --- | --- |
| 27A | 8 | 34 | 948 | 7.4063 |

Table 7 TBS row design for 256QAM option 1

| 840 | 1736 | 2600 | 3496 | 4392 | 5160 | 5992 | 6968 | 7736 | 8760 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9528 | 10296 | 11448 | 12216 | 12960 | 14112 | 14688 | 15840 | 16416 | 17568 |
| 18336 | 19080 | 19848 | 20616 | 22152 | 22920 | 23688 | 24496 | 25456 | 26416 |
| 27376 | 28336 | 29296 | 29296 | 30576 | 31704 | 32856 | 32856 | 34008 | 35160 |
| 35160 | 36696 | 37888 | 37888 | 39232 | 40576 | 40576 | 42368 | 42368 | 43816 |
| 43816 | 45352 | 46888 | 46888 | 46888 | 48936 | 48936 | 51024 | 51024 | 52752 |
| 52752 | 55056 | 55056 | 55056 | 57336 | 57336 | 59256 | 59256 | 59256 | 61664 |
| 61664 | 63776 | 63776 | 63776 | 66592 | 66592 | 66592 | 68808 | 68808 | 68808 |
| 71112 | 71112 | 73712 | 73712 | 75376 | 76208 | 76208 | 76208 | 78704 | 78704 |
| 78704 | 81176 | 81176 | 81176 | 81176 | 84760 | 84760 | 84760 | 87936 | 87936 |
| 87936 | 87936 | 90816 | 90816 | 90816 | 93800 | 93800 | 93800 | 93800 | 93800 |

## Design performance

As shown in Figure 2, the actual code rates of the new 64QAM TBS are within ±2% of the target code rate. Some of code rates can exceed 0.930. Note further that, for the case of NPRB=100, the actual code rate is 0.9289. That is, peak rate of TM9/10 64QAM can be achieved using TBS=66592.

The code rates of the new 256QAM TBS are bound below 0.930.

Figure 2 Comparison of actual code rate and code rate inaccuracy in percentage.

# New TBS table design for TM9/10 Option 3

In this option, for both of the new 64QAM and 256QAM TBS, actual code rates are controlled to be below 0.930.

- One new TBS row is designed for 64QAM.

- One new TBS row is designed for 256QAM.

- No new TBS is introduced.

## New TBS table design for 64QAM

With the assumption of 120 REs/RB and code rate below 0.930, a new TBS row is designed for 64QAM without introducing any new TBS as given in Table 9.

Table 8 TBS table design for 64QAM option 3

| MCS | Mod | TBS | Target r×1024 | Spectral Efficiency |
| --- | --- | --- | --- | --- |
| 28A | 6 | 34 | 948 | 7.4063 |

Table 9 TBS row design for 64QAM option 3

| 632 | 1288 | 1928 | 2600 | 3240 | 3880 | 4584 | 5160 | 5992 | 6456 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7224 | 7736 | 8504 | 9144 | 9912 | 10296 | 11064 | 11832 | 12576 | 12960 |
| 13536 | 14112 | 15264 | 15840 | 16416 | 16992 | 17568 | 18336 | 19080 | 19848 |
| 20616 | 20616 | 21384 | 22152 | 22920 | 23688 | 24496 | 24496 | 25456 | 26416 |
| 26416 | 27376 | 28336 | 29296 | 29296 | 30576 | 30576 | 31704 | 31704 | 32856 |
| 32856 | 34008 | 35160 | 35160 | 35160 | 36696 | 37888 | 37888 | 39232 | 39232 |
| 40576 | 40576 | 40576 | 42368 | 42368 | 43816 | 43816 | 43816 | 45352 | 45352 |
| 46888 | 46888 | 46888 | 48936 | 48936 | 48936 | 51024 | 51024 | 51024 | 52752 |
| 52752 | 52752 | 55056 | 55056 | 55056 | 55056 | 57336 | 57336 | 59256 | 59256 |
| 59256 | 59256 | 61664 | 61664 | 61664 | 63776 | 63776 | 63776 | 63776 | 66592 |
| 66592 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 | 71112 | 71112 | 71112 |

## New TBS table design for 256QAM

With the assumption of 120 REs/RB and code rate below 0.930, a new TBS row is designed for 256QAM without introducing any new TBS as given in Table 11.

Table 10 MCS table design for 256QAM option 3

| MCS | Mod | TBS | Target r×1024 | Spectral Efficiency |
| --- | --- | --- | --- | --- |
| 27A | 8 | 35 | 948 | 7.4063 |

Table 11 TBS row design for 256QAM option 3

| 840 | 1736 | 2600 | 3496 | 4392 | 5160 | 5992 | 6968 | 7736 | 8760 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9528 | 10296 | 11448 | 12216 | 12960 | 14112 | 14688 | 15840 | 16416 | 17568 |
| 18336 | 19080 | 19848 | 20616 | 22152 | 22920 | 23688 | 24496 | 25456 | 26416 |
| 27376 | 28336 | 29296 | 29296 | 30576 | 31704 | 32856 | 32856 | 34008 | 35160 |
| 35160 | 36696 | 37888 | 37888 | 39232 | 40576 | 40576 | 42368 | 42368 | 43816 |
| 43816 | 45352 | 46888 | 46888 | 46888 | 48936 | 48936 | 51024 | 51024 | 52752 |
| 52752 | 55056 | 55056 | 55056 | 57336 | 57336 | 59256 | 59256 | 59256 | 61664 |
| 61664 | 63776 | 63776 | 63776 | 66592 | 66592 | 66592 | 68808 | 68808 | 68808 |
| 71112 | 71112 | 73712 | 73712 | 75376 | 76208 | 76208 | 76208 | 78704 | 78704 |
| 78704 | 81176 | 81176 | 81176 | 81176 | 84760 | 84760 | 84760 | 87936 | 87936 |
| 87936 | 87936 | 90816 | 90816 | 90816 | 93800 | 93800 | 93800 | 93800 | 93800 |

## Design performance

As shown in Figure 3, the code rates of the new TBS are bound below 0.930.

Figure 3 Comparison of actual code rate and code rate inaccuracy in percentage.

# TBS for multiple layer transmissions

Since no new TBS is introduced with all options, there is no need to design new multi-layer mapping for TM9/10 with 4/8 CSI-RS antenna ports.

# Configuration of new tables

In order to be backwards compatible, legacy UEs should continue using the old tables and new tables shall be configured by eNB only for the UEs with new capability. Hence, new UE capability should be introduced and the new tables should be configured by eNB via RRC signalling.

# Conclusions

In this contribution, we provide three design options of TBS for TM 9/10 with 4/8 CSI-RS antenna ports. Option 1 is the simplest design following same design principle as in Rel-8 to 12 for both 64QAM and 256QAM. With option 1, existing TBS row is used for 64QAM and one new TBS row is designed for 256QAM. The peak data rate can be guaranteed and no new TBS is introduced with all options. Therefore, we propose to adopt option 1.

Proposal:

- Adopt option 1 for TM 9/10 with 4/8 CSI-RS antenna ports.

- Use RRC signaling for table configuration to ensure backwards compatibility.

# References

R1-154538, “TBS handling of DL transmission modes with 4/8 CSI-RS antenna ports”,	 NTT DOCOMO, INC.

R1-154773, “Way forward on the maximum TBS handling”, NTT DOCOMO, Ericsson, Huawei, HiSilicon, Qualcomm