3GPP TSG RAN WG1 Meeting #82bis			R1-155848

Malmö, Sweden, 5th - 9th October, 2015

Agenda Item:	            7.2.3.2.1

Source: 		Intel Corporation

Title:	On the LAA DRS design and transmission

Document for: 	Discussion/Decision

# Introduction

In RAN1#82, it was agreed that:

- Subject to LBT, allow the DRS without PDSCH to be transmitted in at least one of different time positions within the configured DMTC where the DMTC timing is relative to the PCell timing. In this case,

  - Duration of the signals in the DRS (which doesn’t include potential initial signal) is less than 1 ms (a single idle sensing interval is used for the LBT procedure prior to transmission of the DRS)

    - FFS: Duration of the DRS occasions when the duration is less than 1 ms

    - FFS: case where duration of the signal in DRS is 1 ms or longer

  - Allow DRS transmission in subframe other than #0 and #5

  - Position of DRS in a subframe is the same for all candidate subframes in the DMTC

  - Following alternatives can be considered for sequence generation for SSS/RS in subframes with DRS

    - Alt. 1: Generate SSS/RS sequences according to the subframe index of DRS

    - Alt. 2: Generate SSS/RS sequences irrespective of the subframe index of DRS

    - FFS: other alternatives

- It should be noted that the transmission of DRS within the DMTC does not preclude the possibility to allow other DRS transmissions outside of the configured DMTC if this is beneficial.

In this document, we present our view on DRS design and transmission.

# DRS Design Options

In the Rel-12 DRS design, as illustrated in Figure 1, the transmission is not contiguous in time when there is no (E)PDCCH/PDSCH. Moreover, the transmit power over different symbols within the DRS is different. It is desirable for the DRS to be contiguous in time with low energy variation. To achieve these desirable properties, we discuss the following design options for LAA DRS.

- To ensure contiguous transmission, additional CRS within the Rel-12 DRS can be considered, as illustrated in Figure 2. Additional CRS can improve the DRS presence detection and RRM measurement. However, the power can still vary over different symbols within the DRS occasion. Also, it is unknown a priori whether the DRS is within a DL data burst or not, which leads to the lack of knowledge on the existence of addition CRS. Thus, UEs may need to have CRS presence detection in the REs where additional CRS may be transmitted.

- An alternative design to ensure contiguous transmission is the shortened DRS, as shown in Figure 3. The DRS is truncated to length of 4 symbols, corresponding to CRS, SSS, PSS and CRS, respectively. However, the energy variation issue still exits, and reduced CRS may have an impact on the DRS presence detection and RRM measurement.

- To achieve contiguous transmission and low energy variation, signals transparent and unknown to UEs can be transmitted in the empty REs. This design can be applied to both the Rel-12 DRS (Figure 1), DRS with additional CRS (Figure 2), and shortened DRS (Figure 3), while applying to Rel-12 DRS (Figure 1) is less desirable due to its inefficiency.

- Besides the above designs, another option is the repetition of PSS/SSS in frequency domain. Duplicating the PSS/SSS over subcarriers may improve the cell identification, DRS presence detection and time/frequency synchronization, but may lead to large PAPR.

| Figure 1: Illustration of Rel-12 DRS Design | Figure 2: Illustration with additional CRS within Rel-12 DRS | Figure 3: Illustration of shortened DRS design |
| --- | --- | --- |

Due to the Listen-before-talk (LBT) requirement, DRS may not be transmitted at fixed subframes #0 and #5 as assumed by legacy LTE design. Thus, the generation of PSS/SSS/RS may need to be modified to be applicable to subframes other than #0 and #5. As the subframe index can be provided by PCell, the subframe index identification based on SSS is no longer needed in LAA SCell. Thus, it is preferred to use existing PSS/SSS/RS sequence with the PSS/SSS/RS generation irrespective of the subframe index, e.g., SSS sequence in subframe # 0 in Rel-12 can be used for all subframes. One benefit to use identical SSS sequence over all subframes is the reduced complexity at receiver, as it is no longer required to have the hypothesis test to determine which SSS sequence is transmitted.

Proposal 1: Consider following options for DRS design when DRS is transmitted outside of DL data burst.

- Additional CRS within the Rel-12 DRS can be transmitted to improve the DRS presence detection and RRM measurement.

- Shortened DRS consisting of four symbols (CRS, SSS, PSS and CRS, respectively) can be considered as an alternative.

- Signal transparent to UE can be transmitted on the REs that are not used for RS and PSS/SSS.

- Repetition of PSS/SSS in frequency domain may help improve the cell identification, DRS presence detection and time/frequency synchronization, but may lead to large PAPR.

- PSS/SSS/RS generation irrespective of the subframe index of DRS is preferred.

# DRS Transmission

It was agreed that subject to LBT, DRS without PDSCH can be transmitted in at least one of different time positions within the configured DMTC. We provide the following designs for DRS transmission.

- Alt 1: Subject to single idle sensing interval, DRS can be transmitted within one or more altervative contiguous subframes of DTMC. If LBT is not successful in any of the subframes within the DMTC, next DMTC occasion is tried.

Figure 4: Illustration of DRS transmission based on Alt 1.

DRS transmission within DL data burst in addition to the DRS transmission within DMTC can be considered.

- Alt 2. DRS within DL data burst in addition to DRS within DMTC: DRS is embedded in the DL data burst, e.g., the DRS is transmitted at the first normal subframe of each DL data burst, as shown in Figure 5. To detect the start of a DL data burst, the following options can be considered.

  - Detection of initial signal if defined.

  - Detection of CRS at least in the first OFDM symbol with CRS in a first TTI within the DL data burst.

  - Detection of a common/UE-specific DCI transmitted over PDCCH or EPDCCH.

Figure 5: An illustration of DRS transmission based on Alt 2.

We evaluate the performance of DRS transmission designs Alt 1 and Alt 2 in a WiFi-LAA co-existence scenario in terms of DRS transmission probability and inter-DRS transmission interval. DRS transmission probability is the probability that at least one DRS is transmitted within DMTC periodicity, while inter-DRS transmission interval is the duration between two consecutive DRS transmissions. We follow the evaluation methodology defined in TR 36.889, with single channel indoor, DL only, FTP traffic scenario. We consider DMTC periodicity of 40 ms.

| Figure 6: DRS transmission probability for different options of DRS transmission schemes. | Figure 7: CDF-plot of the inter-DRS transmission interval for different options of DRS transmission schemes for high load FTP only traffic. |
| --- | --- |

Figure 6 shows DRS transmission probability for different traffic load conditions, while Figure 7 describes the CDF of inter-DRS transmission interval for different options considered for high load. The analysis for DRS transmission probability and inter-DRS transmission interval includes the DRS transmissions within and outside the DL data burst. Alt 1 with multiple DRS candidate locations in one DMTC periodicity, has higher DRS transmission probability and shorter inter-DRS transmission interval compared to DRS transmission within DMTC with only 1 alternative position (i.e., DRS transmitted at fixed subframes). Alt 2 further improves the DRS transmission probability and reduces the inter-DRS transmission interval compared to that of Alt 1, as in Alt 2 the DRS can always be transmitted when there is a DL data burst.

Proposal 2: DRS embedded in the DL data burst (e.g. within the first normal subframe) in addition to the DRS transmission within DMTC should be considered to improve the DRS transmission probability.

# Conclusion

In this contribution, we presented our views and alternatives for DRS design and transmission.

Proposal 1: Consider following options for DRS design when DRS is transmitted outside of DL data burst.

- Additional CRS within the Rel-12 DRS can be transmitted to improve the DRS presence detection and RRM measurement.

- Shortened DRS consisting of four symbols (CRS, SSS, PSS and CRS, respectively) can be considered as an alternative.

- Signal transparent to UE can be transmitted on the REs that are not used for RS and PSS/SSS.

- Repetition of PSS/SSS in frequency domain may help improve the cell identification, DRS presence detection and time/frequency synchronization, but may lead to large PAPR.

- PSS/SSS/RS generation irrespective of the subframe index of DRS is preferred.

Proposal 2: DRS embedded in the DL data burst (e.g. within the first normal subframe) in addition to the DRS transmission within DMTC should be considered to improve the DRS transmission probability.

# References

- 3GPP TR 36.889, “Study on Licensed-Assisted Access to Unlicensed Spectrum”

- R1-15407, “On the dynamic power sharing among multiple LAA SCells for DL-only transmission”, Intel Corporation.