3GPP TSG RAN WG1 Meeting #82bis	R1-155572

Malmö, Sweden, October 5th – 9th, 2015

Source:	Sharp

Title:	System level simulation results of MUST

Agenda Item:	7.2.7.1

Document for:	Discussion and Decision

- Introduction

At the RAN1#82 meeting, the following agreement was obtained for MUST schemes [1].

Agreement: - Multiuser superposition transmission schemes can be categorized as follows - MUST Category 1: Superposition transmission with adaptive power ratio on component constellations and non-Gray-mapped composite constellation - R1-153044 (MediaTek), R1-153798 (Huawei), R1-153985 (Intel), R1-154282 (LGE), R1-154535 (NTT DoCoMo), R1-154701 (Xinwei) - MUST Category 2: Superposition transmission with adaptive power ratio on component constellations and Gray-mapped composite constellation - R1-153798 (Huawei), R1-154055 (ZTE), R1-154184 (Samsung), R1-154282 (LGE), R1-154454 (MediaTek), R1-154535 (NTT DoCoMo) - MUST Category 3: Superposition transmission with label-bit assignment on composite constellation and Gray-mapped composite constellation - R1-153798 (Huawei), R1-153891 (Qualcomm), R1-154656 (Nokia)

In this contribution, we compare the above MUST categories by system-level simulation.

- MUST categories

At the RAN1#82 meeting, the feature of each MUST category is summarized as follow in [2].

Table 1: Classification of MUST schemes and their key characteristics

| Categories | Power ratio | Gray mapping | Label-bit assignment |
| --- | --- | --- | --- |
| MUST Category 1 | adaptive, on component constellations | N | on component constellations |
| MUST Category 2 | adaptive, on component constellations | Y | on the composite constellation |
| MUST Category 3 | N/A | Y | on the composite constellation |

MUST category 1 can superpose plurality of signals non-orthogonally. An eNB can determine the power ratio of each UE adaptively. Therefore, the highest MUST gain can be expected since the opportunities of the transmission based on superposition coding increase by adaptive power allocation. However, for example in 2 UEs case, if the near UE is not implemented with an interference canceller, such as CWIC, it causes degradation of performance for the near UE. In MUST category 1, the near UE needs to uses the interference canceller to detect the desired signal.

MUST category 2 is similar to MUST category 1 in terms of use of the superposition coding and the flexibility of power ratio. With ideal IC, the same MUST gain can be obtained. The difference between MUST category 1 and 2 is whether non-Gray-mapped or Gray-mapped composite constellation. Thanks to the Gray mapping, it is expected that MUST category 2 has little degradation from CWIC even if R-ML receiver is assumed.

MUST category 3 uses the uniform constellations, i.e. 16QAM, 64QAM, and 256QAM, as a MUST signal. Therefore, it comes to use the predetermined power ratio. Therefore, MUST gain would seem to be lower than the others. However, the near UE is not required to have the power ratio information signaled or blindly detected.

- System level simulation

As a receiver to cancel the far UE’s signal, CWIC and R-ML can be assumed. Therefore, in this contribution, we evaluate the both receiver type. However, we assume ideal IC instead of CWIC as a receiver modeling. In this case, MUST category 1 and 2 follow the same modeling. Therefore, MUST category 1 and 2 with ideal IC should show the same gain.

We assume the superposed UEs use the same rank and the same precoder for simplification. Moreover, far UEs always use QPSK modulation. Wideband scheduling is assumed. In RAN1 #82 meeting, MUST category 1 and 2 are evaluated with finite various power ratio [3-9]. In this simulation, the power ratio for the near UE for MUST category 1 and 2 is [0.01, 0.02, … , 0.50]. The other assumptions are described in Annex.

Gains of each MUST category are summarized in Table 2. We can see MUST category 1 and 2 with ideal IC can achieve the highest MUST gain. Moreover, the difference between ideal IC and R-ML is marginal.

Although MUST category 3 has an advantage where near UEs don’t require the power ratio information, the gain is lower than the others.

Table 2: Gains of each MUST category

|  | Cell average throughput gain |  | 5%ile UE throughput gain |  |
| --- | --- | --- | --- | --- |
|  | Ideal IC | R-ML | Ideal IC | R-ML |
| MUST category 1 | 10.3 % | 8.8 % | 17.6 % | 7.8 % |
| MUST category 2 | 10.3 % | 10.2 % | 17.6 % | 16.5 % |
| MUST category 3 | 7.4 % | 7.1% | 9.5 % | 7.0 % |

The ratio of the number of multiplexed UEs is in Table 3. If the ideal IC is assumed, the highest MUST opportunity can be obtained in MUST category 1 and 2. If R-ML is assumed, MUST category 2 has a highest MUST opportunity of the three. However, MUST category 3 has lowest opportunity since the most significant limits are imposed in respect to the power ratio set and modulation order.

Table 3: Ratio of 2 UEs multiplexing

|  | MUST category 1 | MUST category 2 | MUST category 3 |
| --- | --- | --- | --- |
| Ideal IC | 61.4 % | 61.4 % | 33.4 % |
| R-ML | 50.9 % | 58.5 % | 34.1 % |

Observation 1:

- MUST category 1 has 10.3% gain in average cell throughput, and 17.6% gain in 5%ile UE throughput

Observation 2:

- MUST category 2 has the same gain with MUST category 1

  - R-ML receiver has almost the same performance as the ideal IC

Observation 3:

- MUST category 3 has 7.4% gain in average cell throughput, and 9.5% gain in 5%ile UE throughput

Observation 4:

- Adaptive power allocation increases the ratio of superposed signals and performance gains

- Conclusion

In this contribution, we showed the system level simulation results, and we made the following observations:

Observation 1:

- MUST category 1 has 10.3% gain in average cell throughput, and 17.6% gain in 5%ile UE throughput

Observation 2:

- MUST category 2 has the same gain with MUST category 1

  - R-ML receiver has almost the same performance as the ideal IC

Observation 3:

- MUST category 3 has 7.4% gain in average cell throughput, and 9.5% gain in 5%ile UE throughput

Observation 4:

- Adaptive power allocation increases the ratio of superposed signals and performance gains

- References

- Chairman’s Notes RAN1 #82, Beijing, China, Aug. 2015.

- R1-154999, Huawei, HiSilicon, MediaTek, NTT DOCOMO, Sony, CHTTL, HTC, CATR, OPPO, ITRI, “TP for classification of MUST schemes,” RAN1#82, Beijing, China, Aug. 2015.

- R1-153987, Intel Corporation, “System-level performance of downlink multi-user superposition transmission schemes,” RAN1#82, Beijing, China, Aug. 2015.

- R1-154543, LG Electronics, “Initial System Level Simulations for MuST,” RAN1#82, Beijing, China, Aug. 2015.

- R1-154544, Ericsson, “Initial System Level Simulations for MuST,” RAN1#82, Beijing, China, Aug. 2015.

- R1-154456, MediaTek Inc., “System Level Evaluation on MUST,” RAN1#82, Beijing, China, Aug. 2015.

- R1-154638, CATR, “Initial evaluation result of downlink Multiuser Superposition Transmission Scheme,” RAN1#82, Beijing, China, Aug. 2015.

- R1-154657, Nokia Networks, “System-level evaluation of MUST,” RAN1#82, Beijing, China, Aug. 2015.

- R1-154749, NTT DOCOMO, “System level evaluation results for downlink multiuser superposition schemes,” RAN1#82, Beijing, China, Aug. 2015.

- Annex

Table 5 shows the evaluation assumptions.

Table 5: system-level simulation assumptions

| Parameters | Assumptions / Values |
| --- | --- |
| Scenario | MUST Scenario 1 |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| Traffic model | Full buffer (10UE/Cell) |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2 Tx, cross-polarized (+45 degree / -45 degree) UE: 2 Rx, cross-polarized (0 degree / 90 degree) |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| minimum distance from macro-cell to UEs | 35 m |
| UE receiver | For inter-cell and inter-spatial-layer interference - MMSE-IRC - Non-ideal covariance matrix estimation modelled by Wishart distribution (degree of freedom: 8) [3] |
| Transmission mode | TM4 (Fixed to rank-1) |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Handover margin | 3 dB |
| Overhead | 3 symbols for DL CCHs |
| Receiver impairment modelling for demodulation | Non-ideal CRS based channel estimation |
| CSI estimation | Ideal CRS based CSI calculation |
| Feedback | CSI reporting mode 1-1 Feedback periodicity: 5ms Feedback delay: 5ms |
| EVM | Tx: 0%, Rx: 0% |