3GPP TSG RAN WG1 Meeting #83	R1-156475

Anaheim, USA, 15-22 November 2015

Agenda Item:	6.2.6.1.3

Source:	Huawei, HiSilicon

Title:	Exception report latency evaluation in in-band operation

Document for:	Discussion and decision

# Introduction

At RAN#69 meeting, the “NB-IoT” WI was approved based on outcome of the GERAN SI on cellular IoT [1]. The objective on exception report latency in study item is stated in TR 45.820 as

For devices supporting such applications a delay requirement of 10 seconds is appropriate for the uplink when measured from the application ‘trigger event’ to the packet being ready for transmission from the base station towards the core network.

The evaluation results following methodologies in [2]  and following conclusion have been captured in TR 45.820:

This section demonstrates that exception report can be delivered to the base station within 10s for all coverage classes with a reliability of at least 99%.

In this contribution, the evaluation results of exception report delivery latency is presented for in-band operation. Downlink with 15 kHz sub-carrier spacing and uplink with FDMA+GMSK and a 2.5 kHz sub-carrier spacing are assumed. The same methodology for standalone case [5] is used in the analysis except that the time to receive downlink channels is prolonged, due to the limited downlink power boosting in in-band operation.

# Exception report procedure

The different durations for transmitting an exception report with 99% confidence of successful delivery are summarized in Table 1. The assumptions are as following:

- The results on synchronization refer to [3], and the results on broadcast channel refer to [4]. The results in [8] are adopted for NB-EPDCCH reading.

- For downlink channels lasting larger than 10ms, the synchronization channel and broadcast channel are also taken into consideration in calculation the receive time.

- In the worst case, the delay between end of PRACH and next NB-EPDCCH, i.e. waiting for NB-EPDCCH, is assumed as 740ms [7].

- For 99% confidence of delivery, the PUSCH configurations with BLER less than 1% in Table 7 are used in evaluation.

Table 1 Exception report activity duration for 99% confidence

| Activity |  | Report with no header compression (105 byte payload) |  |  |
| --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 |
| Tsync (ms) |  | 140 | 220 | 860 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 160 | 640 | 640 |
|  | Time to decode PSI Duration (ms) | 160 | 640 | 2560 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 |
|  | PSI to RACH gap | 40+4 | 40+4 | 320+4 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 740 | 740 | 740 |
|  | NB-EPDCCH | 2 | 25 | 151 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 80 | 680 | 2560 |
|  | Processing time | 3 | 3 | 3 |
| Total time (ms) |  | 1409 | 3072 | 8198 |

Based on above analysis, the following is observed:

Observation 1: For design with 15 kHz subcarrier spacing downlink and FDMA uplink, the exception report can be delivered to the network with 99% confidence within 10 seconds in in-band operation with 6dB PSD boosting, at all target MCLs, including 164 dB.

For comparison, the exception report latency of design with 15 kHz sub-carrier spacing downlink and SC-FDMA uplink is re-evaluated with following assumptions different to: [7]

- Based on results in [9] and our simulation, the time to receive MIB/PSI (primary system information) is 2560 ms with 35dBm transmission power. A 640 gap is added to between synchronization and start reading of MIB/PSI.

- For downlink channels lasting larger than 10ms, the synchronization channel and broadcast channel are also taken into consideration in calculation the receive time.

- The time to wait for NB-EPDCCH in receiving uplink ACK and uplink assignment is calculated as 171 – (T mod 171), where 171 ms is the NB-EPDCCH periodicity considering sync channel and broadcast channel and T is the time required of preceding steps since last NB-EPDCCH transmission [7].

Table 2 Latency Evaluation in in-band operation for SC-FDMA uplink

| Activity | Size (bytes) | 164 dB |
| --- | --- | --- |
| Synch |  | 1110 |
| MIB |  | 3200 |
| PRACH |  | 1440 |
| Wait for EPDCCH |  | 740 |
| DL assignment | 8 | 171 |
| Wait for PDSCH |  | 0 |
| RA msg 2 (RAR) | 10 | 213 |
| Wait for PUSCH |  | 9 |
| RA msg 3 (TLLI + Access cause + BSR) | 11 | 300 |
| Wait for EPDCCH |  | 162 |
| DL assignment | 8 | 171 |
| Wait for PDSCH |  | 0 |
| RA msg 4 (cont. res.) | 8 | 171 |
| Wait for EPDCCH |  | 0 |
| UL grant | 8 | 120 |
| Wait for PUSCH |  | 9 |
| Uplink report (50 bytes) | 100 | 2300 |
| eNB processing |  | 3 |
| Total (ms) |  | 10119 |

Comparing results in Table 1 and Table 2, the following could be observed:

Observation 2: For SC-FDMA uplink, the exception report latency object in GERAN SI cannot be met in in-band operation with 6dB PSD boosting, for 164dB MCL.

Observation 3: FDMA with GMSK uplink design has lower latency than SC-FDMA based uplink design in delivering exception report to the network in in-band operation.
     - This is without taking into account power back-off for SC-FDMA based uplink, see [10] for details.

The reason why FDMA based uplink has shorter latency than SC-FDMA based uplink is that design with FDMA option uses message based RACH procedures while design with SC-FDMA relies on PRACH to get uplink synchronization and thus has a longer procedure.

The latency for SC-FDMA based solution may be even longer if uplink power back-off is applied. For example, in case of 6dB PAPR, power back-off would induce 4 times more uplink transmission time than that without considering power back-off.

# Impacts of MBSFN

The exception report latency is expected to be significantly longer if MBSFN subframes are configured. As agreed in RAN1#82BIS, one of the options to be evaluated is as follows:

4 downlink subframes are available in a frame, at least 1 PRB available for NB-IoT in a subframe

Then the impacts to exception report latency are:

- The time to transmit downlink channels would be extended by 10/4, with current design.

As a result, the exception report latency results for FDMA uplink and SC-FDMA uplink are shown in Table 3 and Table 4, respectively.

Table 3 Exception report activity duration for 99% confidence, for FDMA uplink, with MBSFN

| Activity |  | Report with no header compression (105 byte payload) |  |  |
| --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 |
| Tsync (ms) |  | 350 | 550 | 2150 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 400 | 1600 | 1600 |
|  | Time to decode PSI Duration (ms) | 400 | 1600 | 6400 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 |
|  | PSI to RACH gap | 40+4 | 40+4 | 320+4 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 740 | 740 | 740 |
|  | NB-EPDCCH | 5 | 36 | 373 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 80 | 680 | 2560 |
|  | Processing time | 3 | 3 | 3 |
| Total time (ms) |  | 2102 | 5333 | 14510 |

Table 4 Latency Evaluation in in-band operation for SC-FDMA uplink, with MBSFN

| Activity | Size (bytes) | 164 dB |
| --- | --- | --- |
| Synch |  | 2775 |
| MIB |  | 7040 |
| PRACH |  | 1440 |
| Wait for EPDCCH |  | 740 |
| DL assignment | 8 | 423 |
| Wait for PDSCH |  | 0 |
| RA msg 2 (RAR) | 10 | 531 |
| Wait for PUSCH |  | 9 |
| RA msg 3 (TLLI + Access cause + BSR) | 11 | 300 |
| Wait for EPDCCH |  | 162 |
| DL assignment | 8 | 423 |
| Wait for PDSCH |  | 0 |
| RA msg 4 (cont. res.) | 8 | 423 |
| Wait for EPDCCH |  | 0 |
| UL grant | 8 | 120 |
| Wait for PUSCH |  | 9 |
| Uplink report (50 bytes) | 100 | 2300 |
| eNB processing |  | 3 |
| Total (ms) |  | 16698 |

Based on above analysis, the following can be observed:

Observation 4: If MBSFN subframes are configured, the exception report latency objective in GERAN SI cannot be met for both FDMA uplink and SC-FDMA uplink in in-band operation.

# Impacts of PSD boosting

This section estimates the exception report latency with 3dB PSD boosting, with the following assumption:

- The time to transmit downlink channels would be doubled, with current design.

As a result, the exception report latency results for FDMA uplink and SC-FDMA uplink are shown in Table 5 and Table 6, respectively.

Table 5 Latency Evaluation in in-band operation for SC-FDMA uplink, with 3dB PSD boosting

| Activity |  | Report with no header compression (105 byte payload) |  |  |
| --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 |
| Tsync (ms) |  | 280 | 440 | 1720 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 320 | 640 | 640 |
|  | Time to decode PSI Duration (ms) | 320 | 1280 | 5120 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 |
|  | PSI to RACH gap | 40+4 | 40+4 | 320+4 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 740 | 740 | 740 |
|  | NB-EPDCCH | 4 | 50 | 302 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 80 | 680 | 2560 |
|  | Processing time | 3 | 3 | 3 |
| Total time (ms) |  | 1871 | 3957 | 11769 |

Table 6 Latency Evaluation in in-band operation for SC-FDMA uplink, with 3dB PSD boosting

| Activity | Size (bytes) | 164 dB |
| --- | --- | --- |
| Synch |  | 2220 |
| MIB |  | 5760 |
| PRACH |  | 1440 |
| Wait for EPDCCH |  | 740 |
| DL assignment | 8 | 342 |
| Wait for PDSCH |  | 0 |
| RA msg 2 (RAR) | 10 | 426 |
| Wait for PUSCH |  | 9 |
| RA msg 3 (TLLI + Access cause + BSR) | 11 | 300 |
| Wait for EPDCCH |  | 162 |
| DL assignment | 8 | 342 |
| Wait for PDSCH |  | 0 |
| RA msg 4 (cont. res.) | 8 | 342 |
| Wait for EPDCCH |  | 0 |
| UL grant | 8 | 120 |
| Wait for PUSCH |  | 9 |
| Uplink report (50 bytes) | 100 | 2300 |
| eNB processing |  | 3 |
| Total (ms) |  | 14515 |

Based on above analysis, the following can be observed:

Observation 5: If 3dB PSD boosting is assumed, the exception report latency objective in GERAN SI cannot be met for both FDMA uplink and SC-FDMA uplink in in-band operation.

# Conclusions

This document provides the latency evaluation for transmitting an exception report to the network in standalone operation. Based on the analysis, the following can be observed:

Observation 1: For design with 15 kHz subcarrier spacing downlink and FDMA uplink, the exception report can be delivered to the network with 99% confidence within 10 seconds in in-band operation with 6dB PSD boosting, at all target MCLs, including 164 dB.

Observation 2: For SC-FDMA uplink, the exception report latency object in GERAN SI cannot be met in in-band operation with 6dB PSD boosting, for 164dB MCL.

Observation 3: FDMA with GMSK uplink design has lower latency than SC-FDMA based uplink design in delivering exception report to the network in in-band operation.
     - This is without taking into account power back-off for SC-FDMA based uplink, see [10] for details.

Observation 3: FDMA with GMSK uplink design has lower latency than SC-FDMA based uplink design in delivering exception report to the network in in-band operation.
     - This is without taking into account power back-off for SC-FDMA based uplink, see [10] for details.

Observation 4: If MBSFN subframes are configured, the exception report latency objective in GERAN SI cannot be met for both FDMA uplink and SC-FDMA uplink in in-band operation.

Observation 5: If 3dB PSD boosting is assumed, the exception report latency objective in GERAN SI cannot be met for both FDMA uplink and SC-FDMA uplink in in-band operation.

# References

RP-151621, “New Work Item: Narrow Band IOT (NB-IOT)”, Qualcomm Incorporated, RAN#69, Phoenix, US, September, 2015.

3GPP TR 45.820, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things, Release 13, version 13.0.0.

R1-156465, Evaluations of network synchronization, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Anaheim, Nov., 2015.

R1-156462, NB-IOT - Downlink physical layer concept description, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Anaheim, Nov., 2015.

R1-156467, Exception report latency evaluation in standalone operation, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Anaheim, Nov., 2015.

R1-156009, Narrowband LTE – Synchronization Channel Design and Performance, Ericsson, RAN182BIS, Malmo, Oct., 2015.

R1-156019, NB LTE – Exception report latency evaluation in inband operation, Ericsson, RAN1#82BIS, Malmo, Oct., 2015.

R1-156020, NB-LTE – Battery lifetime evaluation inband, Ericsson, RAN1#82BIS, Malmo, Oct., 2015.

R1-155984, Performance Evaluations for In-band Operation, Nokia Networks, RAN1#82BIS, Malmo, Oct., 2015.

R1-156466, Uplink coverage evaluation, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Annaheim, Nov., 2015.

- PUSCH configurations

The PUSCH configurations in in-band operation are given in Table 7 for BLER less than 1%.

Table 7 PUSCH configurations with less than 1% BLER

| Burst type | PHY burst size | Coupling loss | Symbol rate | modulation | repetition | code rate | Duration (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH | 105 bytes | 144dB | 30kHz | GMSK | 1 | 0.48 | 80 |
|  |  | 154dB | 3.75kHz | GMSK | 1 | 0.45 | 680 |
|  |  | 164dB | 1.875 kHz | GMSK | 1 | 0.24 | 2560 |