3GPP TSG RAN WG1 Meeting #83                                         	     R1-156680

Anaheim, USA, 15th - 22nd November 2015

Agenda item:	6.2.4.2.1

Source:	NEC

Title:               Proposals for CSI-RS with OCC=2

Document for:	Discussion and Decision

- Introduction

In RAN1#82bis meeting, there were many agreements for the new CSI-RS design. However, there also many details are not be determined, e.g. some working assumptions for 12/16 CSI-RS construction with CDM-2 as below [1]

Agreements:

- 12 and 16 Port CSI-RS construction for CDM-2

  - For 16 port construction:

    - Working assumption (N,K) = (8,2), (2,8)

  - For 12 port construction:

    - Working assumption (N,K) = (4,3), (2,6)

- CSI-RS RE mapping details for CDM-2

  - The ports of the aggregated resource correspond to the ports of component resources according to the following:

    - The aggregated port numbers are 15, 16, … 30 (for 16 CSI-RS ports)

    - The aggregated port numbers are 15, 16, … 26 (for 12 CSI-RS ports)

    - FFS: The aggregated port number is given by n=(k-1)*N+p, p = 15,…,14+N,

      - k (= 1,…,K) correspond to the k-th CSI-RS configuration.

- Working assumption CSI-RS RE mapping details for CDM-4

  - Full-port CSI-RS can be mapped in each OFDM symbol used for CSI-RS mapping.

  - CDM RE set construction

    - Alt 1: time domain only (4 OFDM symbols)

    - Alt 2: time and frequency domain (2 subcarriers x 2 OFDM symbols)

    - Down-selection or merging of the two alternatives FFS.

In this contribution, we focused on CSI-RS design with CDM-2 including 12/16 port CSI-RS construction, CSI-RS pattern with hopping.

- Discussion

2.1 12 and 16 Port CSI-RS construction

Since there are two candidates both for 12 and 16-port CSI-RS construction in the above working assumptions, we propose to down-select the choices to reduce UE implementation complexity based on below reasons

- (N,K) = (8,2) and (4,3) can provide enough flexibility to aggregate the new 16 and 12 CSI-RS ports respectively.

- Since ZP CSI-RS configurations are based on 4 ports, N=2 may introduce some REs wasted. In other words, for PDSCH rate matching, eNB may need to configure more ZP CSI-RS REs for legacy UEs if (N,K) = (2,8) and (2,6) are introduced for 16 and 12 port respectively.

- (N,K) = (2,8) and (2,6) introduces more RRC signaling overhead.

Based on above analysis, we propose that

Propose1: Down-select the candidates of 12 and 16 Port CSI-RS construction for CDM-2 as

  - For 16 port construction:

    - (N,K) = (8,2)

  - For 12 port construction:

    - (N,K) = (4,3)

2.2 2PRB based CSI-RS Hopping for Class A CSI-RS

Since the accuracy of channel estimation is very important, especially for Class A CSI-RS which is non-precoded. Therefore, we propose 2PRB based CSI-RS group hopping as described in Figure1. For 16 CSI-RS ports, two CSI-RS groups map on two 8-port CSI-RS resources. CSI-RS port 0 ~ 7 map on the first 8-port CSI-RS resource, and port 8 ~ 15 map on the second 8-port CSI-RS resource for RB#m. But for RB#m+1, port 0 ~ 7 map on the second 8-port CSI-RS resource and port 8 ~ 15 map on the first 8-port CSI-RS resource. By this hopping, the complete channel information can either be achieved in one PRB pair, or in adjacent PRBs with same OFDM symbols. In addition, by the CSI-RS group hopping, full power utilization can be achieved if maximum power boosting threshold of 6dB is relaxed in current or future Release.

Proposal 2: 2PRB based CSI-RS group Hopping should be considered for Class A CSI-RS.

Figure1. 2 PRB based CSI-RS hopping

2.3 Beam hopping for Class B CSI-RS

In the current standard, CSI-RS locations are not changing in both frequency domain and time domain. Once CSI-RSs of two adjacent cells collide, the collision always exists until next reconfiguration, and this may lead to serious interference across CSI-RSs. The interference will be more serious in BF CSI-RS schemes. In this situation, eNB usually configures non-overlapping CSI-RS resources for adjacent cells, and use CSI-RS muting to avoid serious interference. However, more CSI-RS resources are needed in FD-MIMO and it’s difficult to configure completely non-overlapping CSI-RS resources for all adjacent cells. Then, the BF CSI-RS may incur very serious interference once the CSI-RS collision exists.

To relax this issue, beam hopping within configured CSI-RS resources can be considered in order to achieve interference randomization across CSI-RSs of different cells.

As described in the Figure 1 for 3D BF CSI-RS, 4 beams are configured to one UE. In one CSI-RS transmission, beam 0/1/2/3 are located in the REs as shown in the left figure, but in another CSI-RS transmission, the REs location of beam 0/1/2/3 are changed.

Figure2. Beam hopping for 3D BF CSI-RS

Another example is for vertical BF CSI-RS which is more suitable for cell specific CSI-RS. As described in Figure 2, after vertical virtualization, there are 8 CSI-RS ports in one CSI-RS resource corresponding to one vertical beam. Since in the cell specific scenarios, multiple beams are always configured semi-statically. Therefore, beam hopping within configured resources can be implemented to achieve interference randomization, especially when the number of beams is large.

Figure3. Beam hopping for vertical BF CSI-RS

Proposal 3: Beam hopping within one configured CSI Process should be considered for Class B CSI-RS.

- Conclusion

In this contribution, we propose

Propose1: Down-select the candidates of 12 and 16 Port CSI-RS construction for CDM-2 as

  - For 16 port construction:

    - (N,K) = (8,2)

  - For 12 port construction:

    - (N,K) = (4,3)

Proposal 2: 2PRB based CSI-RS group Hopping should be considered for Class A CSI-RS.

Proposal 3: Beam hopping within one configured CSI Process should be considered for Class B CSI-RS.

- Reference

[1] Chairman notes, 3GPP TSG RAN WG1 Meeting #82bis, Malmo, Sweden, 5th - 9th October 2015