3GPP TSG RAN WG1 Meeting #83	R1-156794

# Anaheim, USA, 15th - 22th November 2015

Agenda item:	6.2.4.3.2

Source: 	Samsung

Title: 	Evaluation results on wideband CSI reporting for FD-MIMO

Document for:	Discussion and Decision

Introduction

According to the WID [1], the objective of the RAN1 is to specify enhancements on CSI reporting for non-precoded CSI-RS and beamformed CSI-RS. In order to enhance CSI reporting, design issues such as reporting granularity and periodicity should be discussed with performance evaluation results. This contribution provides evaluation results to discuss design aspects for wideband CSI reporting.

Performance evaluation

Level of quantization for beam direction and co-phasing, periodicity of CSI reporting is essential parameters in designing wideband CSI reporting. In order to decide such parameters, various design criteria such as UE complexity, reporting overhead, and system performance must be considered. Among such criteria, system performance is a key issue in determining these design parameters.

For CSI reporting, we considered modified PUCCH mode 1-1 to report RI/wideband PMIs/CQIs. In the current specification, PUCCH mode 1-1 support two types of codebook subsampling for 8TX and enhanced 4TX codebooks due to the payload limitation on PUCCH. Codebook subsampling design for 2D codebook was not discussed due to the lack of codebook details in the study item phase. In order to determine the performance characteristics of codebook subsampling for a 2D codebook, we evaluated different levels of quantization for beam direction and co-phasing. Reporting instances for RI/PMIs/CQIs are assumed to be identical for evaluation simplicity.

For MU scheduling, we considered greedy PF scheduling where the scheduler chooses the highest sum PF user pair first and adds co-scheduled UEs based on the reported rank-1 PMI of individual UEs. In order to report UE’s preferred precoding, 2D codebook based on Kronecker product (DFT precoding vectors with size 2 and 4 for vertical and horizontal direction, respectively) is considered. For DFT precoding, precoding vectors of size 2 and 4 with quantization parameter Q are considered as follows.

Vertical direction:

Horizontal direction:

Furthermore, two types of quantization on co-phasing are considered to observe its impact on system performance. Considering co-phasing quantization parameter QC, QPSK symbols {1, j, -1, -j} or BPSK symbols {1, -1} are used for QC=4 or QC=2, respectively.

Subsampling for PMI reporting in both dimensions

In order to achieve the benefits from beam steering for vertical dimension, increment of codebook size is a key point related to CSI enhancement for FD-MIMO. Despite of such merits of codebook size increment, payload size for PMI reporting is limited due to UE computational complexity and PUCCH feedback overheads.

In the current specification, for wideband CSI reporting, two types of subsampling are supported in PUCCH mode 1-1 via submode 1 and 2. In submode 1, RI and first PMI are jointly encoded and reported in a single PUCCH reporting instance. Considering the importance of RI and first PMI, payload for the report is limited to 4 or 5 bits. For this reason, the first PMI is subsampled from 4 bits to 3 bits and overlapped beam groups cannot be selected by using submode 1. Compared to the first PMI, second PMI in submode 1 is not subsampled and can report any beam in the selected beam group with quantized co-phasing of QC=4. For submode 2, first PMI, second PMI and CQI are jointly encoded to be reported in a single PUCCH reporting instance. The first PMI is subsampled in a manner identical to submode 1. However, in submode 2, the second PMI is reduced from 4 bits to 1 bit. Using subsampled second PMI in submode 2, UE can only consider first beam in the reported beam group and reduced quantized co-phasing of QC=2.

For subband CSI reporting, beam group is a crucial factor since UE can select multiple beams only in the selected beams by beam group selection during reporting periodicity for first PMI. However, for wideband CSI reporting, the importance of beam group design can be different from that of subband CSI reporting. Since UE reports only one precoding for a reporting instance, UE can select any precoding in the codebook regardless of beam group design. If precoding changes due to channel variation in the time domain is not severe or periodicity of the first PMI reporting is short enough, overlapped beam group design may not be a crucial issue for wideband CSI reporting.

As mentioned above, for wideband CSI reporting, the importance of beam group design can be different from that of subband CSI reporting since UE reports only one precoding for a reporting instance. However, for periodic CSI reporting, UE reports its CSI components (e.g. rank, beam group selection, beam/co-phasing selection and channel quality indication) separately due to the limitation of payload size for PUCCH. Furthermore, in the current LTE specification, periodic CSI reporting is designed to report rank and beam group selection with larger periodicity than beam/co-phasing selection and channel quality indication to reduce feedback overheads and computational efforts of UE. Since the update of rank and beam group selection are done over large periodicity, having a capability to switch among multiple beams within the selected beam group could be beneficial.

Observations:

- Considering properties of wideband CSI reporting, overlapped beam grouping may not be a crucial design issue.

    - On the other hand, support of multiple beam in a beam group and quantization parameters Q and QC are crucial parameters to design wideband PMI reporting.

Proposals:

- Considering codebook subsampling, prioritize removing overlapped beam group in the codebook as well as legacy PUCCH mode 1-1 submode 1.

- Considering larger periodicity of rank and beam group selection, prioritize supporting multiple beams in a beam group.

Although considerable subsampling is already applied in the wideband reporting, increased codebook size and limited payload size for PUCCH may require additional codebook subsampling for FD-MIMO. For the codebook subsampling, following alternatives can be considered:

- Alternative 1-1: Reduce beam quantization parameter Q in DFT precoding vector.

- Alternative 2-2: Reduce quantized co-phasing from QC=4 {1, j, -1, -j} to QC=2 {1, -1}.

Table 1. Relative average cell throughput and 5% edge UE throughput with different subsampling method

| (8,4,2,16) |  |  | Alt1-1 |  |  | Alt1-2 | Without subsampling |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Q=4,QC=4 | Q=8,QC=4 | Q=16,QC=4 | Q=32,QC=2 | Q=32,QC=4 |
| Vertical beam quantization | Q=4 | Avg. Tput. (bps/Hz/cell) | -11% | -7% | -4% | -3% | -4% |
|  |  | 5% Tput. (bps/Hz/UE) | -10% | -3% | 6% | -14% | 5% |
|  | Q=8 | Avg. Tput. (bps/Hz/cell) | -9% | -5% | -1% | 0% | -1% |
|  |  | 5% Tput. (bps/Hz/UE) | -11% | 6% | 8% | -7% | 12% |
|  | Q=16 | Avg. Tput. (bps/Hz/cell) | -8% | -4% | 0% | 0% | Ref. |
|  |  | 5% Tput. (bps/Hz/UE) | -16% | -7% | 6% | 6% | Ref. |

Based on the above evaluation results, following observations and proposals can be made:

Observations:

- Reducing quantization level of co-phasing causes similar performance degradation with reducing quantization level of beam selection.

    - For reduced quantized co-phasing, 1 bit reduction in horizontal or vertical dimension causes 1% performance degradation in cell average throughput.

    - For reduced quantized co-phasing, bitwidth reduction by more than 1 bit can provide performance degradation up to 11% in cell average throughput.

Proposals:

- For codebook subsampling, consider reducing beam quantization factor Q and reducing quantized co-phasing from QC=4 to QC=2 with same priority.

- To reduce performance degradation, minimize codebook subsampling for PMIs.

Performance impact of periodicity for CSI reporting in PMIH and PMIV

Another way to reduce PMI reporting overhead may be separation of CSI reporting dimensions for horizontal and vertical. Compared to UE’s mobility in horizontal dimension, mobility in vertical dimension is relatively low. Considering such property, PMIV can be more robust compared to PMIH and may not require frequent reporting as much as PMIH. In this section, we provide evaluation results on performance impact of periodicity for CSI reporting in PMIH or PMIV. For this simulation, we assumed delayed CSI only in one of both dimensions and the periodicity of the other dimension is fixed to 5ms.

Table 2. Relative average cell throughput and 5% edge UE throughput with different reporting periodicity

| (8,4,2,16) | Periodicity of change for PMIV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5ms | 20ms | 40ms | 60ms | 80ms | 100ms |
| Avg. Tput. (bps/Hz/cell) | Ref. | -1% | -4% | -7% | -9% | -11% |
| 5% Tput. (bps/Hz/UE) | Ref. | -6% | -9% | -9% | -14% | -17% |

| (8,4,2,16) | Periodicity of change for PMIH |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5ms | 20ms | 40ms | 60ms | 80ms | 100ms |
| Avg. Tput. (bps/Hz/cell) | Ref. | -2% | -6% | -11% | -14% | -16% |
| 5% Tput. (bps/Hz/UE) | Ref. | -6% | -3% | -10% | -35% | -30% |

Based on the above evaluation results, following observation and proposal can be made:

Observation:

- With separated precoding reports for the horizontal and vertical direction, it can be observed that the vertical direction is more robust to longer feedback periodicities and delays.

Proposal:

- Provide specification support to allow separate configuration of CSI reporting for horizontal and vertical dimensions.

Conclusions

This contribution provides evaluation results to discuss design aspects for wideband CSI reporting. The observations and proposals in this contribution are summarized as follows:

Observations:

- Considering properties of wideband CSI reporting, overlapped beam grouping may not be a crucial design issue.

    - On the other hand, support of multiple beam in a beam group and quantization parameters Q and QC are crucial parameters to design wideband PMI reporting.

- Reducing quantization level of co-phasing causes similar performance degradation with reducing quantization level of beam selection.

    - For reduced quantized co-phasing, 1 bit reduction in horizontal or vertical dimension causes 1% performance degradation in cell average throughput.

    - For reduced quantized co-phasing, bitwidth reduction by more than 1 bit can provide performance degradation up to 11% in cell average throughput.

- With separated precoding reports for the horizontal and vertical direction, it can be observed that the vertical direction is more robust to longer feedback periodicities and delays.

Proposals:

- Considering codebook subsampling, prioritize removing overlapped beam group in the codebook as well as legacy PUCCH mode 1-1 submode 1.

- Considering properties of wideband CSI reporting, prioritize supporting multiple beams in a beam group.

- For codebook subsampling, consider reducing beam quantization factor Q and reducing quantized co-phasing from QC=4 to QC=2 with same priority.

- To reduce performance degradation, minimize codebook subsampling for PMIs.

- Provide specification support to allow separate configuration of CSI reporting for horizontal and vertical dimensions.

- References

RP-151085, New WID Proposal: Elevation Beamforming/Full-Dimension (FD) MIMO for LTE, Samsung

R1-156795, Evaluation results on subband CSI reporting for FD-MIMO, Samsung

R1-156793, Discussions on PUCCH based class A CSI reporting for FD-MIMO, Samsung

- Appendix (System simulation parameters)

| Parameters | Value |
| --- | --- |
| Scenarios | 3D-UMi ISD 200m |
| Polarized antenna modeling | Model -2 from 36.873 |
| Traffic model | Full buffer model |
| Wrapping method | Geographical distance based |
| Handover margin | 3dB |
| Metrics | Mean, 5% UPT |
| System bandwidth | 10MHz (50 PRBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Carrier Frequency | 2GHz |
| Network synchronization | Synchronized |
| UE Speed | 3km/h |
| UE distribution | according to 36.873 |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel-12 [71-12] assumptions |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions |
| UE Rx configuration | 2 Rx x-polar (+90/0) |
| Feedback | Modified PUCCH 1-1 |
|  | CQI, PMI and RI reporting triggered per 5ms |
|  | Feedback delay is 5 ms |
|  | Kronecker product based 2D codebook with DFT precoding in horizontal and vertical dimension |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |
| CSI-RS, CRS | CSI-RS, CRS: CSI-RS 1-1 mapping to TXRU, only CRS port 0 is modeled for UE attachment, CRS port 0 is associated with the first column with +45 degree pol, CRS port 0 to TXRU mapping is ideal and given by [1, 0, 0, 0, 0, 0, 0, 0] |
| Downtilt | Antenna downtilting angle θetilt = [100] degree for 3D-UMi |
| CSI-RS/SRS periodicity | 5 msec |