3GPP TSG RAN WG1 Meeting #83     										R1-156925

Anaheim USA, November 15 - 22, 2015

Agenda Item:   	6.2.6.2.3

Source:        	Neul, Huawei, Hisilicon

Title:          	Overall comparison of FDMA and SC-FDMA

Document for:  	Discussion and decision

- Introduction

In RAN#69, a new WI called NB-IoT was created [1]. Two uplink NB-IoT modulation schemes should be evaluated: FDMA and SC-FDMA, derived from the technical report TR45.820 [2].

In this contribution, we compare FDMA and SC-FDMA uplink schemes’ performance and implementation complexity.

- Comparison of FDMA and SC-FDMA uplink

  - General

Observation 1: The key advantage of FDMA is that it results in a simpler and more energy efficient UE transmitter implementation. FDMA has lower PAPR and a lower phase modulation bandwidth.

  - PAPR

PAPR of FDMA vs SC-FDMA is discussed in detail in [6] and [7].

Figure 1: TPSK amplitude vs time

Observation 2: For TPSK waveforms, the low PAPR is misleading in terms of ease of implementation. This is because the dips in signal magnitude at symbol boundaries must be tracked accurately by the PA to avoid spectrum splatter. This prevents the PA being operated with high efficiency in saturation.

Observation 3: While FDMA-GMSK PAPR is 0 dB for all signal bandwidths (all sub-carrier bonding factors), SC-FDMA PAPR can be as high as 6.5 dB. The worst case PAPR with FDMA is 3.8 dB when using 8PSK; however, a low cost UE may use just GMSK and can still achieve higher data rates through sub-carrier bonding, without impacting the 0 dB PAPR property.

Observation 4: SC-FDMA PAPR grows with the modulation bandwidth (number of sub-carriers used), whereas FDMA PAPR does not vary with modulation bandwidth, because it simply scales the symbol rate in order to achieve higher data rates.

Observation 5: The SC-FDMA PRACH has a high PAPR, close to 7 dB for some PRACH sequence indices, whereas the FDMA PRACH reuses the PUSCH modulation formats, so it has the same good PAPR properties.

Observation 6: FDMA and TPSK can both achieve low PAPR, but only FDMA+GMSK provides a genuine constant envelope waveform (exactly 0 dB PAPR), allowing the use of a saturated PA without causing spectral re-growth.

Observation 7: TPSK is not a good replacement ‘zero-PAPR’ scheme for GMSK, because it is not truly constant-envelope.

  - Interference with LTE uplink signals

Both FDMA and SC-FDMA sub-carriers are aligned to a 2.5 kHz frequency grid, therefore they cannot be made orthogonal to LTE uplink transmissions in general. The two schemes are similar in this respect.

In addition to this, in the case of SC-FDMA, interference may be created due to signal discontinuities at symbol boundaries creating spectrum splatter. Note that as illustrated in Table 1, the SC-FDMA symbol boundaries cannot be perfectly aligned to LTE uplink symbol boundaries. In comparison, FDMA has no such discontinuities.

| NB-LTE-style IoT UL |  | LTE UL |  | Difference |  |
| --- | --- | --- | --- | --- | --- |
| Boundary at end of symbol number | Sample offset [samples @30.72 MHz] | Boundary at end of symbol number | Sample offset [samples @30.72 MHz] | Samples @30.72 MHz | us |
| 1 | 13248 | 6 | 13168 | 80 | 2.6 |
| 2 | 26400 | 12 | 26336 | 64 | 2.08 |
| 3 | 39552 | 18 | 39504 | 48 | 1.56 |
| 4 | 52704 | 24 | 54672 | 32 | 1.04 |
| 5 | 65856 | 30 | 65840 | 16 | 0.5 |
| 6 | 79008 | 36 | 79008 | 0 | 0 |
| 7 | 92160 | 42 | 92160 | 0 | 0 |

Table 1: Alignment of LTE and SC-FDMA NB-IoT uplink symbol boundaries

  - Suitability for polar modulator transmitter implementation

As discussed in [7], the trajectories of most SC-FDMA modulations have zero crossings that correspond to a high phase modulation bandwidth.

Figure 2 illustrates this, by comparing the trajectories of SC-FDMA BPSK and FDMA-BPSK signals with 20 kHz bandwidth.

Figure 2: Baseband trajectories of SC-FDMA BPSK and FDMA-pi/2-BPSK signals with 20 kHz bandwidth

  - Implementation complexity

Details of FDMA uplink receiver implementation and its complexity are provided in [8].

The FDMA receiver can use single-tap equalization with channel tracking [9], and it can use a polyphase filterbank to efficiently separate the channel into multiple subcarriers, which means that it has a very low Base Station implementation complexity.

Observation 8: Software update of an LTE eNB to support FDMA with GMSK and PSK is easily feasible.

  - Battery life

Details can be found in [11], [12] and [13].

Observation 9: Compared with the SC-FDMA uplink, the battery life with FDMA uplink is about 25% better in extreme coverage case (164dB MCL).

Observation 10: Since SC-FDMA draws about 20% higher peak transmit current compared with FDMA, the expected battery life of SC-FDMA transmission may be degraded by much more than 20% if these non-linear battery technology effects are taken into account.

  - Spectrum efficiency

Details can be found in [14].

Observation 11: FDMA with GMSK modulation has a comparable spectral efficiency with SC-FDMA including single-tone transmission at MCL=164/154dB whist higher spectral efficiency at MCL=144dB when uplink transmit power back-off is applied.

  - Capacity

The FDMA based uplink solution can achieve higher capacity than the SC-FDMA based uplink solution. One of the reasons is the data rate decrease for SC-FDMA when power back-off is taken into account, as shown in Figure 3. Another reason is the high resource occupation for SC-FDMA in the random access process.

Figure 3: Comparison of data rate between FDMA and SC-FDMA

Detail simulation results can be found in [15], [16] and [17].

Observation 12: The FDMA based uplink solution with an introduction of a 2.5kHz sub-channel spacing achieves higher capacity and lower UL report failure probability than the SC-FDMA based uplink solution in standalone, guard-band and in-band operation mode.

- Conclusions

This document compares the FDMA and SC-FDMA uplink schemes proposed for NB-IoT.

Observation 1: The key advantage of FDMA is that it results in a simpler and more energy efficient UE transmitter implementation. FDMA has lower PAPR and a lower phase modulation bandwidth.

Observation 2: For TPSK waveforms, the low PAPR is misleading in terms of ease of implementation. This is because the dips in signal magnitude at symbol boundaries must be tracked accurately by the PA to avoid spectrum splatter. This prevents the PA being operated with high efficiency in saturation.

Observation 3: While FDMA-GMSK PAPR is 0 dB for all signal bandwidths (all sub-carrier bonding factors), SC-FDMA PAPR can be as high as 6.5 dB. The worst case PAPR with FDMA is 3.8 dB when using 8PSK; however, a low cost UE may use just GMSK and can still achieve higher data rates through sub-carrier bonding, without impacting the 0 dB PAPR property.

Observation 4: SC-FDMA PAPR grows with the modulation bandwidth (number of sub-carriers used), whereas FDMA PAPR does not vary with modulation bandwidth, because it simply scales the symbol rate in order to achieve higher data rates.

Observation 5: The SC-FDMA PRACH has a high PAPR, close to 7 dB for some PRACH sequence indices, whereas the FDMA PRACH reuses the PUSCH modulation formats, so it has the same good PAPR properties.

Observation 6: FDMA and TPSK can both achieve low PAPR, but only FDMA+GMSK provides a genuine constant envelope waveform (exactly 0 dB PAPR), allowing the use of a saturated PA without causing spectral re-growth.

Observation 7: TPSK is not a good replacement ‘zero-PAPR’ scheme for GMSK, because it is not truly constant-envelope.

Observation 8: Software update of an LTE eNB to support FDMA with GMSK and PSK is easily feasible.

Observation 9: Compared with the SC-FDMA uplink, the battery life with FDMA uplink is about 25% better in extreme coverage case (164dB MCL).

Observation 10: Since SC-FDMA draws about 20% higher peak transmit current compared with FDMA, the expected battery life of SC-FDMA transmission may be degraded by much more than 20% if these non-linear battery technology effects are taken into account.

Observation 11: FDMA with GMSK modulation has a comparable spectral efficiency with SC-FDMA including single-tone transmission at MCL=164/154dB whist higher spectral efficiency at MCL=144dB when uplink transmit power back-off is applied.

Observation 12: The FDMA based uplink solution with an introduction of a 2.5kHz sub-channel spacing achieves higher capacity and lower UL report failure probability than the SC-FDMA based uplink solution in standalone, guard-band and in-band operation mode.

- References

  - RP-151621, “New Work Item: NarrowBand IoT (NB-IoT)”, Qualcomm Inc, RAN#69, Phoenix, USA, September 14 - 16, 2015

  - 3GPP TR 45.820, “Technical Specification Group GSM/EDGE Radio Access Network; Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things,” Aug. 2015, v.2.1.0.

  - R1-155970, “UE complexity analysis”, Neul Limited, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN #82 bis, Malmo, Sweden, October 5-9, 2015.

  - R1-155965, “Base Station complexity analysis”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN #82 bis, Malmo, Sweden, October 5-9, 2015.

  - R1-156012, “NB-LTE – UL Modulation”, Ericsson, RAN1 #82bis, Malmo, Sweden, October 5-9, 2015.

  - R1-155804, “PAPR comparison of uplink modulation schemes for NB-IoT”LG Electronics, RAN1 #82bis, Malmo, Sweden, October 5-9, 2015.

  - R1-15xxxx, ” Properties of UL modulations proposed for NB-IoT”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015

  - R1-156463, ”Uplink receiver design and complexity analysis for FDMA based approach”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015

  - R1-156478, ”On GMSK receiver performance”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015

  - RP-156010, “NB LTE – General L1 Concept Description”, Ericsson et al., Malmo, Sweden, October 5-9, 2015.

  - R1-156469, “UE Energy consumption evaluation in standalone operation”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015

  - R1-156472, “UE Energy consumption evaluation in guard-band operation”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015

  - R1-156474, “UE Energy consumption evaluation in in-band operation”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015

  - R1-156466, “Uplink coverage evaluation”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015

  - R1-156468, “Uplink Capacity Evaluation in Standalone Operation”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015

  - R1-156471, “Uplink Capacity Evaluation in Guard-band Operation”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015

  - R1-156476, “Uplink Capacity Evaluation in In-band Operation”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. RAN1 #83, Anaheim, USA, November 15-22, 2015