3GPP TSG-RAN WG1#83	R1-157203

Anaheim, USA, 15th - 22nd November 2015

Source:	Ericsson

Title:	Remaining Details of Codebook Subset Restriction

Agenda Item:	6.2.4.1

Document for:	Discussion and Decision

# Introduction

In RAN1#82bis, it was agreed that codebook subset restriction (CSR) is supported for FD-MIMO with Class A CSI reporting where [1]

- A 2D-beam corresponds to (l1,l2) in X1⨂X2

- CSR is configured via RRC signaling

- A subset of 2D-beams l1,l2 are forbidden, i.e. not allowed to be reported according to the CSR configuration

  - A forbidden 2D-beam is not allowed in reporting with any rank

- Rank restriction is also supported

- Codebook Subset Restriction can be also applied to W2

- Number of PMI bits does not vary according to restricted subset

  - Note: Codebook subset restriction targets e.g. performance/capacity, as in Rel-8 to Rel-12

In [2], it was further agreed that

- For W1 CSR, a bitmap of (N1O1N2O2) bits indicates 2D-beams subset restriction (referred to as Beam-Subset-Restriction in the rest of the contribution),

- 8 additional bits bitmap indicates rank restriction

- An RRC parameter for CSR on Class A i2 (i.e, W2) will be introduced.

  - Bitmap of all possible codewords per rank

In this contribution, we provide further details on the specification impact of supporting CSR for FD-MIMO. As the final details of the Class A codebook have only been agreed for rank 1, we make an assumption of the codebook for higher ranks, for the sake of this discussion, based on the contribution [4]. This codebook is also given in the appendix in Section 5.

# Specification Impact for CSR

A 2D beam is identified by the parameters  l1 and l2 and corresponds to two vectors for the first and second dimension respectively.

| , |  |
| --- | --- |
| Since l1=0,…,N1O1-1, l2=0,…,N2O2-1, there are a total N1O1N2O2 such 2D beams |  |

## Mapping between Beam-Subset-Restriction bits and 2D Beams

Let the bitmap for the Beam-Subset-Restriction RRC parameter be formed by the bit sequence  aS-1,aS-2,⋯a3,a2,a1,a0 where a0 is the least significant bit (LSB), aS-1 is the most significant bit (MSB), and S=N1O1N2O2.  The index  of a bit  in the Beam-Subset-Restriction bitmap can be mapped to a 2D beam using the ‘first dimension first’ mapping scheme shown in Figure 1.

Now let (l'1,l'2) identify a forbidden 2D-beam (i.e., a 2D beam that is not allowed to be reported in any rank) corresponding to a DFT vector with index l'1 in the first dimension and a DFT vector with index l'2 in the second dimension.  The indices l'1 and l'2 can be referred to as forbidden beam indices.  If a bit in the Beam-Subset-Restriction bitmap indicates a forbidden 2D beam, then this bit corresponds to only one combination of a forbidden beam index l'1 in the first dimension and a forbidden beam index l'2 in the second dimension out of the O1N1O2N2 possible combinations of the first beam index l1 and the second beam index l2 .

If we assume a ‘first dimension first’ mapping scheme, then the bit an forbids a 2D beam with indices

l'1=n-N1O1nN1O1

l'2=nN1O1.

Equivalently, n can be calculated from l'1 and l'2 with n=l'1+N1O1l'2.  An example of the first dimension first mapping scheme with N1,N2=(2,2) and O1,O2=(4,4) is illustrated in Figure 2.

Proposal 1:  The nth bit an of the Beam-Subset-Restriction bitmap is used to forbid a 2D beam with indices (l'1 , l'2), where l'1=n-N1O1nN1O1 and l'2=nN1O1.

| Figure 1. A ‘first dimension first’ mapping between Beam-Subset-Restriction bits and 2D beams. |  | Figure 2. Example of ‘first dimension first’ mapping. |
| --- | --- | --- |

## Beam Restriction across Ranks

Considering the rank 1 Class A codebook agreed in [1], a given rank-1 codeword  as defined in (1) of the Appendix is forbidden (i.e., not allowed to be reported), if m1=l'1 and m2=l'2 simultaneously, where the range of (m1,m2) are determined by the value of Config (see [1] for details) and (l'1,l'2) represents the forbidden beam indices of any 2D beam that is not allowed to be reported (as signaled by the Beam-Subset-Restriction bitmap).

For the Class A codebooks of ranks 2, 3, and 4 described in Sections 5.2, 5.3, and 5.4, respectively, a given codeword (which, depending on the rank, can be defined by (4), (5), (6), or (7) in the Appendix) is forbidden (i.e., not allowed to be reported) if either or both of the following two conditions are met

Condition 1:  m1=l'1 and m2=l'2

Condition 2:   m'1=l'1 and m'2=l'2

where possible values of m1,m2 and (m'1,m'2) are determined by the value of Config (see [3]-[4] for details).  Furthermore, (l'1,l'2) represents any 2D beam that is not allowed to be reported (as signaled by the Beam-Subset-Restriction bitmap).

For the rank 5-8 Class A codebooks of Section 5.5, the rules for applying beam restriction has to be formulated slightly differently since codewords for ranks 5-8 are indexed with i1,1 and i1,2 as opposed to the parameters m1, m2, m'1,m'2.  For the rank 5-8 Class A codebooks, a given rank 5-8 codeword is forbidden (i.e. not allowed to be reported) if the codeword contains at least one forbidden 2D beam (as defined jointly by l'1 and l'2). This happens if at least one of the conditions given in Table 1 is met.  In Table 1, the δi,j values are those from the rank 5-8 codebooks which are determined by the parameters N1, N2, and the configuration associated with the codebook (see [4] for details).

The i1,1 and i1,2 indices are the PMI indices corresponding to the first and second dimensions.  The conditions in each column of the table are checked a column at a time according to the rank (denoted by r).  Conditions 1-3 apply to codewords with ranks 5, 6, 7, or 8, while Condition 4 only applies to codewords with ranks 7 or 8. For at least one applicable column (i.e., condition), if forbidden beam index l'1 is equal to the table entry on the first row of the column, and if forbidden beam index l'2 is simultaneously equal to the table entry on the second row of the column, then codeword  is forbidden (i.e., not allowed to be reported).

Table 1: Rank 5-8 Codeword Restriction

|  | Condition 1 | Condition 2 | Condition 3 | Condition 4 |
| --- | --- | --- | --- | --- |
| Rank | 5,6,7,8 | 5,6,7,8 | 5,6,7,8 | 7,8 |
| l'1 | s1i1,1 | s1i1,1+δ1,1 | s1i1,1+δ1,2 | s1i1,1+δ1,3 |
| l'2 | s2i1,2 | s2i1,2+δ2,1 | s2i1,2+δ2,2 | s2i1,2+δ2,3 |

Proposal 2:  Specify the following rules for applying beam restriction of a forbidden 2D beam (l'1 , l'2)  across ranks:

- For rank 1-4: A codeword indicated by the parameters m1, m2 or parameters m1, m2, m'1,m'2  shall not be reported if either or both of these conditions are met:

  - Condition 1:  m1=l'1 and m2=l'2 simultaneously

  - Condition 2:   m'1=l'1 and m'2=l'2 simultaneously

- For rank 5-8: A codeword shall not be reported if at least one of the conditions given in Table 1 are met by both rows corresponding to l'1 and l'2  simultaneously.

## CSR Configuration Conflict Cases

When a UE signaled with more than one of the CSR restrictions ( i.e.  Beam-Subset-Restriction, CLASS A i2 restriction and Rank Restriction)  , different codewords may be forbidden by the different bitmaps.  Some rules are needed in terms of which restriction bitmap has the higher priority in case of conflicts, e.g. a codeword is allowed in one restriction bitmap but not allowed in another restriction bitmap(s).  The more straightforward rule is to not allow a codeword belonging to a particular rank to be reported if the codeword is forbidden by any one of the multiple restriction bitmaps.

Proposal 3:  A codeword belonging to a particular rank is not allowed to be reported if the codeword is forbidden by any one of the multiple CSR restriction bitmaps.

# Conclusion

In this contribution, we provide further details on the specification impact of supporting CSR for FD-MIMO based on the CSR parameters agreed in RAN1#82bis.  We make the following proposals:

Proposal 1:  The nth bit an of the Beam-Subset-Restriction bitmap is used to forbid a 2D beam with indices (l'1 , l'2), where l'1=n-N1O1nN1O1 and l'2=nN1O1.

Proposal 2:  Specify the following rules for applying beam restriction of a forbidden 2D beam (l'1 , l'2)   across ranks:

- For rank 1-4: A codeword indicated by the parameters m1, m2 or parameters m1, m2, m'1,m'2 not be reported if either or both of these conditions are met:

  - Condition 1:  m1=l'1 and m2=l'2 simultaneously

  - Condition 2:   m'1=l'1 and m'2=l'2 simultaneously

- For rank 5-8: A codeword shall not be reported if at least one of the conditions given in Table 1 are met by both rows corresponding to l'1 and l'2  simultaneously

Table 1: Rank 5-8 Codeword Restriction

|  | Condition 1 | Condition 2 | Condition 3 | Condition 4 |
| --- | --- | --- | --- | --- |
| Rank | 5,6,7,8 | 5,6,7,8 | 5,6,7,8 | 7,8 |
| l'1= | s1i1,1 | s1i1,1+δ1,1 | s1i1,1+δ1,2 | s1i1,1+δ1,3 |
| l'2= | s2i1,2 | s2i1,2+δ2,1 | s2i1,2+δ2,2 | s2i1,2+δ2,3 |

Proposal 3:  A codeword belonging to a particular rank is not allowed to be reported if the codeword is forbidden by any one of the multiple CSR restriction bitmaps.

# References

R1-156217, “WF on class A and class B CSI reporting for Rel.13 EB/FD-MIMO”, AT&T, Beijing Xinwei Telecom Tech., CATR, CATT, CHTTL, CMCC, Deutsche Telekom, Ericsson, ETRI, Huawei, HiSilicon, ITRI, Kathrein-Werke KG, KDDI, KT Corporation, Nokia Networks, NTT DOCOMO, Samsung, Sony Corporation, 3GPP TSG-RAN WG1#82bis, Malmö, Sweden, October 5-9, 2015.

RAN1 Chairman’s Notes, 3GPP TSG-RAN WG1#82bis, Malmö, Sweden, October 5-9, 2015.

R1-156335, “Joint Proposal on Rank 2 Codebook for Class A CSI Reporting”, Samsung, Ericsson, 3GPP TSG-RAN WG1#82bis, Malmö, Sweden, October 5-9, 2015.

R1-156390, “Joint proposal on rank 3-8 codebook,” Samsung, Ericsson, NTT DOCOMO, CATT, 3GPP TSG-RAN WG1 #83, Anaheim, USA, November 16-20, 2015.

# Appendix: Class A Codebook Agreements and Proposals

## Rank-1 Class A Codebook

In [1], a class A codebook for rank-1 was agreed.  As per the agreement, a selected rank-1 codeword  can be represented as

| , | (1) |
| --- | --- |

wherein φn=ejπn/2 and Q∈{8, 12,16}.  In (1), the single layer of data is transmitted on the 2-dimensional beam involving the m1th beam in the first dimension and the m2th beam in the second dimension where

| , | (2) |
| --- | --- |
| . | (3) |

Further details of the agreed class A codebook rank-1 codebook can be found in [1].

## Rank-2 Class A Codebook

A class A codebook for rank-2 is proposed in [3].  In [3], a selected rank-2 codeword  can be represented as

| . | (4) |
| --- | --- |

In (4), the first layer of data is transmitted on the 2-dimensional beam involving the m1th beam in the first dimension and the m2th beam in the second dimension; the second layer of data is transmitted on the 2-dimensional beam involving the m1'th beam in the first dimension and the m2'th beam in the second dimension.  Further details of the proposed class A codebook for rank-2 can be found in [3].

## Rank-3 Class A Codebook

A joint proposal for rank-3 class A codebook is presented in [4].  A selected rank-3 codeword can either be represented by  or , where

| , | (5) |
| --- | --- |
| . | (6) |

Further details of the proposed class A codebook for rank-3 can be found in [4].

## Rank-4 Class A Codebook

A class A codebook for rank-4 is proposed in [4].  In [4], a selected rank-4 codeword  can be represented as

| . | (7) |
| --- | --- |

Further details of the proposed class A codebook for rank-4 can be found in [4].

## Class A Codebooks for Ranks 5-8

Class A codebooks for ranks 5-8 are proposed in [4].  A precoding matrix codeword for rank r (r=5,6,7,8) is denoted as .  The precoding matrix codewords , r=5,6,7,8 are then defined as

|  | (8) |
| --- | --- |
|  | (9) |
|  | (10) |
|  | (11) |

In (8)-(11), the values of ,,,,, are determined by the parameters N1, N2, and the configuration associated with the codebook.  Further details of these values can be found in [4].