3GPP TSG RAN WG1 Meeting #84bis                                                            R1-162218

Busan, Korea, 11th – 15th April 2016

Source:	ZTE

Title:	Performance evaluation of MUST Category 2 with 2Tx antennas

Agenda Item:	7.3.4.1

Document for: 	Discussion and Decision

# Introduction

The work item on downlink multiuser superposition transmission for LTE was approved in RAN plenary #71[1]. One of the objectives in [1] is to specify downlink multiuser superposition transmission scheme(s) for MUST Category 2 with multiple transmission power ratios or MUST Category 2 with single transmission power ratio & legacy constellation for co-scheduled MUST users in each constellation combination, therefore, system simulation results of multiple transmission power ratios and single transmission power ratio are compared in this contribution.

# System Simulation Results

In this section, system level simulations of MUST category 2 with multiple transmission power ratios and MUST category 2 with single transmission power ratio on Full buffer model are carried out. Only consider one layer of each UE is paired. Detailed simulation assumptions are described in Table A1.

Near UEs use SIC receiver to remove the interference from edge UEs. Interference to edge UEs can hardly be eliminated.

Alt 1: multiple transmission power ratios, power set of Far UE: [0.99: 0.02: 0.7].

Alt 2: single transmission power ratio in [2]

Table 1 Transmission power ratio for each modulation pairing to achieve legacy QAM in the composite symbol

|  | Far UE | Near UE | Power ratio of Far UE |
| --- | --- | --- | --- |
| 16QAM | QPSK | QPSK | 0.8 |
| 64QAM | QPSK | 16QAM | 0.762 |
| 64QAM | 16QAM | QPSK | 0.952 |
| 256QAM | QPSK | 64QAM | 0.753 |
| 256QAM | 16QAM | 16QAM | 0.941 |
| 256QAM | 64QAM | QPSK | 0.988 |

Table 2 Performance of MUST category 2 with multiple transmission power ratios and MUST category 2 with single transmission power ratio for full buffer model

| Wideband schdulling | Cell spectral efficiency (bps/Hz/cell) | Cell edge user spectral efficiency(bps/Hz/cell/user) |
| --- | --- | --- |
| Alt 1 | 1.3932 | 0.0244 |
| Alt 2 | 1.3980 | 0.0232 |
| Gain | 0.34% | -5.12% |

Observations:

- Compared with MUST category 2 with multiple transmission power ratios, MUST category 2 with single transmission power ratio has lower cell edge performance.

- MUST category 2 with single transmission power ratio adds some restrictions which are not necessary for pairing UEs, so it may not obtain the best performance in some scenarios.

Proposal:

- MUST category 2 with multiple transmission power ratios should be prioritized.

# Conclusions

In this contribution, we provide simulation results of MUST category 2 with multiple transmission power ratios and MUST category 2 with single transmission power ratio on full buffer traffic. Compared with MUST category 2 with multiple transmission power ratios, MUST category 2 with single transmission power ratio has lower cell edge performance .We have the following proposal.

Proposal:

- MUST category 2 with multiple transmission power ratios should be prioritized.

# References

- 3GPP RP-160680, New work item proposal: Downlink Multiuser Superposition Transmission for LTE, MediaTek Inc., CMCC.

- 3GPP R1-157075, Multiuser superposition schemes, Qualcomm Incorporated.

# Annex

Table A1:  System-level simulation assumptions of DL MUST

| Cellular Layout | Hexagonal grid, 3 sectors per site, 7 macro sites |
| --- | --- |
| Inter-site distance | 500 m |
| Minimum distance between BS and UE | 25m |
| Distance-dependent path loss | ITU UMa, with 3D distance between an eNB and a UE applied |
| Penetration loss | For outdoor UEs: 0 dB For indoor UEs: (20+0.5din) dB (din: independent uniform random value between [0, 25] for each link) |
| Shadowing | ITU UMa |
| Fast fading channel between eNB and UE | ITU UMa |
| eNB antenna gain + connector loss | 17 dBi |
| UE antenna gain | 0 dBi |
| Channel Estimation | realistic |
| Channel Measurement | realistic |
| UE speed | 3 km/h |
| Carrier Frequency | 2.0 GHz |
| System bandwidth | 10 MHz |
| Antenna configuration | eNB: 2Tx, 0.5 lambda, cross-polarized UE: 2Rx, 0.5 lambda, cross-polarized |
| Number of UEs per cell | 10 |
| Maximum number of multiplexed UE | 2 ,1ayer/per UE |
| Delay time of scheduling | 6 ms |
| Traffic model | Full buffer, Wideband scheduling |
| UE dropping | 20% UEs are outdoor; 80% UEs are indoor |
| Total BS TX power (total per carrier) | 46 dBm |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| Antenna Height | 25 m |
| UE antenna Height | 1.5 m |
| Codebook | LTE Rel. 8 |
| OLLA | Yes |
| Receiver | MMSE with IRC and SIC |
| EVM | EVM is modeled, 8% TX, 4% RX |
| Duration of the simulation | 5s for full buffer |