3GPP TSG RAN WG1 Meeting #84bis 	R1-162247

Busan, Korea, April 11-15, 2016

Agenda Item:	8.2.1

Source:	Huawei, HiSilicon, Ericsson, NTT DOCOMO, Samsung

Title:	Pathloss modeling for NLOS propagation

Document for:	Discussion and decision

Introduction

In the Channel Modeling Ad hoc meeting, pathloss and shadowing fading modeling were discussed [1]. For LOS propagation case and O2I propagation case, it has been agreed to adopt the research results in [2] and [3]. For NLOS propagation case, there was discussion on how to choose the ABG and CI/CIF model. Pros and cons of both models were discussed [4]. One of the concerns from member companies is to understand the background information about the measurement results and modeling procedure.

In this contribution, the background information about the measurement results and modeling procedure for NLOS pathloss in are introduced based on the information from [5].  The selection of pathloss model is also discussed.

- 2	Pathloss modeling for NLOS propagation.

UMi

For UMi scenarios, the results from 10 companies and universities are summarized as in Table 1. Both measurement and ray-tracing results are included.

Table 1. Summary on UMi pathloss measurement results

Based on the measurement results, pathloss models were derived as in Table 2.

Table 2. pathloss modeling results for UMi street canyon

| Scenarios |  | Parameters |  |  |  | Valid Freq [GHz] | Valid Dist. [m] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n/ |  |  | SF | [min~max] | [min~max] |
| Street Canyon | CI | 3.17 | N/A | N/A | 8.09 | 0.8~73 | 10~959 |
|  | ABG | 3.53 | 22.4 | 2.13 | 7.82 |  |  |

InH

For indoor hotspot, the measurement results are from 8 companies and universities as summarized in Table 3.

Table 3. Summary on indoor hotspot pathloss measurement results

| Source | Scenarios | Frequency band(GHz) |
| --- | --- | --- |
| Aalto Univ. | Shopping mall, LOS/NLOS | 28.5, 63 |
| CMCC | Indoor office, LOS/NLOS | 14, 28 |
| DOCOMO | Indoor office, LOS | 20 |
| Ericsson | Indoor office, LOS/NLOS | 2.44, 5.8, 14.8, 60 |
| Huawei | Indoor office, LOS/NLOS | 28, 73 |
| NYU | Indoor office, LOS/NLOS | 28, 73 |
| Qualcomm | Indoor office, LOS/NLOS | 2.9, 29 |
|  | Shopping mall, LOS/NLOS | 2.9, 29, 61 |
| Samsung | Shopping mall, LOS/NLOS | 28 |

Based on the measurement results, pathloss models were derived as in Table 4 and Table 5.

Table 4. NLOS pathloss modeling results for indoor office

| Scenario |  |  | Parameters | RMSE | Freq. | Dist. |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor office | Dual slope | ABG | =1.7, =33.0, =2.49, =4.17, dBP = 6.9 m | 7.78 | 2.44~73 GHz | 1~86m |
|  |  | CIF | n1=2.51, b1=0.12, f0= 24.1 GHz, n2=4.25, b2=0.04, dBP = 7.8 m | 7.65 |  |  |
|  | Single slope | ABG | =3.83, =17.3, =2.49 | 8.03 |  |  |
|  |  | CIF | n=3.19, b=0.06, f0= 24.2 GHz | 8.29 |  |  |

Table 5. NLOS pathloss modeling results for shopping mall

| Scenario |  |  | Parameters | RMSE | Freq. | Dist. |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor office | Dual slope | ABG | 1=2.9, 1=22.17, 2.24, 2=11.47, dBP = 147.0 m | 6.36 | 2.9~63 GHz | 2~229m |
|  |  | CIF | n1=2.43, b1=-0.01, f0= 39.5 GHz, n2=8.36, b2=0.39, dBP = 110 m | 6.26 |  |  |
|  | Single slope | ABG |  | 6.97 |  |  |
|  |  | CIF | n=2.59, b=0.01, f0= 39.5 GHz | 7.40 |  |  |

UMa

For UMa, the results for both industry and academy are included, the frequency bands for both below 6GHz and above 6GHz band are covered. Both measurement and ray-tracing results are adopted to derive the final results.

Table 6. NLOS pathloss modeling results for UMa street canyon

| Scenarios |  | Parameters |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | n/ |  |  | SF |
| UMa | CI | 3.0 | N/A | N/A | 6.8 |
|  | ABG | 3.4 | 19.2 | 2.3 | 6.5 |

- 3	Considerations on model selection

For model selection, following aspects should be considered:

- Accuracy of the model

For ABG model, it can improve the modeling accuracy and reduce the Shadowing/RMSE during model fitting.

- Backward compatibility with existing models

For current pathloss model used in 3GPP and ITU, it is mainly ABG model.

- Consideration on model robustness

A robust model should be stable when new data introduced. For CI/CIF model, since it is anchored to fixed point FSPL(f, d0) with d0=1m, when data sample is limited, the robustness of CI/CIF model is better. But when data samples are abundant, considering the number of data source, both CI and ABG can provide robust performance.

- Physics behind

For CI model, one thing should be further investigate is the anchor point of FSPL(f, d0) with d0=1m. It is still not clear about the physics for 1m reference distance. Furthermore, there are diffraction loss, vegetation loss and blockage loss of the office furniture in the actual NLOS environment and these losses have own frequency dependency which is not like frequency dependency of free space loss which is utilized in CI model. Therefore, it is not appropriate to use CI model which cannot reflect these influences.

Based on the considerations above, and the shadow fading modeling in [1], Ii is proposed to adopt the pathloss model in table below as working assumption for NLOS propagation.

Table 6. proposed pathloss model for NLOS propagation

| Scenarios |  |  | Pathloss (dB), d in m and f in GHz | σSF(dB) d in m and f in GHz |
| --- | --- | --- | --- | --- |
| UMa |  |  |  | 6.5 |
| UMi | Street canyon |  |  | 7.8 0.8~73GHz, 10~959m |
| InH | Indoor office | Opt.1 |  | 2.44~73GHz, 1~86m |
|  |  | Opt.2 |  | 2.44~73GHz, 1~86m |
|  | Shopping mall | Opt.1 |  | 2.9~63GHz, 2~229m |
|  |  | Opt.2 |  | 2.9~63GHz, 2~229m |

Proposal 1: for NLOS propagation case, the pathloss model in Table 6 should be adopted as working assumption.

Proposal 2: The inclusion of path loss dependency on the UE and eNB height similar to that in the 3D SCM model should be further considered

Proposal 3: The consistency with the 3D SCM model for below 6 GHz should be further considered

- 4	Conclusion

In this contribution, the NLOS propagation pathloss modeling has been discussed. Based on the considerations for model selection, the proposal below is proposed.

Proposal 1: for NLOS propagation case, the pathloss model in Table 6 should be adopted as working assumption.

Proposal 2: The inclusion of path loss dependency on the UE and eNB height similar to that in the 3D SCM model should be further considered

Proposal 3: The consistency with the 3D SCM model for below 6 GHz should be further considered

- References

R1-161718, “Joint proposal on the modelling of Pathloss”, Nokia Networks, AT&T, CMCC, Ericsson, ETRI, Huawei, HiSilicon, Intel, KT Corporation, NTT DOCOMO, Qualcomm, Samsung, Ljubljana, March. 14-16, 2016.

R1-161744, “WF on LOS Pathloss Modeling”, Huawei, HiSilicon, Samsung, Ericsson, DOCOMO, CMCC, Keysight, Fraunhofer HHI, Ljubljana, March. 14-16, 2016.

R1-161737, “WF on outdoor to indoor propagation”, Huawei, HiSilicon, Ericsson, Ljubljana, March. 14-16, 2016.

R1-161723, “WF on NLOS Pathloss Modeling”, Huawei, HiSilicon, Ericsson, Samsung, DOCOMO, CMCC, Keysight, Ljubljana, March. 14-16, 2016.

Aalto University, AT&T, BUPT, CMCC, Ericsson, Huawei, Intel, KT Corporation, Nokia, NTT DOCOMO, New York University, Qualcomm, Samsung, University of Bristol, University of Southern California, “5G Channel Model for bands up to 100 GHz (v2.0)”, Mar. 2016 (http://www.5gworkshops.com/5GCM.html).