3GPP TSG RAN WG1 #84bis	R1-162690

# Busan, Korea, 11th – 15th April 2016

Agenda item:	7.3.3.2.2

Source: 	Samsung

Title: 	CSI reporting for hybrid CSI-RS

Document for:	Discussion and Decision

# Introduction

One of the objectives for Rel.14 WI on enhanced FD_MIMO (eFD-MIMO) is to specify the following three enhancements on CSI reporting [1]:

- Extend specification support for CSI reporting in the following areas [RAN1]

  - Codebook(s) associated with the newly supported number of non-precoded CSI-RS ports for a subset of possible port layouts, both 1D and 2D

  - CSI reporting mechanism to support joint utilization of different CSI-RS types at the UE such as between non-precoded CSI-RS and beamformed CSI-RS as well as between different types of beamformed CSI-RS

  - As second priority, evaluate and, if needed, specify enhancement on CSI reporting based on non-precoded and beamformed CSI-RS to improve eNB precoding (such as new feedback methodologies in addition to codebook-based CSI feedback) and interference measurement to support efficient multi-user transmissions (e.g. further enabling interference estimation from NZP or ZP CSI-RS)

    - Analog feedback is not precluded

This contribution presents Samsung’s view on the second CSI reporting enhancement, that is, joint utilization of different CSI-RS types (also termed hybrid CSI-RS in [2]). In particular, possible options and specification enhancements on this item utilizing one CSI process will be discussed.

# Hybrid CSI-RS

Hybrid CSI-RS is largely motivated by the need for enabling UE-specific beamformed (BF) CSI-RS when sufficiently accurate CSI is not available at the eNB. For TDD, UL-DL short-term reciprocity can be exploited by the eNB to derive UE-specific beamforming for CSI-RS. But this is not applicable for FDD except when UL-DL duplexing distance is sufficiently small to allow long-term reciprocity. To facilitate UE-specific BF CSI-RS for FDD regardless of the eNB capability, the UE can also be configured with non-precoded (NP) CSI-RS in conjunction with BF CSI-RS. The CSI reports associated with NP CSI-RS can be used by the eNB to derive UE-specific beamforming for CSI-RS.

It is known that as the number of TXRUs at the eNB and/or the number of CSI-RS ports increase, BF CSI-RS starts outperforming NP CSI-RS not only due to overhead, but also due to increased CSI-RS penetration from UE-specific beamforming gain (see, e.g. [2]). Therefore,

In Rel.13, this joint usage of NP and UE-specific BF CSI-RS can be implemented with 2 CSI processes where one CSI process is configured with CLASS A eMIMO-Type and the other CSI process with CLASS B eMIMO-Type with 1 CSI-RS resource. However, such an implementation is inefficient for the following reasons:

- Unnecessary CSI reporting overhead: The use of 2 CSI processes imply that the CSI reporting for CLASS A and CLASS B are performed independently. Thus, CQI and RI are to be reported for both CSI processes. Since the purpose of CLASS A is to facilitate eNB UE-specific CSI-RS beamforming, CQI is not needed. The 4- to 7-bit unnecessary payload (which is large considering the 11-bit capacity of PUCCH format 2/2a/2b) degrades coverage. In addition, a complete CLASS A P-CSI reporting requires 3 subframes which makes it susceptible to error propagation and droppings.

- Unnecessary usage of multiple CSI processes: As the number of supportable CSI processes P (per band per band-combination) is a UE capability, CSI process is a limited resource. Since CSI processes are used for various other purposes (e.g. COMP, eCA), the use of multiple CSI processes should be avoided when necessary.

In light of the above drawback of using two CSI processes for hybrid CSI-RS, it is proposed that for Rel.14 eFD-MIMO, one CSI process can be configured with both CLASS A (‘nonPrecoded’) and CLASS B (‘beamformed’) with K=1 NZP CSI-RS resource. To avoid unnecessary CSI reporting overhead, CSI calculation for CLASS B is conditioned on that for CLASS A. Therefore, CSI reporting for CLASS A can be ‘partial’. An example is given in Figure 1 for P-CSI mode 1-1. Here the UE reports only i1 (which can consist of i1,1,i1,2) and RI as it measures the NZP CSI-RS resource associated with CLASS A eMIMO-Type. When the UE measures the NZP CSI-RS resource associated with CLASS B (K=1) eMIMO-Type, the UE reports CQI and i2 conditioned on the last reported value of RI associated with CLASS A reporting. Since the CLASS A CSI is intended to provide UE-specific beamforming information to the eNB, it is reported less frequently compared to the CLASS B CSI.

Therefore, a UE shall measure two CSI-RS resources (or resource configurations) within one CSI process and calculate the second CSI report conditioned upon the first CSI report.

Note that the CLASS A CSI used in this example requires, e.g. format 3 PUCCH since its payload is likely to exceed 11 bits.

For A-CSI, a capability to trigger CLASS A CSI and CLASS B CSI separately can also be beneficial.

Figure 1 Combination 1 of hybrid CSI-RS: CLASS A and CLASS B K=1 (P-CSI mode 1-1)

However, other possible combinations of two eMIMO-Types within one CSI process can be beneficial as well. Two other possibilities, referred as combinations 2 and 3, are listed in Table 1.

In combination 2, the first eMIMO-Type configuration is CLASS B with K>1 CSI-RS resources for implementing cell-specific BF CSI-RS. As the UE measures these K CSI-RS resources, it reports only CRI to the eNB. The CSI-RS resource assigned to the UE (selected based on the CRI feedback) is associated with CLASS B K=1 eMIMO-Type where the UE reports CQI+PMI+RI.

In combination 3, the first eMIMO-Type configuration is CLASS B with K>1 CSI-RS resources for implementing partial-port CSI-RS (see [3]). In this case, CRI feedback is not configured. This use case is useful to support a large number of CSI-RS ports. In principle, it supports a similar CLASS B K=1 operation to combination 1 except that the CSI calculation procedure associated with partial-port CSI-RS quite differs from CLASS A.

Table 1 Possible combinations for hybrid CSI-RS using one CSI process with two eMIMO-Type configs

| Combination | First eMIMO-Type | Second eMIMO-Type |
| --- | --- | --- |
| 1 | CLASS A | CLASS B K=1 |
| 2 | CLASS B K>1 (with CRI reporting, cell-specific BF CSI-RS) | CLASS B K=1 |
| 3 | CLASS B K>1 (without CRI reporting, partial-port CSI-RS [3]) | CLASS B K=1 |

Proposal:

- One CSI process can be configured with both CLASS A (‘nonPrecoded’) and CLASS B (‘beamformed’) K=1 NZP CSI-RS resource

  - P-CSI: CLASS B CSI (CQI+i2) is calculated conditioned on CLASS A CSI (RI+i1)

  - A-CSI: capability to trigger CLASS A CSI and CLASS B CSI separately

- Other combinations between two eMIMO-Type configurations should also be studied. Examples are:

  - CLASS B K>1 (with CRI reporting, cell-specific BF CSI-RS) + CLASS B K=1 (UE-specific BF CSI-RS)

  - CLASS B K>1 (without CRI reporting, partial-port CSI-RS) + CLASS B K=1 (UE-specific BF CSI-RS)

# Conclusions

In this contribution, the motivation for efficiently supporting hybrid CSI-RS (a combination of two eMIMO-Type configurations) is given. Potential specification impacts are also discussed. The proposal can be summarized as follows:

- For Rel.14 eFD-MIMO, one CSI process can be configured with both CLASS A (‘nonPrecoded’) and CLASS B (‘beamformed’) K=1 NZP CSI-RS resource

  - P-CSI: CLASS B CSI (CQI+i2) is calculated conditioned on CLASS A CSI (RI+i1)

  - A-CSI: capability to trigger CLASS A CSI and CLASS B CSI separately

- Other combinations between two eMIMO-Type configurations should also be studied. Examples are:

  - CLASS B K>1 (with CRI reporting, cell-specific BF CSI-RS) + CLASS B K=1 (UE-specific BF CSI-RS)

  - CLASS B K>1 (without CRI reporting, partial-port CSI-RS) + CLASS B K=1 (UE-specific BF CSI-RS)

# References

- RP-160623, New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE, Samsung

- 3GPP, TR36.897v1.0.1, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”

- R1-162685, Port-based CSI-RS Overhead Reduction, Samsung

- 3GPP, TS36.211v13.0.1, “E-UTRA Physical Channels and Modulation”

- 3GPP, TS36.212v13.0.1, “E-UTRA Multiplexing and Channel Coding”

- 3GPP, TS36.213v13.0.2, “E-UTRA Physical Layer Procedure.”